Report on the preliminary OA&M specifications

for the  test-bed

| Status and Version: | Public Deliverable |  |
| --- | --- | --- |
| Date of issue: | 22.09.2000 |  |
| Distribution: |  |  |
| Author(s): | Stefano Brunazzi | SICN |
|  | Carlo Cavazzoni | CSELT |
|  | Roberto Cadeddu | CSELT |
|  | Giampaolo Bendelli | OTC |
| Checked by: | Ennio Turconi | SICN |
|  | Salvatore Rotolo | SICN |
|  | Georg Lehr | T-Nova |
|  | Antonio Manzalini | CSELT |

Table of Contents

1	Introduction	3

1.1	Purpose and Scope	4

1.2	Reference Material	4

1.2.1	Reference Documents	4

1.2.2	Abbreviations	4

1.2.3	Definitions	6

1.3	Document History	6

1.4	Document overview	7

2	Identification of OA&M information for field trial implementation	8

2.1	OA&M information in the OTS layer	8

2.2	OA&M in the OMS layer	10

2.3	OA&M in the OCh layer	11

2.4	Synoptic view of the OA&M flow in the three OTN layers	12

2.5	OA&M flow through CNI	13

3	Physical support for OA&M processes and information flow	15

3.1	OSC (Optical Supervisory Channel)	15

3.2	Channel associated overhead	18

3.2.1	Pilot Tones	18

3.2.2	Digital Wrapper	18

3.3	Capability of detection of physical defects	19

4	Assignment of OA&M functionalities to control entities	21

4.1	OXC by CSELT	21

4.2	OXC by Siemens ICN	22

4.3	OLA	24

5	Preliminary definition of OA&M role for protection/restoration	25

6	Relationship between the OA&M and Management systems	26

7	Implications of interworking functionality on OA&M specification	28

8	Conclusions	30

# Introduction

Under the definition Operation, Administration and Maintenance (OA&M) several functions and processes are intended, which play a critical role in the actual capability to effectively deploy a network. This is particularly true when the objective is to design and demonstrate an advanced multi-client and multi-layer transport network, as the OTN considered in LION is. In this context, the experiences gained in the SDH environment and in OTN standardization (e.g., in G.872) form a consistent basis, which should be however significantly extended.

It is opportune to clarify by the beginning that a theoretical foundation of OA&M topics, as well as a precise definition of which they are, is foreseen for future steps of WP6 activity. Instead, according to the project planning, the initial efforts by WP6 have been focused on the identification of a basic preliminary set of OA&M specifications, on the items directly related to implementation activities. A project internal milestone was associated with this task (WP6 M1).

During this early specification phase, a critical aspect has been the following dichotomy:

- on one hand, detailed requirements from the “theoretical” LION workpackages (in WPG1) were not yet  completely available, since WPG1 activities are still in progress

- on the other hand, the providers of NE equipment and of the control/management systems for the test-bed needed enough information to proceed in the design and implementation of the related hardware and software.

A trade-off has been looked for. The specifications reported in this document are still preliminary, outlining a minimum set of information required by initial implementation activities. Other aspects are intentionally left unspecified to allow a later definition, able to incorporate meaningful results from WPG1 activity.

Further variables came from the more and more rapid changes of the reference scenario for the OTN. New ideas are appearing about the functions and the nature itself of an optical network. For instance, the concepts of a “control plane” and of a signalling-based client-driven connection set-up, once confined in the “switching network” arena, are now considered relevant and possibly useful for the OTN. Just to mention another example, the Multi-Protocol Label Switching approach, originally conceived as an advanced feature for router-based IP networks, is now also considered a promising perspective, under the definition of “Multi-Protocol Lambda Switching”, when applied to the wavelength cross-connection in the OTN.

The evolution roadmap envisaged in LION (in short, a future-proof transport network, where advanced management and OA&M systems allow client-independence and capability of interworking with client layers) is for several aspects compatible with these emerging approaches. Interesting opportunities of enhancing the LION view, by incorporating some of these new ideas, have been identified. Therefore, an in-depth project internal discussion has been started on these topics (see Chapter 7).

This activity is going over the pure scope of WP6, but, although it will mainly cover theoretical aspects, some outcomes for the field trial demonstration are expected too. From the viewpoint of designers responsible for implementation, there are several commonalities in the problems they have to face to ensure OA&M and signalling/routing functions in the field trial. That’s why, these new features are taken into account in this deliverable. Thus, a wide meaning has been assigned to the term OA&M, just for the purpose of this document, covering several aspects to be anyway defined for the LION field trial. Given the preliminary nature of the deliverable, in some cases open issues are just identified, leaving the final specifications for a further step.

In the prosecution of the project, the focus of WP6 will be progressively restricted, as the implementation (WP7) and field trial (WP8 and WP9) workpackages will mark their progresses and WP6 will concentrate its theoretical activity on “pure” OA&M topics.

## Purpose and Scope

This deliverable, according to its definition, is a report of the preliminary OA&M specification activity carried out during the first months of the project. The term OA&M will be intended in a wide sense, for the reasons above mentioned in the “Introduction”.

The deliverable also fulfils a requirement by workpackages WP5 and WP7, where an early input, although not completely detailed, is needed for the activities of implementation of hardware and software sub-systems of the Network Elements to be included in the LION test-bed.

The document, besides its public nature, can be at the same time considered as a “working framework” for the partners responsible for implementation. It will be internally refined up to the final OA&M specifications, which will be summarized in a future WP6 deliverable.

Some theoretical references are included, reflecting the effort of keeping investigation and implementation activities as much as possible aligned, but the present document is not meant to provide theoretical results. A more complete study on the role of OA&M in the OTN and for the interworking between layers will be reported in a future deliverable foreseen in WP6.

## Reference Material

### Reference Documents

- ITU-T Recommendation G.872 “Architecture of optical transport networks” – Draft 1.2 - March 2000

- LION Technical Annex 1 “Description of Work” – Section 9.2 “General Description” – sub-section WPG3

- R. Cadeddu et al. “PEGASO OTN test-bed” – presentation at ECOC’99 – source: CSELT

- J. Luciani et al. "IP over Optical Networks - A Framework"  IETF Internet Draft, Work in Progress, April  2000

- D. Awduche, Y. Rekhter, J. Drake, R. Coltun, "Multi-Protocol Lambda Switching: Combining MPLS Traffic Engineering Control With Optical Crossconnects," Internet Draft, Work in Progress, Nov. 1999.

- Draft of Rec. G.ason, "Architecture for the Automatic Switched Optical Network" (ASON) – update September 2000

- G. Bendelli et al. “PEGASO OTN test-bed” – ECOC’99 proceedings, vol. II, pp. 114-117

- T. Edhag, A. Manzalini et al., “OA&M signalling and fault management” – ACTS DEMON deliverable, WP2 D5 – January 2000

- ITU-T Draft Recommendation G.709 “Network node interface for the optical transport network (OTN)” – V.07 – July 2000

### Abbreviations

| APS | Automatic Protection Switching |
| --- | --- |
| BDI | Backward Defect Indication |
| BQI | Backward Quality Indication |
| CSL | Client Signal Label |
| CNI | Client Network Interface |
| CTP | Connection Termination Point |
| DCC | Data Communication Channel |
| DCh | Digital Channel |
| DCN | Data Communication Networks |
| DW | Digital Wrapper |
| EDFA | Erbium Doped Fibre Amplifier |
| FAS | Frame Alignment Signal |
| FDI | Forward Defect Indication |
| IaDI | Intra – Domain Interface |
| IP-GR | IP GigaRouter |
| IrDI | Inter – Domain Interface |
| LOC | Loss of Continuity |
| LOF | Loss of Frame |
| LOP | Loss of Optical Payload |
| LOS | Loss of Optical Signal |
| LOSC | Loss of Optical Supervisory Channel |
| MPLS | Multi-Protocol Label Switching |
| NC | Node Controller |
| NE | Network Element |
| NM | Network Manager |
| NNI | Network Node Interface |
| OA&M | Operation, Administration & Maintenance |
| OADM | Optical Add Drop Multiplexer |
| OCh | Optical Channel |
| ODU | Optical Channel Data Unit |
| OFA | Optical Fibre Amplifier |
| OH | OverHead |
| OHFIS | OverHead Failure Indication Signal |
| OLA | Optical Line Amplifier |
| OMS | Optical Multiplex Section |
| OPU | Optical Channel Payload Unit |
| OSC | Optical Supervisory Channel |
| OSNR | Optical Signal to Noise Ratio |
| OTN | Optical Transport Network |
| OTS | Optical Transmission Section |
| OTU | Optical Channel Transport Unit |
| OXC | Optical Cross Connect |
| PLC | Power Level Control |
| PLM | PayLoad Mismatch |
| POS | Packet Over SONET |
| PTI | Payload Type Indication |
| SSC | Sub-System Controller |
| SWM | Switching Matrix |
| TCM | Tandem Connection Monitoring |
| TDM | Time Division Multiplexing |
| TIM | Trail-trace Identifier Mismatch |
| TTI | Trail Trace Identifier |
| WDM | Wavelength Division Multiplexing |

### Definitions

It should be noted that the terminology used in this deliverable is still the one adopted at the beginning of the specification activity, for the simplicity’s purpose. One example: OSC (Optical Supervisory Channel) is still used, instead of the definition adopted in the more recent version of G.709 (OAC, i.e., Optical Auxiliary Channel).

Nonetheless, the project partners are monitoring the continuous changes proposed by standardization  (especially, by ITU-T) and considering the possibility of a terminology update for the final specifications.

## Document History

| Date | Author | Comment |
| --- | --- | --- |
| 07/06/2000 | S. Brunazzi | first draft of WP6 M1 document – basis for D5 |
| 27/07/2000 | S. Brunazzi | final draft of WP6 M1 document – basis for D5 |
| 22/08/2000 | C. Cavazzoni, R. Cadeddu, G. Bendelli | Checked version by CSELT Addition of DW specs by OTC |
| 31/08/2000 | S. Brunazzi | Addition of Chapter 7 |
| 15/09/2000 | S. Brunazzi | Draft circulated for a final check by all the project partners |
| 22/09/2000 | All the Authors | Final Version |

## Document overview

The document is structured as follows. Chapter 2 is devoted to the identification of the OA&M information types to be implemented, starting from the general basic list drawn from G.872. Chapter 3 is focused on the physical support for the propagation of OA&M information through the network (the Optical Supervisory Channel and the Digital Wrapper overhead). In Chapter 4 an overview of the control architecture of the optical nodes (in particular, the aspects related to OA&M functions) is reported. A preliminary definition of the role assigned to OA&M in protection procedures is outlined in Chapter 5. In Chapter 6 the topic of inter-relationship between the OA&M and the Management System is considered. Finally, a first investigation on the impact of implementing enhanced interworking functionalities (related to signalling/routing and dynamic connection set-up) is proposed in Chapter 7.

# Identification of OA&M information for field trial implementation

The objective is to identify the types of OA&M information that will be implemented.

As already noted in Section 1, a wide definition for OA&M is considered, to allow an overview, although not exhaustive, on the control/supervision/management information that has to be propagated between optical nodes on the managed Optical Network.

In the following sections, some tables summarize the OA&M information flow in the OTS, OMS, OCh layers, respectively, and between a client network and the OTN, through CNI. The “Network level management requirements” table, included in G.872 [1], has been adopted as a guideline. Some rows are added to that list, where relevant, as possible original contributions by LION (their content is marked in bold red characters).

In each table, it is specified whether a certain kind of OA&M information will be implemented or not in the field trial. The dashed grey cells are related to types of information that are not relevant for the considered optical layer, according to G.872.

In the column “PEGASO”, a reference is done to the current situation in the pre-existing test-bed [2], on which the LION test-bed will be built [3]. The short name “PEGASO” will be used hereafter, for simplicity. Some features, already existing in “PEGASO”, will be extended to the overall LION field trial; some others, not currently present, will be added.

For each row, for each layer, the OA&M messages usually related to that kind of OA&M information are reported too, but only as a reference. The exact naming to be adopted for the OA&M messages is still under agreement among partners, taking into account the opportunity of aligning as much as possible the used terminology with existing and emerging standards.

## OA&M information in the OTS layer

| Type of OA&M info (G.872 + new items) | Related messaging | PEGASO | LION overall test-bed | Notes |
| --- | --- | --- | --- | --- |
| Connectivity Supervision Info | Trail Trace Id. (TTI) | Yes | Yes |  |
| Maintenance Information | Forw. Def. Indic. (FDI) Backw. Def. Ind. (BDI) Backw. Qual.Ind. (BQI) OH Failure Ind. (OHFIS) | Yes Yes No No | Yes Yes No Yes | Note 2.1.1 |
| Signal Quality Supervision | Performance monitor | No | No | see chapter 3.3 |
| Adaptation Management | Payload Type Indication | n.a. | n.a. |  |
| Protection Control | APS control Hold-off / interworking | n.r. | n.r. |  |
| Subnet/tandem Connection Supervision | Connection supervision | f.f.s. | f.f.s |  |
| Management Communications | DCC Management Ch. | Yes | Yes |  |
| Remote Node Supervision | - | No | Yes | Note 2.1.2 |
| Routing/Reconfig. Signalling | t.b.d. | No | Possibly | Note 2.1.3 |

Table 1 - OA&M information in the OTS layer

Note 2.1.1

As “Maintenance Information”, an “OverHead Failure Indication Signal” message has been introduced, related to the loss of an overhead/supervisory channel. Its usefulness was proved in previous research projects (e.g., ACTS MOON). However, two different kinds of anomalies – which should not be confused - were implied under this name:

- Loss of the optical supervisory channel

- Loss of the overhead in a particular layer (OTS-OHFIS, OMS-OHFIS, OCh-OHFIS),  independently on how it is transported (e.g., due to a failure in the related termination)

Only the first one is implemented in the LION field trial, i.e. Loss of Optical Supervisory Channel (pertaining to the OTS layer). This message will implicitly indicate the loss of the overall overhead of OTS and OMS layers, and the loss of that part of the OCh overhead transported by OSC.

Note 2.1.2

The “Remote Node Supervision” is an additional type of OA&M information with respect to the G.872 list. The term is referred to a flow of basic control/supervision information coming from remote nodes not provided with a local NE Agent. In the LION field trial, an example of such “low hierarchy” node (that may also exist in a real network) is the bi-directional Optical Line Amplifier (OLA) node, placed along one of the WDM links between the two OXCs.

From a logical viewpoint, the “Remote Node Supervision” information plays the same role as the "low level" control flow, within the OXC, between the microprocessors of different boards and a Node Controller; this is also used as basic information by the NE Agent. The difference is that, unlike the OXC, the OLA has no local NE Agent. The NE Agent functions for the OLA are implemented in one of the two adjacent OXCs. Consequently, the OLA needs a way to propagate this information to the remote OXC where its NE Agent is implemented. A solution is just offered by the OA&M flow here defined as “Remote Node Supervision”.

In principle, this flow has not to be confused with the information between Agents and Manager (defined in the table as “Management Communications”), which belongs to a quite different level of abstraction. From a practical viewpoint, both kinds of information will be propagated through the same DCC, transported via OSC, because this solution has been recognized as effective and relatively simple (see Section 3.1).

Note 2.1.3

A further additional type of OA&M flow, with respect to [1], is the routing/reconfiguration signalling to be exchanged between optical nodes. The nature as well as the importance of this kind of information, to support interworking between layers and the development of an “intelligent” OTN, have already been outlined in Section 1. There, the reasons of considering it, although not strictly belonging to OA&M, are also explained.

Under the name “routing/reconfiguration signalling” are here collected different types of information such as:

- messages belonging to “routing protocols” to be developed in the optical layer, like the well established ones in the IP layer;

- messages related to “Multi Protocol Lambda Switching” to be developed in the optical layer, like the emerging MPLS protocols (see for example [4] and [5]);

- signalling for the client-driven dynamic connection set-up, as envisaged in G.ason (see [6])

A more precise specification depends on the results of an internal project investigation on this topic (see Chapter 7). Here, just some remarks from the particular WP6 perspective are reported:

- a full implementation of “routing protocols” and “optical intelligence” capabilities in the LION OXCs, strictly speaking, goes beyond the original LION work plan, and seems to be unrealistic, because of practical restrictions due to legacy equipment. Nonetheless, the project aims at implementing a partial sub-set of these features. Before going into deeper details in the signalling definition, an activity has been started to identify which tests are sufficiently significant to prove interesting functionalities and, at the same time, feasible for the field trial;

- as a positive contribution on this item, WP6 has specified the way to support the physical exchange of “routing/reconfiguration signalling” between optical nodes (independently on the logical content), i.e., through a message-based communication via DCC in the OTS layer. This is consistent with an emerging – although not definitive - standardization trend (e.g., explicit proposals of using DCC and not an in-band solution to carry ASON signalling have been recently presented in ITU-T Q19.13).

Note 2.1.4

While classifying OA&M for the purpose of specifications, “Management Communications”, “Remote Node Supervision”, “Routing/Reconfiguration Signalling” are considered as pertaining to the OTS layer. This is consistent with the fact that these kinds of information will be carried by a DCC transported via OSC. It should be however noted that these types of OA&M flow, not strictly bound to OTN layering functions, could be as well gathered under the category “management overlay network” information, according to recent proposals put forward in the context of ITU-T.

## OA&M in the OMS layer

| Type of OA&M info (G.872 + new items) | Related messaging | PEGASO | LION overall test-bed | Notes |
| --- | --- | --- | --- | --- |
| Connectivity Supervision Info | Trail Trace Id. (TTI) | n.r. | n.r. |  |
| Maintenance Information | Forw. Def. Indic. (FDI) Backw. Def. Ind. (BDI) Backw. Qual.Ind. (BQI) OH Failure Ind. (OHFIS) | Yes Yes No No | Yes Yes No No | Note 2.2.1 |
| Signal Quality Supervision | Performance monitor | f.f.s. | f.f.s. |  |
| Adaptation Management | Payload Type Indication | f.f.s. | f.f.s |  |
| Protection Control | APS control Hold-off / interworking | Yes No | Yes No | See chapter 5 |
| Subnet/tandem Connect. Supervision | Connection superv. | f.f.s. | f.f.s |  |
| Management Communications | DCC Management Ch. | f.f.s. | f.f.s. |  |

Table 2 - OA&M information in the OMS layer

Note 2.2.1

The meaning of OHFIS, in this case, would be the Loss of OMS OverHead [ref. to note 2.1.1]. This could be significant if the OMS-OH termination were in a different sub-system with respect to the OSC Termination, and therefore OMS-OHFIS would be decoupled from OTS-Loss Of Supervisory Channel: it might happen that the OSC is active, but the OMS-OH is unavailable because the OMS Termination fails. In the LION test-bed, this message will not be implemented; the loss of OMS overhead will be deduced only in case of loss of the OSC that carries it.

## OA&M in the OCh layer

In Table 3 the OA&M flow in the OCh layer is reported. OCh sub-layering is not considered here, as far as OA&M message classification is concerned.

The concept of OCh sub-layering is particularly related to the issue of sub-network/tandem connection monitoring. For this function, LION will exploit the Digital Wrapper (DW). The DW implementation in LION will be as far as possible aligned with G.709 (see Section 3.2.2). In G.709 the OCh sub-layering is considered, in that the OCh structure consists of OTU (Optical Channel Transport Unit), ODU (Optical Channel Data Unit), OPU (Optical Channel Payload Unit). In this sense, the OCh sub-layering will be implicitly taken into account in LION. An explicit investigation of some different proposals for OCh sub-layering (e.g., the one configured as DCh, TCM, OCh) is carried out within the WP6 theoretical activity.

| Type of OA&M info (G.872 + new items) | Related messaging | PEGASO | LION overall test-bed | Notes |
| --- | --- | --- | --- | --- |
| Connectivity Supervision Info | Trail Trace Id. (TTI) | Yes | Yes | By DW |
| Maintenance Information | Forw. Def. Indic. (FDI) Alarm Ind. Sig. (AIS) Backw. Def. Ind. (BDI) Backw. Qual.Ind. (BQI) OH Failure Ind. (OHFIS) | Yes No Yes No No | Yes Yes Yes Yes No | Note 2.3.2 Note 2.3.2 By OSC/DW By DW Note 2.3.3 |
| Signal Quality Supervision | Performance monitor | Yes | Yes | By DW see chapter 3.3 |
| Adaptation Management | Payload Type Indic. Unequipped | Yes Yes | Yes Yes | By OSC/DW By OSC/DW |
| Protection Control | APS control Hold-off / interworking | No No | No No | see chapter 5 |
| Subnet/tandem Connect. Supervision | Connection supervision | Yes | Yes | By DW |
| Management Communications | DCC Management Ch. | n.r. | n.r. |  |
| Routing/Reconfig. Signalling | t.b.d. | No | No | Note 2.3.4 |

Table 3 - OA&M information in the OCh layer

##### Note 2.3.1

While OTS and OMS overheads will be entirely transported by OSC, the OCh overhead will be carried partially by OSC, partially by DW, as indicated in the column “Notes”. The issue will be  considered in chapter 3.

##### Note 2.3.2

As stated in Rec. G.709 [9]: “AIS and FDI are similar signals. AIS is used as term when the signal is in the digital domain. FDI is used as term when the signal is in the optical domain; FDI is transported as non-associated overhead in the OTM Overhead Signal (OOS)”. In LION, the OCh-FDI will be transported by OSC; the OCh-AIS will be carried by DW.

##### Note 2.3.3

The meaning of OHFIS in this case would be the Loss of OCh Overhead. This could be significant to indicate the loss of that part of the OCh-OH carried by DW (the loss of the part carried by OSC can be deduced by the OTS-LOSC). However, it has been considered that this message would be of limited importance in the test-bed, and therefore it will not be implemented.

##### Note 2.3.4

General remarks on this issue were reported in note 2.1.3. In this table, it is just stated that signalling/routing information will not be transported in the OCh-associated overhead.

## Synoptic view of the OA&M flow in the three OTN layers

The OA&M information flow that will be implemented in the LION field trial (already reported in the previous tables) is synoptically summarized in Table 4.

| Layer | OA&M information type |
| --- | --- |
| OTS | Trail Trace Identifier (TTI) |
| OTS | Forward Defect Indication (FDI) |
| OTS | Backward Defect Indication (BDI) |
| OTS | OverHead Failure Indication Signal (OHFIS) – in the form of an FDI related to the OTS-LOSC (Loss of Optical Supervisory Channel) |
| OTS | Management Communication between NE Agents and NE Manager – through DCC |
| OTS | Remote Node Supervision info (only to/from the OLA node) – through DCC |
| OTS | Routing / Reconfiguration Signalling - through DCC |
| OMS | Forward Defect Indication (FDI) |
| OMS | Backward Defect Indication (BDI) |
| OMS | Automatic Protection Switching (APS) protocol |
| OCh | Trail Trace Identifier (TTI) |
| OCh | Forward Defect Indication (FDI) |
| OCh | Alarm Indication Signal (AIS) |
| OCh | Backward Defect Indication (BDI) |
| OCh | Backward Quality Indication (BQI) |
| OCh | Performance Monitoring information |
| OCh | Payload Type Indication – in the form of a Client Signal Label |
| OCh | Unequipped |
| OCh | Subnetwork / Tandem Connection Supervision |

Table 4 – Summary of the OA&M information types to be implemented in the LION field trial

## OA&M flow through CNI

The definition of a minimum/optimum set of OA&M messages to be exchanged between different layers (actually, between Network Elements, belonging to different layers) is a key task for LION. In the following table, a preliminary specification is proposed.

In principle, a generic CNI has been considered. In other words, both SDH and IP clients are taken into account, consistently with the multi-client nature of the LION OTN.

However, when going into details, some differences emerge. Mainly, the “Routing/Reconfiguration Signalling” is more relevant in the case of an IP client; therefore, when focusing the attention on this kind of information flow, the following significant scenario is considered: OTN is a “server layer”; IP is a “client layer”. Accordingly, the Optical Cross-Connect (OXC) is a “server node”; the IP Giga-Router (IP-GR) is a “client node”. This terminology will be used hereafter.

| Type of OA&M info (G.872 + new items) | Related messaging | Flow through CNI | Notes |
| --- | --- | --- | --- |
| Connectivity Supervision Info | Trail Identifier Mismatch (TIM) | No |  |
| Maintenance Information | Client AIS Other particular frames recognized by the client equipment Forward Def. Indic. (FDI) Backward Def. Ind. (BDI) Backward Qual.Ind. (BQI) | Yes Yes No No No | Note 2.5.1 |
| Adaptation Management | Client Type Indication | f.f.s. |  |
| Protection Control | APS control Hold-off / interworking | No Possibly | see chapter 5 Note 2.5.2 |
| Routing/Reconfig. Signalling | t.b.d. | Yes | Note 2.5.3 |

Table 5 - OA&M flow through CNI

Note 2.5.1

This message is used to inform the client about a detected defect in the optical channel layer; it may also trigger control plane actions in the client equipment. The generation of a Client-AIS is often related to the detection of FDI in the associated OCh; however, OCh-FDI can not directly flow to the client layer, because, strictly speaking, it is terminated by the OCh trail termination; only a particular “alarm” frame (such as Client AIS) can be propagated to the client.

Note 2.5.2

The point is still open because the actual specification of the “hold-off/interworking” protection messages will follow the completion of WP2 activity, and is subject to a more detailed definition of the modifications that are possible on the IP-GR for the field trial.

Note 2.5.3

The same general remarks already reported in the note 2.1.3 are valid. A difference is that a local information exchange between the client and the server nodes (IP-GR and OXC) may be easier to be implemented than the signalling flow between the whole OTN considered in note 2.1.3. Some relatively simple test cases can be envisaged, such as a forced reconfiguration of the OXC switching matrix, driven by the IP-GR through either an explicit message or an implicit request. This might be significant to demonstrate an alternative way of configuring a transport node, besides the normal way (via Network Management) that will be anyway available. From an implementation viewpoint, the exact definition of the functionalities to be guaranteed is for further discussion, because these imply more or less complex upgrades of both the IP-GR and the OXC control systems. The possibility of some modifications on both sides has been in principle envisaged, although possible practical restrictions have to be taken into account.

Note 2.5.4

A first specification result has been fulfilled: the identification of a way to support the local OA&M flow between the IP-GR and the OXC (i.e., between client and server layers, logically through a CNI). The solution is based on a common access to an Ethernet bus by both the OXC and the IP-GR control systems. A message-based IP over Ethernet protocol will run over this interface. It is worth noting that, in this solution, the “Digital Wrapper” transponder that implements the main CNI functions is physically by-passed by the inter-layer OA&M flow. This does not prevent from defining this inter-layer OA&M flow as “flow through CNI”, from a logical viewpoint.

Note 2.5.5

The “companion” topic, i.e., the definition of OA&M flow through NNI is currently under study. The framework defined up to now will allow great flexibility in the future specification, thanks to the following foreseen features:

- the IaDI NNI Digital Wrapper technique will be implemented – following the G.709 approach

- all the information carried by DCC via OSC, including routing/reconfiguration signalling, will be fully available at NE where NNI is implemented

# Physical support for OA&M processes and information flow

The necessary pre-requisite for OA&M implementation is the availability of a physical support for the propagation of the OA&M flows on the optical network. The capability of monitoring the status of the network (and of the equipment it is composed of) is also essential for OA&M processes.

As general specification guidelines, it has been decided that the OA&M information will be for the major part carried by an Optical Supervisory Channel (OSC), with the exception of a sub-set of the OCh layer overhead, for which a better solution is available, thanks to the implementation in LION of the Digital Wrapper (DW) technique.

## OSC (Optical Supervisory Channel)

A first basic decision has been to extend the “PEGASO” OSC solution [7] to the whole LION OTN field trial. This guarantees consistency between the legacy equipment and the new parts of the test-bed. This decision also implies the necessity of integrating the “PEGASO” OSC-Terminations (OSC-T) in the new nodes for the field trial, provided by Siemens ICN (OXC and OLA).

The OSC in LION is an out-of-band channel at 1510 nm (the term out-of-band is used here to indicate the optical spectral region outside the 1550 nm amplifying bandwidth of the EDFA optical amplifiers).

In each OSC-T, interfaced to a bi-directional WDM port, WDM multiplexers and demultiplexers are included, to separate and recombine the 1550 nm WDM payload from the 1510 nm OSC.

The structure of the logical channels carried by OSC is shown in Figure 1 and then described.

Figure 1 – The logical structure of the Optical Supervisory Channel

A 2.048 Mb/s signal is carried by OSC, in a frame composed of thirty-two 64 kb/s Time-Slots (TS).

The TDM logical channels are organized as follows:

- FAS (Frame Alignment Signal)		one TS (TS0)  	– with a 64 kb/s capacity

- OTS-OH (OTS-OverHead)			two TS (TS 1-2) 	– with a 128 kb/s capacity

- the first time slot is used for OTS-TTI

- the second time slot is used for OTS OA&M messages (FDI and BDI)

- three bits are devoted to three different kinds of OTS-FDI

OTS-FDI-OMU-LOC (Optical Multiplex Unit Loss Of Continuity), i.e., the loss of continuity of the WDM payload (the 1550 nm bandwidth wavelengths)

OTS-FDI-OSC-LOC (Optical Supervisory Channel Loss Of Continuity), i.e., the loss of continuity of the 1510 nm OSC

OTS-FDI-OTM-LOC (Optical Transport Module - Loss Of Continuity), i.e., the loss of the overall optical signal (“OMU-LOC” AND “OSC-LOC”)

- one bit for OTS-BDI

- four bits left “for further use”

These OTS-OH bytes are the practical implementation of the OA&M information reported in the first four rows of Table 4.

- OMS-OH (OMS-OverHead)			two TS (TS 3-4) 		– with a 128 kb/s capacity

- the first time slot carries an OMS-Index, i.e., the number of multiplexed wavelengths

- the second time slot is used for OMS OA&M messages (BDI) and Automatic Protection Switching (APS) protocol

- one bit is used for OMS-BDI

- two bits are used for APS (i.e., for the protocol needed for protection in the “OADM-based ring” part of the field trial)

- five bits are for further use

- the OMS-FDI is indicated as an “all ONEs” signal in TS3.

These OMS-OH bytes are the practical implementation of the overall OMS OA&M information reported in Table 2.

- OCh-OH (OCh-OverHead)				nine TS (TS 5-13) 	- with a 576 kb/s capacity

This channel transports a sub-set of the OCh overhead, for all the channels belonging to the WDM payload associated with the OSC. It should be recalled once more that the remaining part of the OCh-OH overhead is propagated through the channel-associated DW. The actual implementation is thought for a WDM payload composed of a maximum of eight channels (i.e., eight wavelengths). This is an acceptable assumption for the field trial.

  - One time slot is used for OCh OA&M messages; more specifically, each bit is used for the OCh-BDI message related to one of the eight optical WDM channels

  - Each of the other eight time slots is used to carry OCh-Identifier (14 bytes) and Client Signal Label (2 bytes) for one of the eight optical channels, with a 16-frame long multi-frame

  - the OCh-FDI is indicated as an “all ONEs” signal in the Client Signal Label

  - the OCh Unequipped  is indicated as an “all ZEROs” signal in the Client Signal Label

- DCC (Data Communication Channel)		twelve TS (TS 17-28) – with a 768 kb/s capacity

From a conceptual viewpoint, the DCC is considered a part of the OTS layer overhead (see Table 1). Actually, it will transport:

    - Management information, i.e., the information flow between a NE Agent and a NE Manager, when not co-located;

    - Remote Node Supervision information, i.e., the information flow between the control system of a low-hierarchy node (where no local NE Agent is loaded), and a higher-hierarchy node where a NE Agent for the remote node is available – see note 2.1.2.

Moreover, this DCC is designed to carry possible additional information (e.g., some kind of signalling information, as in note 2.1.3), thanks to the peculiar feature it offers: the capability of transporting several different types of OA&M information, under the form of IP messages. This is possible because each OSC-T is supplied with a service IP router, i.e., a “DCN router” to which the DCC is interfaced (see the scheme of the OSC-T structure reported in Figure 2). The DCN router is then interfaced to the local control system.

Thus, the implementation of possible additional features will require only a software upgrade in the peer control systems which need to communicate, without implying the enhancement of the overall supervisory system: a potential communication channel is already available.

In other words, the OSC will act as a physical support for two distinct control/supervision networks, i.e., a TDM based supervision network carrying fixed bit-rate overhead channels (OTS-OH, OMS-OH, OCh-OH) and an IP-based DCN. Any signalling that may be implemented between network elements of the LION network (e.g., channel set up, or distribution of routing tables among the OXC) will take place over the IP-based-DCN. This allows more flexibility in the design.

Figure 2 – Scheme of the OSC-T structure

- The remaining six TS (TS 14-16 and TS 29-31), corresponding to a free capacity of 384 kb/s, are left “for further use” (FFU).

## Channel associated overhead

### Pilot Tones

Pilot tones were used in the “PEGASO” trial to transport a limited sub-set of OCh overhead. This feature will remain in LION, only in the equipment coming from the PEGASO trial that will be still equipped with 2R transponders. No further functionality will rely on pilot tones other than a possible usage of monitoring of analogue power, OSNR and crosstalk.

### Digital Wrapper

The specification activity about Digital Wrapper implementation is in progress in the context of WP7 activity. However, no specification document is foreseen in the WP7 framework, in this phase of the project. Therefore, it has been decided to include here a summary of the preliminary DW specifications, as defined up to now, even if this goes over the strict WP6 scope.

Indeed, DW is essential for the OA&M signalling and for possible interworking between OTN and client networks; the DW and OA&M implementation activities are strictly related.

Some basic decisions, relevant for OA&M, have been taken up to now:

  - the Digital Wrapper (DW) technique will be implemented, since it is recognized as one of the major milestones towards the deployment of a client-independent OTN. In LION, the DW implementation is possible thanks to the development of NNI (between the ring and the mesh sections of the OTN) and CNI (between optical nodes and client nodes) including 3R-Regeneration and full OCh Termination functions;

  - the Digital Wrapper will be used to transport a sub-set of OA&M information, more specifically, a part of the Optical Channel layer overhead. In spite of the DW implementation, it is anyway opportune the availability of a channel, within the OSC, to carry the other part of OCh overhead (see Section 3.1);

  - A high-level specification concerning an effective division of OCh overhead content between OSC and DW is summarized in the following table (mirroring the column “Notes” of Table 3).

| OCh-OH information | Carried by |
| --- | --- |
| Channel Identifier | OSC |
| TTI | DW/Pilot tone |
| FDI | DW |
| AIS | DW |
| BDI | DW/OSC |
| Client Signal Label / PTI | OSC/DW |
| UNEQ | OSC/DW |
| Subnetwork / tandem connection supervision | DW |
| BQI OA&M messages associated with performance monitoring | DW |

Table 6 - Partition of OCh-OH info between OSC and DW

Preliminary specifications for Digital Wrapper interfaces in the LION test bed are also summarized here below. They constitute on one side the guidelines that WP7 is following, and, on the other side, an input to WP8 about the functionalities that are going to be realized in the LION test bed.

These specifications reflect a trade-off between functionalities and implementation constraints.

- DW interfaces type: CNI and NNI-IaDI;

- DW-clients: POS-STM16; SDH-STM16;

- DW bit-rate: 2.66 Gb/s;

- DW framing: in line with G.709 rec.;

- V.35 interface for access to one of the general communication channels supplied by G.709 frame; the V.35 interface is foreseen in order to give the DW a flexibility towards new future functionalities (e.g., interworking with a client which implements the G.709 frame);

- FEC is not implemented, because it has a heavy impact on hardware development; the decision also reflects the fact that the focus in LION is on OA&M functionalities rather than on physical transmission features.

From the above specifications, it can be deduced that the client-bit rate will actually be 2.48832 Gb/s (i.e., STM16-like).

As far as interworking between OTN and Client networks is concerned, the following functionalities are considered:

- TX power ON/OFF control, as a mandatory feature;

- generation of specific bit sequences toward the client -  this is an optional feature that could be useful to have the flexibility to emulate specific defects towards the client;

- read/write of the SDH-RSOH bytes (optional).

## Capability of detection of physical defects

In the following table, a preliminary list of the physical defects to be detected as a basis for OA&M processes is reported. Equipment providers need the results, to know the features required to the different types of nodes. A first basis for the compilation of Table 7 has been the experience of a previous research project (ACTS DEMON – see [8], chapter 3).

The logical dependence between physical defects and OA&M specific messages is not reported. This will be the matter for a next WP6 activity, towards final OA&M specifications.

For each kind of physical defect, the points where detection is required are highlighted: these can be a CNI, a NNI, a transparent OCh connection point (e.g., before or after the optical matrix in the OXC), or a WDM port (at OXC, OADM and OLA nodes).

As far as this classification is concerned, OCh sub-layering is not considered.

The important topic of the definition of timing constraints for defect detection is still open, for further study.

| Defect | Meaning | at CNI | at 3R-NNI | at transparent OCh CTP | at WDM ports |
| --- | --- | --- | --- | --- | --- |
| OTS-LOS | Loss of continuity of WDM payload & OSC – i.e., loss of the overall optical power | X | X | X | X |
| OTS-LOP | Loss of continuity of WDM optical payload – i.e., loss of the WDM payload optical power | X | X | X | X |
| OTS-LOSC | Loss of continuity of OSC – i.e., loss of the OSC optical power | X | X | X | X |
| OCh-LOC | Loss of continuity of OCh – i.e., loss of optical power related to a single OCh | X | X | X |  |
| OCh-DEG | degrad. of OCh perform. – digital detection | X | X |  |  |
| OCh-TIM | Trail Trace Identifier Mismatch | X | X |  |  |
| OCh-PLM | PayLoad Mismatch | X | X | X |  |
| OCh-LOF | Loss Of Frame - digital detection | X | X |  |  |
| CS-LOC | Loss Of Cont. of Client Signal – dig. detect. | X |  |  |  |
| CS-LOF | Loss Of Frame of Client Signal -dig. detect. (see note 3.3.2) | X |  |  |  |

Table 7 – List of physical defects to be detected

Note 3.3.1

The topic dealt with in this section is strictly related to Performance Monitoring issues. The following approach has been adopted.

- A basic performance monitoring (mainly, on the optical power) will be ensured at OTS and OMS layers. This will enable effective defect detection. On the contrary, a more detailed performance monitoring will not be done at OTS and OMS layers because:

- not all the optical nodes in the trial include instruments for the optical frequency measure;

- the Digital Wrapper technique will be implemented in LION, so that performance monitoring issues will be effectively faced, in the digital domain, at the OCh layer.

As a consequence, OTS-DEG and OMS-DEG defects are not considered in Table 7, nor anomalies related to channel frequency deviations.

- Advanced performance monitoring (and Signal Quality Supervision) will be ensured at the OCh layer. Besides the optical power monitoring (anyway foreseen), the Digital Wrapper will allow a “digital” performance monitoring. Accordingly, OCh-DEG and OCh-LOF defects are considered in Table 7. Optical measures of crosstalk and OSNR will be performed only in the legacy equipment where pilot tones are implemented (see section 3.2.1). These last measures will not be extended to the overall LION field trial, because of the limited dimensions of “transparent islands” in the actual test-bed, that makes more than sufficient the DW-based monitoring, which provides moreover an enhanced information.

Note 3.3.2

The monitoring of client signal frame will be ensured by examining the Regenerator Section of the client signal. This can be done because the actual client signals will be SDH or “SDH-like” (POS).

# Assignment of OA&M functionalities to control entities

The implementation of OA&M features is strictly related to the control architecture and functionality of the optical nodes.

A complete description of node control systems is out of the scope of this document. However, the basic decisions agreed up to now, in WP6 and WP7, are summarized for the different optical nodes that will be included in the test-bed. The two different OXCs, provided by CSELT and Siemens ICN, are distinguished by the short name of the supplier (CSELT OXC, SICN OXC).

Several aspects are common to the node control architecture; nonetheless, they are repeated in the different sub-section, for clarity’s purpose.

## OXC by CSELT

A simplified scheme of the control architecture of CSELT OXC is reported in  Figure 3.

The “lower level” sub-system controllers - i.e.,  the controllers of optical node sub-systems, of the client-to-OTN transponders and of the OSC Termination - are responsible for the detection of primary defects.

At a “Node Control” level, the information from lower level controllers is collected and the necessary correlations to generate OA&M information and drive OA&M processes are performed. This includes the logical functions of OTS, OMS and OCh OverHead Terminations. In the CSELT OXC (as well as in CSELT OADM) these functions will be implemented through a set of software processes running on the Node Controller (NC). The NC, besides its principal functions, performs also the function of “OA&M Controller”.

In order to act as OTS-OH-T and OMS-OH-T, the NC is interfaced to the OSC-T (which performs the ”physical” termination of the OSC and the extraction of the TDM slots related to OTS-OH and OMS-OH). In order to ensure the functions of OCh-OH-T, the NC is interfaced to both the OSC-T (which performs the ”physical” termination of the OSC and the extraction of the TDM slots related to OCh-OH) and the controller of the transponder implementing DW.

At an “Agent” level, a PC is used to implement the NE Agent, which is interfaced to the NC. The NC has the further task of taking care of the interrelationships between OA&M and NE Management systems, by properly filtering the OA&M information (see chapter 5).

As far as OA&M interworking between layers is concerned, an interface between the Client Node and the Optical Node is foreseen. For this task, it has been decided to by-pass the “lower level” CNI/DW controller, and to implement an IP over Ethernet interface (with a message based protocol) between the control system of the client node (i.e., IP-GR) and the control system of the optical node (see section 2.5, note 2.5.3). In the CSELT OXC the same PC where the NE Agent runs is also used for this purpose (it will be enhanced with the proper interface and the necessary software processes).

In Figure 3, the term “client” is used exclusively with reference to the “client network” (i.e., IP) and “client node” (i.e., IP-GR) to which the OTN and the OXC, respectively, offer a “transport service”.

The “info to/from NE Agent” is distinguished into three different green lines (related to the “OA&M info for the Management System”, “Info exchange between node control and NE Agent”, “Manager/Agent communication”, respectively) to highlight the different logical contents. Actually, the flows are merged, using the same communication channel (physically, the Ethernet bus).

Figure 3 – Scheme of the control architecture aspects relevant for OA&M – (CSELT OXC)

## OXC by Siemens ICN

A simplified scheme of the control architecture of the OXC by Siemens ICN (SICN OXC) is reported in Figure 4.

The “lower level” sub-system controllers are responsible for the detection of primary defects. This category includes:

- the controllers of the basic optical node sub-systems: Optical Amplifiers (OFA), Optical Attenuators and Power Equalizers (indicated in Figure 4 as PLC, i.e., Power Level Controllers), Optical Switch Matrix (SWM);

- the controller of the transponder between IP-GR and OXC, implementing CNI functions and Digital Wrapper;

- the controller of the OSC Terminations (equal to the ones implemented in the CSELT OXC), which will be integrated in the SICN OXC.

At a “Node Control” level, the information from all these lower level controllers is collected and the necessary correlations to generate OA&M information and drive OA&M processes are performed. This includes the logical functions of OTS, OMS and OCh OverHead Terminations. In the SICN OXC, this task will be accomplished by a dedicated controller (defined “OA&M Controller”), available in addition to the System Controller (“SSC”); in this case, a proper communication is necessary between the SSC and the OA&M-Controller, implemented as a message based protocol on a physical RS-232 interface.

The “OA&M Controller” is interfaced to the OSC-T (which performs the ”physical” termination of the OSC and the extraction of the TDM slots related to OTS-OH and OMS-OH) to guarantee the functions of  OTS-OH-T and OMS-OH-T. In order to act as OCh-OH-T, the “OA&M Controller” is interfaced to both the OSC-T (which performs the ”physical” OSC termination and the extraction of the TDM slots related to OCh-OH) and the controller of the DW transponder.

At an “Agent” level a PC is used to implement the NE Agent. This “Agent PC” is interfaced to the  SSC of the OXC. The SSC has the further task of taking care of the inter-relationships between OA&M and NE Management systems, by properly filtering the OA&M information (see chapter 5).

An interface between the Client Node and the Optical Node is foreseen to support OA&M interworking between layers (as in the CSELT OXC). For this task, it has been decided to by-pass the “lower level” CNI/DW controller, and to implement an IP over Ethernet interface (with a message based protocol) between the control systems of the IP-GR and the OXC (see section 2.5, note 2.5.3).

The SSC will be upgraded by adding the applications related to inter-layer signalling, and it will be supplied with the necessary Ethernet interface.

About Figure 4, the same notes already done for Figure 3 - about the use of the term “client” and the logical partition of the “Info to/from the NE Agent” – are valid. The meaning of the “Remote Node Supervision” information, exchanged between SSC and  OSC-T (via the DCN Router), has already been explained in note 2.1.2. This feature is present only in the SICN OXC, because it includes the NE Agent of the remote OLA node.

Figure 4 – Scheme of the control architecture aspects relevant for OA&M – (SICN OXC)

## OLA

The OLA node (including two Optical Line Amplifiers) is an intermediate-site node of a WDM line system, where only the OTS layer is terminated.

Following the guideline of the predecessor project MOON, the OLA node also represents all those low-hierarchy nodes, in a generic OTN, where no local NE Agent is present (or, equivalently, where no interface towards the Network Manager is available). The applied control/management strategy demonstrates one of the possible solutions for this case. The NE Agent of the OLA node is at a higher-level remote node  (actually, the SICN OXC, at one end of the WDM line system), ensuring an interface to the NM. A communication channel (via  DCC, via OSC) is available to exchange information between the local OLA control system and the remote Agent – as already explained in note 2.1.2.

The OLA node control can be consequently described as a simplified two-level architecture (see Figure 5). The scheme is further simplified by the fact that, obviously, no Client Node can be interfaced to the OTN at the OLA.

Figure 5 – Scheme of the control architecture aspects relevant for OA&M – (OLA)

# Preliminary definition of OA&M role for protection/restoration

A detailed specification for protection/restoration procedures is out of the scope of WP6. These aspects will be defined, in the project, after consolidating the results of the workpackage WP2.

However, it is interesting to perform a first investigation of the possible usefulness of OA&M to support and improve protection/restoration procedures. What is proposed in this chapter is the definition of some “boundaries” for the role of OA&M in this area, taking into account realistic implementation constraints for the field trial.

Some significant protection schemes are here briefly considered, from the implementation viewpoint.

- An OCh 1+1 end-to-end protection would be possible, by duplicating the transmitted channel and taking decisions at the receiver side. In this case, however, no OA&M messaging would be involved.

- An OCh end-to-end fast restoration (through meshes and rings) will not be fully implemented, being unrealistic for the field trial. The possibility of a fast reconfiguration of single NE, to test, at least partially, advanced restoration schemes, is under investigation. This issue is related to the possibility of implementation of an “intelligent control plane”, although simplified, in the optical nodes.

- An OCh end-to-end slow restoration (e.g., for clients transporting non-real-time services) is not excluded in principle; it is still to be discussed whether this should involve the Management System or it should be triggered by the client (e.g., IP) through the mediation of OA&M signalling.

- OMS ring protection schemes will be implemented in the ring-structured part of the field trial (inherited by the legacy “PEGASO” test-bed): OA&M is involved, in this case, in that an APS protocol will be active for this purpose (see table 2 and Figure 1). The APS allows to quickly propagate information for OADM reconfiguration, after a fault detection.

- An OMS 1+1 linear protection will be in principle possible, in the meshed part of the trial, by upgrading the OXCs  (e.g., by simulating protection through fast switching of the whole OTS signal in the unused third output fibre of the OXC). OA&M would be indirectly involved, in that OA&M messages or detected defects might be the triggering information for the OTS protection. A definitive decision has not yet been taken about its implementation, since this feature, already demonstrated in predecessor projects, has not the highest priority in LION.

- About the interworking between protection procedures of different layers, the basic step will be a simple test of simultaneous coexistence of different protection mechanisms in the OTN (e.g., OMS ring protection) and in the IP (e.g., OSPF based, MPLS-based, DPT-based) layers. A second step could be the implementation of some hold-off mechanisms; in this case OA&M would be heavily involved to provide the necessary information flow between OXC and IP-GR, through CNI.

# Relationship between the OA&M and Management systems

The task of this chapter is to identify the reciprocal influence between OA&M and Management specifications. This work is still in progress, being carried out in cooperation by workpackages WP6 and WP7.

The first investigations suggest the following approach.

- The Management and OA&M systems must act in a complementary and consistent way. However, it is a good policy to reduce to a minimum the direct interactions, to avoid functional duplications and to minimize complexity. One of the basic tasks of OA&M is just to filter secondary alarm messages, to avoid huge (and useless) cascade of notifications to the Management system.

- No direct requirement for the Management system specification arises from OA&M definition.

- The definition of communication alarms necessary to the Management system implies the identification of a set of physical defects that must be detected by the node equipment. However, this should not be considered as an information flow between OA&M and Management. More precisely, the detected physical anomalies can be considered as a common root for both OA&M and Management processes, which can be in principle independent. For example, a loss of the optical power of a transit channel in the OXC is used by OA&M as a root for an OCh-FDI message; by the NE Agent as a cause to generate a “Loss of Signal” communication alarm to the Manager, e.g., associated to an OCh_CTP_sink.

Actually, the requirements imposed by OA&M and Management are very similar. As an early specification, the list of detected defects imposed by OA&M (reported in Table 7) can be considered representative also for the Management requirements.

- In addition to the communication alarms related to primary defects, it can be useful for the NE Manager to be explicitly informed about Maintenance messages flowing in the network, and received at a particular termination or connection point. This can be opportune, for example:

  - if the “filtering” of “secondary” alarms is more efficiently done at Network Element Manager level rather than at Agent level (this is one of the lessons learnt by the experience of the project MOON);

  - if it is easier for the Manager’s operation to receive notifications directly deriving from some OA&M messages;

  - if the Agents reveal lacks in producing specific notifications about status of equipment/ systems, and Maintenance messages can easily provide an equivalent information.

Since Maintenance messages are generated by OA&M, this can be considered as a requirement on the OA&M system by the Management system, strictly speaking. From a preliminary specification of Management system communication alarms it can be deduced that the following received Maintenance messages belong to the foreseen list:

- OCh-FDI

- OCh-BDI

- OTS-FDI

- OTS-BDI

The forwarding of this explicit information by the node control to the NE Agent will be therefore ensured. It is still to be discussed whether it is opportune to filter secondary alarms at NE Agent rather than at Node Control level.

- In the case the OA&M will be charged of the task of triggering protection procedures, it will be asked to inform the Management system. It is still to be decided whether this would require the definition of an explicit notification or it will be sufficient to forward to the Manager the OA&M messages related to the protection procedure.

- A very important topic is related to the “interworking” between layers; this is faced:

  - at Management level, by achieving interoperability between different Network Managers (this is the key concept of “umbrella management”);

  - at OA&M level, by allowing a signalling between the IP and the OTN Network Elements through CNI.

The two previous approaches are in principle independent. It is to be investigated which kind of impact is to be expected about the OA&M/Management interrelationship. For example, will it be necessary to explicitly inform the Manager about the OA&M signalling flowing through CNI?

More generally, how to ensure consistency between traditional Network Management and possible alternative way of reconfiguring the optical nodes?

These important topics will be covered by the future theoretical studies.

From a practical viewpoint, as stated in chapter 2, the sub-set of functions related to connection set-up to be implemented in the field trial is limited by practical reasons.

What is relevant here is that any of these actions (not yet specified in detail) must be explicitly notified to the Network Manager.

# Implications of interworking functionality on OA&M specification

In this document, a wide meaning has been assigned to the term “OA&M”, because this approach was practical to collect a first significant set of specifications for the partners providing equipment and control/management systems for the field trial.

In the context of a wide view on OA&M, even the items of “signalling” and “routing” in OTN have been taken into account. “Signalling” and “routing” are used here as concise terms to allude to a major evolution trend that is observed in OTN: in short, it is becoming evident that it will be useful or even necessary to propagate more and more information between optical NE, preferably through the OTN itself, to support more and more diversified functions, in addition to the “traditional” Maintenance and Supervision of transport networks.

This is a common lesson that can be drawn from different approaches emerging in research and standardization arenas.

For example, Automatic Switched Optical Networks (ASON), proposed in ITU-T, aims at allowing the user-driven set-up of end-to-end connections by means of signalling. In some way, the well-known concept of signalling in circuit switched networks is re-discovered, and applied to a very new context, where the “user” may be a client-network node and the end-to-end connection may be a high bit-rate pipe, provided through cross-connect nodes.

In IETF, a significant effort is currently made to extend the MPLS principles to the switching at Layer 1 (TDM or WDM). This approach is widely known as “Multi-Protocol Lambda Switching”, when applied in particular to the cross-connection of wavelengths in the OTN. Obviously, the implementation of this idea implies the addition of “optical intelligence” to the optical nodes, and the exchange of routing information, through proper “routing protocols” between them. The final objective is to render the optical network capable of reconfiguring itself like an IP transport network, i.e., through a “distributed” rather than centralized intelligence.

LION is promoting an internal in-depth discussion to find a fair way to reconcile these emerging approaches with its own view, which is under several aspects compatible with them, although different and original. LION, in fact, is considering the problem of client interworking from the perspective of a client-independent OTN carrying different clients such as SDH and IP-based networks. In other words, the LION roadmap is towards a data-centric view, but preserving for the OTN the fair property of being a server layer also for legacy transport networks (mainly, SDH).

In this sense, a more natural convergence can be identified with the ASON approach. The ASON is intended as an OTN also capable of switching Optical Channel connections automatically. ASON is still based on client-server relationships (i.e., there are two independent control planes: the client control plane and the ASON control plane) and IP is not necessarily the only envisaged client. Nevertheless as IP is gaining the role of the most promising Layer 3 protocol supporting upper applications, IP-based networks are likely to become the most common ASON-OTN client. This is why in the LION test-bed the interworking between IP and OTN is being considered primarily.

The evaluation of the above outlined important topics has just started, from the conceptual as well as from the implementation point of view.

The conceptual implications of this discussion go over the pure WP6 scope, and will involve almost all of the project workpackages. The results of the detailed investigation that is currently carried out will be reported in future documents.

It is instead a task of this deliverable to outline preliminary considerations from the viewpoint of implementation feasibility.

Firstly, it should be noted that while defining the implementation of interworking mechanisms based on both OA&M and management systems, limitations imposed by existing infrastructure have to be taken into account.

Then, some outcomes from the preliminary investigation, carried out up to now, can be summarized:

- a survey of possible “transport functions” to be offered by OTN to IP, through signalling/routing, has been identified in WP1 (frame forwarding, OCh connection set-up, bandwidth provisioning, etc.);

- among these functions, the dynamic OCh connection set-up (driven by the client) seems to be particularly worth of implementation;

- about the choice on how to implement different advanced functionalities, an effective assignment of tasks to the Network Manager, on one hand, and the control plane responsible for signalling/routing, on the other hand, will be looked for. Besides feasibility issues, based on the evaluation of the required time-scales for different functions, the more general requirement of consistency with the emerging technical developments promoted by standardization will be taken into account.

In order to implement a more or less complex dynamic OCh set-up, some additional features, with respect to the originally planned control architecture, are necessary:

- a control plane in the OXC, in addition to the NE Agent, the Node Control system, the OA&M controller functions; the OXC control plane should include, in principle, modules implementing signalling and routing protocols;

- an enhanced control plane in the IP-GR, supporting the communication with the control plane of the OXC, and the inter-layer signalling;

- a proper interface (logically, through CNI) for the interworking between the mentioned control planes (in IP-GR and OXC, respectively);

- a communication channel for the transportation of signalling (and possibly routing) information between different optical NE, across the OTN;

- a modification of the NE Agents, to reflect the new features and to ensure consistency between the “control plane” and the “management” world.

From the viewpoint of feasibility, the provisional answers by WP6 to these topics are as follows:

- a physical communication interface is provided between IP-GR and OXC  through a common Ethernet interface, to support client/server NE signalling information exchange (see details in Chapter 4)

- a physical communication channel is available between optical nodes by exploiting DCC transported by the OSC (see details in Chapter 3);

it should be noted however that the implementation of processes related to a remote signalling involves complex issues, related to the overall network consistency, well beyond the physical possibility of this communication: this task might be difficult to be accomplished in the field trial;

- the extent of the possible enhancements of the control plane in the IP-GR for the field trial is currently matter of discussion, within the project;

- the upgrade of the control systems of optical nodes, to implement some of the “control plane” functions is in principle possible; different degrees of complexity can be identified, and will be investigated from the feasibility viewpoint; the actual complexity of these upgrades will be limited, owing to practical constraints;

- the upgrade of NE Agents and, moreover, the strategy to ensure coexistence of an Agent and a Control Plane in a NE are a completely open issue. The impact of this upgrade is expected to be more critical for the TMN based agents.

# Conclusions

The specifications provided by this deliverable are on one side provisional (as the document title itself suggest), but on the other side cover several items (even over the strict OA&M definition).

This profile has been chosen to give to the implementation workpackages as much information as possible; moreover, to reflect not only the pure WP6 work, but also the activity carried out in cooperation with WP5 and WP7. References to general technical discussions, currently promoted within the project, are included too.

One of the consequences is that several of the touched issues are still open.

In the deliverable, the status of the single items is clearly stated, so that the already fixed points can be used as basis for implementation activities; the list of the open issues might be a sort of map of the areas on which to focus the future work.

Where relevant, references to WP6 theoretical activities have been reported too.

On the basis of this deliverable, the WP6 work, while concentrating its view on strict OA&M topics, will deepen the investigation and consolidate the results, in two parallel directions:

- the finalization of OA&M specifications (to be reflected in deliverable D13 “First report on the definition of the OA&M role in the OTN” – foreseen for June 2001);

- a comprehensive theoretical activity on OA&M (ultimately leading to deliverable D23 “Final report on OA&M concepts in a multi-layer network” - scheduled for June 2002).