Milestone WP1M1: “Network requirements”

Part 2: “Products and technologies”

| Status and Version: |  | Version 2.02, Draft |  |
| --- | --- | --- | --- |
| Date of issue: |  | 30.08.2000 |  |
| Distribution: |  | Project Internal |  |
| Author(s): | Didier Colle |  | IMEC |
|  | Pim Vanheuven |  | IMEC |
|  | Steven Van den Berghe |  | IMEC |
|  | Mateusz Kosciuszko |  | AGH |
|  | Jacek Rzasa |  | AGH |
|  | Stefano Brunazzi |  | SICN |
|  | Albert Rafel |  | UPC |
|  | Giuseppe Marone |  | CSELT |
|  | Julio Moyano |  | UPC |
|  | Josep Solé-Pareta |  | UPC |
|  | Lampros Raptis |  | NTUA |
|  | Giorgos Chatzilias |  | NTUA |
|  | Janusz Maliszewski |  | TPSA |
| Checked by: | Giorgos Chatzilias |  | NTUA |
|  | Chris Develder |  | IMEC |
|  | Sophie Demaesschalck |  | IMEC |
|  | Mario Pickavet |  | IMEC |
|  | Piet Demeester |  | IMEC |
|  | Artur Lason |  | AGH |
|  | Antonio Manzalini |  | CSELT |
|  | Fritz-Joachim Westphal |  | T-NOVA |
|  | Janusz Maliszewski |  | TPSA |

Table of Contents

1	Introduction to Part 2	4

1.1	Purpose and Scope	4

1.2	Reference Materials	5

1.2.1	Reference Documents	5

1.2.2	Abbreviations	8

1.2.3	Definitions	10

1.3	History of Part 2 of Milestone WP1M1 document	12

1.4	Part 2 overview	12

2	IP over WDM: overview on current technology and products	12

2.1	Currently Deployed IP technology	12

2.2	Integrated and differentiated services	13

2.2.1	Introduction	13

2.2.2	Integrated Services (intserv)	13

2.2.3	Differentiated services	15

2.3	Overview of IP mapping solutions	18

2.3.1	The “Classical Telecom” mapping	18

2.3.2	Simplified mappings	19

2.4	IP over WDM as transmission technology	24

2.4.1	Enabling technologies	25

3	Mapping Solutions taken into consideration in LION	26

3.1	Multi-Protocol Label Switching	26

3.1.1	What is MPLS?	26

3.1.2	MPLS architecture	27

3.1.3	Label Distribution Protocol (LDP)	30

3.1.4	Diffserv in MPLS-based networks	31

3.1.5	Traffic Engineering (TE) support in MPLS	32

3.2	Dynamic Packet Transport	32

3.2.1	Introduction	32

3.2.2	DPT Foundations	33

3.2.3	The Spatial Reuse Protocol	34

3.3	Gigabit Ethernet	39

3.3.1	Architecture	39

3.3.2	Standards	42

3.3.3	Gigabit Ethernet and Asynchronous Transfer Mode (ATM) technologies	42

3.3.4	Topology	43

3.3.5	10Gigabit-Ethernet	44

3.3.6	Conclusion	45

3.4	IP over SDH and Packet over SONET	45

3.4.1	New Backbone Network Technologies: IP over SDH/SONET	45

4	IP over OTN as a new transport network	53

4.1	The role of a Transport Network	53

4.2	The “IP over OTN” scenario	55

4.3	Open issues	57

4.3.1	3R-Regeneration vs Transparency	57

4.3.2	Performance Monitoring and OA&M	58

4.3.3	Protection vs Restoration	59

4.4	Enabling technologies	60

4.4.1	Technologies for OTN transport nodes	60

4.4.2	Technologies for advanced network functionalities	61

5	IP over “intelligent” OTN	62

5.1	Optical “intelligence”	63

5.1.1	The IP driven roadmap towards an Intelligent OTN	64

5.1.2	The classical telecom roadmap towards an Intelligent OTN	65

5.2	Enabling technologies	65

6	Product overview	66

6.1	IP-over-WDM	66

6.1.1	IP over WDM as transmission technology	66

6.1.2	IP over OTN as a new transport network	67

6.1.3	IP over “intelligent” OTN	72

6.2	A survey of optical networking products for the transport network	72

6.3	DPT Applications and Products	73

7	Relevant standardisation processes	74

# Introduction to Part 2

## Purpose and Scope

The goal of the overall task is to provide the Project Partners with an overview of all technologies relevant to the LION Project. The important technologies have been recognised and the scope of required description has been agreed during discussion of the working document “Some Discussion Topics” [WPG1-DISC]. The following conclusion results from this discussion: evolution of the “IP over WDM” to “IP-based over OTN” technology is fundamental for the LION Project (end generally for telecom operators “taking in conceiving and building next generation of telecommunications networks”). The economic superiorities of these technologies - considered separately for two main layers – was taken as basic reasons for this study pre-selection. The advantages of this main layers are following:

- IP-based layer: As carriers attempt to marshal investment capital to fund new network construction, their business cases are strongly influenced by the time value of investment, which in turn are driven by the ratio of innovation and economic learning effects in the core technologies chosen for the networks. These learning-curve effects are characterised in the best way by the length of the period observed in which the performance/price ratio doubles. The performance-doubling period for TDM is the longest of these shown, at about 60 to 80 months. Packet-based IP and ATM technologies have displayed intervals of, respectively, 20 and 40 months, [TECH-SOFT].

- WDM layer evolving to OTN layer: It is estimated that implementation to transport network WDM technology together with “intelligent optical transport nodes” might result in saving the Capital Cost from 30 to 50% and in saving the Operation Cost from 50 to 60%. Additionally, in comparison with solutions of transport network in “SDH over fiber” technology, implementing OTN on dark fiber may give savings even over 85%.

The IP layer will evolve in opposite direction than Best Effort to support business application and to face requirements resulting from expected traffic explosion. Thus the evolution of IP is necessary to guarantee QoS and to develop of the traffic engineering function. IP is the best platform to realisation the services convergation. IP enables the world wide communication, it is open platform and connectionless technology, sufficient to implementation of new services. It guarantee the reduction of prices for the telecom services.

The main focus of this part of the document is the Wavelength Division Multiplexing (WDM) technology. Giving the profile assumed to the network requirement studies in LION WP1, a particular viewpoint has been chosen: how can WDM be the server layer of the emerging IP-based data-centric packet-switched network?

This viewpoint, first of all allows to describe the main advances of the WDM technology and its expected future developments, and also it is consistent with the perspective assumed by major manufacturers (and network operators), who recognise that the IP Network (and the IP Giga/TeraRouters, at Network Element level) will be the main client for the emerging optical network. This perspective takes into account the strongest driving forces for the WDM (more generally, OTN) evolution.

The considered scenario can be roughly described as follows: WDM already is and will be a “server” layer for the emerging data-centric network, mainly based on the IP technology. The “basic service” offered by WDM evolves from the very high-capacity transmission infrastructure (which WDM already offers) to the functions of a complete transport network (through the ongoing evolution towards an OTN).

## Reference Materials

### Reference Documents

| [AGH 1] | M. Ohta, IETF and Internet Standards, IEEE Communications Magazine, September 1998 |  |  |  |
| --- | --- | --- | --- | --- |
| [ARCH] | E.C.Rosen, A.Viswanathan, R.Callon, "Multiprotocol Label Switching Architecture", draft-ietf-mpls-arch-06.txt, Internet Draft, August 1999. |  |  |  |
| [BGPLAB] | Y. Rekhter, E. Rosen, “Carrying Label Information in BGP-4”, draft-ietf-mpls-bgp4-mpls-04, work in progress. |  |  |  |
| [CONC-TE] | K. Takashima, K. Nakamichi, and T. Soumiya, “Concept of IP Traffic Engineering”, <draft-takashima-te-concept-00.txt>, work in progress, October 1999. |  |  |  |
| [CRLDP] | B.Jamoussi, "Constraint-Based LSP Setup using LDP", draft-ietf-mpls-cr-ldp-03.txt, Internet Draft, September 1999. |  |  |  |
| [DIFF-NEW] | D.Grossman, "New Terminology for Diffserv", draft-ietf-diffserv-new-terms-02.txt, Internet Draft, November 1999. |  |  |  |
| [DPS] | I.Stoica et al, “Per Hop Behaviors Based on Dynamic Packet States”, <draft-stoica-diffserv-dps-00>, work in progress, February 1999 |  |  |  |
| [DPT-1] | Cisco Systems, “Cisco Optical Internetworking”. http://www.cisco.com/warp/public/779/servpro/solutions/opt/oi_brochure.html |  |  |  |
| [DPT-2] | Cisco Systems, “Dynamic Packet Transport Technology and Applications Overview”. http://www.cisco.com/warp/public/cc/cisco/mkt/servprod/opt/dpt/dpta_wp.htm |  |  |  |
| [DPT-3] |  | Internet Draft (Work in progress), “The Cisco SRP MAC Layer Protocol <draft-tsiang-srp-01.txt>”, D. Tsiang, G. Suwala, March 2000 (This draft expires on 1 September 2000) http://search.ietf.org/internet-drafts/draft-tsiang-srp-01.txt |  |  |
| [DPT-4] |  | Executive Summary, “Metro Optical Networks: Metro DWDM and the New Public Network”, Pioneer Consulting |  |  |
| [DPT-5] |  | Cisco DPT Products http://www.cisco.com/warp/public/cc/cisco/mkt/servprod/opt/dpt/ |  |  |
| [DPT-6] |  | RingStar8000 Product Data Sheet http://www.penta-com.com/ringstar8000.htm |  |  |
| [DPT-7] |  | Internet Draft (Work in progress), “Definitions of Managed Objects for Spatial Reuse Protocol (SRP) <draft-jedrysiak-srp-mib-00.txt>2”, Stan Jedrysiak, March 2000 (This draft expires in September 2000) http://search.ietf.org/internet-drafts/draft-jedrysiak-srp-mib-00.txt |  |  |
| [DSEDGE] |  | Y. Bernet, D. Durham, F. Reichmeyer, “Requirements of Diff-serv Boundary Routers”, <draft-bernet-diffedge-01 >, work in progress, November 1998 |  |  |
| [DSFWK] |  | Y. Bernet et al., “A Framework for Differentiated Services”, <draft-ietf-diffserv-framework-02 >, work in progress, February 1999 |  |  |
| [DSMIB] |  | F. Baker, “Management Information Base for the Differentiated Services Architecture”, <draft-ietf-diffserv-mib-00 >, work in progress, July 1999 |  |  |
| [DSROUT] |  | Y. Bernet, A. Smith, S. Blake, “A Conceptual Model for Diffserv Routers”, <draft-ietf-diffserv-model-00 >, work in progress, June 1999 |  |  |
| [ENCAPS] |  | D. Farinacci et al., “MPLS Label Stack Encoding”, draft-ietf-mpls-label-encaps-07, work in progress. |  |  |
| [FR] |  | A. Conta, P. Dool an, and A. Malis, “Use of Label Switching on Frame Relay Networks Specification”, draft-ietf-mpls-fr-03, work in progress. |  |  |
| [ISDS] |  | Y. Bernet et al., “Integrated Services Operation Over Diffserv Networks”, Internet Draft, <draft-ietf-issll-diffserv-rsvp-02 >, work in progress, June 1999 |  |  |
| [LANE] |  | “LANE v2.0 LUNI Interface”, (af-lane-0084.000) July, 1997. |  |  |
| [LDP] |  | L.Andersson, P.Doolan, N.Feldman, A.Fredette, B.Thomas., "LDP Specification", draft-ietf-mpls-ldp-06.txt, Internet Draft, October 1999. |  |  |
| [LOADCONTR] |  | L.Westberg, Z.R.Turanyi, “Load Control of Real-Time Traffic”, <draft-westberg-loadcntr-03>, work in progress, April 2000 |  |  |
| [LOOP1] |  | Y. Ohba “Issues on loop prevention in MPLS networks”, IEEE communications magazine, Januari 2000. |  |  |
| [LOOP2] |  | Y. Ohba et al., “MPLS Loop Prevention Mechanism”, draft-ohba-mpls-loop-prevention-02, work in progress. |  |  |
| [MECN] |  | K. K. Ramakrishnan et al., “A Proposal to Incorporate ECN in MPLS”, draft-ietf-mpls-ecn-00, work in progress. |  |  |
| [MICMP] |  | R.Bonica et al., “ICMP Extensions for MultiProtocol Label Switching”, draft-ietf-mpls-icmp-01, work in progress. |  |  |
| [MPLS-DS] |  | F. Le Faucheur et al., “MPLS Support of Differentiated Services”, draft-ietf-mpls-diff-ext-03, work in progress. |  |  |
| [MPLS-FRAME] |  | R.Callon, P.Doolan, N.Feldman, A.Fredette, G.Swallow, A.Viswanathan, "A Framework for Multiprotocol Label Switching", draft-ietf-mpls-framework-05.txt, Internet Draft, September 1999. |  |  |
| [MPLS-QoS] |  | J. Ash et al., “QoS Resource Management in MPLS-based Networks”, <draft-ash-qos-routing-00.txt>, work in progress, 1999 |  |  |
| [MPLS-TE] |  | D. Awduche, “MPLS and Traffic Engineering in IP Networks.”, IEEE Communications Mag., December 1999. |  |  |
| [MPOA] |  | “Multi-Protocol Over ATM v1.0”, (af-mpoa-0087.000) July, 1997. |  |  |
| [MYTH] |  | G. Armitage, “MPLS: The magic behind the myths”, IEEE communications magazine, January 2000. |  |  |
| [NR-AoS] |  | “Network Requirements – Analysis of Services”, July 2000, LION/WPG1/WP1/T1 – working document: DT-LION-WP1Task1_01j.doc. |  |  |
| [NR-P&T] |  | “Network Requirements – Products and Technologies”, June 2000, LION/WPG1/WP1/T2 – working document: CM1T2-IMEC_v201.doc. |  |  |
| [OTN-DELIV] |  | Eurescom P918, D1, "IP over WDM, Transport and Routing", October 1999. |  |  |
| [OTN-PUBLI 1] |  | L.Thylén et al., "Switching technologies for future guided wave optical networks: potentials and limitations of photonics and electronics", IEEE Comm. Mag., pp. 106-113, February 1996. |  |  |
| [OTN-PUBLI 2] |  | A. Ware (Agilent) “New photonic-switching technology for all-optical networks” – Lightwave, March 2000 |  |  |
| [OTN-PUBLI 3] |  | Communication Magazine (March 2000). |  |  |
| [OTN-PUBLI 4] |  | "Switching in IP Networks", Morgan Kaufmann Publishers, Inc., 1998. |  |  |
| [OTN-PUBLI 5] |  | Report from Communications Industry Researchers, Inc. (CIR) on “Wave Division Multiplexing, Photonic Switching and the Coming of All Optical Networks 1999-2000, Volume1” |  |  |
| [OTN-STD 1] |  | IETF draft "IP over Optical Networks - A Framework" |  |  |
| [OTN-STD 2] |  | IETF draft "An architecture for MPLS control plane for Switched Optical Networks" |  |  |
| [OTN-STD 3] |  | IETF draft "Multi-Protocol Lambda Switching: combining MPLS traffic engineering control with Optical Cross-connects" |  |  |
| [OTN-STD 4] |  | IETF RFC 2702 "Requirements for Traffic Engineering Over MPLS" |  |  |
| [OTN-STD 5] |  | ITU-T G.ason “Architecture for the Automatic Switched Optical Network” |  |  |
| [OTN-STD 6] |  | ITU-T X.ipos “IP over SDH using LAPS” |  |  |
| [OTN-STD 7] |  | ITU-T X.eos “Ethernet Frame over SDH/WDM” |  |  |
| [OTN-STD 8] |  | ANSI T1X1 draft 99-269 (Nortel) “Generic Format for Carrying Ethernet MAC Frames over SONET” |  |  |
| [OTN-STD 9] |  | “ODSI Functional Specifications” |  |  |
| [OTN-WEB 1] |  | www.cisco.com |  |  |
| [OTN-WEB 10] |  | www.lucent.com |  |  |
| [OTN-WEB 11] |  | www.agilent.com |  |  |
| [OTN-WEB 12] |  | www.tellabs.com |  |  |
| [OTN-WEB 13] |  | www.telcordia.com |  |  |
| [OTN-WEB 2] |  | www.marconicomms.com |  |  |
| [OTN-WEB 3] |  | www.sycamorenet.com |  |  |
| [OTN-WEB 4] |  | www.ciena.com |  |  |
| [OTN-WEB 5] |  | www.alcatel.com |  |  |
| [OTN-WEB 6] |  | www.xros.com |  |  |
| [OTN-WEB 7] |  | www.siemens.com |  |  |
| [OTN-WEB 8] |  | www.nortelnetworks.com |  |  |
| [OTN-WEB 9] |  | www.juniper.com |  |  |
| [OTN-WHITE 1] |  | C. Webb, T. Krause (Alcatel) “Drivers and Enabling Technologies for Wavelength Based Services in the All Optical Network”. |  |  |
| [OTN-WHITE 10] |  | Monterey White Paper: “Scaling Optical Data Network with Wavelength Routing” |  |  |
| [OTN-WHITE 11] |  | N. De Vito (Tellium) "Delivering Optical Layer Services with Optical Switching". |  |  |
| [OTN-WHITE 12] |  | C.A. Brackett (Tellium) "Cost Effective Optical Networks: the role of Optical Cross-Connects". |  |  |
| [OTN-WHITE 2] |  | Ciena White Paper: “The new economics of Optical Core Networks” |  |  |
| [OTN-WHITE 3] |  | “Cisco’s Packet over SONET (POS) technology support: mission accomplished”. |  |  |
| [OTN-WHITE 4] |  | "Cisco Optical Internetworking." |  |  |
| [OTN-WHITE 5] |  | Dynarc White Paper: “Distributed Switching and Routing” |  |  |
| [OTN-WHITE 6] |  | R.A. Goudreault et al. (Lucent) “Impact of an Integrated Architecture for Bandwidth Management in a Broadband network Infrastructure” |  |  |
| [OTN-WHITE 7] |  | J. Anderson et al. (Lucent) “Protocols and architectures for IP optical networking” |  |  |
| [OTN-WHITE 8] |  | P. Bonenfant (Lucent) "WaveWrapper Technology, A major leap in optical transport networking". |  |  |
| [OTN-WHITE 9] |  | B.T. Doshi (Lucent) “A Simple Data Link Protocol for high speed packet networks” |  |  |
| [PoS-1] |  | James Manchester, et altri, ”IP over SONET”, IEEE Communications Magazine, May 1998. |  |  |
| [PoS-10] |  | http:/www.lucent.com |  |  |
| [PoS-2] |  | Kevin Thompson, Gregory J. Miller and Rick Wilder, “Wide-Area Internet Traffic Patterns and Characteristics”, IEEE Network, November/December 1997. |  |  |
| [PoS-3] |  | W. Simpson, “PPP in HDLC-like Framing”, RFC 1662, July 1994, ftp://ftp.upc.es/pub/doc/rfc/16xx/1662 |  |  |
| [PoS-4] |  | W. Simpson, “PPP over SONET/SDH”, RFC 2615, June 1999, ftp://ftp.upc.es/pub/doc/rfc/26xx/2615 |  |  |
| [PoS-5] |  | http://www.lucent.com/micro/tic/docs/sdl.pdf |  |  |
| [PoS-6] |  | Integration of IP over Optical Networks: Networking and Management, Eurescom project P918-GI, Deliverable 1 (IP over WDM, Transport and Routing). http://www.eurescom.de |  |  |
| [PoS-7] |  | W. Stallings, “ISDN and Broadband ISDN with Frame Relay and ATM”. Ed. Prentice Hall, 1998 (4th. Edition). |  |  |
| [PoS-8] |  | R. Ramaswami, K.Sivarajan "Optical Networks, A practical Perspective", Morgan Kaufmann Publishers, 1998. |  |  |
| [PoS-9] |  | http:/www.cisco.com |  |  |
| [RFC-1349] |  | P. Almquist., “Type of Service in the Internet Protocol Suite”, RFC 1349, July 1992 |  |  |
| [RFC-1633] |  | R. Braden, D. Clark, S. Shenker, R. Braden, L. Zhang, S. Berson, S. Herzog, S. Jamin, "Integrated Services in the Internet Architecture: an Overview", RFC 1633, June 1994. |  |  |
| [RFC-2205] |  | R. Braden et al., “Resource ReSerVation Protocol (RSVP) – Version 1 Functional Specification”, RFC 2205, September 1997 |  |  |
| [RFC-2206] |  | F. Baker, J. Krawczyk, A. Sastry, “RSVP Management Information Base using SMIv2”, RFC 2206, September 1997 |  |  |
| [RFC-2210] |  | J. Wroclawski, “The Use of RSVP with IETF Integrated Services”, RFC 2210, September 1997 |  |  |
| [RFC-2211] |  | J. Wroclawski, “Specification of the Controlled-Load Network Element Service”, RFC 2211, September 1997 |  |  |
| [RFC-2212] |  | S. Shenker, C. Partridge, R. Guerin, “Specification of Guaranteed Quality of Service”, RFC 2212, September 1997 |  |  |
| [RFC-2213] |  | F. Baker, J. Krawczyk, A. Sastry, “Integrated Services Management Information Base using SMIv2”, RFC 2213, September 1997 |  |  |
| [RFC-2214] |  | F. Baker, J. Krawczyk, A. Sastry, “Integrated Services Management Information Base, Guaranteed Service Extensions using SMIv2”, RFC 2214, September 1997 |  |  |
| [RFC-2215] |  | S. Shenker, J. Wroclawski, “General Characterization Parameters for Integrated Service Network Elements”, RFC 2215, September 1997 |  |  |
| [RFC-2216] |  | S. Shenker, J. Wroclawski, “Network Element Service Specification Template”, RFC 2216, September 1997 |  |  |
| [RFC-2430] |  | T.Li, Y.Rekhter, "A Provider Architecture for Differentiated Services and Traffic Engineering (PASTE)", Informational, October 1998 |  |  |
| [RFC-2474] |  | K. Nichols et al., “Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers”, RFC 2474, December 1998 |  |  |
| [RFC-2475] |  | S. Blake et al., “An Architecture for Differentiated Services”, RFC 2475, December 1998 |  |  |
| [RFC-2597] |  | J. Heinanen et al., “Assured Forwarding PHB Group”, RFC 2597, June 1999 |  |  |
| [RFC-2598] |  | V. Jacobson, K. Nichols, K. Poduri, “An Expedited Forwarding PHB”, RFC 2598, June 1999 |  |  |
| [RFC-2638] |  | K. Nichols, V. Jacobson, L. Zhang, “A Two-bit Differentiated Services Architecture for the Internet”, RFC 2638, July 1999 |  |  |
| [RFC-791] |  | J. Postel., “Internet Protocol”, RFC 791, September 1981 |  |  |
| [RMMIB] |  | A. Bierman, “Remote Monitoring MIB Extensions for Differentiated Services Enabled Networks”, <draft-bierman-dsmon-mib-01>, work in progress, June 1999 |  |  |
| [RSVPT] |  | D. Awduche et al., “Extensions to RSVP for LSP Tunnels”, draft-ietf-mpls-rsvp-lsp-tunnel-04, work in progress. |  |  |
| [SWITCH] |  | B. Davie, P. Doolan, and Y. Rekhter, “Switching in IP Networks”, Morgan Kaufmann, 1998. |  |  |
| [TECH-SOFT] |  | “The Lucent Technologies Softswitch – Realising the Promise of Convergence”, Bell Lab Technical Journal, April-June 1999. |  |  |
| [VCID] |  | K. Nagami et al., “ VCID Notification over ATM Link”, draft-ietf-mpls-vcid-atm-04, work in progress. |  |  |
| [VCSW] |  | B. Davie et al., “MPLS using LDP and ATM VC Switching”, draft-ietf-mpls-atm-02, work in progress. |  |  |
| [WPG1-DISC] |  | Answers from Discussion Meeting, April 13th-14th 2000 |  |  |

### Abbreviations

| AAL5 | ATM Adaptation Layer 5 |
| --- | --- |
| AAU | ATM to ATM User |
| ADM | Add Drop Multiplexer |
| AF | Assured Forwarding |
| APD | Avalanche PhotoDiode |
| APS | Automatic Protection Switching |
| ASON | Automatically Switched Optical Network |
| ATM | Asynchronous Transfer Mode |
| AWG | Arrayed Wave Guide |
| BA | Behavior Aggregate |
| BGP | Border Gateway Protocol |
| BSHR | Bidirectional Self Healing Ring |
| CR-LDP | Constraint Routed – LDP |
| CU | Currently Unused bits |
| DBR | Distributed Bragg Reflector |
| DCC | Data Communication Channel |
| DCF | Dispersion Compensating Fibre |
| DFB | Distributed FeedBack |
| Diffserv | Differentiated Services |
| DLCI | Data Link ConnectionIdentifier |
| DPT | Dynamic Packet Transport |
| DS | DiffServ |
| DSCP | Differentiated Service Code Point |
| DTM | Dynamic Transfer Mode |
| DWDM | Dense Wavelength Division Multiplexing |
| DXC | Digital cross Connect |
| ECN | Early Congestion Notification |
| EDFA | Erbium Doped Fiber Amplifier |
| EDFFA | Erbium Doped Fluoride Fibre Amplifier |
| EF | Expedited Forwarding |
| E-LSP | EXP-inferred-PSC LSP |
| ER-LSP | Explicit Routed – LSP |
| FBG | Fibre Bragg Grating |
| FEC | Forward Equivalent Class (in context of MPLS) |
| FEC | Forward Error Correction |
| FR | Frame Relay |
| GbE | Gigabit Ethernet |
| GSMP | General Switch Management Protocol |
| HDLC | High-level Data Link Control |
| HEC | Header Error Correction |
| IAB | Internet Architecture Board |
| IANA | Internet Assigned Number Authority |
| ICMP | Internet Control Message Protocol |
| IESG | Internet Engineering Steering Group |
| IETF | Internet Engineering Task Force |
| Intserv | Integrated Services |
| IP | Internet Protocol |
| IPS | Intelligent Protection Switching |
| IS | IntServ |
| ISOC | Internet Society |
| ISP | Internet Service Provider |
| ITU | International Telecommunication Union |
| LAN | Local Area Network |
| LAPS | Link Access Procedure on SDH |
| LDP | Label Distribution Protocol |
| LER | Label Edge Router |
| L-LSP | Label-only-inferred-PSC LSP |
| LSA | Link State Advertisement |
| LSP | Label Switched Path |
| LSR | Label Switch Router |
| MAC | Media Access Control |
| MAN | Metropolitan Area Network |
| MAPOS | Multiple Access Protocol Over SONET |
| MEMS | Micro-Electro-Mechanical System |
| MPLS | Multi-Protocol Label Switching |
| MPS | Multiplex Section Protection |
| MPS | Multi-Protocol Lambda Switching |
| MTU | Maximum Transfer Unit |
| NE | Network Element |
| NHRP | Next Hop Resolution Protocol |
| NL | Network Level |
| NM | Network Management |
| O/E/O | Opto-Electro-Optical |
| OA | Ordered Aggregate |
| OA&M | Operation Administration & Maintenance |
| OADM | Optical Add Drop Multiplexer |
| OC | Optical Carrier |
| OCh | Optical Channel |
| OMS | Optical Multiplex Section |
| OSC | Optical Supervisory Channel |
| OSPF | Open Shortest Path First |
| OTN | Optical Transport Network |
| OTS | Optical Transmission Section |
| OXC | Optical cross Connect |
| PHB | Per-Hop-Behavior |
| PLC | Planar Lightwave Circuit |
| POL | Packet Over Lightwave |
| POS | Packet Over SONET/SDH |
| PPP | Point-to-Point Protocol |
| PSC | PHB Scheduling Class |
| QoS | Quality-of-Service |
| Rspec | Request Specification |
| RSVP | Resource reSerVation Protocol |
| SDH | Synchronous Digital Hierarchy |
| SDL | Simple Data Link |
| SNCP | Sub Network Connection Protection |
| SOA | Semiconductor Optical Amplifier |
| SONET | Synchronous Optical NETwork |
| SPE | Synchronous Payload Envelope |
| SRP | Spatial Reuse Protocol |
| SRP-fa | Spatial Reuse Protocol Fairness Algorithm |
| SSG | Super-Sampled Grating |
| STM | Synchronous Transfer Module |
| STS | Synchronous Transport Signal |
| TCB | Traffic Conditioning Block |
| TCP | Transmission Control Protocol |
| TDM | Time Division Multiplexing |
| TE | Traffic Engineering |
| TMN | Telecommunication Management Network |
| ToS | Type of Service |
| Tspec | Traffic Specification |
| TTL | Time-To-Live |
| UDP | User Datagram Protocol |
| UPSR | Unidirectional Path Switched Ring |
| VC | Virtual Channel (ATM context) |
| VC | Virtual Container (SDH context) |
| VCI | Virtual Channel Identifier |
| VP | Virtual Path |
| VPI | Virtual Path Identifier |
| VPN | Virtual Private Networks |
| WAN | Wide Area Network |
| WDM | Wavelength Division Multiplexing |

### Definitions

| Connectionless | Network property, specifying that the network treats each individual packet of a session independently |
| --- | --- |
| Best-effort | Service property, specifying that the network attempts to deliver the data to the right destination but without any guarantee |
| Connectivity | Network property, specifying if there exists a route from source to destination node |
| Flow | Sequence of packets belonging to the same session or connection |
| End-node | Network node residing at the edge of the network, where flows originate or terminate |
| Router | Network node at the network layer (L3). A router is based on the store-and-forward principle and routes each packet independently, based on its destination address |
| Congestion | Overloaded network status, resulting in packet losses |
| Packet Loss | Loss of packets due to buffer overflow. Packet losses can be specified on a node or network level |
| Packet Delay | Time required for a packet to travel across the network form source to destination node. |
| Streaming Application | delay-sensitive (real-time) traffic [WPG1-DISC] |
| Multimedia Application | Application based on more than one medium (e.g., video-conferencing which is based on voice and video media |
| Multicast | One-to-many communication |
| Guarantee | In this context referring to QoS guarantees. Note that such a guarantee is not necessarily very strict (e.g., elastic traffic) |
| Controlled Load Service | A service defined in the IS framework and perceiving a quality as it would receive on an unloaded network |
| Guaranteed quality of service | Another service defined in the IS framework, receiving a QoS which is mathematically guaranteed for bandwidth and delay (but not delay variation) requirements |
| Soft-State Protocol | A protocol which requires to refresh the states in all positions over the network from time to time. |
| EF | A PHB defined in the DS framework and aiming to provide leased-line or premium services |
| AF | Another PHB defined in the DS framework and giving the traffic at least a certain probability to be forwarded |
| Intserv | An architecture providing QoS and reserving resources on a per-microflow basis |
| Diffserv | Another architecture aiming to provide QoS by providing aggregated traffic with a particular per-hop-behavior |
| RSVP | Soft-state signaling protocol to request and confirm resource reservations |
| PATH-message | An RSVP message sent from source to destination specifying the traffic requirements (Tspec) and being used in the intermediate nodes to find the previous upstream node |
| RESV-message | RSVP message send from destination to source on receipt of a PATH message, requesting to reserve the required resources in intermediate nodes |
| LDP | Protocol to distribute the label bindings across the network |
| LSR | An MPLS router, which switches LSPs based on its label (similar to ATM nodes) |
| LSP | A concatenation of label bindings representing a logical connection from ingress to egress LSR. |
| CR – LDP | Label distribution taking into account some constraints in the routing of LSPs |
| ER – LSP | An LSP which is not routed following the IP routing tables |
| FEC | An IP prefix or host address. |
| Flow Driven | Same as data driven |
| Topology Driven | LSP setup is triggered by and following the layer 3 routing table changes |
| Downstream Allocation | Upstream node receives label mappings from the next-hop or downstream node. (LDP supports only downstream allocation) |
| Upstream Allocation | Downstream node receives label mappings from upstream nodes (this mode is not supported by LDP) |
| Unsolicited Distribution | Label distribution, which is not triggered by label requests |
| On-demand Distribution | Label distribution, which is initiated by label requests of upstream nodes |
| Pushed Distribution | See unsolicited distribution |
| Pulled Distribution | See on-demand distribution |
| Independent Control | Label distribution, towards the previous upstream node, which is not triggered by a label distribution from the next downstream node |
| Ordered Control | Distribution, allowing label advertisement, towards the previous upstream node, which is triggered by the receipt of a downstream label mapping |
| Liberal Retention | Mode which keeps temporary unused LSPs. |
| Conservative Retention | Mode which destroys LSPs which are not used anymore due to routing table changes |
| HELLO LDP-message | Message used by LDP to discover LDP neighbors and remote peers |
| INITIATION LDP-message | Message used in LDP, for negotiation about LDP session parameters |
| Label Request Message | Message used in LDP, to request a label binding advertisement from the next downstream LSR |
| Label Mapping Message | Message used in LDP, to advertise label bindings to the previous upstream LSR |
| Label Withdraw | Message sent upstream in LDP, to request the tear down of an LSPs |
| Label Release | Message sent downstream in LDP, to tear down LSPs |
| Notification LDP-message | Message used in LDP, to report failures and exceptions |
| Traffic Trunk | An aggregation into a single LSP.of traffic flows belonging to the same class |
| Induced MPLS graph | Represents the logical topology induced by the end-to-end LSPs (links) between LSRs (nodes) |
| Traffic Trunk Attributes | Attributes describing the behavior of the traffic |
| Resource Attributes | Attributes constraining the placement of traffic trunks through the corresponding resource |
| BA | Aggregation of traffic with similar behavior characteristics. The BA is based on the DSCP. |
| OA | A set of BAs that share an ordering constraint |
| PSC | A set of PHBs for traffic preserving an ordering constraint |
| E-LSP | An LSP with a PHB to EXP-field mapping |
| L-LSP | An LSP with a PSC to label mapping and drop preference to EXP-field (or appropriate field in other headers than the shim header) mapping |

## History of Part 2 of Milestone WP1M1 document

| Version | Date | Authors | Comment |
| --- | --- | --- | --- |
| 1.00 | 19/5/2000 | Didier Colle | Initial compilation |
| 1.01 | 22/5/2000 | Didier Colle | Minor editing changes |
| 1.02 | 25/5/2000 | Mario Pickavet | Minor editing changes |
| 1.03 | 6/5/2000 | Didier Colle | Adding SICN contribution |
| 1.04 | 7/5/2000 | Didier Colle | Revision UPC contribution |
| 1.05 | 7/5/2000 | Didier Colle | Adding AGH contribution |
| 1.06 | 8/5/2000 | Mario Pickavet | Minor editing changes |
| 2.01 | 27/6/2000 | Didier Colle | Major reorganisation |
| 2.02 | 30/08/2000 | Janusz Maliszewski | Accommodation to WP1M1dokuments form |

## Part 2 overview

The document is structured as follows. Section 2 gives an overview on the current technologies and products for an IP over WDM environment. Section 3 digs into the details of the mapping solutions to deploy IP over WDM, which will be studied in the LION project. Section 4 and 5 discuss respectively IP over OTN as a new transport network and IP over a more “intelligent” OTN. Section 6 and 7 give an overview on the current available products and standards respectively.

The part one of the milestone WP1M1 was made on working document base: “Network requirements – Products and Technologies” prepared by WP1/Task2 (LION/WPG1/WG1/T2) [NR-P&T].

“IP over WDM” (as transmission support) and “IP over OTN” (as a transport network) are therefore the two major chapters in this contribution. An introductory chapter, including an overview of the current mapping solutions from the IP protocol down to the physical transmission medium, is proposed, in order to highlight where and when the WDM technology is absolutely necessary and/or highly desired. Next chapter deals with the addition of “routing intelligence” to the OTN, and the consequent linking of IP and OTN intelligence, towards integrated solutions.

# IP over WDM: overview on current technology and products

## Currently Deployed IP technology

The Internet Protocol (IP) [RFC-791] was developed to transport data over a mixture of underlying technologies (e.g., Ethernet, PPP, etc). IP provides a connectionless best-effort service in order to provide a global any-to-any connectivity.

Connectionless implies that all packets are treated in an identical way and thus IP does not investigate to which flow the packet belongs. This responsibility to treat the individual flows separately is handed over to the end-nodes. Using store-and-forward routers to provide any-to-any the global connectivity. Each packet entering an IP-router contains at least the destination address. All incoming packets are buffered (or stored) and processed one after the other. Processing means that the destination address is read from the packet header and used to find out on which interface the packet has to be forwarded.

Best-effort means that, although the network aims to provide a global any-to-any connectivity, the network is not able to guarantee that an incoming packet will be delivered to its destination. Of course the goal (or intention) of the network is to deliver the packets properly to their destination: but packets may get lost due to network congestion for example.

Since the main goal of IP is to provide, in all circumstances, a global any-to-any connectivity (over any mixture of underlying technologies) it is based on adaptive and distributed routing protocols. Routers are periodically exchanging topology or routing information in order to be able to react properly to potential configuration updates (e.g. due to failures, network upgrades, etc.).

The conclusion is that IP is very suitable to provide any-to-any connectivity, but suffers from the fact that it treats all packets in the same way and that it guarantees nothing (about packet loss and/or delay for example). TCP is a widely used flow-control protocol to deal with these potential packet losses. However, the growing amount of streaming applications is based on UDP, since packet loss is not that critical for such applications.

The protocols being focussed upon in this document are developed in order to cope with other drawbacks of the IP protocol. Integrated and differentiated services (see section 2.1) aim to provide some QoS guarantees by treating packets belonging to different flows or services in another way. MPLS (see section 3.1) introduces labels in order to avoid the (longest match) calculations to find the next hop: treating labels with direct entries is much easier. MPLS can be used for many purposes (avoiding longest match calculations, traffic engineering, faster network recovery, etc.), as will be shown in section 3.1.

## Integrated and differentiated services

### Introduction

Within the IETF, two mainstream efforts are put forward to implement QoS-extensions in the IP forwarding path. On one hand, there is the Integrated Services (intserv) working group, which is closely associated with the development of the Resource ReServation Protocol (RSVP). The Differentiated Services (diffserv) working group forms the other part. The following sections will go through these two proposed standards and will provide a view on their current status. A common aspect of these standards is that they don’t impose any specific implementation on the network elements. They only define how the element should behave.

### Integrated Services (intserv)

#### Framework

Two evolutions were important for the development of the IS architecture (Integrated Services):

- Multimedia applications often require multicast functionality.

- Streaming applications require some strict guarantees on (for example) end-to-end packet delays.

An important property of the IS architecture is its end-to-end scope. In order to provide each streaming sess ion with strict QoS guarantees, it is required that resources are allocated on a per flow (or session) basis. It is also necessary that IS routers treat packets by taking into account to which flow (or session) they belong, which is in clear contrast to legacy IP routers.

In the framework [RFC-2215] proposed by the intserv working group, a template is given to be able to specify services available for applications [RFC-2216]. Such a service description defines (amongst other elements):

- The motivation for defining that service.

- The data-handling functions that are needed within the network elements.

- The set of parameters needed to be found in a valid user-request. These are typically the traffic specification (Tspec) that describes the characteristics of the data that will need to be treated and the service request specification (Rspec) that describes the QoS-specifications that are being asked for.

- Guidelines for developers: this is an optional part of the service definition. Every designer of a network-element has complete freedom in working out the implementation, only the externally visible behaviour is being standardised.

To be able to implement intserv in a network, two aspects have to be fulfilled:

- Individual network elements must be able to support the services. This means that resource reservations have to be supported.

- Applications must be able to request services. This can be done in several ways: using manual configuration, simple network management protocol (SNMP), RSVP, etc.

For the moment, two services have been defined, and RSVP has been promoted to the standard way to request them. The following sections will elaborate on these issues.

#### Defined services

Next to the best-effort traffic class (which, by the way, allows applications that are not intserv-aware to make use of the network), the intserv-working group is offering two services today.

The first service, the controlled load service, is being defined in [RFC-2211] and it gives the packet stream of the user the quality as it would receive on an unloaded network, no matter what the real load is (e.g. useful for adaptive real-time applications like video-conferencing).

A second one, by the name of guaranteed quality of service ([RFC-2212]) gives a mathematical guarantee concerning bandwidth and delay. This service guarantees that packets that are in conformance with their requested profile (given in Tspec, e.g. requested bandwidth), experience a delay that is lower than the one specified in Rspec. An important remark here is that no guarantee is given towards the instantaneous packet delay variation.

#### RSVP

The resource reservation protocol (RSVP) has the task of making reservations in every network element located on the source-to-destination path. It must be stressed that RSVP is only a signalling protocol that transports the need for reservation to the network elements and returns an indication of its success to the requesting side.

The protocol makes the reservation in two phases [RFC-2205]. In the first one, PATH-messages are send from source (requesting side) to destination. Some of the parameters that are carried in this message are the Tspec and one or more POLICY_DATA objects . Also the IP address of the previous hop is available in the message, to enable to send responses in the other direction (reverse path forwarding). These messages only set the so-called PATH-state in the network elements: no attempt is made to perform any form of reservation. When this phase is completed, RESV-messages are sent back from the destination to the receiver. These messages follow the reverse path (based on the PATH-state) of the future data stream. In every network element a reservation attempt is made in response to the receiving of an RESV-message. If the attempted request is granted, the message is forwarded to the next hop on the reverse path. If it is not granted (e.g. due to inadequate number of resources or policy-collisions), an error message is send. The fact that the protocol is receiver-oriented has a lot of benefits in a multicast environment (e.g. every leaf of the multicast tree can have its own level of QoS and reservations can be merged in the nodes). Another remark, that needs to be expressed, is that RSVP is a soft-state protocol. This means that at regular intervals, the reservations must be refreshed (so the PATH/RESV-signalling must be repeated).

#### Model of a RSVP/Intserv router

Figure 1shows a model of a router in an integrated services network, which has the capability to process RSVP-reservations. The definitions of database elements in the Intserv and RSVP planes are provided in [RFC-2213]. These are necessary to support the overall intserv and RSVP processes. At the forwarding layer, three main functions are necessary: a multifield classification, policing actions and scheduling of the outgoing packets to implement the negotiated service.

Figure 1: RSVP/intserv router

A major disadvantage of the IS architecture is the end-to-end scope, due to treatment of individual flows. This may result in scalability problems in high-speed (backbone) networks.

### Differentiated services

#### Framework and architecture

The DS architecture is designed with the aim to be scalable. This is achieved by following design rules:

- Per–flow state information is kept at the edge of the network.

- Only nodes at the network edge perform per-flow processing, like shaping and policing.

This is obtained by aggregating traffic inside the network into a small number of classes. To specify the class of each packet, the ToS byte of the IP header is used and renamed to DS field ([RFC-2474]). This byte has two fields:

- Differentiated Services Code Point: bits 0-5.

- Currently Unused: bits 6 and 7.

Note that a class does not correspond to a specific service per se, but to a certain Per-Hop-Behavior (PHB) and resources in the routers are allocated per PHB instead of per flow.

#### Defined PHBs

Up till now, the working group has specified two PHBs. Next to these, the codepoint ‘000000’ is reserved for best effort traffic. Due to the fact that only a few applications set the TOS-bits, this solution should keep the compatibility with legacy applications.

The first PHB, called Expedited Forwarding [RFC-2598], is aimed for creating services with virtual leased line-characteristics (these are called Premium Services in [RFC-2638]). This means that the connection should behave as if it was running on its own dedicated link. To put it in more QoS-like terms: the connection should experience a low end-to-end delay, low jitter and assured bandwidth.

A second class of PHBs is being named Assured Forwarding [RFC-2597] (referred to as the Olympic services in [RFC-2638]). The traffic in such a class will have at least a given probability to be forwarded (as long as it is conforming with the agreed profile), but it can be placed in a buffer (this means nothing can be said about delay or jitter). If an AF-class is over its limit and the corresponding queue is getting congested, there is the choice between marking and dropping packets. For this reason, four AF-classes are defined, each with three drop-probabilities. This way a weighted dropping-behavior can be implemented. Furthermore, a diffserv node should not reorder IP-packets of the same microflow if they belong to the same AF-class.

#### Model of a diffserv-router

One of the major strengths of diffserv is its scalability. This is obtained by moving the complexity of the architecture towards the edge (where the packet-density is lower) and keeping the core as simple as possible. This also means that two types of models must be given for the routers in a diffserv domain. The following subsections will describe the basic model, the core router ([DSROUT]) and then combine that with some extensions to find the boundary device ([DSEDGE]).

Core router

The router at the core of the diffserv-network (see Figure 2) must only classify the incoming IP-packets based on their DSCP and apply the right PHB. This is all being done on behavior aggregates, so a scalable solution is obtained.

Figure 2: Diffserv core router

##### Boundary router

As Figure 3 shows, the boundary router is the core router with addition of the traffic-conditioning block (TCB). Four traffic-conditioning functions are defined in [RFC-2475]: metering, marking, shaping and dropping. This is combined with classification based on a DSCP (for host marked traffic) or a certain set of fields.

Figure 3: Diffserv boundary router

## Overview of IP mapping solutions

Classical mapping:			IP/ ATM/ SDH/ fibre

Simplified mappings:

- Simplified layer 2:	IP/ PPP-HDLC/ SDH/ fibre 	(Packet over SONET)

IP/ GbE / SDH / fibre		(Ethernet over SONET)

IP/ SDL/ SDH/ fibre		(SDL)

IP/ LAPS / SDH / fibre		(ref. X.ipos)

IP/ MAPOS / SDH / fibre	(Packet over Lightwave)

- Simplified layer 1:	IP/ ATM (cell-based)/ fibre

“Intelligent” mappings: 		IP/ MPLS (with ATM switches)/ SDH/ fibre

IP/ GbE (with GbE switches)/ SDH/ fibre

IP/ DPT/ (SDH frame)/ fibre

IP/ DTM/ (SDH frame)/ fibre

### The “Classical Telecom” mapping

The classical telecom mapping IP/ ATM/ SDH/ Fibre is:

- a viable option for the Internet world-wide extension:

- lowing the extension of IP protocol beyond the native LAN/Ethernet environment,

- exploiting the existing and effective networks – SDH at layer 1 and ATM at layer 2 - each one optimised to provide the functions of its layer,

- an effective solution in the present scenario, where:

- IP is just one of the clients of the transport network (voice-centric scenario),

- P router interfaces and links between routers (< 622 Mb/s) are not yet “very big pipes“.

In this scenario, IP packets are segmented into ATM cells, assigned different Virtual Connections (switched through ATM core switches) packed into an SDH frame, according to the SDH multiplexing hierarchy, and transported through the SDH transport network. ATM provides variable granularity by the Permanent Virtual Channels using the ATM management system or by Switched Virtual Channels (SVC) dynamically set-up, all within Virtual Paths (VP). It can also use statistical multiplexing to allow certain users to access extra bandwidth for short bursts. This can help to guarantee a fixed and arbitrary bandwidth from less than 1Mb/s to several hundred Mb/s to many different customers. In addition the fine granularity can enable IP routers to be connected into a logical mesh easily, thus minimising delays from intermediate routers.

Several critical issues can be however identified if this solution is applied to the emerging network requirements:

- Entire networks are necessary at each layer:

- Difficult end-to-end provisioning.

- High global cost.

- Possible multiple bottlenecks.

- Partial overlapping of functions (e.g., protection, management functions).

- Heavy overall overhead.

  (1)  How to simplify the “downstairs”  path from IP to WDM?

- It is hard to face the exponential increase of the overall traffic:

- Possible IP router congestion (especially in the backbone).

  (2)  How to reduce the IP Router congestion / latency delay?

- It is difficult to face the increase of the traffic granularity (bit-rate of router I/F )

- IP Routers perform statistical “packet” multiplexing into ‘high capacity pipes’, through high bit-rate I/F.

- ATM bandwidth flexibility is no longer useful on these large granularities.

- SDH equipment is partially redundant (e.g., the use of SDH multiplexing hierarchy is no longer necessary).

- A new “transport” service for IP is necessary.

  (3)  How to optimise this transport service?

### Simplified mappings

In order to answer the issue (1) how to simplify the “downstairs” path from IP to WDM? several simplified mappings have been proposed by different vendors.

In this first step, the use of WDM is not yet exploited for advanced network solutions. The rationale behind these solutions is:

- try to simplify/optimize layering,

- keep layer functions, avoid as much as possible entire stacked networks.

According to the considered context and target applications, simplified mappings can be roughly classified into mappings simplifying layer 2 and mappings simplifying layer 1.

#### Mappings simplifying Layer 2

The mappings simplifying layer 2 are based on the following steps:

- enhancement of IP Router performances:

- higher internal throughput (from Giga to Tera Routers),

- increased number of interfaces (to support enhanced logical mesh),

- increased bit-rate per interface,

- addition of new functions to IP Routers,

- statistical packet multiplexing,

- traffic aggregation into large pipes,

- layer 2 point-to-point data link functions (e.g., packet delineation, error correction),

- consequent simplification of layer 2, avoiding the necessity of a whole ATM network,

- necessity of an efficient underlying transport network, providing fixed bit-rate circuits (close to the peak rate, to guarantee QoS) to realise “rich mesh” connectivity between routers. Actually, SDH can offer these capabilities. The same rationale – just scaled towards higher traffic, higher bit-rates, higher throughputs – is pushing requirements for the OTN (see chapter 3).

The advantage of this solutions, with respect to classical mapping, is mainly the simplification made possible by the elimination of the ATM network. On the other hand, the fact that the SDH network is kept at layer 1 still ensures all the advantages of a transport network, amongst which protection, rich overhead for OA&M and digital performance monitoring, complete network management capabilities.

The disadvantage of this solution is mainly the difficult scalability with respect to the emerging scenarios (increase of overall traffic, traffic granularity, etc.). Disadvantages are exacerbated by the typical multi-ring topology of SDH networks: it is difficult (and expensive) to guarantee a logical mesh on a physical ring. Moreover, bandwidth efficiency is reduced by the necessity of dimensioning the interfaces and the related fixed SDH circuit to a bit-rate close to the maximum peak and not to the average traffic. This may lead to inefficiencies, in spite of the fact that, on the other hand, the weight of the overhead is fairly reduced.

Some of these solutions are listed here below:

- IP / PPP-HDLC / SDH / Fibre 		(POS  = Packet Over SONET)

The basic IP over SONET uses PPP encapsulation and HDLC framing. This is also known as POS (Packet over SONET). POS (one of Cisco’s solutions) places the IP layer directly above the SONET layer, and while offering QoS guarantees, eliminates the overhead needed to run IP over ATM over SONET. It is possible to transport the SDH-framed IP over an SDH transport network along with other traffic, which may then use WDM links. SDH can be used to protect IP traffic links against cable breaks by automatic protection switching (APS) in different ways.

PPP is a standardized way to encapsulate IP and other types of packets for transmission over many media from analogue phone lines to SDH, and also includes functionality to set up and close links. The HDLC framing contains delimiting flag sequences at the start and end of the frame, and also has a CRC field for error control.

The line-card in the IP router can be enhanced to perform PPP/HDLC framing. The optical signal is then suitable for transmission over optical fibre either into an SDH network element, an adjacent IP router, or a WDM transponder. There are different types of IP over SDH interfaces:

- VC4 or Concatenated VC4 “fat pipes” which provide aggregate bandwidth without any partitions between different IP services which may exist within the packet stream.

- Channelized interfaces, where an STM16 optical output may contain 16 individual VC4s, with a possible service separation for each VC4.

This solution is already implemented by some of the major IP carriers:

- Sprint is deploying packet over SONET to boost its Internet backbone speed to 622 Mbps.

- Qwest’s OC-192 SONET network is using packet over SONET to connect its regional terabit points of presence (TeraPOPs), which are essentially IP-based central offices (COs).

- UUNET’s recently announced OC-12 network will use packet over SONET technology.

- GTE Internetw. is deploying Cisco 12000 routers to operate over its nationwide mesh of OC-3 SONET circuits.

- Many European operators have introduced SDH elements with add/drop bit rate up to 140 Mb/s or STM1. In order to support POS, these SDH elements should be upgraded to have the STM-4c and STM-16c (concatenated) interface.

- IP / GbE / SDH / Fibre 		(Ethernet over SONET)

A large part of native IP traffic comes from Ethernet/Gigabit Ethernet LAN (Ethernet accounts for over 85% of LANs world-wide). The new Gigabit Ethernet standard can be used to extend high-capacity LANs to MANs and maybe even WANs, using Gigabit line-cards on IP routers, which can cost 5 times less than SDH line-cards with similar capacity. For this reason, Gigabit Ethernet could be a very attractive means to transport IP over “metropolitan” WDM rings, or even over longer WDM links. Furthermore, 10Gbit/s Ethernet ports are likely to be standardized in the near future.

Ethernet is both a layer 1 physical interface and the most diffused layer 2 protocol. It can be used:

- on the LAN/access side, for layer 1 and layer 2 functions (e.g., access to shared resources)

- on the WAN side, for layer 2 data link functions, while it is interfaced to the SDH/SONET physical layer for transport functions (transmission over medium/long distances, protection/ restoration, fault and performance management, etc.). In this case, the Ethernet layering is valid at level 2 for MAC (the 8B/10B encoding may be avoided). On the contrary, the physical Ethernet 802.3 layer is substituted by SDH/SONET. It should be recalled that when Gigabit Ethernet (1000Base-X) is used in full-duplex mode, it becomes just an encapsulation and framing method for IP packets, and the CSMA-CD functionality is not used.

This solution may assume a particular interest if Ethernet switching is enabled (see section 2.3.2.3). Some options for the mappings Data (IP) over Ethernet and Ethernet over SONET/SDH have been proposed by Nortel (e.g., in ANSI T1X1.5).

- IP / SDL / SDH / Fibre 		(SDL)

Simple Data Link (SDL) is a framing method proposed by Lucent Technologies, which can replace HDLC framing for PPP-encapsulated packets. It is simpler than HDLC and advantageous at high bit rates. Compared with the HDLC frame the SDL frame has no delimiting flag sequences. Instead the SDL frame is started with a packet length field. This is advantageous at high bit rates where synchronisation with the flag sequence is difficult. The SDL format can be inserted as a payload in an SDH frame or directly onto a WDM optical channel. SDL associated functionality is the minimum required. It does not support protection functionality.

- IP / LAPS / SDH / Fibre 		(X.ipos)

LAPS (Link Access Procedure SDH) is a type of HDLC including data link service and protocol specification to adapt directly IP to SDH. It is a substitute for PPP-HDLC. SDH is seen as an octet-oriented synchronous point-to-point links. The SDH frames are an octet-oriented synchronous multiplex mapping structure which specifies a series of standard rates, formats and mapping method. The LAPS shall be encapsulated into a 32-bit word-oriented frame as for the need of providing frame delineation, transparent transferring, and error detection.

- IP / MAPOS / SDH / Fibre 	(POL  = Packet Over Lightwave)

Multiple Access Protocol over SONET (MAPOS): it is a link layer protocol supporting IP over SDH. It is a connectionless packet switching protocol based on a simple extension of POS framing.

#### Mappings simplifying Layer 1

The mappings simplifying layer 1 are based on the following rationale:

- exploitation of layer 2 switching,

- traffic by-pass at layer 2,

- packets are segmented / re-assembled in cells, and handled as virtual circuits,

- use of ATM as transport infrastructure,

- consequent simplification of layer 1 - avoiding a further underlying transport network, possibly using layer 1 standardized physical interfaces.

Although ATM can be an option for this approach, there are severe limitations related to the max. capacity that can be served and the max. granularity which can be offered. This solution is good when the bit-rates of the interconnection pipes are relatively low (155 Mb/s or less) but not suitable to the emerging scenario for the core backbone. It is mentioned here just for completeness.

- IP / ATM (Cell Based) / Fibre

It is possible to have a scenario where ATM cells are transported directly on an optical channel. ATM cells are not encapsulated into SDH frames; instead, they are sent directly on the physical medium by using an ATM cell-based physical layer. Cell-based physical mechanisms have been developed specifically to carry the ATM protocol; this technique can not support any other protocol except if these protocols are emulated over ATM. Some benefits of using a cell-based interface instead of SDH are:

- Simple transmission technique: ATM cells are directly sent over the physical medium after scrambling.

- Lower physical layer overhead (around 16 times lower than SDH).

- As ATM is asynchronous, there is no stringent timing mechanism to be put on the network.

The drawbacks are that the overhead (i.e., the cell tax) is the same as for transport on SDH, the technology has not been endorsed by the industry yet and this transmission technique can only carry ATM cells.

#### “Intelligent” Mappings

In order to answer the second issue raised in section 2.3.1 - (2) how to reduce the IP Router congestion/latency delay? – the following approaches can be adopted:

- Traffic by-pass at lower layers: typically, exploitation of layer 2 switching.

- Enhancement of the logical connectivity seen by IP Routers, through improvement of the physical connectivity at lower layers.

- Introduction of “interworking” at control plane level.

Several interesting new technologies are emerging, but, for the moment, as single wavelength solutions limited to metro-net scenarios and not yet demonstrated for very high capacity backbone. Moreover, these technologies are not yet ready to face the remarkable increase of traffic granularity currently required.

Some solutions are here just mentioned (since they are dealt with in other contributions to the same WP 1 – Task 2):

- IP / MPLS (with ATM switches) / SDH / Fibre

This is a successful solution, emerged from the “IP standardization world”. It is la label switching, driven by the IP control plane, and actuated by ATM switches. It allows to by-pass the express traffic at layer 2, giving relief to the layer 3 routers. A whole part of the WP1 Task 2 document is devoted to this technology.

- IP / GbE (with GbE switches) / SDH / Fibre

The idea is that of exploiting Ethernet switching at each access node to a ring. In this way, the flow of transit packets is distinguished from the flow of packets destined to (or coming from) the local IP Router. A layer 2 by-pass is performed, and the sharing of a common resource (ring big-pipe) is allowed (up to the 1.25 Gb/s allowed by Gigabit Ethernet) achieving all the benefits of a statistical multiplexing applied by means of packet switching.

To obtain these advantages, it is necessary to change (and enhance) the Ethernet MAC through proper SRP protocols and fairness algorithms. Different physical options can be implemented (according to whether Ethernet switch cards are integrated in the layer 3 router or in the layer 1 node, or in a stand-alone further node).

An example of product proposal is the interWAN Packet Transport (iPT), by Nortel, which is claimed to deliver lower cost IP-optimized networking capabilities to today’s wide area networks over existing SONET/SDH and DWDM optical networks. IPT essentially multiplexes and switches packet traffic in its native format over SONET/SDH platforms. The benefits are optimized use of transport network bandwidth, reduced port costs, and the ability to support a mix of packet and TDM traffic over the same network. With Ethernet add/drop and switching capabilities at each node, interWAN Packet Transport optimizes the optical network usage through packet multiplexing and through a unique spatial reuse algorithm. Instead of dedicating a number of links for each connection, iPT provides logical connectivity for optimum bandwidth sharing.

- IP / DPT / (SDH frame) / fibre

Dynamic Packet Transport – DPT (Cisco proprietary). It is associated to a transport ring topology. It provides the Spatial Reuse Protocol (SRP) to optimize bandwidth optimization and a protection mechanism called Intelligent Protection Switching (IPS). It is based on a further enhancement of IP Router functions: traffic by-pass, queue handling at layer 2, statistical “packet” multiplexing on shared pipes, protection features. This approach not only avoids the use of ATM, but also of SDH (only SDH interfaces are kept).

- IP / DTM (with DTM switches) / (SDH frame) / fibre

DTM is an effort to combine the advantages of asynchronous and synchronous data transfer. It is a TDM scheme, which guarantees each host a certain bandwidth and uses a large fraction of the available bandwidth for effective data transfer. The DTM scheme has in common with ATM, support for dynamic reallocation of bandwidth between hosts. This means that the network can adapt to variations in the traffic and divide its bandwidth between hosts according to their demand.

Hosts connected to a DTM network communicate with each other on channels (circuits). A DTM channel is a dynamic resource that can be set up with a bandwidth ranging from 512 kb/s in quantum steps of 512 kb/s up to the maximum bandwidth. The total capacity is divided into frames of 125 microseconds, which are further divided into 64-bit time slots. This framing structure makes it interoperable with SDH/SONET. Different types of slot reservation can be assigned according to the QoS a client wants: constant delay, minimum bandwidth, best effort.

To interconnect different DTM links, DTM switches should be used. The switching in DTM is synchronous, which means that the switching delay is constant for a channel.

DTM channels are multicast in nature: any channel at a given time can occupy one sender and any number of receivers. DTM is suited as a backbone technology because of its high bit rate throughput. DTM is seen as an alternative for ATM over SDH because it operates at layer 1 to 3 and includes switching and a signaling protocol.

Flows of IP can be mapped on DTM channels. However, DTM is slightly inefficient for IP since it uses channels with a minimum capacity of 512 kb/s.

DTM has enough switch capacity to handle WDM. Hereby the basic assumption is that one WDM colour will go on one DTM channel, which is only possible when the transmission method is supported by DTM.

A drawback of DTM is that currently only three vendors are supporting DTM, Dynarc, Net Insight and Ericsson. Moreover, their solutions are proprietary and not compatible with each other.

## IP over WDM as transmission technology

All the mapping solutions mentioned in chapter 2.1 may be ultimately carried by WDM, rather than being transmitted through a dark fibre. This is advantageous, since very high-capacity WDM systems have already been deployed (especially in the backbone transport). Moreover, this is becoming strictly necessary, because WDM is the only technology whose capacity can support the traffic growth related to the new data-centric network.

More specifically:

- All the mapping solutions including SDH as layer 1 may be supported by the WDM transmission systems already installed for the long-haul SDH network. What is necessary is simply a transponder between the SDH STM-16/STM-64 termination and the “coloured” interface towards the WDM line system.

- IP/ ATM/ SDH/ WDM

- IP/ PPP-HDLC/ SDH/ WDM

- IP/ GbE / SDH / WDM

- IP/ SDL/ SDH/ WDM

- IP/ LAPS / SDH / WDM

- IP/ MAPOS/ SDH/ WDM

- IP/ MPLS (with ATM switches)/ SDH/ WDM

- Even the other mappings – which do not include SDH – can be transported by WDM, through proper transponders between the router/switch and the ends of the WDM line system. Once more, this is done just to increase the capacity of the links, without changing the working principle of the adopted technique.

- IP/ ATM (cell-based)/ WDM

- IP/ DPT/ (SDH frame)/ WDM

- IP/ DTM/ (SDH frame)/ WDM

Therefore, “indirect” IP over WDM is already a reality, especially in backbone network segments. The huge capacity increase allowed by WDM, although not involving advanced network aspects, is by itself an unavoidable pre-requisite for all the consequent progresses, and should not be under-evaluated.

However, this is not yet “IP directly over WDM”: layer 2 and layer 1 functions (and often entire layer 2 and layer 1 networks) are still necessary in between. In other words, WDM plays here the role of a “layer 0”.

In general (as partially discussed in chapter 2.1), a layer 2 network might be useful to allow:

- fine granularity bandwidth management,

- differentiation of QoS on different flows,

- traffic by-pass at layer 2,

- point-to-point layer 2 functions (packet delineation, error correction, etc.).

In general (as further discussed in chapter 3), a layer 1 network might be useful to provide:

- high-bit-rate interconnections, through high capacity point-to-point links,

- advanced protection/restoration functions,

- performance, fault, configuration management on connections,

- traffic by-pass at layer 1.

For clarity’s purpose, the following table is proposed:

| OSI layer | # | Function | Handled Entities | Network | Ref. Technology |
| --- | --- | --- | --- | --- | --- |
| network | 3 | Routing | Packets | Internet | IP |
| data link | 2 | Switching | cells, virtual circuits | Switching Network | ATM |
| physical | 1 | cross-connection | “TDM” circuits | Transport Network | SDH |
| - | 0 | high-capacity transmission | wavelengths | - | WDM |

Table 1:  Overview on different interpretations of the layering concepts

### Enabling technologies

WDM technologies for line systems are mature and effective. Nonetheless, new technologies are emerging to allow further performance improvements. A quick overview of the main optical components of a WDM line system and of the related enabling technologies is here recalled.

| Functional Block | Consolidated Technology | Advanced Technology |
| --- | --- | --- |
| Source | Distributed FeedBack (DFB) Lasers Distributed Bragg Reflector (DBR) Lasers | SSG (Super-Sampled Grating) DBR for wide tunability Vertical Cavity Surface Emitting Lasers (VCSEL) for low-cost arrays |
| Intens. Modulator | Mach Zehnder interferometer in LiNbO3 | Integrated semiconductor Electro-Absorption modul. |
| Receiver Photo-Detector | PIN photodetectors (up to 10 Gb/s ch.) Avalanche-Photo-Diode - APD (up to 2.5 Gb/s ch.) | Avalanche Photo-Diodes – APD (also for 10 Gb/s ch.) |
| WDM Mux/Demux | micro-optics diffraction gratings cascade of interferential filters cascade of Fibre Bragg Gratings (FBG) | Arrayed WaveGuides (AWG), built as Planar Lightwave Circuit (PLC) |
| Optical Amplifier | Erbium-Doped Fibre Amplifiers (EDFA) (optimized for multi-channel WDM, also through external control gain systems) | Different dopings for extended bandwidth (e.g. Erbium-Doped Fibre Fluoride Amplifiers -EDFFA): the objective is to have two usable bandwidths, C and L, over a whole range between 1530 and 1620 nm |
| Dispersion Compensation Module | Dispersion Compensating Fibre - DCF (on multi-channel WDM) Chirped Fibre Grating (single channel) | Chirped Fibre Grating (also on multi-channel WDM) |

Table 2:  Overview on different interpretations of the layering concepts

# Mapping Solutions taken into consideration in LION

## Multi-Protocol Label Switching

Because of its features, MPLS is widely considered as a viable technology for traffic engineering. This section first presents MPLS notions, concepts and the architecture and then focuses on MPLS-based traffic engineering aspects and approaches.

### What is MPLS?

MPLS can be a lot of things depending on the point of view. The following sections will describe what MPLS is, seen from different points of view. These sections also try to point out what MPLS is not.

MPLS is a technique for IP over ATM inter-working.

MPLS is a convergence of a number of “IP switching” schemes. IP switching is a technique that uses ATM hardware to speed up the forwarding of IP packets. It is important to know that the ATM hardware is controlled by IP routing and not by ATM signalling. There are a number of different IP switching implementations: Cisco Systems Tag Switching, IBM’s Aggregated Route based IP Switching (ARIS), Toshiba’s Cell Switch Router (CSR) and NEC’s Ipsofacto (now renamed to Lcatm) [SWITCH]. In order to standardise all these IP switching techniques a new IETF working group came to life in 1997. The MPLS working group has since then been working on forming a common technology for IP switching.

MPLS is NOT an overlay technique.

There are a number of techniques for IP over ATM which are overlay techniques, MPLS is not an overlay technique. Examples of overlay techniques are LANE [LANE], MPOA [MPOA] and the work done in the IETF ION working group. In the case of overlay techniques there are two different networks: a network at layer 2 (ATM) and a network at layer 3 (IP). This leads to a number of disadvantages: the two networks both have to be managed (more management overhead), both L2 (switches) and L3 (router) equipment is necessary (more equipment) and the scalability is limited (due to full meshed peering) [MYTH].

An MPLS network is one single network in contrast with the overlay techniques. In MPLS there is however a separation between control and forwarding. The control is based on standard IP control mechanisms like IP routing and an extra control protocol called Label Distribution Protocol (LDP) [LDP]. LDP will be introduced in the next section and will be described further in detail in the following sections. The advantage of separating forwarding and control is that changing control will have no (or limited) consequences for the forwarding. This makes MPLS a very flexible approach because it makes it easier to introduce new features in the control layer.

MPLS is an advanced forwarding scheme.

In regular (that is non-MPLS) IP networks packets are forwarded in a router after consulting the routing table. A routing table contains information about the next hop and the outgoing interface for a certain destination address (found in the IP header). The destination addresses in this table are aggregated in order to reduce the number of entries in this table. The entries are aggregated, by indicating the length of the destination addresses (from 0 to 32 bits). If n is the length of address a then only the first n (most significant) bits of a are considered. The resulting address is called a prefix. This aggregation of addresses has the drawback that searching through the table becomes more complex. Instead of looking for an exact match in the table, the result of the search must be the entry with the longest address that matches the address. This process is called a longest prefix match.

MPLS takes another approach to route the IP packets through the network: labels. Labels are fixed length entities that have only a local meaning. In the case of ATM based MPLS a label is a VPI, VPI/VCI or a VCI identifier [VCSW], [VCID]. In traditional ATM networks these identifiers are installed with UNI or PNNI signalling. Since MPLS does not use ATM signaling another protocol is needed. The MPLS working group developed the Label Distribution Protocol (LDP). LDP is responsible for distributing labels across the different MPLS routers (called LSRs: Label Switching Routers) so that a packet can travel across the network by switching labels in the core. The concatenation of these installed labels in the different LSRs is called a Label Switched Path (LSP). How these labels are installed and what event (trigger) leads to the installation of the labels will be described later on.

MPLS is NOT a QoS framework, however facilitates QoS delivery.

MPLS is sometimes seen as a technique that stands on the same level as Intserv and Diffserv. This point of view is not correct: MPLS is an advanced forwarding scheme that can have a better forwarding performance then traditional routers (although gigabit routers can be made as fast as MPLS routers) but this performance gain does not make it a QoS framework.

There are however a number of relations between QoS and MPLS. With the use of CR-LDP (extensions to LDP for traffic engineering) [CRLDP] it becomes possible to signal the requested QoS level for a LSP. Another approach is to use a modified version of RSVP for traffic engineering [RSVPT]. Even with the use of CR-LDP or RSVP topics like admission control algorithms, scheduling, constrained based routing etc. are not a part of MPLS.

MPLS is NOT constrained to only ATM as a link layer.

Although the IP switching started out as a technique for IP over ATM this is no longer true, MPLS is now available over a number of link layers. These link layers can be divided in two categories. To the first category belong the link layers that already switch on “labels”. Examples are of course ATM but also Frame Relay (the DLCI values are used as label) [FR]. The second category contains link layers that do not have a field in the header that can be used to transport the labels. In this case an extra header, the shim header is used [ENCAPS]. This shim header is not link layer specific. The header is inserted after the layer 2 header and before the IP header, this is necessary to support IP fragmentation. Ethernet and PPP are examples of link layers that use a shim header.

### MPLS architecture

The MPLS architecture consists of a control layer and a forwarding layer [ARCH], [MPLS-FRAME]. The functionality of the forwarding layer is simple: to switch labels. The control layer is responsible for the IP control functionality of a regular IP router and the distribution of labels.

A closer look at labels.

It is apparent that “labels” constitute the centre of the MPLS architecture. However these labels do not look the same for every link layer. There are two types of link layers in this context: link layers that already support a kind of “label” to switch and link layers that need to use a “shim header” which contains a label. The former category includes ATM and Frame Relay, the latter includes PPP and Ethernet.

ATM and Frame Relay already have an identifier that can be used as a label. In ATM this is the VPI/VCI field in the cell header and in Frame Relay this is the DLCI field. These two link layers have support for switching on these fields in hardware.

On the other hand PPP and Ethernet need a shim header. This shim header (see Figure 4 and Figure 5) contains everything that is needed to forward packets through a MPLS domain. First of all the shim header contains a label but the header also contains three experimental bits (used in Diffserv [MPLS-DS] and ECN [MECN]), a bottom of stack indicator (needed for label stacking, see next section) and a TTL field (will be explained later).

Figure 4: The shim header

Figure 5: the position of the shim header

Label stacks

The labels used in MPLS (ATM VPI/VCI, Frame Relay DLCI and the label field in the shim header) are just identifiers; they are used to create label switched paths. In traditional ATM the VPI field can be used to aggregate multiple PVCs or SVCs. MPLS has a similar but more generic concept to aggregate LSPs. Multiple shim headers can be stacked. We will first look at link layers that only use shim headers (like PPP and Ethernet) and afterwards look at ATM and Frame Relay.

At every node only the top label is considered when making the forwarding actions. On inspection of the top label the LSR can do the following operations on the label stack: replace the top label with a new label (swap), pop the label stack or replace the top label and push a number of new labels on the label stack.

ATM and Frame Relay switch on VPI/VCI and DLCI identifiers and not on labels in the shim headers. However (limited) support is provided for label stacks in an ATM or FR segment of a MPLS network. When the packet enters the ATM/FR segment the top entry of the label stack is copied into the VPI/VCI or DLCI field.  Then the value of the label at the top entry of the label stack is set to zero. In the ATM/FR segment of the MPLS network it is not possible to do operations on the label stack, the only operations possible are the operations on the copy of the top entry of the label stack. In the ingress of the ATM/FR segment it is possible to push the label, in the core to swap the label and in the egress to pop the label. After the packet has passed the ATM/FR segment the VPI/VCI or DLCI label is copied back into the top entry of the label stack.

TTL

The TTL field in the IP header has to be decreased at every hop in an IP network. This mechanism is used to prevent packets from living forever. The TTL field is also used in utilities like ping and traceroute. The section describes the support for TTL decrement in MPLS networks.

Again there’s a difference between ATM/FR and PPP/Ethernet. Since the shim header contains a TTL field the LSRs are able to decrement the TTL just like in the normal IP forwarding case.

ATM/FR LSRs can not decrement a TTL field since they can only act on their native identifiers (VPI/VCI, DLCI). The solution for these link layers is to compute the whole TTL decrement for the ATM/FR segment a LSP traverses [MICMP]. This TTL decrement is computed before the LSP is used. When a packet arrives at the first ATM/FR LSR the calculated TTL is subtracted from the current TTL (found in the shim header). Appropriate actions must be taken on this result (e.g. sending ICMP messages). If the result is positive then this result is written in the top entry of the label stack. So a packet travelling through a LSP which goes through an ATM/FR segment will have the correct TTL before the ATM/FR segment and after the ATM/FR segment. The TTL will have a constant value in the ATM/FR segment (the same value as just after the ATM/FR segment).

Label merging

Some link layer technologies are capable of merging labels. Label merging means that if there’s a merge point (a multi-point to point subtree) in the network then only one label is needed as outgoing label.

Figure 6: multiple LSPs with non-merge capable LSRs

When dealing with non-merge capable media one must use a different outgoing label for every incoming branch of the merge point. This leads to a situation where multiple LSPs needs to be set up between two adjacent LSRs, as shown in Figure 6. Most ATM switches and Frame Relay equipment are non-merge capable.

The distribution of labels

There are two issues when looking at the distribution of the labels in a MPLS network. The first one is what effect triggers the distribution of the labels (data flowing through the network, routing changes or reservations) and the second issue is which protocol is used to distribute the labels. Also some of the protocols have a number of modes in which to operate.

When looking at the protocols to distribute the labels there are again two possibilities: devise a new protocol (LDP – Label Distribution Protocol- and Constraint Routed-LDP) or piggy back it on an existing protocol (tunnel extensions for RSVP or BGP [BGPLAB]).

In the following sections we will look at hop-by-hop routed LSPs. This means a LSP that is set up according to the routing tables of the LSRs in the MPLS network. The alternative is to use an explicit routed LSP (ER-LSP): a LSP that is set up following the path that is included in the request message.

The control protocol that is proposed by the MPLS working group as the control protocol for hop-by-hop routed LSPs is LDP. Two approaches are discussed for explicit LSP set up in the MPLS working group: CR-LDP and extensions to the RSVP protocol. The following section describes the LDP.

### Label Distribution Protocol (LDP)

As mentioned before, LDP distributes labels in the MPLS network. Labels are distributed for “Forward Equivalent Classes” (FECs). A FEC can be either an IP prefix or an IP host address. Addresses that belong to the same FEC can be forwarded in the MPLS domain in the same way. As a result it suffices to construct one label switched path for each FEC. When a label is mapped to a certain FEC one speaks of a FEC-Label binding. A LSP can be regarded as a concatenation of FEC-Label bindings over a number of LSRs. An important consequence of this definition is that a LSP is an unidirectional path.

LDP gives the user a great deal of freedom how to set up the LSPs. This is reflected in a number of different modes [ARCH]. These modes will be described in the following subsections. The reason for the multitude of modes is that support is needed for multiple link layers (some link layers mandate some modes), the desire to “emulate” the proprietary IP switching techniques and the desire for flexibility.

Topology driven versus data driven

The LSPs for the different FECs in a network can be set up according to the routing table before traffic begins to flow through to the network (topology driven). Alternatively the LSPs can be set up after a certain amount of traffic has passed through two points of the network (data driven or flow driven). Topology driven MPLS has the advantage that the number of LDP messages is small (LDP activity takes place initially when MPLS is enabled and after routing changes). However topology driven MPLS has the disadvantage that generally a full mesh of LSPs is constructed between all the edge LSRs (also called Label Edge Routers LERs). Depending on whether or not the LSRs are merge capable the number of LSPs is O(n) or O(n2) respectively.

Data driven MPLS has the advantage that it potentially requires less LSPs. The disadvantages of flow driven MPLS are the greater LDP overhead, extra functionality needed (flow detection), the set up delay and a more unpredictable network behaviour (different behaviour when traffic is switched).

Downstream versus upstream allocation

Since labels only have a local meaning, these labels can be allocated decentralised by the switch controllers. A core LSR has always two neighbors: one upstream and one downstream. It also has an incoming label and an outgoing label. With that in mind there are two approaches possible: the LSR supplies his upstream neighbor with a label and receives a label from his downstream neighbor or the other way around. When the LSR receives the label from his downstream neighbor this is called “downstream allocation”. LDP only supports downstream allocation.

Unsolicited distribution versus distribution on demand

The previous subsection described that labels are chosen (allocated) by the downstream LSRs. When these labels are distributed spontaneously this is called “unsolicited label distribution”. When the upstream LSR always sends a request to his downstream neighbor this is called “distribution on demand”. Unsolicited label distribution is also called “Pushed” distribution while distribution on demand is called “Pulled” distribution.

Independent versus ordered control

In “independent control” an LSR, recognizing a particular FEC, makes an independent decision to bind a label to that FEC and to distribute that binding. In “ordered control” an LSR only binds a label to a particular FEC if it is the egress LSR for that FEC, or if it has already received a label binding for that FEC from its next hop for that FEC.

Liberal retention versus conservative retention

Consider the situation where an upstream LSR has received and retained a label mapping from his downstream peer. When the routing changes and the original downstream peer is no longer the next hop for the FEC then there are two possible actions the LSR can take. The LSR can release the label, this is called “conservative retention” or it can keep the label for later use: “liberal retention”.

Conservative retention (“release on change”) has the advantage that it uses fewer labels, liberal retention (“no release on change”) has the advantage that it allows for faster reaction to routing changes.

Label use method

Labels can be used as soon as a LSR receives them (“use immediate”) or the LSR can use the label unless a loop has been detected (“use loop free”) [LOOP1], [LOOP2].

Supported LDP modes

Media that do not support label merging (most ATM and Frame Relay switches) must use distribution on demand.

Media where the labels are a scarce resource (again ATM and FR) should use conservative retention and distribution on demand.

Important LDP messages

This section briefly describes the important LDP messages. The “Hello” message is used to discover LDP neighbors and remote peers (with the use of “Targeted Hello”). The “Initialisation” message is used to negotiate about the LDP session parameters. LSPs are set up with the “Label Request” and “Label Mapping” messages. “Label Withdraw” (upstream) and “Label Release” (downstream) messages are used to tear down LSPs. Exceptions and failures are reported with “Notification” messages.

### Diffserv in MPLS-based networks

The recent work of the MPLS workgroup has been described in [MPLS-DS]. Special about the MPLS architecture is its connection-oriented nature, which implies that packets of a particular LSP will not be delivered disordered. Therefore the Ordered Aggregate (OA) concept has been defined [DIFF-NEW]. An OA is a set of BAs that share an ordering constraint. The corresponding set of PHBs is called a PHB Scheduling Class (PSC). More precisely, a PHB can be seen as the PSC plus the drop precedence.

To support DiffServ on an MPLS-based network, there exist mainly two possibilities

- EXP-Inferred-PSC LSPs (E-LSPs): in this case a single LSP can support at most 8 BAs, by mapping the EXP-field in the shim header (see Figure 4) to the appropriate PHB. This mapping can be pre-configured or explicitly signaled at label setup.

- Label-Only-Inferred-PSC LSPs (L-LSP): this case requires that at LSP setup the label-to-PSC mapping is explicitly signaled. The drop precedence is carried in the MPLS header. In the case of the shim header, the EXP field is used for this purpose. In the other case, the right field in the link layer header has to be used.

Note that these binding schemes don’t specify anything about resource allocation. Resource (e.g. bandwidth) requirements may be signaled explicitly at LSP setup. Those signaled requirements can then be used to perform admission control or to update the current amount of allocated resources. In the case of L-LSP binding resources can be explicitly allocated for the PSC corresponding to the LSP being established. In the other case, for E-LSP bindings, resources can only be allocated for the whole set of PSCs corresponding to that particular LSP.

### Traffic Engineering (TE) support in MPLS

Although MPLS is not a framework developed for traffic engineering, it provides some significant tools for this purpose due to its path-oriented nature [CONC-TE], [MPLS-TE]. Concepts like Forwarding Equivalent Class (FEC), traffic trunk and explicit routed LSPs (ER-LSPs) are important features for traffic engineering. A traffic trunk aggregates traffic flows belonging to the same class into a single LSP [RFC-2430]. It can be seen as a routable object: this means that its path can be changed.

These concepts make it possible for MPLS to allow sophisticated routing control capabilities and QoS resource management techniques [MPLS-QoS], needed for traffic engineering.

Important to support TE in MPLS, is the induced MPLS graph [MPLS-TE]: the nodes of this graph represent the LSRs of the network and the links represent the logical end-to-end links between these LSRs, provided by the LSPs. There are two main problems to solve:

- Which traffic flows to aggregate in the same traffic trunk or LSP? And what resources (e.g. capacity) is required for each traffic trunk?

- How to map this logical induced MPLS graph on the physical topology, taking into account the limited resources on this physical topology?

To be able to solve this problem traffic trunks and resources are assigned a set of attributes.

- Traffic trunk attributes: describe the behavior of the corresponding traffic trunk (e.g., traffic parameter, generic path selection and maintenance attribute, priority, pre-emption, resilience and policing attributes).

- Resource attributes: constraint the placement of traffic trunks through the corresponding resource.

A constraint-based routing framework should come up with a route for each LSP, without violating the constraints implied by the traffic trunk and resource attributes. Note that the framework may calculate these routes off-line.

## Dynamic Packet Transport

### Introduction

Telecommunications networks around the world are facing a significant change in their network architectures: the shift from a voice based (circuit) to a data based (packet) network. The explosion of bandwidth fueled by the growth of data traffic, particularly IP traffic, is disrupting the nature of networks and is driving next generation architectures. Data traffic, which has been accommodated as well as possible on the voice-centric network, can no longer be handled by optimizing the old circuit-switched infrastructure.

The existing circuit-oriented public infrastructure, based primarily on time-division multiplexing (TDM), must evolve and incorporate data packet technology that combines high-performance switching and routing with new and existing optical technologies and standards: it requires optical internetworking.

Optical internetworking combines high-performance data and optical networking technologies to create new optical networking solutions that can efficiently support the exponential growth of data traffic.

Cisco offers a five-phase plan to face the deployment of optical internetworking while optimizing the existing technology and investment (Figure 7). The latter phases of the plan enable the optical internetworking concept providing data optimized transport over optical transmission technologies.

Figure 7: Cisco’s Optical Internetworking Strategy.

The Dynamic Packet Transport (DPT) is placed in the fourth phase of Cisco’s optical internetworking strategy as metropolitan and wide area network solution providing an innovative and optimized network architecture for ring-based delivery of IP services [DPT-1]. Even if the advantage and applicability of DPT as wide area network solution is not clear in the moment, it seems to be a interesting solution for local - and metropolitan area applications.

Some key benefits of DPT technology are listed below:

- Like other optical internetworking solutions, DPT reduces costs and complexity by eliminating intermediate layers between the IP layer and the optical layer. In this case, it would be eliminated the SONET/SDH equipment.

- DPT increments bandwidth efficiency because of the spatial reuse of bandwidth and the statistical multiplexing of packets.

- DPT enables services such as voice and video over IP and Virtual Private Networks (VPN) to be transparently and robustly extended to the metropolitan and wide area networks.

The DPT functionality is enabled by a new Media Access Control (MAC) layer protocol called Spatial Reuse Protocol (SRP) operating over a dual-ring network topology.

### DPT Foundations

This section presents basic terminology used with the DPT system and the SRP protocol.

As shown in Figure 8, a DPT ring is composed of nodes that are interconnected by a dual ring consisting of two counter-rotating fibers. In order to differentiate between the two rings, the rings are referred to as “outer” ring and “inner” ring. Both rings transport concurrently data and control packets and while data information is sent in one direction, called ‘downstream’, the corresponding control messages are sent in the opposite direction, called ‘upstream’ [DPT-2]. Therefore, control signal propagation that is necessary for access control and self-healing purposes can be accelerated following the shortest path.

Figure 8: DPT Ring.

DPT provides the following features inherited from the SRP protocol:

- Efficient use of bandwidth.

- Support for priority traffic and multicasting.

- Scalability across a large number of nodes.

- Topology discovery.

- Fairness among nodes accessing to the ring.

- Redundancy and protection in the event of a failed node or fiber cut.

These features are explained in the next section when defining and describing the SRP protocol.

### The Spatial Reuse Protocol

The Spatial Reuse Protocol is a new MAC layer protocol for ring configurations and takes its name from the spatial reuse concept. The entire protocol is exhaustively treated in an Internet Draft [DPT-3].

In principal the SRP protocol is layer 1 (media) independent and can be used over a variety of underlying technologies such as SONET/SDH, WDM, dark fiber, or mixed environments. The initial SRP implementation makes use of SONET/SDH framing. This mapping is the same as the Packet over SONET (POS) mapping, using the point-to-point protocol (PPP) and the High-level Data Link Control (HDLC) to encapsulate SRP packets within a SONET/SDH frame. This initial implementation also supports SDH OAM-flows.

The use of DPT on top of a SDH or WDM infrastructure will enhance the possible link span because the maximum link span depends on these underlying transport technologies only.

Among other important features such as bandwidth efficiency or priority and multicast supporting, two protocols or algorithms lay the foundations of the SRP protocol: the SRP fairness algorithm (SRP-fa) and the Intelligent Protection Switching protocol (IPS). The former controls the access to the shared media ensuring fairness, bounding latency and avoiding privileged nodes or conditions while undertaking to prevent congestion, and, the latter consists of a protection scheme of the dual-ring. These two algorithms will be explained later in this section.

#### SRP Features

As mentioned before, DPT provides a set of features inherited from the SRP protocol. Next, these and other features are explained:

##### Bandwidth Efficiency

The spatial reuse concept, which is used in rings to increase the bandwidth available of the ring, refers to the fact that packets (unicast) only circulate along spans between the source and the destination node rather than the whole ring as in other protocols such as FDDI and Token Ring.

This operation, in which destination nodes remove the packets from the ring, is known as destination stripping. Unlike protocols such as Token Ring and FDDI, there are no shared tokens to access to the ring (source stripping) and it is used the SRP-fa to control the access to the media. Due to the spatial reuse (or destination stripping), the bandwidth is used in a more efficient manner since bandwidth is only consumed on traverse segments allowing nodes to transmit concurrently. In addition, the use of the bandwidth is dynamic; there are no bandwidth reservation or provisioned connections.

##### SRP Packet Formats

As depicted in Figure 9, there are two types of SRP packets: SRP data packets and SRP control packets. The figure shows the packet formats of the SRP version 2 where the SRP header has passed from 4 bytes to 2 bytes, thus, reducing overhead.

The maximum transfer unit (MTU) for data packets is 9216 octets and the minimum transfer unit is 55 octets. The minimum limit corresponds to ATM cells transported over SRP since the version 2 of the protocol has added a SRP cell mode to carry ATM as payload (2 bytes SRP header + 53 bytes ATM). Nevertheless, IP datagrams are the main payload of SRP packets.

Figure 9: SRP Data and Control Packet Formats.

There are three different control messages used in SRP rings: Usage, Topology Discovery and Intelligent Protection Switching control packets.

##### Packet Prioritization and Processing

In order to provide support for real time, mission critical applications and control traffic, the SRP protocol has been designed to support packet prioritization.

The node utilizes a mapping between the IP precedence bits in the Type of Service (ToS) field into the SRP MAC header priority field (mapping 16 levels of priority to 8 levels).

It is worth noting that, although the SRP priority field size are 3 bits (8 possible levels), there are only two levels of priority working within SRP rings. Thus, a priority threshold is used to determine if the packets should be placed in the high or low priority queues (mapping 8 levels of priority to 2 levels). However, the full 8 levels of priority in the SRP header can be used prior to transmission onto the ring as well as after reception from the ring.

The motivation of these two levels of priority is the use of a transit buffer in the node that consists of two separate fifo queues (high and low priority) in order to forward packets to other nodes. To send their own packets, the nodes make use of a transmit buffer also divided into two different queues (high and low). The following figure illustrates the SRP packet processing (reception and transmission) in which both the transit and the transmit buffer are shown.

Figure 10: SRP Packet Processing Flow.

The figure shows only the operations made on a single ring (the outer or the inner). The other part of the dual-ring is symmetric. When a packet arrives to the node, it is checked its source and destination address (address lookup) and its mode (type of packet) since packets can either be data or control packets. Simplifying the reception algorithm:

- If the destination MAC address corresponds to the node MAC address and the packet (data packet) is unicast, then the packet is copied to the reception buffer and stripped from the ring. If the packet is multicast then is forwarded since multicast packets need to be stripped by the source instead of the destination node.

- If the destination MAC address match is not made or the packet (data packet) is multicast, the packet is placed into the transit buffer to be forwarded to the next node if the packet passes Time to Live and CRC tests.That means all packets which are not stripped in the node will be forwarded at the SRP-MAC layer directly and do not stress the corresponding IP-router in the node.

- Control packets are always stripped once the information is extracted and copied to the reception buffer.

A transmission algorithm is needed since the packets can be sent either from the transit buffer (data generated by other nodes) or the transmit buffer (data generated by the own node) and both handling different priorities (high or low). The transmission algorithm is the following:

High priority packets from the transit buffer are always sent first.

High priority packets from the transmit buffer are sent as long as the low priority transit buffer is not full.

Low priority packets from the transmit buffer are sent as long as the low priority transit buffer has not crossed a threshold indicating this situation and the SRP-fa rules allow it.

If nothing else can be sent, low priority packets from the low priority transit buffer are sent.

It is worth noting that the SRP-fa contributes to the transmission algorithm controlling the access to the ring (step 3).

##### Multicasting

SRP provides direct support for IP multicasting. IP multicast uses class D address space and this class D multicast address is mapped to the appropriate 48-bit MAC address for transport on the ring.

Unlike unicast packets, multicast packets are source stripped. The multicast packets are placed into the transit buffer for continued circulation.

##### Pass-through Mode

DPT nodes have an optional mode of operation known as Pass-through Mode. In this mode of operation, the node appears invisible to the ring since the node cannot source packets onto the ring and both control and data packets received are immediately forwarded without any type of processing (neither Time to Live and CRC processing nor SRP-fa are executed).

This functionality allows nodes to handle router hardware or software problem avoiding ring wraps or segmentation.

##### Topology Discovery

The SRP nodes dispose of a mechanism to discover the topology existing on the ring. This functionality utilizes a specific control packet know as Topology Discovery control packet.

Figure 11 illustrates the topology discovery mechanism. The discovery process begins with a node sending out topology discovery packets on one or both rings. The packet travels from node to node appending MAC binding information (the node MAC address and MAC status) until it arrives to the node originating. The packet has to be received on the same ring before it can be accepted and the topology is changed only after receiving two packets that are identical.

Figure 11: Topology Discovery Mechanism

Therefore, each node can create/update its topology map following different situations such as wrapping conditions. The topology discovery can be used by routing and management applications.

#### SRP Fairness Algorithm

The SRP-fa is a distributed algorithm that takes charge of ensuring:

- Global fairness: Each node gets a fair share of the ring bandwidth.

- Local optimization: Each node maximally leverages the spatial reuse properties of the ring

- Scalability: SRP-fa is able to handle efficiently large rings with many nodes. SRP-fa allows maximal 128 nodes and data rates up to STM-64c. This bandwidth is independent from the number of nodes and has to be shared between the nodes.

In addition, the algorithm is a preventive control of congestion. It is worth noting that the SRP-fa only applies to low priority traffic.

Simplifying, the algorithm works as follows:

- A set of usage counters monitor the rate at which low priority transmit data and forwarded data are sent (MY_USAGE and FWD_USAGE respectively).

- There is a congestion threshold in the low priority transit buffer (see Figure 10) used to detect congestion. If the congestion threshold is crossed (congestion detected), the node begins to advertise to upstream nodes the value of its transmit usage counter (MY_USAGE). In order to send this bandwidth information, Usage control packets are used. These usage messages are generated periodically even if there is no new bandwidth information to send (a null value is sent) since these packets also inform to the destination node that a valid data link exists.

- Nodes receiving usage messages adjust their transmit rates so as not to exceed the advertised value and propagate the usage messages received whenever the node is not congested. If the node receiving the usage information is also congested, propagate the minimum value of their transmit usage and the usage message received.

A more detailed explanation of the SRP-fa (included the SRP fa pseudo-code) can be found in the SRP Internet draft [DPT-3].

#### Intelligent Protection Switching

The SRP protocol utilizes a protocol known as Intelligent Protection Switching to provide the ability of the SRP ring (DPT ring) to recover from events and faults such as fiber cuts or node failures.

IPS is analogous to the self-healing properties of SONET/SDH (APS) but the wrapping of traffic onto the alternate fiber is done without the need to allocate protection bandwidth providing service restoration times 50msecs. This wrapping is transparent to layer 3 routing protocols. IPS monitors and handles events at layer 1, 2 and 3 instead of just layer 1 events.

Since the SRP packet is encapsulated in a SONET/SDH frame, the IPS makes use of both Usage control packets and SONET/SDH overhead usage to detect events and faults.

IPS makes use of the Intelligent Protection Switching control message in order to process and communicate fault detections and clearances. The IPS protocol maintain a protection switching event hierarchy that handles concurrent multiple events (e.g. signal fail and signal degrade events) without partitioning the ring into separate sub-rings. The following table (Table 3) lists the request types ordered from the highest to the lowest priority:

| Priority | Type of Request |
| --- | --- |
| 1 | Lockout of Protection (LO) |
| 2 | Forced Switch (FS) |
| 3 | Signal Fail (SF) |
| 4 | Signal Degrade (SD) |
| 5 | Manual Switch (MS) |
| 6 | Wait to Restore (WTR) |
| 7 | No Request (IDLE) |

Table 3: Protection Request Hierarchy

In order to maintain this protection request hierarchy, each node in the IPS protocol is in one of the following states:

- Idle: The node is ready to perform a protection switch

- Pass-through: The node enters this state when it receives a long path IPS packet. (This state is different from the Pass-through Mode explained before).

- Wrapped: The node enters this state when it receives a local request or detects a fault or receives a short path IPS packet from an adjacent node. In this state, the node performs a wrap.

Figure 12 shows the difference between short and long paths and the states of the nodes after a fiber cut occurs.

Figure 12: IPS Operation.

All the operation rules of the IPS protocol can be found in the SRP protocol Internet draft [DPT-3].

## Gigabit Ethernet

### Architecture

In comparison to previous Ethernet architectures, Gigabit Ethernet differs only in the physical layer and resembles them from the data link layer upward. In order to obtain a 1 Gbps speed, IEEE 802.3 Ethernet is enhanced by ANSI X3T11 Fiber Channel in the physical layer. What it does is adopting the high-speed physical interface of Fiber Channel, while maintaining the traditional Ethernet frame format.

Figure 13: Architecture of IEEE 802.3z Gigabit Ethernet

Figure 14

#### Physical Layer

Gigabit Ethernet supports 4 physical media types. These are defined in 802.3z (1000Base-X) and 802.3ab (1000Base-T).

##### 1000Base-X

The 1000Base-X standard is based on the Fiber Channel Physical Layer. Fiber Channel is an interconnection technology for connecting workstations, supercomputers, storage devices and peripherals. Fiber Channel has a 4-layer architecture. The lowest two layers FC-0 (Interface and media) and FC-1 (Encode/Decode) are used in Gigabit Ethernet. Since Fiber Channel is a proven technology, re-using it greatly reduced the Gigabit Ethernet standard development time. Three types of media are included in the 1000Base-X standard:

- 1000Base-SX 850 nm laser on multi mode fiber.

- 1000Base-LX 1300 nm laser on single mode and multi mode fiber.

- 1000Base-CX Short haul copper "twinax" STP (Shielded Twisted Pair) cable.

##### 1000Base-T

1000Base-T is a standard for Gigabit Ethernet over long haul copper UTP. It provides 1 Gbps Ethernet signal transmission over four pairs of category 5 UTP cable, covering cabling distances of up to 100 meters or networks with a diameter of 200 meters. This standard will outline communications used for horizontal copper runs on a floor within a building using structured generic cabling, taking advantage of the existing UTP cable already deployed. This effort requires new technology and new coding schemes in order to meet the potentially difficult and demanding parameters set by the previous Ethernet and Fast Ethernet standards.

#### MAC Layer

The MAC Layer of Gigabit Ethernet uses the same CSMA/CD protocol as Ethernet. The maximum length of a cable segment used to connect stations is limited by the CSMA/CD protocol. If two stations simultaneously detect an idle medium and start transmitting, a collision occurs. Ethernet has a minimum frame size of 64 bytes. The reason for having a minimum size frame is to prevent a station from completing the transmission of a frame before the first bit has reached the far end of the cable, where it may collide with another frame. Therefore, the minimum time to detect a collision is the time it takes for the signal to propagate from one end of the cable to the other. This minimum time is called the Slot Time. ( A more useful metric is Slot Size, the number of bytes that can be transmitted in one Slot Time. In Ethernet, the slot size is 64 bytes, the minimum frame length.)

The maximum cable length permitted in Ethernet is 2.5 km (with a maximum of four repeaters on any path). As the bit rate increases, the sender transmits the frame faster. As a result, if the same frames sizes and cable lengths are maintained, then a station may transmit a frame too fast and not detect a collision at the other end of the cable. So, one of two things has to be done:

- Keep the maximum cable length and increase the slot time (and therefore, minimum frame size) or

- Keep the slot time same and decrease the maximum cable length or

- both.

In Fast Ethernet, the maximum cable length is reduced to only 100 meters, leaving the minimum frame size and slot time intact. Gigabit Ethernet maintains the minimum and maximum frame sizes of Ethernet. Since, Gigabit Ethernet is 10 times faster than Fast Ethernet, to maintain the same slot size, maximum cable length would have to be reduced to about 10 meters, which is not very useful. Instead, Gigabit Ethernet uses a bigger slot size of 512 bytes. To maintain compatibility with Ethernet, the minimum frame size is not increased, but the "carrier event" is extended. If the frame is shorter than 512 bytes, then it is padded with extension symbols. These are special symbols, which cannot occur in the payload. This process is called Carrier Extension.

#### GMII (Gigabit Media Independent Interface)

The GMII is the interface between the MAC layer and the Physical layer. It allows any physical layer to be used with the MAC layer. It is an extension of the MII (Media Independent Interface) used in Fast Ethernet. It uses the same management interface as MII. It supports 10, 100 and 1000 Mbps data rates. It provides separate 8-bit wide receive and transmit data paths, so it can support both full-duplex as well as half-duplex operation.

The GMII provides 2 media status signals : one indicates presence of the carrier, and the other indicates absence of collision. The Reconciliation Sublayer (RS) maps these signals to Physical Signaling (PLS) primitives understood by the existing MAC sublayer. With the GMII, it is possible to connect various media types such as shielded and unshielded twisted pair, and single-mode and multi mode optical fiber, while using the same MAC controller. The GMII is divided into three sublayers: PCS, PMA and PMD.

##### PCS (Physical Coding Sublayer)

This is the GMII sublayer which provides a uniform interface to the Reconciliation layer for all physical media. It uses 8B/10B coding like Fiber Channel. In this type of coding, groups of 8 bits are represented by 10 bit "code groups". Some code groups represent 8 bit data symbols. Others are control symbols. The extension symbols used in Carrier Extension are an example of control symbols.

Carrier Sense and Collision Detect indications are generated by this sublayer. It also manages the auto-negotiation process by which the NIC (Network Interface) communicates with the network to determine the network speed (10,100 or 1000 Mbps) and mode of operation (half-duplex or full-duplex).

##### PMA (Physical Medium Attachment)

This sublayer provides a medium-independent means for the PCS to support various serial bit-oriented physical media. This layer serializes code groups for transmission and deserializes bits received from the medium into code groups.

##### PMD (Physical Medium Dependent)

This sublayer maps the physical medium to the PCS. This layer defines the physical layer signalling used for various media. The MDI ( Medium Dependent Interface), which is a part of PMD is the actual physical layer interface. This layer defines the actual physical attachment, such as connectors, for different media types.

### Standards

Gigabit Ethernet was standardised in June 1998.

To recap the recent history of the Gigabit Ethernet standards process, in July 1996, after months of initial feasibility studies, the IEEE 802.3 working group created the 802.3z Gigabit Ethernet task force. The key objectives of the 802.3z Gigabit Ethernet task force were to develop a Gigabit Ethernet standard that does the following:

- Allows half- and full-duplex operation at speeds of 1000 Mbps

- Uses the 802.3 Ethernet frame format

- Retains the CSMA/CD access method with support for one repeater per collision domain

- Addresses backward compatibility with 10BASE-T and 100BASE-T technologies

The task force identified three specific objectives for link distances: a multimode fiber-optic link with a maximum length of 550 meters; a single-mode fiber-optic link with a maximum length of 3 kilometres (later extended to 5 kilometres); and a copper based link with a maximum length of at least 25 meters. The IEEE is also actively investigating technology that would support link distances of at least 100 meters over Category 5 unshielded twisted pair (UTP) wiring.

### Gigabit Ethernet and Asynchronous Transfer Mode (ATM) technologies

When ATM (Asynchronous Transfer Mode) was introduced, it offered 155 Mbps bandwidth, which was 1.5 times faster than Fast Ethernet. ATM was ideal for new applications demanding a lot of bandwidth, especially multimedia. Demand for ATM continues to grow for LAN's as well as WAN's.

ATM was touted to be the seamless and scaleable networking solution - to be used in LANs, backbones and WAN’s alike. However, that did not happen. And Ethernet, which was for a long time restricted to LANs alone, evolved into a scalable technology.

As Gigabit Ethernet products enter the market, both sides are gearing up for the battle. Currently, most installed workstations and personal computers do not have the capacity to use these high bandwidth networks. So, the imminent battle is for the backbones,

the network connections between switches and servers in a large network. Both ATM and Gigabit Ethernet solve the issue of bandwidth. ATM provides a migration from 25 Mbps at the desktop, to 155 Mbps from the wiring closet to the core, to 622 Mbps within the core. ATM also promises 10 Gbps of bandwidth via OC-192, which was available and standard at the end of 1997. Ethernet currently provides 10 Mbps to the desktop, with 100 Mbps to the core.

ATM still has some advantages over Gigabit Ethernet:

- ATM is already there. So it has a head start over Gigabit Ethernet. Current products may not support gigabit speeds, but faster versions are in the pipeline.

- ATM is better suited than Ethernet for applications such as video, because ATM has QOS (Quality of Service) and different services available such as CBR (constant bit rate) which are better for such applications. Though the IETF (Internet Engineering Task Force, the standards body for internet protocols) is working on RSVP which aims to provide QOS on Ethernet, RSVP has it's limitations. It is a "best effort" protocol, that is, the network may acknowledge a QOS request but not deliver it. In ATM it is possible to guarantee QOS parameters such as maximum delay in delivery. Ethernet promises to provide Class of Service (CoS) by mapping priority within the network to mechanisms such as Resource Reservation Protocol (RSVP) for IP as well as other mechanisms for Internetwork Packet Exchange (IPX).

Gigabit Ethernet has its own strengths:

- The greatest strength is that it is Ethernet. Upgrading to Gigabit Ethernet is expected to be painless. All applications that work on Ethernet will work on Gigabit Ethernet. This is not the case with ATM. Running current applications on ATM requires some amount of translation between the application and the ATM layer, which means more overhead.

- Currently, the fastest ATM products available run at 622 Mbps. At 1000 Mbps, Gigabit Ethernet is almost twice as fast.

It is not clear whether any one technology will succeed over the other. It appears that sooner or later, ATM and Ethernet will complement each other and not compete.

### Topology

Gigabit Ethernet supports full duplex operating modes for point-to-point connections, e.g. switch-to-switch and switch-to-server. Further, Gigabit Ethernet offers half duplex modes for shared connections using repeaters and the CSMA/CD scheme.

|  | Ethernet | Fast Ethernet | Gigabit Ethernet |
| --- | --- | --- | --- |
| Data rate | 10 Mbps | 100 Mbps | 1000 Mbps |
| Cat 5 UTP | 100 m | 100 m | 25 m |
| STP/Coax | 500 m | 100 m | 25 m |
| Multimode Fiber | 2 km | 2 km | 500 m |
| Single-mode Fiber | 25 km | 20 km | 2 km |

Table 4. Ethernet topology rules for maximum network distance

Gigabit Ethernet is essentially a "campus technology", that is, for use as a backbone in a campus-wide network. It will be used between routers, switches and hubs. It can also be used to connect servers, server farms (a number of server machines bundled together), and powerful workstations.

Essentially, four types of hardware are needed to upgrade an exiting Ethernet/Fast Ethernet network to Gigabit Ethernet:

- Gigabit Ethernet Network Interface Cards (NICs)

- Aggregating switches that connect a number of Fast Ethernet segments to Gigabit Ethernet

- Gigabit Ethernet switches

- Gigabit Ethernet repeaters (or Buffered Distributors)

Two likely upgrade scenarios are given below:

#### Upgrade a shared FDDI Backbone

Fiber Distributed Data Interface (FDDI) is a common campus or building backbone technology. An FDDI backbone can be upgraded by replacing FDDI concentrators or Ethernet-to-FDDI routers by a Gigabit Ethernet switch or repeater.

Figure 15

#### Upgrade a Fast Ethernet Backbone

A Fast Ethernet backbone switch aggregates multiple 10/100 Mbps switches. It can be upgraded to a Gigabit Ethernet switch which supports multiple 100/1000 Mbps switches as well as routers and hubs which have Gigabit Ethernet interfaces. Once the backbone has been upgraded, high performance servers can be connected directly to the backbone. This will substantially increase throughput for applications which require high bandwidth.

Figure 16

### 10Gigabit-Ethernet

The huge of demand for high-speed networking has stirred development of the next generation 10-Gigabit Ethernet. The 10-Gigabit Ethernet standard will support the data rate of 10 Gbps, which is 10 times faster than the current transmission rate of 1-Gigabit Ethernet, but the cost is targeted around 2 to 3 times the cost of the current 1-Gigabit Ethernet technology. It will not support the half-duplex operation, but it will maintain the compatibility with the preceding Ethernet technologies by using the same Ethernet frame format, enabling seamless integration among the existing networks and the new technology. 10-Gigabit Ethernet may be used in LAN, MAN, and WAN, implying technology convergence and faster switching.

In addition to the data rate of 10 Gb/s,10-Gigabit Ethernet shall be able to accommodate slower date rates such as 9.584640 Gbps (OC-192). This is likely to be done by using the word-by-word pacing mechanism via the 10GMII interface. The current proposal for 10GMII uses 32-bit data paths with 4 control bits (one control bit per one data byte) and a TX_hold line. This structure provides scalability and can support various variations of the physical layer implementation. The discussions regarding issues in the physical layer are still ongoing. The standard may provide different specifications for different applications. The main considerations include the implementation architectures (serial, parallel, or WDM), coding techniques, supporting technologies, and media.

The 10 Gbps serial transmission solution appears to be the easiest and lowest-cost option. The recent technology demonstration by Lucent validates the 10 Gbps serial transmission using 850-nm VCSEL sources on the enhanced multi-mode fibers over the link longer than 300 meters. However, it cannot support the existing multi-mode fiber infrastructure. On the other hand, the WWDM solution supports the existing multi-mode fiber infrastructure but the implementation is more complicated and the devices are still more expensive. The 4 x 3.125 GBaud WDM solution using 1300-nm uncooled DFB sources on the existing 62.5-um multi-mode fiber can reach the link distance up to 300 meters. In addition to the WDM solution, the parallel fiber option may be suitable for short-haul applications such as computer rooms. In addition to fibers, copper wires may be used for a very short link (10-20 meters) but may not be considered in the early version of the standard.

The coding issue still remains opened. Several coding techniques are considered. The trend is that the 8B/10B encoding technique may be specified for the LAN rate Ethernet because it is compatible with 1-Gigabit Ethernet, while the SONET-like scrambled encoding techniques may be specified for the WAN rate Ethernet because it is compatible with SONET. These coding techniques are proven and well understood, but the problem with the 8B/10B is the large 20% overhead. Other coding techniques such as 15B/16B, MB810, and PAM-5 are considered as well.

In short, 10-Gigabit Ethernet will be a low-cost solution for high-speed and reliable data networking and it will dominate the LAN, MAN, and WAN markets in the near future.

### Conclusion

Gigabit Ethernet is a viable technology that allows Ethernet to scale from 10/100 Mbps at the desktop to 100 Mbps up the riser to 1000 Mbps in the data center. By leveraging the current Ethernet standard as well as the installed base of Ethernet and Fast Ethernet switches and routers, network managers do not need to retrain and relearn a new technology in order to provide support for Gigabit Ethernet.

## IP over SDH and Packet over SONET

### New Backbone Network Technologies: IP over SDH/SONET

Existing transport networks for Internet backbones are circuit-based, based on the time division multiplexing TDM hierarchy. The Synchronous Optical NETwork/Synchronous Digital Hierarchy (SONET/SDH) is a TDM technology, which is widely used in the large-scale, high-speed, IP-based networks. Packet over SONET (PoS) technology allows the efficient transport of data over SONET/SDH and is consider a major player in accommodating the explosive growth of the Internet.

#### Reasons to turn to IP directly over SDH/SONET

The advent of ATM in the mid of 80s, was considered as the remedy for the aggregation of voice and data traffic. However, the PoS approach offers a new backbone architecture that preserves existing investments and supports at the same time the deployment of IP-based video and voice applications.

Internet Service Providers (ISPs) are increasingly turning to IP directly over SDH/SONET. The main reasons given by the ISPs are [PoS-1]:

- The use of ATM to transport IP adds up to 25% of overhead (cell tax). The overhead includes:

- The ATM cell header (5 bytes), the AAL5 trailer (8 bytes), the LLC/SNAP encapsulation overhead (8 bytes), and the PAD (last segment alignment to 48 bytes).

This is especially significant for short packets because the statistics say that nearly 50% of IP packets are 40-44 bytes long, which require 2 ATM cells to be transmitted, but the second cell goes almost empty. As shown in Figure 17 [PoS-2], the 50% (approximately) of the packets are very short packets. On the other hand, the PoS approach adds only 4% overhead, which is significantly less than the ATM cell tax.

Figure 17: Packet size distribution measured in a domestic link.

- SDH has better scalability than ATM in terms of transmission rate.

While IP/ATM is limited to 622 Mbps, IP/SDH can scale up to 10 Gbps (see Table 5). The IP/ATM limitation is due to the following fact: ATM SAR function becomes increasingly complex as the interface speed increases. Currently, only interfaces up to OC-12 speed can use ATM SAR chips.

#### SONET Frame Format

SONET is a TDM technology optimised for voice. PoS allows the use of SONET to transport IP data in an efficient way. The SONET frame is depicted as a table with 9 rows and 90 columns. It is divided into two parts:

- The transport overhead. The transport overhead is divided further into the Section overhead and the Line overhead.

- The payload. The payload is divided into the path overhead and the payload capacity.

Figure 18: SONET frame format.

#### SONET Data Rates

The data rates of SONET are multiple of the 51.840 Mbps. That is the minimum rate unit and is referred to as Synchronous Transport Signal level 1 (STS1). Table 5 includes the some transmission rates and their SDH- equivalent rates.

| SONET | Optical Signal | SDH | Data Rates (Mbps) |
| --- | --- | --- | --- |
| STS-1 | OC-1 |  | 51,840 |
| STS-3 | OC-3 | STM-1 | 155,520 |
| STS-12 | OC-12 | STM-4 | 622,080 |
| STS-48 | OC-48 | STM-16 | 2488,320 |
| STS-192 | OC-192 | STM-64 | 9953,280 |
| STS-n | OC-n |  | N x 51,840 |

Table 5. SONET/SDH Data Rates.

#### IP over SDH/SONET

##### Packet Over SONET Protocol Stack

The protocol stack for operating IP over SONET is depicted in the Figure 19.

Figure 19: Packet over SDH/SONET Protocol stack.

The IP datagrams are encapsulated into PPP packets, which, in turn are framed using HDLC, and mapped into the SONET synchronous payload envelope (SPE). The PPP protocol provides multi-protocol encapsulation, error control and link initialisation control features. The PPP adds 1 or 2 octets “protocol field” and optional padding. The PPP protocol is described in IETF RFC 1661. The HDLC delineate the PPP packets.  The HDLC adds a flag byte indicating frame start, 2 more overhead bytes, and a 2 byte frame check sequence (FCS), resulting in a frame up to 1500 octets long. Together with PPP, HDLC results in a 7 or 8 octet overhead being added to the IP packet. The PPP in HDLC-like encapsulation is described in IETF RFC-1662 [PoS-3] (see Figure 19).

Finally the HDLC frames are mapped byte aligned into a VC4 or concatenated VC4 payload, according to RFC 2515. The SONET layer adds the SONET section overhead (81 bytes including AU pointers), and a 9 VC4 byte Path overhead, to the 2340 byte SONET VC4 payload.

##### Encapsulation

IP do not provides bit synchronisation or packet delineation (demarcation).

- In IP/LANs, MAC layer provides bit synchronisation and frame delineation.

- In IP/ATM, SDH provides byte synchronisation, ATM provides the cell synchronisation and the AAL5 frame delineation (HEC byte + AAU bit).

- In IP directly over SDH/SONET, SDH/SONET also provides bit synchronisation, but something is needed to provide IP packets delineation.

Thus, IP over SDH/SONET requires an additional encapsulation for IP packet delineation. IP over SDH/SONET uses PPP HDLC-like encapsulation. Figure 20 depicts the PPP HDLC-like frame format, which consists of the PPP part and the HDLC part.

Figure 20: PPP HDLC-like frame format.

The PPP part consists of three fields:

- The protocol field, which identifies the protocol encapsulated in the Information field
(for IP this field takes the value 0x0021).

- The information field, which contains the encapsulated IP packet. The maximum length of this field can be set when the link is first configured. The default value is 1500 bytes.

- Pad: The information field may be padded up to the maximum length. The encapsulated protocol has to disambiguate padding octets from real information. No padding is used in IP over SDH/SONET.

The HDLC part consists of four fields:

- The Flag used for delineation.

- The Address field set to the HDLC All-Station Address.

- The Control field indicating that a HDLC Unnumbered Information (UI) frame is used.

- The Frame Check Sequence field of 32-bit, which is mandatory for all SDH/SONET rates except for 155 Mbps. For 155 Mbps, it can be chosen between the 32-bit FCS and the 16-bit FCS.

One might wonder whether there is a need to use the PPP HDLC-like encapsulation. The answer is that PPP is needed because it is a standard method for transporting multi-protocol datagrams multiplexed over a point-to-point link.

As above mentioned, PPP provides for IP packet encapsulation, control of errors (discarding corrupted frames) and link initialisation. HDLC provides for error reporting and delineation of the PPP encapsulated IP packets (delineation is accomplished using “byte stuffing” in the flag).

The PPP Lick Control Protocol (LCP) provides the link initialisation. Figure 21 depicts the flow chard for this protocol.

Figure 21: PPP Link Control Protocol (LCP)

In the following a short description for the PPP Lick Control Protocol is included.

LCP is a three-way handshake protocol for configuring, maintaining and terminating the PPP link. Figure 21 show the different states of this protocol, which role are the following:

- Dead state: The PPP link always begins and ends in the dead state.

- Link establishment state: Is reached when an event such as a carrier detection or network administrator intervention indicates that a physical layer is present and ready to be used.

In the link establishment state, one end of the link sends its desired link configuration options using an LCP configure-request frame (a PPP frame with the protocol field set to LCP and the PPP information field containing the specific configuration request). The other side then responds with a configure-ack frame (all options acceptable), a configure-nak frame (all options understood but not acceptable) or a configure-reject frame (options not recognizable or not acceptable for negotiation).

LCP configuration options include a maximum frame size for the link, the specification of an authentication protocol (if any) to be used, and an option to skip the use of the address and control fields in the PPP frames.

- Network layer configuration state: Exchange network-layer-specific network control packets with each other. It uses the IP Control Protocol (IPCP) [RFC 1332]. IPCP packets are carried within a PPP frame (with a protocol field value of  0x8021). IPCP allows the two IP modules to exchange or configure their IP addresses and negotiate whether or not IP packets will be set in compressed form.

- Open state: Once the network layer has been configured, PPP may then begin sending network-layer datagrams. In this state, the LCP echo-request packet and echo-reply packet can be exchanged between the two PPP endpoints in order to check the status of the link.

- Terminating state: is reached when a LCP terminate-request packet is sent.  If a terminate-request LCP packet is sent by one end of the PPP link and replied to with a terminate-ack LCP packet, the link then enters the dead state.

IP datagram can also be directly encapsulated over SDH, without PPP interface, using HDLC frame switches. This configuration provides multiple access, broadcast/multicast-capable switched LAN and WAN environment using essentially point-to-point SDH lines transmission media.

##### RFC 2615 specifications

PPP over SONET/SDH was specified by the IETF RFC 2615 [PoS-4]. Next, the main specifications included in RFC 2615 are summarised:

- The octet stream is mapped into the SDH Virtual Container (VC) with the octet boundaries aligned with the VC boundaries.

- “Scrambling” is performed during insertion into the VC (this function may be switched off if desired).

The X43 + 1 Scrambler description is the following:

- The label C2 of the POH is used to indicate the contents of the VC.

		The value of 22 (0x16) is used to indicate PPP with scrambling.

		The value 207 (0xcf) is used to indicate PPP without Scrambling.

- The label H4 of the POH, which is used to indicate multi-frame is unused (it is set to 0).

- The PPP frames are located by row within the VC.

- PPP frames are allowed to cross VC boundaries (because they are variable in length).

The proper order operation during data transfer is the following:

- When transmitting:

- IP -> PPP -> FCS generation -> Byte stuffing -> Scrambling -> 
-> SONET/SDH framing and transmission

SONET/SDH frame transmission is preceded of the SONET/SDH standard scrambling process.

- When receiving:

- SONET/SDH receiving and framing -> Descrambling -> Byte destuffing -> 
-> FCS detection -> PPP -> IP

SONET/SDH frame reception is followed of the SONET/SDH standard descrambling process.

Figure 22 depicts the process described above.

Figure 22: IP over SONET using PPP in HDLC-like encapsulation.

##### IP over SDH/SONET Advantages

The transport of IP data over SDH/SONET enhances the functionality of the IP networks. The IP directly over SDH/SONET provides robust transmission, path-level fault and performance management and protection switching. The SDH/SONET layer provides alarms and error messages using the overhead bytes of the frame. It enables the monitoring of the connections, the detection and isolation of faults. Such functionality is desirable for any network operator and the IP layer alone cannot provide such.

Moreover, SONET/SDH networks provides protection against failures by automated protection switching. Self-healing ring restore any connection within 50 ms. from the time that a link went down. Taking into consideration that the average time for the routing algorithms in the IP layer (IS-IS, OSFP) to converge is some seconds, compared to some miliseconds in the case of the SDH/SONET networks, justify the appeal of the PoS approach to the ISP and network operators.

##### IP/SDH Drawbacks

SDH/SONET was designed taking into account that voice has going to be transmitted through the network. Thus, SDH/SONET technology is not optimised for the transport of IP-based data.  Moreover, there are some scalability problems beyond 2.4 Gbps data rates: at the transmitter side, very outgoing byte needs to be monitored and stuffing performed, and the receiver needs to monitor every incoming byte to do the destuffing.

In order to scale well beyond OC-48, simpler protocols are needed. Such approach is the Simplified Data Link (SDL) protocol designed by Lucent Technologies. SDL frame consists of a payload length indicator, a cyclic redundancy check (CRC) over the header only, and a separate CRC over the payload.

Two means for initial acquisition of SDL frames are under discussion, namely the header CRC based acquisition as is done in ATM and the H4 pointer of the SDH Path Overhead.

Another important drawback of IP over SDH/SONET is the incremental capacity growth: SDH does not provide much flexibility in terms of capacity expansion. This is one of the motivations for the migration of the IP backbone networks to transport based on optical Wavelength Division Multiplexing (WDM) technology.

#### Applications of PoS

IP over SDH/SONET can be used in all the parts of the network, giving the flexibility to network providers to cater for different needs and demands. PoS interfaces can be used in the backbone, edge or the metro network. These interfaces perform the PPP/HDLC framing and then the optical signal is suitable for transmission. There are 2 types of interfaces:

- VC4 or Concatenated VC4 that provide aggregate bandwidth without any partitions between different IP services which may exist within the packet stream.

- Channelised interfaces, where an STM16 optical output may contain 16 individual VC4s, with a possible service separation for each VC4.

In the backbone, the PoS interface is used to connect different WANs. The different WANs can be connected either with a SDH/SONET network or with the use of WDM systems. In the former case the PoS interface is directly connected to an ADM whereas in the latter case the PoS interface is directly connected to the WDM transponder. It is also possible to connect the different WAN directly using regenerators, referred to as dark fiber connectivity.

In the edge of the network, the traffic from different regions is aggregated and the bit rates are usually less than 155,520 Mbps (OC-3).  In this case the use of channelised interfaces are a preferable solution by most network operators.

Finally, in the metro area, SDH rings are used to provide inter-office connectivity. The PoS approach is again dominant since other alternatives like Gigabit Ethernet, although can operate at the same speed, cannot provide the rich functionality (management, performance, protection) of the SONET/SDH technology.

#### Migration to WDM

##### Motivation to migrate to WDM

The traditional technique to cope with the lack of bandwidth is either to deploy more fiber, which is, of course, an expensive solution, or to increase the TDM system rate (e.g. Move from OC-3 to OC-12), which does not provide much flexibility and has also some scalability problems.

The combination of an unprecedented demand for new transmission capacity and high utilization of the existing infrastructure (cable, equipment), together with the fact that existing traditional technique has reach their limits, makes the introduction of WDM indispensable since it provides a flexible, cost-effective capacity expansion mechanism.

##### Benefits

Some benefits from the introduction of WDM are:

- WDM allows the flexible incremental capacity growth that IP over SDH/SONET can not provide: The SDH/SONET granularity is 51 Mbps, 155 Mbps, 466 Mbps, 622 Mbps, etc., while the SDH/WDM granularity can be 51 Mbps, 2x51 Mbps, 3x 51 Mbps, 4x 51 Mbps, etc.

- WDM maximises the reuse and minimises the life-cycle cost of existing fiber facilities. Example: WDM can be used to provide four OC-12  (622 Mbps) IP/ATM /SDH interface on a single fiber pair. To achieve the same without WDM we would require four fiber pairs.

- WDM provides a transport network solution for high capacity TDM signals.  For the emerging Gigabit Routers with high-speed interfaces (e.g. OC-48), the existing transport infrastructure may become a bottleneck. Using WDM is a cost-effective solution.

- WDM takes advantage of the commercial capacity of the fiber. Example: Consider a Single-mode fiber (low loss region Bw = 30 THz). Existing commercial WDM systems offer 16-32 wavelengths. Assuming OC-192 (10 Gbps) for each wavelengths, we can have 160 Gbps and 320 Gbps. Furthermore, a fiber serving as the transport medium for an OC-12 (622 Mbps) circuit is only using a 0.4% and a 0.2% of the fiber´s commercial capacity for 16 and 32 wavelengths respectively. If we take an OC-48 (2.4 Gbps) circuit we are only using a 2% and a 1% respectively.

- WDM allows the coexistence of multiple interface types on the same fiber. Example: IP/ATM/SDH, IP/PPP/HDLC/SDH, or some future yet-to-be-determined link protocols.

# IP over OTN as a new transport network

## The role of a Transport Network

The advent of packet-switched data-centric networking does not eliminate the necessity of a transport network. In the rush to get to service convergence, it is easy to neglect the fundamental value of transport networking. This is a mistake, because transport networking continues to be essential for real-world networks:

- to provide/manage high bit-rate interconnection pipes between Giga/Tera-Routers in the core backbone; transport networking, increasingly based on the optical layer technology, continues to be essential, because broadband data networks will need more transport functionality than just point-to-point "big dumb pipe" interconnects between routers and switches,

- to establish a unifying infrastructure for multiple-service layers providing large-granularity bandwidth management,

- to support the world-wide extension of the Internet, where the network usage patterns are completely changing. The old rule was that network traffic was 80 % local, and 20 % wide area. Today, the ratio is closer to 50/50, and the growing number of applications that require all communication from clients to traverse the backbone to reach central servers means more stress on network backbones. It is expected that the 80/20 rule will eventually return, but with wide-area applications accounting for 80 % of traffic,

- to achieve, on the whole, a lower-cost network.

A transport network ensures several important functionalities, among which:

- capability of traffic by-pass at layer 1:

- this implies the availability of more high-capacity circuits for improved connectivity and less “hops” for end-to-end paths, the reduction of congestion and latency time. On the contrary, without a transport network, the mesh between routers (through point-to-point links) may be relatively poor, so that a large fraction of the traffic traversing each router is “through” traffic,

- scalability for the service layer:

- without an efficient transport network, it may be necessary to expand the entire network to face sudden spikes in demand for a sub-set of client nodes, even the intermediate service-layer nodes that provide only transport-like bandwidth management. The problem is that the service layer logical topology is tied to the network physical link topology. Transport networking provides a solution, by freeing the service-layer logical topology from the physical link topology, and by implementing networking and bandwidth management in a most efficient way:

- upgrade only the routers where spikes occur,

- provide a new high-capacity end-to-end circuit

- keep the rest of the network unchanged

- fast survivability and support of service-layer restoration with shared-protection architectures:

- IP can achieve survivability by its re-routing algorithms, but when each fibre carries a huge traffic, the convergence time of IP re-routing may be very slow. Network survivability/reliability improves if large aggregates of traffic are recovered by fast layer 1 protection/restoration (at least in case of fibre breaks).

Today’s transport network is SDH/SONET (already mentioned as “layer 1 reference technology”) which exploits WDM as a transmission support. SDH is an excellent solution under many aspects:

- Layered network functional modelling.

- Managed Network (TMN).

- Rich “overhead” for Operation & Maintenance purposes.

- Advanced protection schemes supported (e.g., Optical Multiplex Section Shared Protection Ring - OMSPRing).

However, it is optimised for “voice / PSTN primary clients”, not as a “data-centric” network.

The data-centric network operates in a fundamentally different way from the voice-centric network. Aggregation of physical circuits tends to happen at the edge of the carrier network. The speed of the transmission facilities between switches directly affects the performance of these networks. This drives the evolution towards routers and switches characterised by higher and higher capacity interfaces. The newest IP routers and ATM switches for the core are scaling to Tb/s throughputs, with 2.5 and 10 Gb/s ports to match. The rapid emergence of high bit-rate ports on routers and switches is probably the single most disruptive force in today’s transport networks. The data-service layers are suddenly scaling faster than SDH/SONET.

When OC-192c/STM-64c interfaces are used, the current transport network architectures would call for OC-768/STM-256 ADMs; alternatively, for DXCs with OC-192c/STM-64c interfaces and an accompanying switch fabric. These upgrades will not only be expensive, but they also will not be readily available in the near future.

Another possible answer by SDH is the “overlay rings” architecture. However, from an operational standpoint, the overlay model is complex because each overlay ring must be managed individually. This introduces the issue of selecting the correct ring to provision services, which becomes a challenging exercise in large networks. Provisioning across rings adds even more complexity because ring selection must be performed across multiple rings. To provision capacity in today’s network, there typically are a number of steps and different organisations involved.

On the other hand, the availability of OC-48c interfaces on a router/switch (and the subtended aggregation function) may eliminate the need for an entire layer of TDM multiplexing. There is little benefit to connecting a switch OC-48c output to an OC-48 SONET multiplexer, because the port speeds have already reached the interface speed of the optical layer.

The obvious consequent solution is a “direct” connection of routers/switches to the already deployed and/or emerging WDM systems (described in chapter 2.4), taking advantage also of the fair matching between the mentioned IP Gigarouters interfaces and the typical bit-rate modulation of wavelength channels in WDM. The number of OC-48 channels made possible by DWDM seems to be ideally suited to broadband IP routers. In this case, however, traffic engineering (for provisioning and protection) and more generally all the function of a transport network should be performed by the WDM layer on the wavelength granularity.

To summarize, the future Core Transport Network will be required to offer a substantially increased overall capacity and a flexible provision of long-reach high bit-rate pipes:

- to meet the huge overall traffic growth,

- to answer the new traffic distribution paradigm (80% long reach),

- to serve the future main client: IP Giga/Tera Router with several high-capacity ports,

- to allow switching at a higher level of granularity.

The Optical Transport Network is actually the only candidate solution to fulfil these requirements.

## The “IP over OTN” scenario

- IP Core Routers will continue to do what they can do well:

- routing, forwarding, packet switching and aggregation are performed by IP Routers,

- “point to point” layer 2 (data link) functions are added to IP Routers,

- high bit-rate interfaces SDH- or WDM- compliant are included in the IP Routers.

- WDM supports IP by offering it the functions of a more advanced transport network:

- cross-connection on the new wavelength granularity,

- transport network (layer 1) functions: protection/restoration, monitoring, circuit provisioning, management and supervision – again, on the wavelength granularity.

The high bit rate data pipe – internally organised as a flow of aggregated and forwarded IP packets, with a given priority – is mapped into one wavelength, which is the new “managed entity” in the new OTN transport network. Actually, IP nodes extends their functions to layer 2 (at least point-to-point data link functions) whereas WDM extends its functions from layer 0 (transmission support) to layer 1 (transport network), thus becoming OTN.

The former “transmission support” and “transport network” are substituted by the new “OTN”, realising the evolution from WDM to OTN.

With respect to the three main issues raised in chapter 2.1, with respect to the emerging data-centric backbone network, the situation appear as follows:

(3) how to optimise the transport service for the IP core backbone?

- OTN is the only solution.

(2) how to reduce congestion/latency delays in core IP Routers?

- It is possible to develop solutions for the traffic by-pass at layers lower than layer 3, driven by the layer 3 control plane (e.g., MPLS) – see section 2.3.2.3.

- OTN can be a solution, in that it allows traffic by-pass at level 1, on wavelength granularity.

(1) how to simplify the downstairs path from IP to WDM?

- several different “simplified mappings” have emerged – see section 2.3.2,

- OTN can be a solution, in that it may substitute SDH as transport layer.

This means that the evolution is from a conflicting to a converging telecom and datacom view:

- telecom view: IP is just one more client (the risk of this view is that of under-evaluating the emerging data-centric network requirements),

- datacom view: the IP router is the multi-service aggregation node, fulcrum of the new data-centric network (the risk of this view is that of under-evaluating the necessity and the role of the underlying transport network),

- converging view: the best network architecture for a cost-effective, reliable, scalable evolution employs both transport networking and enhanced service layers, working together in a complementary and interoperable way.

A summary of properties and advantages of the IP over OTN solution are summarised here below:

- Optimisation of the data transport plane, so that:

- It is ready to serve the rapidly increasing long reach data traffic.

- It allows the implementation of the “unlimited bandwidth” scenario imagined in forecasts.

- It respects the evolving IP network (no major change required in the IP world).

- It is a platform for the wide area extension of the network, able to accept different physical interfaces, e.g.,

    - SDH/Sonet OC-48/STM-16, OC-192/STM-64

    - SDH/Sonet compliant (DPT, EoS)

    - ATM 622 Mb/s

    - GbE – 10 GbE

- It is consistent with the development of Gigabit Routers as the main backbone nodes

For example: the Cisco 12016 GSR Terabit System offers modular scalability to 5 Tb/s by integrating up to sixteen 12016 GSR nodes using the GSR Scalability Module and the GSR Terabit Fabric interconnect. The system will support 256 slots and the entire range of existing and future GSR line cards from DS-3 to OC-192c/STM-64c. In particular, Cisco claims it has delivered thousands of OC-48/STM-16 interfaces for the Cisco 12000 series gigabit switch router (GSR) to large IP network operators throughout the world. The vast majority of these interfaces connect directly to DWDM systems, making this a widely accepted approach to building high-capacity IP backbones.

Another example is the “Juniper M20 Backbone Router”, whose architecture delivers a 4xOC-48/STM-16 router that runs in the most traffic-intensive parts of the Internet.

- Improvement of the logical connectivity for the Gigarouters:

- enriched meshed connectivity at layer 1,

- traffic by-pass at layer 1,

- possibility of avoiding a layer 2 network,

- Improvement of the scalability for the service layers.

- Simplification of the network layering:

- the necessity of an entire ATM (more generally, a layer 2) network is avoided,

- the necessity of an entire SDH network with the related equipment is avoided,

- the evolution is from TDM Voice Infrastructure (hierarchical model, cross-connection through 64-kbps crosspoints, capacities limited by TDM, restoration via full duplication) to a Data-Optimised Infrastructure (flattened network model, statistical multiplexing, dynamic usage characteristics, intelligent protection switching, WDM capacities),

- keeping the already standardized SDH interfaces, when opportune – evolving from OC-48 (channelized) optimized for TDM Switching to OC-48c (concatenated) enabling Optical Internetworking,

- the new mapping is: IP / layer 2 point-to-point functions / layer 1 interfaces / OTN.

Thus potentially achieving lower cost, complexity, overhead, and improved scalability.

- Simplification of the network structure, i.e., substantial reduction of the number of separate physical devices.

- Capability of switching at the right granularity.

Historically, the increase of higher level switching granularity has been in proportion to the increase of the overall link capacity (between one and two orders of magnitude below). It is clear that a switching granularity of the order of magnitude of 2.5 Gb/s is becoming necessary in the new scenario. This is perfectly consistent with the wavelength-granularity switching – which is on the other hand the only practical solution to achieve this objective

- Potential to become a unifying transport layer.

Once deployed for the new IP main client, the OTN may substitute the SDH network and accept the existing “SDH voice-centric network” just as a client. The evolution will be smooth:

- IP will continue to be carried by ATM up to e.g., OC-3 capacity.

- ATM and voice will continue to be carried by SDH up to e.g., OC-12.

- SDH and IP will be direct client of OTN at OC-48 and above.

Rather than disappearing, SDH/SONET will transition through this evolution. SDH/SONET will continue to be competitive for lower-capacity transport, especially toward the network edge. And it will enjoy wide acceptance as a stable, interoperable format for multi-gigabit data-service ports.

## Open issues

Some of the still open issues in the roadmap towards the full development of an OTN are here recalled, as an introduction to the actual enabling technologies and to the further evolutions, outlined in the next sections.

First of all, the “analogue” nature of optical transmission must be highlighted: this means that optical signals in WDM networks are susceptible to a number of impairments which may cause bit errors or loss of service, including noise addition, dispersion, fibre non-linearities, jitter accumulation and wander, mis-connections, optical power transients and variations.

This effects have an impact on two substantial aspects:

- Possible necessity of 3R Regeneration.

- Possible necessity of advanced performance monitoring.

### 3R-Regeneration vs Transparency

Transparency: the channel path, from end to end, is purely optical, without the necessity of passing through the electronic domain.

Main advantages:

- it would allow to build a universal platform, ready for any type of client,

- the complete independence from client format and bit-rate

Main problems:

- transmission issues, i.e., the necessity of 3R-regeneration after a certain path length. This necessity can be counteracted in different ways – but it is ultimately unavoidable

- ‘geographical’ scalability not ensured

- pure optical overhead handling and performance monitoring: not yet capable to offer the desired performances (which are on the contrary offered by  “electronic” overhead)

3R-Regeneration: the signal is regenerated in certain points along its path. Today, an established technology for all optical regeneration does not exist. After dispersion and signal-to-noise limits have been reached through a series of cascaded amplifiers, the WDM signal must be demultiplexed and each channel be electrically Retimed, Reshaped, and Retransmitted (3R).

Main advantages:

- transmission issues are solved,

- effectiveness with respect to the overhead transport and handling.

Main problem

- client dedicated regenerators are necessary – this may have a very detrimental impact on the network scalability and on the capability of being dynamically reconfigured and optimised.

Possible trade-off solutions are:

- The definition of “all-optical islands” where the quality of optical channel transmission is anyway guaranteed; at the boundaries of these optical islands, 3R Regeneration and electronic overhead monitoring are performed. This approach is consistent with the concepts of “domains”, leading to the definition of IntraDomain Interfaces (possibly “transparent”) and InterDomain interfaces (certainly not transparent). This solution does not solve the limitations imposed by the necessity of client dedicated regenerators.

- The development of “dynamic multi-bit-rate” regenerators (with programmable clock), associated with the definition of an overhead related to the optical channel (although in the “electronic” domain) and not client-specific (concept of Digital Wrapper – see section 4.4.2.2). An alternative is to define a common bit-rate for all the wavelengths transported in the OTN and fill this maximum capacity with several (possibly different) clients, through a further (but simplified) TDM layer in between. A method to provide the transition between lower speed transparent networks and  high speed fixed bit rate or opaque networks is more and more needed.

### Performance Monitoring and OA&M

An effective transport network must offer performance monitoring capabilities, such as:

- optical transmission performance monitoring,

- “digital” error monitoring,

- fault detection and allowance for protection switching protocols,

- optionally, error correction, for example Forward Error Correction (FEC),

- sufficient timing information to allow robust synchronisation.

More generally, the OTN should offer to its clients the same functions offered by SDH:

- allowance for management information transfer (DCC),

- overhead information handling,

- connection supervision (e.g., path trace, to check that the correct “wavelength” is received),

- OA&M features.

Possible complementary answers for these issues are:

- the Digital Wrapper (associated with each Optical Channel),

- the Optical Supervisory Channel (for the Optical Multiplex and Optical Transmission Sections).

The idea is that anyway a “TDM” electronic overhead is necessary, at least for a sub-set of supervision function. This would allow to implement advanced OA&M features, that may be  defined on the basis of the SDH experience. This aspect is a strong point towards a consistent overall solution based on Opaque Cross-Connect, and Digital Wrapper for each channel.

### Protection vs Restoration

This aspect is particularly important, and rather complex, since it involves the properties required to transport nodes, the control/management systems, and even the topological structure of the whole network.

A thorough discussion on this topic is out of the scope of this contribution. Here, the main aspects are just recalled, to introduce the following steps of the OTN evolutionary paths.

The two solutions to be compared are the automatic protection possible in ring topologies and the end-to-end restoration, suitable to meshed networks.

- Ring Protection:

- well known SDH-like solution,

- fast actuation allowed (~ 50 msec.),

- non optimal bandwidth efficiency,

- associated with a network architecture that may introduce multiple bottlenecks

- .

- Mesh Restoration:

- improved bandwidth efficiency,

- associated with a “fair” network architecture,

- the problem is to perform it within the required strict timescale,

- different options are related to the strategy of reserving capacity used by low-priority traffic, or to avoid a pre-reservation, leaving to intelligent algorithms the task of finding capacity for restoration when required,

- the necessity of adding “intelligence”  to the optical layer emerges here as a very important requirement (see chapter 4).

The assumption is that anyway a survivability mechanism must be provided by the transport network, at least to counteract huge losses of traffic, due for example to a fibre break. This does not prevent the possibility that client layer survivability mechanisms are kept, and in some way harmonised with the transport network ones.

Moreover, it is desirable to offer differentiated kinds of survivability to different classes of traffic. Current SONET/SDH implementations are effective but expensive; all services are treated uniformly, i.e., protected using the same mechanism; this means that, besides being expensive, the solution proposed by SDH is useless, for a significant fraction of the traffic. Many services may tolerate slightly restoration times longer than 50 ms, while cannot tolerate the performance provided by DXC-based restoration mechanisms, which generally have been several orders of magnitude too slow.

The OTN can be a solution, by offering optical survivability mechanisms, and a differentiation on the basis of the wavelength granularity. The key to offer effective restoration in a wide meshed OTN is to provide intelligence to the optical layer, define proper algorithms, and find an effective way to exchange relevant information between remote nodes. This is a decisive driving force for the “intelligent OTN” (see chapter 4).

## Enabling technologies

In order to achieve the goal of developing a new transport network, it is necessary that WDM completes its transition towards a fully functional OTN. This is not an unrealistic objectives, but some steps are still to be done.

In this section, a wide meaning will be assigned to the term  “enabling technologies”, which will be classified in two groups:

- Technologies for the physical realization of the new generation of OTN transport nodes

- Technologies for overcoming the still open issues (outlined in the previous section 4.3).

### Technologies for OTN transport nodes

Optical network nodes can use space, time and wavelength to transport and switch optical channels. Networks can be categorised depending on the lightpath establishing techniques they use:

- Circuit-switched: the circuit route is fixed and is established before transmission.

- Virtual circuit-switched: the route is fix but statistically allocated

- Datagram-switched: the route is selected per packet during transmission, being statistically allocated

Optical nodes participating in the establishment/release of light-paths can be distinguished depending on the switching technique. Thus, they are called cross-connects when the connection pattern is semi-permanent and light-path set up is done by the network operator. They are referred to as switches (intelligent nodes) when light-paths are established using signalling protocols. Routers are nodes that perform connectionless switching of datagrams.

The core of a switching optical node is the Switch Matrix. It can be optical or electrical. An electronic Switch Matrix uses optical to electronic conversion at the input and electrical to optical conversion at the output. This nodes are known as Opto-Electric Cross-Connects (OEXC) – or Opaque Optical Cross-Connects. The Switch Matrix can also be optical and then, many different options can be used.

The main components of an Optical Node are here briefly recalled, and the consolidated and emerging technologies are mentioned, with a particular focus on the switch matrix.

| Functional Block | Consolidated Technology | Advanced Technology |
| --- | --- | --- |
| WDM Mux/Demux (as for WDM line systems) | micro-optics diffraction gratings cascade of interferential filters cascade of Fibre Bragg Gratings (FBG) | Arrayed WaveGuides (AWG) built as Planar Lightwave Circuits (PLC) |
| Optical Amplifier | Erbium-Doped Fibre Amplifiers - EDFA (optimized for multi-channel WDM, also through external control gain systems) Semiconductor Optical Amplifiers - SOA (on single channels, can be used as “on/off gates”) | Different dopings for extended bandwidth e.g Erbium-Doped Fibre Fluoride Amplifiers - EDFFA the objective is to have two usable bandwidths, C, L, over a whole range between 1530 and 1620 nm SOA for multi-wavelengths channels |
| Wavelength Converter | Based on O/E/O conversion | All-Optical Wavelength Converter (AOWC) built by SOA-based interferential structures (this is not a short term solution) |
| Splitter / Combiner | Fibre-based splitter/combiner | Planar Lightwave Circuit (PLC) in SiO2/Si |
| Tunable Filter (e.g., for split & select nodes) | Mechanic. tunable Fibre Fabry-Perot (FFP) Mechanic. tunable interferometric filter (angle-tilting) | thermo-optically tunable PLC in SiO2/Si acousto-optically tunable in LiNbO3 |
| Add/Drop Filter | Fibre Bragg Gratings (FBG) based | thermo-optically tunable in SiO2/Si |
| Var. Attenuator | thermo-optical PLC in SiO2/Si |  |
| 2x2 switch (e.g. for protection switching) | micro-optics electro-mechanical (MEMS, e.g., based on a movable cantilever) piezo-electric electro-mechanical passive PLC in LiNbO3 | thermo-optical PLC in SiO2/Si active PLC in InP micro-mirror based |
| 1xN selector (N < 8) | micro-optics electro-mechanical (MEMS, e.g., based on a movable cantilever) | polymeric waveguide based |
| Switch Matrix | Bulk electro-mechanical (not larger than 64x64) Micro-Optics Electro-Mechanical | Micro-Mirror based matrix (see section 6.1.2.4) Intersecting Waveguides + InkJet bubbles (see section ) Active matrix based on SOA gates |

Table 6:  Optical technologies for Optical Nodes

| Switch Matrix | Cross-point matrix Open issues are: max. frequency that can be switched (suitable to 2.5 Gb/s ch. not to 10 Gb/s ch.) electronic interconnection between matrix units (short reach optics maybe required) | Cross-point matrices able to handle higher frequencies (consistent with 10 Gb/s channels) with optimized backplanes for electronic interconnection (at least up to 2.5 Gb/s channels) |
| --- | --- | --- |

Table 7:  Electronic technologies for Optical Nodes

### Technologies for advanced network functionalities

Several strategies are actually applied to overcome the obstacles described in section 4.3, including addition of functions to the optical layer and interworking mechanisms with client layers. From the viewpoint of the technologies which are necessary, two items emerge:

- the Optical Supervisory Channel technology,

- the Digital Wrapper technology.

#### The Optical Supervisory Channel technology

The Optical Supervisory Channel (OSC) is an idea derived from the WDM line systems (e.g., see the products mentioned in section 6.1.1). The OSC in line systems is a means for remote monitoring of line stations; it can also be used to carry service communication channels. Typically, the OSC is a low bit-rate channel (e.g., 2 Mb/s) modulated onto a wavelength out of the EDFA amplification bandwidth, to avoid the under-exploitation of an in-band wavelength (better used as a payload). The OSC can be in the 1310 nm band, or at 1510/1520 nm, or around 1625 nm. Obviously, it is extracted before and re-inserted after EDFAs, which are not transparent to the OSC wavelengths.

When evolving from point-to-point WDM to OTN, the OSC may play a more important role:

- it keeps the function of remote monitoring of line sites, properly enhanced to exchange also information related to the remote management of sites where no local NE Agent is present

- it supports optical OA&M functions (maintenance messages, protection protocols, etc.)

- more generally, it transports the overhead information related to the OTS and OMS optical network layers, and that part of the OCh layer overhead which is not strictly associated with the channels itself (e.g., OCh OA&M messages) – the part of OCh overhead strictly associated with each optical channel needs another type of support (e.g., the Digital Wrapper)

- in the evolutionary scenario towards the Intelligent OTN (see chapter 4), the OSC is a possible means to carry optical routing protocol information.

#### The Digital Wrapper technology

A requirement of the WDM-OTN is that client layer provides a continuous bit-stream. Therefore packet based client networks cannot be mapped onto the server layer directly. The trend for OTN server/client network interworking is to define a common frame structure in the optical channel layer to carry clients regardless of their type. With state-of-the-art technology, WDM-OTN networks are unable to provide neither all-optical 3R (re-amplification, reshaping, and re-timing) regeneration nor QoS guarantee using optical protocols to client signals. Therefore, a highly desirable feature of a TDM optical container is to provide somehow means for signal quality monitoring analogously to SONET/SDH.

The Digital Wrapper (DW) is a Lucent's proposal. It comes after results of different studies that have shown the convenience of carrying the associated optical channel overhead in a TDM format. DW structure is based on the  ITU-T Rec. G.975 and is aimed at carrying the OCh-OH as well as an FEC (Forward Error Control) and possibly ring protection on a per-wavelength basis. DW is a means to effectively manage wavelengths or Optical Channels (OCh), irrespectively from the type of client. Until now, the only feasible way to support signal regeneration and to monitor, analyze and manage optical channels (wavelengths) was to rely on SONET/SDH signals and equipment throughout the network. However, as already noted several times, it will be required to the new OTN to be a “universal” platform, able to support the multiplicity of "IP over WDM" signal mappings (see chapter 2.1), i.e., to reliably carry a wide variety of client signals-including SONET/SDH, IP, ATM, GbE, and SDL-directly over the optical layer of the network..

The Digital Wrapper technology will provide functionality and reliability similar to SONET/SDH, but potentially at a lower cost and without adding more equipment to the network.

# IP over “intelligent” OTN

The importance of the OTN as a new transport network, mainly to serve IP, has been widely discussed in the previous chapter, and by itself, can justify the development of new optical nodes.

However, a further important step can be done: the addition of “intelligence” to the optical layer:

- in section 2.1 the importance of linking the control planes of IP and of the underlying server layer has been pointed out; this is up to now realized by MPLS, when the underlying layer is ATM

- in section 3, the possible usefulness of “intelligent” protocols has emerged from the opportunity of implementing fast and efficient mesh restoration schemes on large (up to nationwide) networks

- in almost any forecast on the telecommunication evolution, the key words are no longer simply “optical networks” but “intelligent optical networks”

A flash on a market forecast:

Purchases of DWDM and optical cross-connects will grow to about $7.6 billion in 2003, compared to $3.5 billion today, according to the new report from Communications Industry Researchers, Inc. (CIR).  This growth is expected to occur as DWDM evolves out of being just a way of saving money on new fibre and, together with optical cross-connects, becomes an enabling technology for an entirely new type of network — the intelligent optical internetwork. The new CIR report notes that "fibre exhaust" alone cannot drive the DWDM market forward. But the most important innovations will come from adding intelligence that will enable the interworking of DWDM transmission systems and optical cross-connects to create an all optical internetwork over which ATM, IP and other traffic will travel. In this new optical internetwork it will be possible to provision high bandwidth services in minutes, not months, turning the new optical technology into a revenue spinner for the service providers and not just a way of saving money.

It can be useful to reach some agreements on what we really mean by “intelligence”. Some items for discussion are here proposed.

In the scenario “IP over OTN”, presented in chapter 3, no major interaction between control planes is foreseen. However, both the server layer (whichever it is) both the client layer (whichever it is) have their own management system and their own OA&M features (OA&M intended in a wide sense).

In particular, for the OTN, the extension of TMN principles and of the layered functional model (based on three optical layers, OCh, OMS, OTS) has already been demonstrated. The application of OA&M functionalities has been implemented too in the main research OTN field trials.

The focus now is on the interworking of management and OA&M between different networks – LION is just an example of this trend.

- Interworking at Network Management level

- Necessary for and suitable to all that management and supervision functions which need to be centralized and can be performed on slow timescales

- Different network elements – in the same or in different network layers – can be equipped with different NE Agents, presenting a different interface to the remote Network Management

- Network Element Managers on one hand are interfaced to the NE Agents, on the other hand present a common view of network resources to the Network Manager

- Network Managers may be built with different technologies (e.g., CORBA, WBEM) and must interoperate

- Interworking at OA&M level

- Useful to co-ordinate mechanisms which need very fast actuation (e.g., protection switching) that are present in both layers – these are often partially overlapping and not efficient when completely independent

- a sub-set of OA&M messages related to each layer can usefully flow through inter-layer interfaces (e.g., CNI) – also to identify different kinds of faults and avoid unnecessary fault indication propagation

These items are essential, but still do not realize an “intelligent optical layer” and do not imply an interworking between control planes.

## Optical “intelligence”

The following steps for the introduction of intelligence in the optical layer can be envisaged.

- Each optical transport node should know the topology and the status of all the network, or of a part of it (e.g., a domain), the partition of the available and occupied capacity, possibly taking into account the priorities of the information supported by its resources. This allows the actuation of fast restoration procedures, or a node reconfiguration for optimization of traffic engineering, on the basis of local decisions by the optical node itself.

- The obvious way to proceed is to define routing/signalling protocols for the optical layer, by exploiting concepts already developed in the IP world (e.g., MPLS, OSPF, LDP – each one with its specific functionality)

- Finally, a link between existing layer 3 protocols and emerging layer 1 protocols should be established, in order  to optimize the behaviour of the overall network.

Two different approaches can be adopted towards this evolution, based on a IP-driven view and a classical telecom view, according to which answer is given to the question: “Should the optical layer be treated like a circuit-switched network, with data devices requesting 2.5- or 10-Gb/s lightpaths just like 64-kb/s ISDN B channels? Or should the optical layer have an IP-centric control plane”? The dominance of data makes today the data-centric approach stronger.

In the standardization arena, the IP-driven approach is pushed by IETF, OIF, and is recently supported by a new informal study group called ODSI (Open Domain Service Interconnect).

### The IP driven roadmap towards an Intelligent OTN

This approach, supported by the IETF standardization forum, is based on the following principles:

- The end-to-end circuit provision is treated as a route provision according to IP concepts/protocols

- Semi-permanent big-pipes are offered (as in traditional transport network), but they are reconfigured according to “Internet rules”: each route in this case is biunivocally related to a set of wavelength (one for each OTN link)

This proposal can be also summarized as the evolution from MPLS (Multi-Protocol Label Switching) to MPS (Multi-Protocol Lambda Switching):

| MPLS | MPS |
| --- | --- |
| “label based” routing - traffic by-pass at layer 2 | “label based” routing - traffic by-pass at layer 2 |
| Actual solution: IP/MPLS over ATM | Proposed solution: IP/MPS over OTN |
| the routing “decided” by MPLS is performed by switching ATM virtual circuits in ATM switches | the routing “decided” by MPS is performed by switching WDM wavelengths in OXC |

Table 8:  Multi-Protocol Label Switching and Multi-Protocol Lambda Switching

QoS by using label switching

Wavelength paths can be used to support MPLS real circuits where each MPLS label points to a wavelength channel. With this approach, WDM dynamic cross-connects are under control of the IP layer as setting a MPLS circuit means WDM cross-connects reconfiguration.

One method that has been suggested is “Optical Label Switching”, which uses an optical header or Label that contains routing information. A problem with this implementation is that optical processing in the time domain is needed, which can be quite difficult to implement. On the other hand, it is very similar to conventional Label switching which means that protocol issues may be quite easily solved.

Another possible method is to use the wavelength itself as a label. One issue that has to be further investigated is how many wavelengths are needed for implementing wavelength labels. It is clear that if wavelength reuse is not implemented in the nodes, it is not possible to map the number of labels possible in e.g. MPLS. However, with reasonable number of neighbouring nodes, and wavelength reuse, it is probable that 40 wavelengths per fibre are more than adequate. One solution is to use OEXC (Opto-Electric Cross-Connects), since they will provide wavelength conversion in every node.

Labelling can be used for providing different service levels; different routes that give different QoS. They could also be used for implementing VPNs, which can be considered as privileged QoS.

For the purpose of implementing MPS, several kinds of protocols are proposed for extension to the optical layer, e.g.:

- Constraint-based routing + RSVP or CRLDP signalling mechanisms applied to the optical layer

- Optical Link Discovery Path (OLDP)

- Optical LSA (Link State Advertisement) for extension of OSPF, IS/IS

Some more information is reported in the following section 5.2.

### The classical telecom roadmap towards an Intelligent OTN

This approach is emerging in ITU-T SG15 and SG19, in the draft recommendation G.ason (ASON stands for “Automatic Switched Optical Networks”) – whose introduction and content are here summarised.

The basic proposal is the use of signalling protocols to set up OCh connections in real time.

There is clearly a benefit from automatic switching of optical channels, in particular, there is a clear opportunity to create a global switched optical network. There are a number of value added capabilities proposed for such a capability:

- Reactive traffic engineering. This is a prime attribute and allows the network resources to be dynamically allocated to routes

- Restoration and recovery.  These attributes can maintain graduated preservation of service in the presence of network degradation.

- Linking optical network resources to data traffic patterns automatically will result in a highly responsive and cost effective transport network.

In order to achieve automatic OCh level switching, which we consider as automation of the OTN, a certain degree of global standardisation is required.

The proposed ASON consists of multiple layers and partitions, such as OCh layer, fibre layer, conduit layer (a bundle of fibres), domain/sub-network layer, network layer, which consists of domains/sub-networks.

A preliminary identification of the main interfaces, in a ASON, is reported in the G.ason:

- the User-Network Interface (UNI) between the optical network elements and users such as the router, ATM switch,

- the ASON Node-Node Interface (NNI) between network elements within the optical network,

- the Connection Control Interface (CCI) between the control plane and network elements,

- the ASON Inter_Domain Node-Node Interface (IrDI_NNI) between ASON control planes in different administration domains,

- the ASON NMI between the ASON network and a network management.

The extension of signalling protocols for the Wavelength Path set-up/release will be a major goal of the future activity related to G.ason.

## Enabling technologies

Routing and signalling protocols form the control plane: in order to interwork an IP based over an WDM-OTN network, the optical control plane must be defined.

Routing & Wavelengths Assignment (RWA) paradigms: Current WDM-OTN assign wavelengths to routes given a set of requested access-egress nodes. This facilitates finding a "best" solution in a quasi-static scenario.

IP over WDM-OTN demands dynamic route computation schemes to solve practical cases where channel requests may arrive randomly at arbitrary frequencies. Therefore it will be necessary to improve the performance interaction between IP and WDM-OTN protocol layers. Different aspects of this interworking will need to be standardised. Currently there are many candidates for routing protocols in the WDM-OTN control plane. Here a brief summary is given to some of them. Trying to be an overview, not all possibilities are addressed.

- Wavelength Routing Protocol (WaRP): this routing protocol is Cisco proprietary. It views the network as a bandwidth pool and provides end-to-end (from access to egress nodes) wavelength paths (light-paths). Depending on the required class of service, the Wavelength Router (OXC) can provide bandwidths with varying guarantees of availability, latency, and restoration times. Any wavelength added to the network becomes available for service and can be provisioned as part of an arbitrary light-path.

- Multi-protocol Lambda Switching (MPS): it uses an MPLS (Multi-Protocol Label Switching) signalling entity to control OXC treating them as IP capable. MPS extends an analogy between MPLS labels and WDM wavelength channels, allowing end-to-end light-paths between optical nodes to be treated as high bandwidth, large, discrete granularity lambda label-switched path entities.

- General Switch Management Protocol (GSMP): it allows communication between an IP switch controller and an ATM or FR switch (i.e. is responsible for installing and deleting connections in a switch). An access or egress router would signal each OXC between access and egress nodes along a light-path. It would signal using GSMP to ask the OXC's to set up its cross connect management table.

- Next Hop Resolution Protocol (NHRP): it is used to enable cut-through paths to be established between logical IP subnets on an ATM or FR network. It may be used to request a shortcut. Therefore, it may be applied as signalling mechanism for optical flow switching (flow can be defined as the minimum manageable traffic).

- Resource Reservation Protocol (RSVP): signalling protocol, used to reserve network resources to provide Quality of Service guarantees to the flows that have to be routed from access to egress nodes. It can be used analogously to NHRP.

- Label Distribution Protocol (LDP): signalling protocol, it is used to distribute label binding in label switching protocols.

- Constraint-based LDP (CR-LDP): signalling protocol

# Product overview

## IP-over-WDM

### IP over WDM as transmission technology

Some of the most advanced line system products, by major manufacturers, are summarised in [NR-P&T], as significant examples. Reported there data had been considered just as a qualitative information – drawn from different public sources – and obviously not exhaustive. The exact commercial situation (what is already deployed, what is ready to be sold, what is at prototype level) is not specified in detail, because analytical comparisons between different products is out of the scope of that document. Anyway, that rough overview should be sufficient to have an idea of the state of the art and of the ongoing short-term evolution.

The following advanced line system products had been analysed as examples in [NR-P&T]-p.6.1.1: Lucent: WaveStar OLS 400G, Cisco Photonics: TeraMux HyperDWDM, Nortel: OpTera LH, Ciena: MultiWave Sentry 4000 and MultiWave CoreStream, Siemens: TransXpress Infinity, Alcatel: OPTINEX 1640 WM.

### IP over OTN as a new transport network

A non-exhaustive, brief overview of the emerging products by major vendors, in the area of optical transport nodes, is provided in [NR-P&T] - Appendix 1 p. 6.1.2.

#### Transport Nodes for the Core Backbone Network

In traditional networks, the physical layer is unchanging unless a manual reconfiguring or fiber reconnection is performed. This method is disadvantageous, expensive and error-prone. Moreover, it cannot face the faster and faster network evolution, often hardly predictable. The answer to all of this is to provide the flexibility of switching high-speed optical channels in the core network. This functionality is provided by new kinds of transport nodes i.e. optical cross-connects (OXC) and optical add-drop multiplexers (OADM).

These optical transport nodes provide several benefits. They offer a protected, restorable optical layer transport network of wide flexibility. They allow to achieve lower network cost and higher fiber efficiency. A typical traffic pattern at a node in the core of a network is that between 50 and 75 percent of the traffic is “through” traffic. Such traffic can be routed straight through a cross-connect without a need for any additional processing or switching. Instead of devoting high-cost ports on DXC or IP Switches to such traffic, much lower cost optical cross-connect switches can be used to express route the trough traffic. Cost savings of 30 to 40 percent are projected. Operations costs are also reduced because of the automated and remote control functions available.

Optical transport nodes also enable ring network interworking and the building of networks of complex ring and mesh architectures. Networks can be configured in the same cross-connect on a wavelength-by-wavelength basis: several wavelengths, or channels, can be configured as self-healing rings, while other channels are connected as meshes. Protection and restoration protocols are selectable on a wavelength-by-wavelength basis.

There are different ways to pursue these objectives:

- One means is to use large port-count digital cross-connect switches and many time division multiplexers. This is an expensive approach. It also is difficult to perform restoration and protection switching in reasonable time limits in large networks because the switching action is done on sub-rate channels, sequentially, rather than at the high-speed channel rate. Configuration or provisioning of high-speed interconnection paths requires the coordinated switching of all of the sub-channels and re-assembly.

- On the other hand, all-optical switching fabrics are proposed as a means of cross-connecting signals regardless of bit rate or data format, therefore providing a “future proof” design. These would be designed as optically transparent switches. However, the actual state of the art of all-optical switches is that they are expensive, and they are of insufficient port count to be competitive with electronic fabrics.

- The alternative is to deploy “electronic” cross-connects at the optical layer, able to switch at the line rate of the input channels, providing restoration and protection switching on a full OC-48 channel in one operation and avoiding the need for large banks of TDM multiplexers. The size of the required cross-connect is also commensurately reduced, both in port count and in physical dimension. Management of the optical layer is done at the optical layer and directly on each of the optical channels.

These optical cross-connects, so called because they operate at the optical line rate and on whole optical channels at a time, require a significant amount of information about the optical channels in order to perform fault management, protection switching and restoration, and to verify connectivity through a switch. This can be done by accessing the various overhead bytes of the SONET carrier, or by introducing a new kind of digital overhead, associated to the Optical Channel (Digital Wrapper). Both solutions lead to the use of electronic switching fabrics as the most cost-effective way of implementing the cross-connect switch function.

Therefore, among the products for the core backbone the following types of nodes are considered:

- “Multi-service” nodes, directly evolving from SDH DXC nodes e.g., Lucent WaveStar Bandwidth Manager, Tellabs Titan 6500, Alcatel 1680 OGX

- Opaque OXC: wavelength cross-connection, using electrical core switching matrix e.g., Ciena Lightworks CoreDirector, Cisco ONS15900, Lucent Aurora512, Nortel OpteraConnect, Sycamore SN16000

- Transparent OXC:

- Automatic large size optical matrices (e.g., Lucent LambdaRouter, Nortel/X-Ros)

- Manually reconfigurable OXC plus automated protection, remote monitoring (e.g., Siemens OSN)

#### Multi-Service nodes for the Core Backbone Network

A starting point for the discussion can be a look on the classical broadband DXC, optimized for SDH (excellent examples of this category are the Siemens SXD and the Nortel S/DMS nodes).

Typical feature of these nodes are:

- large-size synchronous switching matrices – optimized for compactness (for example, 2048 – 4096 STM-1 equivalent, in a single rack),

- capability of working on high level Virtual Containers (typically VC-4, at a STM-1 line rate of 155 Mb/s),

- their role is at the upper level of an SDH-compliant core switching network, whereas a set of different Network Elements (often belonging to the same vendor’s product family) are optimized for different functions (regenerators, terminals, add-drop multiplexers, wideband cross-connects, etc.),

- capability of supporting OC-48/STM-16 interfaces towards the high-capacity transmission systems,

- often already equipped with (or at least pre-disposed for) OC-192/STM-64 interfaces,

- often already equipped with (or at least pre-disposed for) coloured DWDM-compliant interfaces,

- operations on concatenated interfaces/channels possible,

- support to different protection schemes developed for SDH (linear MSP, SNCP, ring MSP, BSHR, UPSR) – to ensure survivability,

- optimized OA&M and Network Management features, through the capability of handling DCC for all the throughput channels.

However, despite their good features, these “pure” SDH nodes cannot keep the pace with the evolving requirements for transport network.

The first category of “new nodes” include the “multi-service” nodes, directly evolving from SDH DXC nodes. The main additional features in these nodes may typically be:

- the capability of switching aggregates at large granularities (comparable to the wavelength granularities) although in a synchronous electronic domain,

- the capability of switching lower granularities, with respect than the classical VC-4 of broadband DXC,

- the possible integration of layer 2 switching and layer 3 routing sub-systems,

- the addition of local intelligence for protection/restoration, beyond the network management features.

This is the roadmap preferred by manufacturers already present in the SDH and in the transport networking market. Different evolutionary paths are pursued. The following three examples of different and original trends, (Lucent WaveStar Bandwidth Manager, Tellabs Titan 6500 MTS, Alcatel 1680 OGX) are reported in [NR-P&T]-p. 6.1.2.

Optical Gateways: Where optical layer backbone and access networks interconnect, optical gateways and photonic cross-connects will emerge. The optical gateway is a necessary element of true optical networking, providing a platform with integrated Dense Wave Division Multiplexing (DWDM) optics while internally managing broadband services in the electrical domain. Gateways become the bridge between flexible access networks and highly efficient backbone networks.  Optical gateways will displace the broadband cross-connect to manage the transparent to opaque conversion between the access and long haul networks, and to manage the broad range of payloads in wavelength services. They will simultaneously support cell based routing and aggregation of ATM or IP payloads and legacy STM services. In this way, lower rate wavelengths from the access will be aggregated to high speed ITU compliant channels for the backbone network. And as DWDM systems grow in channel count (currently projected up to 240 channels), optical gateways will emerge as the central wavelength service management device.

#### Opaque Optical Cross-Connects for the Core Backbone Network

The second category include the “Opaque” OXC, i.e., cross-connect nodes able to perform “wavelength management” and switching on wavelength granularity, able to handle optical channel overhead (pre-disposed for Digital Wrapper techniques), which have however an internal electronic core. It must be highlighted that, although electronic, the switching matrix is an asynchronous space switch, and has the only limitation of the maximum bit-rate that can be handled electronically (today, up to 2.5 Gb/s).

This category is today the most important one, since it allows the realization of large-size matrices, and is consistent with the necessity of 3R regeneration, digital performance monitoring, channel-associated digital overhead.

The principles behind this choice are:

- Networks are Becoming Data-Centric,

- High- speed Data Signals do not need STS- 1 grooming,

- WDM has made bandwidth plentiful,

- managing OC- N is still difficult & expensive.

- Mesh Protocols will dominate the Optical Network Core.

- OC- 48 is the “brick” for building Optical Networks.

- SONET monitoring is essential for intelligent layer 1.

- Opaque will continue to win over Transparent.

- Network Management is key to survivable Optical Layer.

Many vendors think that the architecture of preference for an optical cross-connect is that of an electronic switching fabric, with input and output interface boards that provide optical detection and regeneration, and overhead processing. All of these functions are performed at the line rate of the optical channel, most commonly at OC-48 (SDH-16) rates. Extensions to OC-192 interfaces and switching will be developed soon. The following product lines are presented as example in [NR-P&T]-p.6.1.2: Ciena: MultiWave CoreDirector, Cisco: ONS15900, Lucent: Aurora 512, Nortel: OpTera Connect, Sycamore: SN16000.

#### Transparent Optical Cross-Connects for the Core Backbone Network

The third category include the “Transparent” OXC, i.e., cross-connect nodes able to perform “wavelength management” and switching on wavelength granularity, through optical core matrices.

Two different levels of transparency can be identified:

- Complete transparency – avoid 3R regenerators at all, save costs, allow end-to-end transparency for each type of client – but problems related to transmission performances and overhead remain

- Core switching transparency – 3R Reg. and electronic overhead processing are still present, but the internal optical core allow to properly switch 10 Gb/s channels, differently from the electronic core.

In any case, the optical switching matrix of sufficient dimensions is required.

No consolidated products are available in this category. The focus is still on the availability of a core optical matrix characterised by proper dimensions.

Two different approaches are considered in the following examples:

- Focus on technology, to achieve the objective of a fully functional large size Optical Cross-Connect (a “true” OXC) – this is pursued by Lucent (LambdaRouter), Nortel (former X-Ros technology), Agilent; what is offered is not yet a complete product but a solution for the optical matrix, that is the main bottleneck for the realisation of the whole system..

- Focus on a smooth evolution from what is possible right now – this is pursued e.g., by Siemens– “optical” features are progressively added to existing nodes, accepting intermediate steps in the short term (e.g., manual cross-connection or wavelength Add/Drop - rather than full cross-connect - capability).

[NR-P&T]-p.6.1.2 briefly describes following technology for transparent optical cross-connects solutions: Lucent: LambdaRouter, Nortel/X-Ros: X-1000, Agilent Photonic Switching technology, Siemens: OSN.

#### Nodes for the Metropolitan Transport Network

The argument for WDM as simply a network infrastructure tool is not as compelling for metropolitan or other short haul networks. It must have more benefit to the operator to gain acceptance into the network. However, the appearance of wavelengths in both long haul and access networks opens new revenue generating service opportunities.

While it is sometimes assumed that the widespread deployment of DWDM in core networks will inevitably spread into short-haul optical transport networks, very different issues come into play if DWDM is to benefit from widespread application beyond fibre exhaust situations. The cost savings that result in core networks from replacing electrical regenerators with optical amplifiers are largely irrelevant in short-haul optical transport networks, especially metropolitan areas, due to the limited distances involved.

The need to add/drop traffic from the metro network at many locations places a premium on cost-effective optical add/drop rather than maximum channel density.

Another driver for metro DWDM deployment may be the provisioning of dedicated wavelengths for high-speed access. For customers who require very high access bandwidths, above OC-12/STM-4 for example, providing a dedicated wavelength over a DWDM ring may become an attractive alternative. Today these requirements are often addressed by point-to-point fibre between customer and central office, by two node SONET/SDH rings, or by allocating a large portion of a SONET/SDH ring. DWDM access rings may provide a cost-effective answer to this challenge as the requirement for reliable gigabit access grows.

Standalone metro DWDM systems are already available for ring and point-to-point applications. However, these systems are offered at a high cost-per-wavelength and require additional platforms to deliver services at DWDM nodes. Integration of DWDM into optical transport equipment provides a cost-effective way for service providers to scale beyond the speed limitations of serial optics and meet increasing bandwidth requirements.

Metropolitan, interoffice and access networks consist of a diverse range of architectures, bit rates, traffic patterns and protocols, in contrast to the long distance environment that is evolving from an established, standardized synchronous optical network (SONET) base. The economics of WDM compared to alternative solutions are, therefore, much more complex.

Cost is clearly a major issue. Unlike the long distance environment, there has been no obvious "killer application"--an economically compelling reason to deploy WDM in the majority of cases, over all other solutions.

In the short term, therefore, the decision to deploy WDM in the local environment appears to be driven by simple network congestion and cost considerations, where WDM is cheaper and/or more readily available than the alternatives for increasing capacity, then it will be deployed. This is occurring in those networks that are "fibre poor", i.e. where existing fibre capacity is running out and there is a lot of traffic on interoffice facilities. On the contrary,  in "fibre rich" networks, where there are spare conduits for laying new fibre and/or network congestion is not so acute, WDM may not be deployed widely in the near term.

However, despite the relative lack of success thus far for metro WDM, simply looking at the technology in terms of point-to-point capacity increase is misleading. Optical networking, deploying WDM ring architectures as opposed to simple point-to-point WDM, potentially will provide a common method for transport regardless of signal format in the metro environment, giving Local Exchange Carriers (LEC) the ability to deliver new services more quickly while reducing the costs associated with delivery.

For example, service providers will be able to assign the end customer a set of wavelengths and route those wavelengths through the network independent of whatever service is being carried. The marketing of transport on a wavelength basis will enable operators to offer end users a way to enter the high-speed backbone in native formats, avoiding the costs of aggregating multiple service types.

Because of the reasons expressed in the previous rationale, the commercial availability of optical nodes for metropolitan systems is one step behind with respect to the situation valid for the core network. Nonetheless, some examples can be mentioned, by looking at medium capacity / medium throughput products, optimized for ring topologies (typical of metro networks).

Two lines of evolution can be identified:

- Smooth evolution from DWDM line systems, which gradually offer linear and then ring wavelength add/drop capabilities (some of these products may be found in section 6.1.1, where add/drop capabilities offered by commercial DWDM line systems have been highlighted, e.g., in Lucent OLS 400G, Nortel OpTera LH, Ciena MultiWave CoreStream, Alcatel 1640 WM)

- Development of “optical nodes” – in parallel to the ones proposed for the core transport network – whose features are optimized to the metro transport environment (smaller size but also lower cost, ring instead of mesh topology, wavelength add/drop multiplexing rather than full wavelength cross-connection/routing). The optical nodes are in this case Optical Add Drop Multiplexers (OADM), Four examples of these systems will be briefly considered in [NR-P&T]-p.6.1.2 (Marconi: Smart PhotoniX PMA8, Ciena: MultiWave Metro, Nortel: OpTera Metro, Siemens: WaveLine).

### IP over “intelligent” OTN

An overview of the products in the area of Optical Transport Nodes has already been done in previous section, where the availability of intelligent protocols has been highlighted among the node properties. Existing intelligent nodes mainly belong to the category of Opaque OXC.

The emerging “Intelligent Optical Nodes” are just recalled in the following list:

- Ciena MultiWave CoreDirector with Optical Signaling and Routing Protocol (OSRP),

- Cisco ONS 15900 Wavelength Router with Wavelength Routing Protocol (WaRP),

- Lucent 512 Aurora with StarNet (Signaling Algorithm for Restoring Networks),

- Sycamore SN16000 with automatic topology discovery and optical routing algorithms (e.g., OSPF), LDP for circuit set-up.

## A survey of optical networking products for the transport network

The results deriving from survey of optical networking products for transport network are presented in [NR-P&T]-p.6.2. This section lists manufacturers in alphabetical order and gives short descriptions of the main products pertaining to the optical networks area of each company.

Below we are listing main manufacturers of products and solutions for the long haul transport network. [NR-P&T]-p.6.2.3. contains short descriptions of the main products or families of products offered by each vendor.

Main manufacturers:

ALCATEL (http://www.alcatel.com, http://www.alcatel.europe.fr )

AVICI (http://www.avici.com )

Charlotte’s Web Networks (http://www.cwnt.com )

CIENA (http://www.ciena.com )

CISCO (http://www.cisco.com )

CORVIS (http://www.corvis.com )

ECI Telecom Ltd. (http://www.ecitele.com )

ERICSSON (http://www.ericsson.com )

FOUNDRY Networks (http://www.foundrynet.com )

FUJITSU Network Communications (http://www.fnc.fujitsu.com )

HITACHI Telecom (http://www.hitel.com )

JUNIPER Networks (http://www.juniper.net )

LUCENT (http://www.lucent-optical.com )

MARCONI Communications (http://www.marconi.com )

NEC (http://www.necpng.com )

NOKIA (http://www.nokia.com )

NORTEL Networks (http://www.nortelnetworks.com )

PIRELLI TELECOM SYSTEMS Division (http://www.pirelli.com )(now acquired by CISCO)

PLURIS Inc. (http://www.pluris.com )

SIEMENS (http://www.siemens.com )

SYCAMORE Networks (http://www.sycamorenet.com )

TELLABS (http://www.tellabs.com )

TELLIUM (http://www.tellium.com )

Other manufacturers:

ALIDIAN Networks (ex TERABIT Net.) (http://www.alidian.com )

ARGON (purch. by Unisphere Solutions) (http://www.argon.com )

ASTRAL POINT Communications (http://www.astralpoint.com )

ATMOSPHERE NETWORKS (http://www.atmospherenet.com )

MAYAN Networks (http://www.mayannetworks.com )

OSICOM TECHNOLOGIES (http://www.osicom.com )

OPTICAL NETWORKS Inc. (http://www.opticalnetworks.com )

QEYTON Systems (http://www.qeyton.com )

SILK ROAD Corp. (http://www.silkroadcorp.com )

Zaffire (http://www.new-access.com ) (previously New-Access Communications)

## DPT Applications and Products

The DPT system can be used in LAN, MAN and WAN ring applications due to its new MAC layer protocol (SRP protocol). But it is in the local and the metropolitan environment where the DPT technology is more interesting.

The metropolitan area market, concretely the metro DWDM market, can be divided at least in three different segments [DPT-4]:

- Metro Core: Connections are between carriers PoPs and do not directly interface with end users.

- Metro Access: Consists of the rather nebulous segment between carriers’ PoPs and access facilities.

- Enterprise: Includes multichannel systems used to create high-bandwidth, multiprotocol links between sites within an enterprise.

DPT can be used in all these three segments and an enumeration of some DPT applications is listed below:

- IntraPOP connectivity for leased-line services aggregation and IntraPOP connectivity via high-speed regional rings (Metro Core segment). The DPT provides an ideal solution since uses efficiently the bandwidth and eliminates the complexity associated with intermediate layer 2 switching solutions, among other benefits.

- Metropolitan area IP rings for shared business and residential access loop services (Enterprise and Metro Access segment) allowing robust, self-healing transport for premium services including VoIP, video over IP, VPNs, and protected IP managed bandwidth services.

- Campus rings for enterprise services for large businesses covering MANs and WANs (Enterprise segment).

DPT products can be currently found on several Cisco router series, Cisco 12000 series Gigabit Switch Routers (GSRs) and Cisco 7500 and 7200 series routers [DPT-5].

PentaCom, recently acquired by Cisco, is another provider of products implementing the SRP technology. Its RingStar8000 is a layer one concentrator conceived to complement the Cisco’s Dynamic Packet Transport family of products [DPT-6].

Upcoming DPT developments include the following:

- DPT running at speeds ranging from OC-3c/STM-1c to OC-192c/STM-64c with speed mismatch adaptation.

- To provide tools for planning, monitoring, provisioning and configuration, and trouble resolution.

- To provide architectural extensions including routing and QoS enhancements.

- To extend DPT technology to the edge of the network using DPT access multiplexor products.

# Relevant standardisation processes

Information regarding the organisation of the process of standardisation together with its results are presented in working document of WPG1/WP1/T2 [NR-P&T]. Section 7 (appendix 2) of this document contains:

- Organisational structure of the Internet standards process and Internet Standards (The Internet standardisation process and documents of standardisation for: MPLS, IntServ, DiffServ).

- Standardisation on OTN (ITU-T, ANSI, IETF, ETSI, ODSI, OIF).

- Main standards related to Dynamic Packet Transport (Spatial Reuse Protocol).

- References to Gigabit-Ethernet Standards.

- Main standards related to IP over SONET/SDH.

e.g. bandwidth, burstiness, … .

e.g. maximum delay

The POLICY_DATA elements are depending on the requested service, and have their meaning defined by the service (e.g. the Intserv-objects are defined in the integrated service/RSVP usage-document[RFC-2210]).

Based on [RFC-2475], [DSFWK]

From now on we will use VPI/VCI for the three possibilities.

Or CNI in LION terminology

one LSP per ingress and FEC in the case of non-merge capable switches

Note that the peering is always O(n) in contrast with overlay models which require O(n2) peering which leads to O(n3) IGP rerouting.

And wavelengths in MPS.