Milestone WP1M1: “Network Requirements”

Part 1:  “Analysis of Services”

| Status and Version: | Version 1, Draft |  |
| --- | --- | --- |
| Date of issue: | 30.08.2000 |  |
| Distribution: | Project Internal |  |
| Author(s): | Name | Partner |
|  | Andreas Gladisch | T-NOVA |
|  | Regine Liebenstein | T-NOVA |
|  | Jochen Knapp | T-NOVA |
|  | Josef Röse | T-NOVA |
|  | Ralph-Peter Braun | T-NOVA |
|  | Sabine Szuppa | T-NOVA |
|  | Frank Tetzlaff | T-NOVA |
|  | Monika Jäger | T-NOVA |
|  | Fritz-Joachim Westphal | T-NOVA |
|  | Katsuhiro Shimano | NTT |
|  | Naohide Nagatsu | NTT |
|  | Artur Lasoń | AGH |
|  | Piotr Pacyna | AGH |
|  | Robert Chodorek | AGH |
|  | Jacek Roman | AGH |
|  | Janusz Gozdecki | AGH |
|  | Jan Derkacz | AGH |
|  | Jacek Rząsa | AGH |
|  | Antonio Manzalini | CSELT |
|  | Janusz Maliszewski | TPSA |
|  | Adam Matzke | TPSA |
|  | Josep Sole-Pareta | UPC |
|  | Salvatore Spadaro | UPC |
|  | Davide Careglio | UPC |
|  | Didier Colle | IMEC |
| Checked by: | Janusz Maliszewski | TPSA |

Table of Contents

1	Introduction	4

1.1	Purpose and Scope	4

1.2	Reference Material	4

1.2.1	Reference Documents	4

1.2.2	Abbreviations	6

1.2.3	Definitions	7

1.3	Document History	7

1.4	Document overview	7

2	Application Services	7

2.1	Overview of Most Important Applications	8

2.1.1	Plain Old Telephony Service	9

2.1.2	Voice over IP	10

2.1.3	Videotelephony	11

2.1.4	Videoconferencing	14

2.1.5	Internet Access	16

2.1.6	TV Broadcast	19

2.1.7	Delayed TV Broadcast	21

2.1.8	Movies on demand	22

2.1.9	TV Listings	24

2.1.10	News on Demand	25

2.1.11	Teleworking	27

2.1.12	Distance learning	29

2.1.13	Tele-shopping	31

2.2	Definition of Application Services	33

2.3	Requirements of Application Services on Transport Services	34

2.4	Conclusion	35

3	Client Interface Types and Client Formats	37

3.1	Classic SDH	37

3.1.1	Description of Classical SDH Services	37

3.1.2	Parameters of Interfaces	39

3.1.3	Requirements on Transport Service	39

3.2	SDH frame only	39

3.3	Ethernet	40

3.3.1	The Gigabit Ethernet Standard—IEEE 802.3z	41

3.3.2	Full- and Half-Duplex	41

3.3.3	The OSI layering Model and Gigabit Ethernet	41

3.4	ESCON	44

3.4.1	Protocol Characteristics	44

3.4.2	Physical Parameters of Interfaces	44

3.5	Fibre Channel	44

3.5.1	Protocol Characteristics	44

3.6	Other Formats	46

4	High Level Description of OTN-MCN related Transport Services	48

4.1	TDM-Transport Connection Service	48

4.1.1	SDH Leased Line Services	48

4.1.2	SDH VPN	52

4.1.3	Basic Mapping of native Signals into TDM – Frame	53

4.2	Optical Channel Transport Service	54

4.2.1	Digital Channel Transport Service – Digital Wrapper	55

4.3	IP-based Transport Services	69

4.3.1	IP Transport Service	69

4.3.2	IP Transport Service Characterization	71

5	Additional Information	82

# Introduction to Part 1

## Purpose and Scope

The part one of the mailstone WP1M1 was made on working dokument base: “Network requirements –Analysis of Services” prepared by WP1/T1 (LION/WPG1/WG1/T1) [48].

In the following two definitions of services are distiguished:

a) application-oriented services; these are services which are offered to the clients of the network.

b) transport services; these are transport services which may be offered by the lower layer (=transport) part of the network, especially the OTN.

The focus within this document is on the transport services.

The report is an internal OTN-MCN report describing briefly the basic  OTN-MCN related services including services provided by the optical Channel Layer and Services provided by a new TDM-Layer. In the following, application services will be described briefly, possible transport services will be identified and their requirements on the network will be derived.

## Reference Material

### Reference Documents

| [1] | ITU-T Recommendation G.872 |
| --- | --- |
| [2] | ITU-T Recommendation G.803 |
| [3] | ITU-T Recommendation G.709 Draft: NETWORK NODE INTERFACE FOR THE OPTICAL TRANSPORT NETWORK (OTN) |
| [4] | ATM Forum, Audiovisual Multimedia Services: Video on Demand Specification 1.0, af-saa-0049.000, 1995 |
| [5] [6] [7] | DAVIC 1.4 Part 1: Description of Digital Audio-Visual Functionalities, 1998 DAVIC 1.4 Part 3: Service Provider System Architecture and Interfaces, 1998 ETS 300 264: Integrated Services Digital Network (ISDN); Videotelephony teleservice. Service description, March 1994 |
| [8] | ETS 300 264: Integrated Services Digital Network (ISDN); Videotelephony teleservice. Functional capabilities and information flows, February 1994 |
| [9] | ITU-T Recommendation G.114: Transmission systems and media - One way transmission time, February 1996. |
| [10] | ITU-T Recommendation F.700: Framework Recommendation for audio-visual/multimedia services, July 1996 |
| [11] | ITU-T Recommendation F.720: Videotelephony Services– General, August 1992 |
| [12] | ITU-T Recommendation H.100: Visual Telephone Systems, 1993 |
| [13] | ITU-T Recommendation H.323: Packet-based multimedia communications systems, February 1998. |
| [14] | ITU-T H.222.0, "Information Technology – Generic Coding of Moving Pictures and Associated Audio Information: Video", 1995 |
| [15] | ITU-T H.222.0, "Information Technology - Generic Coding of Moving Pictures and Associated Audio Information: Video", 1995 |
| [16] | ITU-T H.222.0, "Information Technology - Generic Coding of Moving Pictures and Associated Audio Information: Video", 1995 |
| [17] | EURESCOM Project P714 Advanced CSCW Tools for Telecommuting, Deliverable 1, Volume 1: ”Configuration used for the Trials on Telecommuting and Evaluation Criteria”, December 1997 |
| [18] | ISO 13818-2, "Information Technology - Generic Coding of Moving Pictures and Associated Audio Information: Video", 1995 |
| [19] [20] [21] [22] [23] [24] [25] [26] [27] [28] [29] [30] [31] [32] [33] [34] [35] [36] | ISO 13818-3, "Information Technology - Generic Coding of Moving Pictures and Associated Audio Information: Audio", 1995 M. Bossert: Kanalcodierung. Teubner Verlag, Stuttgart, 1998 S. Haykin: Communication Systems. Wiley & Sons, 1994 N.S.Bergano, ”Margin Measurements in Optical Amplifier Systems”, IEEE Photonics Technology Letters, Vol. 5, No. 3, March 1993 C. Kurtzke, ”Kapazitätsgrenzen digitaler optischer Übertragungssysteme”, TU Berlin, Dissertation D83, 1994 F.Tetzlaff, R.-P. Braun, N. Hanik, ”QoS Monitoring of Optical Transmission Links using Amplitude Histograms: Field Implementation and Experimental Evaluation”, NOC 2000 G. Grau, ”Optische Nachrichtentechnik”, Springer Verlag, 1981 C. Hentschel, ”Fiber Optics Handbook”, Hewlett Packard, 2. Edition, Jan. 1988 ITU-T G.703, ”Physical/electrical characteristics of hierarchical digital interfaces”, 1998 ITU-T G.957, ”Optical interfaces for equipments and systems relating to the synchronous digital hierarchy”, 1999 ITU-T G.691, ”Optical interfaces for single channel STM-64, STM-256 systems and other SDH systems with optical amplifiers”, draft Recommendation, April 2000 ITU-T G.707, ”Network Interface for the synchronous digital hierarchy (SDH)”, 1996 ITU-T G.825, ”The control of jitter and wander within digital networks which are based on the synchronous digital hierarchy (SDH)”, 1993 ITU-T G.826, ”Error performance parameters and objectives for international, constant bit rate digital paths at or above the primary rate”, 1999 ITU-T G.828, ”Error performance paramteres and objectives for international constant bit rate synchronous digital paths”, draft new Recommendation, October 1999 ITU-T G.827, ”Availability parameters and objectives for path elements of international constant bit-rate digital paths at or above the primary rate”, 1996 Uyless Black, ”TCP/IP and related Protocols”, McGraw-Hill, 1998 (3rd Edition). William Stallings, ”Data Computer Communications”, Prentice Hall, 1999, (5th Edition). |
| [38] | ETS 300 417-3-1, ”Generic requirements of transport functionality of equipment; Part 3-1: Synchronous Transport Module-N (STM-N) regenerator and multiplex section layer functions”, 1999 |
| [39] | ETS 300 417-4-1, ”Generic requirements of transport functionality of equipment; Part 4-1: Synchronous Digital Hierarchy (SDH) path layer functions” ,1999 |
| [40] | EN 301 164, ”SDH leased lines; Connection characteristics”, 1999 |
| [41] | EN 301 165, ”SDH leased lines; Network and terminal interface presentation”, 1999 |
| [42] | ETS 300 462-2-1, ”Generic requirements for synchronization networks; Part 2-1: Synchronization network architecture”, 1999 |
| [43] | I.432, ”B-ISDN User-Network Interface – Physical layer specification”, 1996 |
| [44] | M.3010, “Principles for a Telecommunications management network”, 1996 |
| [45] | G.781, “Synchronization layer functions”, 1999 |
| [46] | G.782, “Types and general characteristics of synchronous digital hierarchy (SDH) equipment”, 1994 |
| [47] | G.783, “Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks“, 1997 |
| [48] | “Network requirements – Analisys and Services”, LION/WPG1/WP1/T1, July 2000, working document, DT-LION-WP1Task1_01j.doc |
|  | ITU-T Recommendation F.720: Videotelephony Services– General, August 1992 |

### Abbreviations

| BIP |  | Bit Interleaved Parity |  |
| --- | --- | --- | --- |
| TCM |  | Tandem Connection Monitoring |  |
| CNM |  | Customer Network Management |  |
| DWDM |  | Dense Wavelength Division Multiplex |  |
| FDM |  | Frequency Division Multiplexing; is used as a synonymon for DWDM |  |
| VPN |  | Virtual Private Network |  |
| MS |  | Multiplex Section |  |
| MSP |  | Multiplex Section Protection |  |
| RS |  | Regenerator Section |  |
| RDI |  | Remote Defect IndicationI |  |
| REI |  | Remote Error Indication |  |
| APS |  | Automatic Protection Switching |  |
| CRC |  | Cyclic Redundancy Check |  |
| OTN |  | Optical Transport Network |  |
| FEC |  | Forward Error Correction |  |
| OPU |  | Optical Payload Unit |  |
| ODU |  | Optical Data Unit |  |
| OTU |  | Optical Transport Unit |  |
| AIS |  | Alarm Indication Signal |  |
| BDI |  | Backward Defect Indication |  |
| SLA |  | Service Level Agreement |  |
| SAP |  | Service Access Point |  |
| NSP |  | Network Service Provider |  |
| ICMP |  | Internet Control Message Protocol |  |
| TCP |  | Transmission Control Protocol |  |
| UDP |  | User Datagram Protocol |  |
| IP |  | Internet Protocol |  |
| BEI |  | Backward Error Indication |  |

### Definitions

| TMF | Telemanagement Forum , formerly NMF, Network Management Forum |
| --- | --- |

## Part 1 Milestone WP1M1 history

| Version | Date | Authors | Comment |
| --- | --- | --- | --- |
| 0.01 | 29/2/2000 |  | Initial documents |
| 0.02 | 3/3/2000 |  | Update based on comments |
| 1.00, draft | 28/07/2000 | T-Nova | Working document WP1-Task1 “Analysis of Services” |
| 2.00, draft | 30/08/2000 | Janusz Maliszewski | Accomodation WP1Task1document to WP1M1 document form |

## Part 1 overview

The part 1 of the milstone WP1M1 is structured as follows. Section 2 describes briefly the application services. Requirements to the transport services are included. Section 3 describes several client interface types and client formats of the transport network. Section 4 describes the transport services. Section 5 contains same additional information.

# Application Services

The section presents results of studies on the most important application services which in opinion of authors have to be taken into account in evaluation of the Optical Transport Network Requirements.  The work has been based mostly on the DAVIC 1.4 specification and ITU-T Recommendations. Some other relevant reference documents had been also analyzed and taken into account.

The pre-selected set  of services has been briefly described in order to provide Project Partners with clear depiction of functionality of applications characterized. The following applications have been included: POTS (Plain Old Telephony Service), Voice over IP, Videotelephony, Videoconferencing, Internet Access, TV Broadcast, TV Delayed Broadcast, TV Listing, Movies on Demand, News on Demand, Teleworking (Telecommuting), Distance Learning, Tele-shopping and Virtual CD-ROM. Description of each  application is concluded with Application Technical Requirements specification. These specifications were defined in order to assist the progress of Applications classification into specific Application Services and evaluation of Application Services Technical Requirements.

Figure 1: Evaluation of OTN Network Requirements

The goal of the definition of Application Services Technical Requirements is to provide the information necessary to identify basic requirements on Optical Transport Network with regard to applications and services to be supported in the near term future. The methodology of Network Requirements evaluation is depicted in the Fig.1.

## Overview of Most Important Applications

Classification of Applications:

Human to Human

- real time voice and video, tele-medicine, tele-immersive VR, etc

- very sensitive to jitter and delay

- very symmetric & growing linearly

Human to Computer\

- web, voice mail, video servers, call centers, fax - mostly TCP

- jitter and delay can be compensated with client buffering

- fractal & very asymmetric & growing exponentially

Computer to Computer

- bio-informatics, IP appliances, off site backups, web caching

- insensitive to jitter and delay, but extremely fractal

- extremely asymmetric & growing exponentially plus

### Plain Old Telephony Service

Plain Old Telephony Service (POTS) is defined as a service which provides voice connection between two or more users at separate locations. The user initiates and controls the conversation. The analogue transmission of the voice signal in the local loop is provided typically in the 0.3-3.4 kHz bandwidth. In the central office analog voice signal is digitized and transmitted over digital, circuit switched network. Digital voice signal is transmitted with 64 kbps bit rate. POTS is Human-Human, real-time service.

#### Functional specification

List of end-user functions follows:

Establish a new voice session.
Disconnect the call.
Send audio, low quality video and data to another participants by means of voice band modem.
Receive audio, low quality video and data from interlocutor by the use of voice band modem.
Send and receive data with the use of fax.
Allow a user to subscribe to the value added services.

List of service provider functions:

Enable a user to subscribe to the service. 
Authenticate and authorize a user attempting to use the service.
Establish permanent connection links to service subscribers.
Introduce data and control flows towards the user.
Support billing by generating charge records.
Provide billing information.
Interoperate with other POTS systems offered by other service providers. 
Support value added services

#### Technical requirements

Voice services are very sensitive to delay and delay variation (jitter). Transport Network must guarantee that the delay limits defined in ITU-T G.114 will not be violated. One-way delay limits defined for standard telephone service follows:

- delay of  0 to 150 ms: acceptable for most user applications

- delay of 150 to 400 ms: acceptable provided that service provider is aware of the transmission time impact on the transmission quality of user applications

- above 400 ms: unacceptable for general network planning purposes.

Delay jitter can be neglected in the PSTN networks, similar requirements have to be placed on optical transport networks. Recovery time of the transport network supporting POTS should be less then 50ms.

- Table 1: Technical requirements of the POTS service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | Point-to-point Multipoint-to-Multipoint | The session comprises of two or more POTS subscribers controlled by Control Unit. |
| Session control | Distributed | Each of the POTS subscribers can control the session |
| Session duration | 3 min.  30 min. |  |
| Interactivity level | Real-time |  |
| System response time | Very high | System response time less than 250 ms |
| Flow | Symmetric |  |
| Data flow bandwidth | 64 kbps | Compression used in some systems (32 kbps) |
| Data flow encoding | G.711 PCM, G.721 ADPCM |  |

### Voice over IP

Voice over IP is a service which provides voice connection between two or more users at separate locations. Digitized and compressed voice signal is transmitted through networks based on the IP family protocols.

#### Functional specification

Voice over Internet Protocol (VoIP) systems digitize and transmit analog voice signals as a stream of packets over IP network. The transmission is connectionless in that no continuous circuit connection need to be maintained for the packets to traverse the network. The transmissions typically take place over the transport network used by providers of long distance circuit-switched services. Public IP service provider (VoIP provider) must establish points of presence where POTS-VoIP translation can occur. Translation occurs in gateways,  which form the building blocks of an IP voice network. A gateway router or switch performs several operations on the voice signal entering it — or leaving it. First, a voice router or IP switch will generally compress the digitized voice signal with a vocoder or codec. Compression is employed — frequently down to an 8–16 kbps rate. The emerging trend in IP voice network equipment is the variable vocoder, which can be programmed for various rates or can change rates based on network conditions to maintain a predetermined level of voice quality. Next, the gateway will packetize the digitized, compressed voice, adding addressing information, and possibly error correction code, to allow its transmission over an IP network.

Today VoIP does not seem as substitute to carrying voice and probably in the near future will be used most where PSTN calls are expensive. For company with offices in different cities or countries, VoIP can diminish expenses on  long-distance calls and as some researches show expenditures on equipment can return after several months. Industry reports estimate that between $2 billion and $8 billion will be spent on Internet telephony technology by 2001. Some market research reports predict that the worldwide IP market will be worth $60 billion by 2005.

List of end-user functions follows:

- Establish a new voice session.

- Disconnect the session.

- To allow a user to subscribe to the value added services.

List of service provider functions:

- Enable a user to subscribe to the service.

- Authenticate and authorize a user attempting to use the service.

- Introduce data and control flows towards the user.

- Support billing by generating charge records.

- Provide billing information.

- Interoperate with other voice systems (PSTN/GSM/VoIP) offered by other service providers.

#### Technical requirements

IP based transport network must guarantee that the delay limits defined for POTS will not be exceeded. There is a very common proposal to extend typical 150 ms limit up to the 250-300 ms. Some proposals are based on the results of research on one way transmission time impact on quality of voice service (ITU-T G.114).

Recovery time of IP transport network supporting VoIP has to be much more relaxed due to the problems with fast and reliable distribution of routing information. Exact recovery timing is still under study.

- Table 2: Technical requirements of the VoIP service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | Point-to-point Multipoint-to-Multipoint | The session comprises of two or more VoIP subscribers controlled by Control Unit. |
| Session control | Distributed | Each of the VoIP subscribers can control the session |
| Session duration | 3 min.  30 min. |  |
| Interactivity level | real-time |  |
| System response time | High | System response time less than 250 ms [1] |
| Default access mode | Protected |  |
| Aggregate flow | Symmetric |  |
| Data flow bandwidth | 8-32 kbps | 5.3 kbps compression used in some systems |
| Data flow direction | G.728 LDCELP, G.729 CS-ACELP |  |

### Videotelephony

Videotelephony is defined as a service which provides connection between two users at separate locations. Optionally connection to a service provider should be enabled. The user initiates and controls the conversation, some additional information may be exchanged between the users. The application provides the user a real time, bi-directional exchange of audio, video and other data [2].

#### Functional specification

Functional specification of videotelephony follows.

List of end-user functions:

- To allow a user to subscribe to service.

- Establish a new videophone session.

- Send ID, PIN for identification and access permission.

- Disconnect the videophone call.

- Send audio, video and data to another participants of the videoconference.

- Receive audio, video and data from interlocutor.

- Share the same application by videophone participants.

- View data concerning the service & the relationship between the user & the service provider.

- To allow a user to cancel the subscription, to give permission for payment.

List of service provider functions:

- Enable a user to subscribe to the service and define user profile.

- Provide a consistent user interface by means of application.

- Authenticate and authorize a user attempting to use the service.

- Establish connection links to service subscribers.

- Introduce data and control flows towards the user.

- Support billing by generating charge records.

- Provide billing information.

- Resource reservation to support quality of service.

- Interoperate with other videophone systems offered by other service providers.

- Provide means for conditional access – service security.

#### Technical requirements

The level of acceptable delay between videotelephony service users is the most important issue of videotelephony technical requirements. The effects of delay on video phone service were studied by numerous R&D centers. Some results are presented  in ITU-T Recommendation G.114. The comparison of results of studies on round trip delay sensitivity of videotelephony and standard telephony is shown in Fig. 2.  The degradation of service MOS (Mean Opinion Score) is similar for both services. The substantial differences were not observed neither in the case of highly interactive task of reading out random numbers (see Fig.2, Task 1) nor in the case of free conversation (Fig. 2, Task 2). The MOS rating is the standard (ITU) quality of service measure and it takes values from range from 0 to 5.

- Task 1						       Task  2

Figure 2: Comparison of delay sensitivity of telephony (circle marker) and video telephony (triangle marker) services. Results of NTT research, 1991.

The level of videophone connection acceptability was also studied by COMSAT (1992)[6]. Results are summarized in the table 1. The table is extracted from the ITU-T Recommendation G.114. The videophone connection acceptability remains very high even with delays much higher than 
200 ms.

| Table 3: Variation of subjective performance for three end-to-end videophone connections [6] | One-way transmission delays |  |  |
| --- | --- | --- | --- |
|  | 200 ms | 450 ms | 700 ms |
| MOS connection quality | 3.74  0.52 | 3.69  0.51 | 3.48  0.48 |
| MOS ease of interruption | 4.00  0.55 | 3.79  0.53 | 3.56  0.49 |
| Communication difficulty | 28  4% | 35  5% | 46  6% |
| Connection acceptability | 80  11% | 78  11% | 73  10% |
| All errors are defined at a 95% level of confidence. |  |  |  |

Taking into consideration results presented above it is reasonable to adopt for video telephony one-way delay limits defined for standard telephone service. These limits follows [6]:

- delay of  0 to 150 ms: acceptable for most user applications

- delay of 150 to 400 ms: acceptable provided that service provider is aware of the transmission time impact on the transmission quality of user applications

- above 400 ms: unacceptable for general network planning purposes.

The delays above 400 ms are unavoidable in the case of  double satellite hops, satellites used to restore terrestrial routes, fixed satellite and digital cellular interconnections, video telephony over satellite circuits and very long international connections with two digital cellular systems connected by long terrestrial facilities. In such cases the delay can be conditionally accepted.

Based on above requirements for video telephony services it is possible to evaluate the delay requirements for OTN transport network The delay introduced by network of video phone call originator (PSTN, ISDN, packet network) should not exceed 150 ms.

Technical requirements of the Videotelephony service are presented in Tab. 4.

- Table 4: Technical requirements of the Videotelephony application

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | Point-to-point | The session comprises of two videophone subscribers controlled by Control Unit. |
| Session control | Distributed | Each of the videophone subscribers can control the session |
| Session duration | 3 min.  30 min. |  |
| Interactivity level | real-time |  |
| System response time | Very high | System response time less than 250 ms[2] |
| Default access mode | Protected | Password protected |
| Aggregate flow | Audio – symmetric video – symmetric |  |
| Data flow bandwidth | video: n * 64 kbps per video stream audio: codec dependant | n of the range 4  30. 64 kbps max. |
| Data flow direction | Two-way |  |
| Data flow encoding | video: H.261, H.263 audio: G. 7xx |  |

There are two optional networking protocols which can be adapted to support videotelephony service. The videotelephony service can be based on ATM/AAL-5 transport or can be based on IP family of protocols.  The choice of networking protocol will depend on the application software which may use native ATM network service or use IP. The IP based approach is now much  more common solution for voice audio and video based communications systems.

### Videoconferencing

Videoconferencing is a service which provides a real time bi-directional exchange of audio, video, and data information between multiple users. Involved are one or more people at two or more geographically separated sites, and a multipoint service provider. A user can establish, announce and control the conference. During the conference shared applications may be used to facilitate exchange of information.

A videoconferencing terminal can communicate with other terminals through the Multipoint Control Unit (MCU). This is a multi-port device, which manages session and controls individual terminals - members of the session. The physical realization of the MCU is such that several simultaneous sessions may be set up within the same unit. The MCU is the main component of this service. Numerous MCUs may exist in the network provider system to scale the system up. Then the MCUs must implement common resource management policy and perform load balancing. To enable this they elect a master MCU for superior control of the slave MCUs.

#### Functional specification

List of end-user functions for videoconferencing service are based on the list of videotelephony functional requirements. Some modifications have been introduced in order to reflect multi-user type of the service. List of end-user functions follows:

- To allow a user to subscribe to service.

- Get a list of currently active videoconferencing sessions.

- Connect to an active session.

- Get a list of a session participants, if not restricted.

- Establish a new session.

- Elect a session manager.

- Pass the attribute of a session manager to another session member.

- Send ID, PIN for identification and access permission.

- Leave a videoconferencing group.

- Control a videoconferencing group by the session manager.

- Send audio, video and data to another participants of the videoconference.

- Receive audio, video and data from another participants of a videoconference.

- Share the same application by all session participants.

- View data concerning the service and the relationship between the user and the service provider.

- Perform action related to subscription, e.g. to subscribe, to cancel the subscription, to give permission for payment (credit card, money transfer).

List of service provider functions:

- Enable a user to subscribe to the service and define user profile.

- Advertise active sessions.

- Provide a consistent user interface by means of application.

- Authenticate and authorize a user attempting to use the service.

- Establish connection links to session participants.

- Introduce data and control flows into the SPS-SCS network towards the user.

- Support billing by generating charge records.

- Provide billing information.

- Resource reservation to support quality of service.

- Interoperate with other videoconferencing services offered by other service providers. 
Provide means for conditional access  - security.

#### Technical requirements

Technical requirements of the Videoconference service are presented in Table 5. The technical requirements specification is based on the videotelephony description.

- Table 5: Technical requirements of the Video Conference service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | Multipoint-to-multipoint | The session comprises of a single server and two or more users. |
| Session control | Centralised, by session manager | Session manager has full control over session |
| Session duration | 5 min.  90 min. |  |
| Interactivity level | Real-time |  |
| System response time | Very high | System response time less than 250 ms [2] |
| Default access mode | Protected | Password protected |
| Aggregate flow | Audio – symmetric video – asymmetric | Dialog with frequent pass-over. One flow upstream, several flows downstream |
| Data flow bandwidth | Video: n * 64 kbps per single video stream Audio: codec dependant | N is in the range 6  30. 64 Kbps max. |
| Data flow direction | two-way |  |
| Data flow encoding | Video: H.261, H.263 audio: G. 7xx |  |

### Internet Access

Internet Access service provides connectivity to the worldwide Internet over TCP/IP protocol suite, and the ability to use applications implemented there.

#### Functional specification

General specification of the Internet Access service is presented to provide the list of end user and service provider most important and common functions. The list is probably not exhaustive. The services and application used in the Internet are very dynamic and task of the service functional specification is very complex. List of end-user required functionality follows.

- Allow a user to subscribe to service.

- Connect to a service on a remote host using that hosts' IP address.

- Request a data available at the specific location (URL).

- Introduce data by means of typing (e.g. type in a password/query/search criterion)

- Submit data.

- Search for specific information.

- Navigate through the content.

- Initiate upload or download of a given data (file, e-mail).

- Initiate streaming of time-critical data.

- Cancel previously initiated action.

- Set a bookmark, go to bookmark.

- View data concerning the service and the relationship between user and service provider (such as usage, billing and account details).

Perform action related to subscription, e.g. to cancel the subscription, to give permission for payment (credit card, money transfer).

List of service provider functions:

- Enable to subscribe a user to the service and define user profile.

- Authenticate and authorize a user attempting to use the service.

- Make content available and advertise the availability of the content.

- Provide a consistent user interface by means of appropriate application.

- Provide data download via the Internet.

- Provide data upload via the Internet.

- Provide a proxy Web server to retrieve data from the Internet and store it locally.

- Provide mailboxes to users.

- Provide routing function.

- To handle subscriber billing/accounting.

- To keep track of user data (who accessed what, etc.).

- Automatic insertion of information into downloaded data (e.g. graphical commercial insertion into web pages, ads, etc.)

- Transport various data formats down to the user including motion video, still pictures, audio, text and graphics.

#### Technical requirements for WWW access service

The WWW service allows to access web pages that are stored in the provider systems or that are available in the global Internet. The www service allows to transfer data in various formats down to the user including formatted text, graphics, still pictures, audio and motion video.

All data accessible to users is downloaded from a www server. The server manages arbitrarily large number of www pages. In addition to the pages provided by the www content providers, residential and institutional users should have possibility to create their own www pages. Technical requirements of the www access service are presented in Table 6.

- Table 6: Technical requirements of www access service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-point | The session comprises of a single server and single user. |
| Session control | Centralised, by user | The user has full control over the session |
| Session duration | Undefined |  |
| Interactivity level | not applicable | The service is not interactive |
| System response time | Moderate | Delays of 2 sec. max. are tolerable |
| Default access mode | Unprotected | No password required |
| Media delivery | Undefined | Typically: non continuous media – file download |
| Aggregate flow | highly asymmetric | High bandwidth downstream small bandwidth upstream |
| Data flow bandwidth | Undefined | The bandwidth is content dependant |
| Data flow direction | Downstream (to user) |  |
| Data flow encoding | HTTP 'data', GIF, JPEG, MPEG1, content dependant (eg. pdf, ps, doc) |  |
| Data flow encryption | Optional |  |
| Control flow encoding | HTTP, ftp |  |
| Networking protocol | TCP/IP |  |

#### Technical requirements for e-mail service

The e-mail is among the most popular services in IP based networks. Originally e-mail service was used only to send short text messages to other users. As the functionality of the mailing applications improves, it is more frequently used to send binary files, like word processing documents, spreadsheet documents and graphical files. Continually increasing volume of e-mail puts greater requirements to the e-mail server file systems. Technical requirements of the e-mail service are presented in Table 7.

- Table 7: Technical requirements for e-mail service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-point | The session is set-up between a single server and a single user. |
| Session control | Centralised, by user | The user has full control over the session |
| Session duration | 3 min. 15 min. |  |
| Interactivity level | non interactive |  |
| System response time | High |  |
| Default access mode | Protected | Password protected |
| Media delivery | Undefined | Non continuous media (download) |
| Aggregate flow | Asymmetric | Higher bandwidth downstream small bandwidth upstream |
| Data flow bandwidth | 64 kbps |  |
| Data flow direction | two way |  |
| Data flow encoding | SMTP, text, X.400 | All encoding converted to plain text before exchange |
| Control flow encoding | POP2, POP3, IMAP |  |
| Networking protocol | TCP/IP |  |

#### Components of the Internet Service Provider System

The ISP System consists of devices used to process and display content information downloaded by the user from the ISP. In the case of Internet Access Service the user terminal need to be configured prior to operation. The network configuration may by stored in local nonvolatile memory or can be obtained from ISP by means of DHCP or BOOTP services.

The DHCP or BOOTP services are used to read configuration parameters from DHCP and BOOTP servers located inside the ISP System. These servers manage databases in which network parameters for each user terminal are defined. The terminal is identified by a unique identifier – MAC address. The parameters necessary to perform base configuration are: IP address, IP netmask, IP address of the DNS server.

For the reasons of reliability the DHCP (or BOOTP) server should be replicated. The full list of the DHCP parameters is available in the DAVIC 1.4 part 12 DHCP Options for Internet Access [6].

The Domain Name Service (DNS) is required for all user IP-based applications. It resolves user-friendly symbolic addresses of the servers into IP dotted notation. It is also used for internal purposes by the IP-based application servers and network services of the SPS, and by the applications running on the users terminal. The access network and the SPS will typically constitute a single DNS domain. However it is possible to define two or more domains there, e.g. to distinguish between the residential and institutional users. Then there should exist one DNS server per DNS domain (the master server for that domain). In order to improve the reliability of the DNS service the servers should be replicated.

### TV Broadcast

Users subscribe many broadcast services, for instance radio and TV broadcasting. These services were recently moved to network environment. DAVIC 1.4 specification [2, 3] defines two Broadcast services: Broadcast and Delayed Broadcast. This deliverable is focused on TV Broadcast.

#### TV Broadcast

Broadcasting is an application providing multiple users with immediate, real-time access to multiple sources of TV, Radio or Data Programming [2]. Broadcasting includes applications such as Pay-Per-View (PPV), subscription TV (Pay-TV) or conventional open access TV. TV broadcast service can be set up with the functionality assuring interactivity between the user and the service/network/content provider. The Service Provider, in many cases, acts as a broker for material between the User and the Content Provider. Service contents can comprise real-time video, audio, text and data streams from which the user can select a program for immediate access. Thus, the user can be enabled to participate in a program or get control over a program or parts of it. User inputs to elections, polls and games can possibly alter the content of the broadcasting program.

The next service configuration, which is a subset of the first one, has no requirements for an upstream information flow from the user to the service/network/content provider. The service is unidirectional. Specification of the TV broadcast service user follows.

- Allow a user to subscribe to service.

- Get a list of available content.

- Select content.

- To get at any time and to browse through information about the network, the services (programs, data) and the events through easy to use user interfaces such as on screen menus and graphics

- To initiate a self-configuration of the STU for the available channels of the connected network
To perform fast channel hopping

- To select among the available service providers, a service or a single event

- To select language for specified content or additional data (e.g. subtitles)

- To check account data and to retrieve billing record

- Participate in an interactive program

- To personalize their selection of services, for example through easy connection to preferred services, pre-selection of favorite services and applications, and personal filtering

List of Service Provider Functions:

- Provide information about the services (service names, service provider names, free/CA-mode, etc.).

- Provide information about the program events (start times, duration, free/CA-mode, audio modes, languages, subtitles, content types, additional text, kind of data, version of uploaded software) to support the end user selection.

- Provide means for conditional access (CA).

- Allow an end user to subscribe to services and define user profile (define automatic upgrading of software).

- Control conditional access.

- Establish control channels.

- Provide Service-Provider-specific "Look-and-Feel" of the graphical user interface for selecting between optional program parts

- Make content available and advertise the availability of the content.

- Provide a consistent user interface by means of appropriate application.

- Provide a consistent content manager interface.

- Classify the content for easy access and selection.

- Authenticate and authorize a user attempting to use the service.

- Perform access control (blocking certain types of content from being received at certain times and/or by certain people).

- Charging and billing

#### Technical requirements

Technical requirements of the Broadcast service are presented in Table 8.

- Table 8: Technical requirements of the Broadcast service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-multipoint | The session comprises of a single server and at least one user. |
| Session control | Centralised, by User or Service Provider | The user can select to join to proper broadcast transmission |
| Session duration | 1 min.  8 hours | Depends on content: can be short news, full length movie or updating large database |
| Interactivity level | real-time |  |
| System response time | High | System response time less than 250 ms (in interactive mode) |
| Default access mode | Protected | Password protected |
| Flow types | Asymmetric | High bandwidth downstream Low bandwidth upstream |
| Data flow bandwidth | video: 2-8 Mbit/s data: undefined |  |
| Data flow direction | Very asymmetric downstream |  |
| Data flow encoding | ISO 13818-3 ISO 13818-2 | MPEG2 video MPEG2 audio |
| Control flow encoding | Video and audio: DSM-CC or H.245 Data: HTTP, MFTP |  |

TV Broadcast service can use two stacks networking protocols. The service can be based on ATM/AAL-5 transport or it could use IP (IPv6) over ATM or over SDH.  In layers above IP protocols like RTP/UDP for real-time stream and/or RMTP/UDP for reliable data distribution should be used.

### Delayed TV Broadcast

The Broadcast service is similar to classical TV or radio distribution. Delayed Broadcast services was defined to provide more functionality to Broadcast services. The Delayed TV Broadcast service provider can perform all function of Broadcast service provider. The main extension of the Delayed Broadcast is that the Service Provider or the end user selects a scheduled broadcast program to be stored at the service provider for delivery at a later time. Later delivery may be performed using MOD (Movies On Demand), NVoD or repetitively Broadcast.

#### Functional specification

Delayed Broadcast is a specific broadcasting application. The Service Provider or the end user selects a scheduled broadcast program to be stored at the service provider (or at the network) for delivery at a later time. The time of delivery may or may not be scheduled at the time of the application request. Later delivery may be performed using MOD, NVoD or repetitively Broadcast. Computer software, electronic newspaper and other kind of data could be broadcasted and fetched from the network in the same manner.

List of end-user functions:

- Authentication/Identification. To use in the dialogue with the service provider.

- Allow a user to subscribe to service.

- Navigation. To look for what is, or will be (after a request) available to view.

- Interactively view the content (video). This includes the VCR functions.

- Get a list of available content (e.g. video or new software, data).

- Select content (also language for data).

- Content presentation (audio, video) or downloading (data, software).

- Content decoding.

- Billing/Charging. Information about the account (also payment for received data or software).

List of Service Provider Functions:

- Authentication/Identification. Known users could be stored in a distributed database.

- Billing/Charging.

- Intellectual property protection. Management of royalty tasks (e.g. software and data).

- Content management. Recording, (encoding & compression of analogue/uncompressed information), storing, managing, sending and deletion of broadcasted information.

- Exception procedures. Signaling downstream when recording failed.

- Content presentation.

- Ad (commercial) insertion. Could be distributed with geographical differences.

#### Technical requirements

Technical requirements of the Delayed Broadcast service are presented in Table 9.

- Table 9: Technical requirements of the Delayed Broadcast service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-multipoint | The session comprises of a single server and at least one user. |
| Session control | centralised, by user, CPS or SPS | The user has control over the session or the user can select to join to proper broadcast transmission |
| Session duration | Undefined | Depends on content: can be short news, full length movie or updating large database |
| Interactivity level | audio and video: real-time data non real-time |  |
| System response time | High | System response time less than 250 ms |
| Default access mode | Protected | Password protected |
| Media delivery | audio and video: continuous media data: non continuos |  |
| Flow types | Asymmetric | High bandwidth downstream Low bandwidth upstream |
| Data flow bandwidth | video: 2 –8 Mbit/s data: undefined |  |
| Data flow direction | Asymmetrical down stream |  |
| Data flow encoding | ISO 13818-3 ISO 13818-2 HTML, MHEG-5 | MPEG2 video MPEG2 audio data object |
| Control flow encoding | DSM-CC or H.245 data: HTTP, FTP |  |

### Movies on demand

The Movies on Demand (MoD) service is an asymmetrical service that transfers a digital, compressed and encoded video and audio information from a server (typically a video server), to a client. At the consumers' set-top terminal the streams are reassembled, uncompressed, decoded and presented on a display [1].

#### Functional specification

Movies on Demand is a video service where the end user has a pre-determined level of control over the selection of the video content as well as the time of viewing. One implication of this is, that the MoD sessions are point-to-point, connecting a Video Information Provider  with an individual user.

MOD offers the functionality of a home VCR including select/cancel, start, stop, pause (with or without freeze frame), fast forward and reverse. More advanced implementation may enable scanning forward/ reverse, setting and resetting memory markers, showing counters, jumping to different indexes. The MoD service ensures synchronization of the audio and video streams as well as time base recovery. The MoD service is likely to be used for entertainment and educational/training purposes to allow subscribers access to a library of programs (e.g., movies) collected in a digital repository.

List of end-user functions:

- Allow a user to subscribe to service.

- Get a list of available content.

- Select content.

- Interactively view the content. This includes the VCR functions.

- To be able to get, before and during viewing, additional information with regard to the selected content.

- To switch on/off the subtitles or closed captioning.

- Make reservations in the case of limited availability of content or network resources.

- Set up personal preferences by users such as ‘I normally want to see only sports items’.

- To perform action related to subscription, e.g. to subscribe, to cancel the subscription, to give permission for payment (credit card, money transfer).

- To view data concerning the service and the relationship between the user and the service provider (such as usage, billing and account details).

- Select movie by rating.

- Set a bookmark.

- Go to bookmark.

- Enable to choose dynamically between several levels of quality of service.

List of Service Provider Functions:

- Enable to subscribe a user to service and define user profile.

- Make content available and advertise the availability of the content.

- Provide a consistent user interface by means of appropriate application.

- Provide a consistent content manager interface.

- Classify the content for easy access and selection.

- Provide indexes of content scenes to ease fast access to a given part of movie.

- Authenticate and authorize a user attempting to use the service.

- Perform access control (blocking certain types of content from being viewed at certain times and/or by certain people).

- To keep track of user data.

- To keep track of usage data, (used e.g. to mark where the user ‘left’ a movie, so that at a later point in time the movie can be resumed from that exact point)

- Introduce S1 flow into the SPS-SCS network delivery system in accordance with its time-critical nature over A9 interface.

- Support billing by generating charge records.

- Provide billing information.

- Perform Resource reservation to support quality of service.

- Interoperate with other MoD services offered by other service providers (proxy).

#### Technical requirements

Technical requirements of the Movies on Demand service are presented in Table 10.

- Table 10: Technical requirements of the Movies on Demand service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-point | The session comprises of a single server and a single user. |
| Session control | centralised, by user | The user has full control over the session |
| Session duration | 3 min.  2 hours | Depends on video content: can be video clip or full length movie |
| Interactivity level | real-time |  |
| System response time | high | System response time less than 250 ms [1] |
| Default access mode | protected | Password protected |
| Media delivery | continuous media |  |
| Flow types | asymmetric | High bandwidth downstream Low bandwidth upstream |
| Data flow bandwidth | 750 Kbps – 2 Mbps |  |
| Data flow encoding | ISO 13818-3 ISO 13818-2 | MPEG2 video MPEG2 audio |
| Control flow encoding | DSM-CC or H.245 |  |

The standard way of carrying video traffic over the (core and access part) is the Asynchronous Transfer Mode [1, 3].  An interesting alternative for MoD stack of networking protocol is the family of IP protocols.

### TV Listings

TV Listings (EPG - Electronic Program Guide) is a broadcast application provided by the broadcast service provider that gives the user information about the services, upcoming events, etc. The receiver has a resident Navigator that provides the user means to invoke a TV Listings of a selected service provider.

#### Functional specification

Once the TV-listings are selected, the end-user is provided with a (scrolling) display of distribution programs and their related schedules. The end user may select a listed distribution program to get additional information (situation, actors, when made, etc.), potentially in progressive queries.

In practical cases, several selections can be chained from general searches producing large sets through specific searches within sets to final direct selection from the remaining list of items.  Selection can be user initiated or agent initiated.  Selected programs can be either a specific program or a collection like a series or a thematic set.

Selection mechanisms include:

- Search-based selection: A search engine or an agent accepts a request and produces a list of items supposed to fit the criteria.  The search may use many data sources, including EPG data or program metadata.

- Direct selection (e.g. activates link on a web page or chooses from a list): Direct selection may be from an EPG, from a promotion, or from other printed or electronic sources.

List of end-user functions:

- Allow a user to subscribe to service.

- Get a list of available content.

- Select content.

- Interactively view the content.

- Receive notification about new subscribed content (subscribed TV Listings).

- Scan TV Listings for relevant content to a viewer’s interest.

- To perform action related to subscription, e.g. to subscribe, to cancel the subscription, to give permission for payment (credit card, money transfer).

- To view data concerning the service and the relationship between the user and the service provider (such as usage, billing and account details).

List of Service Provider Functions:

- Enable to subscribe a user to service and define user profile.

- Make content available and advertise the availability of the content.

- Provide a consistent user interface by means of appropriate application.

- Provide a consistent content manager interface.

- Classify the content for easy access and selection.

- Authenticate and authorize a user attempting to use the service.

- Perform access control (blocking certain types of content from being viewed at certain times and/or by certain people).

- Provide billing information.

- Perform Resource reservation to support quality of service.

- Interoperate with other TV Listings services offered by other service providers.

#### Technical requirements

Technical requirements of the TV Listings service are presented in Table 11.

- Table 11: Technical requirements of the TV Listings service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-point | The session comprises of a single server and a single user. |
| Session control | centralised, by user | The user has full control over the session |
| Session duration | 1 min.  20 min | Depends on looking range of TV Listing |
| Interactivity level | real-time |  |
| System response time | high | System response time less than 250 ms |
| Default access mode | protected | Password protected |
| Media delivery | continuous media |  |
| Flow types | Asymmetric | High bandwidth downstream (when user required multimedia data) Low bandwidth upstream |
| Data flow bandwidth | text and graphics: 64 kbps video trailer: 750 Kbps - 2 Mbit/s |  |
| Data flow direction | two way |  |
| Data flow encoding | text and graphics: MHEG-5, HTML video trailer: ISO 13818-3 ISO 13818-2 | MPEG2 video MPEG2 audio |
| Network protocol | ATM AAL5 or RTP/IPv6 or for data distribution RMTP/IPv6 transfer) |  |

### News on Demand

News is specific kind of content. It changes quickly and user would like to receive content (news) in real-time or offline.  Service defined for delivery news are called “News on Demand”.

#### Functional specification

News on Demand service delivers news determined by the end user. The user interactively obtains information about news items. The service also provides summaries and headlines. The user can choose news items to view, the presentation level and the presentation material (e.g. only text, text with graphics or also moving video).

Extensions to the News on Demand can be:

- collecting news from different sources based on user queries;

- (generating and) using a personal profile to change the items presented and the way of presentation.

These two extensions combined give an application that can be called “Personalized News”. Function listed bellow was defined for News on Demand and Personalized News.

- Allow a user to subscribe to service.

- Get a list of available content.

- Select content (news).

- Interactively view the content. (This includes the VCR functions for video material and hypertext viewer for text with graphics).

- Select language of collected news.

- Set up personal preferences by users such as ‘I normally want to see only sports news’.

- Receive notification about new subscribed content (subscribed news e.g. next information about accident).

- Scan news for relevant content to a viewer’s interest.

- To perform action related to subscription, e.g. to subscribe, to cancel the subscription, to give permission for payment (credit card, money transfer).

- To view data concerning the service and the relationship between the user and the service provider (such as usage, billing and account details).

List of Service Provider Functions:

- Enable to subscribe a user to service and define user profile.

- Make content available and advertise the availability of the content.

- Provide a consistent user interface by means of appropriate application.

- Provide a consistent content manager interface.

- Classify the content for easy access and selection.

- Authenticate and authorize a user attempting to use the service.

- Perform access control (blocking certain types of content from being viewed at certain times and/or by certain people).

- Provide billing information.

- Perform Resource reservation to support quality of service.

- Interoperate with other News on Demand services offered by other service providers.

- Establish multipoint connection to current news viewed by more than one user (for most popular news).

#### Technical requirements

Technical requirements of the News on Demand service are presented in Table 12.

- Table 12: Technical requirements of the News on Demand service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-point point-to-multipoint | The session comprises of a single server and a single user. For current news use session of single server and multiple user (most important news transmitted in real time) |
| Session control | centralised, by user | The user has full control over the session |
| Session duration | 2 min.  2 hours | Depends on news content: can be short news or full review of news |
| Interactivity level | real-time |  |
| System response time | high |  |
| Default access mode | protected | Password protected |
| Flow types | asymmetric | High bandwidth downstream Low bandwidth upstream |
| Data flow bandwidth | video: 750 Kbps - 2 Mbit/s data: not less than 10kb/s |  |
| Data flow direction | one way: downstream |  |
| Data flow encoding | ISO 13818-3 ISO 13818-2 HTML, MHEG-5 | MPEG2 video MPEG2 audio data object |
| Control flow encoding | DSM-CC or H.245 data: HTTP, FTP |  |
| Network protocol | ATM AAL5 or for real-time data: RTP/IPv6 or for data distribution RMTP/IPv6 transfer) |  |

### Teleworking

Teleworking (telecommuting) service recently gets an increased interest. There is observed an increasing number of teleworking trials and experiments. The European Commission actively supports R&D projects related to teleworking.  The economic and environmental benefits of new work regime are very well known and widely accepted. Teleworking has become the reality as a result of advantages in computer and communications technology of recent years. The service of teleworking has been recognized as a key service for multifunction Service Provider System.

The teleworking service is known also as telecommuting, multimedia collaboration, telecooperation, CSCW (Computer Supported Co-operative Work). DAVIC 1.4 specification uses the term of teleworking. The service is described as a complex multimedia communication session with end user (teleworker), service provider and one collaborator involved [2]. The user establishes a session via a service provider.  He activates and controls local and distant applications and communicates with a collaborator (through audio, video and data).

#### Functional specification

The deliverables of EURESCOM P714 Project contain the list of tasks that are suitable for teleworking. The list is completed and modified with regard to personal experiences of authors and results of some teleworking trials. Suitable tasks to be performed at home by teleworkers can be grouped into the following categories:

Consulting/Answering E-mail

Reading and writing of e-mail messages forms an important part of today's work. The messages can be sent out for discussions, for the answering questions, demanding information, etc. An e-mail client and a connection to the corporate e-mail server are needed to access the mail system from home. The activity can be supported by any Service Provider System  supporting Internet Access service. Important issue is to guarantee appropriate security of the system.

Writing of reports and presentations

A major part of everyday work can be done at home with a computer and the standard software packages which are used in the company (word processor, spreadsheet, presentation software, statistical packages, etc.). A connection to the office LAN is essential. Security issues are important.

Finding and reading information

People need to collect documents, books and information from databases, libraries, the World Wide Web, etc. A lot of information is available in electronic form which can be retrieved, printed and studied at home.  Information from the Web services is very important. WWW browser and an Internet connection has to be provided.

Answering and making telephone / videophone calls

Answering and making telephone or videophone calls should not pose any technical limitations on telecommuters. All what is needed is a telephone or videophone with a call-forwarding facility in the office.

Contacts with colleagues

Personal contact with colleagues is an important part of the work. The contact is useful for information access, discussions about certain topics, management of people, social contacts, etc. These contacts often require physical presence, but sometimes they can be realized remotely by using e-mail and telephone.

Attending meetings

Meetings are one of the main reasons for which people are required in the office. However, it is felt that certain meetings could be handled remotely, in particular if videoconference service is available.

List of end-user functions necessary to perform specified actions follows.

- Establish session via service provider.

- Send ID, PIN for identification and access permission.

- Select collaborator.

- Send message to bulletin/message board.

- Send mail with MM attachments to correspondent.

- Activate/deactivate video information (at front-end or far-end).

- Activate shared application at front-end, at service provider, or far-end

- Common application overview at far end site(s)- without any control (not synchronized)- only with pointer control- with full control (scrolling, sizing, moving) (synchronized)

- Common edition of shared application- Send/accept edit request(s) - Synchronize application control and output management (may also be Service Provider)- Request, grant or deny control of shared application.

- Send/ accept electronic signatures.

- Remote LAN functionality, rapid access to remotely stored data.

- Have access to on-line services (Internet).

- Have integrated access to telecommunication services.

- Receive mail announcements.

- Receive mail.

- Read bulletin board.

Service Provider Functions:

- Provide directory service.

- Prove bulletin/message board.

- Accept session scheduling.

- Accept and distribute MM-mail.

- Establish collaboration session with requested collaborator(s).

- Manage and control collaboration session.

- Synchronize application control and output management.

- Secure access and session.

- Allow on-line subscription.

- Enable and control signature commutation.

- Send billing information.

- Provide access to telecommunication services and on-line services.

- Mail announcement to end user.

#### Technical requirements

The teleworking service can be implemented as a composite service, it is a subscription to the teleworking service should corresponds to the subscription to the predefined set of services necessary to perform all specified teleworking tasks and activities. The telework session should be started with high throughput, low delay Internet connection. After the selection of the collaborative the access to the remote LAN and e-mail service should be granted. The access to the POTS, videotelephony and/or videoconference service should be also guaranteed. Technical requirements for Teleworking service corresponds to the technical requirements for POTS, videotelephony, videoconferencing, Internet access and possibly others services.

### Distance learning

This application aims at providing ‘the virtual classroom’ regardless of the whereabouts of the instructor or students. During a class, the audio and visual communication between lecture participants (teacher and students) is provided, and some other functions are available. These include file exchange, document sharing, and shared whiteboard. Lecture participants are  students and the teacher. The teacher can establish virtual class, end class and has a control over who can join the class and what is displayed on student’s monitors. Students can interact with the teacher and communicate with other students. This service may incorporate videoconference service, movies on demand service and other. It is a composed service.

#### Functional specification

Virtual classes are based on videoconferencing groups. Session is point-to-multipoint, but can be switched to multipoint-to-multipoint, over the session manager control. It may also be possible to set-up point-to-point communication channels between the session manager (the teacher) and the selected user. Advanced systems may allow to set-up point-to-point connections between session participants (students/pupils). In each case the session manager should have the full control over the session, and can grant or deny the channel set-up request.

The session manager (teacher) is the one who has the full control over the session and content distribution. Point-to-multipoint, and multipoint-to-multipoint communication is managed by a modified videoconferencing Multipoint Control Unit (MCU) extended with a control interface to the Movies on Demand system.

The functional specification of the Videoconferencing Service as defined in ITU-T recommendations is very rich, and extends far beyond the common understanding of the service. Among others, it allows shared workspace, application sharing, data exchange and others. For this reason the videoconferencing MCU and distance learning MCU may be the same, if the two incorporate service-specific functions of Distance Learning.

The whiteboard, as defined in rec. T.126 uses a protocol supporting the management of common visual spaces and the multipoint exchange of graphical information directed to these spaces including images, pointers and drawing elements (points, lines, polygons and ellipses). Support for rendering out-of-band video streams within T.126 workspaces is also included. In addition, keyboard and pointing device exchanges are specified to support basic user interaction. Protocol elements for creating and referencing archived visual spaces are defined to allow pre-stored or pre-distributed graphical materials to be referenced.

Application Sharing (AS) is a process whereby two or more terminals cooperate to share the output of applications running on one or more terminals to the other terminals and to provide input for  these applications.

The AS protocol enables application sharing by allowing to advertise a view of a shared application within a session. Each site can, under specified conditions, take control of the shared computer application by sending remote keyboard and pointing device information. This style of application sharing does not require and does not make provision for synchronizing multiple instances of the same computer application running at multiple sites. Instead, it enables remote viewing and control of a single application instance to provide the illusion that the application is running locally.

The Distance Learning application concentrates much of the functionality in the client part. The client part of the application is responsible for content preparation, processing and performing management (e.g. application sharing). Among the unique features of the Distance Learning service is the presence of the MoD module used to play high quality video based educational material.

List of end-user (common to teacher and students) functions:

- Enable to subscribe to service and define user profile.

- Send data (audio, video, text and still pictures) to another users.

- Provide visual communication between session participants.

- To browse/view teaching materials (data).

- Receive data from another users.

- Share applications between users.

- To view data concerning the service and the relationship between user and service provider (such as usage, billing and account details).

- To perform action related to subscription, e.g. to cancel the subscription, to give permission for payment (credit card, money transfer).

End-user functions specific to the teacher:

- Establish a new classroom session.

- End a classroom session.

- Accept or reject requests to join the classroom.

- Control what is displayed on student’s displays.

- Initiate distribution or dispatch of teaching materials (data) to students.

- Initiate/pause/stop/resume data streaming to students

End-user functions specific to the students:

- Join a classroom.

- Leave a classroom.

- Send ID, PIN for identification and access permission

Service Provider Functions

- Allow an end user to subscribe to services

- Perform session management.

- Support application/data sharing.

- Enable visual communication between session users.

- Transports audio/video/data to user.

- Simultaneous delivery of different media types.

- Transports control signals to and from user.

- Can support billing by generating charge records.

- Establish connection links to requested users

- Control connection during session

- Provide billing information

#### Technical requirements

Technical requirements of the Distance Learning service are summarized in Table 13.

- Table 13: Technical requirements of the Distance Learning service

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | multipoint-to-multipoint | The session comprises of a single server and two or more users. |
| Session control | centralised, by session manager | Session manager – teacher – has full control over the session |
| Session duration | 5 min.  90 min. |  |
| Interactivity level | real-time |  |
| System response time | very high | System response time less than 250 ms[2] |
| Media delivery | continuous media |  |
| Aggregate flow | audio – symmetric video – asymmetric data – asymmetric | Dialog with frequent pass-over. One flow upstream, several flows downstream |
| Data flow bandwidth | video(1): n * 64 kbps per single video stream video(2): up to 4 Mbit/s audio(1): codec dependant audio(2): 128256 Kbps data: undefined | n is in the range 4  30. For video education material 64 Kbps max. for audio (1) 256 Kbps max. for audio (2) depending on the audio coding scheme: layer 1, layer 2, layer 3. |
| Data flow direction | Two-way |  |
| Data flow encoding | video(1): H.261, H.263 video(2): H.262 audio(1): G. 7xx, audio (2): ISO 13818-2 | - - for basic communication - - used when MoD service module is used for lectures |

### Tele-shopping

Tele-shopping allows the user to browse video catalogues or virtual shops to purchase products and services. The user may select items to get more information that may be presented using many different media, e.g. video, text, motion video with audio, audio or graphics (still or animated). After the user has selected a product, he may order the product. Once the product has been ordered, the method of delivery depends on the service provider’s implementation and user agreements.

#### Functional specifications

The following parties are involved in Tele-shopping:

- The end user uses the application and possibly purchases goods,

- A VASP (Value Added Service Provider) provides this service,

- A content provider may provide media content delivered during the use of the application,

- A second VASP may provide the Back-End of the application.

List of end-user functions:

- Move through the shopping environment.

- Select items of interest.

- Receive pictures of items.

- Receive text describing the items.

- Receive audio describing items.

- Receive motion video (with or without audio) describing items.

- Receive graphics (still and animated) describing items.

- Talk to a real sales person (audio only or audio video), in function of the context of the application (for future consideration).

- Control media clips, including repeat, pause, and abort (note: since media clips are of short duration it is not required to have fast forward/rewind type functions although they may be provided).

- Authorize payment / purchase of goods.

- Inquire about and alter  previous purchase (orders) including requesting exchange / return authorization.

- Being able to make a hard copy.

- Reserve products/services.

- Select payment method.

List of Service Provider Functions:

- Provide the shopping environment.

- Request media clips to be sent to the user.

- Send media clips to the user.

- Process user’s order.

- Keep an intermediate list of acquired items.

List of Content Provider Functions:

- Provide media clips for products.

- Provide information about price, availability, delivery times, special conditions.

- Categorization of material for electronic selection.

- Determine layout of virtual store.

- Assign products to virtual departments.

List of Network Provider Functions:

- Transport various data formats down to the user including: motion video, still pictures, audio, text and graphics.

- Transport information from the content providers or service providers to the server, in order to have rapid updates on product information

- Allow for the dynamic addition / deletion of connections between the end user and additional servers (i.e., if the user "clicks" on an item that has a video clip, then a video "pipe" must be set-up to the user).

#### Technical requirements

Technical requirements of the tele-shopping services can be categorized into two subgroups. The first one concerns presentation and selection of the products. This task can be done with the use of WWW service. Technical requirements for WWW access service are presented in Table 4. The process of product ordering and payment services have to be considered as a sub-service with special attention paid to the security issues. Details of technical requirements for this sub-service category are summarized in Table 14.

- Table 14: Technical requirements for tele-shopping services – product ordering and payment services

| Parameter | Value or range | Comments |
| --- | --- | --- |
| Session type | point-to-point | The session is set-up between a single server and a single user. |
| Session control | centralised, by user | The user has full control over the session |
| Session duration | 3 min. 15 min. |  |
| Interactivity level | High |  |
| System response time | High |  |
| Default access mode | Protected | Password protected, electronic signature |
| Media delivery | Undefined | Non continuous media (download) |
| Aggregate flow | Asymmetric | Higher bandwidth downstream small bandwidth upstream |
| Data flow bandwidth | less than 64 kbps |  |
| Data flow direction | two way |  |
| Data flow encoding | SMTP, text, X.400, X.500, X.435, UN/EDIFACT | All encoding converted to plain (optionally ciphered) text before exchange |
| Control flow encoding | POP2, POP3, IMAP, MAPI |  |
| Networking protocol | TCP/IP |  |

Tele-shopping service is based on a few generic functions: Internet Access, Virtual CD-ROM (Data Retrieval) and (optionally) Videotelephony. Session type is point to point. Issues related to Internet Access architectures were discussed before.

Tele-shopping can be implemented in a wide range of complexity. All user actions (selections and movement through the shopping environment) should be acknowledged immediately. Many different multimedia objects with different data size must be sent dynamically downstream. A single bi-directional, symmetric channel should remain during the entire session.  This channel is expected to have bursty traffic characteristics.

## Definition of Application Services

Application defined and characterized in this deliverable has been classified into two, very general categories. The classification has been done based on the technical requirements determined for given applications. The first category of applications are based on the information sent to user in a streaming manner. The source continuously produces data stream during the end-user session. The source is bursty, mostly due to the adaptive nature of video coding  scheme. The application classified into the Streaming Service is sensitive or highly sensitive to the delay introduced by the transport network.

The other group of applications are characterized as a much less delay sensitive. This group has been assigned to the Elastic Service. Data bit rate variation of this group is much higher in comparison to the applications of Streaming Service. Elastic Service applications are in most cases based on information retrieval. Next parameter which differentiate Streaming and Elastic services is delay sensitivity.

The application classification depends in some cases on the content of the service is offering. For example News on Demand application categorized into Elastic Service group can be moved to the Streaming Service class in the case of on-line financial markets tracking. Tele-shopping service can be redefined with on-line auction systems taken into account.  It is also possible to assign some Streaming Service applications to the Elastic Service group by the specific modification of the application definition.

## Requirements of Application Services on Transport Services

Two parameters should be defined to identify an application services scenario: the bandwidth requirements and the service’s degree of penetration in terms of percentage spread across customers for each type of service. The second parameter is the percentage of customers who subscribe the service, and generally it is obtained through market surveys.

As an example, urban network areas could be classified as follows:

1.	Minor urban areas (less than 50,000 inhabitants)

¨	Industrial sub-area

¨	Rural sub-area

¨	Residential sub-area

¨	Service sector sub-area

¨	Tourist sub-area

2.	Intermediate urban areas (50,000 to 200,000 inhabitants)

¨	Residential-housing sub-area

¨	Industrial sub-area

¨	Service sector sub-area

3.	Major urban areas (over 200,000 inhabitants)

¨	Residential sub-area

¨	Business sub-area

¨	Industrial sub-area

¨	Service sector sub-area

It should be noted that a regional or metropolitan network covering a quite large territory will contain all three types of “urban area”, and as such it will include all of the “areas” previously listed.

For each “area” and “service”, it is necessary to estimate the percentage of customers subscribing for the service, i.e. the service spread. These values should be used then to define the short, medium and long term scenarios.

Figure 3 - Example of Service penetration

## Conclusion

Applications described and analyzed in this document are summarized in the Table 15. The assignment of given application to the Streaming or Elastic Transport Service group is also indicated in the following table. Applications are summarized first of all with indication of the data bit rate specific to the service and level of expected data rate variation. Application delay sensitivity has been indicated with the grade ranging form 0 up to 5. The highest grade means that the application is highly delay sensitive, the grade o 0 would mean that the delay in the transport network can be neglected. The same rating has been used for need for protection. Each of the applications has been also analyzed with regard to its expected importance in the short and mid-term perspective. It was assumed that short-term prognosis means prognosis in three years perspective and mid-term forecast relates to the application importance in 10 years. The importance of the application is give in the scale from –5 to +5. The rate of 0 means no changes in the applications importance. The negative ratings is adequate to forecasted degradation of the application, the positive value corresponds to increased importance of the service. The aggregate rating of  importance prognosis is higher then zero due to the belief of continuing increase of networked services. The applications were analyzed with regard to the fiber optic based transport networks. Degradation of the TV broadcast services are resulted from belief of fast expansion of digital TV systems based on satellite links.

- Table 15: Application overview and its relationship to the Application Services

|  | Application: | Bit rate | Bit rate variation | Delay sensitivity | Need for protection | Importance |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Short-term | Mid-term |
| Streaming Service | POTS | 64 Kbps–32Kbps | Const. | 5 | 5 | -1 | -3 |
|  | VoIP | 8 Kbps–32 Kbps | Const. | 5 | 5 | 3 | 5 |
|  | Video-telephony | 256Kbps–1920Kbps | High | 5 | 5 | 2 | 3 |
|  | Video-conferenence | 256 Kbps at least | High | 5 | 5 | 3 | 4 |
|  | Tele-working | 64 Kbps–2 Mbps | Very high | 5 | 4 | 3 | 5 |
|  | TV Broadcast | 2 Mbps–8 Mbps | High | 4 | 4 | 1 | 0 |
|  | Delayed TV Broadcast | 2 Mbps–8 Mbps | High | 4 | 4 | 1 | 0 |
|  | Distance Learning | 64 Kbps–2 Mbps | Very high | 5 | 5 | 2 | 4 |
|  | MoD | 750 Kbps–4Mbps | High | 4 | 3 | 1 | 3 |
| Elastic Service | News on Demand | 64 Kbps | Very high | 2 | 2 | 2 | 4 |
|  | Internet Access | 64 Kbps–2 Mbps | Very high | 1 | 2 | 5 | 5 |
|  | TV Listing | 64 Kbps | Very high | 2 | 2 | 1 | 2 |
|  | Tele-shopping | 64 Kbps–2 Mbps | Very high | 2 | 2 | 5 | 5 |

Categorization of most common applications into two groups of transport services: Streaming Service and Elastic Service has been done in order to generalization of modern telecommunication services market and derivation of their requirements on future Optical transport Network.

Most of the applications were find to be appropriate to the Streaming Service. The importance of the streaming services probably will increase in the future. ATM LAN Market Analysis: Quantitative Market Research report prepared for the ATM Forum by Sage Research presents the forecast of delay-sensitive traffic volume increase from 17% in 1997 up to 28% in 1999. The forecast has not been verified (nor data are accessible). Own analysis performed for this deliverable shown that the trend probably will observed and even intensified in the future. The basis of this prognosis is active support of services like teleworking and distance learning from many governments and organizations (including EU)  and increased interest in these services from end-users.

Optical Transport Network has to be designed in a way enabling easy and reliable transport of this type of data. Data bit rate of streaming applications vary from 8 Kbps up to the 8 Mbps. The data bit rate required in the transport network by specific application vary with coding scheme applied to the incoming stream (video, audio stream). Notable progress in the video coding is still observable. New, faster and more effective codecs are available at the market. Application of new coding techniques probably will result in decreasing requirements for data rate. To forecast required throughputs complex techno-economic studies would be necessary. This is far away from the scope of the LION Project.

Streaming Service data is highly sensitive to the networking delay. Even in the case of TV like services short system (video server) response time in necessary, mostly due to the requirements of assuring VCR like functions  (freeze, stop, forward, etc). The value of 250 ms is the most common in the subjective literature.

Need for protection for given applications varies with the type of end-user. Transport of digital video signal between movies server and end-user and between TV studios is characterized with  different protection requirements. The most important conclusion is that Streaming Service need to be supported with protection mechanism of future transport network.

Forecasting of importance of the given applications are very difficult. Presented rating was based on available market research reports and discussed in the team working in LION project. The assigned importance of the applications is highly reliable to its definition. Presentation of application definitions were one of the goals of this deliverable.  Importance rating will be discussed during project duration. Change of the application importance rating at the beginning and at the end of project was found to be the most reliable and interesting parameter of applications analysis.

Some applications have been assigned to the Elastic Service. Data bit rate of this application group vary from 64 kbps up to 2 Mbps. Data bit rate variation is very high. This effect is observed because Elastic Service applications are in most cases based on information retrieval. The requested information is gathered by the end user (session is seen as an active one) and then some period of gathered data analysis can be noticed (at the transport network session seen as an inactive).

Elastic Service is not as highly as Streaming Service delay sensitive. Probably this parameter will have to be modified in the near future because of the possible change in the end users expectations. WWW service is likely to evolve toward fast, on-line services with no delay toleration.

The work done on the applications analysis should be continued. There is necessary to carry out the quantitative analysis of most likely applications in order to provide WP3 with clear view what share bandwidth will be consumed by Streaming Service. At this moment simple extrapolation of available data shows that streaming services will take about 46% of bandwidth of future transport networks.

# Client Interface Types and Client Formats

## Classic SDH

In the following sections the most important features of classic SDH services and interfaces are depicted, for further details refer to Appendix Error: Reference source not found.

### Description of Classical SDH Services

##### General

SDH is a managed transport network that consists of a Synchronous Transport Module of order N (STM-N) plus higher and lower order path layer. The STM-N is further subdivided into 3 section layers, electrical or optical section layer, regenerator and multiplex section layer. The bit rates of the SDH entities are summarized in table 16. In addition, a number of VCs could be virtual concatenated (inverse multiplexing) in order to form a signal of a higher bit rate.

Figure 4: Relation of SDH section and path layers

| STM-1 | STM-4 | STM-16 | STM-64 | STM-256 |
| --- | --- | --- | --- | --- |
| 155.52 Mbit/s | 622.08 Mbit/s | 2.488 Gbit/s | 9.953 Gbit/s | 39.813 Gbit/s |

Table 16: Overview of STM-N bit rates

|  | VC-4 | VC-4-4c | VC-4-16c | VC-4-64c | VC-256c |
| --- | --- | --- | --- | --- | --- |
| VC size | 150.336 Mbit/s | 601.344 Mbit/s | 2.405 Gbit/s | 9.622 Gbit/s | 38.486 Gbit/s |
| payload size | 149.790 Mbit/s | 599.040 Mbit/s | 2.396 Gbit/s | 9.585 Gbit/s | 38.339 Gbit/s |

Table 17: Overview of higher order VC bit rates

|  | VC-12 | VC-2 | VC-3 |
| --- | --- | --- | --- |
| VC size | 2.240 Mbit/s | 6.848 Mbit/s | 49.536 Mbit/s |
| payload size | 2.176 Mbit/s | 6.784 Mbit/s | 48.960 Mbit/s |

Table 18: Overview of lower order VC bit rates

##### SDH services for customers

SDH provides transport and mappings for clients and transport of higher and lower order VCs. Transport of SDH multiplex section signals over SDH regenerators is possible but hardly used, since the management of customer and operator network require in practice the resources of both regenerator and multiplex section layer.

##### Timing, jitter and wander

SDH is a synchronous network, i.e. the timing of each SDH NE is locked to a Primary Reference Clock. The timing information is distributed via the STM-N signal and shall, therefore, meet the requirements of jitter and wander. The VCs could transport data or PDH signals. The latter is also sensitive to jitter and wander and, therefore, the phase jumps of the VCs shall not exceed the defined limits.

##### Supervision

SDH provides functions for continuity, connectivity and quality supervision for:

- STM-N,

- higher order VCs,

- lower order VCs and

- subnetwork connections of higher and lower order VCs.

In addition, an OAM signal (AIS) is defined for fault isolation. A NE, which detects a defect inserts downstream AIS, i.e. a primary defect is detected only once.

##### Protection

SDH provides linear protection for:

- Multiplex Section (1+1, 1:1 and 1:n)

- higher and lower order VCs (1+1 and 1:1)

and Multiplex Section Shared Protection Ring (self-healing rings)

The switching criterion could be a defect or quality degradation, which depends on the protection scheme. The switching time from working to protection path is  50 ms, while the detection time depends on the switching criterion.

### Parameters of Interfaces

#### Physical Layer

Classical SDH interfaces are based on one of the following physical section layers:

- STM-1 electrical and optical interface according to Recommendations G.703 and G.957 [27, 28];

- STM-4, -16 optical interface according to Recommendation G.957 [28];

- STM-64, -256 optical interface according to Recommendation G691 [29];

The optical interfaces are classified as intra-office, short-haul, long-haul, very-long-haul and ultra-long-haul with respect to different values of launched power and range. Intra-office and short-haul interfaces operate in 850 nm and 1300 nm wavelength range whereas long-haul interfaces use either the 1300 nm or the 1550 nm optical window.

#### Overhead Functions

Section- and Path Layer overheads are defined in Recommendation G.707 [30] and support the function listed in subsection 3.1.1.

### Requirements on Transport Service

The OTN transport service for STM-N signals has to meet the following requirements:

- Transparent transport of the binary content of the STM-N signal;

- Transfer of the timing associated to the STM-N signal;

- Adaptation of OTN FDI to SDH client layer in the direction from OTN server to SDH client

## SDH frame only

This term is often used for high bitrate interfaces that present a single contiguous concatenated VC-4-Xc. As such it is one specific SDH solution which is covered by the description of Classic SDH in section 3.1.

## Ethernet

Ethernet accounts for over 85% of LANs world-wide. The Gigabit Ethernet standard can be used to extend high-capacity LANs to MANs and maybe even WANs, using Gigabit line-cards on IP routers, which can cost 5 times less than SDH line-cards with similar capacity. For this reason, Gigabit Ethernet could be a very attractive means to transport IP.

Lower bit rate Ethernet networks (e.g. 10Base-T or 100Base-T) have been used a lot in a half-duplex mode, where the bandwidth available for transmission is shared between all users, and between both directions of transmission. To police access to the shared bandwidth, CSMA-CD is used. This imposes limits on the physical size of the network, where the transit time cannot exceed the “slot time” which is the minimum frame length (512 bits for 10Base-T and 100Base-T). For a bit rate of 1Gb/s, using a minimum frame length of 512 bits would imply an Ethernet network only roughly 10m long, and for this reason the minimum frame length has been redefined to be 4096 bits for Gigabit Ethernet. However, this still limits the network size to 100m, so full-duplex mode is more attractive when using Gigabit Ethernet.

When Gigabit Ethernet (1000Base-X) is used in full-duplex mode, it becomes simply an encapsulation and framing method for IP packets, and the CSMA-CD functionality is not used. Ethernet switches can also be used to extend the network topology beyond a point-to-point link.

The frame structure for Gigabit Ethernet is shown in Figure 5. 1 The maximum payload length of Gigabit Ethernet is 1500 bytes, but the possibility to extend this to 9000 Octets (Jumbo Frames) is discussed. To use larger payload size would however be to give up the compatibility with previous Ethernet standards, and the subject is under debate and no standard is presently available.

Before the Destination Address (DA) a preamble of 8 bytes is sent for synchronization. The free space between two Ethernet packets is filled with IPG (idle intervals)  also for synchronization purposes.  As seen in Figure 5, after the Destination Address, one finds the Source Address (SA) followed by the length/type indication. The Data has a minimum size of 46 bytes and a maximum size of 1500 bytes and it is located  between the Length/Type indication and the Frame Check Sequence (FCS).  The Frame  Check Sequence provides the error indication by using a Cyclic Redundancy Check (CRC) algorithm.

Figure 5: Gigabit Ethernet Frame

The Ethernet frame is encoded onto an optical carrier using 8B/10B encoding. For a 1Gb/s throughput, the line rate is therefore 1.25Gb/s. The encoding also ensures that idle periods when packets are not being transmitted, are filled with symbols having a sufficient density of transitions between 1 and 0 logical states to ensure that clock recovery is maintained.

Gigabit Ethernet provides some support of CoS as defined in the standards IEEE 802.1Q and 802.1p. These standards facilitate CoS over Ethernet by providing a means for “tagging” packets with an indication of the priority or class of service desired for the packet. These tags allow applications to communicate the priority of packets to internetworking devices. RSVP or DiffServ support may also be achieved by mapping into 802.1p service classes.

### The Gigabit Ethernet Standard—IEEE 802.3z

To recap the recent history of the Gigabit Ethernet standards process, in July, 1996, after months of initial feasibility studies, the IEEE 802.3 working group created the 802.3z Gigabit Ethernet task force. The key objectives of the 802.3z Gigabit Ethernet task force were to develop a Gigabit Ethernet standard that does the following:

- Allows half- and full-duplex operation at speeds of 1000 Mbps

- Uses the 802.3 Ethernet frame format

- Uses the CSMA/CD access method with support for one repeater per collision domain

- Addresses backward compatibility with 10BASE-T and 100BASE-T technologies

The task force identified three specific objectives for link distances: a multimode fiber-optic link with a maximum length of 550 meters; a single-mode fiber-optic link with a maximum length of 3 kilometers (later extended to 5 kilometers); and a copper based link with a maximum length of at least 25 meters. The IEEE is also actively investigating technology that would support link distances of at least 100 meters over Category 5 unshielded twisted pair (UTP) wiring.

### Full- and Half-Duplex

Gigabit Ethernet supports new full-duplex operating modes for switch-to-switch and switch-to-end-station connections, and half-duplex operating modes for shared connections using repeaters and the CSMA/CD access method. Initially operating over optical fiber, Gigabit Ethernet will also be able to use Category 5 UTP cabling.

Much of the effort of the IEEE 802.3z task force was devoted to the definition of tranceiver or PHY standards for Gigabit Ethernet. Like other standards based on the International Standards Organization (ISO) model, Gigabit Ethernet implements functionality adhering to a physical layer standard. In general, the PHY or physical layer is responsible for defining the mechanical, electrical and procedural characteristics for establishing, maintaining and deactivating the physical link between network devices. For Gigabit Ethernet communications, several physical layer standards are emerging from the IEEE 802.3z effort.

Two PHYs provide Gigabit transmission over fiber-optic cabling. 1000BASE-SX is targeted at lowest cost multimode fiber runs in horizontal and shorter backbone applications. 1000BASE-LX is targeted at longer multimode building fiber backbones and single-mode campus backbones. For multimode fiber, these standards define gigabit transmission over maximum distances ranging from 220 to 550 meters. Single-mode fiber, which is covered by the long-wavelength standard, is defined to cover distances of 5 kilometers.

### The OSI layering Model and Gigabit Ethernet

Gigabit Ethernet is located in two OSI layers: the data link layer and  the physical layer, as shown in Figure 6.

Figure 6: Architectural position of Gigabit Ethernet with its different sublayers

The Physical Coding Sublayer (PCS) encodes and decodes the transmission data in an 8B/10B coding scheme based on the Fiber Channel protocol. An 8 bits group of data is encoded in a 10 bits group.  The coding helps the synchronization of the receiver timing clock. The extra capacity can be used for signal control instruction between devices. The PMA (physical media attachment) serializes and deserializes the data bits and the PMD (physical media dependent) performs alignment of the data stream.

Table 19: Physical Properties of Gigabit Ethernet

| Client | Classification | Datarate | Standard | Distance | Code | NTR |
| --- | --- | --- | --- | --- | --- | --- |
| Ethernet |  |  |  |  | Manchester Codierung |  |
|  | 1Base-5 | 1 Mbit/s | IEEE 802.3 | 500 m STP | " |  |
|  | 10Base-2 (Cheapernet) | 10 MBit/s | IEEE 802.3 | 185 m Koax RG-58 | " |  |
|  | 10Base-5 | 10 MBit/s | IEEE 802.3 | max. 500 m RG-58 Kabel | " |  |
|  | 10Base-T | 10 MBit/s | IEEE 802.3 | 100 m STP; UTP | " |  |
|  | 10Base-F | 10 MBit/s | IEEE 802.3 ISO/IEC8802-3 | 4500 m Netzwerk 2000 m MMF(FA) 500 m MMF(FP) | " |  |
|  | 10Base-FB | 10 MBit/s |  | Max. Linksegment 2000 m MMF 25 km SMF (vollduplex) | " |  |
|  | Switched Ethernet | n x 10 Mbit/s | kein Standard, IEEE 802.3 kompatibel |  | " |  |
|  | Full Duplex Ethernet | 20 Mbit/s | kein Standard, IEEE 802.3 Datenformat |  | " |  |
|  | Isochrones Ethernet | 10 + 6.144 Mbit/s | IEEE 802.9 |  | 4B/5B |  |
| Client | class | Data rates | Standard | Distance | Coding | NTN |
| Fast Ethernet | 100 Base-T | 100 Mbit/s | IEEE 802.30 |  |  |  |
|  | 100 Base-TX | " | " | 200 m UTP (Kat. 5) oder STP | MLT-3; 4B/5B |  |
|  | 100 Base-T2 | " | " | 200 m 4 x UTP (Kat.3) |  |  |
|  | 100 Base-T4 | " | " | 100 m 8 x UTP (Kat.3) | 8B/6T |  |
|  | 100 Base-FX | " | ISO 9324-3 (FDDI-Standard) | 400 m MMF 2 km MMF bei Verbindung zwischen Brücken oder Switches 20 km SMF (vollduplex) | 4B/5B/NRZI |  |
|  | 100 Base-VG - auch VG-Anylan (nur Hewlett-Packard) | 100 Mbit/s 4 Leitungspaare mit jeweils 25 Mbit/s (künftig 400 / 800 Mbit/s) | IEEE 802.12 | 100 m UTP | 4B/5B/NRZI (5B/6B NRZ) |  |
| Gigabit-Ethernet |  | 1000 Mbit/s | IEEE 802.3Z IEEE 802.3AB |  |  | Ja |
|  | 1000 Base-LX (1300 nm) | Interoperate | " | 550 m MMF 5 km SMF (bei Full Duplex) | 4B/5B 8B/10B | Ja |
|  | 1000 Base-LH (LH für long houl) |  | Kein IEEE-Standard LX-Ausrüstung mit GBIC | Nur SMF 1 – 49 km bei 1300 nm 50 – 100 km bei 1550 nm |  | Ja |
|  | 1000 Base-SX (850 nm) |  |  | 270 m MMF 62.5 µm 550 m MMF 50 µm (bei Full Duplex) | 4B/5B 8B/10B | Ja |
|  | 1000 Base-CX |  |  | 25 m STP | 8B/10B | Ja |
|  | 1000 Base-T |  |  | 100 m Kabel Kat.5 | 1000 Base-T Kodierung | Ja |
|  | Full-Duplex-Gigabit-Ethernet |  |  |  |  | Ja |
| 10 Gigabit-Ethernet |  | 9.58 Gb/s | IEEE 802.3 Higher Speed Study Group | In Arbeit: 300 m short-haul 10 km long-haul |  | Ja |

## ESCON

### Protocol Characteristics

For data back-up and data storage in storage area networks a special data transmission protocol exists. Its name is ESCON (Enterprise System CONnection). Usually data will be transmitted from the master disc to the slave disc via optical multimode fibres. The data rate is 200 Mbit/s. Data from the master disc will be mirrored on the slave disc. For transmission the data from the master will be devided into a sequence of packets and will be sent to the slave. After each sequence of data packets a checkbyte will be sent to the slave and the data stream from the master will be interrupted. If the checkbyte of slave and master are identical the slave sends a corresponding word to the master and the next sequence of data packets will be sent. If there is a failure in the transmission link and/or bit errors appear the checkbytes are not identical. Then the master sends the failed data sequence again and the failed data sequence will be overwritten on the slave disc. This process will be repeated until the checkbytes are identical. That means an errorless data mirroring. The maximum transmission length between master and slave via optical multimode fibre is about 2.5 km. The distance can be extended by data transponse from multimode to singlemode fibre. But the longer the distance between master and slave the slower the total data recording rate due to time delay between data stream and answer of the slave limited by the velocity of light in the optical fibre.

### Physical Parameters of Interfaces

Table 20: Physical properties of ESCOn

| Client | Classification | Datarate | Standard | Distance | Code | NTR |
| --- | --- | --- | --- | --- | --- | --- |
| ESCON Enterprise System Connection/ SBCON Single Byte Command Code Sets Connection | ESCON/ SBCON | 200 Mbit/s 17 Mbyte/s 10 Mbyte/s | IBM/ ANSI | 60 km SMF 3 km MMF |  | Ja |
| FICON |  | 100 Mbyte/s |  | 20 km |  | Ja |

## Fibre Channel

### Protocol Characteristics

Fibre Channel is the general name of an integrated set of standards being developed by the American National Standards Institute (ANSI). It is designed to significantly improve the speed at which data is transmitted between workstations, mainframes, supercomputers, storage devices and displays, while providing one standard for networking, storage and data transfer. In general Fibre Channel provides speed, reliability, distance, and connectivity; all at a low cost. It guarantees bandwidth and delivery while being able to handle any protocol or topology.

Fibre Channel is designed to combine the best of both channel and network data communication. A channel is a direct or switched point-to-point connection. One benefit of a channel is that it is mostly hardware-intensive for speed and efficiency of data transport. This is compared to the higher overhead of network data transfer, which is slower because it is software-intensive. The benefit of networks is that they can handle a greater range of tasks because they operate in an environment of  unanticipated connections. Fiber Channel avoids the downside of network data transfer by providing a way to transfer data from a buffer at the source device to a buffer at the destination device. Fibre Channel does not need to know what the data is or how it is formatted. What the individual protocols do with the data before or after it is in the buffer is independent of the function of Fibre Channel. All a Fibre Channel port has to do is to manage a simple point-to-point connection between itself and the Fabric. A Fabric is a circuit switch, active hub, or a loop.

Fibre Channel is a switched medium that works similar to a telephone network: any user will have a temporary, direct connection that provides the option of the full bandwidth of the Fibre Channel as long as the connection is established. Fibre Channel's acknowledgment and flow control supports connection-less traffic by using time division multiplexing. Fibre Channel is designed to transport many protocols, such as FDDI, serial HIPPI , SCSI, IPI, and many more that will be listed in the section describing the FC-4 layer.

The connections provided for by Fibre Channel are:

- High performance workstations

- Intelligent mass-storage systems

- Heterogeneous workstation clustering

- Parallel processing

- Load leveling

- Host-to-Host

- Host-to-mass-storage

- Server-to-mass-storage

- server-to-server

- Bulk data transfer

- Multi-media

- Printers linked to a LAN without a cpu

- Disk drives linked to a LAN without a cpu

The transfer rates of Fibre Channel are currently (133 Mbps,266 Mbps,530 Mbps, and 1 Gbps).

However, data rates of 2 to 4 Gbps should be available soon. Fibre Channel will allow simultaneous transmission of different protocols over a single optical-fiber pair and it can allow a number of existing services, such as network, point-to-point, and peripheral interfaces, to be accessed over a single medium using the same hardware connection. Fibre Channel also provides control and complete error checking.

The Fibre Channel structure is defined as a multi-layered stack of functional levels, not unlike those used to represent network protocols, although not mapping directly to OSI layers. The layers of the Fibre Channel standard define the physical media and transmission rates, encoding scheme, framing protocol and flow control, common service, and the upper-level applications interfaces. The five layers are: FC-0,FC-1,FC-2,FC-3,FC-4

FC-0, the lowest layer, specifies the physical link in the system, including the media, transmitters, receivers, and connectors that can be used with Fibre Channel. This also includes electrical and optical characteristics, transmission rates, and other physical components of the standard. The physical level is designed to be able to use a large number of technologies to meet the widest range of system requirements.

FC-1 defines all transmission protocols including serial encoding and decoding rules, special characters and error control. Every 8 bits of data are encoded into a 10 bit Transmission Character. A Transmission Word is composed of four contiguous Transmission Characters. The transmission code is DC balanced and the Transmission character is used to ensure that clock recovery is possible by having enough transitions present in the serial bit stream.

FC-2 is called the signaling protocol level. It is responsible to break the data to be transmitted into frame size, and reassemble the frames after transport. It specifies the framing rules of the data to be sent from one port to another, ways for controlling the three service classes, and controlling the sequence in whichthe data is transferred. All frames in a transfer have a sequence number from 0 to N so that the receiver is able to tell if a frame is missing and exactly which frame(s) is missing. There are five different building blocks defined to provide efficient means of data transport across a link:

Ordered Sets: Ordered Sets are Transmission Words that allow bit or word synchronization and word boundary alignment.

Frame: The frames contain the data(payload), address of the source and destination ports, and the link control information. Each frame payload has a maximum 2112 byte long.

FC-3 is in the process of being defined for use by common services required for advanced features such as:

Striping: To multiply bandwidth

hunt groups: The ability for more than one port to respond to the same alias address

Multicast: Which delivers a single transmission to multiple destination ports

FC-4 This layer defines the application interfaces that can be run over Fibre Channel. Both network and channel protocols can concurrently run over the same physical interface providing seamless integration of standards. Current or proposed protocols are SCSI, IPI, HIPPI, IP, ATM (AAL5), SBCCS, IEEE 802.2.

To accommodate a range of communications needs, Fibre Channel defines three different classes of service. Each class can be utilized between as few as two points, and each class can be integrated within the framework of other network protocols.

Class 1 is used when the need is for dedicated circuit-switched connections. This service guarantees the maximum bandwidth between two N_Ports and is good for high-throughput transactions.

Class 2 is a frame-switched, connection-less service where the bandwidth can be shared by different.

sources by multiplexing frames. Frames may be delivered out of order, but this service is good when the connection set-up time is greater than the latency of a short message. This class also sends an acknowledgment to confirm frame delivery.

Class 3is the same as class 2 except that frame delivery is not confirmed. This is a datagram type of service that is good for real-time broadcasts.

## Other Formats

Table 21: Physical properties of other formats

| Client | Classification | Datarate | Standard | Distance | Code | NTR |
| --- | --- | --- | --- | --- | --- | --- |
| HIPPI 800 High Performance Parallel Interface |  | 800 Mbit/s (Duplex) 1600 Mbit/s (Simplex) | ANSI X3T9.3 | 25 m Twinax 2 km Gf | NRZ | Ja |
| Serial HIPPI (S-HIPPI) |  | 1.2 Gbit/s | ANSI | 10 km STP, Gf |  | Ja |
| GSN Gigabyte System Network | (auch Super HIPPI oder HIPPI 6400) | bis zu 6.4 Gbit/s (10 G – 1.6 G Fehlerkontrolle–2 G Codierung) | Entwicklung von Silicon Graphics | 40 m STP 300 m MMF 1 km SMF | 4B/5B | Ja |

# High Level Description of OTN-MCN related Transport Services

The transport services described in the following are based on the network architecture depicted in Figure 7.

Figure 7: Network architecture

Network transport services are those functions and utilities supporting connectivity, communications and control required by applications operating across the network applications. The definition of these network tranport services will determine the flexibility of an enabling infrastructure to support current and unforeseen new network application. The tight link between network infrastructure and transport services will benefit Network and Service Providers from the reduced complexity of the overall network environment. This complexity reduction could lead to simpler management and cheaper overall network costs.

Figure 8 - Example of Transport Services

## TDM-Transport Connection Service

### SDH Leased Line Services

SDH Leased Line Services provide for transport of bidirectional lower order and higher order VC. Interconnection takes place at STM-N interfaces. A subset of possible solutions is defined in EN 301 164 [40] and EN 301 165 [41].

##### Timing Tolerance

The leased line connection shall carry user timing with a tolerance of 4,6 ppm. It is recommended to use SDH synchronization as specified in ETS 300 462-2-1 [42]. The network termination point shall transmit a SDH section signal that could be used for generating the VC timing at the terminal equipment.

##### Transfer Delay

The one way end-to-end transfer delay of the connection mainly depends on geographical distance and the transmission media.

##### Jitter

In order to meet the jitter requirements the AU/TU pointers shall comply to the specification in EN 300 417-3-1 [38] and EN 300 417-4-1 [39] respectively.

The leased line shall operate as specified when the jitter at the STM-N interface is within the limits given in Recommendation G.825 [31].

##### Information Transfer Susceptance

The connection should be capable of transparently transferring a complete VC expect of the N1/N2 byte of the path overhead, which is used for Tandem Connection Monitoring application.

##### Error Performance

Measurement of the error performance of an SDH connection is based on the following parameters which are defined in Recommendation G.826 [32].

- Errored Block (EB):

- An EB is a block in which one or more bits are in error.

- Errored Second Ratio (ESR)

An errored second (ES) is a one second period with one or more errored blocks or at least one defect. The ESR is the ratio of ES to total seconds in available time during a fixed measurement interval.

- Severely Errored Second Ratio (SESR)

- A severely errored second (SES) is a one-second period which contains 30% errored blocks or at least one defect. SES is a subset of ES. The SESR is the ratio of SES to total seconds in available time during a fixed measurement interval.

- Background Block Error Ratio (BBER)

A background block error (BBE) is an errored block not occurring as part of an SES. The BBER is the ratio of  BBE to total blocks in available time during a fixed measurement interval. The count of total blocks excludes all blocks during SESs.

- Unavailable Seconds (UAS)

A period of unavailable time begins at the onset of ten consecutive SES events. These ten seconds are considered to be part of unavailable time. A new period of available time begins at the onset of ten consecutive non-SES events. These ten seconds are considered to be part of available time. A bidirectional path is in the unavailable state if either one or both directions are in the unavailable state.

Performance objectives are given in Recommendation G.828 [33].

##### Availability Performance

In Recommendation G.827 [34] the following availability performance parameters are defined:

- Availability Ratio

Availability ratio (AR) is defined as the proportion of time that a path element (PE) is in the available state during an observation period.

- Mean Time Between Digital Path Outages

The mean time between digital path outages (Mo) for a digital path portion is the average duration of any continuous interval during which the portion is available. The reciprocal of the Mo is called Outage Intensity (OI).

##### Provisioning Time

The time to set-up a VC connection (with existing infrastructure) is mainly determined through provider’s administrative processes. The duration of the provisioning action through the NMS is in the range of minutes depending on the size of the network.

#### Concatenation of VC-4

TDM multiplexing and TDM framing in new optical transport networks is focused on higher order SDH path layer (VC-4).  In order to achieve payload bandwidths larger then the basic VC-4 container the principle of concatenation has been defined. The following description of the concatenated formats is based on ITU-T G.707 [30].

Concatenation definition: A procedure whereby a multiplicity of Virtual Containers is associated one with another with the result that their combined capacity can be used as a single container across which bit sequence integrity is maintained

##### Concatenation of AU-4

AU-4s can be concatenated together to form an AU-4-Xc (X concatenated AU-4s) which can transport payloads requiring greater capacity than one Container-4 capacity. In principle it is distinguished between contiguous concatenation and virtual concatenation

##### Concatenation of contiguous AU-4s

A concatenation indication, used to show that the multi Container-4 payload carried in a single VC-4-Xc should be kept together, is contained in the AU-4 pointer. The capacity available for the mapping, the multi Container-4, is X times the capacity of the Container-4 (e.g. 599 040 Mbit/s for X = 4 and 2 396 160 kbit/s for X = 16). Columns 2 to X of the VC-4-Xc are specified as fixed stuff. The first column of the VC-4-Xc is used for the POH. The POH is assigned to the VC-4-Xc (e.g. the BIP-8 covers 261 X columns of the VC-4-Xc). The VC-4-Xc is illustrated in figure 9.

Figure 9:VC-4-Xc structure

##### Virtual concatenation of AU-4s

Up to now G.707 [30] has not specified the virtual concatenation of VC-4. However it is specified the  method for TU-2s, and the virtual concatenation of vc-4 will be based on the same basic principles.

Based on the specification in G.707 [30] this method of concatenation allows for the transport of a single VC-2-mc in m  TU-2 without the use of Concatenation Indication in the pointer bytes. The method only requires the path termination equipment to provide concatenation functionality.

Virtual concatenation requires the concatenated Tributary Unit signals at the origin of the path to be launched with the same pointer value. The so formed Tributary Units at each interface shall be kept in a single higher order VC-4.

When the higher order VC-4 is terminated, the restrictions that apply in passing the concatenated Tributary Units from one interface to another is that all of the concatenated Tributary Units are connected to a single higher order VC-4 and that the time sequencing of the concatenated Tributary Units is not altered.Differences in delay of the individual concatenated VC-2 signals may occur due to pointer processing at intermediate equipment. The maximum difference in pointer value within a concatenated group at any interface is for further study. At the path termination the VC-2-mc can be reconstructed by using the pointer values for alignment.

Each concatenated VC-2 signal will carry its own POH. At the VC-2-mc path termination, the individual BIP-2s are aggregated to give a single BIP error monitor.

### SDH VPN

SDH VPN is a service provided by the SDH TMN and does not require any equipment functions in addition to classical SDH equipment.

A VPN comprises a set resources to which authorized customers can apply special customer network management functions according to different authorization levels.

##### VPN Resources

- Ports/Interfaces of Network Elements

- Connections between Network Elements

- Possible Sub-VPN levels

##### CNM Functions/CNM Interface

- Configuration Management Functions

- Fault Management Functions

- Performance Management Functions

- CNM Security Functions

- Accounting Functions

### Basic Mapping of native Signals into TDM – Frame

Table 22: Mapping of Protocols

##### Example of application of concatenated VC-4 – mapping of ATM-cells

The mapping of ATM cells is performed by aligning the byte structure of every cell with the byte structure of the Virtual Container used including the concatenated structure (VC-x or VC-x-mc, x1). Since the relevant Container-x or Container-x-mc capacity may not be an integer multiple of the ATM cell length (53 bytes), a cell is allowed to cross the Container-x frame boundary.

The ATM cell information field (48 bytes) shall be scrambled before mapping into the VC-x or VC-x-mc. In the reverse operation, following termination of the VC-x or VC-x-mc signal, the ATM cell information field will be descrambled before being passed to the ATM layer. A self-synchronizing scrambler with generator polynomial x43 + 1 shall be used. The scrambler operates for the duration of the cell information field. During the 5-byte header the scrambler operation is suspended and the scrambler state retained. The first cell transmitted on start-up will be corrupted because the descrambler at the receiving end will not be synchronized to the transmitter scrambler. Cell information field scrambling is required to provide security against false cell delineation and cell information field replicating the STM-N frame alignment word.

When the VC-x or VC-x-mc is terminated, the cell must be recovered. The ATM cell header contains a Header Error Control (HEC) field which may be used in a similar way to a frame alignment word to achieve cell delineation. This HEC method uses the correlation between the header bits to be protected by the HEC (32 bits) and the control bit of the HEC (8 bits) introduced in the header after computation with a shortened cyclic code with generating polynomial g(x) = x8 + x2 + x + 1.

The remainder from this polynomial is then added to the fixed pattern "01010101" in order to improve the cell delineation performance. This method is similar to conventional frame alignment recovery where the alignment word is not fixed but varies from cell to cell.

More information on HEC cell delineation is given in Recommendation I.432 [43].

The ATM cell stream is mapped into a Container-4-Xc with its byte boundaries aligned with the Container-4-Xc byte boundaries. The Container-4-Xc is then mapped into VC-4-Xc together with the VC-4-Xc POH and (X1) columns of fixed stuff. The ATM cell boundaries are thus aligned with the VC-4-Xc byte boundaries. Since the Container-4-Xc capacity (X  2340 bytes) is not an integer multiple of the cell length (53 bytes), a cell may cross a Container-4-Xc frame boundary.

Figure 9: Mapping of ATM cells into VC-4-Xc

## Optical Channel Transport Service

An OTN is a transport network bounded by optical channel access points.

An OTN Transport Service could be defined as the capability of an OTN to transport client information between two optical channel access points with a given degree of transparency. The OTN transport service could be provided by means of wavelengths, optical channel connections with transponders and dark fibre. Wavelength and dark fibre transport services are not studied in this document.

Wavelength

Optical Channel Connection

An OTN should be capable of supporting the following types of optical channel connection:

- a permanent optical channel set up from the network management system by means of network management protocols;

- a soft permanent optical channel set up from the management system, which uses network generated signalling and routing protocols to establish connections;

- a switched optical channel which can be set up by the customer on demand using signalling and routing protocols.

A set of parameter could be used to qualify the Optical Channel Connection:

- access mode

- rate of service availability

- multiple service quality classes

- multiple service protection classes

- routing configuration (VPN)

- bandwidth management

- security

- provisioning time

### Digital Channel Transport Service – Digital Wrapper

#### Current definition of an ONNI

Draft Recommendation G.709 (April/2000) contains the basic information of an ONNI. Futher details will be specified within the year 2000. The text hereafter describes the draft ONNI specification and addresses, in addition, the ongoing discussion regarding a detail specification of the ONNI frame structure.

The Optical Transport Network as specified in ITU-T Recommendation G.872 defines two ONNI interface classes:

- Inter-Domain Interface (IrDI)

- Intra-Domain Interface (IaDI)

Figure 4-10 gives a possible network configuration to illustrate the location of OTM IrDI and OTM IaDI.

The Optical Transport Network will consist of a set of network operator specific networks, which are interconnected via OTM IrDIs (Figure 4-10). Each network operator specific network may consist of one or more manufacturer specific subnetworks. Those manufacturer specific subnetworks are interconnected via OTM IrDIs. OTN equipment within a manufacturer specific subnetwork may be interconnected via OTM IaDIs and OTM IrDIs.

Figure 4-10: Locations of OTM IrDI and IaDI

The IrDI is subject to standardization (see Figure 4-11). The IaDI is not subject to standardization, except for the properties necessary for network management. However it includes the ODUk as a standard information structure which is used through out the network in order to allow network wide interworking of optical channels.

NOTE – Any non-standard interface is defined as an IaDI. Any standardized interface is defined as an IrDI.

The Optical Transport Module-n (OTM-n) is the information structure used to support ONNI interfaces. Two OTM-n structures are specified per ONNI interface class:

- OTM-n (n1)

- OTM0

This results in four OTM-n interface structures: OTM-n IrDI, OTM0 IrDI, OTM-m IaDI, OTM0* IaDI. Definitions of the ONNI unit are given in .

Figure 4-11: Overview of the basic structure of standardiszed and propriety optical interfaces

The frame structure of the ODU/OTU bases on the Digital Wrapper (DW). The basic assignment of overhead functions to the frame is illustrated in Figure 4-12 and Figure 4-13. It is agreed to specify a 4 times 16 byte overhead plus a 4 times 256 bytes FEC field that may be unused or filled with RS239/255 FEC code. The size of the ODUk payload is not yet fixed. It depends on mapping scheme for STM-N (byte synchronous or byte asynchronous).

Figure 4-12: Frame synchronous mapping of ODUk into OTUk signal

Figure 4-13: OTUk and ODUk Overhead

##### Network functions provided by the DW

The DW provides functions for:

- Continuity supervision (loss of frame, AIS generation and detection for OTU and ODU).

- Connectivity supervision (trail trace for OTU and ODU).

- Quality supervision (BIP-8 and 4 bit REI for OTU and ODU).

- Transport for information for network operators.

The supervision functions listed under item a) to c) are provided for 8 nested levels and are called connection monitoring. The coding for each level is identical. One level is assigned for the end-to-end path and one for the OTU. Figure 4-14 shows 3 levels of connection monitoring of an ODU (OTU monitoring is not illustrated). One level is used by the user for end-to-end monitoring, one for the leased line (along domains A, B and C) and one for each domain. In addition, network operator B uses a non-intrusive monitor for reading the end-to-end quality and the leased line quality.

Figure 4-14: Example of nested connection monitoring

##### Relation between transparent domains and DW

As shown in Figure 4-11 the ONNI consists of a common part (ODU) and parts that may be standardized or propriety (OTU, OMS and OTS). An ODU can be transported over administrative domains while an OTU is a signal within or between administrative domains or transparent domains respectively. This is illustrated in Figure 4-15.

Figure 4-15: Relation between transparent domains, OTU and ODU

#### Future developments of the DW

The current specification of the ONNI include only a structure of a single ODU per OTU, i.e multiplexing of ODUs in order to increase the bit rate per wavelength is foreseen for the 2nd version of the ONNI. However, the automatic switched optical network (ASON) and Time Division Multiplexing (TDM) of DW signals influence the design of the OTN and, therefore, structure and bit rate of the DW.

First generation OTN NEs could be expected for year 2003. The availability of ASON switches and 2nd generation OTN NEs will not be available before year 2005. The first generation of ONNIs has to be specified such that it is compatibly with ASON and possible TDM in the OTN, i.e. the bit rate of OTUs should be independent on TDM.

##### Effect of ASON on OTN

The discussion on an ASON has just started. The relation between ASON and OTN is not yet defined nor which signal to be switched. It may be that ASON and OTN each uses a fibre exclusively or that the share a fibre. The latter case implies that that ASON switches uses transport capacities of the OTN. ASON will provide switched ODUs when it is intended to offer switch connections between any locations of the world. Switched OTUs can be only provided within a transparent domain and therefore cannot be used for a world-wide switching network.

It is assumed that ASON will offer switched ODUs. It is the purpose of switched network to provide quickly bandwidth with an appropriate granularity. On the other hand, it is the purpose of the OTN to design OTN spans and nodes for a maximum capacity or throughput respectively. Example: Switched ODU1s of 2,5 Gbit/s provide the smallest granularity for ASON while an OTN may be optimised for OTU2s (10 Gbit/s) or OTU3s (40 Gbit/s). Hence, the forthcoming ASON gives reason for the standardisation of ODU multiplexing in future.

##### TDM of ODUs

It is the intention of network operators to run the spans between nodes with the optimum capacity, which is the product of bit rate per channel and number of channels. A best bit rate or a best number of channels exist only for a certain period. Both, best bit rate and number of channels increases over time. Unlike digital cross connect, transparent OCX have fixed cost per cross connection that independent on the bit rate per channel. These are two further reasons for TDM in the OTN, even when there is currently no need for it.

Three alternatives are discussed for the TDM scheme:

- Layered ODU scheme like PDH (see Figure 4-16)

- Lower and higher order ODUs (see Figure 4-17)

- Flat TDM hierarchy (see Figure 4-18)

The layered ODU scheme has two disadvantages. It requires stuffing overhead (about 1% of an ODU) for each multiplex level. Cross connections of ODU1s requires the termination of all higher order ODUs.

The scheme of lower and higher order ODUs offers both, traffic aggregation and transport of aggregated traffic over administrative domains. This scheme has the disadvantage that the higher order ODU has to be terminated in case of lower order cross-connections.

The flat TDM hierarchy provides traffic aggregation, but no transport of aggregated traffic over administrative domains. It has the advantage of direct access to each ODU level.

Two orders of ODUs and the flat TDM hierarchy consume probably the same amount of stuffing overhead, i.e. both schemes lead to the same OTU bit rate. For these reasons, the lower and higher order ODU scheme may be favoured.

Figure 4-16: Layered ODU hierarchy

Figure 4-17: Lower and higher order ODUs

Figure 4-18: Flat TDM hierarchy

#### Excerpt of Draft Recommendation G.709 (April/2000)

OTS OH are transported via the OSC.

The physical layer parameters and provisional values for multichannel inter-domain interfaces are given in Table 23.  It is distinguished between a multi channel interface and a single channel interface.

Table 23: G.959.1 - Physical layer parameters and provisional values for multichannel IrDI applications

| Status |  | Provisionally agreed | Provisionally agreed | Provisionally agreed | Provisionally agreed |
| --- | --- | --- | --- | --- | --- |
| Parameter1 | Units | 16S1-10G-2-III 16S1-10G-3-III 16S1-10G-5-III | 16S1-10G-2-II 16S1-10G-5-II | 16I1-10G-2-IV 16I1-10G-5-IV | 16S1-2.5G-2-IV 16S1-2.5G-5-IV |
| General information |  |  |  |  |  |
| Maximum number of channels | - | 16 | 16 | 16 | 16 |
| Bit rate/format of client signals | - | NRZ 10G | NRZ 10G | NRZ 10G | NRZ 2.5G |
| Maximum bit error ratio | - | 10-12 | 10-12 | 10-12 | 10-12 |
| Fibre type | - | G.652/G.653/G.655 | G.652/G.655 | G.652/G.655 | G.652/G.655 |
| Interface at point MPI-SM |  |  |  |  |  |
| Max. mean channel output power | dBm | -7 | +3 | -3 | -4 |
| Min. mean channel output power | dBm | -11 | 0 | -6 | -10 |
| Max. mean total output power | dBm | +5 | +15 | +9 | +8 |
| Central frequency | THz | 192.1+0.2m, m=0 to 15 | 192.1 + 0.2m, m=0 to 15 | 192.1 + 0.2m, m=0 to 15 | 192.1 + 0.2m, m=0 to 15 |
| Channel spacing | GHz | 200 | 200 | 200 | 200 |
| Max. central frequency deviation | GHz | 40 | 40 | 40 | 40 |
| Min. channel extinction ratio | dB |  |  |  | 8.2 |
| Eye mask | - |  |  |  | STM-16 per G.957 |
| Optical path (single span) from point MPI-SM to MPI-RM, |  |  |  |  |  |
| Maximum attenuation | dB | 11 | 11 | 6 | 11 |
| Minimum attenuation | dB | 0 | 0 | 0 | 2 |
| Maximum chromatic dispersion | ps/nm | 800 | 800 | 400 | 800 |
| Minimum optical return loss | dB | 24 | 27 [24?] | 24 | 24 |
| Maximum discrete reflectance | dB | -27 | -27 | -27 | -27 |
| Maximum differential group delay | ps | 10 [30?] | 10 [30?] | 10 [30?] | 40 [120?] |
| Interface at point MPI-RM |  |  |  |  |  |
| Maximum mean channel input power | dBm | -7 | +3 | -3 | -6 |
| Minimum mean channel input power | dBm | -22 | -11 | -12 | -21 |
| Maximum mean total input power | dBm | +5 | +15 | +9 | +6 |
| Maximum channel power difference | dB | 2 | NA |  | NA |
| Maximum optical path penalty | dB |  |  |  | 1 |
| Maximum reflectance of optical network element | dB |  |  |  | -27 |

##### Single channel IrDI

The physical layer parameters and provisional values for single channel inter-domain interface are given in Table 24 through Table 25.

Table 24: G.959.1 - Proposed Single Channel IrDI parameters/values for client signal class NRZ 2.5G and possibly lower rate classes

| Proposal status |  | Under study | OK | OK | OK |
| --- | --- | --- | --- | --- | --- |
| Parameter | Units | 1I1-2.5G-1-IV | 1S1-2.5G-1-IV | 1S1-2.5G-2-IV | 1L1-2.5G-1-IV |
| General information | - | "I-16" | ”S-16.1” | "S-16.2" | "L-16.1" |
| Maxi. # of channels | - | 1 | 1 | 1 | 1 |
| Bitrate/format client signals | - | NRZ 2.5G | NRZ 2.5G | NRZ 2.5G | NRZ 2.5G |
| Maximum bit error ratio | - | 10-12 | 10-12 | 10-12 | 10-12 |
| Fibre type | - | G.652 | G.652 | G.652 | G.652 |
| Interface at point SM-S |  |  |  |  |  |
| Operating  range | nm | 1266-1360 (G.957) | 1260-1360 (G.957) | 1430-1580 (G.957) | 1280-1335 (G.957) |
| Source type |  | MLM (G.957) | SLM (G.957) | SLM (G.957) | SLM (G.957) |
| Maximum RMS width () | nm | 4 (G.957) | - (G.957) | - (G.957) | - (G.957) |
| Maximum –20 dB width | nm | - (G.957) | 1 (G.957) | <1 (G.957) | 1 (G.957) |
| Minimum side mode suppression ratio | dB | - (G.957) | 30 (G.957) | 30 (G.957) | 30 (G.957) |
| Max. mean output power | dBm | -3 (G.957) | 0 (G.957) | 0 (G.957) | +3 (G.957) |
| Min. mean output power | dBm | -10 (G.957) | -5 (G.957) | -5 (G.957) | -2 (G.957) |
| Minimum extinction ratio | dB | 8.2 (G.957) | 8.2 (G.957) | 8.2 (G.957) | 8.2 (G.957) |
| Optical path from point SM-S to RS-M |  |  |  |  |  |
| Attenuation range | dB | 0 - 6 | 0-11 | 0 - 11 | 10 - 22 |
| Maximum dispersion | ps/nm | 12 (G.957) | NA (G.957) | - (G.957) | NA (G.957) |
| Minimum optical return loss at SM-S | dB | 141 | 141 | 14 | 24 (G.957) |
| Max discrete reflectance between SM-S and RS-M | dB | -27 (G.957) | -27 (G.957) | -27 (G.957) | -27 (G.957) |
| Max. differential group delay | ps | 40 [120?] |  | 40 [120?] | 40 [120?] |
| Interface at point RS-M |  |  |  |  |  |
| Receiver type | - |  |  |  |  |
| Maxi. mean input power | dBm | -3 (G.957) | 0 (G.957) | 0 (G.957) | -9 (G.957) |
| Min. mean input power | dBm | -17 | -17 | -17 | -25 |
| Max. optical path penalty | dB | 1 (G.957) | 1 (G.957) | 1 (G.957) | 1 (G.957) |
| Maximum reflectance | dB | -141 | 141 | -14 | -27 (G.957) |

Note 1 - The values for maximum reflectance and minimum ORL are subject for final confirmation in April 2000.

Table 25: G.959.1 - Proposed Single Channel IrDI parameters/values for client signal class NRZ 10G and possibly lower rate classes

| Proposal status |  | Agreed | Under study1 | Agreed | Agreed | Agreed |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Units | 1I1-10G-1-IV |  | 1I1-10G-2-IV | 1I1-10G-3-IV | 1I1-10G-5-IV |
| General information | - | ”I-64.1” |  | ”I-64.2” | ”I-64.3” | ”I-64.5” |
| Max. number of channels | - | 1 | 1 | 1 | 1 | 1 |
| Bit rate/format of client signals | - | NRZ 10G | NRZ 10G | NRZ 10G | NRZ 10G | NRZ 10G |
| Maximum bit error ratio | - | 10-12 | 10-12 | 10-12 | 10-12 | 10-12 |
| Fibre type | - | G.652 |  | G.652 | G.653 | G.655 |
| Interface at point SM-S |  |  |  |  |  |  |
| Operating wavelength range | nm | 1290- 1330 | 1500-1580 | 1500-1580 [1530-1565] | 1500-1580 [1530-1565?] | 1500-1580 [1530-1565?] |
| Source type |  | SLM | SLM | SLM with ext. mod. | SLM with ext. mod. | SLM with ext. mod. |
| Max. RMS width () | nm | NA |  | N/A | N/A | N/A |
| Maximum -20 dB width | nm | 1 | 1 | TBD | TBD | TBD |
| Minimum side mode suppression ratio | dB | 30 | 30 | 30 | 30 | 30 |
| Max. mean output power | dBm | -1 | 0 | -1 | -1 | -1 |
| Min. mean output power | dBm | -6 | -5 | -5 | -5 | -5 |
| Min. extinction ratio | dB | 6 | 8.2 | 8.2 (G.691) | 8.2 (G.691) | 8.2 (G.691) |
| Optical path from point SM-S to RS-M |  |  |  |  |  |  |
| Attenuation range | dB | 0 – 4 | 0-7 | 0-7 | 0-7 | 0-7 |
| Maximum dispersion | ps/ nm | NA | 40 | 500 (G.691) | 80 (G.691) | TBD (G.691) |
| Minimum optical return loss at SM-S | dB | 14 1 | 24 | 24 (G.691) | 24 (G.691) | 24 (G.691) |
| Max. discrete reflectance between SM-S and RS-M | dB | -27 | -27 | -27 (G.691) | -27 (G.691) | -27 (G.691) |
| Maximum differential group delay | ps | 10 | 10 | 10 [30?] (G.691) | 10 [30?] (G.691) | 10 [30?] (G.691) |
| Interface at point RS-M |  |  |  |  |  |  |
| Receiver type |  |  |  | PIN | PIN | PIN |
| Max. mean input power | dBm | -1 | 0 | -1 | -1 | -1 |
| Min. mean input power | dBm | -11 | -13 | -14 | -13 | -13 |
| Max. optical path penalty | dB | 1 | 1 | 2 (G.691) | 1 (G.691) | 2 (G.691) |
| Maximum reflectance | dB | -14 2 | -27 | -27 (G.691) | -27 (G.691) | -27 (G.691) |

Note 1 - This proposed application is similar to the ”I-64.2” application, except for an intended target distance of up to 2 km, as implied by the proposed value for maximum chromatic dispersion.  The need for a separate application in addition the ”I-64.2” application is under study.

Note 2 - The feasibility of these proposed values is under study.

#### SDH-Wrapped

(1) Leased VC

This service provides leased lines with conventional granularities in the OTN. To achieve this service, three options of overhead-relay functionality is considered. The first one passes through all user overheads with the capability of B1 and J0 monitoring at the gateway equipment. The second one terminates RS at the gateway to obtain OCh overhead capacity, while passing through MS overheads. The last one terminates both MS and RS to make more room for OCh overheads. In this option user’s  VC network is completely separable from the OTN in terms of management capability if all the user overhead is terminated. As such, some portion of user overheads might be relayed to guarantee users’ OA&M capability.

#### Service Parameters

Provisioning Time

Class of Restoration / Protection

Time to Restore

Type of Resilience

- Unprotected

Two classes are recognized: normal unprotected OCh and extra traffic type OCh. Neither of them offers restorability in case of failure. The normal unprotected OCh is managed as a working OCh. It has no protection bandwidth, and as such shall be disrupted during the period of its failure. The extra traffic type OCh is offered by using the protection bandwidth (wavelengths). Its service shall be disrupted and replaced by protection OChs for other protected OChs in case of failure.

- Restored

OChs categorized in this resilience class have no dedicated or shared protection OCh capacity. The only way to be recovered from a failure is the OCh restoration. This service has slower rapidity to restore traffic than the protected OCh service.

- Protected

This resilience class is for the highest priority traffic. Normally, the dedicated OCh protection such as 1+1 and 1:1 is supported. The shared protection (1:N) architecture can be applied if a protection OCh shall not be accessed by multiple of working OChs at the same time. The example of 1:N protection is an OCh shared protection ring. For the multiple failure cases, since not all the protected traffic can be restored, the remaining portion of traffic will be restored by the restoration scheme.

WDM Inter-Channel Jitter/Delay

Performance Parameter

- Performance Parameter of DW bit interleaved parity level eight (BIP-8)

- Check Sum (Example SDH) ??

Bit Interleaved Parity (BIP)

The BIP calculation is used to measure the error performance of an SDH connection according to ITU-T G. 826/828 . The received BIP-Bytes are compared with the calculated BIP-Bytes on the receiver side. In a case of bit errors you get a difference between these compared BIP-Bytes

One byte is allocated for regenerator section error monitoring. This function shall be a Bit Interleaved Parity 8 (BIP-8) code using even parity. The BIP-8 is computed over all bits of the previous STM-N frame after scrambling and is placed in byte B1 of the current frame before scrambling.

The B2 bytes are allocated for a multiplex section error monitoring function. This function shall be a Bit Interleaved Parity N  24 code (BIP-N  24) using even parity. The BIP-N  24 is computed over all bits of the previous STM-N frame except for the first three rows of SOH and is placed in bytes B2 of the current frame before scrambling.

One byte is allocated in each VC-4-Xc/VC-4/VC-3 for a path error monitoring function. This function shall be a BIP-8 code using even parity. The path BIP-8 is calculated over all bits of the previous VC-4-Xc/VC-4/VC-3 before scrambling. The computed BIP-8 is placed in the B3 byte of the current VC-4-Xc/VC-4/VC-3 before scrambling.

Calculation of BIP-X:

The monitored signal is divided in bit sequences each with a size of  X bits. The 1st bit of the BIP-X is the result of  an even parity calculation over all 1st bits from all bit sequences.   The 2nd  bit of the BIP-X is the result of  an even parity calculation over all 2nd bits from all bit sequences, etc.

The even parity calculation is done by set the corresponding bit in the BIP-X in such a manner that you have an even number of one´s.

Example for calculation of BIP-24 :

The three B2 bytes (24 bits) in an STM-1 signal monitors a number of:

(270*9-3*9)*8 bits = 19224 bits.

The number of bit sequences each with a size of 24 bits is therefore:

19224/24 = 801.

- The corresponding bit in the BIP-24 is set to one or zero in such a manner that you get an even number of one´s over the 802 bits (801 bit + BIP-24 bit).FEC : Reed-Solomon (255, 239)

Supervision Functions / requirements

Continuity: support of loss of signal detection and loss of frame detection

Connectivity: support of 64-byte TTI aligned with 64-multiframe

TCM: support of eight level of nested tandem connection monitoring as specified in G.709 and G.872

Defect Indications: support of BEI, BDI, and ODU-AIS

- FEC based Performance parameter

##### Forward Error Correction (FEC)

Error control to guarantee integrity of the transmitted data may be exercised using Forward Error Correction (FEC). In principle, the FEC encoder in the transmitter accepts data bits and adds redundancy according to a prescribed rule, thereby producing encoded data at a higher bit rate. The corresponding decoder on the receiver side exploits this redundancy to decide which message bits have been actually transmitted. The goal of this encoding-decoding mechanism is to reduce the degrading effects of transmission on the Bit Error Ratio to a tolerable level. The use of FEC, however, adds complexity to the system. Thus, an appropriate trade-off has to be found between acceptable error performance and increased bandwidth demand and system complexity

There is a high number of different error correcting codes which can be applied. Historically, these codes are classified into block codes and convolutional codes, depending on the implementation (or absence) of memory in the encoders. In optical transmission systems, block codes are used throughout, due to their efficient hardware implementation.

To generate a (n, k) block code, the channel encoder collects data in successive k-bit blocks. For each block, it adds n-k redundant bits generated by some mathematical rules, thus producing an overall encoded block of n bits (with n > k). Hence, the channel data rate coming out of the coder is always higher than the source data rate. Depending on the amount of added redundancy (i. e. the number of added n-k bits per block), and the type of the code, a specific amount of errors within every encoded block can be identified.

An important subclass of block codes are the so-called cyclic codes. A binary code is classified as cyclic, if

- The sum of any two code words is also a code word;

- Any cyclic shift of a code word is also a code word.

For cyclic codes, an extended mathematical theory has been developed, using binary polynomials do describe the coding and decoding procedures. Linear feedback shift registers are applied to implement coding and decoding algorithms derived from these polynomials.

Examples of block codes are:

- Repetition codes, where a data bit is simply transmitted n times;

- Hamming codes, Maximum Length Codes, Cyclic Redundancy Check (CRC-) Codes, Bose-Chaudhury-Hocquenghem (BCH-) Codes are block codes with specific parameters;

- Reed Solomon Codes (RS-codes) are an important subclass of nonbinary BCH Codes, as the encoder operates on multiple bits (i. e. m-bit symbols) rather than individual bits. Hence, the encoding algorithm expands a block of k symbols to n symbols by adding n-k redundant symbols. These RS-codes make highly efficient use of redundancy, and block lengths and symbol sizes can be adjusted in a wide range of data sizes.

## IP-based Transport Services

In this section, the concept of IP Transport Service will be defined and qualified by a set of parameters.

### IP Transport Service

Before introducing the concept of IP Transport Service some preliminary definitions should be fixed. As known the OSI model provides a conceptual framework for communication between systems, but the model itself is not a method of communication. Actual communication is made possible using communication protocols.  In this context the protocol is a formal set of rules that governs how nodes exchange information over a network medium. A protocol implement the functions of one or more OSI layers.

A given OSI layer generally communicates with three other OSI layers: the layer above, the layer below and its peer layer in the other networked systems. The Service User is the OSI layer that requests services from an adjacent OSI layer. The Service Provider is the OSI layer that provides a service to a Service User. OSI layers can provide services to multiple service users. The Service Access Point (SAP) is the conceptual location where one OSI layer can request a service to another OSI layer. The service provided by adjacent layers help an OSI layer to communicate its peer layer in another system.

Figure 19 - Communication between OSI layers

The Internet protocols consist of a suite of communication protocols of which the two best known are the Transmission Control Protocol (TCP) and Internet Protocol (IP). The IP is a network-layer (Layer 3) protocol that contains addressing information and some control information that enables packets to be routed. IP has two primary responsibilities: providing connectionless, best-effort delivery of datagrams; providing fragmentation and reassembly of datagrams to support data links with different maximum-transmission unit size.

The Internet protocols consist of a suite of communication protocols of which the two best known are are the Transmission Control Protocol (TCP) [RFC793 and RFC1122] and Internet Protocol (IP). TCP is a transport-layer (Layer4) protocol that controls the reliability of the end-to-end data transfer,  while IP is a network-layer (Layer 3) protocol that contains addressing information and some control information that enables packets to be routed. Both software and hardware operating on TCP/IP networks typically consist of a wide range of functions to support data communications. In general, an IP Transport Service could be defined as the capability of an IP-based network to deliver datagram payloads from a Service Access Point to anyone of the interfaces with the IP-address for that SAP. In this sense, an IP Transport Service is intended as an added value service to the simple data transmission. As such an IP Transport Service needs to be characterised.

If the end-user always acts via an application protocol, the Service Access Point of a Network Service Provider (NSP) could be either at the network-layer, or at the transport-layer, via intermediate protocols (e.g. TCP) or at the application layer via application protocols. This is shown in Figure 20.

Figure 20 - Definition of IP Transport Service on the OSI framework

Figure 20 depicts the IP Transport service on the conventional OSI framework. Nevertheless, the IP Transport service has to be placed on the Internet layer architecture shown in [35], which is the OSI framework adapted to the TCP/IP networks.

Figure 21: Definition of IP Transport Service on the Internet Layer Architecture

The bottom layer of the Internet model contains subnetworks and is thus called the subnetwork layer; it includes the subnetwork interfaces. Examples of subnetworks are Ethernet, in local area environment, and Frame Relay, ATM and OTN in wide area environments. The Internet subnetwork layer includes the link and physical layers of the OSI model.

The next layer is the internetwork layer. The internetwork layer provides the functions for connecting networks and gateways into one coherent system. This layer includes the IP and ICMP (Internet Control Message Protocol [RFC 792]) protocols.

The third layer is known as service provider protocol layer. This layer is responsible for end-to-end communications and includes the TCP and UDP protocols.

Finally, the upper layer is called the Application service layer. This layer supports the direct interface to an end-user application (FTP, HTTP, SNMP, SMTP, etc.)

According to this model, a more detailed definition of an IP Transport Service could be formulated including the following capabilities:

- Data transfer

- Reliability

- Flow and congestion control

- Operation and maintenance (O&M)

- Management

Data transfer is provided by IP. IP has two primary responsibilities: providing connectionless, best-effort delivery of datagrams; providing fragmentation and reassembly of datagrams to support data links with different maximum-transmission unit size.

Reliability: An IP transport based service can provide both reliable and unreliable data transfer between host computers attached to the Internet. TCP provides a connection-oriented reliable data transfer, while UDP provides a connectionless unreliable data transfer.

Flow and congestion control: TCP also provides end-to-end flow control. Flow control is performed in order to ensure the proper rate adaptation between sender and receiver, and secondly to prevent and survive possible congestion within the network.

Some sort of Operation and Maintenance capabilities can be set up using ICMP. ICMP includes two key mechanisms, namely ping and traceroute. The former mechanism provides an estimation of the mean delay to reach certain destination. The latter tells the user the current route provided by the network for reaching certain destination. These two mechanisms can be used, either manually or automatically, to monitor the IP transport service. For an automatic monitoring the ping and traceroute mechanisms can be enabled from an internal process, which would be periodically executed.

Management: The protocol used for the management of TCP/IP networks is the Simple Network Management Protocol (SNMP) [RFC1157 and RFC1441 to RFC1452]. SNMP includes three key capabilities, namely get, set and notify. Get enables the management station to retrieve the value of objects at the agent. Set enables the management station to set the value of objects at the agent. Finally, notify enables the agent to notify the management station of significant events [19]. SNMP is at the application layer and goes on top of UDP.

### IP Transport Service Characterization

According to the above definion, an IP Transport Service could be qualified by a set of parameters (to be adopted in the SLA), such as:

- Access mode

- Rate of service availability

- Multiple service quality classes vs performance

- Routing configuration

- Multicast

- Througthput management and traffic shaping

- Security

- Provisioning time

- Charging

The access mode defines how the user can assess the service: for example, switched access via ISDN or dedicated access via leased line such as 2 Mbit/s.The rate of service availability should indicate the service unavailability in hours per year.

Multiple service quality classes tied to the network latency or throughput characteristics provide an opportunity to differentiate between carrier services as well as to satisfy a full range of customer transport requirements via a single common interface. For example the DiffServ proposed by IETF, which makes use of the Type of Services (TOS) bit in the header of every IP datagram, permits the network to recognise how each packet should be handled. Premium (or Virtual Leased Line) service could include application services such as real time video-conferencing; on the other end the Assured service might include most of non-time sensitive application services such as file-transfers.

As the IP layer could run the role of integration layer for multiple services, particular attention should be payed in relating the IP performance parameters (such as packet loss ratio, end-to-end transfer delay, delay variance) with the performance requirements of the services. As an example, in Table 26 performance requirements for both standard services and IP-based services are reported.

| APPLICATION SERVICE | STANDARDS | BER | JITTER | DELAY |
| --- | --- | --- | --- | --- |
| Voice | G.711- G.723.1 G.727 – G.729 | < 10-4 | < 3 ms | < 200 ms |
| Data Trasmission | RFC 959 | n/a | n/a | < 60 s |
| On – Line | RFC 1945 | n/a | n/a | < 400 ms |
| Broadcast-Multicast | MPEG1 – MPEG2 | < 10-6 | < 3 ms | n/a |
| Video Communication | H.320 – H.323 | < 10-6 | < 3 ms | < 250 ms |

Table 26 – Application Service Performance Requirements

Routing configuration should indicate, for example, the belonging to one or more IP-based VPN.

As the offering of multicast services for streaming audio and video-conferences are also foreseen, the IP transport services should support also multicast protocols. There a number of multicast protocols being developed and adopted such as MOSPF (Multicast OSPF), DVMRP (Distance Vector Multicast Routing Protocol) and PIM (Protocol Independent Multicast).

Throughput management and traffic shaping relates to the capability to be used to decrease the burstiness of UDP and TCP traffic an as such to decrease the load on traffic buffers as well as the latency jitter caused by long queues. Traffic shaping does this by identifying traffic flows and then managing the maximum transmission rates. In this sense the rate control works directly with TCP or UDP to manipulate transmission rate, instead of relying only on the existing TCP channel-capacity feedback mechanism.

Security relates to the capacity of end-users to select both secure and non-secure exchange. Support for this feature in mandatory in IP transport service based on IPv6 and optional for IPv4. In both cases the security features are implemented as extension headers that follow the main IP header. Two extension header are defined, one is designated for authentication (the Authentication Header, AH) and the other for privacy (the Encapsulating Security Payload, ESP) [36] [RFC1825 to RFC1829].

The provisioning time is the time required to set-up the service.

Charging is the way the NO get payed for the provided service. Charging implies a tariff specification and an accounting policy. For an IP Transport service, the accounting policy is a still open critical issue. Yet it has to be defined its parameters and implementation.

#### IP Service Level Agreements

In a corporate environment, the IT department takes on the role of network provider, and various internal departments take the position of the customers. Applying certain use policies, so called Service Level Agreements (SLAs), are already common in modern IT strategies. For example, in many enterprises, it is no longer acceptable that employees’ personal web traffic takes the same priority as crucial business-critical traffic, such as order-entry systems or SAP traffic. It must be possible to differentiate the various types of traffic on a private network and prioritise them according to some global strategy.

When dealing with public network services, with the increasing importance of IP-based communications for business, guarantees about the behaviour of the service are becoming more and more important. Offering an IP transport service, if an IP network provider is to make some form of guarantee to the customer, a SLA must be formed. In this case a SLA is a contract between the customer and the provider. The customer lays out their requirements for the network, and the provider makes some level of guarantee to provide those requirements. The guarantee may not be 100%, but could take the form of a probability, for example: the requirements will be met 99.9% of the time, over a one-month period.

Based on the parameters qualifying the IP transport services described above (Section 4.4.2) Table 4.4.2.2-1 gives an example of a level of guarantee for a possible set of IP transport service classes that currently could be offered by a Network Operator. Three classes are described, namely GOLD, SILVER and BRONZE:

- The GOLD service represents the transport class providing the best performances for all potential applications. It is foreseen for supporting very time sensitive applications.

- The SILVER service is a transport class for supporting time sensitive applications.

- The BRONZE service is the transport class for applications that require no particular performances.

| PARAMETERS | GOLD | SILVER | BRONZE |
| --- | --- | --- | --- |
| Availability | 99.97 | 99.95 | 99.95 |
| End-to-End delay | 90 ms | 150 ms |  |
| Packet Loss Ratio | 2 % | 5 % | 10 % |
| Security | - | - | - |
| Provisioning Time | - | - | - |
| Access | 2 - 34 Mbit/s | N x 64 kbit/s | ISDN |

Table 4.4.2.2-1. Example of parameters qualifying an IP Transport Service

Note that the only network parameter for which a valuable guarantee is offered is the End-to-End delay. The announced Packet Loss Ratio is very poor even for the GOLD service class. Also note that no security guarantee is offered.

The case presented in Table 4.4.2.2-1 is, at present, the most common situation: Some ISPs are already offering SLAs to their customers, but very limited. One major ISP is offering their customers guarantees on timely installation, and 100% availability. The only network parameter for which they offer a guarantee is latency - "Average network latency of less than 85 ms roundtrip between designated U.S. hubs and an average latency of 120 ms roundtrip or less between Trans-Atlantic gateway hubs located in New York and London". As we can see, only average latency times are guaranteed, as the ISP’s connections could still congest for short periods. Simple over-provisioning is used here. If stronger network guarantees could be offered, on parameters such as bandwidth, delay or jitter, the IP provider would have many new services to offer. To illustrate this case Table 4.4.2.2-2 includes the IP transport services level of guarantee provided by two different USA network operators, UUNET and AT&T WorldNet.

| UUNET | Internet Access Guarantee | Internet Access Make Good | IP-VPN Service Guarantee | IP-VPN Service Make Good |
| --- | --- | --- | --- | --- |
| Network Availability | 100% | 1 day credit for up to each hour downtime | 99.9% 99.8% 99.6% | 25% credit on entire VPN MSC |
| Maximum Latency | 85 ms (US) 120 ms (int) | 1 day credit after 30 day grace make good period | 100 ms (US) 200 ms (int) | 25% credit on entire VPN MSC |
| Maximum Packet Loss | No |  |  |  |
| Local Access included in Guaranties | Yes |  | Yes |  |
| AT&T WorldNet | Internet Access Guarantee | Internet Access Make Good | IP-VPN Service Guarantee | IP-VPN Service Make Good |
| Network Availability | 99.5% | 1/30 of MSC refund per 15 min. downtime per day; Up to 100% MSC refund per quarter | 99.7% | 5% MSC refund per 10 min downtime per day; up to 25% MSC refund |
| Maximum Latency | N/O | N/O | 100 ms (US) 250 ms (int) | 1/30 of MSC credit |
| Maximum Packet Loss | N/O | N/O | N/O | N/O |
| Local Access included in Guaranties | Yes | N/A | Yes | N/A |

Table 4.4.2.2-2. Two real live examples of SLAs.

SLA metrics

From above, it can be stated that one important aspect which form part of the SLA is the QoS characteristics required for the service. The QoS requirements of the SLA can be categorised into three main groups:

- Bandwidth parameters, describing the bandwidth requirements of the service. An example of these is average throughput.

- Delay parameters, describing the delay requirements of the service. Examples of these are average delay and jitter.

- Loss parameters, describing the loss requirements of the service. Examples of these are average packet loss and packet loss sensitivity.

The SLA also covers all aspects of the customer-provider relationship, which are relevant to the service. Some metrics that SLAs may specify include:

- What percentage of the time services will be available

- The number of users that can be served simultaneously

- Specific performance benchmarks to which actual performance will be periodically compared

- The schedule for notification in advance of network changes that may affect users

- Help desk response time for various classes of problems

- Dial-in access availability

- Usage statistics that will be provided

Once a SLA is agreed, the provider has committed to deliver a QoS and must deliver on that commitment. When the customer and provider have agreed a SLA, it is the responsibility of the provider to monitor and maintain that SLA. If the SLA is not met, the customer is entitled to some form of compensation. This is really a form of insurance policy, which - like most insurance policies - is a last resort for the holder - the customer does not want this money back, they want their service to perform as expected.

In order to minimise customer compensation (increasing profitability and increasing customer satisfaction), management applications for SLA and Quality of Service management are required by the provider. Management of the network must attempt to ensure that SLAs are respected at all times. If this is not possible, the violated SLAs must be prioritised.

This requires tools for measuring the performance of the network, analysis of the performance information to identify violations of SLAs, and the provision of decision support capabilities for aiding the provider in the resolution of such problems. Such tools must be multi-vendor (capable of managing network elements from a range of vendors in a consistent manner) and be capable of SLA management in a multi-provider domain.

Policy-based Management

Quality of Service is an end-to-end concept; if the quality at one hop in the network is not sufficient then the service is affected. This means that in order to manage QoS, all of the various devices on a path must be managed in a co-ordinated manner. It is this issue that policy-based management initiatives within IETF working groups (such as the Network Working Group, [8]) aims to address.

Policies may be specified independent of a particular device and can refer to end-to-end targets (e.g., for a particular Class of Service). Policies are also specified in a vendor-independent notation (there is currently a Policy Definition Language being defined within the IETF [9]). The policy service then takes care of determining which devices the policies are applicable to and propagates the policies to these devices. If the device does not support standard protocols for setting of policies then the policy service also handles the mediation between the policies and their representation on the device. Because the policy service propagates the policies to the devices, the network manager does not have to explicitly update a MIB or issue the appropriate CLI4 commands on each device.

Put simply, a policy represents a control rule within the network, and is normally composed of a Policy Condition and a Policy Action. A policy condition is a filter related to the IP packet header, it identifies a subset of packets to which the policy should apply. A simple example of a policy condition could be all ftp traffic originating at IP address 10.10.15.3 and terminating at IP address 10.10.16.3. A policy action is the action to be taken on traffic relevant to the policy. An example of a policy action is to assign the traffic 5% of the link bandwidth, and a priority of "critical". Whenever a policy condition is met, a policy action is carried out.

The policy service is composed of a number of components:

- The Policy Entry Console (PEC), which is used by the operator to manage the policies in the network domain.

- The Policy API (PAPI) which is the exported interface of the policy service used by applications programs in the management of policies.

- The Central Policy Controller (CPC), which assures the consistency of policies and ensures that changes in policies are affected in a predictable manner.

- The Policy Decision Point (PDP) makes decisions about which devices new policies should be implemented upon and changes existing policy configurations on devices where necessary. A protocol called the Common Open Policy Specification (COPS) is being proposed as the protocol for communication between PDPs and network elements (or network element proxies).

- The Message Passing Service is an event service for communication between components.

- The Directory, which acts as a common repository for shared information relating to policies.

- The Policy proxy, which mediates with the policy service for legacy network elements not supporting the standard protocols and schemata.

There are a number of policy-based management products on the market, which allow the setting of QoS policies. Most of these offerings are proprietary to the vendor’s own packet-shaping devices (e.g., Packeteer, www.packeteer.com). Other offerings from router and switch vendors are beginning to appear, and they will allow the setting of policies on these network elements (e.g., Cisco Assure, www.cisco.com). Finally, there are also products from independent software vendors, which are multi vendor and enable the setting of policies on multiple device types (e.g., Orchestream, www.orchestream.com). In most - if not all - of these cases, the policy manager does not communicate with network elements using COPS as this is not supported by the network elements in question. However, as COPS becomes adopted this situation is expected to change.Policies can provide a mechanism for provisioning services across the network. However, they tend to be static, and do not take into account the actual QoS being received by the service.

Management of DiffServ-capable Networks

As it has been said, operators will increasingly provide each customer/user of the network with a Service Level Agreement to specify the terms and conditions of the subscription. There will then be a requirement on the network to honour all these SLAs. However, coarse QoS solutions such as DiffServ do not provide guarantees at this level, as the QoS is defined for the Class of Service rather than for a particular customer’s flow. So it will fall to sophisticated management systems to fill the gap to help to assure the SLAs. In particular, in the case where the network is congested, the management system must define how and which SLAs will be respected with the finite resources that exist.

The end-to-end nature of QoS further complicates this. In many cases, the two communicating entities may be in geographically distributed locations, connected to different providers, who themselves may not be directly connected to each other. This means that the flow of communication must cross multiple provider domains, any of which can affect the QoS experienced. Within a provider’s domain, it is relatively easy to honour a SLA, as the provider has absolute control over all traffic within the domain. However, once the traffic steps out of that domain then the control is lost. Also, the meaning of different Classes of Service may vary from provider to provider.

One possible solution to this issue is where the providers themselves also have SLAs between them relating to the traffic passing through their domains. This means that each peering pair of providers will have to reach an agreement over how each other’s traffic will be handled within their respective domains. The agreement would specify how each class of traffic should be handled by the other6. In effect, the providers are customers of other providers.

As an example, consider the following case:

- Traffic originating in Domain 1 and terminating in Domain 2 will be placed into Class X.

- Traffic coming from Domain 1 but not originating in Domain 1 will be placed into Class Y.

- DiffServ byte ‘A’ in Domain 1 will be mapped into DiffServ byte ‘B’ in Domain 2.

Conclusions

Proposed solutions to the challenges of QoS are being standardised within the IETF. Of these, the DiffServ approach appears to be gaining significant momentum. The relative simplicity and coarseness of the QoS characteristics in DiffServ mean that it will require significant management in order to assure Service Level Agreements (SLAs). In particular, a management system is required which measures the actual SLA performance, relates it to the targets in the SLA and provides decision support in the resolution of problems with the resolution of the SLA.

#### Overview IP – QoS and Traffic Engineering Framework

Definition of IP QoS: Quality of Service is defined as the set of application requirements concerning the time and reliable conditions under which the traffic that these applications generate must be supported. The Quality of Service requirements are established according end-user subjective parameters (such as human-eye sensitivity, human-ear sensitivity, etc.). These parameters impose the required values for the transport service performance parameters (such as packet loss ratio, end-to-end transfer delay, delay variance). Thus, an IP Transport service ambitioning to integrate multiple types of applications, need to provide a wide range of QoS requirements (as an example, see Table 21).

There are essentiality two QoS schemes:

- Resource reservation (or integrated services model, IntServ): Network resources are apportioned according to an application’s QoS request, and subject to a per-flow bandwidth management policy. RSVP provides the mechanisms to support IntServ in an IP Transport service.

- Prioritisation (or differentiated services model, DiffServ): Incoming traffic to the network is aggregated and classified. Preferential treatment is given to applications identified as having more demanding requirements. Network resources are apportioned according to a per hop behaviour policy applied to the different aggregated flows enabled within the network. To support DiffServ, IP Transport services use scheduling and buffering at nodes.

Definition of traffic engineering: Internet Traffic Engineering is defined as that aspect of Internet network engineering which is concerned with the performance optimization of operational networks. It encompasses the application of technology and scientific principles to the measurement, modeling, characterization, and control of Internet traffic; and the application of such knowledge and techniques to achieve specific performance objectives, including the reliable and expeditious movement of traffic through the network (i. e. the QoS provisioning), the efficient utilization of network resources, and the planning of network capacity. Definition and recommendations done by IETF: The IETF is organized in several working areas which consist of several working groups. In the following the work of the working groups dealing with traffic engineering and QoS issues are reviewed.

Multiprotocol Label Switching (mpls) working group in the Routing Working Area

The working group was set up in 97. The working group defines its goal as follows:

“Recently the use of label-swapping based forwarding ("label switching") in conjunction with network layer routing has attracted much attention. Several vendors have proposed techniques based on this paradigm. Among the problems this paradigm is expected to address the following:

1. Scalability of network layer routing

Using labels as a means to aggregate forwarding information, while working in the presence of routing hierarchies.

2. Greater flexibility in delivering routing services

Using labels to identify particular traffic which are to receive special services, e.g. certain QoS.

Using labels to provide forwarding along an explicit path different from the one constructed by destination-based forwarding (Explicit Routing).

3. Increased performance

Using the label-swapping paradigm to optimize network performance.

4. Simplify integration of routers with cell switching based technologies

a) making cell switches behave as peers to routers (thus reducing the number of routing peers that a router has to maintain),

b) by making information about physical topology available to Network Layer routing procedures, and

c) by employing common addressing, routing, and management procedures.

The working group is responsible for standardizing a base technology for using label swapping forwarding paradigm (label switching) in conjunction with network layer routing and for the implementation of that technology over various link level technologies, which may include Packet-over-Sonet, Frame Relay, ATM, Ethernet (all forms, such as Gigabit Ethernet, etc.), Token Ring, etc…”

There exist one RFC (Requirements for Traffic Engineering Over MPLS (RFC 2702)) and around 70 current Internet drafts. Some of them are:

General:

- draft-ietf-mpls-framework-05.txt A Framework for MPLS

- draft-ietf-mpls-arch-06.txt Multiprotocol Label Switching Architecture

Traffic Engineering:

- draft-takashima-te-concept-00.txt Concept of IP Traffic Engineering

QoS:

- draft-fedyk-mpls-te-metrics-00.txt Metrics and Resource Classes for Traffic Engineering

- draft-ietf-mpls-diff-ext-04.txt MPLS Support of Differentiated Services

VPNs:

- draft-muthukrishnan-mpls-corevpn-arch-00.txt Core MPLS IP VPN Architecture

- draft-rosen-rfc2547bis-00.txt BGP/MPLS VPNs

Protection/Restoration:

- draft-andersson-reroute-frmwrk-00.txt Requirement Framework for Fast Re-route with MPLS

- draft-chang-mpls-path-protection-00.txt A Path Protection/Restoration Mechanism for MPLS Networks

- draft-shew-lsp-restoration-00.txt Fast Restoration of MPLS Label Switched Paths

- draft-makam-mpls-protection-00.txt Protection/Restoration of MPLS Networks

- draft-makam-mpls-recovery-frmwrk-00.txt Framework for MPLS Based Recovery

OAM and Management:

- draft-theimer-mpls-oam-requ-00.txt Requirements for OAM Functionality in MPLS

- draft-gibson-manage-mpls-qos-01.txt The management of MPLS LSPs for scalable QoS Service Provision

The working group is recently also very active in investigating the use of MPLS principles and control mechanisms in Optical networks:

- draft-krishnaswamy-mpls-son-00.txt MPLS control plane for Switched Optical Networks

- draft-ceuppens-mpls-optical-00.txt Performance Monitoring in Photonic Networks in support of MPL(ambda)S

- draft-awduche-mpls-te-optical-01.txt Multi-Protocol Lambda Switching: Combining MPLS Traffic Engineerin...

- draft-saha-rsvp-optical-signaling-00.txt RSVP Extensions for Signaling Optical Paths

- draft-kompella-mpls-optical-00.txt

- draft-basak-mpls-oxc-issues-01.txt

- draft-mannie-mpls-sdh-control-00.txt MPLS for SDH control

The TE and QoS topic is seen mainly correlated to MPLS today. However, a separate working group Internet Traffic Engineering was established recently. The work on lamda Switching was integrated in the MPLS group as well, but there is an ongoing acivity to coordinate this work in an own working group IPO.

Internet Traffic Engineering (tewg) working group in the Operations and Management Working Area

This working group was set up around October 99. The intention was to concentrate all the TE topics in this working group. The working group defines its primary focus as follows: “...concerns the control aspects of intra-domain Internet traffic engineering. This includes control aspects pertaining to intra-domain routing and control aspects pertaining to intra-domain network resource allocation. Techniques already in use or in advanced development for Internet TE include ATM and Frame Relay overlay models, MPLS based approaches, constraint-based routing, and TE methodologies in Diffserv environments...”

There exists one Internet draft:

- draft-ietf-tewg-framework-01.txt “A Framework for Internet Traffic Engineering”

However, the working group seems still in the set-up phase, most of the work on TE is still discussed in the mpls group.

DiffServ working group in the Transport Area

The working group specifies its working area as follows:

“There is a clear need for relatively simple and coarse methods of providing differentiated classes of service for Internet traffic, to support various types of applications, and specific business requirements. The differentiated services approach to providing quality of service in networks employs a small, well-defined set of building blocks from which a variety of aggregate behaviors may be built. A small bit-pattern in each packet, in the IPv4 TOS octet or the IPv6 Traffic Class octet, is used to mark a packet to receive a particular forwarding treatment, or per-hop behavior, at each network node.... the Working Group has standardized a common layout for a six-bit field of both octets, called the 'DS field'. ...

…The group will not work on:

- mechanisms for the identification of individual traffic flows

- new signalling mechanisms to support the marking of packets

- end to end service definitions

- service level agreements…”

The work resulted in a list of Internet drafts and 4 RFCs, This RFCs are the following:

- Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers 
(RFC 2474)

- An Architecture for Differentiated Services (RFC 2475)

- An Expedited Forwarding PHB (RFC 2598)

- Assured Forwarding PHB Group (RFC 2597)

IntServ working group in the Transport Area

The working group specifies its working area as follows:

“Recent experiments demonstrate the capability of packet switching protocols to support Integrated Services --- the transport of audio, video, real-time, and classical data traffic within a single network infrastructure. These experiments suggest that expanding the Internet service model would better meet the needs of these diverse applications. The purpose of this working group is to specify this enhanced service model and then to define and standardize certain interfaces and requirements necessary to implement the new service model.

The working group will focus on the three problems listed below:

- Clearly defining the services to be provided.

- Defining the application service, router scheduling and (general) subnet interfaces.

- Developing router validation requirements which can ensure that the proper service is provided.

The group will not work on:

- Enhancements to individual protocols will be left to other working groups, except in those rare cases where detailed definitions of behaviour are critical to the success of the enhanced architecture…”

The work resulted in a list of 1 Internet draft: and 2 RFCs

The Internet Draft:

- draft-ietf-intserv-compress-02.txt

RFCs:

- General Characterization Parameters for Integrated Service Network Elements (RFC 2215)

- The Use of RSVP with IETF Integrated Services (RFC 2210)

The protocol RSVP, one possible resource reservation protocol which is needed oin an IntServ architecture is specified in a separate working group ”Resource Reservation Setup Protocol” in the Transport Area.

RSVP working group in Transport Area

Its major features include: (1) the use of ``soft state'' in the routers, (2) receiver-controlled reservation requests, (3) flexible control over sharing of reservations and forwarding of subflows, and (4) the use of IP multicast for data distribution.

The primary purpose of this working group is to evolve the RSVP specification and to introduce it into the Internet standards track.

The task of the RSVP Working Group, creating a robust specification for real-world implementations of RSVP, will require liaison with two other efforts: (1) continuing research and development work on RSVP in the DARTnet research community, and (2) the parallel IETF working group that is considering the service model for integrated service. Although RSVP is largely independent of the service model, its design does depend upon the overall integrated service architecture and the requirements of real-time applications. As an additional task, RSVP will maintain coordination with the IPng-related working groups.

The work resulted in a several Internet drafts and RFCs:

Internet Drafts:

- draft-ietf-rsvp-state-compression-03.txt RSVP Refresh Overhead Reduction by State Compression

- draft-ietf-rsvp-refresh-reduct-04.txt RSVP Refresh Overhead Reduction Extensions

- draft-ietf-rsvp-proxy-00.txt RSVP Receiver Proxy

RFCs:

- Resource ReSerVation Protocol (RSVP) Version 1 Applicability Statement Some Guidelines on

- Deployment (RFC 2208)

- Resource ReSerVation Protocol (RSVP) -- Version 1 Message Processing Rules (RFC 2209)

- Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification (RFC 2205)

- RSVP Management Information Base using SMIv2 (RFC 2206)

- RSVP Extensions for IPSEC Data Flows (RFC 2207)

- RSVP Diagnostic Messages (RFC 2745)

- RSVP Operation Over IP Tunnels (RFC 2746)

- RSVP Cryptographic Authentication (RFC 2747)

# Additional Information

Some detailes regarding the information structure for the ONNI are located in working document LION/WPG1/WP1/T1 [48] – p.5. There are also located same information respekting to SDH protocol of characteristics and functionality.