WP3-M1

Specification of functionalities of available planning and simulation tools

| Status and Version: | Version 4.0 |  |
| --- | --- | --- |
| Date of issue: | 17.11.2000 |  |
| Distribution: | Project Internal |  |
| Contributors(s): | Marco Quagliotti, Fausto Saluta, Marco Bettin | CSELT |
|  | Gabriella Colomba | SIRTI |
|  | Mario Pickavet, Sophie De Maesschalck, Peter Arijs, Chris Develder | IMEC |
|  | Julio Moyano, Josep Sole-Pareta | UPC |
|  | Jan Dercatz | AGH |

Table of Contents

Executive summary	4

1	Introduction	6

1.1	Purpose and Scope	6

1.2	Reference Material	6

1.2.1	Reference Documents	6

1.2.2	Abbreviations	7

1.3	Document History	7

1.4	Document overview	8

2	Overview on available tools	10

2.1	Planning tools	10

2.1.1	IP layer	10

2.1.2	MPLS/DPT/GbEthernet/POS	11

2.1.3	SDH layer	11

2.1.4	OTN	12

2.2	Simulation environments and simulation models	13

2.2.1	IP based network simulator	14

2.2.2	DPT simulator	14

2.3	A tool for strategic planning	15

APPENDIX A: IP Networks Planner	16

IP Networks Planner description form	16

APPENDIX B: CCN Optimiser	20

CCN Optimiser description form	20

APPENDIX C: SIREN-PLAN-SDH/RING	25

SIREN-PLAN-SDH/RING description form	25

APPENDIX D: WDMNetDesign	31

WDMNetDesign description form	31

APPENDIX E: WDMRing	41

WDMRing description form	41

APPENDIX F: IP-based network simulator	49

PART 1: description of the simulation tool/environment NS	49

PART 2: description of available model/simulation scenarios	54

APPENDIX G: DPT (SRP-fa and ISP) simulator	55

PART 1: description of the simulation tool/environment OPNET	55

PART 2: description of available model/simulation scenarios	60

Appendix H: OPTIMUM	61

Key Inputs	61

Architecture scenario	61

Service scenario	61

Subscriber density	61

Service demand	61

Key Outputs	62

Revenues	63

Investments	63

Life Cycle Cost	63

Cash Balance	64

Net Present Value (NPV)	64

Internal Rate of Return (IRR)	64

Geometric modeling	64

The tool database	64

# Executive summary

This document includes information about planning and simulation tools available to the project team. Information was collected by means of charts, which the partners filled in with details concerning the tools at their disposal.

As regards the planning tools, all the participants contributed to the identification of software programs that can cover planning issues about the IP/MPLS, SDH and WDM technologies/layers.

The information provided on the characteristics/ functionalities of the planning tools (IMEC, SIRTI and CSELT) is quite detailed. It must however be stressed that the planning tools are able to handle one network layer at a time; so far we have been unable to identify any multi-layer tools. It follows that when we have to use the tools in order to plan / dimension a case study network we will have to use the tools separately, one by one, from the higher to the lower layer and not simultaneously in an integrated way, as it would be possible with a multi-layer tool.

In addition, when we use the tools  it is a very hard task – or an impossible task- to take issues like multi-layer resilience strategies into consideration. This aspect should be taken into account in the other tasks of the WP3.

A couple of partners (UPC and IMEC) contributed with the description of simulators that cover IP network layer and DPT technology (SRP-fa Protocol). Although these simulators cannot directly support a network planning process they can be used to derive dimensioning criteria for network elements or –alternatively- to evaluate the performance of a roughly designed network.

The following figure shows the relationships between the tools described in this document and the protocol stack assumed as a reference in the LION project.

The protocol stack and the available planning and simulation tools

This collection of contributions also includes a strategic planning tool called OPTIMUM (proposed by AGH). This tool is not a planning tool like the tools that we mentioned above, which are tools based on dimensioning and optimization criteria and whose purpose is the technical design of the network.

OPTIMUM is a tool that enables its users to evaluate economic features such as investments, revenues, life cycle costs, cash balance and so on connected with the construction and running of the network in a given period.

Even though the tool achieves a rough dimensioning of the network it doesn’t perform a detailed project of the network and in particular it doesn’t take into account factors like architectural details, resilience or grade of service. In addition, the tool carries out evaluation on the access portion of the network while transport on the backbone is disregarded.

This is the reason why OPTIMUM doesn’t seem to be workable at present within our project, but in the future it could be used to carry out case studies focused on economic evaluation of network solutions.

In addition some partners declared that new computer programs or upgrades of existing tools (both for network planning and simulation) is under development or planned for the near future. For example NTUA is thinking about the development of a network simulator for evaluating network performances in terms of robustness/recovery against failures. IMEC is refining its computer programs for planning and dimensioning robust MPLS/MPS networks. UPC is completing its DPT simulator (to add the IPS part). CSELT is also thinking about the introduction of resilience strategies in IP planner. So we shouldn’t consider the work connected to collelct information about planning and simulation tool finished and all the partner are encouraged to circulate information about new functionalities of existing tools or new computer programs when they will be reach a stable version and they could be employed in WP3 activities.

# Introduction

## Purpose and Scope

The goal of the M1 of WP3 is to provide the Project Partners with an overview of all planning and simulation tools available to LION Project participants.

M1 includes the all the work done on Task 1 of WP3. Further information concerning other tasks – and in particular task 2 - will be contained in other official reports.

## Reference Material

### Reference Documents

The following table collects the significant contributions that have been received to be integrated into this report.

The first two documents (No 1 and 2) on the list are the frameworks proposed by CSELT in order to collect information about the characteristics of planning tools and simulation model and environment.

The documents from No 3 to No 7 provided by CSELT, IMEC  and SIRTI concern the tools for planning the network layers that the partners decided to take into account within WPG1, (i. e. IP, SDH and OTN layer). For this reason other documents provided by the partners, and in particular documents on ATM planning tools, were not taken into account as material to be inserted in the present report.

Documents No 8 and No 9 concern some simulation models built up on simulation environment like NS and OPNET. This simulation model could be applied to an planning context in order to study end evaluate some aspects connected with the use of a specific technology or protocol. In particular the available models deal with the two following aspects: the evaluation of DPT technology (and in particular the performance of the SRP protocol) and the protection switching delay on TCP flows.

Document 10 provided by AGH concerns OPTIMUM, a tool for “strategic planning”. This kind of tool is not conceived for a detailed network planning and dimensioning like the other planning tools proposed and described but it is able to perform economic analysis connected to the building and the operation of a telecommunication network. At present the use of such a  tool is not yet  considered or discussed but should economic issues  be considered , this tool can support this kind of activity.

List of documents

| No | Document file name | Source/e-mailed FROM | TO | Date |
| --- | --- | --- | --- | --- |
| 1 | Planning Tool Functionalities v2.doc | CSELT/Marco Bettin | WPG1 partners | 28/03/2000 |
| 2 | Simulation_tool_template.doc | CSELT/Marco Quagliotti | WPG1 partners | 07/07/2000 |
| 3 | IP Net Palnner Functionalities.doc | CSELT/Marco Bettin | WPG1 partners | 07/04/2000 |
| 4 | CCNOptimiser.doc | CSELT/Marco Bettin | WPG1 partners | 07/04/2000 |
| 5 | SIREN-PLAN_SDH_functionalities.doc | SIRTI/Gabriella Colomba | WPG1 partners | 11/04/2000 |
| 6 | LION_IMEC_WdmNet Design.doc | IMEC/Mario Pickavet | CSELT | 29/05/2000 |
| 7 | LION_IMEC_WdmRing.doc | IMEC/Mario Pickavet | CSELT | 29/05/2000 |
| 8 | Simulation_tool_ns_IMEC.doc | IMEC/Chris Develder | CSELT | 24/07/2000 |
| 9 | WP3_simulators_dpt-upc.doc | UPC/Josep Sole-Pareta | CSELT | 28/07/2000 |
| 10 | Optimum_tool_description.doc | AGH/Jan Dercatcz | AGH | 11/09/2000 |

### Abbreviations

| TCP | Transmission Control Protocol |
| --- | --- |
| OTN | Optical Transport Network |
| ISP | Intelligent Switching Protection |
| DPT | Dynamic Packet Transport |
| SRP | Spatial Reuse Protocol |
| IP | Internet Protocol |
| VC | Virtual Container (SDH context) |
| WDM | Wavelength Division Multiplexing |
| ILP | Integer Linear programming |
| OTS | Optical Transmission Section |
| OMS | Optical Multiplex Section |
| DCL | Digital Client Layer |
| D&C | Drop And Continue |
| OCh | Optical Channel |
| WP | Wavelength Path |
| VWP | Variable Wavelength Path |

## Document History

| Version | Date | Authors | Comment |
| --- | --- | --- | --- |
| 1.00 | July 2000 | Marco Quagliotti | First version, with contribution on planning tools only |
| 2.00 | 31/8/2000 | Marco Quagliotti | Second Version with additional contributions on simulation environments and models |
| 3.00 | 28/9/2000 | Marco Quagliotti | Third Version with additional contribution on tool for economic evaluation and conversion from WP3/T1 report into M1 document |
| 4.00 | 17/11/2000 | Marco Quagliotti | Fotrth Version with some rearrangement on the document contents suggested by IMEC during Krakov Meeting (a short paragraph describing the CCN optimizer and a picture showing relationships between the protocol layer stack and the described tools) |

## Document overview

The document is structured as follows.

Section 2 gives an overview on available planning and simulation tools and tries to make some suggestions on how those tools can be employed in the following activity of WP3.

In particular, as the most interesting IP over OTN scenario seems to be the one that uses a direct mapping of the IP network layer on the OTN (i.e. without any SDH intermediate network infrastructure, only with an adaptation/framing in-between mechanism) and the one that uses an intermediate SDH network layer, we can concentrate our attention on these two types of layering scenarios.

The last sub-section of section 2  includes a brief presentation of a tool for strategic planning for economic issues.

Appendixes from A to E concern the detailed description of the planning tools while Appendixes F and G include the description of the simulation models and environments.

Appendix  H gives a description of the strategic planning tool called OPTIMUM.

The following tables report the main characteristics of the tools described in detail the appendixes.

List of appendixes which contain planning tool characteristics

| Appendix | Planning Tool | Company | Layer/Net structure |
| --- | --- | --- | --- |
| APPENDIX A | IP Network Planner | CSELT | IP/mesh |
| APPENDIX B | CCN Optimiser | CSELT | Any cross-connected network/mesh |
| APPENDIX C | Siren SDH | SIRTI | SDH/ring |
| APPENDIX D | WDMNetDesign | IMEC | OTN/mesh |
| APPENDIX E | WDMRing | IMEC | OTN/ring |

List of appendixes which contain simulation models and environments

| Appendix | Simulation model | Simulation environment | Company | Layer/technology |
| --- | --- | --- | --- | --- |
| APPENDIX F | SRP-fa and ISP algorithm evaluation on a DPT ring | OPNET | UPC | IP employng DPT technology |
| APPENDIX G | TCP flows, OSPF behaviour and performance | NS | IMEC | IP |

# Overview on available tools

## Planning tools

Available planning tools cover the following network layers:

- IP

- SDH (only rings)

- OTN (both meshes and rings)

Other layers, and in particular layer 2 or 2/3 implemented in Router-Switches like DPT or MPLS are not covered by any planning tool declared available within the consortium.

So, when the WP3 participants will decide about which scenarios have to be analyzed through numerical techno-economic case studies, the ways to perform the case studies have to be discussed. In particular it will be necessary to decide about the need to develop some custom “informal tools” (through spreadsheets for example) or develop/adapt new functionalities on existing tools to cover the planning and dimensioning issues for layer 2 or 2/3 (OSI).

### IP layer

The tool available for optimising and dimensioning the IP layer is IP networks planner (see APPENDIX A for a detailed description).

The assets of the tool are the following.

IP Nertworks Planner allows a full topology capacity and flow assignment in an IP network. The tool allows us to derive an optimal minimum cost IP network topology when the packet traffic matrix, the mean network delay requirement and the cost parameters of transport services (which are modeled as “generic” transportation service) are given.

The tool assumes the OSPF algorithm as a principle to route the traffic flows on the network. As a secondary output of the algorithm the weight of the links in the OSPF topology is provided by the tool.

The principal weaknesses of the tool are the following:

- no possibility to differentiate the traffic into classes of services (for instance, Diffserv offer a kind of CoS inIP networks). This happens because IP Net planner implements a model for which all the packets are routed, queued and forwarded with the same policy rule.

- no possibilty to make a robust project: the optimal network structure doesn’t take into account any diversity criterion to face with link or node failures. So it is not possible to carry out a project taking into account  resilience requirements.

- no possibility to take into account special kinds of traffic management (MPLS) or level 2 transportation (DPT)

- no possibility to take into account equipment costs: only transportation costs are modeled

- only dimensioning starting from a green-field context

So, the tool is good to perform a dimensioning of the resources requested by IP level only when the protection relies on the lower layers of the network.

### MPLS/DPT/GbEthernet/POS

Up to now there aren’t any tools available for planning/dimensioning or DPT or GbEthernet network functionalities while there are some computer programs (not planning tools but prototypes) for planning and dimensioning network that use MPLS/MPS techniques available in IMEC.

In fact , these functionalities should be modeled and handled, for planning/dimensioning/simulating purposes, in a proper way when they are present in the IP over OTN layer stack.

For instance, if MPLS is the technique used to make traffic engineering on the network, a proper tool for planning and dimensioning the network should take into account the mechanism for routing and managing the traffic flows typical of MPLS technology and, for example, the presence of  given resilience strategies to protect the Label Paths. This is what it is implemented in the research programs under refinement in IMEC.

GbEthernet should be employed in two options: only framing or full switching.

In the former case IP flows are framed into GbEthernet frame and use only point to point connection between GbE interface cards installed on the routers. In this case no specific tool is required and the implications on the project involve only the overhead of GbEthernet framing and the cost.

In the latter case a full layer 2 infrastructure, that uses layer 2 switches, is used to interconnect the network.

POS could require a point to point SDH flows that can either use or not an SDH infrastructure as a in-between layer between IP and OTN. Also in this case an infrastructure (SDH) can be present or not depending on the scenario considered.

### SDH layer

SIREN-SDH

A tool for planning and dimensioning SDH infrastructure called SIREN-SDH is currently used by SIRTI.

The principal asset of the tool is that SIREN-SDH produces a very detailed project of a transmission SDH network. In fact it is a tool oriented to Operators (it is not a research tool). In particular the tool gives as outputs the configurations of each piece of equipment and the details of the routing of the single flows. SIREN-SDH allows accurate cost comparisons among different dimensioning scenarios.

A WDM infrastructure under SDH is also modeled, but only as a point to point systems in order to economically compare alternative projects in which new fibers are supposed to be deployed or WDM system is introduced in order to multiply the transportation capacity of existing fibers.

In addition a functionality for grooming lower order flows is offered. A number of protection strategy, associated to single flows, is also allowed and routing strategies can be chosen from a set by the user.

The principal weakness of the tool is that SIREN-SDH is not an optimization tool but only a dimensioning tool, so the structure of each ring, the point of the interconnections between rings and other architectural details have to be assigned from the user and they are not the results of an optimization process. In addition the economic evaluation of the network seems to be too accurate in a context like WP3 of LION, in which techno-economic evaluation should be done on general case study, and not on detailed project of a real network.

CCN Optimiser

CCN Otimiser is a computer program for ATM, SDH and WDM network optimisation available in CSELT. It is not focused on a particular technology and it is a designed to optimise the topology and sizing the link groups of a generic cross connected meshed network (Network which is supposed to use Deterministic Multiplexing).

Only two network layers are considered by the program: a generic client layer which relies on a generic server layer.

CCN Optimiser requires a definition of the cross connecting nodes, the definition of the demands for permanent connections (circuits of the client layer, each one characterised by source, destination, bandwidth, directionality, maximum hops permitted along its path), the cost of transport services offered by the server layer.

On the basis of these inputs, CCN Optimiser provides

- the optimal topology of the network (set of topology edges)

- the number of links required on each edge

- the optimal path for each circuit

- the minimum cost of the network

The principal weakness of the algorithm implemented in CCN optimiser is the fact that the optimisation process doesn’t take into account any resilience strategy; so the resources allocated by the algorithm are enough only for routing the whole traffic over the network without any chance of recovery in case of failure.

### OTN

A complete set of planning tools for planning and dimensioning OTN is provided by IMEC.

With WDM_NetDesign and WDMRing (see APENDIXES D and E) both meshed and ring OTN networks can be optimised and dimensioned.

WDMNet Design

WDM Ring is a planning tool that allows to perform the planning and dimensioning of a multy-layer network:a digital client layer (DCL), an optical channel layer (OCh), and a Physical layer (OMS and OTS).

At DCL demands are expressed as a list of STM1/OC3 to be transported (demands are also entered at DCL in Mbit/s). At OCh layer demands are expressed in number of optical channel (2.5 or 10 Gbit/s) or also in raw bandwidth (Gbit/s).

Physical topology is assigned and the dimensioning algorithm can work either with or without an optimization algorithm that tries to minimize (reduce) the network installation cost by changing some OCh routes that imply an inefficient use of the network. The tool outputs the topology, the routing of the demands (both working and protection) and costs.

Assets:

- many kinds of protection and routing strategies are available

- detailed model for equipment

- for both green field and existent networks (during optimization step, already existent facilities and demands are not changed)

- accurate model of cost components

Weaknesses:

- no optimization strategy in mapping DCL level on OCh level (DCL traffic routing require that OChs has already been set up)

- no assurance that diverse routes for protected DCL demands are physically disjoint.

WDMRing

WDM Ring is a planning tool that allows to perform the planning and dimensioning of a two layer network: a client layer (OCh) and a server layer (OMS and OTS). Demands are expressed as a  list of optical channel transported over the network. The physical topology in terms of the meshed structure of the fiber/cable mesh is assigned. The tool selects the optimal rings to route the Optical Channel demands. Channels are supposed to carry STM-16 SDH flows.

Assets:

- full optimization tool for ring OTN (on the basis of both the physical topology and the Och demands WDM Ring build up the structure of the network in terms of configuration and dimensioning of the rings following an optimization criterion that minimizes the overall cost of the network)

- many kinds of protection and routing strategies are available

- for both green field and existent networks

- cost model: taking into account fiber and equipment costs, the cost of some summarized network components are evaluated (single specific ring cost, cost for routing a specific wavelength on a specific ring, cost for transiting a wavelength between two rings). This can be very useful in techno-economic evaluations and comparisons.

Weakness:

- the matrix of optical channels to be transported must be symmetric and channels are supposed carrying STM-16 flows

- wavelength conversion is allowed only when a channel transiting between two rings

## Simulation environments and simulation models

Available simulation models deal with two aspects:

- IP based network, in particular available models allows to analyze the behavior and the performance of TCP and OSPF protocols

- DPT evaluation, and in particular simulator for SRP-fa and ISP protocols/algorithms

The two models rely on two general purpose simulation environments OPNET and NS respectively. A detailed description of the main characteristics of NS and OPNET is given in the first part of appendixes F and G respectively.

### IP based network simulator

The IP-based networks simulation models developed with the support of NS and available in IMEC concern the following two aspects of IP networks:

- The study of the influence of recovery time on TCP flows

- The analysis of the time it takes OSPF to recover from link failure.

In general models developed with the support of NS, as they can employ a fairly detailed traffic model, could be used to validate numbers used as parameters for more high-level simulations or planning algorithms. Such parameters values (throughput for instance) may be used as input for capacity planning algorithms.

In order to consider the use in planning context the first model could be used in order to find out whether fast protection is really necessary in networks carrying TCP/IP traffic, while the second for evaluating link recovery times when OSPF is running on a IP-based network.

More details of both the available models are given in part 2 of appendix F.

### DPT simulator

The DPT simulator owned by UPC was mainly developed for evaluating the performance of the SRP fairness algorithm (SRP-fa).

Basically, the SRP-fa controls the access to the shared media ensuring fairness, bounding latency and avoiding privileged nodes or conditions while undertaking to prevent congestion.

The current version of DPT simulator allows to verify some DPT features such as bandwidth efficiency, fairness among the different nodes of the ring, support for priority traffic, etc.

For network planning purposes simulation results that can be obtained with the SRP-fa algorithm simulation model could be useful for supporting the following activities:

- evaluation of the maximum number of nodes of a ring when the traffic generated by a node is known. In fact, if we assume that each node forming part of a DPT ring connected through a given type of transmission system  offers a certain amount of traffic, the higher is the number of  nodes that constitute a ring the less is the traffic that could be offered by a node. This is true when a certain level of service is imposed.

- evaluation of the maximum traffic that could be generated by each router that forms part of a DPT ring when the number of nodes of the ring is fixed, the transmission interconnecting system is assigned and the traffic is supposed (for instance, but not necessarily) equally directed to the other routers of the ring.

- a-posteriori evaluation of the performance of a given DPT network (traffic, number of nodes, transmission systems are supposed assigned) in order to validate a network structure obtained without any other design supports or with low-accurate planning tools. Following that approach refinements and changes of the project are then possible.

The other simulator concerning DPT technology (not yet available but under development at UPC) is designed to perform studies on the time IPS takes to recover from link and node failures, and the impact of the failures on the packet loss rate.

ISP provides the ability of SRP ring to recover from events and faults as fibre cuts or node failures. IPS is able to monitor and handles events at layers 1, 2 and 3 instead of just layer 1 events monitored and recovered by SONET/SDH protection.

This kind of simulator could be employed in a planning context too, for instance in order to evaluate in a DPT network the recovery performance and , eventually, the opportunity of an additional protection mechanism at the lower layer (SDH transmission layer using its own protection, for instance).

## A tool for strategic planning

OPTIMUM is a tool that enables its users to evaluate economic features such as investments, revenues, life cycle costs, cash balance and so on connected with the construction and running  of the network in a given period.

For a detailed description of the main characteristics of the tool see Appendix H.

In short, the main strength of the tool seems to be the accurate financial analysis of the project, performed through a set of parameters like cash balance, net present value (NPV), internal rate of return and life cycle cost. The weaknesses are that the tool uses a very generic (it is not devoted to IP network) and not very accurate model for the network and that the portion of network considered for the investments is the access portion and not the backbone, which is the interesting aspect of our investigation.

# APPENDIX A: IP Networks Planner

The IP Networks Planner is a software module that implement the well known Kleinrock’s approach to the computer networks design problem, referred to in the literature as TCFA (Topology, Capacity and Flow Assignment problem)[A1].

Modelling assumptions are:

- nodes in the networks exhibit null or constant packets processing delay;

- queueing at each node is FIFO with independent, exponential distributed service times with a given mean;

- packets arrival processes at each node are independent Poisson processes;

- network cost is associated to links and have a linear dependency on the distance and linear or concave dependency on the capacity.

The aim of the tool is to find a minimum cost network able to carry the traffic demands, satisfying a performance constraint on the average delay experimented by packets traversing the network.. Additional features are:

- capability of adjusting the links capacity taking into account a second-order statistic on the packet length;

- support of mono and bi-directional links;

- filtering capabilities on the found solutions.

## IP Networks Planner description form

General information

| Name | IP Networks Planner |
| --- | --- |
| Company | CSELT SpA (IT) |
| HW & SW environment | HW: - any platform SW: - any OS - ANSI C compiler Interface: I/O with text files. No window or graphical interface. |

Modelled Network Layers

| Possible stacks of layers modelled | Layer 3 (IP) only |
| --- | --- |

Demand matrices

| Layer 3 |  |
| --- | --- |
| Type of matrix (meaning of the elements, unit of measurement) | For each source-destination pairs the packets emission frequency |
| Associated parameters | packet length, mean and variance (optional) |

Network topology

| Layer 3 |  |
| --- | --- |
| Physical/logical topology models | One topology level: only a general mesh starting topology can be defined; the optimisation process tends to reduce the meshing |
| Subnetworks/domains definition/interconnection | Not applicable |

Recovery mechanisms available

| Layer 3 |  |
| --- | --- |
| Recovery mechanism | Not modelled |
| Recovery time | Not applicable |
| others |  |
| Co-ordination of recovery mechanisms among layers (if applicable) | Not modelled |

Routing algorithm

| Layer 3 |  |
| --- | --- |
| General approach | Shortest path |
| Protocol/algorithm | OSPF like; i.e. any shortest path routing protocol with administrative weights associated to links. |
| Route diversity | Not available |

Grooming/traffic aggregation rules

| Layer 3 |  |
| --- | --- |
| Rules of grooming/traffic aggregation | Not available |

Network dimensioning

| Layer 3 |  |
| --- | --- |
| Traffic models | Poisson arrivals of packets with exponential length distribution |
| Models of network elements (equipment, systems, cable/infrastructure) | Only queueing at outgoing links is modelled. Queueing is FIFO. |
| Topology optimisation strategy | The optimisation process tends to reduce the topology deviating traffic flows over more convenient paths and eliminating links not traversed by traffic any longer. |
| Dimensioning optimisation approach | Given a routing plan, capacity assignment is performed using the Lagrangian method. |
| Greenfield dimensioning and/or dimensioning of existing networks | Greenfield only. Only the starting topology can be defined; the optimisation process won’t surely increase the meshing. |
| Dimensioning of resources in case of protection/restoration | Not applicable |
| Other information |  |

Cost models

| Layer 2: Client Layer |  |
| --- | --- |
| Cost models of network elements | None |
| Cost models of the lower layer transport services | Continuous cost function: liner with respect to the physical distance, linear or concave with respect to the capacity; Discrete cost function: can be defined as a matrix where rows correspond to values of capacities available and columns to a given distance (intermediate distances must be referred to the shortest distance associated to a column, greater than the considered one) |
| others |  |

Input

| Layer 3 |  |
| --- | --- |
| Main input parameters/information | Three files: Traffic file (traffic matrix and packet length statistics) Topology file (starting topology defined as a matrix of distances between nodes; 0 means “link between nodes is forbidden”) Cost file (matrix of discrete costs as explained above) |
| Import from external input files | YES |
| Format of input files (if any) | Text files |

Output

| Layer 2: Client Layer |  |
| --- | --- |
| Main output parameters/information | Optimised topology; Links capacity and utilisation; Administrative weights for the links Routing paths. |
| Export to external output files | YES |
| Format of output files (if any) | Text files |

Capacity/performance limits

| Layer 2: Client Layer |  |
| --- | --- |
| Maximum number of nodes | There is not a predefined limit on number of nodes because the memory is dinamically allocated. |
| Maximum number of circuits | The same as above |
| Other significant limits | Runtime depends on the number of nodes and on the number of demands to be routed. |

References

[A1]	L. Kleinrock “Queueing Systems”, vol. II, J. Wiley & Sons, 1976

# APPENDIX B: CCN Optimiser

Cross Connected Network (CCN) Optimiser is a computer program designed to optimise the topology and sizing the link groups of a generic cross connected meshed network (Network which implements Deterministic Multiplexing). It is not focused on a particular technology. Optimisation of ATM, SDH and WDM networks are some examples for which CCN Optimiser can be employed.

Only two network layers are considered by the program: a generic client layer which relies on a generic server layer.

CCN Optimiser requires a definition of the cross connecting nodes, the definition of the demands for permanent connections (circuits of the client layer, each one characterised by source, destination, directionality, maximum hops permitted along its path), the cost of transport services offered by the server layer.

On the basis of these inputs, CCN Optimiser provides

- the optimal topology of the network (set of topology edges)

- the number of links required on each edge

- the optimal path for each circuit

The following description form deals mainly with dimensioning capabilities of CCN Optimiser.

## CCN Optimiser description form

General information

| Name | CCN Optimiser |
| --- | --- |
| Company | CSELT SpA (IT) |
| HW & SW environment | HW: Platform: PC SW: OS: Microsoft Windows Language: numerical algorithm developed in C++ (Microsoft Visual C++) Interface: the executable file read and write from/to text files. No window or graphical interface. |

Modelled Network Layers

| Possible stacks of layers modelled | Layer 2: Generic Client Layer Layer 1: Generic Server Layer |
| --- | --- |

Demand matrices

| Layer 2: Client Layer |  |
| --- | --- |
| Type of matrix (meaning of the elements, unit of measurement) | List of Circuits (to be transported over the network. (source, destination, bandwidth, directionality, maximum number of hops in the route) |
| Associated parameters | None |
| Layer 1: Server Layer |  |
| Type of matrix (meaning of the elements, unit of measurement) | List of transport links used to transport the Client layer resources (Circuits). For example list of STM1 links between nodes. Multiple parallel links on the same topology edge could be used between a couple of nodes. It is an output of the program. |
| Associated parameters | Capacity of the link (only one kind of link can be defined within the same network) and cost of a single module between each couple of nodes (matrix of link costs). They are inputs for the program |

Network topology

| Layer 2: Client Layer |  |
| --- | --- |
| Physical/logical topology models | The list of traffic demands are freely defined between network |
| Subnetworks/domains definition/interconnection | Not applicable |
| Layer 1: Server Layer |  |
| Physical/logical topology models | A mesh, without restrictions |
| Subnetworks/domains definition/interconnection | None |

Recovery mechanisms available

| Layer 2: Client Layer |  |
| --- | --- |
| Recovery mechanism | Not modelled |
| Recovery time | Not applicable |
| others |  |
| Layer 1: Server Layer |  |
| Recovery mechanism | Not modelled |
| Recovery time | Not applicable |
| others |  |
| Co-ordination of recovery mechanisms among layers (if applicable) | Not modelled |

Routing algorithms

| Layer 2: Client Layer |  |
| --- | --- |
| General approach | Static routing using a single route. CCN Optimiser determines the single path each VP is routed along. |
| Protocol/algorithm | The program doesn’t consider any particular type of routing protocol: the program find the routes which minimise the network cost, satisfying at the same time the maximum hop constraints |
| Route diversity | Not available |
| Layer 1: Server Layer |  |
| General approach | Not modelled |
| Protocol/algorithm | Not applicable |
| Route diversity | Not applicable |

Grooming/traffic aggregation rules

| Layer 2: Client Layer |  |
| --- | --- |
| Rules of grooming/traffic aggregation | Not available |
| Layer 1: Server Layer |  |
| Rules of grooming/traffic aggregation | Not available |

Network dimensioning

| Layer 2: Client Layer |  |
| --- | --- |
| Traffic models | Deterministic multiplexing of CBR Circuits on transport resources. Transport resources (links) are modelled as bi-directional link of given capacity |
| Models of network elements (equipment, systems, cable/infrastructure) | Nodes: not modelled Links: simple model taking into account capacity and cost |
| Topology optimisation strategy | Link group topology optimisation is performed during the VP routing and link group sizing process |
| Dimensioning optimisation approach | VP routing and link group sizing is performed by a heuristic algorithm based on local search and tabou search techniques |
| Greenfield dimensioning and/or dimensioning of existing networks | Greenfield only |
| Dimensioning of resources in case of protection/restoration | Not applicable |
| Other information |  |
| Layer 1: Server Layer |  |
| Traffic models | None |
| Models of network elements (equipment, systems, cable/infrastructure) | None |
| Topology optimisation strategy | None |
| Dimensioning optimisation approach | None |
| Greenfield dimensioning and/or dimensioning of existing networks | None |
| Dimensioning of resources in case of protection/restoration | None |
| Other information | None |

Cost models

| Layer 2: Client Layer |  |
| --- | --- |
| Cost models of network elements | None |
| Cost models of the lower layer transport services | Cost of a single link module between each couple of nodes (link cost matrix) |
| others |  |
| Layer 1: Server Layer |  |
| Cost models of network elements | Not taken into account |
| Cost models of the lower layer transport services | Not applicable |

Input

| Layer 2: Client Layer |  |
| --- | --- |
| Main input parameters/information | circuits of the client layer, each one characterised by source and destination nodes, directionality, maximum hops permitted along its path, matrix of cost for transport services offered by the server layer (it is possible to define only one kind of transport link of a given capacity). |
| Import from external input files | YES |
| Format of input files (if any) | Text files |
| Layer 1: Server Layer |  |
| Main input parameters/information | Not applicable |
| Import from external input files | Not applicable |
| Format of input files (if any) | Not applicable |

Output

| Layer 2: Client Layer |  |
| --- | --- |
| Main output parameters/information | Transport resouces (number of links on each edge), link utilisation, circuit routes |
| Export to external output files | YES |
| Format of output files (if any) | Text files |
| Layer 1: Server Layer |  |
| Main output parameters/information | Not applicable |
| Export to external output files |  |
| Format of output files (if any) |  |

Capacity/performance limits

| Layer 2: Client Layer |  |
| --- | --- |
| Maximum number of nodes | There is not a predefined limit on number of nodes because the memory is dinamically allocated. Up to now the program has been used for networks whit 45 nodes or less. |
| Maximum number of circuits | For the same reason reported above there are not limits on number of circuits. Up to now the program ghas been run for network with 2000 circuits or less. |
| Other significant limits | Runtime depends on the number of nodes and on the number of circuits to be routed. Network with 45 nodes and 2000 circuits takes a couple of hours to reach a stable solution. The user can define a maximum CPU time to find a solution. |
| Layer 1: Server Layer |  |
| Maximum number of nodes | The same as layer 3 (50 nodes) |
| Maximum number of link groups | Fully mesh physical topology for 50 nodes (1225 link groups) |
| Other significant limits | Up to 10 types of connections grouped in a up to 5 classes of switching connections. The layer2 dimensioning algorithm is faster than the layer 3 algorithm and run in |

# APPENDIX C: SIREN-PLAN-SDH/RING

The SIREN-PLAN/RING System is a support system for the planning of SDH ring networks.

It allows to define network structures starting from a network model whose components are a network topology (nodes + segments), infrastructures, carriers and cables, a set of traffic requirements, and a set of  cost and reliability parameters for each network element.

SIREN-PLAN/RING allows to calculate, evaluate and compare different solution options in order to choose the optimal network structure after comparing each solution with a set of parameter values.

The solution selected by the operator becomes a reference guideline during the network creation process.

An available addition module to SIREN-PLAN-SDH(RING) system is called WDM.

This module permits to find out, inside the network, fibre critical points; it analyzes and compares planning SDH solution with the use of fibre with WDM point-to-point systems (λ = 4, 8, 16, 32) and then it chooses and presents the best solution.

## SIREN-PLAN-SDH/RING description form

General information

| Name | SIREN-PLAN-SDH/RING |
| --- | --- |
| Company | Sirti S.p.A. (IT) |
| HW & SW environment | HW: SUN (Sparcstation o UltraSparc) SW: OS: UNIX Sun Solaris C, C++ compiler DB: ObjectStore (Object Oriented) Softw. Tool: Power Model UI: Microstation |

Modelled Network Layers

| Possible stacks of layers modelled | Layer 1 SDH Layer 0 WDM |
| --- | --- |

Demand matrices

| Layer 1 |  |
| --- | --- |
| Type of matrix (meaning of the elements, unit of measurement) | Point-to-point and point-to-multipoint request flows at 2,34,140 and 155 Mbit/s bit rates between network transmission nodes. |
| Associated parameters | Each matrix has the following features: name description speed type of protection reference year network graph flow demand flow usage type flow protection mode |
| Layer 0 |  |
| Demand matrices | The same as Layer 1 |

Network topology

| Layer 1 |  |
| --- | --- |
| Physical/logical topology models | SDH rings. The system can manage also rings collapsed in segments. |
| Subnetworks/domains definition/interconnection | The application software supports the management of hierarchical or flat networks. The rings can be interconnected by single or dual nodes. |
| Layer 0 |  |
| Network topology | The system compares SDH and WDM solutions and, if it is the case, it plans point-to-point WDM. |

Recovery mechanisms available

| Layer 1 |  |
| --- | --- |
| Recovery mechanism | Reliability parameters(MTBF and MTTR) permit the user to choose the optimal network structure comparing reliability/safety data. |
| Recovery time | Not applicable |
| others |  |
| Layer 0 |  |
| Recovery mechanism | Not modelled |
| Co-ordination of recovery mechanisms among layers (if applicable) |  |

Routing algorithms

| Layer 1 |  |
| --- | --- |
| General approach | The application software allows the operator to choose among 4 different routing typologies: Vertical routing Horizontal routing Minimum number of crossed rings Minimum length |
| Protocol/algorithm | The routing of flow demands is reduced to the problem of finding one optimum path (Dijkstra’s algorithm) or two separate optimum paths (Sourballe’s algorithm). |
| Route diversity | The system routes flow demands with one of the following protection types: 1+1 Drop & Continue 1+1 diversified protection two ways diversified protection no protection |
| Layer 0 |  |
| Routing algorithms | The same as Layer 1 |

Grooming/traffic aggregation rules

| Layer 1 |  |
| --- | --- |
| Rules of grooming/traffic aggregation | The user can define grooming groups (services that can be transmitted together in the same VC-4s), their grooming thresholds and protection level (HO or LO). |
| Layer 0 |  |
| Rules of grooming/traffic aggregation | The same as Layer 1 |

Network dimensioning

| Layer 1 |  |
| --- | --- |
| Traffic models | SNCP, B-SNCP and MS-Spring rings architectures are managed. |
| Models of network elements (equipment, systems, cable/infrastructure) | Equipments: ADM-1,ADM-4,ADM16, ADM16C, ADM-64, DXC, regenerators Ring systems: STM-1,STM-4,STM-16,STM-64 Optical fibres, radio links, pipes, submarine,.. |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach | Economic/financial data, inserted as parameters by the user, are used in order to have the best cost/benefit ratio. |
| Greenfield dimensioning and/or dimensioning of existing networks | The system can plan new network architectures or also incremental flows on already existing network. |
| Dimensioning of resources in case of protection/restoration | Not applicable |
| Other information |  |
| Layer 0 |  |
| Models of network elements (equipment, systems, cable/infrastructure) | Equipments: WDM-MUX, WDM-DEMUX, boosters, line optical amplifiers, Fabry-Perot filters, Transponders, ecc. |
| Dimensioning optimisation approach | Extra parameters that are considered inside WDM are: Cost for the use of existent fibres Cost for lay new fibre Cost for lay new pipe Cost for the use of existent pipe Cost for lay new duct |

Cost models

| Layer 1 |  |
| --- | --- |
| Cost models of network elements | The system makes the sum of all planned elements cost |
| Cost models of the lower layer transport services | Not applicable |
| others |  |
| Layer 0 |  |
| Cost models of network elements | The same as Layer 1 |

Input

| Layer 1 |  |
| --- | --- |
| Main input parameters/information | Network graphs (at physical and logical level) Matrixes with their flow requests Protection types and grooming type requested Every equipment, port, ducts, carriers and regenerator type cost and characteristics |
| Import from external input files | The following elements can be imported by the system from ASCII file: - Graphs - Matrixes - Already existing networks - Equipment costs - Equipment characteristics |
| Format of input files (if any) | The ASCII files must be readable from Excel |
| Layer 0 |  |
| Main input parameters/information | Every WDM equipment cost and characteristics must be inserted by the user. |

Output

| Layer 1 |  |
| --- | --- |
| Main output parameters/information | Evaluation parameters for an alternative structure are grouped into five data categories: planned resources technical data economic/financial data reliability/safety data cost/benefit ratio |
| Export to external output files | All the input data detailed before can be also written in ASCII file as output. |
| Format of output files (if any) | The ASCII files must be readable from Excel |
| Layer 0 |  |
| Main output parameters/information | The same as Layer 1 |

Capacity/performance limits

| Layer 1 |  |
| --- | --- |
| Maximum number of nodes for each layer | No limits |
| Maximum number of links for each layer | No limits |
| Other significant limits | The only present limits are: MS-Spring ring nodes limit: 16 (not due to the system but to technology) 60 rings displayed by Microstation |
| Layer 0 |  |
| Other significant limits | The same as Layer 1 |

# APPENDIX D: WDMNetDesign

WDMNetDesign is a tool for the design of meshed WDM networks. The functionality comprises topological design, routing, network dimensioning and evaluation.

The WDM network is modelled in a layered way compliant with ITU recommendation G.805.

## WDMNetDesign description form

General information

| Name | WDMNetDesign |
| --- | --- |
| Company | IMEC |
| HW & SW environment | HW: Platform PC (Windows NT, Windows 95/98) SW: Language: C++ Window interface: MFC |

Modelled Network Layers

| Possible stacks of layers modelled | Layer 4: Digital Client Layer (DCL) Layer 3: Optical Channel Layer (OCh) Optical line systems and optical cross-connects Layer 2: Optical Multiplex Section Layer (OMS) Equipped fibres Layer 1: Optical Transmission Section Layer (OTS) Buildings and cables |
| --- | --- |

Demand matrices

| Layer 4: DCL |  |
| --- | --- |
| Type of matrix (meaning of the elements, unit of measurement) | List of traffic expressed in STM-1/OC-3 (155Mbit/s) to be transported over the network between source and destination. Traffic can also be entered in Mbit/s. Multiple demand matrices can be set-up simultaneously in the network |
| Associated parameters |  |
| Layer 3: OCh |  |
| Type of matrix (meaning of the elements, unit of measurement) | List of traffic expressed in number of optical channels to be transported over the network between source and destination. Bit rate per channel is 2.5 or 10 Gbit/s. Traffic can also be entered in Gbit/s. For converting a demand expressed in Gbit/s into optical channels a threshold (value between 0 and 100%) is used. Multiple demand matrices can be set up simultaneously in the network. |
| Associated parameters |  |
| Layer 2: OMS |  |
| Type of matrix (meaning of the elements, unit of measurement) | Not applicable |
| Associated parameters |  |
| Layer 1: OTS |  |
| Type of matrix (meaning of the elements, unit of measurement) | Not applicable |
| Associated parameters |  |

Network topology

| Layer 4: DCL |  |
| --- | --- |
| Physical/logical topology models | Logical meshed network |
| Subnetworks/domains definition/interconnection |  |
| Layer 3: OCh |  |
| Physical/logical topology models | Meshed network |
| Subnetworks/domains definition/interconnection |  |
| Layer 2: OMS |  |
| Physical/logical topology models | Meshed network |
| Subnetworks/domains definition/interconnection |  |
| Layer 1: OTS |  |
| Physical/logical topology models | Meshed network |
| Subnetworks/domains definition/interconnection |  |

Recovery mechanisms available

| Layer 4: DCL |  |
| --- | --- |
| Recovery mechanism | Dedicated path protection (1+1), path and link based restoration to protect against single link failures. |
| Recovery time | Not applicable |
| others | The protection path will follow a disjoint route in the DCL layer, but this does not imply that the routes are physically disjoint. |
| Layer 3: OCh |  |
| Recovery mechanism | Dedicated path protection (1+1), path and link based restoration to protect against single link failures. |
| Recovery time | Not applicable |
| others | The protection path is guaranteed to follow a physically disjoint route (node or link disjoint). |
| Layer 2: OMS |  |
| Recovery mechanism | Not applicable |
| Recovery time | Not applicable |
| others |  |
| Layer 1: OTS |  |
| Recovery mechanism | Not applicable |
| Recovery time | Not applicable |
| others |  |
| Co-ordination of recovery mechanisms among layers (if applicable) | Not modelled |

Routing algorithms

| Layer 4: DCL |  |
| --- | --- |
| General approach | Maximise throughput at minimum cost. Two routing possibilities: Shortest path routing Routing table with possible routes (e.g. k shortest paths) |
| Protocol/algorithm | Before DCL traffic can be routed, OCh traffic must first be set up. Two routing algorithms: The connections are one by one allocated along the shortest path found. Different options exist to specify the routing cost. For each node pair a list of possible routes is built up (prior to the routing). The algorithm evaluates the candidate routes of the connections and allocates capacity along the cheapest route. Different cost functions are implemented (hop length, fibre length, real cost). The order in which the connections will be routed can be changed. The routing approach can be sequential or distributive. |
| Route diversity | The protection route follows a diverse path in the DCL layer. This does not imply that the protection path will be physically disjoint. |
| Layer 3: OCh |  |
| General approach | Maximise throughput at minimum cost. Two routing possibilities: Shortest path routing Routing table with possible routes (e.g. k shortest paths) Cross-connection with (VWP) or without (WP) wavelength conversion. |
| Protocol/algorithm | Two routing algorithms: The connections are one by one allocated along the shortest path found. Different options exist to specify the routing cost. For each node pair a list of possible routes is built up (prior to the routing). The algorithm evaluates the candidate routes of the connections and allocates capacity along the cheapest route. Different cost functions are implemented (hop length, fibre length, real cost). The order in which the connections will be routed can be changed. The routing approach can be sequential or distributive. |
| Route diversity | Unprotected and diverse path protected routing (link or node disjoint) |
| Layer 2: OMS |  |
| General approach | Not modelled |
| Protocol/algorithm | Not applicable |
| Route diversity | Not applicable |
| Layer 1: OTS |  |
| General approach | Not modelled |
| Protocol/algorithm | Not applicable |
| Route diversity | Not applicable |

Grooming/traffic aggregation rules

| Layer 4: DCL |  |
| --- | --- |
| Rules of grooming/traffic aggregation | Not modelled |
| Layer 3: OCh |  |
| Rules of grooming/traffic aggregation | Not modelled |
| Layer 2: OMS |  |
| Rules of grooming/traffic aggregation | Not modelled |
| Layer 1: OTS |  |
| Rules of grooming/traffic aggregation | Not modelled |

Network dimensioning

| Layer 4: DCL |  |
| --- | --- |
| Traffic models |  |
| Models of network elements (equipment, systems, cable/infrastructure) |  |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach |  |
| Greenfield dimensioning and/or dimensioning of existing networks |  |
| Dimensioning of resources in case of protection/restoration |  |
| Other information |  |
| Layer 3: OCh |  |
| Traffic models | Static demand matrices. |
| Models of network elements (equipment, systems, cable/infrastructure) | Node: Cross-Connect The OXC dimension is specified by the number of bidirectional wavelength ports that can be handled. The internal connectivity is considered to be full in case of VWP, partial for WP. Two options: Discrete OXC: Multiple discrete OXC sizes can be specified (eg a 32 by 32, 128 by 128 stc.) Continuous OXC: it is assumed that any size of OXC exists. An OADM can be used instead of an OXC in nodes with degree 2. Link: Line system with fixed number of wavelengths. |
| Topology optimisation strategy | Not modelled |
| Dimensioning optimisation approach | The network can only be dimensioned for a load in the OCh layer. Network dimensioning can be done with or without reservation of protection paths. For both approaches a shortest path based approach and a heuristic are foreseen. Both 1+1 path protection and path/link restoration algorithms are available. As for the routing algorithms various routing costs can be selected. If an optimisation algorithm is selected, the optimisation aims at reducing the network installation cost maximally, hence rerouting is performed to remove inefficient used line facilities starting with the link incurring the highest redundant cost. |
| Greenfield dimensioning and/or dimensioning of existing networks | Both are possible. In general, the network dimensioning function performs an upgrade dimensioning. The network capacity is upgraded to accommodate the selected traffic forecast. Network facilities and traffic already in place will not be removed or changed during the upgrade neither during a potential optimisation step. |
| Dimensioning of resources in case of protection/restoration | Shortest cycle algorithm or heuristic based on shortest cycle (shortest cycle + optimisation step). |
| Other information | In the WP mode, wavelength assignment is performed. |
| Layer 2: OMS |  |
| Traffic models |  |
| Models of network elements (equipment, systems, cable/infrastructure) |  |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach |  |
| Greenfield dimensioning and/or dimensioning of existing networks |  |
| Dimensioning of resources in case of protection/restoration |  |
| Other information |  |
| Layer 1: OTS |  |
| Traffic models |  |
| Models of network elements (equipment, systems, cable/infrastructure) |  |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach |  |
| Greenfield dimensioning and/or dimensioning of existing networks |  |
| Dimensioning of resources in case of protection/restoration |  |
| Other information |  |

Cost models

| Layer 4: DCL |  |
| --- | --- |
| Cost models of network elements |  |
| Cost models of the lower layer transport services | Not applicable |
| others |  |
| Layer 3: OCh |  |
| Cost models of network elements | Channel cost Line cards at both sides |
| Cost models of the lower layer transport services | Not applicable |
| others |  |
| Layer 2: OMS |  |
| Cost models of network elements | Fibre cost Fixed cost: multiplexers at both sides, measuring and testing the fibre Cost per length: fibre, splices Cost per OA/ Regenerator Location: OA/ Regenerator for a single direction |
| Cost models of the lower layer transport services | Not applicable |
| others |  |
| Layer 1: OTS |  |
| Cost models of network elements | Cable cost Fixed cost: project management cost Cost per length: digging, ducts and cable Cost per OA/Regenerator Location: cabinets, powering, patch panels |
| Cost models of the lower layer transport services | Not applicable |
| others |  |

Input

| Layer 4:DCL |  |
| --- | --- |
| Main input parameters/information | Traffic matrix, routed traffic |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |
| Layer 3: OCh |  |
| Main input parameters/information | Traffic matrix, routed traffic |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |
| Layer 2: OMS |  |
| Main input parameters/information | Fibre topology |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |
| Layer 1: OTS |  |
| Main input parameters/information | Cable topology |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |

Output

| Layer 4:DCL |  |
| --- | --- |
| Main output parameters/information | Topology information (number of nodes, number of lines, network connectivity, node degree, used capacity, total capacity), traffic information (protection, routed capacity,…), information on the connections (load, capacity,…) |
| Export to external output files | Screen output + file output |
| Format of output files (if any) | Excel file |
| Layer 3: OCh |  |
| Main output parameters/information | Connectivity, wavelength usage, topology information (number of nodes, number of lines, network connectivity, node degree, used capacity, total capacity), traffic information (protection, routed capacity,…), information on the connections (load, capacity,…) |
| Export to external output files | Screen output + file output |
| Format of output files (if any) | Excel file |
| Layer 2: OMS |  |
| Main output parameters/information | Topology information (number of nodes, number of lines, network connectivity, node degree, used capacity, total capacity, total fibre length) |
| Export to external output files | Screen output + file output |
| Format of output files (if any) | Excel file |
| Layer 1: OTS |  |
| Main output parameters/information | Topology information (number of nodes, number of lines, network connectivity, node degree, used capacity, total capacity, cable length) |
| Export to external output files | Screen output + file output |
| Format of output files (if any) | Excel file |
| General |  |
| Main output parameters/information | Availability, node cost, line cost, network cost, connectivity information (node degree, node connectivity, line connectivity) |
| Export to external output files | Screen output + file output |
| Format of output files (if any) | Excel file |

Capacity/performance limits

| Layer 4: DCL |  |
| --- | --- |
| Maximum number of nodes |  |
| Maximum number of links |  |
| Other significant limits |  |
| Layer 3: OCh |  |
| Maximum number of nodes |  |
| Maximum number of links |  |
| Other significant limits | Two possible granularities: STM-16 or STM-64 |
| Layer 2: OMS |  |
| Maximum number of nodes |  |
| Maximum number of links |  |
| Other significant limits |  |
| Layer 1: OTS |  |
| Maximum number of nodes |  |
| Maximum number of links |  |
| Other significant limits |  |

# APPENDIX E: WDMRing

WDMRing is a tool for the design of WDM ring networks. The functionality comprises topological design, routing, network dimensioning and evaluation.

The WDM network is modelled in a layered way compliant with ITU recommendation G.805.

## WDMRing description form

General information

| Name | WDMRing |
| --- | --- |
| Company | IMEC |
| HW & SW environment | HW: Platform PC (Windows NT, Windows 95/98) SW: Language: C++ Window Interface: WXWin |

Modelled Network Layers

| Possible stacks of layers modelled | Layer 4: Digital Client Layer (DCL) Layer 3: Optical Channel Layer (OCh) Optical rings (line systems and OADMs) Layer 2: Optical Multiplex Section Layer (OMS) Equipped fibres Layer 1: Optical Transmission Section Layer (OTS) Buildings and cables |
| --- | --- |

Demand matrices

| Layer 4: DCL |  |
| --- | --- |
| Type of matrix (meaning of the elements, unit of measurement) | Not applicable |
| Associated parameters |  |
| Layer 3: OCh |  |
| Type of matrix (meaning of the elements, unit of measurement) | Only a symmetric traffic matrix can be used. List of traffic expressed in number of optical channels to be transported over the network between source and destination. It is assumed that the service layer is using STM-16 SDH systems (16 circuits of 155 Mbit/s per wavelength). |
| Associated parameters | A scale factor can be used to scale all the values of the traffic matrix. A random generator can be invoked to obtain the values in the traffic matrix. All routed traffic will be protected. |
| Layer 2: OMS |  |
| Type of matrix (meaning of the elements, unit of measurement) | Not applicable |
| Associated parameters |  |
| Layer 1: OTS |  |
| Type of matrix (meaning of the elements, unit of measurement) | Not applicable |
| Associated parameters |  |

Network topology

| Layer 4: DCL |  |
| --- | --- |
| Physical/logical topology models |  |
| Subnetworks/domains definition/interconnection |  |
| Layer 3: OCh |  |
| Physical/logical topology models | Interconnected ring network, with or without Drop&Continue interconnection. |
| Subnetworks/domains definition/interconnection |  |
| Layer 2: OMS |  |
| Physical/logical topology models | Meshed network (equivalent to OTS layer). |
| Subnetworks/domains definition/interconnection |  |
| Layer 1: OTS |  |
| Physical/logical topology models | Meshed network. Location of OADM groups (nodes) / cables can be chosen. |
| Subnetworks/domains definition/interconnection |  |

Recovery mechanisms available

| Layer 4: DCL |  |
| --- | --- |
| Recovery mechanism |  |
| Recovery time |  |
| others |  |
| Layer 3: OCh |  |
| Recovery mechanism | All routed traffic is protected. Dedicated Protection with or without Drop&Continue (OCh-DP rings) Three options for the protection mode can be chosen: No D&C D&C level 1: D&C with the possibility of performing the D&C in the same OADM group. D&C level 2: Highest level of interring protection, D&C with the obligation of performing D&C in 2 separated OADM groups. |
| Recovery time | Not applicable |
| others |  |
| Layer 2: OMS |  |
| Recovery mechanism |  |
| Recovery time |  |
| others |  |
| Layer 1: OTS |  |
| Recovery mechanism |  |
| Recovery time |  |
| others |  |
| Co-ordination of recovery mechanisms among layers (if applicable) | Not applicable |

Routing algorithms

| Layer 4: DCL |  |
| --- | --- |
| General approach | Not applicable |
| Protocol/algorithm |  |
| Route diversity |  |
| Layer 3: OCh |  |
| General approach | Traffic can only be routed in the OCh layer. In the rings only WP (Wavelength Path) mode is considered (no wavelength conversion). Between rings wavelength conversion is possible. |
| Protocol/algorithm | Two solution techniques: Shortest path first Optimised ILP: the maximum amount of solutions, the maximum time the ILP solver can spend and the number of routes the algorithm should consider between any source-destination pair can be specified. |
| Route diversity | Working path and protection path are node disjunct. |
| Layer 2: OMS |  |
| General approach | Not applicable |
| Protocol/algorithm |  |
| Route diversity |  |
| Layer 1: OTS |  |
| General approach | Not applicable |
| Protocol/algorithm |  |
| Route diversity |  |

Grooming/traffic aggregation rules

| Layer 4: DCL |  |
| --- | --- |
| Rules of grooming/traffic aggregation | Not modelled |
| Layer 3: OCh |  |
| Rules of grooming/traffic aggregation | Not modelled |
| Layer 2: OMS |  |
| Rules of grooming/traffic aggregation | Not modelled |
| Layer 1: OTS |  |
| Rules of grooming/traffic aggregation | Not modelled |

Network dimensioning

| Layer 4: DCL |  |
| --- | --- |
| Traffic models |  |
| Models of network elements (equipment, systems, cable/infrastructure) |  |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach |  |
| Greenfield dimensioning and/or dimensioning of existing networks |  |
| Dimensioning of resources in case of protection/restoration |  |
| Other information |  |
| Layer 3: OCh |  |
| Traffic models | Static demand matrix (number of wavelengths). |
| Models of network elements (equipment, systems, cable/infrastructure) | Line systems: One type of line system can be specified in terms of number of wavelengths. OADMs: separate from line system, fully non-blocking Transponders between OADMs of interconnected rings. |
| Topology optimisation strategy | Three steps: Ring identification: identification of all possible rings in underlying network (restricted by maximum number of nodes, maximum length,…). Ring selection: Keep rings with best traffic characteristics from the ring identification phase. Ring optimisation: Two possibilities: Exhaustive search of all ring combinations (only for small networks) Tabu Search heuristic |
| Dimensioning optimisation approach | Three possibilities: Shortest path routing Heuristic method ILP method |
| Greenfield dimensioning and/or dimensioning of existing networks | Both are possible. |
| Dimensioning of resources in case of protection/restoration | All traffic on the rings is protected (traffic uses both sides of the ring for working + protection traffic). |
| Other information | Topology optimisation phase and dimensioning optimisation phase are separated. |
| Layer 2: OMS |  |
| Traffic models |  |
| Models of network elements (equipment, systems, cable/infrastructure) |  |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach |  |
| Greenfield dimensioning and/or dimensioning of existing networks |  |
| Dimensioning of resources in case of protection/restoration |  |
| Other information |  |
| Layer 1: OTS |  |
| Traffic models |  |
| Models of network elements (equipment, systems, cable/infrastructure) |  |
| Topology optimisation strategy |  |
| Dimensioning optimisation approach |  |
| Greenfield dimensioning and/or dimensioning of existing networks |  |
| Dimensioning of resources in case of protection/restoration |  |
| Other information |  |

Cost models

| Layer 4: DCL |  |
| --- | --- |
| Cost models of network elements |  |
| Cost models of the lower layer transport services |  |
| others |  |
| Layer 3: OCh |  |
| Cost models of network elements | The following generic cost model is considered: A ring cost for each ring to be deployed: this cost includes the fixed installation costs of the ring, fibre costs, amplifier costs, OADM costs, … A routing cost for routing one wavelength on a ring: this costs accounts for the modular components of the ring that can be installed on a wavelength basis (e.g. tributary cards, transponders). An interconnection cost: the cost for interconnecting a wavelength between 2 rings (e.g. transponder cost). The first two costs are ring specific, the latter not |
| Cost models of the lower layer transport services | Not applicable (single-layer tool). |
| others |  |
| Layer 2: OMS |  |
| Cost models of network elements |  |
| Cost models of the lower layer transport services |  |
| others |  |
| Layer 1: OTS |  |
| Cost models of network elements |  |
| Cost models of the lower layer transport services |  |
| others |  |

Input

| Layer 4: DCL |  |
| --- | --- |
| Main input parameters/information |  |
| Import from external input files |  |
| Format of input files (if any) |  |
| Layer 3: OCh |  |
| Main input parameters/information | Traffic matrix, deployed rings, routed traffic |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |
| Layer 2: OMS |  |
| Main input parameters/information | Fibre topology |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |
| Layer 1: OTS |  |
| Main input parameters/information | Cable topology |
| Import from external input files | Yes |
| Format of input files (if any) | Text files |

Output

| Layer 4: DCL |  |
| --- | --- |
| Main output parameters/information |  |
| Export to external output files |  |
| Format of output files (if any) |  |
| Layer 3: OCh |  |
| Main output parameters/information | Number of wavelengths used, wavelength utilisation, routes of connections (working and protection path), percentage of traffic routed,… |
| Export to external output files | Screen output + restricted file output |
| Format of output files (if any) | Text files |
| Layer 2: OMS |  |
| Main output parameters/information | Number of fibres used |
| Export to external output files | Screen output + restricted file output |
| Format of output files (if any) | Text files |
| Layer 1: OTS |  |
| Main output parameters/information | Number of cables used |
| Export to external output files | Screen output + restricted file output |
| Format of output files (if any) | Text files |
| General |  |
| Main output parameters/information | Cost of used resources in all layers, availability (given certain values for MTBF and MTTR of the network elements) |
| Export to external output files | Screen output |
| Format of output files (if any) |  |

Capacity/performance limits

| Layer 4: DCL |  |
| --- | --- |
| Maximum number of nodes |  |
| Maximum number of links |  |
| Other significant limits |  |
| Layer 3: OCh |  |
| Maximum number of nodes | Less than 100 nodes |
| Maximum number of links |  |
| Other significant limits | Restrictions on size of network for topology dimensioning (calculation time). |
| Layer 2: OMS |  |
| Maximum number of nodes | Less than 100 nodes |
| Maximum number of links |  |
| Other significant limits |  |
| Layer 1: OTS |  |
| Maximum number of nodes | Less than 100 nodes |
| Maximum number of links |  |
| Other significant limits |  |

# APPENDIX F: IP-based network simulator

This appendix includes a description of the simulation tool "Network Simulator", probably more familiar by the name of NS. In a planning context, NS is useful for studying behaviour of technologies and protocols. For example, NS can be used to verify whether fast protection switching is useful or rather harmful in networks carrying TCP/IP traffic.

As NS employs a fairly detailed traffic model, it can also be used to validate numbers used as parameters for more high-level simulations or planning algorithms. For instance, link throughput can be measured rather accurately. Such throughput values, possibly backed up by some real-life measurements, may be input parameters to capacity planning algorithms.

Also, the models of various protocols (e.g. OSPF, TCP, MPLS) included in NS are useful to study for instance the influence of network topology on the convergence of OSPF, the round trip times and resulting throughput for TCP, the time to set up a LSP, etc. Again, these are all considerations to take into account when planning an (IP-based) network.

## PART 1: description of the simulation tool/environment NS

General information

| Name | Network Simulator (NS) version 2, non-commercial (freeware) |
| --- | --- |
| Company | VINT project (http://www.isi.edu/nsnam/vint) |
| HW & SW requirements | A computer with a C++ compiler. Preferably a Unix machine (FreeBSD, Linux, SunOS, Solaris), but it should run on a Windows platform too. |
| Type of simulator | An object-oriented discrete event simulator. |

Modelled Network Elements

| Node sub-components | The function of a more or less real-life node (a router) is split up in following parts: - Routing component(s): calculation of routes, updating of routing tables, … This is done by so-called "routing agents". - Forwarding component(s): forwarding incoming packets to the appropriate outgoing link(s). This is done by a classifier component. - Queuing components: the buffer behaviour. The simulation model covers output buffering schemes. These queues are part of the link model (which is fine, as long as we agree to restrict our simulations to output buffered nodes). |
| --- | --- |
| Nodes | - A node corresponds more or less to a router: in a node routing tables are kept, and routing protocols are active - The node model is quite high-level: it only covers the routing functions; buffering is modelled as part of the link (see below) - No libraries with typical apparatus and/or equipment available on the market |
| Links | - The link model covers the buffering and scheduling parts: the output buffers of a real-life router are a part of the link in the simulation model. - A link has following parameters: - link bandwidth (determines serialisation delay); - propagation delay; - queue type: capacity of buffer and queue management (e.g. RED); - link cost: the weight to be used when calculating (shortest) routes. - Link losses can be accounted for, so-called error modules can be included (error rates in packets or bits). - Apart from point-to-point link models, also LAN models (with MAC) are available. |
| Communication Protocols | NS is aimed to allow the study of scale and protocol interaction in the context of current and future network protocols. As such, various protocols are included: - IP (Internet Protocol) - TCP (Transmission Control Protocol): various variants are included (Tahoe, Reno, Newreno, Vegas, Sack…) - UDP (User Datagram Protocol) - RTP (Real Time Protocol) - SRM (Scalable Reliable Multicast) - MPLS (MultiProtocol Label Switching) has recently been added to NS. |
| Routing Protocols/algorithms | The available routing protocols include: - Unicast routing: - a RIP-like "Distance Vector (DV)" routing protocol; - an OSPF-like "Link State (LS)" routing protocol. - Multicast routing: - "Centralised Multicast" is a sparse mode implementation similar to PIM-SM. - "Dense Mode Multicast" can run in two modes: one is very similar to PIM-DM, the other is loosely based on Distance Vector Multicast Routing (DVRMP). - "Shared Tree" and "Bi-directional Shared Tree" are shared tree multicast protocols, where the rendezvous points of the multicast groups are fixed during the entire simulation (no support of network dynamics). |

Traffic/commodities to be transported

| Model for traffic and/or commodity to be transported | The transported traffic always consists of individually treated packets. |
| --- | --- |
| Traffic sources/generators | Several traffic generators are available: - The "Exponential On/Off" traffic source generates traffic according to an exponential on/off distribution. Packets are sent at a fixed rate during on periods, and no packets are sent during off periods. - A "Pareto On/Off" source generates traffic according to a Pareto on/off distribution. The behaviour is similar to the exponential on/off source, except that the on and off period durations are taken from a Pareto distribution. These sources can be used to generate traffic exhibiting long-range dependency. - The "CBR-traffic" source generates traffic according to a deterministic rate. Packets are of constant size. Optionally, some randomising jitter can be enabled on the inter-packet departure times. - A "TrafficTrace" source generates traffic according to a trace file containing pairs of inter-packet times and packet lengths. - An "FTP" source can be used to simulate bulk data transfer. One can choose to continuously emit packets during a certain time interval, or to generate a specified number of packets. - A "Telnet" source produces individual packets with exponentially distributed inter-arrival times (IATs) or with IATs taken from the "tcplib" telnet distribution. Note: For simulation of HTTP traffic, a "PagePool" model can be used by server models to generate web page information (name, size, modification time, lifetime, etc.), by cache models to describe which pages are in storage, and by client models to generate request stream. Inter-arrival times of requests by the clients will be generated by a random generator of the user's choice. |
| Traffic profile | There are no explicit means to define traffic profiles. The only way it can be done is by using different traffic generators during different time intervals. |

Type of simulations

| Scenarios | NS can be used to simulate both small and larger portions of a network. However, the larger the network and especially, the more traffic it is loaded with the longer it takes to run a simulation. Only layer 3 (or 2.5 when using MPLS) and above are modelled in detail. As all traffic flows are modelled on a packet-per-packet basis, this somewhat restricts the network size for which simulation takes a reasonable time. |
| --- | --- |
| Performance evaluation | As every traffic flow is modelled as a series of packets, measurements of losses, delays and jitter are quite straightforward. |
| Survivability analysis | In NS, only failures are simulated. Node failures are emulated by letting all the links connected to the particular node fail. There are four models available to introduce link failures: - The exponential on/off model lets the link be up and down during intervals whose length is exponentially distributed. - The deterministic on/off model allows alternating the link status between up and down during time intervals of fixed length. - The trace driven model takes the times for links going up and down from a trace file. - The "manual" model allows the user to set the times of link failures explicitly. |

Routing algorithm and Recovery mechanisms

| Recovery mechanism | IP layer recovery: dynamic routing protocols will send out packets with topology updates. Consequently, routing tables will be recalculated. MPLS layer recovery: e.g., one can have pre-established back-up paths, or installation of new paths due to altered layer 3 forwarding. |
| --- | --- |
| Co-ordination of recovery mechanisms among layers (if applicable) | In a way, there is interaction between MPLS and layer 3 is present in NS: when a link failure prevents sending packets along the installed MPLS path, one can choose to fall back to layer 3 routing. In addition, link failures are immediately visible to the (layer 3) routing components at the link's terminating nodes. One could consider this as an interaction between layer 2 (or lower) and layer 3. |

Inputs, Outputs and post processing

| Type of input data | The input to the simulator is a simulation script (a text file) written in the Tcl language. From within that simulation script, other files can be opened and processed, e.g. to create the network topology. Another way to provide simulation parameters is to pass them via the command line. |
| --- | --- |
| Type of data provided | The output of the simulation usually is a set of one or more text files (or some info written to the screen). One can create standard NS trace files containing packet "events" (arrivals, departures, and drops), link failures, queue occupation, etc. It is also possible to write only the information of interest to a file, e.g. the link throughput during a certain interval. |
| Simulation data processing/post-processing tools | To process the output files, one can use standard text processing tools (e.g. awk, perl scripts, …). The output files (i.e. the outcome of post-processing) can then be fed into classical tools to produce graphs (xgraph, gnuplot, MS Excel, …). There is also a Network Animator (nam) available to visualise the packets flowing through the network. |

Capacity/performance limits

| Maximum number of nodes | In theory, the maximum number of nodes is 232, as 32 bits are used for node addresses. In practice, it depends on the memory available and on the amount of traffic flowing through the network. Of course, larger simulation topologies need longer running times. |
| --- | --- |
| Maximum number of links | Per node, an identifier is kept for each interface. For this, again 32 bits are used, so there is a theoretical limit of 232. Again, the available memory will be the real limiting factor. |
| Other significant limits | Runtime depends on the number of subsystems (nodes, links, included in the simulated scenario and on the level of detail used to model each subsystem. |

## PART 2: description of available model/simulation scenarios

Available developed model

| Simulation Scenario 1: Study of protection switching delay on TCP flows | - Context: backbone IP(/MPLS) network - Aim: studying the influence of recovery time on TCP flows in order to find out whether fast protection is really necessary in networks carrying TCP/IP traffic. - Components: backbone nodes are LSRs (IP+MPLS), interconnected by point-to-point links. (Layers below MPLS are not modelled explicitly.) - Routing: we use MPLS paths between source and destination nodes, with a pre-established alternative path for the path on which we introduce a link failure. We vary the delay between the link failure and the switch to the protection path. - Traffic: we use TCP sources, which get a continuous stream of data to transport. (One can see this as a model for FTP transfer of very large files.) |
| --- | --- |
| Simulation Scenario 2: Study of OSPF recovery time after link failure | - Context: backbone IP network - Aim: studying the time it takes OSPF to recover from link failure. (From this, we may draw conclusions regarding the need for lower layer recovery.) - Components: backbone nodes are IP routers, interconnected by point-to-point links. (Layers below are not modelled explicitly.) - Routing: we use a somewhat simplified model of OSPF (the link state routing model that comes with NS, called rtProto/LS). - Traffic: initially, we do not load the network with traffic. This way, we see only the OSPF-generated packets (link state advertisements etc.). By avoiding extra traffic, we speed up the simulation time, thus allowing us to consider larger topologies. |

# APPENDIX G: DPT (SRP-fa and ISP) simulator

This appendix includes a description of the DPT (Dynamic Packet Transport) model built up with the OPNET network simulator tool.

The DPT simulator available at the UPC was mainly developed in order to evaluate the performance of the SRP fairness algorithm (SRP-fa). Basically, the SRP-fa controls the access to the shared media ensuring fairness, bounding latency and avoiding privileged nodes or conditions while undertaking to prevent congestion. Thus, with the current version of our DPT simulator it is possible verify some DPT features such as bandwidth efficiency, fairness among the different nodes of the ring, support for priority traffic, etc.

The DPT model is being extended with the Intelligent Protection Switching (IPS) algorithm. The IPS algorithm is a protection and restoration mechanism that provides the ability of the DPT ring to recover from events and faults such as fiber cuts or node failures. Once the ISP algorithm will be in place, it will be possible to evaluate recovery times and packet loss rates in case of eventual failures.

The Part 1 of this appendix reports the main characteristics of OPNET while the part 2 deals with the two models developed (the second one is still under development) for evaluating the SRP fairness and IPS algorithms respectively.

## PART 1: description of the simulation tool/environment OPNET

General information

| Name | OPNET |
| --- | --- |
| Company | OPNET Technologies, Inc. (http://www.opnet.com) MIL 3, Inc. (http://www.mil3.com) |
| HW & SW requirements | Solaris, Windows NT, Windows 2000, and HP-UX supported OPNET Modeler Requirements version 6.0 http://www.opnet.com/products/modeler/modlr_reqirmnts_60.html OPNET Modeler Requirements version 7.0 http://www.opnet.com/products/modeler/modeler_requirements.html |
| Type of simulator | An object-oriented discrete event simulator. |

Modelled Network Elements

| Nodes | - The nodes implemented in the DPT simulator are mainly the DPT interfaces, and only model the MAC layer protocol (SRP) it is not considered neither the IP layer nor the physical layer. - The packets used as SRP payloads are IP datagrams. - The SDH framing proposed in the initial version of the SRP protocol is not implemented. |
| --- | --- |
| Node sub-components | OPNET may develop any kind of device or network element using the node editor The node of the DPT simulator consists of the following components: - A Receiver: a module realizing the address lookup of each incoming packet and the destination stripping. - A Transit Buffer (Insertion Register): used to forward packets already circulating onto the ring and composed by two finite FIFO queues. - A Transmit Buffer: used to store the SRP packets generated by the node and composed by two infinite FIFO queues. - A Transmitter: a module implementing the transmission algorithm and the SRP fairness algorithm. These elements can be found in both the inner and the outer part of the DPT node. |
| Links | - OPNET offers wireless, point-to-point and multipoint links. - Link behavior is open and programmable. - Accurately account for delay, availability, and throughput characteristics of links. - Incorporate physical layer characteristics and environmental effects. |
| Communication Protocols | OPNET allows to define communication protocols using a finite state machine modeling (simulating any required behaviour with C/C++ logic in FSM’s states and transitions) and offering a comprehensive support for protocol programming (library functions simplifying writing protocol models). Library of detailed protocols: - HTTP (HyperText Transfer Protocol) - TCP (Transmission Control Protocol) - IP (Internet Protocol) - RSVP (Resource Reservation Protocol) - SNA (Systems Networks Architecture) - Token Ring - FDDI (Fiber-Distributed Data Interface) - Ethernet - ATM (Asynchronous Transfer Mode) - 802.11 Wireless LAN - Frame Relay DPT modeled protocols: - SRP (Spatial Reuse Protocol) |
| Routing Protocols/algorithms | OPNET includes the available following routing protocols: - BGP (Border Gateway Protocol) - EIGRP (Enhanced Interior Gateway Routing Protocol) - OSPF (Open Shortest Path First) - RIP (Routing Information Protocol) |

Traffic/commodities to be transported

| Model for traffic and/or commodity to be transported | In an OPNET scenario, the transported traffic always consists of individually treated packets. |
| --- | --- |
| Traffic sources/generators | OPNET offers the following modules in order to model different traffic sources: - A traffic generator in which both the packet size distribution and the interarrival distribution can be defined using any probability density function (pdf). Moreover, OPNET uses a PDF Editor in order to create, edit, and view probability density functions. - A clock generator in order to create traffic with a periodic characteristic. Two traffic sources are available in the DPT simulator: - A Poisson IP traffic source using the traffic generator provided by OPNET and defining an exponential interarrival distribution of packets as a parameter. - An On-Off IP traffic source specifically developed on OPNET for the DPT simulation in order to model burstiness. In both cases the IP packet size distribution was set according real Internet trafficprovided by Lucent |
| Traffic profile | As OPNET has the capability to model any device, it can be used to model specific traffic profiles. Besides the already mentioned traffic sources, other traffic models could be implemented. |

Type of simulations

| Scenarios | OPNET can be used to simulate any kind of network scenarios. It can be used in LAN, MAN and WAN applications. Since, typically the DPT solution is aimed to the metropolitan area market, the DPT simulator focus in a MAN scenario. |
| --- | --- |
| Performance evaluation | OPNET can evaluate any kind of performance measure. Furthermore, OPNET has incorporated the calculation of determined statistics associated with several modules such as queues, ports and links. The DPT simulator provides network throughput and packet delays figures. |
| Survivability analysis | A survivability analysis of a DPT ring is not implemented yet. |

Routing algorithm and Recovery mechanisms

| Recovery mechanism | The Intelligent Protection Switching (IPS) algorithm, which provides protection and restoration to a DPT ring, is under construction. |
| --- | --- |
| Co-ordination of recovery mechanisms among layers (if applicable) | The DPT simulator only deals with the MAC layer. Thus, besides the implementation of the IPS algorithm, to check the co-ordination among layers would also require the implementation of the recovery mechanisms of both the network layer (IP) and the physical layer (SDH). |

Inputs, Outputs and post processing

| Type of input data | All the simulation parameters are defined within OPNET. However, the simulation makes use of two external files in order to load the traffic matrix of each node and to define some global constants and variables. |
| --- | --- |
| Type of data provided | The output of the simulation can be either a vector file or a scalar file. Both files can be only processed by OPNET. |
| Simulation data processing/post-processing tools | OPNET offers an integrated analysis tool to display simulation results. These results can also be exported to spreadsheets. |

Capacity/performance limits

| Maximum number of nodes | The SRP protocol makes use of 48-bit MAC addresses so the maximum number of nodes is 248. The number of nodes used may be a limitation for ther DPT simulations. This depends on the memory available, |
| --- | --- |
| Maximum number of links | There is no limit in the number of links to be used. |
| Other significant limits | As the OPNET simulator is an event-driven, runtime depends on the number of events to be treated. The greater the events are the longer the simulation takes. The number of events can vary according to the number of nodes of the network, the complexity (level of processing) of these nodes, the offered load, etc. |

## PART 2: description of available model/simulation scenarios

Available developed model

| Simulation Scenario 1: Performance evaluation of the SRP-fairness algorithm | - Context: Metropolitan Area Application. However, the model could be used in LAN and WAN environments. - Aim: To evaluate the performance of the SRP-fa in terms of network throughput and packet delays and verify bandwidth efficiency, fairness among nodes and support for priority traffic within a DPT ring. - Components: DPT nodes forming a DPT dual-ring network. The links are point-to-point. Upper and lower layers from the SRP layer are not modelled. - Offered Load: Two traffic sources are used. A Poisson traffic source and an On-Off traffic source modelling IP packet streams. - Load Distribution: In order to check the DPT networks under two different conditions, two traffic load distributions can be selected, namely balanced load distribution (all against all) and unbalanced load distribution (all against one). |
| --- | --- |
| Simulation Scenario 2: Evaluation of the IPS recovery mechanism | - Context: Metropolitan Area Application. However, the model can be used in LAN and WAN environments. - Aim: To study the time IPS takes to recover from link and node failures, and the impact of the failures on the packet loss rate. - Components: DPT nodes forming a DPT dual-ring network. The links are point-to-point. Upper and lower layers from the SRP layer are not modelled. - Offered Load: Two traffic sources are used. A Poisson traffic source and an On-Off traffic source modelling IP packet streams. - Load Distribution: In order to check the DPT networks under two different conditions, two traffic load distributions can be selected, namely balanced load distribution (all against all) and unbalanced load distribution (all against one). - Failure Triggers: The failures are generated randomly without the need of using SDH framing. |

# Appendix H: OPTIMUM

## Key Inputs

### Architecture scenario

The architecture scenario indicates how the network is built out during the study period. The amount of equipment needed in the network should be defined and, the time when the various components actually are implemented.

### Service scenario

The service scenario defines how many subscribers are connected to a certain service at a certain year and how much revenue one service user means for the network operator per year. The penetration of the service, expressed in terms of number of users versus time is a key input to the calculation tool of OPTIMUM.

### Subscriber density

The OPTIMUM methodology contains the possibility to use a geometric model to calculate the cable and duct lengths in the network. The main input to the geometric modelling is the subscriber density. This parameter is often used to study the sensitivity of the cost of the system to various geographic conditions.

### Service demand

The demand for the applications depends on the tariff. One possibility is to investigate the willingness to pay for given applications. This leads to demand forecast which in turn allow the calculation of penetrations based on the calculated costs of providing the service.

Another possibility is to estimate the maximum potential use for the applications and then make an S-curves (logistic model etc.) as an input to the forecasts.

The third alternative is to enquire directly by the potential customers about their expected forecasts for the applications.

Another alternative is to make a simpler analysis focused on the development of demanded capacity for the customers and not look too much at the applications. Based on historical data and also subjective evaluations, forecasts of demand can be produced.

A mix of the methods suggested may be used or one specific method which may change from business case to business case.

The demand will vary depending of the type of company. It is possible to identify the variation if the survey is carried out among different type of companies.

#### Traffic

Some assumptions have to be made regarding traffic generated from each application. For some applications the estimated traffic characteristics can be found in the literature. It is also possible to differentiate these results by carrying out a market survey asking different type of companies about their expected traffic behaviour.

#### Demographic Characteristics.

The traditional classification of companies is the «Standard Industrial Classification» (SIC). The classification is given down to 6 digits. In the National Bureau of Census all establishments in the country are classified by this standard. Because of limited number of establishment in the business cases it is recommended only to use a one digit classification.

Other important characteristics for the communication need are the following:

- Number of employees (Especially number of white collar workers)

- Economic budget

- Telecommunications budget

A stratification of the companies can be made based on the given characteristics. The total number of strata must be limited because of the size of market survey for the different business cases.

#### Geographic demand

If a comprehensive market survey is carried out and the results from each company is grouped in defined demographic strata, it is possible to estimate the communication need also in other geographic areas by taking into account the distribution of number of companies in each strata.

#### Traffic between geographic areas

The approach so far has been to estimate the subscription and traffic demand generated in each area. Another aspect is to estimate the traffic between geographic areas.  One  method for making traffic matrix forecasts between geographic areas based on accumulated broadband traffic (generated from separate geographic areas), is to map the broadband traffic proportional with some correlated traffic - for example the financial transaction flow between the same geographic areas.

## Key Outputs

In the evaluation of investment scenarios there can be several points of view. Depending on the complexity of the scenario various commonly used indicators, like Payback Period, NPV or IRR, can give differing results in comparisons. Because of this, it is often necessary to use several figures of merit for the studies to get an thorough understanding of the economic issues related to each scenario.

In most of the cases these evaluations have some less known inputs. In these cases it is advisable to apply sensitivity analysis and/or Risk assessment methodologies to these inputs using multiple figures of merit as indicators.

### Revenues

In most of the evaluations the calculation includes the revenues from the services. OPTIMUM methodology handles the revenues simply by estimating a certain annual tariff for each service per connected customer. However, it must be noted that revenues in OPTIMUM refers to the part going to the access operator, not the total amount customers will pay for their telecommunications services.

The above revenues input is critical for the subsequent calculation of further economic outputs like Profits, Retained Profits and Cash Flows.

### Investments

The first output of the OPTIMUM methodology is the investment cost of the network project. Because the OPTIMUM methodology studies scenarios, the investments are usually spread over the study period. To get a single figure of merit for the total investment, the future investments are discounted to the start of the study period using the conventional discounting formula. The total discounted investment cost is usually called First Installed Cost.

In the OPTIMUM methodology the network is subdivided into a hierarchy of flexibility points and link levels. Links interconnect flexibility points. All links or flexibility points in the same hierarchical level form a so-called network level.

The current implementation of the methodology allows the investments to be analysed based on physical location of the cost components in the network (by hierarchical network level).

### Life Cycle Cost

Life Cycle Cost is defined as the sum of global discounted investments and global discounted running costs. This gives the total costs for constructing and running the network over the study period.

Often the sole reason for a techno-economic study is the evaluation of the cost of the proposed network build. The reason might be a new network or an upgrade to the existing network. In these cases it is not always necessary to add any service revenues to the study and the final result of the study can be either First Installed Cost or Life Cycle Cost of the project. In this case this methodology and the tool are especially suitable for comparative studies between competing network technologies.

A typical Cash balance curve for a network scenario goes first deeply down to the negative side because of the high initial investments. If the scenario is profitable, the cash flow turns positive fairly soon and the Cash Balance curve starts to rise. The lowest point in the Cash Balance curve gives the amount of funding required for the project. The point in time when the Cash Balance turns positive gives the Payback Period for the project. This is the usual case when studying a single technology scenario in green field situation.

In an investment scenario where most of the expenditure happens in the beginning of the study period, the Payback Period gives a good indication of the efficiency of the investment. If the scenario is more complex, that is, if there are for example several technology steps in an upgrade situation, it is not possible to define a single Payback Period. It is still possible to use the Cash Balance curve as an indicator for the profitability of the scenario. In these cases it is important to study the trend of cash flow at the end of the study period.

### Cash Balance

The Cash Balance (Cumulative Cash Flow) time-series is a very informative figure for a network and service scenario. Especially for a green field case it gives much information in a single picture.

### Net Present Value (NPV)

The Net Present Value gives a single figure of merit for a project. Its definition is the Sum of Discounted Cash Flows plus Discounted Rest Value of the project. It gives the monetary value of the whole project in today’s money. It is a good indicator for the profitability of the scenario especially in these cases where the Payback Period cannot be used because major investments are spread out in time.

The weakest point in this figure of merit is the definition or calculation of the rest value of the network. There are several ways to try to define this value. The usual approach uses the bookkeeping value of the network as the Rest Value because it is the only figure that can be calculated from the inputs already available.

### Internal Rate of Return (IRR)

IRR is the discount rate at which the NPV is zero. If the IRR is higher than the opportunity cost of money (that is, interest of an average long term investment), the project is viable.

If the scenarios to be compared are not similar enough, for example if the size of these networks is different, these can not be easily compared using Net Present Values. In these cases Internal Rate of Return gives a good indication for how “good value for the money” these networks give.

### Geometric modeling

The geometric model is used to estimate the amount of cable/fibre and ducting required in the network. On a conceptual level the geometric model is a function that takes several inputs such as subscriber density, network topology (star, ring, bus), average cable over length, duct availability, etc. and gives two outputs, which are the total amount of cable/fibre required in the network and the total amount of new duct required. Various geometric models were developed and used in the TITAN project. The most advanced model (TITAN model) is implemented in this tool and described below. We renamed the model to OPTIMUM model.

## The tool database

The database includes costs at a given reference year for components, installation, civil works, operations and maintenance. The price-curve for each cost component can be calculated by means of learning curve data and volume data. Learning curve data and volume data are classified in relevant groups. This simplifies the addition of new components and makes it easier to compare price evolution of various components with different functionalities. For each component, a learning curve class and volume class have to be specified in order to predict the price curve. The classes are stored in the database.

Another important financial figure is the depreciation. The depreciation time is also classified in a few depreciation classes. The class number refers to the number of years in which a component is completely depreciated.

The final financial figure is the OAM-class. This class specifies the percentage of the investment that contributes to the yearly running cost of the network.

Other fields are:

1.	Cost unit, or the currency of the cost data

2.	TypeID, a type identification of the component

3.	ConfClass, The confidence class of the cost component which defines the uncertainty and the relative standard deviation in the price

4.	Importance, the financial importance of the cost component.

The cost evolution of every components in the cost database is described by the following parameters:

- Price (cost) at a given reference year

- Reference year

- «Learning curve class » which gives the cost of the component as a function of produced volume. One parameter K (optimistic or pessimistic)

- «Volume class» which gives volume as a function of time (forecasted volume) 2 or 3  parameters D t, n(0)  and g  characterise the volume class

- D t - the time it takes for the  accumulated production volume to  increase from 10% to 90% of total accumulated production volume

- accumulated production volume by t=0 (relative to total accumulated volume)

- g  - described the asymmetry of the logistic curve

It is worth noting that not all functionalities offered by the Spatial Reuse Protocol are implemented.

Paul Langner, “SDL Data Link Specification”, Lucent Technologies – White Paper.

The extension of the available DPT simulator to include the IPS algorithm is under construction.