Project P803-PF

European IP Testbed (definition phase)

Deliverable 1

Proposal for Setting-up the Testbed

Suggested readers:

Participants in P803-PF

Organisations aiming to install international IP Testbeds in Europe

Organisations aiming at corporate advanced IP development in Europe

R&D engineers working on advanced IP network solutions

People interested in “hot” IP test topics

May 1998

EURESCOM PARTICIPANTS in Project P803-PF are:

Belgacom, S.A. de Droit Public

BT

Deutsche Telekom AG

FINNET Group

France Télécom

Koninklijke PTT Nederland N.V.

MATÁV Hungarian Telecommunications Company Ltd.

Tele Danmark A/S

Telecom Eireann

This document contains material which is the copyright of certain EURESCOM PARTICIPANTS, and may not be reproduced or copied without permission

All PARTICIPANTS have agreed to full publication of this document.

The commercial use of any information contained in this document may require a license from the proprietor of that information.

Neither the PARTICIPANTS nor EURESCOM warrant that the information contained in the report is capable of use, or that use of the information is free from risk, and accept no liability for loss or damage suffered by any person using this information.

This document has been approved by EURESCOM Board of Governors for distribution to all EURESCOM Shareholders.

Preface

(Prepared by the EURESCOM Permanent Staff)

The Definition Phase of the P803 Project was scheduled to last 3 months in order to clarify the goals of the next phase (implementation phase). Rather than continuing the previous P716 Project, it was felt that a clearly delimited and separated definition phase would simplify administration.

Nine Shareholders took part in the definition phase. (AF, BT, DK, DT, FT, HT, NL, RB, TI). The Project Leader was Rüdiger Geib (Deutsche Telekom).

This Deliverable 1 contains a description of WHY a European IP Testbed should be built, HOW it could be built and WHAT it could be used for. This These topics was were also described in the P716 Deliverable 1. and t This Deliverable supersedes the Deliverable of P716, but the this Deliverablepresent document:

contains a more specific description of a set of tests to be performed on the Testbed

contains a more realistic set of interconnectivity alternatives

additionally discusses some political issues and proposes ways in which they can be handled

Late in the definition phase a proposal from P702 was received and it was agreed to add this as a separate Task in P803. The reason for this was that the work done in P702 could continue without losing momentum and substantial synergy effects could be achieved by placing the work inside the P803 Project.

This Deliverable should be regarded as an input to the description and management of a subsequent phase. As such it should be useful as a reference document. It could also be used by other Projects being started in this area.

Executive Summary

This Deliverable describes WHY a European IP Testbed should be built, HOW it could be built and WHAT it should be used for.

WHY should a European IP Testbed be built?

Most EURESCOM Shareholders are building or upgrading IP networks. To do so it is important to be able to follow the technical developments and to test easily the different technologies that may be used. A European testbed has several advantages in order to make the most suitable selection of new products and features.

Rather than each Shareholder carrying out all tests separately, resources can be saved by doing tests and evaluations together. Selection of technology and products can then be done individually (outside the scope of the Project).

A Testbed comprising a network (rather than doing the different tests locally and then sharing the results) has benefits since a potentially larger collection of technologies and products can be tested for interoperability. Such a heterogeneous network is almost impossible to build/simulate in a laboratory.

Any individual attempt by single European institutions, PNOs or carrier consortias to influence the currently developing advanced IP standards will most probably fail due to the prevailing influence of the USA on this technology. Combined activities with practical results proven by operation in a European IP Testbed will allow the Europeans to play a role in this undoubtedly very important field.

There is also a strategic/practical reason for building a European IP Testbed. The European Union, (EU) needs experimental broadband networks to support research within the “Fifth Framework”. A joint effort within EURESCOM can help the EURESCOM Shareholder’s situation in their relation to the EU.

HOW should a European IP Testbed be built?

Initial planning and some of the results (e.g. migration scenarios for IPv6) can be achieved through theoretical studies.

Some tests (mainly interworking tests) can be performed on the ISDN-based “medium” speed network already available in EURESCOM. (Up to 1 Mb/s connectivity can be achieved using this network).

In a second step, the Project can negotiate to get access to some “bandwidth-on-demand” connections between selected labs within the Project.

WHAT should a European IP Testbed be used for?

The areas needing most attention at the moment are:

IPv6 where problems such as migration strategies, interoperability and multicast usage should be investigated from a PNO point of view.

Support of differentiated Quality of Service in IP networks. Many techniques are being proposed for supporting QoS in IP networks. (examples are Class of Service schemes and Resource Reservation Protocol,  RSVP). Different solutions need to be tested and evaluated.

Network architectures. Apart from IP techniques able to support differentiated QoS there are a number of network architecture solutions that are being proposed. These solutions represent a wider field and are currently more immature than the ones mentioned above. This area is thus not as highly prioritised as the two previous ones.

IP over NBMA (Non Broadcast Multiple Access). A technique that has already been tested in P702 is to select the type of underlying network based on Quality requirements of the user. Further work is required on this issue.

Contributions to standard bodies and setting of standards by support of European agreements (e.g. service or carrier interconnection MoUs).

List of Authors

Rüdiger Geib, Deutsche Telekom

Monika Jaeger, Deutsche Telekom

Christian Siebel, Deutsche Telekom

Jan Gerald Snip, KPN

Markus Isomäki, FINNET GroupHelsinki Telephone Company

Hannu Nikkanen, FINNET GroupHelsinki Telephone Company

David O’Meara, Telecom EireannBroadcom Eireann

Godefroid Cantraine, Belgacom

Ferenc Telbisz, MATÁV Hungarian Telecommunications Company

Jan Anderson, Tele Danmark

Table of Contents

TOC \o "1-3" \t "EUHeading 1;1;EUHeading 2;2;EUHeading 3;3" µPreface	 GOTOBUTTON _Toc418216494   PAGEREF _Toc418216494 µi§§

Executive Summary	 GOTOBUTTON _Toc418216495   PAGEREF _Toc418216495 µiii§§

List of Authors	 GOTOBUTTON _Toc418216496   PAGEREF _Toc418216496 µv§§

Table of Contents	 GOTOBUTTON _Toc418216497   PAGEREF _Toc418216497 µvi§§

Abbreviations	 GOTOBUTTON _Toc418216498   PAGEREF _Toc418216498 µviii§§

1	Introduction	 GOTOBUTTON _Toc418216499   PAGEREF _Toc418216499 µ1§§

1.1	Background	 GOTOBUTTON _Toc418216500   PAGEREF _Toc418216500 µ1§§

1.2	Benefits of a EURESCOM European IP Testbed	 GOTOBUTTON _Toc418216501   PAGEREF _Toc418216501 µ1§§

1.3	Structure of this Deliverable	 GOTOBUTTON _Toc418216502   PAGEREF _Toc418216502 µ2§§

2	Content of P803 Project Description	 GOTOBUTTON _Toc418216503   PAGEREF _Toc418216503 µ3§§

2.1	Task 0	 GOTOBUTTON _Toc418216504   PAGEREF _Toc418216504 µ3§§

2.2	Task 1, Dissemination of results	 GOTOBUTTON _Toc418216505   PAGEREF _Toc418216505 µ3§§

2.3	Task 2, Preparation of the EU IP Testbed	 GOTOBUTTON _Toc418216506   PAGEREF _Toc418216506 µ3§§

2.4	Task 3, IPv6	 GOTOBUTTON _Toc418216507   PAGEREF _Toc418216507 µ4§§

2.5	Task 4, IP services supporting differentiated Quality of Service	 GOTOBUTTON _Toc418216508   PAGEREF _Toc418216508 µ5§§

2.6	Task 5, Network architecture	 GOTOBUTTON _Toc418216509   PAGEREF _Toc418216509 µ6§§

2.7	Task 6, IP over NBMA	 GOTOBUTTON _Toc418216510   PAGEREF _Toc418216510 µ6§§

3	Specific content of P803	 GOTOBUTTON _Toc418216511   PAGEREF _Toc418216511 µ7§§

3.1	Task 2, Preparation of the European IP Testbed	 GOTOBUTTON _Toc418216512   PAGEREF _Toc418216512 µ7§§

3.1.1	Objectives	 GOTOBUTTON _Toc418216513   PAGEREF _Toc418216513 µ7§§

3.1.2	PIRs (Project Internal Results)	 GOTOBUTTON _Toc418216514   PAGEREF _Toc418216514 µ7§§

3.2	Task 3, IP version 6 testing	 GOTOBUTTON _Toc418216515   PAGEREF _Toc418216515 µ8§§

3.2.1	Heterogeneous IPv4/IPv6 network testing	 GOTOBUTTON _Toc418216516   PAGEREF _Toc418216516 µ8§§

3.2.2	Homogenous IPv6 network	 GOTOBUTTON _Toc418216517   PAGEREF _Toc418216517 µ9§§

3.2.3	PIRs (Project Internal Results)	 GOTOBUTTON _Toc418216518   PAGEREF _Toc418216518 µ10§§

3.3	Task 4, Support of differentiated QoS by IP	 GOTOBUTTON _Toc418216519   PAGEREF _Toc418216519 µ10§§

3.3.1	Background	 GOTOBUTTON _Toc418216520   PAGEREF _Toc418216520 µ10§§

3.3.2	Need for research	 GOTOBUTTON _Toc418216521   PAGEREF _Toc418216521 µ11§§

3.3.3	Need for testing	 GOTOBUTTON _Toc418216522   PAGEREF _Toc418216522 µ12§§

3.3.4	Task objectives	 GOTOBUTTON _Toc418216523   PAGEREF _Toc418216523 µ12§§

3.3.5	PIRs (Project Internal Results)	 GOTOBUTTON _Toc418216524   PAGEREF _Toc418216524 µ12§§

3.3.6	Usage of the output	 GOTOBUTTON _Toc418216525   PAGEREF _Toc418216525 µ13§§

3.3.7	Risks	 GOTOBUTTON _Toc418216526   PAGEREF _Toc418216526 µ14§§

3.4	Task 5, Network architecture	 GOTOBUTTON _Toc418216527   PAGEREF _Toc418216527 µ14§§

3.4.1	Overview	 GOTOBUTTON _Toc418216528   PAGEREF _Toc418216528 µ14§§

3.4.2	Test and testbed definition	 GOTOBUTTON _Toc418216529   PAGEREF _Toc418216529 µ14§§

3.4.3	Tools, equipment	 GOTOBUTTON _Toc418216530   PAGEREF _Toc418216530 µ17§§

3.4.4	PIRs (Project Internal Results)	 GOTOBUTTON _Toc418216531   PAGEREF _Toc418216531 µ17§§

3.4.5	Risks	 GOTOBUTTON _Toc418216532   PAGEREF _Toc418216532 µ17§§

3.5	Task 6, IP over NBMA	 GOTOBUTTON _Toc418216533   PAGEREF _Toc418216533 µ17§§

3.5.1	Scenario	 GOTOBUTTON _Toc418216534   PAGEREF _Toc418216534 µ18§§

3.5.2	Technical Options	 GOTOBUTTON _Toc418216535   PAGEREF _Toc418216535 µ18§§

3.5.3	PIRs (Project Internal Results)	 GOTOBUTTON _Toc418216536   PAGEREF _Toc418216536 µ19§§

4	Connectivity	 GOTOBUTTON _Toc418216537   PAGEREF _Toc418216537 µ21§§

4.1	Narrowband ISDN	 GOTOBUTTON _Toc418216538   PAGEREF _Toc418216538 µ21§§

4.2	ATM connectivity	 GOTOBUTTON _Toc418216539   PAGEREF _Toc418216539 µ21§§

4.3	Leased lines	 GOTOBUTTON _Toc418216540   PAGEREF _Toc418216540 µ21§§

4.4	Gigabit Ethernet	 GOTOBUTTON _Toc418216541   PAGEREF _Toc418216541 µ21§§

4.5	WDM field trials	 GOTOBUTTON _Toc418216542   PAGEREF _Toc418216542 µ22§§

5	Conclusions	 GOTOBUTTON _Toc418216543   PAGEREF _Toc418216543 µ23§§

References	 GOTOBUTTON _Toc418216544   PAGEREF _Toc418216544 µ24§§

Appendix 1: The EURESCOM ISDN network	 GOTOBUTTON _Toc418216545   PAGEREF _Toc418216545 µ25§§

§

Abbreviations

ACTSAdvanced Communications Technologies and Services (EU Research Programme)ARPAddress Resolution ProtocolATMAsynchronous Transfer ModeBGPBorder Gateway ProtocolBRIBasic Rate Interface (ISDN)BUSBroadband and Unknown ServerCBQClass-based queuingCoSClass of Service (CISCO)DG XIIIDirectorate-General nro. XIII (Telecommunication, Information Market and Exploration of Research)DiffServDifferentiated ServicesETNOEuropean Public Telecommunications Network Operators’ AssociationFreeBSDBerkeley Standard Distribution Unix implementation for PCsFTPFile Transfer ProtocolHDLCHigh Level Data Link ControlIETFInternet Engineering Task ForceIntServIntegrated ServicesIPInternet ProtocolIPRIntellectual Property RightsIPv4/IPv6Internet Protocol version 4/version 6ISDNIntegrated Services Digital NetworkISPInformation Service ProviderITUInternational Telecommunication UnionLANLocal Area NetworkLANELAN EmulationLISLogical IP SubnetMARSMulticast Address Resolution Server MLRMulti Layer RoutingMPLSMultiProtocol Label SwitchingMPOAMultiProtocol Over ATMNBMANon Broadcast Multiple AccessNHRPNext Hop Routing ProtocolOSIOpen Systems InterconnectionPIMProtocol Independent MulticastPMCProject Management CommitteePNNIPrivate Network-Network InterfacePNOPublic Network OperatorpTLApseudo Top Level Aggregation identifier PRIPrimary Rate Interface (ISDN)PVCPermanent Virtual CircuitQoSQuality of ServiceREDRandom Early DiscardRIPRouting Information ProtocolRIP2Routing Information Protocol version 2RSVPResource reSerVation ProtocolSBR3Statistical BitRate version. 3. An ATM service with a guaranteed quality up to a Sustained Cell Rate (which is smaller then the Peak Cell Rate) of an ATM connection.SCSPServer Cache Synchronisation Protocol SDHSynchronous Digital HierarchySMDSSwitched Megabit Data ServiceSTMSynchronous Transport ModuleSVCSwitched Virtual ConnectionSWSoftwareTCPTransport Control ProtocolTENTrans European Networks (within DG XIII)TMTraffic ManagementTOSType Of ServiceUBRUnspecified Bit RateUDPUser Datagram ProtocolUNIUser-Network InterfaceVATA real-time, multi-party, multimedia application for audio conferencing over the InternetVBRVariable Bit RateVCVirtual ChannelVICA real-time, multi-party, multimedia application for video conferencing over the InternetVPNVirtual Private NetworkWANWide Area NetworkWDMWavelength Division MultiplexingWFQWeighted Fair QueueWREDWeighted Random Early DiscardWWWWorld Wide Web 1	Introduction

1.1	Background

Many PNOs are building IP networks and are testing new technologies like: IP switching, Label (Tag-) Switching, Ethernet Switching and Cell Switching to be able to provide the IP users with more bandwidth and QoS.

The IP business forces PNOs to adopt their business and networks to the different roles of the information industry. Existing PNOs will become both network and service providers. As network provider it is important to be part of a seamless network which can carry all types of services via a limited set of open standardised interfaces.

As new network architectures and technologies as well as new algorithms and protocols emerges, these need to be tested to enable the PNOs to select the appropriate technology to fulfil the great demand for higher bandwidths and more structured network interconnection schemes.

1.2	Benefits of a EURESCOM European IP Testbed

Since there are many different products to be tested and evaluated, much effort could be spent by the individual PNOs. The area of IP evolves very rapidly which leads to a need to spend resources following the development.

By testing and evaluating the technologies (and products) in a neutral way together, resources (and thus money) can be saved. The actual selection of technologies and products to be used in the networks is outside the scope of the Testbed.

Since an expanded market may be built by interconnection of different networks, it is essential that these networks can interwork in a standardised way. The technical prerequisites for this can be tested in the Testbed (leaving the legal and business issues to others).

The IP community is still relatively immature and “de-facto” standards are being defined. A group of PNOs that presents relevant measurement data and proposes an action that has wide support CAN make an impact on standardisation even against the prevailing influence of the USA.

The question may be raised; “Why not to run all the tests on local testbeds and then compare (or share) the results?” While this will definitely save connection costs, such an attempt has shortcomings compared to a somewhat more expensive European IP Testbed:

Many of the proposed tests are interworking tests (“do these two pieces of equipment actually talk with each other?”). The heterogeneity inherent in the Testbed would give access to a variety of hardware combinations, a situation similar to a commercial inter carrier network.

If problems are encountered, it is easier to isolate them in a separate test network than in public operational facilities (e.g. Internet).

1.3	Structure of this Deliverable

This Deliverable is the result of the P803 Set-up phase. As a continuation of the P716 activities, the document describes the European IP Testbed and defines a detailed Project Description for the next phase, the P803 Implementation phase. It is important to note, that overall aims, mentioned in some chapters, may not be reached until the end of Project. From this perspective, the requirement for follow up activities is clear to all participants already today. This Deliverable is structured as follows:

Chapter 2 provides information on the organisation and the budget estimated for P803. The various tests are briefly described and prioritised.

Chapter 3 gives a more detailed description of the specific contents of P803 subdivided into a number of tasks and discusses the background for choosing the test topics:

Preparation of the legal terms for a European IP testbed,

IPv6 test cases,

How can differentiated QoS be supported by IP,

Network Architectures.,

IP over NBMA.

Chapter 4 discusses various solutions to the interconnectivity requirements of the testbed.

Chapter 5 contains conclusions and recommendations.

2	Content of P803 Project Description

Below you will find the proposed Task structure of P803, a prioritisation of each task and some start dates as well as a preliminary budget distribution.

2.1	Task 0

The PMC (Project Management Committee) will be built from technical and political/legal experts and thus allow a good exchange of information between both groups. Important questions to be handled on PMC level are connectivity (especially in 1998) and liaison with other Projects (e.g. Internet2, TEN155/DANTE).

Priority: 	High
Start: 		Beginning of Project
Budget: 	10 MM

2.2	Task 1, Dissemination of results

The dissemination will focus on results of the technical groups and focus on the Deliverables, presentations and so on. It is expected that Task 2 will also do some promotion of Project results.

Priority: 	Middle
Start: 		Beginning of Project
Budget: 	5 MM

2.3	Task 2, Preparation of the “EU IP Testbed”

In the following, the term “EU IP Testbed“ is used to describe a Project consisting of more than the current P803 or EURESCOM members. Whenever P803 only is meant, the term “P803“ is used.

Task 2 is responsible for the legal and political background of the “EU IP Testbed” (meant as a Project that EU may start in 1999). The Task aims at the construction of an MoU (to allow involvement of any new joiners, suppliers and, in a later stage, users) and liaison with the EU. Additionally Task 2 is expected to consider rules for a “merchandising“ of the Project. This aims to create a protected Project name and rules how interested parties can claim to have participated in it (e.g. a supplier whose equipment was used by a PNO). The R&D sections are expected to get the required lab equipment on better terms if such a Project branding is done actively. Ideas on how to do that may be taken from Internet2 [I2].

If the EU IP Testbed is to result in standards, broad support of the European Internet community will be needed. The same is true if these standards are to be accepted on a broader basis than just Europe. A future activity aiming at new or improved IP standards will have to look for co-operation with all interested parties.

The European organisations which might be interested in ways of co-operation to develop common new or improved standards may be:

EURESCOM Shareholders

Internet Service Providers

the European R&D community

the European Commission

Hardware/Software Suppliers

The still high expenses caused by broadband lab to lab connectivity on a European level is a further incentive to aim at a broad co-operation within the cited community of interest in order to minimise network costs of each single institution connected to the EU IP Testbed.

There is no demand for P803 to be the leading force when an “EU IP Testbed” is constructed in the way as described above. Participation in any activities aiming at the establishment of a Project in the mentioned framework will, however, contribute largely to the success of P803, as the question of connectivity may be solved easier (which will lead to broader interest in the Project) and acceptance of the P803 results will rise if they are shared with a bigger community of interested IP researchers.

The above mentioned aims will require the involvement of managers with a technical background but a broader scope of work in P803. Task 2 participants should be able to market and promote the idea and aims of an EU IP Testbed as seen from the position of a EURESCOM Shareholder. Additionally, legal experts need to be involved at least in an indirect way. They will have to construct Non Disclosure Agreements covering multiple aspects and deal with some aspects resulting from a protected brand name of a EU IP Testbed (e.g. conditions of the usage of such a name for company specific advertisement).

Further Task 2 should play an active role when discussing the terms of access to the EU IP Testbed with all interested parties. (Note that this does not necessarily mean that P803 is providing a network and access to third parties. It seems to be more realistic to expect the opposite situation!)

TEN 155 is an important contact for P803. First, TEN 155 plans to work on similar IP related contents. Second, the “Quantum“ part of TEN 155 aims at the provision of connectivity for European R&D Projects.

Other possible contacts are the EU and the Projects which could run under the Fifth Framework call for international networks. DG XIII may be an appropriate body for this. Note that many universities participate in such Projects. In general, contacts to national research networks and universities may result in interested Project partners.

Priority: 	Very High
Start: 		Beginning of Project
Budget: 	30 MM

2.4	Task 3, IPv6

Besides the topics given in the headlines, Security, Management, Scalability and Performance issues may be regarded as part of all subsections The topics are subdivided as follows (if not explicitly mentioned the priority is given in the order of appearance):

T3.1	Heterogeneous IPv4 and IPv6 networks.

T3.1.1 Migration scenarios

T3.1.2 Addressing

T3.1.3 Interworking

T3.1.4 End to end testing

T3.2	Protocol related IPv6 tests

T3.2.1 Compatibility testing (router to router)

T3.2.2 Addressing

T3.2.3 Routing

T3.2.4 Mobility

T3.2.5 Validation tests

Priority: 	High
Start: 		Beginning of Project
Budget: 	30 MM

2.5	Task 4, IP services supporting differentiated Quality of Service

Besides the topics given in the headlines, Security, Management, Scalability and Performance issues may be regarded as part of all subsections. The task is subdivided as follows (if not explicitly mentioned the priority is given in the order of appearance):

T4.1	General description of mechanisms and testing conditions.

Description of the mechanisms and differences between RSVP and the flow- or topology-based QoS differentiation mechanisms.

General testing and measurement principles to benchmark IP services supporting differentiated QoS (e.g. background load, test load, traffic mix etc.)

Duration should be limited to 3 months.

T4.2	Flow- and topology based IP QoS differentiation

T4.2.1	Useful parameters to describe IP QoS

One of the important questions to be answered here is whether there are specific parameters describing e.g. a flow based QoS and whether it makes sense to determine numbers for these parameters (e.g. throughput or delay).

T4.2.2	Methods to ensure the different QoSs

An example of scalability issues to be covered here: A high QoS user most probably is using a broadband IP access. If the QoS should not be ensured by network over-dimensioning, bandwidth limitation could be an appropriate means to allow a reasonable network planning and the availability of a minimum of resources for users with a lower QoS.

T4.2.3	Selection and validation of feasible solutions

The selection of these mechanisms should not only be based on network requirements but also respect those of users and applications.

T4.3	RSVP

T4.3.1	Scalability

T4.3.2	Aggregation of RSVP flows

T4.3.3	Set-up delay

T4.3.4	RSVP connection stability and end to end availability

T4.3.5	Mobility

T4.4	RSVP combined with flow and topology based QoS differentiation mechanisms.

Priority: 	Very High
Start: 		Beginning of Project
Budget: 	30 MM

2.6	Task 5, Network architecture

Task 5 will investigate different network architectures based on different advanced IP systems and different available layer 2 connections. The priority of this task may be changed if the general interest in these aspects is higher than today.

Note that  ATM is providing certain IP specific mechanisms like early packet discard. Features like this one demand a kind of “inter layer“ research and therefore liaison may be sought with P813 (if ATM SBR3 and UBR performance issues are treated there).

Priority: 	Middle (may be changed during the Project)
Start: 		Early 1999
Budget: 	20 MM

2.7	Task 6, IP over NBMA

Task 6 will continue the work of P702, which looked at the benefits of selecting types of transport networks based on Quality of Service requirements issued by the user. The results of this Task can be seen as a complement to the results, which come out of Task 4 (but looking at the option of selecting between different types of networks rather than tuning one network to different requirements).

Priority: 	High
Start: 		Beginning of Project
Budget: 	40 MM

3	Specific content of P803

This chapter discusses how to prepare the establishment of a test bed and goes into more details about the background for choosing the focus issues. The chapter also suggests the specific content of the implementation phase of Project P803. Each section provides a description of one of the Tasks T2 through T6.

3.1	Task 2, Preparation of the European IP Testbed

This section contains the description of a proposed Task 2, which will deal with support for (and political acceptance of) the European IP Testbed as an instrument to support an EU initiative in the area of “Advanced European Experimental Testbeds” (as described in the discussion of the Fifth Framework of EU, [EU5thfw]).

3.1.1	Objectives

Since it is now clear that JAMES will close at the end of March 1998, many Projects (mainly within the ACTS program) are now hunting for alternative solutions. There are many competing offerings, but if every Project will shop around for its own connectivity, it will be more expensive than if a combined effort is made.

Presently, it is unclear how this will be done, but there is a strong need for such a co-ordination. It must be made certain that the European IP Testbed (and other EURESCOM Projects) will not be left outside such a co-ordination, and thus there is a need for resources to follow the development.

Also, if the EURESCOM Shareholders were to join in an effort to create a Broadband Service Network, the “European IP Testbed” should be regarded as a strategically important instrument. The results of this Project could then immediately be used in the construction of such a network.

3.1.2	PIRs (Project Internal Results)

The work can be divided into sub-tasks:

3.1.2.1	PIR 2.1 Co-ordination within EURESCOM/ETNO

The EURESCOM Shareholders (and other PNOs) represent a specific group, which may need a closer co-ordination than other players. This is because these organisations’ own networks can be used both by this Project as well as other Projects.

This PIR will make sure that the interest of the Shareholders is looked after and that close contact is kept with ETNO, especially if ETNO were to offer a Broadband Service Network to the EU.

3.1.2.2	PIR 2.2 Co-ordination with EU

This PIR will follow the development within the different parts of EU (e.g. DG XIII, TEN Telecom, ACTS, TEN155 etc.) that are providing connectivity or are looking for connectivity.

3.1.2.3	PIR 2.3 Legal and political support for “European IP Testbed”

This PIR shall act as the representative of the Project and negotiate (if necessary) with different connectivity providers on behalf of the Project. The aim is to provide the connectivity needed for Task 3, Task 4 and Task 5.-6.

3.2	Task 3, IP version 6 testing

The current world-wide Internet is based on the protocol IPv4 (IP version 4). Although this protocol has some limitations, its usage is increasing dramatically as Internet grows. Almost all known telecommunication services and applications are technically feasible over this network. Examples are: e-messaging, Web-site hosting, telephony, audio, video (still and moving pictures), retrieval and conference services, point-to-multipoint etc.

Despite its undoubted success and ubiquity, IPv4 reflects the limitations of a protocol of the early seventies. It is increasingly unable to cope with the migration towards services with real-time and multicast requirements. Problems also arise for delay and delay variation sensitive multimedia services, where there is a need for specific QoS or for differential service levels.

IPv6 is being developed by the IETF to address many of these problems and to enable scaleable IP services to meet future growth demands. In essence, the protocol has been extended to meet new requirements, such as address space, flow labels, traffic class field and security features. It has also been simplified by removing with old parameters, such as check-sum, type of service, flags and identification options being removed, which allows faster header processing within routers.

In order to gain experience with IP version 6, it is necessary to build a dedicated network connecting the partners of P803. As IPv6 will gradually become more widespread it is most probable that it will be implemented first as a backbone protocol, as it offers many advantages to the ISP. Hopefully this will bring the ISPs in a position to offer new and improved services to the subscriber.

3.2.1	Heterogeneous IPv4/IPv6 network testing

All partners will be required to install and configure one or more IPv6 hosts ( Sun, Linux and FreeBSD all have IPv6 implementations available). These hosts can be either native IPv6 or have a dual IPv4/IPv6 stack. Initially these hosts will communicate in IPv6 through IPv4 tunnels over the Internet, using static routes. Once the hosts are communicating successfully over the Internet, more advanced testing can commence. ISDN connections (again using IPv4 tunnels) will be used if a dedicated network is not yet in place. Then, if possible (i.e. if hardware is available), native IPv6 links can be established. This will require routers capable of supporting IPv6 directly.

This sequence of experiments will allow various migration strategies to be tested. Depending on the hardware available to the partners, two strategies can be tested. If IPv6 capable routers are not available, then the strategy will be to migrate the hosts first to carry the traffic over IPv4 tunnels. If IPv6 capable routers are available, it should be possible to configure the backbone first, then decide on a migration strategy for the users/hosts.

This experiment will automatically force interoperability testing, as it is extremely unlikely that all partners will be using the same software/hardware. The various implementations of IPv6 stack and/or IPv4/v6 hardware being used by the partners will all be required to be compatible for the experiment to succeed. Any inconsistencies will be established recognised at an early stage.

3.2.2	Homogenous IPv6 network

Once partners have acquired IPv6 connectivity, experiments designed to test the capabilities of IPv6 can be designed.

The main areas where IPv6 improves upon the older IPv4, and where testing should be concentrated, are :

3.2.2.1	Auto-configuration

IPv6 routers advertise their presence on all IPv6 networks. Hosts can automatically configure themselves with an IPv6 address by listening for these advertisements, and building an address from the router’s prefix plus the MAC address of the hardware interface.

3.2.2.2	Mobility

The current IPv4 mobility proposal uses Home and Foreign agents to forward packets destined for the mobile node to the node’s current location. The main disadvantage of this scheme is that all packets destined for the mobile node are first routed to the node’s home network, then on to the node’s current network. This so-called ‘triangular routing’ is inefficient. The proposal for mobility in IPv6 allows the two communicating nodes to cache each other’s current location, and use IPv6 routing headers to route packets directly to each other.

3.2.2.3	Routing

All IPv6 hosts are capable of specifying the route to be taken by a packet, in a similar fashion to Source Routing in IPv4. This allows more intelligence in each node, which can take some of the load from the routers. In addition, the new routing protocols being designed to support IPv6 (e.g. BGP4+, RIPv6) are more efficient, and will eventually support QoS-routing directly.

3.2.2.4	Addressing

The currently proposed addressing scheme, known as ‘Aggregatable Global Unicast Address hierarchy’ (draft-ietf-ipngwg-testv2-addralloc-01.txt) is used throughout the 6Bone today. The P803 Project could be allocated a Pseudo Top-Level Aggregation identifier (pTLA) which is the top level of the new testing address allocation. See [6bone] for more details.

3.2.2.5	Traffic Flows

IPv6 contains a field in the header called the ‘Flow Label’. Although the use of this field has not yet been standardised, P803 could, perhaps in conjunction with the QoS experiments, investigate the use of this field.

3.2.2.6	Multicast / Anycast addressing

Multicast addressing will obviously become increasingly more important as Internet traffic increases and streaming applications such as audio or video on demand become more popular. Anycast addressing could be useful for running server farms for example, where the user connects to the server which is ‘closest’ i.e. whichever server replies first.

3.2.3	PIRs (Project Internal Results)

3.2.3.1	PIR 3.1 Heterogeneous IPv4 and IPv6 networks

In this PIR the following tests will be performed:

Migration scenarios

Addressing

Interworking

End to end testing

3.2.3.2	PIR 3.2 Protocol related IPv6 tests

In this PIR the folllowingfollowing tests will be performed:

Compatibility testing (router to router)

Addressing

Routing

Mobility

Validation tests

3.3	Task 4, Support of differentiated QoS by IP

3.3.1	Background

The need for QoS in the IP protocol rises from two issues. First, real-time applications need at least predictable guarantees of the network performance in terms of throughput and delay. Second, there is a need to differentiate QoS between users based on how much they are willing to pay for the service.

Much effort has been spent studying the requirements of the real-time applications in a packet switched network, but it seems that clear results are still missing. In the beginning it was believed that voice and video required very strict delay guarantees and also guaranteed maximum packet loss ratio. However, recent studies have shown that if the applications are designed correctly, they are able to adapt to slight variations in the network conditions. This adaptation is based on buffering, forward error correction and variable bit rate codecs. At the moment it seems that applications like videoconferencing and IP telephony can live with quite loose delay bounds and occasional packet losses, provided that the network is reliable most of the time. This means that what is needed is a QoS scheme that provides high probability of success, but can still run the network with high utilisation taking advantage of statistical multiplexing.

Differentiating service between users is going to be an essential part of competition between operators. At the moment the operators are unable to offer enhanced end-to-end QoS for customers who would be willing to pay for it. The only practical way to give better service is to sell more capacity for the access pipe, and that really does not solve anything network-wide, or even inside an IP VPN. It would also be advantageous to be able to offer lowered QoS for lowered price. These users could still get good performance, when there is room in the network, e.g. at night. These kinds of requirements call for a simple QoS differentiation scheme, where per-application signalling is too heavy.

IETF has already defined a QoS architecture for IP based on two new service models (Controlled Load and Guaranteed Service) and a new signalling protocol (RSVP). This Integrated Services architecture has gained much support from the equipment vendors, and almost everybody is boasting of "RSVP support" in the new products. Most of the criticism has been concerned with scaling of the architecture. At the moment it is recommended that RSVP and Integrated Services would not be deployed in the fastest backbones where the number of simultaneous reservations could rise to thousands.

However, most of the QoS related work in IETF is currently shifting toward Differentiated Services. Differentiated Services architecture is based on the idea of few service classes that are indicated in each packet in IPv4 TOS or IPv6 Class byte. Different Classes perceive different service in the routers based on, for example, WRED queue management and WFQ or CBQ scheduling. The marking of the Class can be done either by the applications or the router at the network "edge". If application marking is enabled in commercial environment, some kind of policing is still needed at the edge to prevent misuse. This means that some kind of user (or application) profiles are needed in the edge routers. The profiles could be composed of token bucket filters with rules for marking of conforming and non-conforming packets. IETF is trying to standardise the mechanism and the number and the meaning of the Classes. There are already some non-interoperable products available, so there is really a need for a standard.

Furthermore, the IETF concludes its work on the Resource Reservation Protocol (RSVP) (Intserv-Model). This protocol is able to reserve end-to-end bandwidth between two or more users. However, the IETF itself is not satisfied with the result. One reason is that it is too complex. Another is that it is thought that RSVP will not scale very well in large networks. As a consequence, the IETF recently created a new working group “Differentiated Services“, which will work on a queue based reservation mechanism with “Classes of Services“ (Diffserv-Model).

3.3.2	Need for research

The research on the European IP Testbed should focus on defining how the new QoS mechanisms can be brought into use in commercial networks. For example, RSVP needs some kind of policy control to authenticate and/or bill for its usage. The problem is how this authentication and billing is to happen when a reservation is signalled through several administrative domains (networks operated by different operators). For Differentiated Services, the important question is how an operator can build reasonable services for sale. Should there be several classes of best-effort with different dropping probabilities and a single class for all real-time traffic with low delay, and how could these classes be provisioned? Is it possible to give any performance figures, such as the average packet loss ratio, for the classes? Is it possible to aggregate RSVP in the backbones by simply mapping Controlled Load into a particular service class, or should there also be admission control? The operators should also agree on how different markings are handled at the interconnection points.

3.3.3	Need for testing

For Integrated Services, the European IP Testbed could concentrate on testing the scalability of the architecture and RSVP. It would be interesting and important to find out how many reservations a particular router is able to handle, and whether it is able to keep its promised service levels under different load conditions. Also the merging of reservations in multicast connections would be interesting to see. There are already RSVP implementations for all major operating systems and even applications that can use RSVP, so the testing could be arranged.

For Differentiated Services, the testing could find out how well the mechanisms implemented in different products really perform under various conditions, such as differentiating packet drop probabilities and delay. Also different policies for non-conforming packets could be tested; Is it better to drop them or downgrade their service class to best eforteffort, or what should be done? How important is the size of the queues when comparing the products? How should WRED thresholds and WFQ weights be set so that optimal performance is achieved? Controlled Load to service class mapping could also be tested. Also the requirements of several applications could be tested using different classes with varying packet loss, delay and jitter conditions.

3.3.4	Task objectives

This task will focus on the following mechanisms:

Integrated Services (Resource Reservation Protocol)

Differentiated Services (Class of Service based QoS)

The aim of this task will be in a first step to give a general description, analysis and comparison of these two models and to define testing conditions. Then in a second step these two models will be analysed separately in more detail, where scalability, routing and addressing issues will play an important role.

Concerning RSVP, special points of interest are flow aggregation mechanisms, set-up delays and mobility; Concerning CoS, special points of interest are the definition of useful parameters to describe QoS and methods to ensure different QoS. Finally feasible solutions will be chosen.

In a third step, protocols belonging to the different Diffserv and Intserv model will be combined. This has to be done as RSVP will only work well in the access network and where CoS based mechanisms are proposed for the backbone area. Also, practical evaluation will only happen in a follow-up Project.

3.3.5	PIRs (Project Internal Results)

Besides the topics given in the headlines, Security, Management, Scalability, Routing and Addressing issues may be regarded as part of all subsections.

3.3.5.1	PIR 4.1 General description of mechanisms and testing conditions

The PIR will provide a description of:

the mechanisms and differences between RSVP and the flow- or topology-based QoS differentiation mechanisms.

general testing and measurement principles to benchmark IP services supporting differentiated QoS (e.g. background load, test load, traffic mix etc.)

3.3.5.2	PIR 4.2 Flow- and topology based IP QoS differentiation

The following topics will be addressed:

Useful parameters to describe IP QoS: are there specific parameters describing e.g. a flow based QoS and does it make sense to determine numbers for these parameters (e.g. throughput or delay)?

Methods to ensure the different QoSs (scalability ): If QoS should not be ensured by network over-dimensioning, bandwidth limitation could be an appropriate means to allow a reasonable network planning and the availability of a minimum of resources for users with a lower QoS.

Selection and validation of feasible solutions: the selection of these mechanisms should not only be based on network requirements but also respect those of users and applications.

3.3.5.3	PIR 4.3 RSVP

The following topics will be addressed:

Scalability

Aggregation of RSVP flows

Set-up delay

RSVP connection stability and end to end availability

Mobility

3.3.5.4	PIR 4.4 RSVP combined with flow and topology based QoS differentiation mechanisms

RSVP is thought to work well in the access network, where CoS based protocols are proposed for the backbone. That is, a network provider has to combine both approaches for its ISP activities.

3.3.6	Usage of the output

The output of this task can be used by the Network Operators to identify the most adequate methods to implement QoS. This is an issue that most of the Operators are looking into now and that has strong business implications in the more or less near future. Relevant results of the work will be carried to standards bodies, e.g. IETF.

3.3.7	Risks

The discussion on the mentioned protocols and products is a hot issue presently being debated in the IETF. RSVP is standardised, CoS is not. So, no advanced hardware platforms may be available during the test (CISCO already implements some proprietary CoS based solution, however it may not be adequate enough).

3.4	Task 5, Network architecture

3.4.1	Overview

ATM is likely to be the core technology of any backbone network. Different IP over ATM models are available: Classical IP over ATM is widely used because of its simplicity and robustness. However,  it does not meet big networks’ requirements (single server, no redundancy, no multicast etc.) and, especially, it does not scale. LAN Emulation incorporates some improvements but still needs a router for traversing a Logical IP Subnet (LIS) boundary. Solutions like NHRP and MPOA are both based on the route server principle (separation of route calculation and forwarding). Shortcut connections over subnet boundaries can be established. These more sophisticated solutions also provide distributed and redundant services and so they do scale. On the other hand, they are highly complex and might be difficult to manage in big networks.

The Multi-Layer Routing (MLR) systems (better known as IP switching systems) have been developed with the aim of combining the best of layer 3 routing and layer 2 switching and reducing a lot of the overlay IP over ATM solutions’ complexity. These systems withdraw from ATM routing and ATM signalling. Individual connections with individual QoS may be established for detected IP flows. IP switching systems promise to be better performing than traditional routers. However the latter also continue to be improved (Gigabit routers, implementation of sophisticated queuing algorithms etc.) to be made more powerful and yet still compete with MLR solutions.

The Project partners’ local networks islands will be interconnected forming a wide-area IP testbed. The LANs may have different underlying network technologies and build a heterogeneous WAN. An application communicating with a remote destination will traverse several subnetworks with different administrations and different technologies. A specific end-to-end QoS must be provided depending on the requirements of the application (bandwidth, delay etc.). The application’s accessible QoS very much depends on the network technology used. Therefore some network architecture testing issues are characterised below The aim of the Project will be to evaluate different network technologies and their interworking in a heterogeneous wide-area IP testbed.

3.4.2	Test and testbed definition

Certain network technologies may have been evaluated locally. The integration of separated network islands and the set-up of the distributed testbed will convey network management experience. Questions concerning interoperability and performance will be answered quantitatively and qualitatively. For the tests a reference configuration and an interface definition has first to be found:

Figure  SEQ Figure \* ARABIC µ1§: Example reference configuration

Specification of

access device (ATM switch, IP switch, router)

interface (UNI, PNNI, UNI signalling 3.x, IP routing protocols and domains)

logical IP subnets

ARP servers, BUSs

bandwidth, ATM QoS class

are required.

A test plan needs to be developed. During the set-up of the testbed basic functionality tests will be carried out. The follow-up tests are defined depending on the structure of the testbed. They will cover interoperability and performance tests. In the following, a list of topics is given which will (partly) form the basis of the test definition.

3.4.2.1	Basic functionality tests

experience with the integration of different networks and configuration of the testbed, identification of problems

stability of the network, failure of components, possibility to install redundant services etc.

behaviour at start-up of the network or during recovery after a breakdown of (part of) the network, existence of failover-mechanisms etc.

configuration, administration and management effort during setting-up the testbed

3.4.2.2	Interoperability tests

evaluation of the scalability of the network, e.g. existence of single/distributed services, behaviour with one or more LISs, etc.

support of virtual LAN definition

routing interaction at the network interfaces on IP layer (and ATM layer)

multicasting over subnet boundaries

security mechanisms on different layers

mobility

3.4.2.3	Performance

The tests will evaluate the performance at the IP level. Specific results of IP-level measurements should be further analysed at the ATM level.

IP level tests

The following tests have the parameters:

1) number of concurrent IP flows;

2) traffic pattern (constant, on/off sources);

3) IP packet size;

IP packet throughput, costs per routed packet

routing delay

Depending on the network technology the mode of the network can be changed from “routed“ to “switched“ (MLR systems) or from “default flow“ to “shortcut flow“ (LANE, MPOA). The process of mode change/establishing end-to-end ATM connections will be investigated regarding processing effort and process duration.

support of differentiated QoS within each network technology, overall QoS in heterogeneous networks

comparing the performance of IP over ATM mechanisms, MLR systems and traditional routers

end-to-end packet delay and jitter

ATM level tests

mapping of RSVP/CoS to ATM QoS

switching delay and jitter

cell throughput

number of connections

signalling performance

3.4.3	Tools, equipment

We must distinguish between active and passive tests. In passive tests the network is monitored at suitable observation points. In active tests traffic is generated and captured by special test tools. Standard IP traffic analysis tools like “tcpdump”, “snoop”, “netpertf” etc., as well as specifically designed generation, monitoring and statistic tools will be used. Test equipment like the HP 75000 ATM tester can perform passive and active tests. Additionally real applications will be run in the testbed. These are traditional TCP/UDP applications (ftp, www, telnet etc.) as well as multimedia applications like VIC and VAT with real-time requirements.

3.4.4	PIRs (Project Internal Results)

3.4.4.1	PIR 5.1 Network definitions

In this PIR a reference architecture will be defined. Also a test plan comprising the selected architectural technologies will be provided.

3.4.4.2	PIR 5.2 Basic Functionality tests

The selected network architecture technologies will be tested and evaluated in isolation.

3.4.4.3	PIR 5.3 Interoperability tests

Several network architectures will be interconnected and tests performed on the resulting total network. The PIR will comprise test results on both IP and ATM level.

3.4.5	Risks

Since it is not yet clarified how the labs will be connected the scope of this task cannot be accurately defined. The technical contents very much depend on the technology, the topology and the bandwidth of the testbed. Test objectives may be limited.

3.5	Task 6, IP over NBMA

Pre-normative experiences in the area of QoS routing are limited and widely dispersed and the P702 Project has provided unique experience in the provision of end-user bandwidth on demand with a choice of QoS options so that traffic flows are carried over a variety of NBMA and packet switched bearer networks. The mapping of each user request to the appropriate network and/or trunking technology remains under the control of the PNO. This creates the opportunity for PNOs to route traffic from preferred corporate clients through superior network links (e.g. non-loaded high capacity transatlantic links as opposed to shared-access lines), or to optimise resource utilisation by intelligently and dynamically routing traffic according to each application's bandwidth or delay requirements e.g.: Email via Internet, Voice via ISDN, Video conference via ATM.

The area of QoS routing and differential services remains one of enormous opportunity and technological uncertainty affecting all European PNOs. It is possible to further develop the advanced work of the P702 group to evaluate how emerging differential services, QoS and multicast functionality can be commercially exploited by the PNOs and to optimise their integration across the IETF, ATM Forum and ITU-T standard tracks.

3.5.1	Scenario

EMBED Word.Picture.8µ §

Figure 2: QoS Multicast Routing

In this scenario, the destination hosts signal to remote Decision Point and Multicast Server that they wish to join a multicast group. In doing so, they specify the QoS with which they wish to receive the multicast session. The Decision Point then intelligently multicasts the traffic according to the requested QoS, Policy, and Admission data that it holds. Traffic is bundled to realise multicast network efficiencies, and in doing so, ensures that the highest common denominators for bandwidth and minimum delay are satisfied.

3.5.2	Technical Options

QoS multicast routing. RSVP is optimised for multicast, but is currently only implemented in a unicast mode. This would require the development of a multicast application, multicast routing support of IPv6 (e.g. PIM), integration of RSVP with multicast routing, development of a multicast QoS routing table and the mapping of multicast to ATM multicast channels.

Integration with a differential services approach making use of the IPv6 Traffic Class field.

Flow aggregation. It is possible to carry more RSVP flows over the same ATM SVC channel.

Use of WFQ to control any Layer 2 link, and not just ISDN, and to reserve resources on LANs.

Support of NHRP. With the aim of better integrating IPv6 and ATM, support of NHRP would allow the optimisation of RSVP initiated IP flows across an ATM network.

Native IPv6 over ISDN. Currently no native ISDN support is provided. A Project to integrated ISDN support could also include MPLS integration.

Dynamic cut-through policy and decision mapping (e.g. based on cost/availability).

SCSP for multiple MARS operation.

Comparison of IPv4 and IPv6 style flow classification.

3.5.3	PIRs (Project Internal Results)

3.5.3.1	Public Relations

This PIR will interact with external bodies, especially IETF.

3.5.3.2	Architecture issues

This PIR will look at problems from an architectural point of view, e.g. where and how policy decisions should be made, how the acheivedachieved results can be mapped to existing (and evolving) standards, etc.

3.5.3.3	Low level issues

This PIR will perform tests concerned with level 2 and usage of different fields in the IpPv6 header.

4	Connectivity

Connectivity between the labs of the P803 participants is a requirement for the success of the Project. Currently no European Research Network is available nor do a majority of the participating PNOs offer any product allowing international broadband connectivity on affordable terms.

Until mid 1999, no requirement for a permanent network can be seen. Even with a limited number of third party users, on demand connectivity between labs should be sufficient. As on demand connectivity should be more inexpensive than permanent connectivity, it could be a good topic in discussions with commercial or network divisions.

In general, fantasy will be required when going for fair price lab to lab connectivity. The involved business and network divisions should be pushed to develop or offer the appropriate services and types of connectivity. As the opened competition allows to expand services into other countries, local interconnectivity may be an issue in some cases.

4.1	Narrowband ISDN

Narrowband ISDN is acceptable as intermediate solution. It has, however, to be respected that it will not be possible to load a system under test if it is connected by low bandwidth connections only. Performance and QoS related testing will not be feasible in such cases.

Protocol related issues may be tested using narrowband ISDN connections.

EURESCOM provides an inverse multiplexer allowing n*64 kbit/s connectivity. The conditions on how to interconnect using this inverse multiplexing equipment may be found in Appendix 1.

4.2	ATM connectivity

ATM provides high flexibility in terms of bandwidth and Quality of Service. Additionally, on demand connections may be offered by management (i.e. without signalling). Therefore, ATM PVC connections may be a good opportunity. 
A basic requirement is the availability of a bilateral ATM connection between two PNOs. Should a particular connection prove to be too expensive to finance, it is recommended to ask for on demand (reserved) UBR connections. Whenever this service is not offered commercially, ATM service development engineers may also be interested in supporting such an attempt and further price reductions may be gained.

4.3	Leased lines

Another option is the use of international 2 Mbit/s leased lines. Considering the usual lack of flexibility in terms of order and activation, duration and changes of destination in combination with the required investments, such connections may be limited e.g. to partners within a telecommunication alliance.

4.4	Gigabit Ethernet

Gigabit Ethernet services may be used as layer 2 transport service for P803.

4.5	WDM field trials

PNOs involved in bilateral WDM field trials may use this transmission technology to get access to bandwidth for P803.

5	Conclusions

Among the EURESCOM Shareholders there is a need for research within the area of IP. Such research is mostly pre-competitive and should be done in a collaborative environment. The competitive part (the actual selection of technology and products to support different offerings to customers) should be kept outside this Project.

The European Union is looking for initiatives in the area of “Experimental Networks”. The European IP Testbed could be an important component for such initiatives. The Project should follow the work of other groups and liaise as appropriate.

There are a number of tests that could be performed on a European IP Testbed. The most prioritised concern IPv6, support for differentiated Quality of Service, network architectures and IP over NBMA.

In the beginning of the Project there is no need for broadband connections nor a permanent network. The European IP Testbed could be interconnected initially through the EURESCOM ISDN platform and later interconnected (where needed) using broadband connections. Several alternatives exist for broadband connectivity, e.g. spare capacity (UBR) in the ATM networks, WDM trials and leased lines between “friendly” Shareholders.

In order to influence the evolving IP network platform and its services, Europe (i.e. PNOs, ISPs, R&D community and suppliers in Europe) will have to build its own large scale test networks in order to allow each participating organisation to stay in competition with rest of the world.

References

[EU5thfw]	The 5th Framework of EU. See http://www.cordis.lu/fifth/home.html

[I2]	The US Internet 2 initiative. See: http://www.internet2.edu

[6bone]	The IPv6 network. See: http://www.6bone.net

Appendix 1: The EURESCOM ISDN network

The Project P715 (EURESCOM Services Platform) uses an ISDN platform with IP over a (dynamically allocable) number of ISDN B-channels. The figure below shows the principle:

EMBED Word.Picture.8µ §

Figure 1: The EURESCOM P715 ISDN platform

The principle of the platform is that the EURESCOM site functions as an intermediate router. Two (or more) local sites can connect to the EURESCOM site. (call back is used so that the actual costs are covered by EURESCOM).

The EURESCOM Router is connected via an ISDN 2 Mb/s PRI (Primary Rate Interface). Local sites can be connected using 2 x ISDN BRI (Basic Rate Interface), which means 4 B-channels (4 x 64 kb/s). This is shown in the figure above. With this configuration the bandwidth is limited to 256 kb/s per site and  7 sites can be connected simultaneously. This is the set-up used for P715.

If the local sites are connected via a PRI (as the EURESCOM site) the allocation of channels can be dynamic in the range of  1 - 30 B-channels. (i.e. two local sites can communicate at 1 Mb/s using 15 B-channels each).