Project P803-PF

European IP Testbed (Implementation Phase)

Deliverable 4

Supporting Differentiated Quality of Service in IP Networks

Volume 1 of 3: State of the Art

Suggested readers:

- Managers, IP network designers/planners

- People interested in Quality of Service in the Internet

For full publication

August 1999

EURESCOM PARTICIPANTS in Project P803-PF are:

- Deutsche Telekom AG

- Eircom

- FINNET Group

- France Télécom

- MATÁV Hungarian Telecommunications Company Ltd.

- Tele Danmark A/S

- TELECOM ITALIA S.p.a.

- Telekom Austria AG

This document contains material which is the copyright of certain EURESCOM PARTICIPANTS, and may not be reproduced or copied without permission.

All PARTICIPANTS have agreed to full publication of this document.

The commercial use of any information contained in this document may require a license from the proprietor of that information.

Neither the PARTICIPANTS nor EURESCOM warrant that the information contained in the report is capable of use, or that use of the information is free from risk, and accept no liability for loss or damage suffered by any person using this information.

This document has been approved by EURESCOM Board of Governors for distribution to all EURESCOM Shareholders.

Preface

(Prepared by the EURESCOM Permanent Staff)

The Implementation Phase of the P803 Project initially had the ambition to construct an international testbed, consisting of broadband connections between the different participating shareholders.  The structure and the use of the proposed testbed were to be clarified by the Project [P803-1].

However, as the Project progressed:

- A lot of time was needed for the development of test tools and test methodologies in advance of actually performing the tests.  This was largely done in the individual shareholder labs

- Many of the tests were interoperability-related rather than performance-related because a European-wide high-performance broadband network was not available to the Project

- Access to cheap connectivity throughout Europe is not generally available now that JAMES no longer exists.  Instead, connectivity needs to be bought at commercial prices

The result is that no European-wide testbed currently exists, but the Project has still managed to produce relevant results.  For the success of future work in this area, broadband connectivity between the participants is absolutely necessary and industry involvement would be desirable (as has been the case in some other EURESCOM Projects).

Eight Shareholders took part in the Implementation Phase of P803.  The Project Leader was Paul Duggan, Eircom.

One of the Tasks in the P803 Project focused on support for differentiated services in IP networks.  This Deliverable 4 is the results of this Task.  (These tests focussed mainly on IPv4 because Deliverable 3 addressed IPv6 testing)

Within the deliverables, the reader may encounter the following EURESCOM Shareholder codes:

- AF		FINNET Group  (Finland)

- AU	Telekom Austria AG  (Austria)

- DK	Tele Danmark A/S  (Denmark)

- DT	Deutsche Telekom AG  (Germany)

- FT		France Télécom  (France)

- HT	MATÁV Hungarian Telecommunications Company Ltd.  (Hungary)

- IT		TELECOM ITALIA S.p.a.  (Italy)

- TI		Eircom  (Ireland)

Further project information can be obtained at the following website:

www.eurescom.de/Public/Projects/P800-series/P803/P803.htm

Executive Summary

The Internet Protocol (IP) has become a de facto standard for almost all data communications.  In addition, even real-time traffic, such as voice and video, is nowadays carried over IP-based networks.  In the future, the IP network could thus replace the circuit-switched networks and become the communications media for all applications.  Traditional IP routers provide best-effort service for all packets within the network making sophisticated resource sharing virtually impossible.  If transmission or switching resources are scarce, as is usually the case, all applications and users suffer equally.  This document is an overview of techniques to support differentiated Quality of Service (QoS) in IP networks.

In order to provide better service for certain applications or users, some form of QoS differentiation is needed at the IP layer.  To solve the problem, several components are needed; new service models for the network (either defining an end-to-end service or a per-hop service), signalling for resource reservations combined with admission and policy control and possibly even a QoS-aware routing mechanism.

The Internet Engineering Task Force (IETF) has already defined some of the needed functionality and the process is still ongoing.  The first solution was the Integrated Services (IntServ) architecture with two new service models; Guaranteed Service and Controlled Load.  IntServ provides either deterministic (guaranteed) or predictable (controlled) end-to-end service for individual flows of packets if applied to each router along the path.  To make a reservation for either a unicast or a multicast flow, the IETF defined a signalling protocol called RSVP (Resource reSerVation Protocol).

The problem with RSVP and IntServ is exactly that it provides individual service for each flow.  This makes it suffer from scalability problems in the backbone environment, where possibly thousands of simultaneous reservations could be present in a particular router.  Per-flow classification, scheduling and policing are not cheap to perform at gigabit speeds.

To circumvent this problem, the IETF launched a new effort called Differentiated Services (DiffServ) which relies on a small number of service “classes” within the backbone.  All per-flow processing, such as mapping individual packets or flows to service classes, is done at the network edges.  DiffServ has proven to be scalable for even fast Internet backbones, and some advanced implementations already exist.

However, simple DiffServ alone is not a very flexible solution as it lacks the ability for dynamic reservations with admission control.  Also, the end-to-end services are not defined in trivial terms, since the actual service is the concatenation of the service provided by each hop, which again, is impacted by the prevailing amount of traffic in each of the service classes.

One possible enhancement would be the combined use of RSVP, IntServ and DiffServ.  RSVP would be the signalling protocol and the actual service offered would be done by IntServ where possible and by DiffServ in the backbone areas.  Also DiffServ “bandwidth brokers” could be used to provide dynamics and admission control for DiffServ set-ups.

Other technologies contributing towards Internet QoS standardisation include Internet Telephony and MPLS (MultiProtocol Label Switching). Internet telephony is an important application benefiting from differentiated QoS while MPLS offers a platform for the easy implementation of QoS-based routing.

List of Authors

| Friedhelm Henkel, | Deutsche Telekom AG |
| --- | --- |
| Monika Jäger, | Deutsche Telekom AG |
| Dave O’Meara, | Eircom |
| Michael Barry, | Eircom |
| Markus Isomäki, | FINNET Group |
| Grégoire Sénéclauze, | France Télécom |
| Laurent Rabret, | France Télécom |
| Ferenc Telbisz, | MATÁV Hungarian Telecommunications Company |
| Jan Anderson, | Tele Danmark A/S |
| Günther Körbler, | Telekom Austria AG |

Table of Contents

Preface	i

Executive Summary	iii

List of Authors	iv

Table of Contents	v

Abbreviations	vii

1	Introduction	1

1.1	Structure of this Deliverable	1

2	IETF Integrated Services (IntServ) & RSVP	3

2.1	Architecture	3

2.1.1	Basic Assumptions	4

2.1.2	The Basic Architectural Components	4

2.1.3	Overview of the Traffic Control Components	5

2.2	Service Models	7

2.3	Resource reSerVation Protocol (RSVP)	7

2.4	IntServ and RSVP over Specific Link Layers	9

2.4.1	RSVP and ATM	10

2.4.2	RSVP and Broadcast Media	10

2.4.3	IEEE 802.1p-Type Networks	11

2.4.3	Subnetwork Bandwidth Manager (SBM)	11

2.5	Scalability Issues	12

2.5.1	Class-Based Aggregation	13

2.5.2	Hierarchical RSVP	13

2.6	Billing for IntServ	13

2.7	Status	14

3	IETF Differentiated Services (DiffServ)	17

3.1	Motivation	17

3.2	Architecture	17

3.2.1	Comparisons with Other Approaches	18

3.2.2	Traffic Classification and Conditioning	19

3.3	Proposed Service Models	20

3.3.1	Differentiated Services (DS) Field Structure	20

3.3.2	Per-Hop Behaviours (PHB)	20

3.3.3	Services Built from PHBs	22

3.4	DiffServ over Specific Link Layers	22

3.4.1	DiffServ and Frame Relay	23

3.4.2	DiffServ and ATM	23

3.4.3	DiffServ and IEEE 802.1p	24

3.5	Billing for DiffServ	24

3.5.1	Dynamic Capacity Contracting	25

3.6	IETF Status	25

3.6.1	Unsolved Issues	26

4	Interoperation of IntServ & DiffServ Networks	27

4.1	Benefits of using IntServ with DiffServ	27

4.1.1	Explicit Admission Control	27

4.1.2	Assistance in Traffic Identification/Classification	27

4.2	Interoperability Framework	28

4.2.1	Interoperability Reference Network	28

4.2.2	Service Mapping	29

4.2.3	Edge Routers and Border Routers	29

4.3	Examples of IntServ/DiffServ Interoperation	29

4.3.1	RSVP-Unaware DiffServ Region	29

4.3.2	RSVP-Aware DiffServ Region	31

4.4	Bandwidth Broker	31

4.5	Interoperability Limitations	32

5	Applying QoS to Voice over IP (VoIP)	33

5.1	Why is a Policy Server (PS) Necessary?	33

5.2	H.323 with RSVP-Capable Routers and a PS	34

5.3	H.323 with DiffServ-Capable Routers and a PS	35

5.4	A Possible Global IP Telephony Network	36

5.5	The Future of VoIP	37

6	IETF MultiProtocol Label Switching (MPLS)	39

6.1	Introduction to MPLS	39

6.2	Overview of Label Switching	39

6.3	Overview of MPLS	39

6.4	MPLS QoS Architecture	40

6.5	Evaluation of MPLS	40

6.6	IETF MPLS Working Group	41

6.6.1	Working Items of MPLS Working Group	41

References	43

Abbreviations

| ABR | Available Bit Rate |
| --- | --- |
| AF | Assured Forwarding |
| ATM | Asynchronous Transfer Mode |
| BB | Bandwidth Broker |
| CAR | Committed Access Rate (Cisco) |
| CBQ | Class-Based Queuing |
| CBR | Constant Bit Rate |
| CL | Controlled Load |
| COPS | Common Open Policy Service |
| DiffServ | Differentiated Services |
| DS | Differentiated Services |
| DSCP | DiffServ CodePoint (IP header field) |
| EF | Expedited Forwarding |
| FF | Fixed Filter |
| FIFO | First-In First-Out |
| FQ | Fair Queuing |
| GS | Guaranteed Service |
| IEEE | Institute of Electrical and Electronics Engineers |
| IETF | Internet Engineering Task Force |
| IntServ | Integrated Services |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| ISP | Internet Service Provider |
| ISSLL | Integrated Services over Specific Link Layers |
| LAN | Local Area Network |
| LDAP | Lightweight Directory Access Protocol |
| LSR | Label Switched Routes |
| MAC | Media Access Control |
| MF | Multi-Field |
| MPLS | MultiProtocol Label Switching |
| MTU | Maximum Transmission Unit |
| nrt-VBR | non-real-time VBR |
| PDP | Policy Decision Point |
| PEP | Policy Enforcement Point |
| PHB | Per-Hop Behaviour |
| PS | Policy Server |
| PSTN | Public Switched Telephone Network |
| QoS | Quality of Service |
| RAP | RSVP Admission Policy |
| RED | Random Early Discard |
| RFC | Request for Comments (IETF standards) |
| RIO | Random early discard with In/Out profiles |
| RSVP | Resource reSerVation Protocol |
| rt-VBR | real-time VBR |
| SBM | Subnetwork Bandwidth Manager |
| SE | Shared Explicit |
| SIP | Session Initiation Protocol |
| SLA | Service Level Agreement |
| SS7 | Signalling System 7 |
| SVC | Switched Virtual Circuit |
| TCA | Traffic Conditioning Agreement |
| TCP | Transmission Control Protocol |
| ToS | Type of Service |
| UBR | Unspecified Bit Rate |
| UDP | User Datagram Protocol |
| VBR | Variable Bit Rate |
| VC | Virtual Circuit |
| VoIP | Voice over IP |
| WAN | Wide Area Network |
| WF | Wildcard Filter |
| WFQ | Weighted Fair Queuing |

1	Introduction

The type and content of the data transmitted over the Internet is undergoing a dramatic change.  So far, the Internet has mostly been used to transmit bursty, delay-tolerant (but loss-“intolerant”) data from a server to the user.  Examples are file transfer, web browsing or email.  While delays in receiving data may be annoying (most Internet users are familiar with the term ‘World Wide Wait’!), the user will tolerate some delay in using these applications, and the data is no less valuable if it takes a longer time to arrive.

However, an increasing proportion of traffic is becoming oriented towards data that requires predictable delays.  Examples are listening to music, watching movie clips, Internet telephony (voice and/or video), multimedia conferencing, distributed simulations, real-time control, network games and other real-time applications.  Some of these traffic types can tolerate a small amount of delay, as the data can be buffered before display, i.e., they are not strictly real-time.  An example is the popular Real Video application.  A VoIP application, however, cannot tolerate delay.  Voice data, which arrives late, is of no use to the receiving party! However, these applications may tolerate reduced quality when sufficient data is not available in time.  Whereas a file transfer cannot tolerate dropped packets, a voice call would simply reduce in quality when packets are lost.

The TCP/IP protocol was never designed with these capabilities in mind.  It was designed strictly as a fault-tolerant, reliable, “best-effort” service.  It does not make any attempt to prioritise packets, or to give any guarantees about the time taken to deliver the packet.  If enough bandwidth is available, TCP/IP is capable of providing real-time services, but this is rarely the case.

Several schemes are being investigated which attempt to retrofit Quality of Service (QoS) into the TCP/IP protocol.  At first glance, datagram networks do not seem suitable for real-time traffic.  Individual packets are routed independently across shared networks, so packet transit times can vary significantly.  (Variation in transit delay is called jitter.)  The price tag attached to sharing bandwidth is congestion, leading to jitter and packet loss.  While real-time multimedia traffic contributes heavily to congestion because of the typically large bandwidth requirements, it also suffers more from congestion than non-real-time traffic.  The only effect of congestion on non-real-time traffic is that a transfer takes longer to complete.  In contrast, real-time data becomes obsolete if it doesn’t arrive in time.  For multimedia applications this could be merely a nuisance, but for real-time control applications, it could be disastrous.  As a consequence, real-time applications deliver poor quality during periods of congestion.  What is needed is some way of ensuring that during these periods of congestion, real-time traffic is not affected at all, or is at least given higher priority than non-real-time traffic.

1.1	Structure of this Deliverable

This deliverable provides an introduction to the field of IP QoS.  Different technologies are presented, and their applicability and standardisation status is considered.

Chapter 2 is an overview of the IETF’s Integrated Services (IntServ) architecture and RSVP (Resource reSerVation Protocol), which is in practice tightly coupled with it.  The known scalability problems are presented, and also possible billing scenarios are discussed, as these are necessary for commercial deployment.  The mapping of IntServ to Layer 2 (Link Layer) mechanisms is also covered.

Chapter 3 presents another of the IETF’s QoS techniques; Differentiated Services (DiffServ).  The details of DiffServ are still not fully standardised at this time, but the basic principles are described.  This chapter also considers the problems involved with billing, accounting and mapping to Layer 2 QoS mechanisms.

In order to provide network-wide end-to-end QoS, it may be necessary to use a combination of both IntServ/RSVP and DiffServ.  Alternatively, DiffServ may use some other mechanisms to enable dynamic reservations and admission control.  Both cases are discussed briefly in chapter 4.

Voice-over IP (VoIP) is seen as one of the main applications benefiting from IP QoS.  Chapter 5 discusses how guaranteed-quality VoIP service could be provided using the QoS mechanisms presented in the previous chapters.  Issues, such as authentication and call set-up signalling are covered, as well as presenting a wider view of the problem area.  A more detailed analysis of VoIP is provided in Annex A (Deliverable 4, Volume III).

Finally, Chapter 6 presents a brief overview of MPLS ((MultiProtocol Label Switching).  It describes label switching, the MPLS QoS architecture and the working of the IETF MPLS Working Group.

The readers may come across the terms egde/border/boundary router/node.  There does not seem to be any consistency in the use of these terms even in the official IETF documents.  The authors have chosen not to attempt to harmonise the usage.

2	IETF Integrated Services (IntServ) & RSVP

The IETF has devised an architecture called the Integrated Services model (IntServ) which attempts to layer QoS on top of the current best-effort model.  Applications must be able to specify their bandwidth requirements to the network, which then tries to provide the required resources.  The IETF IntServ working group has agreed upon a new protocol, called the Resource reSerVation Protocol (RSVP) [RFC2205] which, during session set-up, reserves necessary resources (such as bandwidth on an interface) in every intermediate router along the application’s data path.

If every hop on the journey agrees to reserve sufficient resources, then the user will have a dedicated, reserved stream of guaranteed fixed bandwidth for the duration of the session.  Once the session ends, the reserved resources are dropped.  Of course, it only makes sense to reserve resources if the amount of traffic generated during the session is greater than the amount of traffic necessary to set up the reservation.  Hence, bandwidth reservation is not suitable for traffic such as e-mail or Web browsing.  It is aimed at applications such as video streaming, voice telephony, or any other application where a large amount of real-time traffic is generated.  Of course, some policing of the reservations is necessary to ensure a user has the right to set-up the reservation, and that its traffic respects the contract.

Section 2.1 describes the general architecture and Section 2.2 discusses service models.  Section 2.3 is an introduction to the RSVP signalling protocol, and in Section 2.4 the mappings between IntServ and various Layer 2 (Link Layer) QoS methods are covered.  Section 2.5 is about RSVP scalability issues, while Section 2.6 covers billing for IntServ.  Finally, Section 2.7 presents the status of RSVP and IntServ standardisation at the IETF.

2.1	Architecture

The need for support of new distributed applications mostly arising in the field of multimedia has been recognised by the Internet community.  The current Internet architecture is incapable of providing QoS, which is the main requirement of the newly emerging applications from a communications’ point of view.  The IETF has been for some time examining how the Internet architecture can be enhanced to provide such services.

One proposed model, IntServ, is an attempt to merge the advantages of two different paradigms: datagram networks and circuit switched networks.

Datagram networks, like the Internet, maximise network utilisation by multiplexing multiple data streams.  They also provide for multi-point communication and robustness by adapting to network dynamics.  Up to now, however, datagram networks only provided a best-effort delivery service.

On the other hand, current circuit-switched telecommunications and ISDN networks inefficiently utilise network resources when sending bursty data traffic, but can provide service guarantees.

The aim set by the IETF is to provide for robust yet flexible services that allow for QoS support and multi-point communication while making efficient use of network resources.

2.1.1	Basic Assumptions

Multiparty communication is regarded as one of the main requirements of the new applications and therefore multicasting is considered vital for the new Internet infrastructure.  In addition to the provision of QoS for real-time applications, the sharing of bandwidth between different traffic classes is considered a desirable characteristic of a future Internet infrastructure.  Therefore, the IntServ model shall include, besides best-effort and real-time services, a so-called controlled-link service.  The approach taken to introduce these partially new services is not to design a new Internet architecture from scratch, but to supplement and extend the existing Internet architecture with some new components.  The basic assumption of the IntServ approach is that resources must be explicitly managed, otherwise guarantees cannot be granted, no matter whether they are deterministic or statistical, strict or approximate.

The IntServ approach assumes the existence of flow-related state to perform the resource reservation inside the network, and this means a partial shift away from one of the basic principles in the Internet; the so called end-to-end principle.  This claims that flow-related state should only be maintained in the end-systems.  However, in order to relax this paradigm shift away from connectionless to connection-oriented services, the IntServ model operates on soft state in the network, thereby trying to maintain the robustness characteristics of the Internet.  By using soft state, a hybrid form between connectionless and connection-oriented services is used.

Since resource reservation leads to privileges, the IntServ model recognises the need for policy and administrative control, which in turn will lead to authentication requirements.  This handles the issue of whose packets receive the contracted QoS.

Another fundamental assumption of the approach taken is the integration of real-time and non-real-time communication into a single Internet infrastructure, and thereby enabling statistical sharing between these two traffic classes.  This is in contrast to building an entirely new, parallel infrastructure for real-time communication, leaving the Internet architecture unchanged and dedicated to non-real-time traffic.  Hence, a unified protocol stack for both real-time and non-real-time traffic on the Internet layer is envisaged in the implementation of the IntServ model.

2.1.2	The Basic Architectural Components

The Integrated Services working group in the IETF has developed an enhanced Internet service model that includes best-effort service and real-time service.  Together with RSVP [RFC2210], this architecture is a comprehensive approach to provide applications with the type of service they need, with the quality they choose.

To support this capability, two things are required:

- There must be a method of communicating QoS requirements and management information between applications and network elements, and between network elements themselves (hosts and routers).

- Individual network elements along the data path must support mechanisms to control the QoS allocated to individual packets travelling via that network element.

The first requirement above may be provided in a number of ways, but is often implemented using the RSVP protocol.

RSVP communicates a request for QoS along the data path in a hop-by-hop fashion, resulting in a reservation if the request was successful.  A common misunderstanding is that RSVP alone will give a better QoS.  RSVP is only a control protocol that sets up a reservation, but enforcement of the reservation needs to be performed by some other component of the architecture.  It is like making a travel reservation.  The booking system makes sure a seat is available on every leg of the journey, then marks the seat unavailable for everyone else.  However, if there is nobody at the airport to check that only passengers with reservations (and a few stand-by passengers) are allowed on the plane, a reservation will probably not help to get to one’s destination in time.

To honour reservations, routers on the path need to distinguish between best-effort flows and reserved flows.  A flow is a stream of related packets from one source to a unicast or multicast destination.  Routers use sophisticated scheduling techniques to provide service according to the reservations and to other policies.  Reservation set-up, management and enforcement consume bandwidth and processing power in the router.  The number of RSVP messages and the amount of state information needed in the routers are proportional to the number of reservations.  If a router handles tens of thousands of flows, keeping state information for all these can be a problem.  Enforcement of large numbers of reservations at the same time is also very difficult.  On large interfaces, the bottleneck is usually the processing speed of the router, not the bandwidth of the outgoing link.

2.1.3	Overview of the Traffic Control Components

The traffic control module consists of three components:

- Packet Classifier;

- Packet Scheduler;

- Admission Control.

(See Figure 2.1)

Figure 2.1: Traffic Control Components

2.1.3.1	Packet Classifier

The packet classifier performs a mapping of incoming packets into classes, which are characterised by the fact that all packets of the same class get the same treatment from the packet scheduler.  The possible classifications are, for example, all video flows or all flows attributable to a particular organisation.  However, a single data flow can also represent a whole class.  This will be observed particularly at the edge routers of the network, whereas the Internet core routers should preferably be using aggregation mechanisms.  At the moment, packet classification for routers is complicated by the fact that the destination address, which is the only source of information for routing purposes in the Internet, is not sufficient to select the class of service a packet must receive.  (This problem, however, could be alleviated by the use of the flow label field of IPv6.)

2.1.3.2	Packet Scheduler

The packet scheduler’s task is the forwarding of different packet streams using a set of queues and possibly other mechanisms like timers.  In order to achieve the desired QoS provision, the basic function of the scheduler is to reorder these queues, according to a priority scheme.  A possible sub-module of the packet scheduler might be a component called the estimator, which samples statistics of traffic streams that control packet scheduling and admission control.  Some perceive the estimator as an independent module, but usually, if it is considered at all, it is viewed as a part of the packet scheduler.

2.1.3.3	Admission Control

The admission control component implements the decision algorithm used by a router or a host to decide whether a new data flow can be granted its requested resources or not.  In addition to ensuring that QoS guarantees are met, admission control also has the task of enforcing administrative policies on resource reservations and plays an important role in accounting and administrative reporting.  The admission control is not to be confused with policy control, which is performed at the edge of the network to ensure that hosts do not violate their traffic characteristics and which is considered part of the packet scheduler.

The traffic control components have to be realised in the routers as shown in Figure 2.1.  What is still missing is the component that initialises the traffic control modules with the necessary parameters in a distributed fashion, by conveying the application needs from the end-users to the routers.  This is achieved in the IntServ model by the use of RSVP.

2.1.3.4	Packet Dropping

An augmentation of the service model is the integration of a packet dropping mechanism, in which some of the packets can be marked as pre-emptable.  This is led by the observation that in many audio and video streams, some packets are less important than others, and should therefore, in a case of congestion, be dropped first.  It is therefore a means of delivering application-specific information to the network, in order to assist the network to make more reasonable decisions.

The IntServ model proposes to go one step further by introducing expendable packets, which in contrast to preemptable packets do not have to pass admission control any more.  The expectation for these packets is that many of them are to be dropped, while for preemptable packets most should still be delivered, and therefore they are considered a part of the flow subject to admission control.

2.2	Service Models

The IntServ architecture introduces two service models named Controlled Load [RFC2211] and Guaranteed Service [RFC2212].  These specifications determine how traffic is handled within individual network elements.

Guaranteed Service provides firm (mathematically provable) bounds on end-to-end datagram queuing delays.  This makes it possible to provide a service that guarantees both delay and bandwidth.  It is only useful if supported by every node along the path.

Controlled Load provides the client data flow with a QoS closely approximating the QoS that same flow would receive from an unloaded network element, but uses capacity (admission) control to assure that this service is received even when the network element is overloaded.  Therefore, the application may assume that a very high percentage of its transmitted packets will successfully reach the destination, and that the transit delay experienced by a very high percentage of the successfully delivered packets will not be significantly bigger than the minimum transit delay of any packet.

The main features of Controlled Load and Guaranteed Service are summarised in Table 2.1:

Table 2.1: IntServ Service Models

| Template components | Controlled Load | Guaranteed Service |
| --- | --- | --- |
| End-to-End Behaviour | Approximates best-effort over unloaded network | Guaranteed maximum delay |
| Motivation | Applications sensitive to network congestion | Real-time applications |

2.3	Resource reSerVation Protocol (RSVP)

Each node capable of resource reservation has several modules that work together for reservation set-up and enforcement (see Figure 2.2).  The IntServ architecture of routers and hosts is the same, but the implementation is usually different.  An RSVP process on every network element (both hosts and routers) handles all protocol messages needed to set up and tear down reservations.

Figure 2.2: RSVP Implementation Overview

An application requests a certain quality of service from the RSVP process running on the host.  Note that the receiver initiates reservations; if a sender had to maintain a reservation for each receiver, the protocol would not scale for large multicast groups.  A sender does not need to know the number and specifics of reservations or the location of the receivers.  In this respect, RSVP closely follows the multicast model.  Also, a receiver knows best the level of QoS needed.  Different receivers might request and receive different QoS.

The disadvantage of receiver-initiated reservations is that a receiver does not know which path the data packets are taking.  The data path from the sender to the receiver might be different from the path from the receiver to the sender.  To solve this problem, the sender sends a “PATH” message to the unicast or multicast address of the data receiver.  A PATH message contains the unicast address of the next RSVP-capable host upstream (towards the sender).  PATH messages also contain information about the expected size of the flow.  Every node that receives a PATH message saves this information.  It then passes the PATH message on with its own unicast address as the next upstream RSVP hop.  Thus PATH messages build a trail of path states along the data path.

The PATH message contains various information structures, two of which are of interest here.  The first is the SENDER_TSPEC, which contains the QoS control services that the sender offers and the bandwidth they require.  Intermediate routers record this information.  The other is the ADSPEC, which contains information such as the availability of a particular QoS control service at the router and the actual resource available for each control service.  Intermediate routers modify this information to reflect their capabilities.  Thus, when a PATH message arrives at a receiver, the ADSPEC information structure contains a summary of the data path’s available QoS.  Receivers can then use this information to make a QoS reservation that the data path can sustain.

To make a reservation, a receiver sends a reservation request (“RESV”) message towards the source.  Reservation requests specify the requested service and the size of the expected data flow (FLOWSPEC).  It is also specified which packets can use the reservation (known as the FILTERSPEC).  There are three reservation styles:

- Fixed Filter, (FF)

- Shared Explicit, (SE)

- Wildcard Filter, (WF)

In fixed filter style, a reservation is made for packets sent by exactly one sender that is specified in the FILTERSPEC.  In shared-explicit style, packets from several sources listed explicitly in the FILTERSPEC can use the reservation.  In wildcard style, all sources sending to the multicast group address share the reservation.

Each time a “RESV” message is received at a node, the RSVP process checks with the Policy Control module to see if the user has administrative permission to make a reservation.  Future accounting for reservations will also be carried out by Policy Control.  The RSVP process also checks with the Admission Control module to find out whether the node has sufficient resources to supply the requested QoS.

If either check fails, the RSVP process sends an error notification back to the host.  If both checks succeed, parameters are set in other modules (such as the Packet Classifier & Packet Scheduler modules) to enforce the reservation.  The RSVP process then sends the reservation request to the next hop on the data path.  As soon as every node on the data path accepts the reservation, the flow should receive the requested QoS.

The Packet Classifier and Packet Scheduler modules on every node are responsible for the QoS given to a flow for which a reservation has been made.  The Packet Classifier looks at every data packet to determine whether the appropriate flow has a reservation and what QoS the flow should receive.  Flows are identified in one of several formats.  The simplest format contains the sender IP address and sender port together with the destination address and port.  In this case, the Packet Classifier has to examine the IP and UDP/TCP headers.  In the future, IPv6 and/or MPLS flow labels could be used to specify RSVP flows, making the classification of packets more efficient.

The Packet Scheduler then makes the forwarding decision according to the QoS class.  For example, the Packet Scheduler decides which queue the packet is to be placed in.  The Packet Scheduler also polices the data flow to ensure that the reservations are not being violated.  Packets which are outside the specified flow parameters could be marked as ‘Out-of-Profile’, could be treated as best-effort, or could simply be discarded.  The Packet Scheduler is a key component of the architecture because it actually gives different services to different flows.  To ensure that flows receive their requested QoS, the Packet Schedulers on all nodes and routers must support the distinction between different services.

Of course, on the Internet, routes are subject to dynamic change, and receivers can disappear.  RSVP dynamically copes with such changes by maintaining the reservations as a soft state.  This means that the senders and receivers periodically send refresh PATH and RESV messages.  Reservations automatically time out if such refresh messages are not received within a specified time period.  Senders and receivers can also explicitly terminate reservations via standard RSVP messages.

2.4	IntServ and RSVP over Specific Link Layers

The IETF Integrated Services over Specific Link Layers (ISSLL) working group has been set up to examine these issues.  The work completed so far has resulted in technology-independent protocols and specifications.  Focused engineering work to define the mapping of these universal specifications onto specific subnetwork technologies is now required.

It should be obvious that if the underlying transport network has already implemented some form of QoS, the interworking from layer-3 QoS to layer-2 QoS is really only a mapping as mentioned above.  As we shall see, RSVP over ATM and RSVP over IEEE 802.1p take this route.  RSVP over a transport network that does not have any inherent QoS will also be examined.

2.4.1	RSVP and ATM

One of the key features of ATM technology is the ability to request a point-to-point Virtual Circuit (VC) with a specified QoS.  ATM also supports point-to-multipoint VCs with a specified QoS, which provides a mechanism for supporting IP multicast.  It is only natural that the IntServ model should utilise the QoS properties of the underlying ATM link layer.

A key remaining issue for IP in an ATM environment is the integration of RSVP signalling and ATM Q.2931 signalling [Q.2931].  RSVP is the signalling protocol used at layer 3 to convey QoS requests.  Q.2931 is the native ATM signalling protocol used for setting up and tearing down VCs with specific QoS constraints.  The problem breaks down to several distinct areas; the mapping of IntServ models to ATM service categories and QoS parameters, the operation of RSVP over ATM, and VC management.

Mapping IntServ models to ATM service categories and QoS parameters is a matter of determining which categories can support the goals of the service models and matching up the parameters and variables between the IntServ description and the ATM description(s).  Since ATM has such a wide variety of service categories and parameters, more than one ATM service category should be able to support each of the IntServ models.  This will provide flexibility in configuration and deployment.  This work is ongoing.

2.4.2	RSVP and Broadcast Media

Connectionless packet networks in general, and LANs in particular, work today because of scaling choices in network provisioning.  Typically, excess bandwidth and buffering is provisioned in the network to absorb the traffic sourced by higher layer protocols, often sufficient to cause their transmission windows to run out on a statistical basis, so that network overloads are rare and transient and the expected loading is very low.

With the advent of time-critical high-bandwidth traffic, such over-provisioning has become far less easy to achieve.  Time-critical frames may be queued for annoyingly long periods of time behind temporary bursts of file transfer traffic, particularly at network bottleneck points.  For example, bottlenecks occur at the 100 Mbps to 10 Mbps transition that might occur between the riser to the wiring closet and the final link to the user from a desktop switch.  In this case, however, if it is known a priori that time-critical traffic is a small fraction of the total bandwidth, (either by application design, on the basis of statistics, or by administrative control), it suffices to give it strict priority over the non-time-critical traffic.  The worst case delay experienced by the time-critical traffic is roughly the maximum transmission time of a maximum length non-time-critical frame; less than a millisecond for 10 Mbps Ethernet, and well below the end-to-end delay budget based on human perception times.

When more than one priority service is to be offered by a network element, e.g., one which supports both Controlled Load as well as Guaranteed Service, the requirements for the scheduling discipline becomes more complex.  In order to provide the required isolation between the service classes, it will probably be necessary for them to be queued separately.  There is then an issue of how to service the queues, which requires a combination of admission control and more intelligent queuing disciplines.  As with the service specifications themselves, the specification of queuing algorithms is beyond the scope of this document.

2.4.3	IEEE 802.1p-Type Networks

The IEEE 802.1 Interworking Task Group has recently agreed on enhancements to the basic MAC Service provided in bridged LANs.  As a supplement to the original IEEE MAC Bridges standard, the update 802.1p proposes differential traffic class queuing and access to media on the basis of a “user_priority” signalled in frames.

As with ATM, many options are available for mapping aggregations of flows described by IntServ service models (Best Effort, Controlled Load, and Guaranteed Service are the services considered here) onto user_priority classes (an example mapping is shown below).  Note that the mechanisms or algorithms that a network element (e.g., an Ethernet switch) needs to perform to implement these mappings are not specified; this is an implementation choice and does not matter so long as the requirements for the particular service model are met.

Table 2.2: Mapping of IntServ to IEEE 802.1p

| user_priority | Service |
| --- | --- |
| 0 | Default, (assumed to be Best Effort) |
| 1 | Background, (“less than” Best Effort) |
| 2 | Reserved |
| 3 | Reserved |
| 4 | Controlled Load |
| 5 | Guaranteed Service, 100ms bound |
| 6 | Guaranteed Service, 10ms bound |
| 7 | Reserved |

With this example set of mappings, all traffic that uses the Controlled Load service is mapped to a single user_priority class whilst that for Guaranteed Service is placed into one of two user_priority classes with different delay bounds.  Unreserved best-effort traffic is mapped to another.

2.4.4	Subnetwork Bandwidth Manager (SBM)

Although IEEE Standard 802.1p is emerging, the vast majority of the installed Ethernet equipment does not support this feature.  And even if it gets deployed, it still does not address admission control.

The only possibility to provide any predictable service over an Ethernet segment is to somehow limit the allowed traffic in that segment.  To provide admission control in LAN segments for RSVP reserved IntServ traffic, the IETF ISSLL (Integrated Services over Specific Link Layers) working group has specified an entity called Subnetwork Bandwidth Manager (SBM).  The SBM’s task is to control all RSVP reservations over a specific LAN segment and deny a reservation, if the resource level set by the network administration would be exceeded [SBM].

In the IETF model, there is a designated SBM (elected by a formal election process) in each segment.  The SBM can reside in any network component, host, LAN switch or router.  Every RSVP-capable host in the segment learns the address of the current SBM by listening to a specific IP multicast address.  When any host generates an RSVP path message, it does not send it directly to the next hop router, but rather sends it to SBM.  The RSVP message is extended with additional objects, which for example define the IP and MAC addresses of the next hop router (SBM does not have to be aware of them).  SBM then installs an ordinary PATH state and puts its own IP address to the Previous Hop (PHOP) object and relays the PATH message forward to the router (assuming that there are no chained SBMs).  When the router receives a RESV message corresponding to the PATH message, its PATH state points to the SBM, via which the RESV message is conveyed to the host.  SBM can decide whether to accept the new reservation based on the prevailing reservations and the asked QoS.

The weakness of SBM is that it has no means to control the amount of best-effort traffic in the LAN.  If the network is composed of hubs or if 802.1p is not used, it is not possible to separate best-effort traffic from Controlled Load or Guaranteed Service traffic.

2.5	Scalability Issues

The primary scaling problem with RSVP is that of managing the reservation state for a large number of sessions [SCHW].  The number of RSVP control messages processed by each router is proportional to the number of QoS flows going through the router.  RSVP deals with application-level flows, such as one multicast audio session or a video transmission from a single source.  Reservation state is kept on a per-flow basis.  Thus, managing state and processing control messages scales linearly with the number of flows.  However, managing reservation state puts a heavy strain on routers with large interfaces.  Information about thousands of reservations needs to be stored, accessed and changed.  The primary function of routers is packet forwarding.  Managing a large amount of state information and performing additional lookups will necessarily degrade router performance.  The management capabilities of RSVP routers must scale in proportion to their forwarding path bandwidth to fully utilise the capacities.  Unicast routing tables store information per destination, aggregated by hierarchical routing.  Multicast routing tables store information per multicast session and possibly per sender, depending on the multicast routing protocol.  Aggregation of multicast routing state across groups is impossible with the current addressing scheme.  In addition to unicast and multicast routing state, RSVP needs per-flow state, further straining the router’s management capabilities.  The amount of routing state in a router depends only on the network topology and is insensitive to the size of the router’s links.  In contrast, the state required for RSVP grows with the bandwidth of the links.  The larger the links, the more flows can be served, but the more state information needs to be managed.  Compared to unicast routing state, reservation state is relatively short-lived and thus frequently changed.

As links become even larger and support even more reservations, however, it is unlikely that the management capabilities can keep up.  It is not expected that large routers on inter-domain backbones keep per-flow state.  Some form of aggregation will be necessary.  Aggregation means treating several RSVP flows as one.  Of course, only flows with similar QoS requirements can be aggregated.  Flows that are aggregated into a super-flow share the delay.  Isolation is not possible for aggregated flows.  Two methods have been proposed:

- Class-Based Aggregation;

- Hierarchical RSVP.

2.5.1	Class-Based Aggregation

On entry into the aggregating region, each flow, for which a reservation was made, is assigned to one of the service classes.  Flows with similar service requirements are grouped together into a service class.  (Service class definition and flow assignment is the subject of ongoing research.) Each packet is marked with a tag that identifies which service the flow should receive.  For IP, this tag could consist of the Type of Service (ToS) bits in the packet header or the packet could be encapsulated.  Inside the aggregating regions, packets are scheduled according to their assigned service class.  Because the number of classes is fixed, packet scheduling is far less expensive.  However, there is a risk of congestion within any individual service class.  Instead of simply combining all flows blindly into one service class, the overall bandwidth available for each service class can be specified.  RSVP admission control is used to admit new flows if there is sufficient bandwidth within the service class.  In this fashion, the advantages of Admission Control still apply, but the packets within each service class can be processed and routed far more efficiently.

2.5.2	Hierarchical RSVP

The IETF RSVP Working Group is examining hierarchical RSVP.  While set-up and teardown patterns of single RSVP flows are unpredictable, the aggregation of hundreds of flows seems more stable.  The idea of hierarchical RSVP is that routers at the edge of aggregating regions use RSVP to reserve large “pipes” with particular QoS parameters through the region.

The advantage of hierarchical RSVP is the reduction of reservation state.  Routers in the interior of aggregating regions only keep reservation state for the larger pipe reservations.  Packet scheduling is simplified by offering only a few QoS choices.  The main disadvantage is that packet classifying is still done by looking far into the packet headers and comparing source and destination against the (now shorter) list of reservations.

2.6	Billing for IntServ

Obviously, any ISP providing a bandwidth-reservation service is expected to charge a premium for this service.  At present, very little work has been carried out regarding this subject.  It is expected that billing will be handled by the Policy Control & Admission Control modules.  If the user is administratively permitted (by Policy Control) to make the reservation, and the reservation is technically possible (Admission Control), then a charge can be levied.  Individual ISPs will probably determine a charging structure, based on combinations of bandwidth reserved, time reserved, data transferred etc.

It has been proposed that RSVP could be modified to carry charging-related information.  PATH and RESV messages could carry price quote requests, actual price information, and “willingness to pay” information.  The RAP (RSVP Admission Policy) working group in the IETF has designed a protocol called COPS (Common Open Policy Service) for outsourcing the policy decisions.  The COPS protocol can be used to exchange policy information between a Policy Server (PS) and its clients.  One example of a policy client is RSVP routers that must exercise policy-based admission control over RSVP usage.  It is assumed that at least one Policy Server exists in each controlled administrative domain.

Some advantages of schemes such as these are:

- Carrying charging messages with the reservation messages reduces the protocol overhead.

- Periods of reserved bandwidth could be accounted and charged at each router by using the FLOWSPEC parameter as an alternative to actually ‘measuring’ the bandwidth of the flow.  Allowances would have to be made for when the reserved bandwidth guarantee is not met.

- Granularity of the charging process can be controlled by the frequency of the RSVP refresh messages.

- Asynchronous messages can be used at any time e.g., for querying the current market price / alternative routes.

- RSVP messages are processed and forwarded hop-by-hop.  This allows every provider to collect money for its own resources.

During tests, it has been demonstrated that the processing overhead for large numbers of flows and dynamic pricing schemes was measured at less than 2.3% and protocol overhead was typically 0.75%.

In the future, we can expect economic and network models to be combined and enhanced.  For example, we could see the emergence of new protocols to allow least-cost routing functions (in monetary terms), or intelligent agents which will act as bandwidth brokers.  In an increasingly competitive market, a trend towards ‘dynamic’ pricing models can be envisaged.  Full deployment of multicast services with built-in cost sharing functionality could be the final reason to converge broadcast media (e.g., TV & radio) into the Internet and make them globally available.

2.7	Status

Version 1 of the RSVP protocol is now an Internet IETF standard.  Despite the amount of work which has been done in this area and the ever-increasing number of RSVP implementations available, not many sites appear to be using RSVP yet.  The underlying packet queuing techniques may need some more work before they are reliable, or the inherent scaling problems of RSVP may stop it from taking off as originally intended.  Especially Internet-wide end-to-end IntServ/RSVP seems not to be happening.

Recently, however, RSVP with some extensions has been proposed as a signalling protocol for Multi-Protocol Label Switching (MPLS).  Also IntServ/RSVP could be utilised at the network edges with more scalable solutions, such as DiffServ, in the core.  In these cases the scalability problems are not as critical any more.  After all, RSVP is the only “standardised” method for applications to request specific resources in the network.

3	IETF Differentiated Services (DiffServ)

Differential Service (DiffServ) mechanisms allow ISPs to allocate different levels of service to different users of the Internet.  “Broadly speaking, any traffic management or bandwidth control mechanism that treats different users differently - ranging from simple Weighted Fair Queuing [WFQ] to RSVP and per-session traffic scheduling - counts.  However, in common Internet usage, the term is coming to mean any relatively simple, lightweight mechanism that does not depend entirely on per-flow resource reservation” [DiffServ].

Section 3.1 outlines the motivation behind DiffServ.  An architecture for its deployment in the Internet will be presented in Section 3.2.  Each corporate or ISP network is a DiffServ domain.  Within a domain, traffic classification and conditioning functions in each node implement the Per-Hop Behaviour (PHB) indicated by the DiffServ CodePoint (DSCP) in the packet header, as explained in Section 3.3.  A brief overview of two Per-Hop Behaviours, the Expedited Forwarding PHB and the Assured Forwarding PHB is also given here.  Section 3.4 discusses the use of DiffServ over specific link layers.  Many link layers provide some form of differentiated services.  These can be exploited by DiffServ to preserve the relative priority of the DiffServ PHBs.  A framework for pricing differentiated services based on dynamic capacity contracting is presented in Section 3.5, while Section 3.6 presents the current status of the DiffServ task in the IETF.

3.1	Motivation

In the past, limitations of the IntServ model were recognised as impeding its deployment in the Internet at a large scale.  Basic limitations were due to the reliance of RSVP on per-flow state and per-flow processing, which raises scalability concerns in large networks.

Consequently, simple and coarse methods of providing differentiated ‘classes’ of service for IP traffic were needed.  Since the DiffServ model is biased towards long-term allocation rather than on-demand reservation, the guarantees for any Differentiated Services come through proper provisioning.

The IETF working group specified several principles that the DiffServ model should follow:

- Works with existing applications

- Decouples the service behaviour from the particular application in use

- Does not depends on hop-by-hop application signalling

- Utilises only aggregated classification state within the network core

- Permits reasonable interoperability with non-compliant network nodes

- Provides service differentiation in one direction of traffic flow and is therefore asymmetric

3.2	Architecture

The DiffServ architecture [RFC2475] is based on a simple model where traffic entering a network is classified and possibly conditioned at the boundaries of the network, and assigned to different behaviour aggregates.  Each behaviour aggregate is identified by a single DSCP, which is contained in the packet header.  Within the network, packets are forwarded according to the PHB associated with the DSCP.

Figure 3.1 shows a network architecture for the deployment of DiffServ, consisting of a set of interconnected DiffServ domains.  (A DiffServ domain consists of one or more networks under the same administration, for example an ISP or a corporate Intranet.)

Figure 3.1: DiffServ Network Architecture

DiffServ boundary nodes classify and possibly condition traffic entering the domain to ensure that traffic is appropriately marked to select a PHB which is supported by that domain.  Nodes within the domain map the DSCP value to one of the supported PHBs using either a standardised or customised mapping [RFC2474].

Between DiffServ domains, Service Level Agreements (SLAs) and Traffic Conditioning Agreements (TCAs) are used to specify traffic classifications and the corresponding forwarding service a customer should receive.  Such agreements are established between an upstream domain and a downstream domain.

3.2.1	Comparisons with Other Approaches

The DiffServ architecture can be contrasted with other existing models of service differentiation:

- Relative Priority: e.g., IPv4 Precedence marking, Token Ring priority.  An application, host, or proxy node selects a relative priority or “precedence” for a packet.  Network nodes along the path then apply the appropriate priority forwarding behaviour.  DiffServ can be considered as a refinement to this model.

- Service marking: e.g., IPv4 Type of Service (ToS).  In this example, each packet is marked with a request for a “type of service” (e.g., low delay).  This model is subtly different from DiffServ architecture.  The ToS markings are very generic and do not span the range of possible service semantics.

- Label switching (or Virtual Circuit) Model: (e.g., Frame Relay, ATM, and MPLS).  In this model, path forwarding state and traffic management or QoS state is established for traffic streams on each hop along a network path.  This model permits finer granularity resource allocation to traffic streams, since label values are only significant on a single link.  Label switching semantics govern the next-hop selection, allowing a traffic stream to follow a specially engineered path through the network.

- IntServ/RSVP Model: This model allows senders and receivers to exchange signalling messages, which establish additional packet forwarding states in each transit node.  The amount of state in each node scales in proportion to the number of concurrent reservations.  It also requires application support for the RSVP signalling protocol.

- “Static” Per Hop Classification: A variant of the IntServ/RSVP model eliminates the requirement for hop-by-hop signalling by utilising only “static” classification and forwarding policies which are implemented in each node along a network path.  These policies are updated on administrative timescales and not in response to the instantaneous mix of microflows active in the network.

3.2.2	Traffic Classification and Conditioning

Traffic entering a DiffServ domain must be classified, marked and possibly conditioned according to the SLAs and TCAs in force at that domain.  A logical view of the packet classifying and traffic conditioning functions in a DiffServ node is shown in Figure 3.2.

Figure 3.2: Packet Classifier and Traffic Conditioner

It includes the following:

- Packet Classifiers select packets in a traffic stream based on the content of some portion of the packet header.  Classifiers are used to “steer” packets matching some specified rule to an element of a traffic conditioner for further processing.  Classifiers must be configured by some management procedure in accordance with the appropriate TCA.

- Traffic Conditioners may contain the following functions: meter, marker, shaper, and dropper:

- Meters are used to measure the traffic stream against a traffic profile.  The state of the meter with respect to a particular packet (e.g., whether it is in-profile or out-of-profile) may be used to affect a marking, shaping, or dropping action.

- Markers set the DiffServ field of a packet to a particular CodePoint.

- Shapers delay some or all of the packets in a traffic stream in order to bring the stream into compliance with a traffic profile.  Shapers usually take the form of a Token Bucket, which essentially specifies the continually sustainable data rate and the extent to which the data rate can exceed the sustainable level for short periods of time [PAR94].

- Droppers discard some or all of the packets in a traffic stream in order to bring the stream into compliance with a traffic profile.  This process is known as “policing” the stream.

3.3	Proposed Service Models

At each network node in a DiffServ domain, the DSCP contained in a field in the IP header is mapped to a particular forwarding treatment of PHB.  DSCPs may be chosen to have standardised or local meanings.

3.3.1	Differentiated Services (DS) Field Structure

The IP header field known as the Differentiated Services (DS) Field is shown Figure 3.3.  Six bits of the DS field are used as the DSCP.  The two-bit “Currently Unused” (CU) field is reserved and may be assigned later, e.g., for explicit congestion notification.  It is intended to supersede existing definitions for the IPv4 ToS octet [RFC791] and the IPv6 Traffic Class octet [IPv6-Draft].

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DSCP |  |  |  |  |  | CU |  |

DSCP:	DiffServ CodePoint

CU:	Currently Unused

Figure 3.3: DS Field Structure

DiffServ-compliant nodes select the relevant PHB for the packet by matching the value of the entire six-bit DSCP field.  To maintain partial backwards compatibility with existing implementations of differential services (based on the IP precedence field in the IPv4 ToS octet), a number of CodePoints are reserved to indicate PHBs that are compatible with known current uses of the IP precedence field.  Compatible CodePoints are of the form xxx000, where the first three bits correspond to the equivalent definition for the IPv4 precedence field.  CodePoint 000000 is mapped to the default PHB implemented in a node, and should at least correspond to the common best-effort forwarding behaviour as standardised in [RFC1812]

3.3.2	Per-Hop Behaviours (PHB)

A Per-Hop Behaviour (PHB) is a description of the externally observable forwarding behaviour of a DiffServ node applied to a particular DiffServ behaviour aggregate.  The simplest example of a PHB is one that guarantees a minimal bandwidth allocation of X% of a link (over some reasonable time interval) to a behaviour aggregate.  This PHB can be fairly easily measured under a variety of competing traffic conditions.  A slightly more complex PHB would guarantee a minimal bandwidth allocation of X% of a link, with proportional fair sharing of any excess link capacity.

PHBs may be specified in terms of resources (e.g., buffer, bandwidth), priority relative to other PHBs, or in terms of their relative observable traffic characteristics (e.g., delay, loss).  PHBs may be used as building blocks to allocate resources and may be specified as a group for consistency.  PHB groups will usually share a common constraint applying to each PHB within the group, such as a packet scheduling or buffer management policy.

PHBs are not defined in terms of particular implementation mechanisms.  In general, a variety of implementation mechanisms may be suitable for implementing a particular PHB group.

Standardised PHBs have a recommended CodePoint.  However, [RFC2474] leaves provisions for locally configurable mappings.  A CodePoint-to-PHB mapping table may contain both 11 and N1 mappings.  Two PHBs currently standardised by the IETF are:

- Expedited Forwarding PHB;

- Assured Forwarding PHB.

3.3.2.1	Expedited Forwarding PHB

The Expedited Forwarding (EF) PHB is defined as a forwarding treatment for a particular DiffServ aggregate where the departure rate of the aggregate’s packets from any DiffServ node must equal or exceed a configurable rate [RFC2598].  The EF traffic should receive this rate independent of the intensity of any other traffic attempting to transit the node.  It should average at least the configured rate when measured over any time interval equal to or longer than a packet time at the configured rate.  (The configured minimum rate must be stored in a non-volatile configuration).

3.3.2.1.1	Mechanisms to Implement the EF PHB

A simple priority queue will give the appropriate behaviour as long as there is no higher priority queue that could pre-empt the EF for more than a packet time at the configured rate.  It is also possible to use a single queue in a group of queues serviced by a weighted round-robin scheduler where the share of the output bandwidth assigned to the EF queue is equal to the configured rate.  All of these mechanisms give the basic properties required for the EF PHB though different choices result in differences in auxiliary behaviour, such as jitter, seen by individual flows.

3.3.2.2	Assured Forwarding PHB

The Assured Forwarding (AF) PHB is a means for a DS domain to provide a scale of packet delivery assurance [RFC2597].  There are currently four AF classes being defined, with increasing relative priority.  Within each class, packets can be marked with one of three levels of drop precedence, which determines the probability of the packet being dropped in the case of congestion within that class.

In order to avoid discarding AF packets in case of congestion, the assumption is that the volume of AF packets that enter the provider DS domain is limited by traffic-conditioning actions at the ingress (entry) to the domain.  Such an assumption is not needed for best-effort packets, since the provider DS domain needs to deliver them only as best-effort on the basis of available resources.

3.3.2.2.1	Queuing and Discard Behaviour

A DS node must place all packets of the same flow to the same class in the same order in which they were received.  In order to protect AF packets from being discarded due to congestion, a DS node must start discarding best-effort packets well before the queue space allocated for the particular AF packets starts to fill up.  The discard mechanism may be based, for example, on the RED (Random Early Discard) algorithm.

3.3.2.2.2	Packet Promotion/Demotion

AF packets may be promoted or demoted within the AF PHB by traffic conditioning actions at the edge of a DS domain.  This means moving the packet from one AF class to another AF class with higher or lower relative priority.  In addition to promoting and demoting AF packets, the traffic conditioning actions at the edge of a DS domain may also include re-marking packets with the default (best-effort) PHB.

3.3.3	Services Built from PHBs

PHBs will provide predictable building blocks from which a variety of services might be built.  In theory, there is thus no need to standardise the services as these are realised by combining traffic conditioners at boundaries with Per-Hop Behaviours.  Some proposed services for DiffServ are listed in Table 3.1.

Table 3.1: Examples of DiffServ Services

| Service Name | Description |
| --- | --- |
| Best Effort | “Like today” (as much as possible; as soon as possible) |
| Premium Service | Peak-limited, extremely low-delay service, resembling a leased line |
| Assured Service | Minimum rate (=target rate), i.e., minimum throughput even during congestion period Additional bandwidth based on best-effort There are currently at least two Internet-Drafts available for policing and marking of AF traffic; [AFmark1], [AFmark2]. |
| Olympic Service | Deployment implementation of rate-based link share scheduler behaviour at each hop Three levels of service: e.g., gold 60%, silver 30%, bronze 10%. |

3.4	DiffServ over Specific Link Layers

This section looks at the application of DiffServ over different link layers, specifically ATM, Frame Relay and IEEE 802.1p.  Each of these provide different levels of service based on traffic type and load in the network.  Applying DiffServ to the link layers involves the specification of a suitable mapping between the PHBs implemented in a DiffServ node and the appropriate level of service provided by the link layer.

3.4.1	DiffServ and Frame Relay

Frame Relay allows a basic level of over-subscription of basic point-to-point Virtual Circuits (VCs).  Individual flows can increase their transfer rate, with the intent of occupying otherwise idle transmission capacity which is not being utilised by other virtual circuits sharing the same transmission paths.  When the sender is not using the entire committed rate within any of the configured VCs, other senders can utilise the transmission space with discard-eligible frames.  Using Frame Relay as the link layer for DiffServ involves the setting up of a mapping between the PHBs in the DiffServ node the Frame Relay VCs.  There are two options for this:

- One VC per PHB: Packets entering the node are processed by a PHB according to their DSCP.  Processed packets are then transmitted on a VC associated with the PHB.  In effect, a mapping has been made between the DSCP and the Frame Relay VC.  If the VC for a given PHB is not at full capacity, then other VCs can use the available transmission space to transmit frames marked as discard-eligible frames.  If the network becomes congested, these frames are preferentially dropped.

- One VC for Multiple PHBs: This involves multiplexing packets from different PHBs onto one or more Frame Relay VCs.  When the VC reaches its Committed Information Rate (CIR), then frames for the PHB with lowest priority are preferentially marked as discard-eligible frames.  As the network becomes congested, then the frames associated with the PHB with the lowest priority are dropped first.  This is equivalent to drop precedence within a single AF class, so AF maps nicely onto Frame Relay.

3.4.2	DiffServ and ATM

The ATM Forum defines a number of different classes of services that can be offered over ATM:

- Constant Bit Rate (CBR): This service category is used for virtual circuits that transport traffic at a consistent bit-rate, where there is an inherent reliance on time synchronisation between the traffic source and destination.  CBR is tailored for any type of data for which the end-systems require predictable response time and a static amount of bandwidth continuously available for the lifetime of the connection

This type of service is generally not applicable with bursty, non-real-time Internet services.  However, it can be used to guarantee that a certain amount of resources are constantly available in the network for the transmission of data for high priority PHBs.

- Variable Bit Rate (VBR): The VBR service categories are generally used for any class of applications that might benefit from sending data at variable rates in order to use network resources more efficiently.

Real-Time VBR (rt-VBR), for example, is used for multimedia applications with lossy properties, applications that can tolerate a small amount of cell loss without noticeably degrading the quality of the presentation.

Non-Real-Time VBR (nrt-VBR), on the other hand, is used for transaction-oriented applications, where traffic is sporadic and bursty, and for which there is no inherent reliance on time synchronisation between the traffic source and destination, but there is a need for an attempt at a guaranteed bandwidth or latency.  Non-real-time VBR is ideally suited to DiffServ as it can provide a guaranteed bandwidth for high priority PHBs.

- Available Bit Rate (ABR): The ABR service category is similar to nrt-VBR, because it is also used to transport variable bit-rate traffic for which there is no reliance on time synchronisation and for which no required guarantees of bandwidth or latency exist.  ABR provides a best-effort transport service in which flow-control mechanisms are used to adjust the amount of bandwidth available to the traffic originator.

DiffServ can use ABR for the transmission of data for low-priority PHBs.  When network traffic is low, there is enough bandwidth available for the transmission of data from low-priority PHBs.  As network congestion increases the amount of bandwidth available to the low-priority PHBs decreases.  If higher-priority PHBs are using CBR or VBR services, then the bandwidth available to them will remain constant.

- Unspecified Bit Rate (UBR): The UBR service category is also similar to nrt-VBR, because it is used for connections that transport variable bit-rate traffic for which there is no reliance on time synchronisation between the traffic source and destination.  However, unlike ABR, there are no flow-control mechanisms to dynamically adjust the amount of bandwidth available to the user.  UBR is generally used for applications that are very tolerant of delay and cell loss.  UBR is only suitable for the very lowest priority PHBs.  The services it provides are no better than the standard best-effort routing provided by the Internet.

3.4.3	DiffServ and IEEE 802.1p

A supplement to the original IEEE MAC Bridges standard [ISO10037], the proposed 802.1p specification [IEEEP802.1p] provides a method to allow preferential queuing and access to media resources by traffic class, on the basis of a “priority” value signalled in the frame.  The IEEE 802.1p specification, if adopted, will provide a way to transport this value (called “user_priority”) across the subnetwork in a consistent method for MAC-layer media types (such as Ethernet or Token Ring), using an extended frame format.  Of course, this also implies that 802.1p-compliant hardware may have to be deployed to fully realise these capabilities.

The current 802.1p draft defines the “user_priority” field as a 3-bit value, resulting in a variable range of values between 0 and 7 (decimal), with 7 indicating the highest relative priority and 0 indicating the lowest relative priority.  IEEE 802.1p can support DiffServ by the mapping of PHBs to the appropriate “user_priority” field.  Data to be transmitted for PHBs with higher priority are allocated the higher-priority “user_priority” fields.

3.5	Billing for DiffServ

A key constraint in the DiffServ architecture is the clear distinction between interior routers and boundary routers.  Interior routers cannot do signalling and accounting; they are optimised for forwarding.  Therefore, these routers cannot participate in pricing-related functions directly.  The policy and accounting functions have to be concentrated at the boundary of the DiffServ domain.

Limited term contracts have a specified life during which users get value-added services by paying a fixed price at the time of entering into the contract.  Since contracts are not for an infinite time span, provisioning efficiencies can be gained by avoiding resource reservations.  Term-contracts would also be attractive from a user’s standpoint as they can be invoked, based on needs.

3.5.1	Dynamic Capacity Contracting

Users should pay for the privilege of expecting a premium service.  This can be captured in the form of an “expected capacity” contract.  In such a framework, users would negotiate short-term contracts with the providers.

Consider an example of a simple service contract.  Assuming that the interior routers can support eight priority classes and two levels of drop within each class.  Contracts can be structured using these sixteen mechanisms in combination with time and volume.  For example, a contract can be as follows: 2KB of data every second will be marked as “IN” in class “i” and any excess traffic will be marked as “OUT” in class “i”.  This service allows sending up to 2KB of data every second on a priority basis within class “i”.  Any excess data will be sent in class “i” but with a lower probability of delivery.  In this example, the volume dimension (“2KB”) and the time dimension (“per second”) are used to define the service.  This is an example of a “better-than-best-effort” service, where both the user and provider have more control of how to share the limited bandwidth resources.

3.5.1.1	A Sample Pricing Scheme

A provider could have a set of classes (say 1 to N) with priority scheduling (where 1 is the highest priority).  Further, it might split each class into two subclasses.  For example, we may have subclasses (1a, 1b), (2a, 2b) etc.  One subclass of every class (say the “a” subclass) could be sold as a flat-priced service to satisfy a large number of users who want flat rate.  The only “admission control” is the economic one; the flat price for 1a would be much larger than 2a, or any variable price in 2b and so on.  The higher flat price is because 1a is given priority over 2a, 2b.  The key point is that we do not assure anything during congestion (seen by this class) for this subclass “a”.  The other subclass, “b”, can be charged based on congestion.

3.6	IETF Status

The DiffServ Working Group has produced four RFCs to date; [RFC2474], [RFC2475], [RFC2597], [RFC2598].  The Working Group will assign specific PHBs to a small number of particular DSCPs within the DS field.  The standardised code-points will only apply to PHBs already in widespread common usage within the global Internet, e.g., the forwarding treatment received by best-effort traffic.  Another goal of the Working Group is to experiment with other PHBs that can be used to produce additional services.  These will be documented in Internet drafts.  The Working Group will also investigate the additional components necessary to support differentiated services, including such traffic conditioners as traffic shapers and packet markers that could be used at the boundaries of networks.  The Working Group may also define mechanisms such as the Lightweight Directory Access Protocol (LDAP) schemata to map user service profiles into differentiated services at the network boundary.  In addition to the RFCs above, a number of Internet Drafts have been published.  For up-to-date information on the IETF DiffServ Working Group activities, see:

www.ietf.org/html.charters/diffserv-charter.html

3.6.1	Unsolved Issues

While a number of the published drafts are considered stable, others are not and a number of points pertaining to the adoption of DiffServ and associated PHBs are still not finalised.  Some of the topics under discussion in the DiffServ Working Group include:

- Operation of boundary mechanisms and traffic conditioners,

- Does the DiffServ group produce documents defining behavioural building blocks, which must later be combined and allocated to CodePoints? Or is the group defining different DiffServ CodePoints and their associated behaviour?

4	Interoperation of IntServ & DiffServ Networks

It is broadly accepted that IntServ is an excellent solution for providing IP QoS in comparatively small private networks, but not suitable for use in the core of the Internet.  In contrast, DiffServ was developed to scale well to large networks.  There is no reason, however, why these two solutions should not co-operate; they can be seen as complimentary, not opposed [COMBINED].  The two approaches must be able to coexist and effectively interoperate.  This chapter will attempt to show how this can be achieved.

From the perspective of IntServ/RSVP, DiffServ regions of the network are treated as virtual links connecting IntServ/RSVP-capable routers or hosts.  Within the DiffServ regions of the network, routers implement specific PHBs (aggregate traffic control).  RSVP is used to apply admission control to each PHB in the DiffServ region.  Often IntServ/RSVP network regions are referred to as ‘customers’ of the DiffServ network regions.

4.1	Benefits of using IntServ with DiffServ

4.1.1	Explicit Admission Control

In IntServ networks, quantitative QoS applications use RSVP to request resources from the network.  The network may accept or reject these requests in response.  This is ‘explicit admission control’, which helps to assure that network resources are optimally used.  In contrast, DiffServ networks can only provide ‘implicit admission control’.

Consider a DiffServ network region providing only aggregate traffic control with no signalling.  For example, a network element at the ingress (entry) to a DiffServ network region could be provisioned to accept only 50 Kbps of traffic for the Expedited Forwarding (EF) service.  Consider now that there may be ten IP telephony sessions originating outside the DiffServ network region, each requiring 10 Kbps of EF service.  Since the ingress network element is provisioned to accept only 50 Kbps of EF traffic, it will discard half the offered traffic.  This traffic will be discarded from the traffic aggregate, with no regard to the microflow from which it originated.  As a result, it is likely that of the ten IP telephony sessions, none will obtain satisfactory service when in fact, there are sufficient resources available in the DiffServ network region to satisfy five sessions.  In the case of explicit admission control, the network will signal rejection in response to requests for resources that would exceed the 50 Kbps limit.

4.1.2	Assistance in Traffic Identification/Classification

Within DiffServ network regions, traffic is allotted service based on the DiffServ CodePoint (DSCP) marked in each packet’s IP header.  There are two mechanisms for doing so; host marking and router marking.

In the case of host marking, the host operating system marks the DSCP in transmitted packets.  Host marking requires that the host be aware of the interpretation of DSCPs by the network.  This information can be configured into each host, or IntServ/RSVP hosts can use RSVP signalling to query the network for a mapping from IntServ service type to DSCP.

In the case of router marking, Multi-Field (MF) classification criteria must be configured in the router.  This may be done dynamically, by request from the host operating system, or statically via manual configuration or via automated scripts.

However, the classification criteria may change frequently.  Users may be assigned different IP addresses by DHCP.  Applications may use transient ports.  To further complicate matters, multiple users may share an IP address.  These factors make it very difficult to manage static configuration of the classification information required to mark traffic in routers.  RSVP signalling is ideally suited for this task.

4.2	Interoperability Framework

In this general framework, hosts will use IntServ/RSVP to request reservations for specific services from the network.  The network includes some combination of IntServ/RSVP regions (in which MF classification and per-flow traffic control is applied) and DiffServ regions (in which aggregate traffic control is applied).

Two specific realisations of the framework will be considered.  In the first, resources within the DiffServ regions of the network are statically provisioned and these regions include no RSVP-aware devices.  In the second, resources within the DiffServ region of the network are dynamically provisioned and select devices within the DiffServ network regions, which participate in RSVP signalling.

4.2.1	Interoperability Reference Network

The two realisations of the framework will be discussed in the context of the reference network in Figure 4.1:

Figure 4.1: Interoperability Reference Network Configuration

From an economic viewpoint, it may be considered that the DiffServ region sells service to the IntServ/RSVP regions, which provide service to hosts.  Thus, we may think of the IntServ/RSVP regions as customers of the DiffServ region.  In the following, the term ‘customer’ for the IntServ/RSVP regions is used.

The reference network includes a DiffServ region interconnecting two IntServ/RSVP regions.  In the interest of simplicity, a single QoS sender in one of the IntServ/RSVP network regions and a single QoS receiver in the other will be considered.  Both sending and receiving hosts use IntServ/RSVP to communicate the quantitative QoS requirements of QoS-aware applications running on the host.  It is assumed that RSVP signalling messages travel end-to-end between sender and receiver to support reservations in the IntServ network regions.  It is required that these end-to-end RSVP messages are carried across the DiffServ region.

4.2.2	Service Mapping

IntServ/RSVP signalling requests specify an IntServ service type and a set of quantitative parameters known as a ‘flowspec’.  Admission control agents for DiffServ network regions must map IntServ service types to a corresponding DiffServ service level (DSCP or PHB) that can reasonably extend the IntServ service type requested by the application.  The admission control agent can then approve or reject resource requests based on the capacity available in the DiffServ network region at the mapped service level.

4.2.3	Edge Routers and Border Routers

Routers at the ingress (entry) and egress (exit) points of the network are known as Edge Routers in IntServ and Border Routers in DiffServ.  In Figure 4.1 above, ER1 and ER2 are edge routers residing in the IntServ/RSVP network regions.  BR1 and BR2 are border routers, residing in the DiffServ network region.  As will be seen in Section 4.3, there are two ways to view the functionality of edge and border routers, depending on whether the DiffServ region has any awareness of the IntServ region and its associated RSVP signalling.

4.3	Examples of IntServ/DiffServ Interoperation

Two examples will be discussed, one in which the DiffServ network region is RSVP-unaware, the other in which the DiffServ network region is RSVP-aware.

4.3.1	RSVP-Unaware DiffServ Region

In this example, no devices in the DiffServ network region are RSVP-aware.  The DiffServ network region is statically provisioned.  The owner(s) of the IntServ/RSVP network regions and the owner of the DiffServ network region have negotiated a static contract (i.e., SLA) for the transmission capacity to be provided to the customer at each of a number of standard DiffServ service levels (see Figure 4.2).

Figure 4.2: IntServ/RSVP-Unaware DiffServ Scenario

It is helpful to consider the edge routers in the customer network, to consist of two halves, a standard IntServ/RSVP half, which interfaces to the customer’s IntServ/RSVP network regions and a DiffServ half which interfaces to the DiffServ network region.  The IntServ/RSVP half has full RSVP capability.  It is able to participate in RSVP signalling and it is able to identify and process traffic on per-flow granularity.

4.3.1.1	Sequence of Events in Obtaining End-to-End QoS

The following sequence illustrates the process by which an application obtains end-to-end QoS (see Figure 4.3).

Figure 4.3: End-to-End QoS Process

- The QoS process on the sending host generates an RSVP PATH message.

- At the edge router ER1, the PATH message is subjected to standard RSVP processing The PATH message is sent onward to the DiffServ network region.

- The PATH message is carried transparently through the DiffServ network region and then processed at ER2 according to standard RSVP processing rules.

- When the PATH message reaches the receiving host, the host generates an RSVP RESV message, which is carried back towards the DiffServ network region and the sending host.

- At ER2, the RESV message is subjected to standard IntServ/RSVP processing.  It may be rejected if resources on the downstream interface of ER2 are deemed insufficient to carry the resources requested.  If it is not rejected, it will be carried transparently through the DiffServ network region, arriving at ER1.

- In ER1, the RESV message triggers admission control processing.  ER1 compares the resources requested in the IntServ/RSVP request to the resources available in the DiffServ network region at the corresponding DiffServ service level.  The availability of resources is determined by the capacity provisioned in the SLA.

- If ER1 approves the request, the RESV message is admitted and is allowed to continue upstream towards the sender.  In that case, ER1 updates its internal tables to indicate the reduced capacity available at the admitted service level on it’s transmit interface.

- The RESV message proceeds through the IntServ/RSVP network region to which the sender is attached.  If any subsequent nodes do not reject the request, the RESV message will arrive at the sending host.

- At the sender, the QoS process receives the RESV message.  It interprets receipt of the message as indication that the specified traffic flow has been admitted for the specified IntServ service type (in the IntServ/RSVP network regions).

- The sender may choose to mark the DSCP in the headers of packets that are transmitted on the admitted traffic flow.  The DSCP is the value that maps to the IntServ service type specified in the admitted RESV message.

In this manner, it is possible to obtain end-to-end QoS through a combination of networks that support IntServ/RSVP and networks that support DiffServ.

4.3.2	RSVP-Aware DiffServ Region

In this example, the customer’s edge routers are standard RSVP routers.  The border router, BR1 is RSVP-aware.  In addition, there may be other routers within the DiffServ network region which are RSVP-aware.  Note that although these routers are able to participate in RSVP signalling, they process traffic in aggregate, based on DSCP, not on the per-flow classification criteria used by standard IntServ/RSVP routers (see Figure 4.4).

Figure 4.4: IntServ/RSVP-Aware DiffServ Scenario

When the DiffServ network region is RSVP-aware, the admission control agent is part of the DiffServ network.  As a result, changes in the capacity available in the DiffServ network region can be indicated to the IntServ/RSVP network regions via traditional RSVP.  By including routers interior to the DiffServ network region in RSVP signalling, it is possible to improve the efficiency of resource usage.  This is because admission control can be linked to the availability of resources along the specific path that would be impacted.  This benefit of RSVP signalling is known as ‘topology-aware admission control’.

Various mechanisms may be used within the DiffServ region to support dynamic provisioning and topology-aware admission control.  These include aggregated or hierarchical RSVP, as discussed in Section 2.5.2, and bandwidth brokers, discussed in the Section 4.4.

4.4	Bandwidth Broker

As has been shown, it is possible to use additional mechanisms such as RSVP to dynamically allocate resources for particular DS behaviour aggregates within a DiffServ network.  However, border routers could instead use custom protocols to interact with some hypothetical agent that has sufficient topology awareness to make admission control decisions.  It has been proposed that agents called Bandwidth Brokers (BB), that can be configured with organisational policies, keep track of the current allocation of marked traffic, and interpret new requests to mark traffic in light of the policies and current allocation [TWOBIT] (see Figure 4.5).

Figure 4.5: Bandwidth Broker (BB)

Bandwidth Brokers (BB) have two responsibilities:

- Their primary one is to parcel out their region’s marked traffic allocations and set up the routers within the local domain.

- The other is to manage the messages that are sent across boundaries to adjacent regions’ BBs.

A BB is associated with a particular trust region, one per domain.  When an allocation is desired for a particular flow, a request is sent to the BB.  Requests include a service type, a target rate, a maximum burst, and the time period when service is required.  If a request passes admission and policy control at the BB, the available bandwidth along the path of the requested flow is reduced by the requested amount and the flow specification is recorded.  In this fashion the request may traverse several domains, and end-to-end service is achieved, if the BB in each domain along the way allows it to proceed.

4.5	Interoperability Limitations

The approach presented in this chapter is subject to certain limitations.  Performance and scalability might become problems in admission control.  There has been a lot of discussion on the viability of the Bandwidth Broker scheme, especially if each request needs to be processed.  Also multicast routing causes problems, because reservation merging would be required within the DiffServ region.

5	Applying QoS to Voice over IP (VoIP)

Voice over IP (VoIP) or IP Telephony is the exploding new technology enabling voice to be carried over IP-based, packet-switched local and wide area networks (WANs and LANs).  The most accepted standards for VoIP are the ITU-T Rec. H.323 [H323] and the ETSI TIPHON standards.  Also, the IETF has produced a “competing” set of standards, which may gain momentum in the future, as they are originally planned with the Internet in mind [MMusic].  (In this chapter, only H.323 is covered, but the same issues apply to MMusic’s Session Initiation Protocol (SIP).)

In the brief history of IP telephony, voice connections over the Internet have got a bad reputation because of poor quality.  Due to its real-time nature, an effective voice conversation requires a reasonable level of continuity, but large numbers of packets (representing other types of data) can impact negatively on continuity competing with voice packets for network bandwidth.  The voice quality issue is made more complex because of its subjective nature.  Equipment responsible for processing voice for transport over an IP network must be able to retain all the nuance, inflection and pauses that comprise effective human communication.

A shortcoming of H.323 is that it does not support QoS negotiation capability.  One example to solve this issue is to use RSVP-capable routers in combination with a Policy Server (PS) that can provide different types of services to successfully transport real-time traffics, such as audio.  Another example would be to use DiffServ in combination with a Policy Server.  Also, a combination of RSVP, DiffServ and Policy Server is possible.

5.1	Why is a Policy Server (PS) Necessary?

The Policy Server (PS) with the policy directory is necessary, because service providers must be able to monitor, control, and enforce use of the network resources according to predefined policies that are usually based on the identity and status of users, security considerations, time of day etc.

The IETF RAP (RSVP Admission Policy) working group have proposed the Common Open Policy Service (COPS) protocol to communicate between PS and routers.  Their architecture comprises of two main types of components; Policy Enforcement Point (PEP) and Policy Decision Point (PDP).  PEP is a component at a network node such as router or switch and PDP is a remote entity that may reside at a PS.

The COPS protocol can be used to exchange the policy control information between the PDP and the PEP.  It is a simple query and response protocol based on a client/server model.

Figure 5.1: Communication between the PS and H.323 Gatekeeper

As shown in Figure 5.1, the H.323 Gatekeeper will initiate a QoS request to the PS, which is responsible for setting up the QoS and communicates it back to the H.323 Gatekeeper.  The protocol between the H.323 GK and the PS should provide the capability of handling the temporal information for QoS reservation.

5.2	H.323 with RSVP-Capable Routers and a PS

Figure 5.2 shows a possible H.323 call set-up to support QoS for VoIP.  It contains two H.323 terminals, one H.323 gatekeeper, RSVP and COPS capable routers, and a Policy Server (PS) with a policy directory.

Figure 5.2: H.323 Call Set-up with RSVP and COPS

Terminal A (caller) wants to call Terminal B (callee):

Message flow:

1: Admission request via call set up.

2: Admission Req. from Gatekeeper to PS.

3: PS to check the policy.

4: Policy response.

5: PS grants/rejects the call req.

6: Gatekeeper informs the grant/reject.

7, 8, 9, 10: RSVP Path message.

11: RSVP Resv message.

12, 14, 16: RSVP Req./Response between PS and Router C.  PS checks the directory to make a decision.

13, 15, 17: RSVP Resv. Message.

Terminal A is now ready to transmit.

Terminal B initiates the RSVP process in parallel.

5.3	H.323 with DiffServ-Capable Routers and a PS

The key to provide QoS is the ability to differentiate traffic and to provide differentiated service levels based on the types of traffic.  For example, for mission-critical and real-time applications such as VoIP, the amount of available bandwidth and end-to-end delay is crucial compared to fax and email transmissions, which are quite intensive to bandwidth and delay issues.

The DiffServ architecture is a framework within which service providers can simultaneously offer various levels of performance.  TIPHON has proposed several QoS classes, terminal types and each pair of endnotes to a performance budget in terms of delay (including jitter) packet loss and bandwidth.  Each router, according to its capabilities, will be assigned three delay limits; one for each class of delay.

Whenever an admission request is issued, the gatekeeper checks the capabilities and preferences of the endpoints against the network momentary condition.  The request is confirmed or rejected and the network condition is updated (see Figure 5.3).

Figure 5.3: H.323 and DiffServ

In cases where the access point is a gateway (owned by the service provider), which has the ability to mark DS bytes, the gateway may perform the role of a conditioner.  The gatekeepers (possibly through some PS) inform traffic conditioners near the ingress points how to mark the packets of the call.  When the call is terminated the gatekeepers update the network condition and the billing is carried out according to the QoS class.

5.4	A Possible Global IP Telephony Network

Figure 5.4 shows a complete picture of how some these QoS technologies can work together to provide “end-to-end QoS” using different domains.  DiffServ mechanisms can be used in the core network while end applications will use RSVP to request QoS from the network.  This is an excellent example of using RSVP with the DiffServ approach.

Figure 5.4: A Global IP Telephony Network

End hosts may use RSVP requests with high granularity (e.g., bandwidth, jitter threshold, etc.).  Border routers at backbone ingress points can then map those RSVP “reservations” to a class of service indicated by a DS-byte.  At the backbone egress point, the RSVP provisioning may be checked again, to the final destination.  Ingress points essentially do traffic conditioning on a customer basis to assure that SLAs are satisfied.  As shown in Figure 5.4, COPS may be also be used between Bandwidth Brokers (BBs), which essentially act as PDPs (for dynamic inter-domain policy pricing).

5.5	The Future of VoIP

IP telephony standards, such as H.323, are evolving rapidly.  Most do not support full Signalling System 7 (SS7) capabilities and therefore cannot interoperate with the PSTN.  But progress is being made and a spirit of co-operation exists within the IP telephony community.  As new IP signalling protocols, such as the Resource Reservation Protocol (RSVP) and new prioritisation techniques like DiffServ, are standardised and implemented in network routers, the quality of VoIP will become equivalent to that of the PSTN.  Until then, VoIP technology will be used primarily by large organisations and public carriers over dedicated networks (rather than over the public Internet) where QoS can be monitored and controlled.

The convergence of voice and data networks means that the PSTN users will benefit from rapid innovations in IP technology over the next number of years.  New affordable services, such as video-telephony, promise to herald in a new era of innovative, multimedia communications.  The circuit-switched public telephone network of today will evolve to become the packet-switched public data network of tomorrow, carrying voice, video, and data.

6	IETF MultiProtocol Label Switching (MPLS)

MPLS is another method to implement QoS.  Although it is a mixture of layer 3 routing and layer 2 switching technologies, it is dealt here for the sake of completeness, but otherwise the investigation of MPLS is outside of the scope of the project.  This chapter gives an overview of the MultiProtocol Label Switching architecture, its QoS model, and the MPLS working group in the IETF.

6.1	Introduction to MPLS

Ipsilon’s IP Switching system was the first representative of a new technology which increases the performance of IP routers by combining layer 2 switching and layer 3 routing.  Extended layer 2 switches are able to process layer 3 packet information.  The hardware architecture of these layer 2 switches allows very fast data processing.

Other companies came up with similar, but mostly incompatible label switching concepts.  In order to reach compatibility between products from different vendor, the MultiProtocol Label Switching (MPLS) working group was established at the IETF (see Section 6.6).

6.2	Overview of Label Switching

The term IP Switching was originated by Ipsilon to refer to the company’s particular scheme for IP label switching.  It is now used more generally, to refer collectively to all similar approaches for integrating layer 3 forwarding with layer 2 switching.  Other names for label switching are layer 3 switching or multi-layer routing.  A number of different schemes utilising the concept of label switching have been proposed by major equipment manufacturers, such as Tag Switching from Cisco, Cell Switch Routing from Toshiba, etc.

Schemes implemented by Ipsilon and Toshiba are flow-oriented.  The basic idea is that each IP switch/integrated switch-router/cell switch router consists of two components: a layer 2 switch and an IP router sometimes also called IP switch controller.  Initially the IP hosts are connected via default connections and data is forwarded through the IP controller.  The default IP streams are analysed permanently in order to identify IP flows.  Once a flow has been identified, the switch controller establishes a direct connection between two neighbours.  After establishing a direct connection, all packets of the same flow are switched directly through the node.

Cisco’s Tag Switching is topology-oriented.  In this architecture, the flow identification is based on layer 3 routing information and can aggregate application level flows into network route flows.  Thereby, they can utilise VCs more efficiently than can the original IP Switching.

6.3	Overview of MPLS

The different approaches discussed in Section 6.2 apply layer 2 switching technology (based on label swapping) to layer 3 packets.  The IETF has initiated a working group called MPLS (Multi-protocol Label Switching) with the goal of defining a standard IP-switching architecture.  The goal of this group is to develop a standard for integration of layer 2 switching with layer 3 routing in order to improve price/performance of network layer routing, scalability of network layer and provide greater flexibility in providing new routing services.

Labels are associated with specific streams of data.  Forwarding is simplified by the use of short fixed length labels to identify streams.  Streams are forwarded as a unit and ensure that an identified stream takes a single path, where a stream may consist of the aggregate of multiple flows of user data.  No assumption is made about layer 2 protocols, except that they are capable of forwarding network layer packets.  MPLS enhances the scalability of ATM networks by the so-called VC merge.  This concept is similar to a multi-point-to-point VC.

MPLS can also be used for implementing an entirely new class of VPNs.  MPLS-based IP-VPNs are connectionless IP networks, consisting of disjoint (separate) meshes of label switched routes (LSR) for the different VPNs.  They can offer the same privacy as a Frame-Relay or ATM VPN, but are more cost effective for the Service Providers.

Results from this MPLS working group will have to be integrated into the existing IP-switching prototypes in order to achieve interoperability between different solutions.  The standardisation process is not yet finished.

6.4	MPLS QoS Architecture

The MPLS technology promises to ease the realisation of the IETF Differentiated Services Model.  IP CoS can be provided over an ATM network using ATM Forum QoS, but configuring the many virtual circuits (VC) and allocating bandwidth on a VC basis makes this a difficult network design.  With MPLS, this may be simpler, because direct support of CoS is given, making it simple to configure the bandwidth allocation on a class-by-class basis for end-to-end IP QoS in a routed or ATM environment.

Label headers used within MPLS are simpler than IP headers.  The label contains a 3-bit CoS field.  There are still ongoing discussions about the use of the CoS field.  There are several possibilities to set the CoS field:

- CoS value = IP ToS

- function of the packet’s input interface

- function of the flow type

The MPLS drafts say that a compatibility with the IETF Integrated Services Model, including RSVP should be given.  For realising RSVP over an MPLS system, each RSVP flow must be associated with a certain label.

There are differences between flow-oriented and topology-oriented systems.  With flow-oriented systems like IP Switching, since the architecture is built around individual flows, it is, in principle, possible to provide per-flow QoS with IP switching.  Topology-based systems, like Tag Switching, make per-application flow QoS control difficult.  Aggregation of streams is not an advantage when one is trying to implement per (application) flow QoS control.  Nevertheless, some QoS services can be offered by assigning different IP-VPNs to different types of flow.

6.5	Evaluation of MPLS

The availability of MPLS systems on the market is low.  Only Cisco’s Tag Switching claims to be (pre)standard MPLS conforming.  (An overview of Cisco’s Tag switching is given in [RFC2105].) Some vendor specific solutions like Cell Switch Routing from Toshiba exist.

Actually, there are no systems, which support RSVP so far, but IP CoS will become available on MPLS systems soon.  Even if vendors praise the higher performance and the better QoS support of their MPLS system, there is not much experience running such networks.  A big advantage of MPLS is that is not limited to ATM.

6.6	IETF MPLS Working Group

The description of the working group initiated by IETF and called MPLS (Multi-protocol Label Switching) can be found at:

www.ietf.org/html.charters/mpls-charter.html

Several vendors have proposed techniques based on the paradigm of label-swapping based forwarding.  Currently, none of these solutions that employ label-swapping-based forwarding (“label switching”) in conjunction with network layer routing are based on standard technology.

The working group is responsible for standardising such a base technology and standards for its operation over various link level technologies, which may include Packet-over-Sonet, Frame Relay, ATM, Gigabit Ethernet, Token Ring, etc.

This includes procedures and protocols for the distribution of labels between routers, encapsulations, multicast considerations, use of labels to support higher layer resource reservation and QoS mechanisms, and definition of host behaviours.

High level requirements on the standards include:

- Independence from data link technologies.  (Specific optimisations for particular media will be considered.)

- Compatibility with existing internetwork technologies and routing protocols.

- Independent operation from the routing protocol.

- Support for a wide range of forwarding granularities associated with a given label, from a single application flow to a group of topologically related destinations.

- Support for operations, administration, and maintenance

- Prevention and/or containment of routing loops

- Operation in a hierarchical network.

- Scalability issues, (e.g., In the case of ATM, conservation of VC usage)

6.6.1	Working Items of MPLS Working Group

- General architecture and framework document

- Standard protocol(s) for maintenance and distribution of label binding information for label-swapping based forwarding supporting:

- Unicast destination-based routing

- Multicast routing

- Hierarchy of routing knowledge, (e.g., Complete segregation of intra and inter-domain routing)

- Explicit paths

- Standard procedures of carrying label information over various link layer technologies.

- Standard way for operation over ATM user plane

- Allow operation/co-existence with standard (ATM Forum, ITU, etc.) ATM control plane and/or standard ATM hardware

- Specify a `label swapping’ control plane

- Take advantage of possible modifications/improvements in ATM hardware, for example the ability to merge VCs

- Support for QoS, (e.g., RSVP).

- Standard protocol(s) to allow direct host (e.g., server) participation and procedures for Host Behaviour

The working group have produced a considerable number of Internet drafts, but at this time, no RFC has been produced.

References

| [AFmark1] | J. Heinänen, R. Guerin, “A Single Rate Three Color Marker”, Internet-Draft, <draft-heinanen-diffserv-srtcm-01.txt>, May 1999 |
| --- | --- |
| [AFmark2] | J. Heinänen, R. Guerin, “A Two Rate Three Color Marker”, Internet-Draft <draft-heinanen-diffserv-trtcm-01.txt>, May 1999 |
| [COMBINED] | Y. Bernet et al, “Interoperation of RSVP/IntServ and DiffServ networks”, Internet Draft < draft-ietf-issll-diffserv-rsvp-02.txt>, June 1999 |
| [DiffServ] | John Wroclawski; “Differential Service for the Internet”, http://diffserv.lcs.mit.edu/ |
| [H323] | ITU-T Recommendation H.323, (Feb. 98), Packet-based multimedia communications systems |
| [ID-DiffServ-RSVP] | Y. Bernet (ed.) et al, “Interoperation of RSVP/IntServ and DiffServ Networks”, Internet Draft, March 1999 |
| [IEEEP802.1p] | “Supplement to MAC Bridges: Traffic Class Expediting and Dynamic Multicast Filtering,” IEEE P802.1p/D6, May 1997 |
| [IPv6-Draft] | S. Deering, R. Hinden; “Internet Protocol, Version 6 (IPv6) Specification”, Internet-Draft <draft-ietf-ipngwg-ipv6-spec-v2-02.txt>, August 1998 |
| [ISO10037] | “MAC Bridges,” ISO/IEC 10038, ANSI/IEEE Std. 802.1D, 1993 |
| [MMusic] | IETF Multiparty Multimedia Session Control working group, www.ietf.org/html.charters/mmusic-charter.html |
| [PAR94] | C. Partridge, “Gigabit Networking”, Addison Wesley, 1994 |
| [P803-1] | EURESCOM Project P803-PF, European IP Testbed (definition phase), Deliverable 1, Proposal for Setting-up the Testbed, May 1998 |
| [Q.2931] | ITU-T Recommendation Q.2931, (Feb. 95), Broadband Integrated Services Digital Network (B-ISDN) – Digital Subscriber Signalling System No. 2 (DSS 2) - User-Network Interface (UNI) - Layer 3 specification for basic call/connection control |
| [RFC791] | “Internet Protocol – DARPA Internet Program Protocol Specification”, Jon Postel (ed.), September 1981 |
| [RFC1812] | “Requirements for IP Version 4 Routers”, F. Baker (ed.), June 1995. |
| [RFC2105] | “Cisco Systems’ Tag Switching Architecture Overview”, Y. Rekhter, B. Davie, D. Katz, E. Rosen, G. Swallow, February 1997 |
| [RFC2205] | “Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification”, R. Braden (ed.), L. Zhang, S. Berson, S. Herzog, S. Jamin, September 1997 |
| [RFC2210] | “The Use of RSVP with IETF Integrated Services”, J. Wroclawski, September 1997 |
| [RFC2211] | “Specification of the Controlled-Load Network Element Service”, J. Wroclawski, September 1997 |
| [RFC2212] | “Specification of Guaranteed Quality of Service”, S. Shenker, C. Partridge, R. Guerin, September 1997 |
| [RFC2474] | “Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers”, K. Nichols, S. Blake, F. Baker, D. Black, December 1998 |
| [RFC2475] | “An Architecture for Differentiated Services”, S. Blake, D. Black, M. Carlson, E. Davies, Z. Wang, W. Weiss, December 1998 |
| [RFC2597] | “Assured Forwarding PHB Group”, J. Heinanen, F. Baker, W. Weiss, J. Wroclawski, June 1999 |
| [RFC2598] | “An Expedited Forwarding PHB”, V. Jacobsen, K. Nichols, K. Poduri, June 1999 |
| [SBM] | R. Yavatkar, D. Hoffman, Y. Bernet, F. Baker, M. Speer, “SBM (Subnet Bandwidth Manager): A Protocol for RSVP-based Admission Control over IEEE 802-style networks”, Internet-Draft, November 1998 |
| [SCHW] | U. Schwantag, “An Analysis of the Applicability of RSVP”, Diploma thesis at the University of Oregon, 1997. http://network-services.uoregon.edu/~ursula |
| [TWOBIT] | K. Nichols, V. Jacobson, L. Zhang, “A Two-bit Differentiated Services Architecture for the Internet”, Internet Draft, December 1997. |
| [WFQ] | A. Demers, S. Keshav, S. Shenker, “Analysis and simulation of a fair queuing algorithm”, Proceedings of ACM SIGCOMM, pp. 1-12, Austin, Texas, September 1989. |