Project P803-PF

European IP Testbed (Implementation Phase)

Deliverable 4

Supporting Differentiated Quality of Service in IP Networks

Volume 2 of 3: Tools, Measurements and Evaluations

Suggested readers:

- Managers, IP network designers/planners

- People interested in Quality of Service in the Internet

For full publication

August 1999

EURESCOM PARTICIPANTS in Project P803-PF are:

- Deutsche Telekom AG

- Eircom

- FINNET Group

- France Télécom

- MATÁV Hungarian Telecommunications Company Ltd.

- Tele Danmark A/S

- TELECOM ITALIA S.p.a.

- Telekom Austria AG

This document contains material which is the copyright of certain EURESCOM PARTICIPANTS, and may not be reproduced or copied without permission.

All PARTICIPANTS have agreed to full publication of this document.

The commercial use of any information contained in this document may require a license from the proprietor of that information.

Neither the PARTICIPANTS nor EURESCOM warrant that the information contained in the report is capable of use, or that use of the information is free from risk, and accept no liability for loss or damage suffered by any person using this information.

This document has been approved by EURESCOM Board of Governors for distribution to all EURESCOM Shareholders.

Preface

(Prepared by the EURESCOM Permanent Staff)

The Implementation Phase of the P803 Project initially had the ambition to construct an international testbed, consisting of broadband connections between the different participating shareholders.  The structure and the use of the proposed testbed were to be clarified by the Project [P803-1].

However, as the Project progressed:

- A lot of time was needed for the development of test tools and test methodologies in advance of actually performing the tests.  This was largely done in the individual shareholder labs

- Many of the tests were interoperability-related rather than performance-related because a European-wide high-performance broadband network was not available to the Project

- Access to cheap connectivity throughout Europe is not generally available now that JAMES no longer exists.  Instead, connectivity needs to be bought at commercial prices

The result is that no European-wide testbed currently exists, but the Project has still managed to produce relevant results.  For the success of future work in this area, broadband connectivity between the participants is absolutely necessary and industry involvement would be desirable (as has been the case in some other EURESCOM Projects).

Eight Shareholders took part in the Implementation Phase of P803.  The Project Leader was Paul Duggan, Eircom.

One of the Tasks in the P803 Project focused on support for differentiated services in IP networks.  This Deliverable 4 is the results of this Task.  (These tests focussed mainly on IPv4 because Deliverable 3 addressed IPv6 testing)

Within the deliverables, the reader may encounter the following EURESCOM Shareholder codes:

- AF		FINNET Group  (Finland)

- AU	Telekom Austria AG  (Austria)

- DK	Tele Danmark A/S  (Denmark)

- DT	Deutsche Telekom AG  (Germany)

- FT		France Télécom  (France)

- HT	MATÁV Hungarian Telecommunications Company Ltd.  (Hungary)

- IT		TELECOM ITALIA S.p.a.  (Italy)

- TI		Eircom  (Ireland)

Further project information can be obtained at the following website:

www.eurescom.de/Public/Projects/P800-series/P803/P803.htm

Executive Summary

Both standardisation bodies and router vendors are pushing hard for Quality of Service (QoS) solutions for the Internet.  Many vendors already advertise either standard-compliant or proprietary products equipped with IP “QoS”.  However, there is very little operational experience with such products and performance, management and network planning issues are not yet well understood.  Also, interoperability between products has not been demonstrated in many occasions.

This has made most of the Network Operators and Internet Service Providers reluctant to upgrade their networks with QoS features.  The QoS standards, especially DiffServ, are ambiguous, leaving much of the responsibility for the operators.  On the other hand, working solutions already exist, and both customers and providers could really benefit from them.

In this situation, practical and objective information of the capabilities of different router models has become vital and this information can only be gathered with proper testing tools.

The work done in EURESCOM Project P803 provides insight into the area.  During this Project, two excellent tools for RSVP/IntServ measurements were developed.

- RSVPTester is a Windows software built upon WinSock2 programming interface with the ability to generate RSVP reservations with user defined parameters.  It additionally generates traffic according to several source models including a voice conversation.

- RSVPdump is a tcpdump-based UNIX tool for debugging RSVP messages.

Besides these tools, there is also discussion about the ISDN test network built in P803.  The network was put into operation successfully, but the actual tests were limited to pure signalling interoperability.  Performance tests over wide area networks turned out to be difficult but are indeed an important area of future research.

Many UNIX systems already have a rich set of QoS tools freely available.  One of them is ALTQ, which is an implementation of Class Based Queuing, a scheduling mechanism for enabling DiffServ.  ALTQ for FreeBSD was used to build a DiffServ router.  Some performance tests were done to evaluate the viability of such a scheme.  Also, there was a study on RSVP-to-ATM mapping using UNIX hosts with ATM interfaces and Fore’s ATM switch to interconnect them.

Tests were run with RSVPTester, Windows 2000 and Cisco’s 2500 Series routers.  Initial tests concentrated on interoperability, followed by performance issues, namely TCP throughput and latency.  Voice over IP (VoIP) tests were done in different background load conditions in order to verify the correct operation of the router’s IntServ implementation.  It was noticed that while in some cases the routers performed as expected, it was easy to find conditions where IntServ simply did not guarantee the service required in terms of packet delay distribution.

The conclusion of the performance tests was that there are still some problems with the QoS implementations, especially with older hardware platforms.  On the other hand the basic mechanisms worked very well.

List of Authors

| Olaf Bonness | Deutsche Telekom AG |
| --- | --- |
| Dave O’Meara, | Eircom |
| Michael Barry, | Eircom |
| Jussi Tuominen | FINNET Group |
| Markus Isomäki, | FINNET Group |
| Mika Pyyhtiä | FINNET Group |
| Grégoire Sénéclauze, | France Télécom |
| Laurent Rabret, | France Télécom |
| Ferenc Telbisz, | MATÁV Hungarian Telecommunications Company |
| Jan Anderson, | Tele Danmark A/S |
| Per Randrup Nielsen | Tele Danmark A/S |

Table of Contents

Preface	i

Executive Summary	iii

List of Authors	iv

Table of Contents	v

Abbreviations	vi

1	Introduction	1

1.1	Structure of this Deliverable	1

2	Tools for Testing and Measuring IP QoS	3

2.1	RSVPTester for Microsoft Windows	3

2.1.1	Introduction to RSVPTester	3

2.1.2	Architecture of RSVPTester	4

2.1.3	Flow Description	5

2.2	RSVP Protocol Analyser for UNIX	7

2.2.1	Analysing RSVP Packets	8

2.2.2	RSVPDUMP	9

2.2.3	Results of RSVP Protocol Analyser	9

2.3	The P803 ISDN Test Network	10

2.3.1	Proposed Interoperability Tests	10

3	IP QoS Implementations	13

3.1	Differentiated Services on FreeBSD	13

3.1.1	FreeBSD DiffServ Implementation	13

3.1.2	ALTQ config File	15

3.1.3	Cisco Implementation	16

3.2	RSVP over ATM	18

3.2.1	RSVP over ATM Test Network	18

3.2.2	Conclusions	20

3.3	Signalling and Interoperability	21

3.3.1	Microsoft Windows and Cisco IOS Interoperability	21

3.4	IntServ Performance Tests	22

3.4.1	TCP Throughput with Controlled Load (CL) Service	22

3.4.2	TCP and UDP Performance over Controlled Load Reservations	24

3.4.3	UDP Performance over Controlled Load Service	25

4	DiffServ Performance Tests	29

4.1	DiffServ Testing Scenario	29

4.2	Conclusions of DiffServ Performance Tests	31

4.3	Future Work in DiffServ Performance Testing	32

5	Applying QoS to Voice over IP (VoIP)	33

References	37

Abbreviations

| AF | Assured Forwarding |
| --- | --- |
| ATM | Asynchronous Transfer Mode |
| BE | Best-Effort |
| CBQ | Class-Based Queuing |
| CL | Controlled Load |
| DiffServ | Differentiated Services |
| DS | Differentiated Services |
| EF | Expedited Forwarding |
| FIFO | First In, First Out |
| FQ | Fair Queuing |
| GS | Guaranteed Service |
| HTTP | Hyper-Text Transfer Protocol |
| ICMP | Internet Control Message Protocol |
| IETF | Internet Engineering Task Force |
| IntServ | Integrated Services |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| ISP | Internet Service Provider |
| MTU | Maximum Transmission Unit |
| PHB | Per-Hop Behaviour |
| PVC | Permanent Virtual Circuit |
| QoS | Quality of Service |
| RED | Random Early Discard |
| RFC | Request for Comments (IETF standards) |
| RIO | Random early discard with In/Out profiles |
| RSVP | Resource reSerVation Protocol |
| SVC | Switched Virtual Circuit |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |
| UNI | User-Network Interface |
| VC | Virtual Circuit |
| VoIP | Voice over IP |
| WFQ | Weighted Fair Queuing |

1	Introduction

The standardisation for Quality of Service (QoS) in IP networks is advancing rapidly in various working groups within the Internet Engineering Task Force (IETF).  In some cases, he different approaches can be seen as orthogonal components, whereas in others, here is a direct competition between them.  The overall architecture is still vague and depends on your point of view.

At the same time, a variety of products (routers, switches, operating systems, applications) claiming to support QoS support have emerged in the market.  In most cases, today’s products are built upon a combination of open standards and proprietary technology.  With many of these products it is already possible to build an IP network offering different levels of service.  However, before a Network Operator or an Internet Service Provider (ISP) can do so, the performance issues must be known.  In many cases the QoS support in a router has been implemented simply by a software upgrade to a legacy platform without any notes on the performance implications.  Also the reliability of new code is unknown, as always.  Newer platforms usually have more processing power, but the quantifiable amount is seldom known accurately.

This means that a lot of practical tests concerning the implementations need to be done before launching a commercial service.  The test results can be compared with the IETF RFC specifications or application-specific (voice etc.) target values.  In addition, a new set of tools is needed to monitor and measure the QoS-enabled IP network.  These include measurement tools, protocol monitors and signalling and traffic generators.  This document describes such efforts within this Project.

1.1	Structure of this Deliverable

Chapter 2 describes two useful RSVP testing tools developed within the Project.  The first one is a RSVP signalling and TCP/UDP traffic generator (RSVPTester), which enables the testing for both RSVP signalling interoperability and traffic performance for reserved flows.  The other one is a protocol analyser built on top of tcpdump, capable of interpreting RSVP messages.  Additionally, the chapter discusses the European-wide ISDN test network which was set-up for interoperability testing.

Chapter 3 provides details for two QoS implementations; a queuing system for FreeBSD UNIX (used to implement DiffServ scheduling) and an RSVP-to-ATM-mapping using UNIX and Fore’s ATM switch.  This chapter also describes basic tests carried out using Cisco’s RSVP-enabled routers and RSVPTester on Windows 2000.  There are both signalling interoperability and TCP/UDP performance tests.

Chapter 4 provides some results of DiffServ performance tests done using the FreeBSD queuing scheme.

Chapter 5 describes the tests done with RSVPTester’s voice source emulation tool.  Voice was run as best effort and with RSVP reservations in parallel with different background loads.  The measurements were done using Cisco’s routers and the focused in packet loss and end-to-end delay.

2	Tools for Testing and Measuring IP QoS

2.1	RSVPTester for Microsoft Windows

In order to test QoS features, some applications capable of generating RSVP signalling and loading a network were needed.  At that time, the only application that was QoS aware was NetMeeting™ for Windows.  This application provided very little flexibility and was not suited for testing (multiple reservations, generator, measurements etc.).  Therefore it was decided to develop a specific program for carrying tests.  This program was named ‘RSVPTester’, and it is described in the following sections.

2.1.1	Introduction to RSVPTester

RSVPTester is a piece of software especially realised for EURESCOM Project P803.  Its goal is to provide a generic, easy to use and powerful tool, allowing transaction achievement under the control of the IntServ mechanism (RSVP).  It relies on a standard set of stacks supplied with the latest Microsoft Operating Systems; Windows 98 and Windows 2000.  Therefore, it only works on these two platforms.

Figure 2.1: RSVPTester in the Windows Environment

RSVPTester acts both as a receiver (R) and a traffic generator or sender (S), which is able to handle Unicast TCP and UDP streams.  Relevant information from the R or S side are displayed in a window with respectively a white (R window) or cyan (S window) background colour (see Figure 2.2).

Figure 2.2: View of one R window and two S windows in the background.

2.1.2	Architecture of RSVPTester

In order to be as simple as possible and to avoid confusion it has been decided to configure connections from the R side only.  One R window displays information emanating from a set of streams (‘1’ to ‘n’) all created by a single configuration dialog box.  Therefore, there can be multiple R windows each managing a set of streams (see Figure 2.3).

Figure 2.3: Configuration Dialog Box

The S windows will display information about data transmission and RSVP signalling.  There will be two S windows: one for each transport protocol (TCP and UDP).  Figure 2.4 illustrates this description:

Figure 2.4: Windows Architecture for RSVPTester Application

2.1.3	Flow Description

2.1.3.1	How Configuration Flows are Managed

As seen before, configuration parameters are handled on the R side.  Taking this into account, it is compulsory to echo each new connection request or parameter alteration on the S side.  This is a simple manipulation considering TCP since this protocol guarantees a reliable data transmission mode.  Therefore, in that case the first created connection of a set of streams will be used to forward this information (see Figure 2.5).

Figure 2.5: ‘n’ TCP Connections Managed by Same Configuration Panel

Obviously, this does not work for UDP due to its unreliable data transmission mode.  When testing UDP flows, a TCP connection, responsible for the forwarding of configuration information, has to be created.  This signalling connection is called “meta-connection” in RSVPTester (see Figure 2.6).

Figure 2.6: ‘n’ UDP Connections Managed by Same Configuration Panel

2.1.3.2	How Signalling Flows are Managed

The “PATH” message is generated after the TCP connection or UDP pseudo-connection is achieved.  Getting this message, the receiver will send back a “RESV” message taking into account the client’s expectations.  Only when receiving the “RESV” will the traffic generator be able to transmit data.

Note that the configuration panel includes a field named “Max SDU size”, the reference for the data to be forwarded by the generator.

In case several connections should be created, the receiver will wait for the “PATH” message before initialising a new connection.  This mechanism for a TCP connection is shown in Figure 2.7:

Figure 2.7: Messages Chronograph from a TCP Connection Managed by RSVP

2.1.3.3	The Scheduler

Except for the voice model, the encoded scheduler will try to forward data in an isochronous way.  Hence, time slots between two consecutive data transmissions should be constant (see Figure 2.8).

Figure 2.8: Constant Value between Two Consecutive Data Transmissions

Of course, this vision is theoretical because data transmission is not instantaneous and Windows introduces jitter.  This jitter may be source of problems if it becomes bigger than the theoretical time slot.  In order to avoid precision degradations, it has been decided to send data in a bulk way till having a long enough idle period able to absorb jitter (see Figure 2.9).

Figure 2.9: Jitter Problem and Implementation of a Solution

2.2	RSVP Protocol Analyser for UNIX

Protocol analysers are useful tools for the monitoring of network traffic and for the analysis of individual protocols.  Typical uses for analysers are to “analyse and quantify all network traffic between host A and host B” or to “intercept and present all HTTP traffic between host A and host B”.  In a UNIX environment, the open source analysers snoop [SNOOP] and tcpdump [TCPDUMP] have been widely adopted.  This experiment describes the use of tcpdump for capturing and analysing RSVP packets.

The tcpdump analyser consists of two packages

- libpcap (which captures and filters packets)

- tcpdump (which analyses packets)

The typical operation of a tcpdump analyser is shown in Figure 2.10.  The libpcap package acts as a packet sniffer and extracts packet from the network.  These are then forwarded to the tcpdump package, which analyses the packets and formats them in a style suitable for the user.

Figure 2.10: Structure of tcpdump-based Analyser

Both tcpdump and libpcap are modular and extendable, i.e., it is possible to add new modules to intercept and interpret new protocols, such as RSVP.  Standard modules, included in the tcpdump distribution, provide analysis of IP, TCP, UDP, ICMP and other standard UNIX protocols.

2.2.1	Analysing RSVP Packets

RSVP is a network layer protocol and runs directly over IP or UDP.  While running over IP it has protocol ID 46 (decimal) [RFC2205].  This is used to distinguish IP datagrams carrying RSVP traffic from others.  The structure of an IP datagram carrying RSVP information is shown in Figure 2.11.

Figure 2.11: IP Datagram with RSVP Message

Each RSVP message consists of a header and a set of RSVP objects.  The header contains standard information including the message type, the length of the message and the protocol version.  The number and sequence of RSVP objects varies from message to message.  These are used to describe the status of an RSVP connection between two nodes, e.g., RSVP session information, Token Bucket parameters, error information etc.  A list of messages and objects is given below in Figure 2.12.  (A detailed description of each message and RSVP object can be found in [RFC2205].)

Figure 2.12: RSVP Message and Objects

2.2.2	RSVPDUMP

Implementing the RSVP protocol analyser consisted of modifying the existing tcpdump to parse and dump RSVP packets.  This required creating and modifying a number of files:

- print_rsvp.c, rsvp.h were created to interpret and print RSVP messages and objects.

- print_ip.c was modified to recognise RSVP as a valid protocol, with protocol ID 46, and to forward any messages to the print_rsvp module

2.2.3	Results of RSVP Protocol Analyser

The RSVP protocol analyser has been implemented on a PC with the FreeBSD-based UNIX platform.  It has also been tested on a Sun workstation using the Solaris OS.  Standard tcpdump has a number of verbosity levels, each of which displays a different level of detail of the protocol being analysed.  Currently, the RSVP analyser displays messages at the highest level of detail.  A sample output generated by the analyser for an RSVP path message is given below in Figure 2.13.

Figure 2.13: Output of RSVP Analyser

2.3	The P803 ISDN Test Network

Establishment of connectivity between the partners has been a part of the P803 Project.  As no European research network was available during the Project lifetime and since it turned out that no broadband connectivity could be established within the scope of the Project, it was decided to establish narrow band connections between partners using the public ISDN network.  It was felt that an ISDN solution would be a reasonable start for the test network, and since EURESCOM had purchased an ISDN router, the EURESCOM service platform, it would be possible to run IP over ISDN via Heidelberg.  However due to lack of EURESCOM support resources for configuration and maintenance of the router, a similar solution using an ASCEND ISDN router situated at the DT site in Berlin was configured (see Figure 2.14).

Figure 2.14: P803 ISDN Test Network

All partners configured and connected an ISDN router to their local test network.  Connections were established by autodial from the respective local sites to the ASCEND router and IP packets were routed to the partner site.

2.3.1	Proposed Interoperability Tests

The ISDN connectivity is not the most ideal platform for performing QoS tests.  Due to the low bandwidth, the ISDN connection is easy to overload where the LANs and especially the routers themselves all have much bigger capacity.  So the Project decided to perform QoS tests concerning bandwidth reservation and queuing mechanisms on local testbeds and then share the results.  Nevertheless some early tests concerning the exchange of RSVP signalling have been carried out.

Figure 2.15 shows two local sites connected via the ISDN network.  At both sites the RSVPTest tool is installed on Win98 or Win2000 hosts.  The local network topology includes an RSVP-capable router and RSVP connections are established between the two hosts.  The purpose is mainly to show signalling compatibility between various RSVP router implementations.  A structured test between some participants had been planned but only a few tests were carried out due to lack of resources among the partners.  However, RSVP reservations were successfully made in some routers at the opposite sides of the ISDN network.

Figure 2.15: RSVP Interoperability Test

Various RSVP implementations are already available for research platforms and from commercial router vendors as well, so results from further interoperability tests would be interesting.  Such tests are left as an open matter for future projects.

3	IP QoS Implementations

3.1	Differentiated Services on FreeBSD

In this section, an attempt is made to detail the configuration used to construct a DiffServ network.  It was decided to implement the six PHB’s being planned at the moment, namely Expedited Forwarding (EF), 4 levels of Assured Forwarding (AF) and regular Best-Effort.

In any real-world scenario, DiffServ will be expected to work across many different network platforms.  In this example, two platforms were chosen; one because it is universal, the other because it is inexpensive.  It was decided to use PCs running UNIX as the network edge routers, and a Cisco as the core router (see Figure 3.1).

Figure 3.1: Network Topology

The Edge Routers are Dell 400MHz PCs with 128MB RAM running FreeBSD 2.2.8 and ALTQ 1.1.3.  The Core Router is a Cisco 7200 running IOS 12.0.  (As shall be seen, it appears that the earlier release of IOS, 11.11c, could be more suitable.)

This topology can represent the following scenario:

- Host A is a subscriber to ISP 1, represented by Edge Router 1.

- Host B is a subscriber to ISP 2, represented by Edge Router 2.

- The Cisco Core Router represents any DiffServ-capable backbone network.

3.1.1	FreeBSD DiffServ Implementation

The edge routers are using an implementation of DiffServ based on FreeBSD running on PCs configured as IP routers.  Generally, the IP research community provides releases of experimental software much more quickly then the typical hardware router vendors.  FreeBSD and Linux are two good free-release UNIX operating systems.

(See www.freebsd.org/handbook/install.html)

ALTQ is a Class-Based Queuing [CBQ] scheme, which allows the standard UNIX FIFO (First In, First Out) queuing scheme to be replaced.

(See www.csl.sony.co.jp/person/kjc/kjc/software.html)

The following scheme is configured on the Edge Routers, (as shown in Figure 3.2):

Figure 3.2: CBQ Implementation of DiffServ

The bandwidth of the link is divided into several classes.  The most obvious split is between AF and EF traffic.  The definition of EF from the IETF draft is that as long as the arrival rate of the EF traffic is less than the service rate of the EF traffic, the EF packets should see very little, if any, delay through the router.  So the bandwidth allocated to EF is known and fixed.  For example, if 2 Mbps of EF traffic will be arriving at the router, EF should be configured to be slightly more than 2 Mbps.  This amount of bandwidth is fixed and reserved, and no other traffic can use this bandwidth.  The remainder of the bandwidth is allocated to AF and to BE (Best-Effort) traffic.

At this point, the concept of ‘borrowing’ bandwidth must be explained.  In Figure 3.2, it can be seen that each class (except the Root class) has a ‘parent’ class.  A parent class can have one or more ‘child’ classes.  The class structure must be constructed so that each parent has at least as much bandwidth as the sum of its child classes.

However, as the traffic in the under-limit classes increases (in this case, as the traffic in each class approaches 1 Mbps), the 1 Mbps restrictions on each class are enforced.  As soon as all of the classes are in congestion (i.e., running at just above 1 Mbps or more), the CBQ mechanism will restrict the bandwidth in each class to the configured rate.  Obviously some packets must be dropped.  ALTQ provides two drop schemes, RED (Random Early Discard) [RED] and RIO (RED with In/Out profiles) [RIO].  RIO runs two RED algorithms at the same time, one for In-Profile packets and one for Out-of-Profile packets.

In this DiffServ implementation, there were originally five child classes under the AF parent class: AF1-4 classes and the BE class.  1 Mbps was allocated to each AF class, 1 Mbps to Best-Effort (BE), and 5 Mbps to the AF parent class.  The BE class was marked as ‘default’ which means that if any packet did not match a filter/classifier, it was placed into this default class.  These five classes would share the bandwidth of the Parent class, and a priority mechanism built into the ALTQ software would prioritise the traffic in times of congestion.  (The priority was AF4 highest, AF3 to AF1 progressively lower, and BE lowest)

However, this approach did not function correctly under the ALTQ software for some unknown reason, so a different class hierarchy was tried.  Each of the AF1-4 classes remained at 1 Mbps, and the AF parent class at 5 Mbps, but the specific BE class was removed.  Now the AF parent class was marked as the default class to handle unfiltered or BE traffic.  So even when the AF classes were full, 1 Mbps was available for BE traffic.  Of course this amount can be configured, as the operator prefers.  When not in congestion, the AF1-4 and BE classes share and borrow bandwidth from each other.

3.1.2	ALTQ config File

The config file, used to specify the behaviour of ALTQ, is as follows:

# Configuration for interface XL1

interface XL1 bandwidth 10M cbq

class cbq XL1 ROOT_class NULL pbandwidth 100

class cbq XL1 CTL_class ROOT_class control priority 7 pbandwidth 2

As above, the Ethernet interface in use is ‘XL1’, and it has a maximum bandwidth of 10 Mbps.  ‘CBQ’ will be used on this interface (default).

A root traffic class ‘ROOT_class’ is then defined with 100% of the bandwidth (pbandwidth 100).  This class has no parent, hence the parameter NULL.

A control traffic (ICMP, SNMP etc.) class called ‘CTL_class’ is then defined, which has ‘ROOT_class’ as parent, top priority, and 2% of the bandwidth.

# Class

class cbq XL1 EF_class ROOT_class priority 7 pbandwidth 2 rio

riometer XL1 EF_class 0 8000 0xb8

# Filter

filter XL1 EF_class 10.10.16.3 5501 10.10.15.3 0 0

As above, the EF_class has ROOT_class as parent, top priority, 2% bandwidth and RIO is running in this class.

Regarding the ‘riometer’ command and parameters; the first two parameters are clearly the interface and the class.  The next two parameters are the token bucket rate and the token bucket depth.  ‘0’ is a wildcard in ALTQ.  So a value of ‘0’ for the rate means that the token bucket rate is set to the rate of the class.  The depth parameter sets the bucket depth in bytes.  If 0 is specified, the depth is set to six times the max packet size of the class (which defaults to the MTU of the interface).  Packets that obey this token-bucket profile are considered to be In-Profile.  ALTQ does not give the user the ability to specify the RIO/RED thresholds manually.

But how can the drop precedence of each packet be marked? How do subsequent routers know if the packet was In- or Out-of-Profile? The final parameter is the codepoint used to mark the packets.  “riometer” assumes that the value provided here will be used to mark the In-Profile packets.  (The RIO algorithm uses bit 4 of the codepoint to mark packets as In-or Out-of-Profile.  Bit 4 set to ‘1’ means In-Profile, set to ‘0’ means Out-of-Profile).  For example for EF In-Profile packets are marked with b8h.  “riometer” will automatically mark Out-of-Profile packets with a8h.

0xb8		1011 1000

0xa8		1010 1000

In the AF draft, this is exactly as specified.  However, there are only two levels of drop precedence (for example AF11 and AF13); not three as suggested in the Draft.  At present RIO only handles two levels of Drop Precedence.  Later versions may support three levels.

As shown below, a filter that maps each packet into the correct traffic class.  The filter command is quite simple.  The format is:

filter interface class daddr dport saddr sport proto

where

daddr		: destination address

dport		: destination port

saddr		: source address

sport		: source port

proto		: IP protocol ID

ALTQ attempts to match every incoming packet into a class using the five parameters listed.  ‘0’ is interpreted as a wildcard.  Any packets that do not match any filters are automatically routed into the Default class (in our case the AF parent class).

For example;

filter xl1 AF4_class  10.0.0.1  0  10.2.2.2  0  17

would take all UDP (protocol id 17) packets from 10.2.2.2, any port, addressed to 10.0.0.1, any port, and place them into class AF4_class.

# Class

class cbq xl1 AF_class ROOT_class borrow default priority 0 pbandwidth 18 rio

# Filter

filter xl1 AF_class 10.10.16.3 5510 10.10.15.3 0 0

# Class

class cbq xl1 AF1_class AF_class borrow priority 3 pbandwidth 3 rio

riometer xl1 AF1_class 0 8000 0x28

# Filter

filter xl1 AF1_class 10.10.16.3 5508 10.10.15.3 0 0

filter xl1 AF1_class 10.10.16.3 5509 10.10.15.3 0 0

(Similarly for AF2, AF3 and AF4.)

As above, one can see the AF1_class.  Each AF class gets 3% of the bandwidth, and has AF_class as parent.  Each AF child class can borrow from AF_class.  Within each class RIO is used to manage the queue.  One can also see that AF1_class has the lowest priority while AF4_class has the highest priority.

Note here that the AF parent class (‘AF_class’) also contains the keyword ‘default’.  Note also that it contains 18% of the bandwidth while the four child classes only contain 3% each, 12% total.  This is because the AF parent class has become the Default class.  Any packets not matching any filter are placed in this class.  Best-Effort traffic is placed into this class also.  When the AF1-4 child classes are under-limit, all of the remaining bandwidth is available for the use of the BE traffic.

3.1.3	Cisco Implementation

Cisco’s CAR (Committed Access Rate) was used to implement DiffServ in a core router.  Of course, in the core of the network, DiffServ is not concerned with the microflows passing through the router, only the value of the DS byte.  So only the DS byte is used to filter, not the usual 5-tuple of source/destination address/port and Protocol ID.

It has been discovered that Cisco’s Weighted Fair Queuing (WFQ) could be a better solution.  At present the router is running IOS 12.0, and the necessary configuration options are not available.  The possibility of running IOS 11.11 and configuring WFQ should be examined in the future.

In the Cisco router, filters and traffic classes similar to the ALTQ example above are applied.  In Cisco terms, ‘access-lists’ and ‘rate limits’ are applied.  Access Lists allow the packets to be classified.  Access Lists can be simple (only filter on Source Address) or extended (filter on sa, sp, da, dp, pid, tos, prec).  In this case extended access-lists are used in order to gain access to the DS byte.  Note that Cisco does not give access directly to the DS byte.  Cisco split the DS byte into TOS and PREC, based on the IP Precedence RFCs.  The end result is the same however.  Note that access-lists can be numbered or named, but named access-lists do not work with CAR.  Any access-lists with the same number form an ‘Access Group’.

As an example:

access-list acl-index {deny | permit} protocol source source-wildcard dest dest-wildcard [precedence prec] [tos tos]

- acl-index : an identifier for this access-list

- protocol : protocol ID

- source : source address and port

- source-wildcard :  inverse of a standard mask (!!!)

- dest : destination address and port

- dest-wildcard : inverse of a standard mask (!!!)

- prec : IP Precedence value

- tos : IP ToS value

(Note:  In order to work with the DS byte, the combination of prec + tos must be used.)

Once the packet filters have been decided, traffic classes must be created for them.  The ‘Rate Limit’ command is used.  Rate-limits can be applied to an entire interface, or to an access-group.  Rate limiting also uses a token-bucket to determine whether packets are In- or Out-of-Profile and the action to be taken on each packet can be decided.  If a packet is In-Profile, the “conform-action” is specified, if Out-of-Profile, the “exceed-action” is specified.  Each can have the following values :

- transmit		: simply transmit the packet

- drop		: simply drop the packet

- set-prec-transmit	: re-mark the DS byte and transmit the packet

- set-prec-continue	: re-mark the DS byte and submit the packet for re-

classification

As an example:

rate-limit {input | output} [access-group acl-index] bps burst-normal burst-max conform-action action exceed-action action

- acl-index: access-control-list number

- bps: average rate in bits per second

- burst-normal: determines how large traffic bursts can be before some traffic is considered to exceed the limit

- burst-max: traffic between burst-normal and burst-max exceeds the limit with a probability that increases as burst size increases.

By using several access-lists, one for each incoming DS byte, and by using rate-limits to divide the available bandwidth according to some network management policy, a type of DiffServ network can be built using Cisco routers.

At the moment, an attempt is still being made to create policies that will interwork between the ALTQ routers and the Cisco router to implement DiffServ.  It must be decided how to mark packets at the edges of the network, and how to handle these packets once they arrive at the Cisco.

For example, if an AF-4 packet arrives at the Cisco, marked as ‘Out-of-Profile’, should this packet be filtered into the AF-4 class, or should it be filtered into the Best-Effort class?

3.2	RSVP over ATM

Two main problem areas can be identified in operating IntServ and RSVP over ATM [RFC2382]:

- Mapping of IntServ models to ATM service categories and QoS parameters

- Operation of RSVP over ATM, and VC (Virtual Channel) management.

Mapping IntServ models to ATM service categories and QoS parameters is a matter of determining which categories can support the goals of the service models and matching up the parameters and variables between the IntServ description and the ATM description(s).  Since ATM has such a wide variety of service categories and parameters, more than one ATM service category should be able to support each of the IntServ models.  This will provide flexibility in configuration and deployment.

The operation of RSVP over ATM requires careful management of VCs in order to match the dynamics of the RSVP protocol.  VCs need to be managed for both the RSVP QoS data and the RSVP signalling messages.  A simple test network has been established to explore the management of VCs for RSVP data and signalling messages.

3.2.1	RSVP over ATM Test Network

This network allows two hosts connected via an ATM switch to negotiate a transport connection between them using RSVP.  Once the transport connection has been negotiated, an ATM SVC (Switched Virtual Circuit), meeting the negotiated parameters, is set up through the ATM switch.  The two hosts can then communicate over this link using standard Classical IP over ATM techniques [RFC2225].

The test network used in this experiment is shown in Figure 3.3.  It consists of the following software components:

- The ATM/IF representing the ATM interface supported by the ATM switch.  This is used to set-up Classical IP over ATM links on demand.

- rsvpdump is a modified version of the RSVP protocol analyser described in Section 2.2.  It has been extended to set-up a Classical IP over ATM link between the hosts, mulder and scully, whenever a new RSVP reservation is to be set-up between them.

- A client application, which initiates the set-up of an RSVP session by sending a PATH message.  It then connects to the server and uploads some data to the server.

- A server application, which, on receipt of a PATH message, generates a suitable response in an RESV message.  It then waits to receive information from the client.

- rsvpd is a standard RSVP daemon provided with the SUN Solstice Bandwidth Reservation Protocol [SunRSVP].

Figure 3.3: Experimental Platform for RSVP/ATM Tests

3.2.1.1	Transport Link

Figure 3.4 shows the steps involved in setting up an ATM link between the hosts mulder and scully using RSVP.

Figure 3.4: Set-up of a Transport Link

- An RSVP session is opened between the rsvpd in mulder and the Client.  The client sends an RSVP PATH message.

- The PATH message is received by the rsvpd in scully and forwarded to the Server.

- The Server replies using an RSVP RESV message.

- The RESV message is trapped by the rsvpdump in mulder and scully.

- The rsvpdump modules in mulder and scully interact with the local ATM interface to set-up a Classical IP over ATM link through the ATM switch.

- The link is set-up by the ATM switch.

- The RESV message is received by the Client.  The Client can now exchange information with the server using the newly established link through the ATM switch.

3.2.1.2	Limitations and Future Work

This test network represents a first attempt at providing RSVP services over an ATM network.  The use of the link is limited to traffic between the two hosts, more specifically between the client and the server applications.  As such it does little more than set up a Classical IP over ATM link in response to an RESV message with the capacity of the link equal to the specified Maximum Link capacity in the RSVP message.  Issues regarding the transport of data from multiple sources, some of which have different QoS requirements, are not considered.

To fully explore the integration of RSVP signalling and ATM signalling, in support of the Internet Integrated Services (IIS) model, requires further study in the following areas:

- QoS translation concerning the mapping from the IIS model to a proper ATM QoS.  (This is described in [RFC2381].)

- VC management concentrates on how many VCs are needed and which traffic flows are routed over which VCs.  (This is further described in [RFC2382], [RFC2379], [RFC2380].)

- RSVP and Integrated Services Tests

3.2.2	Conclusions

The operation of RSVP over ATM is a large area of study addressing both signalling and data.  The experiment carried out focussed on a small area, the ability to set-up an ATM PVC (Permanent Virtual Circuit) in response to an RSVP trigger.  More work is needed on the interworking of the two technologies at both a signalling and transport level.

One option is to utilise a single ATM PVC between two RSVP nodes.  This can then be treated by RSVP as a single interface over which data is transported.  RSVP mechanisms can then be applied to this interface in the usual manner.  A more elegant solution would involve interworking between RSVP and ATM UNI (User-Network Interface) mechanisms for the set-up of SVCs for the transport of data.  The use of SVCs will result in a more efficient use of network resources at the ATM level.  At the transport level, this experiment utilised ATM purely as a transport mechanism and each RSVP flow used a separate ATM flow for transport.  In all cases, the ATM flow was over-provisioned for the data it needed to carry.  Thus the experiment did not consider the reservation and policing capabilities inherent in ATM systems.  Further work should concentrate on the interoperability of the ATM reservation and policing mechanisms with those provided by RSVP.

3.3	Signalling and Interoperability

3.3.1	Microsoft Windows and Cisco IOS Interoperability

These tests were done at Helsinki Telephone Corporation Research department using a dedicated research network composed of two Cisco 2500 series routers and two PC hosts running Microsoft Windows NT 5.0 beta 2 operating system.  NetCom System’s Smartbits SBM-2000 traffic generator was used to load the network with background traffic, and RadCom Ethernet protocol analyser provided the measurement information to be further processed with mathematical graphing tool, Matlab.

Figure 3.5: RSVP Test Set-up

The network components are depicted in Figure 3.5.  A 3Com 10 Base-T hub is used to connect the routers, hosts and the Radcom analyser.  The analyser can thus capture all traffic between the routers and the hosts without any interference.  NetCom’s load generator is connected to the two additional Ethernet ports of the routers.

The routers have IOS 11.3 as their operating system, so RSVP and both Controlled Load (CL) and Guaranteed Service (GS) models are supported.  Also, the host operating systems are able to generate RSVP signalling.  The applications used are Microsoft NetMeeting (UDP) and France Télécom’s RSVPTester (TCP), which are both capable of making RSVP reservations (see Section 2.1).  With RSVPTester, the user is able to determine the reservation parameters, but in NetMeeting the reservation parameters are automatically determined by the selected video or audio codec (for example 69 kbps CL for G.711 PCM [G.711]).

The first test case was to set-up an RSVP reservation between the two hosts and to monitor the signalling.  The goal was to find out whether the routers and the hosts behaved according to [RFC2205].  RSVPTester was used as the application to initiate the reservation.

RSVP’s soft state maintenance for a single flow is controlled by two sets of timers.  The generation of the PATH and RESV messages is based on random timers having a uniform distribution between [0.5*refresh period, 1.5*refresh period].  Randomness is used to break the synchronisation among the reservations.  [RFC2205] specifies the default refresh period to be 30 seconds, thus the timer range should be {15s - 45s}.  The timing out of a reservation or path state within a single node is handled by fixed timers having a lifetime (K+0.5)*1.5*refresh period.  This allows at least K-1 successive messages to be lost without state being deleted.  The default value for K is 3, so the lifetime of the local state should be 157.5 seconds.

The test results confirmed that the implementations of both Microsoft and Cisco behaved correctly in terms of the refreshments.  The time interval between successive messages varied between 15 and 45 seconds as expected.  When the Ethernet cable was disconnected from one of the routers, it caused the router to immediately send a RESVTear message toward the session sender.  The conclusion is that the Cisco router immediately tears down all the reservations coming from a certain interface when that interface becomes unusable.  The lack of PATH and RESV messages causes the router to generate a PATHTear message 120 seconds after the last received PATH message and 150 seconds after the last RESV message.  There is thus minor discrepancy toward [RFC2205], but in general the implementations are in line.

3.4	IntServ Performance Tests

The performance tests were run in the environment shown in Figure 3.5.  The main difference compared to the set-up earlier is that NetCom’s traffic generator is used to load the network with best-effort traffic (see the arrows in Figure 3.5).  The most interesting part of the network is the serial link between the routers, operating at rate 2 Mbps.  The traffic between the two hosts as well as the traffic between the two traffic generator ports is forwarded along this link, so it clearly becomes the bottleneck along the route from one host to another.

3.4.1	TCP Throughput with Controlled Load (CL) Service

TCP throughput was measured between the two hosts with a 2 Mbps CBR-type UDP (no congestion control) background load over the serial link generated by NetCom.  There were only a single TCP flow and only a single background flow.  TCP acknowledgements flowing in the reverse direction were served in an otherwise empty network, so they did not affect the results.  In a pure FIFO environment, it is obvious that the TCP throughput would be very small because of TCP’s reaction to packet drops is to half the congestion window.  The packets were captured both before and after the congested link.  In the measurements in front of the congested link it is expected that there would be more packets since retransmitted packets are seen multiple times whereas at the second measuring point only the final transmission is seen.  This is true if packets are only dropped at the congested link, which should be a good assumption in this case.

RSVP was used to make different Controlled Load (CL) reservations for the TCP flow.  The different reservation parameters are shown in Table 3.4.1.  The first column is the time period in seconds during which the corresponding reservation was in place.  Token rate, bucket depth and peak rates are as specified in IntServ documents.  The final column refers to the queuing discipline used at the serial link output buffer.

| Time period (s) | Token rate (bps) | Bucket depth (bytes) | Peak rate (bps) | Queuing mechanism |
| --- | --- | --- | --- | --- |
| 28-59 | 128 000 | 1024 | 128 000 | CL + FQ |
| 59-126 | 128 000 | 2048 | 512 000 | CL + FQ |
| 126-172 | 128 000 | 0 | 512 000 | CL + FQ |
| 172-205 | 256 000 | 1024 | 256 000 | CL + FQ |
| 205-229 | 512 000 | 1024 | 512 000 | CL + FQ |
| 229-263 | 1 024 000 | 1024 | 1 024 000 | CL + FQ |
| 263-293 | 1 440 000 | 1024 | 1 440 000 | CL + FQ |
| 293-329 | 1 440 000 | 0 | 1 440 000 | CL + FQ |
| 329-342 | 1 440 000 | 512 | 1 440 000 | CL + FQ |
| 342-357 | 1 440 000 | 256 | 1 440 000 | CL + FQ |
| 357-380 | 1 440 000 | 1024 | 1 440 000 | CL + FQ |
| 380-396 | 1 440 000 | 2048 | 1 440 000 | CL + FQ |
| 396-413 | 1 440 000 | 64 | 1 440 000 | CL + FQ |
| 413-429 | 1 440 000 | 32 | 1 440 000 | CL + FQ |
| 429-448 | 1 440 000 | 16 | 1 440 000 | CL + FQ |
| 448-466 | 1 440 000 | 0 | 1 440 000 | CL + FQ |
| 466-499 | 1 440 000 | 1024 | 1 440 000 | CL + FQ |
| 499-524 | 0 | 0 | 0 | FQ |
| 524-592 | 0 | 0 | 0 | FIFO |

Table 3.4.1: Reservation Parameters over Time

The measured throughputs over time corresponding to these reservations are shown in Figure 3.6.  The horizontal axis represents time in seconds and corresponds to time periods in Table 3.4.1.  The vertical axis represents data rates measured in bits per second.  There are four graphs in the figure named Link share, Measured throughput, Reservation, and Sent traffic.  Sent traffic is the TCP data rate measured in front of the congested link, so each sent packet should be seen there.  Measured throughput is the TCP data rate measured after the congested link so it is what the receiver really gets.  Reservation corresponds to the CL token rate.  Link share is calculated according to the following equation:

LS=RES+LBW−RESBEF	(1)

where

- LS is Link share

- RES is the CL token rate

- LBW is the total link bandwidth.  (In this case, 2 Mbps.)

- BEF is the number of simultaneous flows.  (In this case, BEF is 2; TCP flow and the loading flow.)

Figure 3.6: TCP Throughput and Reservation over Time

It can be seen that when the bucket size is large enough, the TCP throughput is nicely modelled by equation (1) indicating that the flow gets its theoretical share of the overall bandwidth.  Token bucket size 0 is identical to pure fair queuing environment, since each packet of the flow is policed to be non-conforming.  Larger bucket sizes, even very small (16, 32 or 64 bytes), increase the throughput slightly.  1024 bytes seems to be a large enough bucket size, since doubling it does not noticeably affect the performance.  According to the token bucket algorithm, the bucket size would have to be larger than the incoming packet to get any packets classified as conformant.  In this case, however, the algorithm seems to be implemented differently.

The throughput measurements done before and after the congested link give almost identical results indicating that there is no large number of packet drops.  It seems that TCP is able to adapt to the reserved rate quite well.  At least in the measurement timescale (one second averages) the throughput curves are quite smooth.

Fair queuing seems to work well even when there is no reservation in place.  TCP flow is able to get roughly half of the available bandwidth as expected.  However, when plain FIFO is used, TCP throughput decreases dramatically.

The conclusion is that intelligent queuing mechanisms are indeed able to boost the throughput of the important flows compared to traditional FIFO.  With CL, a TCP source gets a guaranteed throughput according to the reservation, and is able to use the spare bandwidth as well.  The only prerequisite is that the bucket size is not too small.

3.4.2	TCP and UDP Performance over Controlled Load Reservations

In this case, the only difference to Section 3.4.1 is that in addition to the single TCP flow with a reservation.  NetMeeting was used to make reservations for UDP traffic.  The UDP flow is actually an H.263-coded variable bit-rate video, and it is run between the same hosts (and via the same route) as the TCP connection [H.263].  NetMeeting makes an automatic bi-directional 690 kbps reservation for the video.  There is also an additional reservation for voice, but in this set-up, no voice traffic was sent.  For TCP, a 512 kbps reservation was made (bucket size 1024 bytes, peak rate 512 kbps).  The background load is a constant bit rate source sending at 2 Mbps.

Figure 3.7 shows the measured bit rates for the two sources and the reservation token rates.  It is seen that the UDP source stays clearly below its reserved rate at all times.  TCP throughput follows quite well the following equation:

LS=TCPRES+LBW−TCPRES−UDPBEF		(2)

where:

- LS is TCP link share

- TCPRES is the TCP reservation token rate

- LBW is the total link bandwidth (2 Mbps)

- UDP is the UDP traffic rate

- BEF is the number of best-effort flows competing for the spare resources.

Figure 3.7: TCP and UDP Throughput and Reservation over Time

The link share is based on the following reasoning.  First, TCP gets its reserved rate (TCPRES).  Also UDP source always gets what it sends, because it is always below its reserved rate.  The rest (LBW-TCPRES-UDP) is fairly divided between the competing best-effort flows, namely the non-conforming TCP traffic and the background traffic.  From Figure 3.7, no difference can be seen between the theoretical and the measured throughput.  Thus, it can be concluded that the routers behave as expected.

3.4.3	UDP Performance over Controlled Load Service

In this test case, the NetCom traffic generator was connected in place of the PCs in Figure 3.5.  The traffic generator was used to generate four constant bitrate (CBR) UDP-sources from one port to another.  The rate of each source was configured to be 250 kbps, making the traffic aggregate rate 1 Mbps.  The packet size within each flow was 125 Bytes, so the packets were smaller than the average.  The serial link was configured to the capacity of 800 kbps, so the offered traffic was larger than the link could serve.  A Controlled Load reservation with token rate 250 kbps was manually configured for one of the flows.  The reservation was presented by the Cisco routers as follows:

BPS    To              From            Protoc DPort  Sport  Weight Conversation

250K   194.241.226.186 194.241.226.178 UDP    3450   4567   4      264

Delay and throughput over the two routers were measured for each of the flows to find out whether Controlled Load was able to solve the congestion problem for the flow with the reservation.  Figure 3.8 shows the throughput distribution among the flows.  The flow with the reservation gets roughly 32,7 % of the capacity (about 262 kbps) while the others are left to about 22,5 % (180 kbps).  The flow with the reservation is thus better off than its competitors and gets its all packets through the congested serial link.

Figure 3.8: Throughput Distribution among the Competing Flows

Figure 3.9 and Figure 3.10 show the delay variation and distribution of a flow with Controlled Load service and a flow with best-effort service, respectively.  For the flow with the reservation, the delay is around 2-3 milliseconds, while the best effort flow experiences delay of 120-130 milliseconds.  The delay for the best-effort traffic is constantly high because the best-effort buffers are constantly full because of the traffic overload.

Figure 3.9: Delay Variation and Distribution of the Flow with CL Service

Figure 3.10: Delay Variation and Distribution of the Flow with BE Service

The conclusion from the measurements is that at least in this simple case the Controlled Load service offered by the Cisco 2514 router is as good as expected.

4	DiffServ Performance Tests

This section demonstrates some of the operating characteristics of the DiffServ implementation shown in Section 3.1.  Several scenarios were designed where varying amounts of traffic were fed into each class and the results monitored.

The tool used to generate packets is Traffic Generator (TG), a freely available UNIX tool.  The tool used to create the graphs is cbqmonitor from the ALTQ package (see Figure 4.1 and Figure 4.2).

The traffic classes are given as follows:

Default 		:	Best-Effort (BE)

Class 0		:	EF

Class 1		:	AF-1

Class 2		:	AF-2

Class 3		:	AF-3

Class 4		:	AF-4

4.1	DiffServ Testing Scenario

The aim of the following simulation is to demonstrate the behaviour of six traffic types in both non-congested and congested situations.  BE, EF and AF 1-4 traffic flows are examined.  In the graphs, the throughput of the first FreeBSD Edge Router is shown, i.e., this router controls the injection of marked traffic into the network.

In this scenario, the ingress (entry) router is configured to filter packets into one of the six classes and each class has been allocated the following bandwidths:

| EF | 460 kbps |
| --- | --- |
| AF1 | 250 kbps |
| AF2 | 250 kbps |
| AF3 | 250 kbps |
| AF4 | 250 kbps |
| BE | 700 kbps |

In total, the scenario attempts to pass 10 traffic flows through this router:

| EF | 450 kbps |
| --- | --- |
| AF1 | 135 kbps |
| AF1 | 135 kbps |
| AF2 | 135 kbps |
| AF2 | 135 kbps |
| AF3 | 135 kbps |
| AF3 | 135 kbps |
| AF4 | 135 kbps |
| AF4 | 135 kbps |
| BE | 800 kbps |

The objective is to show the following:

- The service rate of the EF traffic is slightly higher than the arrival rate.  In theory, EF should see minimal delays through the router and should not be affected by the behaviour of AF or BE traffic.  EF traffic is assumed to be pre-configured and the bandwidth allocated to EF is strictly reserved.

- Each AF class can be examined in either uncongested or congested modes simply by starting one or two traffic flows.

- BE traffic should always take whatever bandwidth remains after AF traffic is serviced.

Figure 4.1: Throughput per Class

Figure 4.2: Packet Drops per Class

The following table gives a description of these graphs over time:

| Time | Action | Result |
| --- | --- | --- |
| t=0 | Simulation begins |  |
| t=20 | One 840 kbps BE flow starts One 430 kbps EF traffic flow starts Four 135 kbps AF 1-4 flows start | AF and EF not congested. BE congested, some BE dropping occurs. |
| T=90 | Four more 135 kbps AF 1-4 flows start | AF 1-4 now congested. AF 1-4 dropping occurs. BE available bandwidth reduced, greater BE dropping occurs. EF not affected. |
| t=220 | AF1 reduced to 135 kbps | AF1 no longer congested. AF1 drops reduce to zero. BE takes the available bandwidth, BE drops reduce slightly. EF not affected. |
| t=300 | EF traffic stops | AF 1-4 traffic not affected. BE traffic increases slightly? (This should not happen - Possibly due to reduction in processing overhead) |
| t=330 | AF 2-4 reduced to 135 kbps | AF 2-4 no longer congested, dropping reduced to zero. BE takes available bandwidth. BE drops reduce. |

4.2	Conclusions of DiffServ Performance Tests

This simulation demonstrated that the FreeBSD DiffServ implementation functioned as expected.  Some particular results were noted:

- EF traffic was handled as planned.  It is not strictly necessary to reserve the EF bandwidth, but this was how it was decided to implement EF.  The EF traffic was never affected, regardless of the AF and BE traffic levels.

- AF traffic behaved as expected also.  When the four AF classes were all uncongested, they passed the traffic without any dropping.  When all were congested, some dropping occurred in each class.  When one AF class is not congested, no drops occur, and the excess bandwidth is shared among the other AF and BE classes.

- Throughout this simulation, BE traffic is congested.  Whenever AF traffic increases, BE throughput reduces, and vice-versa.  This is as expected.

- By measuring the number of packets in each AF class reaching the destination point, some idea could be gained of the relative priority of each class.  The following results were obtained.

| Class | In-/Out-of-Profile | Number of packets |
| --- | --- | --- |
| AF 1 | In | 29609 |
|  | Out | 14 |
| AF 2 | In | 30453 |
|  | Out | 7 |
| AF 3 | In | 31970 |
|  | Out | 2 |
| AF 4 | In | 31139 |
|  | Out | 0 |
| BE |  | 35025 |

- Unfortunately this result did not quite work out quite as expected.  It can be seen that the number of packets successfully delivered increases from AF 1 to AF2 to AF3; however at this point the number of packets delivered to AF4 decreases.  This experiment was performed several times, and similar results were found.  There is no apparent explanation for this.  It is assumed to be a quirk of the ALTQ software implementation.

4.3	Future Work in DiffServ Performance Testing

So far, most of the parameters necessary to implement a ‘DiffServ-like’ queuing network have been determined.  In the future, an attempt will be made to create policies, which will determine the treatment of packets from edge-to-edge through the network.  For example, in the core of the network how should Out-of-Profile AF packets be treated? Should they be dropped at the first opportunity? Should they be passed as Best-Effort? Should they be re-queued with the In-Profile packets?

5	Applying QoS to Voice over IP (VoIP)

To verify the feasibility of RSVP/IntServ for Voice over IP (VoIP), some tests were performed in the similar test set-up already discussed in Section 3.3.  (See Figure 5.1.)

Figure 5.1: VoIP RSVP Test Environment

France Télécom’s RSVPTester was used to generate emulated voice traffic from one PC to another and also to signal reservations for some of the voice sources with RSVP.  NetCom Smartbits was again used to generate background traffic and RadCom’s analyser captured all the packets from the hubs in order to calculate delay and loss patterns.

All voice sources generated on/off-traffic according to a Markovian model (exponential distribution, on-period mean length of 4 seconds, off-period mean-length of 6 seconds).  The rate of the sources was equal to the rate generated by ITU-T G.711 PCM codec, i.e., the payload rate was 64 kbps.  All sources were independent from each other so statistical multiplexing gain was considerable.  The speed of the serial link was either 500 kbps or 2 Mbps.  The background traffic from the load generator was also on/off-type with bursts that were able to overload the serial link.  All reservations were made so that there was no need for policing, i.e., all traffic was always conformant to the reservation.

Five different tests were performed.  Table 5.1 summarises the parameters for each of them.  The first three rows indicate the number of voice sources and the service class offered to them (GS, CL or BE).  Background traffic discusses the traffic model for the NetCom generator.

|  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- |
| Number of GS flows | 8 | 8 | 2 | 2 | 2 |
| Number of CL flows | 8 | 8 | 2 | 2 | 2 |
| Number of BE flows | 8 | 45 | 2 | 2 | 10 |
| Background Traffic: burst and pause lengths, number of flows | 240 packets at 2.4 Mbps, 0.7 s. pause between bursts, 24 flows | - | 60 packets at 500 kbps, 0.7 s pause between bursts, 4 flows | 60 packets at 500 kbps, 0.7 s pause between bursts, 24 flows | - |
| Serial link Capacity | 2 Mbps | 2 Mbps | 500 kbps | 500 kbps | 500 kbps |
| Serial link Maximum queue length | 64 packets | 64 packets | 64 packets | 64 packets | 64 packets |

Table 5.1: VoIP Test Cases:

(GS = Guaranteed Service, CL = Controlled Load, BE = Best-Effort)

The following items were measured and analysed for each test case:

- Delay distribution of GS, CL and BE flows including the mean and 90th and 95th percentiles

- Packet loss rates of GS, CL and BE flows

- Burstiness and jitter of the delay over time

The test results are provided in Table 5.2.  The results are averages of all flows belonging to a same service class.

|  | Mean Delay | 90th Percentile | 95th Percentile | Packet Loss |
| --- | --- | --- | --- | --- |
| Case 1 | BE: 173.5 ms CL: 40.5 ms GS: 34.4 ms | BE: 360 ms CL: 132.5 ms GS: 115 ms | BE: 705 ms CL: 166.4 ms GS: 156.9 ms | BE: 27 % CL: 0.5 % GS: 0.7 % |
| Case 2 | BE: 79.9 ms CL: 5.3 ms GS: 4.7 ms | BE: 176.3 ms CL: 8.2 ms GS: 7.4 ms | BE: 191.6 ms CL: 10.7 ms GS: 8.7 ms | BE: 10.5 % CL: 0.8 % GS: 1.4 % |
| Case 3 | BE: 524 ms CL: 19.0 ms GS: 19.3 ms | BE: 1130 ms CL: 27.7 ms GS: 29.3 ms | BE: 1220 ms CL: 34.5 ms GS: 34.7 ms | BE: 19 % CL: 3.2 % GS: 0.5 % |
| Case 4 | BE: 657 ms CL: 36.4 ms GS: 47.2 ms | BE: 876 ms CL: 86 ms GS: 97 ms | BE: 1114 ms CL: 103 ms GS: 162 ms | BE: 5.2 % CL: 0 % GS: 0.7 % |
| Case 5 | BE: 127 ms CL: 10.7 ms GS: 11.2 ms | BE: 547 ms CL: 16.3 ms GS: 15.0 ms | BE: 624 ms CL: 18.2 ms GS: 20.2 ms | BE: 4.9 % CL: 1.7 % GS: 0.2 % |

Table 5.2: VoIP Test Results:

(GS = Guaranteed Service, CL = Controlled Load, BE = Best-effort)

Many interesting conclusions can be derived from these results.  The first one is that in Cisco’s implementation, there is no difference between Guaranteed Service and Controlled Load.  Another is that the following three circumstances, in the order of importance, determine the outcome:

- The source of the background traffic.  If it comes in from a different interface than the reserved traffic, the router runs into problems.  If it comes from the same interface, the router is able to cope with it.

- The capacity of the serial link (or in general any interface).  If it is too high, IntServ does not work properly in all cases.

- The number of competing flows.  The lesser the better, especially if they come in from a different port.

In test cases 2 and 5 where all traffic comes in from the same Ethernet interface, GS and CL seem to work very well.  Loss rate is below 2 % and for 95 % of the packets the delay is bounded below 10 ms with the 2 Mbps serial link and below 20 ms with the 500 kbps serial link.  In both cases best-effort traffic suffers considerably.

However, when the background load is fed from the other Ethernet port than the IntServ flows, the situation is worse.  In case 1 where the serial link capacity is 2 Mbps, over 10 % of GS and CL packets suffer from over 100 ms delay.  A good thing is that packet loss is still minimal.  For BE traffic, the loss and the delay are huge in case 1, making the service almost unusable for most applications.

When the serial link capacity is reduced to 500 kbps to decrease the router-processing burden, the situation gets better, but not yet good enough.  This is shown by test case 4, where GS and CL packets have less delay than in case 1 but still 10 % of the packets have over 90 % delay.  The next thing to help the router is to decrease the number of competing flows (still keeping the aggregate load equal), as is done in test case 3.  In that case GS and CL get excellent service; the 95th percentile of the delay distribution is below 35 ms.

In conclusion, it could be stated that only cases 2, 3 and 5 would be good enough for voice, and only cases 2 and 5 could cope with the VoIP end-to-end delay budget (150-200 ms) when combined with the terminal delays.  The main reason for the poor results is probably that Cisco 2500 Series was never designed to do anything other than low-bitrate FIFO packet forwarding.  It has a single centralised CPU, which can easily become a bottleneck while calculating complex scheduling decisions such as WFQ.  In addition, all data within the router is transmitted over a single shared bus, which may cause unpredictable delays.  In the future, it would be interesting to perform similar tests with a more powerful, but otherwise quite similar, router platform (e.g., 2600 Series) or with a totally different router architecture (e.g., 7500 or 12000 Series).

References

| [CBQ] | S. Floyd, V. Jacobson, “Link-Sharing and Resource Management Models for Packet Networks”, Transactions on Networking, Vol. 3, No. 4, August 1995 www-nrg.ee.lbl.gov/floyd/papers.html |
| --- | --- |
| [G.711] | ITU-T Recommendation G.711, (Nov. 88), Pulse code modulation (PCM) of voice frequencies |
| [H.263] | ITU-T Recommendation H.263, (Feb. 98), Video coding for low bit rate communication |
| [RED] | S. Floyd, V. Jacobson, “Random Early Detection Gateways for Congestion Avoidance”, IEEE/ACM Transaction on Networking, Vol. 1 No. 4, pp.397-413, August 1993 www-nrg.ee.lbl.gov/floyd/papers.html |
| [RFC2205] | “Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification”, R. Braden (ed.), L. Zhang, S. Berson, S. Herzog, S. Jamin, September 1997 |
| [RFC2225] | “Classical IP and ARP over ATM”, M. Laubach, J. Halpern, April 1998 |
| [RFC2379] | “RSVP over ATM Implementation Guidelines”, L. Berger, August 1998 |
| [RFC2380] | “RSVP over ATM Implementation Requirements”, L. Berger, August 1998 |
| [RFC2381] | “Interoperation of Controlled-Load Service and Guaranteed Service with ATM”, M. Garrett, M. Borden, August 1998 |
| [RFC2382] | “A Framework for Integrated Services and RSVP over ATM”, E. Crawley, L. Berger, S. Berson, F. Baker, M. Borden, J. Krawczyk, August 1998 |
| [RIO] | D. Clark, W. Fang, “Explicit Allocation of Best Effort Packet Delivery Service”, November 1997 http://diffserv.lcs.mit.edu/Papers/exp-alloc-ddc-wf.pdf |
| [SNOOP] | Daedalus Research Group, University of California, Berkeley, www.cs.berkeley.edu/~hari/snoop |
| [SunRSVP] | Sun Microsystems, Solstice Bandwidth Reservation Protocol v1.0, www.sun.com/software/bandwidth/rsvp/index.html |
| [TCPDUMP] | V. Jacobson, C. Leres, S. McCanne; Lawrence Berkeley National Laboratory; ftp://ftp.ee.lbl.gov/tcpdump.tar.Z |
| [WFQ] | A. Demers, S. Keshav, S. Shenker. “Analysis and simulation of a fair queueing algorithm”, Proceedings of ACM SIGCOMM, pp. 1-12, Austin, Texas, September 1989. |

RSVP messages are transmitted with the ‘2E’ code in the IP header protocol field.

It is, of course, not exactly true for TCP streams because of internal TCP mechanisms, that may modify shaping (slow start and Nagle algorithms, flow control, etc.)

Unfortunately, the autodial capability from the ASCEND router to the partner routers didn’t work properly during the test phases, so it was necessary to simultaneously establish connections from all involved partners to the ASCEND router during tests.

mulder and scully are, respectively, a SPARC20 running Solaris 2.4 and a SPARC5 running Solaris 2.5.1.  They are both a bit unstable, but scully tends to be much more stable and consistent than mulder.  This is only to be expected considering whom they were named for.