Project P803-PF

European IP Testbed (Implementation Phase)

Deliverable 4

Supporting Differentiated Quality of Service in IP Networks

Volume 3 of 3: Annex A - IP Telephony Demonstrator

Suggested readers:

- Managers, IP network designers/planners

- People interested in Quality of Service in the Internet

For full publication

August 1999

EURESCOM PARTICIPANTS in Project P803-PF are:

- Deutsche Telekom AG

- Eircom

- FINNET Group

- France Télécom

- MATÁV Hungarian Telecommunications Company Ltd.

- Tele Danmark A/S

- TELECOM ITALIA S.p.a.

- Telekom Austria AG

This document contains material which is the copyright of certain EURESCOM PARTICIPANTS, and may not be reproduced or copied without permission.

All PARTICIPANTS have agreed to full publication of this document.

The commercial use of any information contained in this document may require a license from the proprietor of that information.

Neither the PARTICIPANTS nor EURESCOM warrant that the information contained in the report is capable of use, or that use of the information is free from risk, and accept no liability for loss or damage suffered by any person using this information.

This document has been approved by EURESCOM Board of Governors for distribution to all EURESCOM Shareholders.

List of Authors

| Günther Körbler, | Telekom Austria AG |
| --- | --- |
| Herwart Wermescher, | Telekom Austria AG |

Table of Contents

List of Authors	i

Table of Contents	ii

Abbreviations	iii

1	Introduction	1

1.1	The Future of VoIP	2

1.2	The Current Problem	4

2	Voice over IP (VoIP)	5

2.1	VoIP Characteristics	5

3	VoIP Performance Issues	7

4	Quality of Service (QoS) in VoIP	9

4.1	IP QoS at the Edge and in the Core	9

4.1.1	QoS on the Edge	9

4.1.2	At the Core	11

4.2	Resource reSerVation Protocol (RSVP)	11

4.3	Market need for QoS	12

4.4	Light at the End of the Tunnel	12

4.5	Order Out of Chaos; The Policy Solution	13

5	Examples of using QoS and VoIP	15

5.1	CiscoAssure Policy Administration	15

5.2	Providing End-to-End Intelligence	16

5.3	Cisco QoS Mechanisms	16

5.3.1	At the Edge	17

5.3.2	In the Core	18

5.4	Cisco NetWorks NFT	18

5.4.1	Integrated Service (IntServ) Model	18

5.4.2	Differentiated Service (DiffServ) Model	19

5.4.3	Cisco NetWorks QoS Model	19

5.5	How ThruQoS Enhances VoIP	20

5.6	Using Packeteer’s PacketShaper™ for Network Analysis	21

5.6.1	What is PacketShaper?	22

5.6.2	Discovery and Classification of Traffic Types	22

6	QoS Testing in the VoIP Environment	25

6.1	VoIP Call Elements	25

6.2	What needs to be Tested, Measured and Evaluated?	25

6.3	How to Test, Measure and Evaluate	26

6.4	Test Methodology	28

6.5	Test Cases	29

6.6	What is Pathchar?	30

6.6.1	Understanding Pathchar	30

6.6.2	Bandwidth Tests with Pathchar	31

6.6.3	Pathchar Execution Output Descriptions	31

6.6.4	Terminal Output of Pathchar from France to Austria	32

6.6.5	Terminal Output of Pathchar from France to Germany	33

References	37

Abbreviations

| ABR | Available Bit Rate |
| --- | --- |
| ACL | Access Control Lists |
| AIN | Advanced Intelligent Network |
| ATM | Asynchronous Transfer Mode |
| BECN | Backward Explicit Congestion Notification |
| CAR | Committed Access Rate |
| CBQ | Class-Based Queuing |
| CBR | Constant Bit Rate |
| CIR | Committed Information Rate |
| CLI | Command Line Interface |
| CLP | Cell Loss Priority |
| COPS | Common Open Policy Service |
| CoS | Class of Service |
| CQ | Custom Queuing |
| DCOM | Distributed Component Object Model |
| DE | Discard Eligibility |
| DHCP | Dynamic Host Configuration Protocol |
| DiffServ | Differentiated Services |
| DNS | Domain Name System |
| DSP | Digital Signal Processor |
| DTMF | Dual Tone Multi-Frequency |
| EIR | Excess Information Rate |
| ETSI | European Telecommunications Standards Institute |
| FECN | Forward Explicit Congestion Notification |
| FRTS | Frame Relay Traffic Shaping |
| FTP | File Transfer Protocol |
| GSM | Global System for Mobile communication |
| GSM-EFR | GSM Enhanced Full-Rate |
| GSM-FR | GSM Full-Rate |
| GTS | Generic Traffic Shaping |
| HTTP | Hyper-Text Transfer Protocol |
| ICMP | Internet Control Message Protocol |
| IETF | Internet Engineering Task Force |
| IntServ | Integrated Services |
| IP | Internet Protocol |
| IQD | Inter-Quartile Distance |
| ISDN | Integrated Services Digital Network |
| ISP | Internet Service Provider |
| ITU-T | International Telecommunication Union - Standardisation Section |
| IVR | Interactive Voice Response |
| LAN | Local Area Network |
| LDAP | Lightweight Directory Access Protocol |
| MAC | Media Access Control |
| MCR | Minimum Cell Rate |
| MOS | Mean Opinion Score |
| MPLS | MultiProtocol Label Switching |
| MPOA | MultiProtocol over ATM |
| MTU | Maximum Transmission Unit |
| NBAR | Networked-Based Application Recognition |
| OC-48 | Optical Carrier - 48 (2.488 Gbps) |
| PBR | Policy-Based Routing |
| PCR | Peak Cell Rate |
| PSQM | Perceptual Speech Quality Measurement |
| PSTN | Public Switched Telephone Network |
| QoS | Quality of Service |
| RED | Random Early Discard |
| RFC | Request for Comments (IETF standards) |
| RSVP | Resource reSerVation Protocol |
| RTCP | Real-Time Control Protocol |
| RTP | Real-Time Transport Protocol |
| RTP-HC | Real-Time Protocol - Header Compression |
| RTT | Round Trip Time |
| SNA | System Network architecture |
| SS7 | Signalling System 7 |
| SUT | System Under Test |
| TCP | Transmission Control Protocol |
| TIPHON | Telecommunications and Internet Protocol Harmonisation Over Networks |
| ToS | Type of Service |
| TTL | Time To Live |
| UDP | User Datagram Protocol |
| URL | Uniform Resource Locator |
| VAD | Voice Activity Detection |
| VoIP | Voice over IP |
| VPN | Virtual Private Network |
| WAN | Wide Area Network |
| WFQ | Weighted Fair Queuing |
| WRED | Weighted Random Early Detection |
| WRR | Weighted Round Robin |

1	Introduction

IP Telephony or Voice over IP (VoIP) started as a hobby among the first Internet enthusiasts, who were trying to make PC-based phone calls over the same WAN links that let them visit Internet sites around the globe, without incurring long-distance telephone charges.  Many of these early pioneers concluded that the connectionless, packet-based Internet Protocol (IP) [7] on which the Internet is built is ill suited to real-time voice traffic.

But those early Web surfers were on to something big.  Pioneer VoIP developers soon began offering commercial applications, like Internet Phone from VocalTec Communications.  In the past three years, the explosive growth of the Internet has allowed the development of key enabling technologies that now make VoIP a viable solution for delivering cost-effective, business-grade voice applications.  Global sales of IP telephony products hit $80 million in 1997, and major research institutions are projecting phenomenal market growth.  In the United States alone, Forrester Research predicts the VoIP market to grow from $30 million in 1998 to a $2 billion market in 2004.

In recent years, the business world has reaped tremendous benefits from the many exciting products and applications made possible by the marriage of data and voice technologies.  Now, the efficiencies and enhanced services resulting from that revolution are about to be eclipsed by the magnitude of change, as data networks become the transport for voice.  VoIP is the exploding new technology enabling voice to be carried over IP-based, packet-switched local and wide area networks.

Why is VoIP attracting so much attention? The advantages to a company adapting the technology are significant.  In traditional circuit switched networks, when a connection is established, a channel is dedicated end-to-end for the duration of the communication.  In the packet switched world, many types of communication share the bandwidth, which fills the available capacity much more effectively.  Speech compression technologies used in preparing voice signals for transport on a packet network eliminate “silent space” and save still more bandwidth.  In addition to this economy of scale, combining all traffic onto a single network represents an opportunity for major savings in physical plant.  The advantage currently attracting the most attention is the ability to bypass the PSTN and its toll charges, while applications made possible by merging voice with the Internet will drive the level of interest even higher.

Because of this enormous potential, the market for VoIP solutions is ramping up quickly.  However, there remain significant quality issues that must be addressed before we see widespread acceptance of VoIP as a mainstream business tool.

In the brief history of IP telephony, voice connections over the Internet have got a bad reputation because of poor quality.  Due to its real-time nature, an effective voice conversation requires a reasonable level of continuity.  That continuity can be negatively impacted by large numbers of packets (representing other types of data) competing with voice packets for network bandwidth, a situation that never existed in the circuit switched world.  The voice quality issue is made more complex because of its subjective nature.  Equipment responsible for processing voice for transport over an IP network must be able to retain all the nuance, inflection and pauses that comprise effective human communication.  It is important that service and equipment providers build into their VoIP solutions the ability to test, measure and evaluate the performance of the various elements needed to create a VoIP transmission.

Basically, the main problem with voice transport over the Internet is limited bandwidth; standard telephone quality speech uses 8,000 bytes per second, but a standard 14.4 kbps modem can send only 1,800 bytes per second.  Moreover, in an Internet connection, the bandwidth can be very inconsistent.

In the past, well-performing computer networks provided best-effort service, where the content of network bandwidth delivered to each application or protocol session varied as a function of time, hardware, available bandwidth, and built-in delay.  The process of dropping bits on the wire in a non-deterministic fashion provided network performance that was, and is still, at times, random and unreliable.

While this type of network performance was, and still is, acceptable in many cases, particularly when store and forward, batch, or non-interactive traffic is predominant, there is an emerging class of applications where best-effort performance is not acceptable.  This class of applications requires performance that is uniform, non-random, and dependable.  This type of “transactional” traffic, defined as applications that are acutely sensitive to variations in bandwidth, latency, or delay, is rapidly becoming a dilemma as networked, mission-critical applications continue to evolve into vital business assets.

Currently the poor quality of VoIP is simply unacceptable for most business-to-business and business-to-customer contact.  Compressed video and audio transmissions are very sensitive to variations in transit delay (latency).  This, combined with the short delay times required, does not allow time for retransmission of data with errors.  Thus, damaged data packets must normally be dropped, causing a break in the data stream, and unacceptable voice quality.

Domestically, in order to be usable by the enterprise, Round Trip Time (RTT) should average at or below 200 ms for interactive use (e.g., telephone or teleconference).  Longer delay or greater latency interferes with normal psychosocial cues on telephones and during teleconferences.  By comparison (average delay):

- Satellite Links  -  300 ms

- International Voice  -  150 ms

- Local Voice  -  30 ms

1.1	The Future of VoIP

Today’s large-scale VoIP implementations are built on managed IP networks and not the Internet, meaning delay, jitter and packet loss are managed.  Reduction in delay makes all the difference, and IP networks are now overcoming three elements of this: transmission, switching and queuing

Transmission delays are reduced in proportion to the size of the pipe.  Bandwidth allocation becomes irrelevant if the sizes of individual data streams are insignificant in comparison to the size of the transmission path.  Switching delays are reduced through highly specialised high-speed switches.  The transmission speeds available in carrier backbone networks mean that packet injection times are meaningless (measured in milliseconds), and compared to the 50 ms maximum delay criteria for voice, these delays are insignificant.  Finally, controlling packet flow reduces queuing delays.  This has three approaches:

- Connection-oriented control: Tag/netflow, RSVP (Resource reSerVation Protocol), MPOA (MultiProtocol over ATM) [4]

- Connectionless control: Next-hop Quality of Service (QoS) routing

- Over-provision:

In terms of network equipment, IP requires simpler, less expensive devices.  Proprietary switching systems need incredibly complex development environments requiring specialised tools and procedures.  Telco billing systems are also exceedingly complex.  Why should smart customer equipment not negotiate with intelligent hosts on the fly for services, leaving the network out of the loop? Smart network billing systems are only necessary with dumb end-points.  Integration of legacy and IP networks will provide support for existing SS7 (Signalling System 7), AIN (Advanced Intelligent Network) and billing systems.  However in a 100% IP world, these telecommunications technologies will not be required.  Clearly a great opportunity resides in the development of new business models, unencumbered by the cost of supporting traditional thinking.

As the above demonstrates, IP is a near and long-term threat to telcos.  In the short-term, Internet telephony carriers will compete on price, regulators will continue to favour Internet-based new entrants and new competitors will build critical mass.  Longer term, quality gaps will continue to close, and voice and data IP networks will converge on the same facilities and the same bandwidth.  The widespread use of integrated IP infrastructure will give rise to a new generation of communications services such as virtual call centres, unified messaging, stereo phone calls and video-on-demand.

Whether or not telecommunications carriers participate in the resultant industry growth remains the central question.  Doing nothing will certainly lead to a further erosion of the most profitable customers.  Telcos need to embark on a ‘big, fat dumb’ crash course, involving short-term steps and the willingness to make bold moves.  These steps might include:

- Discovering why a customer would switch: Price? Customer Service? Quality? Products? Why exactly each customer has left?

- Lowering settlement rates: Benchmark rates for international settlements range from US$ 0.15 to US$ 0.23 per minute.  However BT’s rate is US$ 0.07, which has led to a bad environment for IP-based new entrants in the UK.

- Introducing a competing service: It is always better to cannibalise oneself.

- Erecting regulatory barriers: This helps regulators understand what is going on.  Using market knowledge to help regulators do the ‘right’ thing is a stalling tactic but may be necessary in order to buy time.

Examples of bold moves include:

- AT&T initiative to help Internet Service Providers (ISP) establish IP Telephony services to 140 nations worldwide

- Advanced Radio Telecom is building a wireless ISP that will eventually offer VoIP

- Deutsche Telekom purchased US$ 30 million worth of IP Telephony gateways for deployment across the world including in a cellular operator network in the Czech Republic

The convergence of voice and data networks means that the PSTN users will benefit from rapid innovations in IP technology over the next number of years.  New affordable services, such as video telephony, promise to herald in a new era of innovative, multimedia communications.  The circuit-switched public telephone network of today will evolve to become the packet-switched public data network of tomorrow, carrying voice, video, and data.

1.2	The Current Problem

Network managers are increasingly presented with a variety of bandwidth-hungry applications that compete for bandwidth in both the LAN and WAN.  The simplest approach is to throw bandwidth at the problem, which is known as over-engineering the network.  At times, particularly in a LAN environment, this is the best approach.  In the WAN, over- engineering is less practical.  Declining WAN bandwidth costs will make higher speeds more affordable, somewhat mitigating the need for policy-based networking in the WAN.  However, WAN bandwidth costs will still be a significant expense for most corporations, so over- engineered WANs will never be as prevalent as over- engineered LANs.

However, due to the cost of WAN bandwidth, (particularly in the international area), over- engineering is often not a viable alternative.  Thus, optimisation of available bandwidth is the key to optimal network performance.

Enterprises that want to deploy new bandwidth-hungry applications have concluded that some mixture of bandwidth over- engineering combined with network optimisation is critical to the success of mission-critical applications over both the LAN and WAN.  This can be achieved by defining network policies, which align network resources with business objectives, and are enforced by means of QoS mechanisms.  Without these QoS controls, non-vital applications such as casual Internet browsing or large file transfers can quickly exhaust network resources at the expense of more important ones, such as transactional applications, thus compromising business processes and productivity.

These problems concerning VoIP are solved in different ways, but most share the same general solution in concept; compressing the voice signals at the sending host and decompressing at the receiving host.  Human ears can tolerate missing voice packets and latency, but the voice packets must flow in constant rhythm.  Long set-up time can be tolerated (as in a regular telephone conversation).  There is nothing useful to do with very late voice packets, so they can be discarded.  The connection can be unreliable.  Acknowledgement packets are a waste of valuable bandwidth, and the reliability will be achieved at the “Human Layer”.  This points out the use of UDP for a solution.

2	Voice over IP (VoIP)

The speech is converted into digital packets that are sent through a network and received out of order, with pieces missing.  Since the original was created in real time, a missing packet cannot be resent, thus creating a gap in the data stream, which the recipient hears as clipped words or run-together speech (see Figure 2.1).

Figure 2.1 Voice over IP Illustration

2.1	VoIP Characteristics

In order to take advantage of the promise of VoIP, a product must provide certain features to enable it to be useable.  The key to all of these requirements is that the product must be at least close to the existing telephone voice quality; if it is not, it won’t be used.  The following table attempts to define what the characteristics of a Voice over Data application must have to meet these minimum use requirements.

| FEATURE | REQUIREMENT |
| --- | --- |
| Compression: Sub-PCM compression significantly reduces the amount of bandwidth used by a voice conversation while maintaining high quality. | Must Have. |
| Silence Suppression: The ability to recover bandwidth during periods of silence in a conversation, making that bandwidth available for other users of the network. | Must Have. |
| QoS: Assuring priority for voice transmission is critical. This keeps delay, delay variation and loss to a tolerable minimum. | Must have. (Little current support.) |
| Signalling for Voice Traffic: Support of traditional PBXs and the associated signalling is critical. | Must have for real applications. |
| Echo Control: Echo is annoying and disruptive. Control is key. | Must have for real applications. |
| Voice Switching: Data network equipment can generally support on-net applications. Off-net is also critical. At the very least, the adjunct equipment must decide whether to route a call over the internal data network or to route it to the PSTN. | Ability to route off-net is a must for real applications. |

Table 2.1: Requirements for Voice over Data Applications

3	VoIP Performance Issues

New compression algorithms have significantly improved VoIP application performance.  By reducing voice bandwidth requirements to under 5 kbps, compression techniques have increased the voice-carrying capacity of an IP network by nearly tenfold.  However, the biggest stumbling block to VoIP application performance remains the number of transmission delays (or latency) imposed by the “connectionless”, packet-based IP network.  Unlike the circuit-switched PSTN, which guarantees a dedicated network path (or circuit) for the duration of each phone call, packetised voice traffic for a single VoIP call might take several paths over the IP network.  The inevitable delays associated with multiple network paths for a single voice call can introduce unnecessary latency that degrades audio quality in the time-sensitive VoIP application.

Network congestion exacerbates latency levels, causing IP to randomly drop packets for subsequent retransmission; the packets always get to their destination, they just might take several attempts to do so.  This packet-dropping behaviour is actually a part of the protocol’s design.  IP assigns all packets an equal value, giving them the same level of “best-effort” transport service across the network, with no guarantee of timely delivery.  The result is inconsistent voice quality or “jitter” during peak traffic loads.  When network latency exceeds 800 milliseconds, audio quality becomes unacceptable.  Because IP’s best effort service cannot differentiate between time-sensitive voice traffic and less time-critical traffic, such as e-mail messages, VoIP applications deliver poor voice quality when latency reaches high levels.  Figure 3.1 shows the basic trade-off for initial latency versus true bandwidth.

Figure 3.1: Latency vs. Bandwidth for VoIP

4	Quality of Service (QoS) in VoIP

Assuring priority for packets or datagrams used with voice transmission is critical, in order to keep delay, delay variation and loss to a tolerable minimum.  At the time of writing, the QoS parameters are not widely used; delivering voice over an ATM network that provides CBR (Constant Bit Rate) is an alternative, however most Internet connections today are over traditional packet or Frame Relay routes.

4.1	IP QoS at the Edge and in the Core

QoS has become a hot topic with the introduction of real-time data traffic in an IP network; best-effort approaches are just not acceptable for delivering voice in a business environment.  These are not new subjects; IP networking companies have been dealing with these issues quite successfully ever since SNA was put onto IP networks.  It is, however, an important topic as networks merge data, voice and video onto the same infrastructure.  QoS refers to the ability to ensure that packet flow through the network is sustained at a maximum throughput and that some types of packets are able to get preferential treatment.  There are now many ways to provide QoS in IP networks.  The real question is not “Is IP QoS possible?” but “Which of the many techniques is best?”

Real-time IP traffic, such as VoIP, video and SNA, typically cannot tolerate excessive delays or packet loss in the network.  Most other IP traffic can withstand delays and congestion, (since it is not time-sensitive and is connection-oriented).  Connection-oriented traffic has the ability to recover lost packets by re-transmitting them.  E-mail, for example, is considered critical data, but if it is delayed by 200 ms, it probably won’t matter.  If one or more of the packets associated with e-mail are lost, TCP recovers by re-transmitting the lost packets after the congestion has settled.  Voice, however, experiences quality degradation if packets are delayed for that long or have to wait for retransmission.

The role of IP QoS for real-time traffic is to deliver these packets with as little delay as possible, even if there is congestion on the network.  Most IP networks are already pushed to full capacity, and congestion is quite common.  With QoS techniques, real-time traffic can find ways to avoid the congestion and recover from packet loss, without retransmission or large delays.

QoS is very dependent on the weakest link, which may be the most congested point in the network or a low-speed connection on the access link.  It is very important to look at QoS as an end-to-end issue and design the network appropriately.  Integrated gateways, where the routing and voice handling occur in the same box, have the distinct advantage of being designed for real-time traffic flows and for being able to give this traffic priority treatment.  The typical integrated gateway classifies voice packets by labelling the IP packet.

4.1.1	QoS on the Edge

The edge of the network is typically where bandwidth is most limited.  At this point in the network, it is important to give real-time traffic, such as voice, the highest priority.  If data and voice are sharing limited bandwidth on a single interface, giving voice packets the highest priority will help ensure higher quality audio.

VoIP traffic is typically transferred via RTP (Real-Time Transport Protocol).  This is defined in [13] and detailed in [12] for VoIP.  Many VoIP gateways have the ability to give these packets a special importance, or weight, by modifying some bits in the Type of Service (ToS) field in the IP header that is attached to the RTP packet.  This is considered packet classification and these flagged bits are often called precedence bits.

Some QoS mechanisms treat these packets with special attention by recognising they have been given a special weight.  One example is Weighted Fair Queuing (WFQ), a packet-queuing strategy designed to give fair bandwidth allocation to all types of traffic flows on the outgoing interface.  It does this by giving low volume traffic flows preferential treatment over higher volume traffic flows.  This is ideal for voice since the typical voice call is a low bandwidth flow compared to other flows such as file transfer.  Tagging the RTP packets with a weight gives them an effective lower volume (or lower bandwidth), and they are therefore treated with higher priority.  Treating voice flows in a special way also has the added advantage of reducing the jitter caused by random delay of RTP packets in the network.  Jitter can create garbled voice if the voice gateway does not have mechanisms to adjust automatically to changing network conditions.  Another result of mild jitter is an increase in overall latency, since the ‘playout’ buffers must be larger to accommodate the jitter.

Resource reSerVation Protocol (RSVP) [6] is another QoS mechanism that works in many enterprise networks for real-time traffic.  RSVP is a per-flow QoS mechanism that attempts to provide a quantifiable delay and bandwidth requirement.  It recognises a predefined flow type and sends a request for a specified amount of bandwidth and delay through the network to the receiving node.  A response is delivered back through the flow path that has the ability to open up a guaranteed channel for the requested traffic flow.  RSVP requests can fail if the intermediate hops are not able to fulfil the request because of other restrictions on bandwidth availability.  When reservations are successful, RSVP provides high quality results.  In its current form, RSVP will probably not be the answer for QoS on real-time traffic networks, since the reservations for every voice call on an OC-48 link may be impractical.  However, for larger bandwidth flows of real-time data like video, RSVP may perform very well.

Several other rate-control mechanisms exist today to limit the amount of data being sent out on an interface.  These mechanisms may be important, for example, on a Frame Relay network where the Committed Information Rate (CIR) is less than the actual port speed.  These traffic-shaping algorithms provide the ability to limit throughput into the network.  It is important to make sure these algorithms do not completely restrict the real-time traffic since it would suffer in quality.  Examples of these mechanisms include generic traffic shaping, Frame Relay traffic shaping and Committed Access Rate (CAR).  CAR also has the unique ability to provide class-of-service policing.  If all packets coming into the router are classified with the highest “weight,” then there is no more QoS.  CAR has the ability to reclassify packets, based on port configurations.

Another barrier for quality real-time traffic at the edge of the network is the potential delay created by mixing large packets and small voice packets on the same low bandwidth connection.  On a 56K connection, a 2500-byte packet (such as an Ethernet frame) can take 214 ms to complete delivery from one node to the next.  This is considered serialisation delay.  During this time, many voice packets could be generated and inserted in the queue waiting to be delivered, creating jitter and unacceptable delays.  Some vendors have started to develop solutions to this problem by using a mechanism to chop larger packets into smaller ones and interleaving the real-time traffic.

4.1.2	At the Core

It is equally important to avoid congestion in the core of the network.  Increasing bandwidth can solve this problem, but it is not practical for most networks, since adding bandwidth usually increases costs.  Random Early Discard (RED), a queuing strategy, offers an alternate way to manage congestion at the core.  In IP networks, RED attempts to detect that traffic is coming into the interfaces faster than it can go out.  The algorithm then randomly discards packets.  Connection-oriented TCP traffic realises that packets are getting dropped and throttles back.  By randomly discarding packets before congestion occurs, the network gets much better overall throughput.  Real-time traffic can bypass the discard process by using Weighted Random Early Discard (WRED), which determines that a packet has been weighted or classified and allows it to slip through the random discarding process.

If the core of the network is a Layer 2 (Link Layer) infrastructure, there are now methods available to map the requested QoS directly into the network (done at the packet level).  One Layer 2 example uses MultiProtocol Label Switching (MPLS), which defines how to encapsulate IP traffic with a new header and apply the requested QoS in Layer 2.  It does this by mapping the IP precedence bits into the MPLS Class of Service (CoS) field.  The original IP packet can be restored as it exits the core of the network.  MPLS offers other features that can also improve routing efficiency over time.

Another method currently being proposed to the Internet Engineering Task Force (IETF) is Differentiated Services (DiffServ) [1][2][3], which provides functions similar to MPLS but does so at Layer 3 (Network Layer).  DiffServ will likely play a role at the edge of the network, and MPLS will function in the core of a Layer 2 network.  Like RSVP, DiffServ creates reservations for queues, buffers and schedules, but does not require the core routers to keep state information about the flow.  Instead it works on a packet basis.

With unlimited bandwidth, QoS is not necessary.  However, few networks are fortunate enough to have unlimited bandwidth.  Bandwidth is still not free, and probably won’t be free for many years.  QoS for real-time traffic will remain a valuable tool in the convergence of data, voice and video networks.  The good news is that IP QoS will continue to get better.  In every software release by major networking vendors, new techniques that focus on delivering high quality video or audio (IP telephony) are showing up.  These new techniques often lead to standards, which move the industry forward in improving QoS.

4.2	Resource reSerVation Protocol (RSVP)

An approach to providing and enhancing QoS on IP is RSVP [6].  The IETF developed RSVP specifically for the packet-switched multicast environment.  RSVP has been designed to meet a number of requirements:

- Support for heterogeneous service needs

- Flexible control over the way reservations are shared along branches of multicast delivery trees

- Scalability to large multicast groups

- Ability to pre-empt resources to accommodate advance reservations.

RSVP basically acts according to its name.  An RSVP request specifies the level of resources to be reserved for some or all of the packets in a particular session.  An application requests resources by specifying a flow specification, which describes the type of traffic anticipated (for example, average and peak bandwidths and level of burstiness), and a resource class specifying the type of service required (such as guaranteed delay).  A filter specification is also specified, which determines the sources to which a given reservation applies.

RSVP mandates that the receiver rather than the sender initiate a resource reservation.  While the sender knows the properties of the traffic stream it is transmitting, it has been found that the sender-initiated reservation scales poorly for large, dynamic multicast delivery trees.  A receiver-initiated reservation deals with this by having each receiver request a reservation appropriate to itself; differences among heterogeneous receivers are resolved within the network by RSVP.  After learning the sender’s flow specification via a higher level “out of band mechanism”, the receiver generates its own desired flow specification and propagates it to the sender, making reservations in each router along the way.

RSVP itself uses a connectionless approach to multicast distribution.  The reservation state is cached in the router and periodically refreshed by the end station.  If the route changes, these refresh messages automatically install the necessary state along the new route.

RTP (Real-Time Transport Protocol) and RTCP (Real-Time Control Protocol) information is simply data from the point of view of routers that move the packets to their destination.  RSVP prioritises multimedia traffic and provides a guaranteed QoS.  Routers that have been upgraded to support RSVP can reserve carrying capacity for video and audio streams and prevent unpredictable delays that would interfere with their transmission.

4.3	Market need for QoS

Vendors have been touting QoS as the ultimate solution, but research has found that it has been severely hampered by a lack of easy to use products and vendor hype that has led to rampant confusion in the marketplace.  Last year, 75 per cent of respondents were planning to implement QoS in their networks.  Yet studies conducted for this year found a huge drop in the number of respondents looking to deploy QoS (45 per cent of respondents).  Vendors tend to define QoS in their own way, which often conflicts with others in the market.  In addition, the concentration on QoS as an issue has produced a sense of ‘overkill’ among network mangers, many of which have lost interest.  Vendors may have to approach the issue in a different way.

The most common way of implementing QoS is with the ‘Bandwidth Band-Aid,’ better known as over-provisioning the network.  There are two approaches, both of which have their merits; (i) keep throwing bandwidth at the problems, or (ii) bring the intelligence into the network.  Choosing the correct approach is a company-specific decision; it is up to the user to decide what fits their business needs.

4.4	Light at the End of the Tunnel

However, the emerging technique for controlling network functions called policy-based management holds great promise for easing the administrative burden on network managers.  With policy-based management, a network manager can define specific policies for controlling network resources and security access for different users on the network.  The policies are typically set at a central site and then policy information is distributed to all devices on the network, eliminating the time-consuming task of manually reconfiguring devices.  By focusing implementation at a central site, typically the directory, decisions can be made once and then deployed throughout the network.  Policy-based management also promises to greatly lessen the skill set level required of network staff.  Companies have an extremely difficult time hiring and retaining qualified staff, and policy tools to provide a solution to this problem are being worked on.

Yet while the concept of policy-based management has caught the attention of end users, real products are only just beginning to appear on the market.  While they hold great potential, vendors need to move from the ‘policy ware’ that is flooding the market to real products that deliver the promised benefits.  Once the glitches are gone, policy-based management will help network managers to more effectively and efficiently run their networks.

4.5	Order Out of Chaos; The Policy Solution

Policy Controls can bring order to our current Intranet and Internet chaos [9].  The ultimate solution is a switched world where security and individual levels of service are contracts between users and their network providers (corporate, telcos, and ISPs).  But the world does not change overnight, so we must look at what we can do to implement policy today; what is the first stage?

First, let’s characterise traffic in networks into three types:

- Best Effort is traffic, as we know it today.  It goes out onto the network and we hope it gets to its destination.  No bandwidth controls, no guarantees.  In the future, we will continue to apply best effort traffic characterisation to things like email and unimportant web traffic.

- Profiled traffic has pre-defined rules/policies applied to it.  These policies may include bandwidth limits, priorities, reservations, security and other controls that characterise this as ‘special’ traffic.

- On-Demand traffic requires new policies to be applied as the associated applications are loaded.  An example might be an unscheduled videoconference or audio application.

There are different components in a network that play a role in policy deployment and handling of these traffic types.

- There is a centralised or relatively distributed Policy Management Server somewhere on a network.  It gathers information from hosts, switches and routers and allows a Network Operator to establish rules for users, applications and traffic.  In addition, it can serve as a ‘Mother May I’ system (admission control) to enable on-demand QoS requests.  This system will distribute the rules to policy-enabled boxes (hosts, switches, routers, bandwidth managers, etc.) and will deliver billing information to billing systems.

- On the other extreme of the network, we have hosts: clients and servers.  For some applications, like IP telephony, packet scheduling must be done here.  For countless others, it is desirable to do some traffic shaping, and QoS requests (in whatever form - 802.1p [8], ToS, reservation, etc.) on the host as well.  Host systems will provide information to the policy server(s) and to infrastructure boxes to enable them to identify users, specific application traffic, on-demand requests, and to provide statistical information to make informed policy decisions.

- Network Infrastructure boxes consist of routers, switches, firewalls, proxy servers, and bandwidth managers.  Assuming the backbone of a network is QoS-capable either through switching, dynamic provisioning or aggregate priority schemes, the edge infrastructure devices must apply policy controls to traffic entering the backbone.  In addition, some of these boxes can provide control to local subnets.  To do this, the infrastructure devices must support application of rules onto traffic to perform shaping, scheduling, bandwidth controls, and where possible, to make QoS reservations or priority decisions.  This will ensure that profiled traffic is handled as prescribed, and that on-demand requests can be properly handled.  Infrastructure boxes must get information from hosts and policy servers, and must provide dynamic information to the policy server(s).

Figure 4.1: Policy Solution

Figure 4.1 shows the roles of each of these policy players

- Hosts provide information to Policy Servers to enable them to make policy decisions.

- Policy Servers distribute rules to infrastructure devices and hosts to establish traffic profiles.

- Infrastructure Devices accept the rules and work with hosts to classify traffic and then apply policies.  On-demand requests go from the host or infrastructure device (if acting as a reservation proxy) to the Policy Server, which dynamically adjusts the rules distributed to the hosts and infrastructure devices.

5	Examples of using QoS and VoIP

5.1	CiscoAssure Policy Administration

Even network management teams that have well-planned QoS policies find that implementing and enforcing them throughout a large enterprise network is a daunting, if not overwhelming, task.  Even with advanced, intelligent network devices and the latest management tools, the fact remains that in today’s networks, configuration is often quite complex.  It is often done through a Command Line Interface (CLI), one device at a time.  This is expensive and can lead to inconsistent deployment.  Clearly, there is a need for a simpler, higher-level way to implement policy without requiring a detailed understanding of the QoS mechanisms and how they interoperate.  Cisco recognises that administrators need management tools that automate much of the lower-level configuration and maintenance duties of QoS management.  Such tools would transform management away from the CLI to a policy-based paradigm.  With these tools, administrators could define policies and let the system automatically take care of the “bits-and-bytes” tasks of implementation.  This approach would substantially increase network integrity while dramatically reducing the amount of staff time (and expense) it now takes to implement and maintain QoS.  Cisco Systems answers this need with CiscoAssure Policy Networking [5], a family of products that includes a QoS policy server (see Figure 5.1).

Figure 5.1: CiscoAssure Policy Networking Architecture

How would network managers use the CiscoAssure system to implement a QoS policy?  An administrator could define a policy such as;

“No Webcast traffic allowed inside the firewalls between 7 a.m. and 6 p.m., Monday through Friday.”

The rest of the job is automated through the policy network system.  The underlying policy and registration server identifies the devices that require configuration to reflect the policy.  The servers translate this policy into policy-binding information, then communicates the policy to the relevant network devices using industry-standard protocols such as Common Open Policy Service (COPS).  Cisco IOS® software in the devices themselves translates policy-binding information into device-level QoS enforcement mechanisms.  Pertinent devices adjust their configurations to reflect the new policy.  Now the administrator does not have to choose a QoS mechanism for each device or configure them one by one; it is all done automatically.

QoS in the LAN, as well as the WAN, can help reduce or defray capital, operations, and maintenance costs.  A smart combination of network convergence, QoS for bandwidth management and policy-based administration strategies are ways to manage costs better in a network.

5.2	Providing End-to-End Intelligence

The axiom that a chain is only as strong as its weakest link certainly applies to QoS in networks.  Because QoS is most effective when applied end-to-end, in today’s heterogeneous networks it must be media-independent.  Layer 2 (Link Layer) classification methods are the foundation of end-to-end QoS, but are limited to specific media (such as Frame Relay or Ethernet).  Yet a typical pathway in the enterprise spans several media types.  For instance, it benefits no one to assign high-priority QoS at Layer 2 to a packet on an Ethernet LAN, then send it to a Frame Relay WAN that cannot see the classification and thus ignores it.  For true end-to-end service, QoS must be media-independent; therefore, QoS must operate at Layer 3 (Network Layer).  Cisco QoS services use Layer 3 services to link together existing Layer 2 mechanisms for end-to-end policy enforcement.  These services are IP-based for transparent, ubiquitous deployment.  Each device knows how to map Layer 3 services to the appropriate Layer 2 mechanisms.

A heterogeneous network has varying degrees of congestion, and each application type has separate requirements.  Therefore, while QoS should be ubiquitous, it is not uniform.  A small WAN link needs fine-grained QoS, while coarse QoS mechanisms may be adequate for a large LAN backbone.  Cisco QoS services offers both qualitative and quantitative QoS services to support the different requirements of mission-critical data, voice and video and non-mission-critical data applications.

The actual deployment of QoS in a network requires a division of labour for greatest efficiency.  Since QoS requires intensive processing, the Cisco model distributes QoS duties between edge and core devices, which can be multilayer switches or routers.  This approach assumes a lower-speed, ‘high-touch’ edge and a high-speed, ‘low-touch’ core for optimum efficiency and scalability.  Edge switches and routers perform most of the processor-intensive work, performing application recognition and awareness operations to identify flows and classify packets according to policy.  Core devices expedite forwarding while enforcing QoS levels assigned at the edge.

5.3	Cisco QoS Mechanisms

Cisco provides an extensive palette of end-to-end QoS technologies.  What follows is a brief definition of the operations performed by edge and core devices, with examples of Cisco IOS QoS technologies that enable each one.

5.3.1	At the Edge

5.3.1.1	Congestion Management

These tools use queuing algorithms to sort traffic and determine a method of priority to schedule traffic onto output links.  Examples include:

- Custom Queuing (CQ)

- Weighted Fair Queuing (WFQ)

- Class-Based Queuing (CBQ)

5.3.1.2	Traffic Shaping and Policing

These tools manage traffic and congestion between QoS policy domains, often where there are different amounts of available bandwidth.  Examples include:

- Generic Traffic Shaping (GTS)

- Frame Relay Traffic Shaping (FRTS)

- Committed Access Rate (CAR)

5.3.1.3	Link Efficiency

These mechanisms work in conjunction with queuing and traffic shaping to improve application service-level efficiency and predictability.  Examples include:

- Real-Time Protocol - Header Compression (RTP-HC)

- Voice Activity Detection (VAD)

- Voice/Data Compression

5.3.1.4	QoS Signalling

Signalling mechanisms give QoS domain-edge devices the ability to send requests for priority handling to their neighbours.  Signalling plays a key role in configuring successful end-to-end QoS, especially in Guaranteed Services.  Because it must work from end to end, signalling must be at Layer 3 and be media-independent.  Examples of Layer 3 QoS signalling tools include:

- RSVP: for Guaranteed (Quantitative) QoS

- IP Precedence: for 3-bit (8 levels) Qualitative QoS

- DiffServ: for 6-bit (64 levels) Qualitative QoS (currently undergoing standards review by the IETF)

Layer 3 QoS signalling maps into the appropriate Layer 2 QoS signalling technology.  For example:

- 802.1p over Ethernet

- Cell Loss Priority (CLP), Minimum Cell Rate (MCR), Peak Cell Rate (PCR) over ATM

- Discard Eligibility (DE), Forward Explicit Congestion Notification (FECN), Backward Explicit Congestion Notification (BECN), Committed Information Rate (CIR), Excess Information Rate (EIR) over Frame Relay

5.3.1.5	Application Recognition

These features provide packet classification, which selects appropriate packets traversing a network device or interface for QoS.  CiscoAssure Policy Networking places a dynamic policy control layer on top of these features.  Some examples include:

- Committed Access Rate (CAR)

- Policy-Based Routing (PBR)

- Networked-Based Application Recognition (NBAR)

- Access Control Lists (ACL)

5.3.2	In the Core

5.3.2.1	Congestion Avoidance

This operation monitors network traffic loads in an effort to anticipate and avoid congestion at common network bottleneck points.  Some examples include:

- Available Bit Rate (ABR)

- Weighted Random Early Discard (WRED)

5.3.2.2	Bandwidth Schedulers

These mechanisms provision bandwidth per aggregate queue (Qualitative QoS), or per session (Quantitative QoS).

- Weighted Round Robin (WRR)

- Rate Scheduling

5.4	Cisco NetWorks NFT

Since inception, the Internet has been operating under one service model; best-effort.  All packets are treated the same, no matter what applications they belong to.  Such a model is no longer sufficient for today’s Internet, where real-time applications such as audio and video are becoming important.  Real-time applications are extremely sensitive to packet delays; thus they require a different kind of QoS from traditional Internet applications.  The IETF has set up two working groups to improve the Internet in the QoS area; the Integrated Services (IntServ) Working Group and the Differentiated Services (DiffServ) Working Group.

5.4.1	Integrated Service (IntServ) Model

The IntServ Model defines three classes of services:

- Guaranteed Service: packet delays have an upper bound

- Controlled Load: resembles best-effort found in a network that is lightly loaded

- Best-Effort: the model in the Internet today

In providing Guaranteed Service, the IntServ Working Group has defined RSVP, which allows the receiver to request a certain amount of bandwidth, allocated to its communication flows.  Figure 5.2 shows the how RSVP works.

Figure 5.2: RSVP Operation

The RSVP protocol places many demands on router resources.  Each router along the path must maintain states for each flow.  As the number of flows increases, the demands placed on routers increases proportionally.

5.4.2	Differentiated Service (DiffServ) Model

The DiffServ Model provides a kind of QoS that requires less resources from routers, and at the same time, can treat traffic flows differently according to application needs.  With the DiffServ Model, routers do not need to keep state information for traffic flows.  This architecture uses the ToS field of the IP header and allows the sender to mark the field according to the need.  Routers along the path look at the ToS bits and act accordingly.

5.4.3	Cisco NetWorks QoS Model

Cisco NetWorks software includes the RSVP client library, which allows Internet devices to request, guaranteed bandwidth for their applications.  In addition, Cisco NetWorks software supports the IP ToS DiffServ Model.  The inclusion of both QoS models allows service providers to integrate the two service models and use the advantages of both.  Network QoS is a subject that involves both network devices and routers.  Not only does Cisco NetWorks software provide sufficient QoS for real-time Internet applications; it also enjoys the support of 80% of the routers on the Internet.  For example, a service provider can implement RSVP at the edge of the network and use ToS for core routers (see Figure 5.3).

Figure 5.3: Mixed Use of RSVP and IP Precedence QoS

5.5	How ThruQoS Enhances VoIP

Because it is all too easy for a dissatisfied VoIP application user to return to the ubiquitous ‘tried-and-true’ service of the PSTN, it’s imperative that VoIP applications include proven QoS technology that can deliver performance similar to the PSTN.  Fortunately, a product named ThruQoS provides the technologies required, delivering more predictable, reliable, and guaranteed audio-transmission services on IP networks [10].

On uncongested networks, ThruQoS can improve application performance by allowing the network administrator to use simple traffic prioritisation mechanisms like IP Precedence/ Class-Based Queuing (CBQ).  The assigned IP Precedence level determines the order in which different traffic types are forwarded over the IP network.  By assigning a high priority to VoIP traffic, the network administrator ensures that VoIP data is forwarded over the WAN before ordinary e-mail and other low-priority traffic.  If congestion occurs, the low-priority traffic is delayed to ensure that the high-priority VoIP traffic receives expedited service.  Whereas, if congestion causes packets to be dropped, the low-priority packets are dropped first.  IP Precedence can be used to expedite traffic across the Internet backbone, where it is difficult to guarantee bandwidth without establishing a VPN managed by a single ISP or corporation.

But, time-sensitive two-way voice applications generally require more sophisticated QoS solutions, such as IPHighway’s QoSMasterTM, which supports “policy-based” dynamic bandwidth management capabilities that automatically reserve bandwidth completely through the network to ensure the VoIP application receives a steady rate of throughput.  For example, the network administrator can define a policy that automatically allocates 10 KBPS for international long-distance VoIP calls between the Paris sales office and the corporate headquarters in New York.  By reserving bandwidth end-to-end across the entire enterprise network, ThruQoS technology guarantees the smooth delivery of real-time VoIP applications, minimising gaps, out-of-sequence data and other problems that undermine audio transmission quality.  RSVP-based ThruQoS, which guarantees a steady rate of data delivery, provides superior QoS for point-to-point or enterprise-level VoIP solutions (see Figure 5.4).

Figure 5.4: VoIP Configuration with IPHighway QoSMasterTM Support.

To deliver business-grade voice services, an enterprise VoIP solution needs to incorporate reliable QoS mechanisms that rival the PSTN.  Policy-based ThruQoS provides these capabilities and is available today from IPHighway.  IPHighway provides first policy-based end-to-end ThruQoS that supports both business-driven traffic prioritisation and guaranteed application bandwidth using industry-standard protocols such as IP Precedence (DiffServ), COPS and RSVP.  IPHighway ThruQoS automatically support VoIP bandwidth requirements by enforcing enterprise-wide policies for handling voice packets traversing the network, even if that network includes multiple WANs.

5.6	Using Packeteer’s PacketShaper™ for Network Analysis

What is running on your WAN? Are you sure? How much of your network bandwidth is wasted delivering retransmitted traffic? Who are the top bandwidth consumers? Which servers have the worst performance records?

PacketShaper gives you the answers to these and many more questions about application performance and network utilisation [16].  These network-analysis features are part of PacketShaper’s four-step approach to comprehensive bandwidth management:

- Discovers and classifies network traffic

- Analyses network behaviour

- Enforces policy-based bandwidth allocation

- Generates reports on utilisation and performance

5.6.1	What is PacketShaper?

PacketShaper is an application-adaptive bandwidth-management solution.  It brings predictable, efficient performance to critical applications running over enterprise WANs and the Internet.  PacketShaper provides insight into and control over congested WAN access links.

When a high-speed LAN narrows into a lower-speed WAN access link, the resulting bottleneck causes delays and inconsistent responses.  PacketShaper typically sits behind WAN link routers to analyse and alleviate these bandwidth bottlenecks.  But if you’re interested in using PacketShaper only for analysis, it can sit anywhere it sees the traffic you’d like it to analyse.

Installation is simply a matter of plugging in two cables.  PacketShaper requires no changes to router configuration, desktops or servers.  A web-based user interface provides access to PacketShaper from any desktop with a web browser.

5.6.2	Discovery and Classification of Traffic Types

Most enterprise networks carry a multitude of traffic types.  This becomes evident just moments after plugging PacketShaper’s cables into the network and turning on Traffic Discovery.  PacketShaper immediately builds a hierarchical traffic classification tree, inserting an entry for every distinct type of traffic it observes.

Detecting and identifying the types of network traffic that contends for limited bandwidth is the first step toward understanding that contention.  Relying on low-level protocols to classify traffic prevents you from discovering your real traffic trends.  Differentiating one application’s traffic from another’s allows you to distinguish its behaviour as well.  In addition, administrators are usually surprised to see the diversity of their own network traffic.

PacketShaper automatically detects and identifies over 150 types of traffic (See some examples in Table 5.1).  You can supply additional criteria to differentiate even more categories.  You can classify traffic based on application, service, protocol, port number, URL or wildcard (for web traffic), user speed, host name, precedence bit, and IP or MAC address.  In addition, you can define criteria to recognise proprietary applications so that PacketShaper automatically classifies their associated traffic.  Each distinct traffic category is called a “traffic class”.

Today’s sophisticated traffic requires sophisticated classification techniques.  The simple IP-address or static-port schemes that routers depend on fall short.  PacketShaper can identify traffic that uses dynamic port assignments, a strong competitive advantage over classifying by only well-known port numbers.  PacketShaper can even track an application as it ‘switches’ among dynamically allocated ports.  For example, one passive FTP session can meander from port to port, confusing other network analysers, which interpret this behaviour as multiple sessions or can’t recognise it as FTP.  PacketShaper tracks a passive FTP session throughout its journey.  More examples of complex traffic types PacketShaper handles include VoIP, Oracle 8, TN3270, and DCOM.  These examples can’t be identified using IP address or static-port schemes.

PacketShaper allows network managers to look even deeper into traffic.  Network managers can isolate each published application running within a Citrix-based environment.  For example, PacketShaper can distinguish Microsoft Word from PeopleSoft applications running over Citrix MetaFrame.  PacketShaper even differentiates among various applications using the same port.  For example, web browsing, HTTP traffic from mission-critical web sites, PointCast, and Peoplesoft e7.5 all run on TCP port 80, but can be individually classified using PacketShaper.

PacketShaper can track hosts by their DNS (Domain Name System) name, even if DHCP (Dynamic Host Configuration Protocol) changes the host’s IP address frequently.  In addition, you can use LDAP (Lightweight Directory Access Protocol) host lists to isolate any traffic associated with a host in the list.

| Client/Server Oracle LotusNotes MS DCOM CORBA SunRPC FileMaker Pro MS SQL FIX | Streaming Media NetShow Real Audio Streamworks RTSP MPEG ST2 | Network Management Protocol ICMP SYSLOG SNMP NTP Cisco Discovery | Routing AURP BGP EGP EIGRP OSPF RIP RARP Spanning Tree CBT PIM IGMP DRP | Push Marimba PointCast Backweb File Server NFS NetBIOS-IP Lockd | E-mail SMTP cc:MAIL POP3 IMAP MSSQ MS DCOM (MS Exchange) | Gaming Systems Doom Quake Quake II Kali |
| --- | --- | --- | --- | --- | --- | --- |
| Internet HTTP FTP SSL NNTP IRC Gopher UUCP SSHTCP IP IPv6 TCP UDP TFTP ActiveX | Voice over IP CUSeeMe H.323 T.120 RTP RTCP VDOPhone Micom VIP I-Phone | Directory Services Finger LDAP DNS RADIUS Kerberos TACACS Whois CRS DHCP DPA Ident | Legacy LAN and Non-IP AppleTalk DECnet IPX SNA FNA LAT NetBEUI AFP | Tunneling Protocol DLS L2TP PPTP IPSEC GRE | Print LPR TN5250p TN3287 | Session Citrix TN3270 TN5250 Telnet Timbuktu RDP Xwindows |

Table 5.1: Traffic Types

The combination of PacketShaper’s automatic application discovery, response-time, measurements, utilisation statistics, and efficiency calculations give you insight with both breadth and depth.  Understanding the content, performance, and behaviour of network traffic leads to better network planning and management.  Although the focus here is on monitoring and analysis, remember that PacketShaper also gives you the opportunity to boost application performance, reduce retransmissions, optimise bandwidth capacity and help avoid expensive bandwidth upgrades.  Better network analysis feeds a more effective bandwidth management strategy and PacketShaper facilitates both.

6	QoS Testing in the VoIP Environment

6.1	VoIP Call Elements

A VoIP call can consist of several elements; endpoints, gateways, some type of packet-switched network and sometimes a circuit switched network.  Which of these elements are present in a particular call depends on what types of endpoints are being used.  An endpoint in a VoIP scenario is either a PC with an Internet Telephone type application installed, an Internet Telephone itself or a regular telephone.  In a conference situation all types of endpoints could be participating.

The difference in what elements are present is determined by where the necessary voice signal processing is done to package the voice for transport over the packet network.  If the endpoints are PC’s or Internet Telephones, the speech encoding and packetising functions are incorporated.  When a standard telephone is at one or both ends of the connection, an interface must be provided between the voice network and the packet network.

IP Telephony Gateways are equipped with standard interfaces to the PSTN (analog, T1/E1) as well as interfaces to the packet network.  The necessary encoding/decoding, compression/decompression and packetising/depacketising are done in between.  The processing of a voice signal into the format necessary for transport over a packet network is performed in all cases by an encoding/decoding subsystem called a vocoder.  These systems encode, compress (usually) and packetise the signal.  When the signal reaches its destination, the process must be reversed by the vocoder on the destination end, either in a gateway or the endpoint itself.  The algorithms used for the vocoder functions can differ from manufacturer to manufacturer, a possible performance variable.  The final element is the packet-switched network itself-the “cloud” that provides the data transport between the other elements.  The network, consisting of various physical media, network protocols and the routers and switches controlling the flow of traffic, is the most problematic of the connection elements, as will be seen in the Section 6.2.  (Note that, while the interface to the PSTN is an important component and should be tested, it does not generally impact voice quality and is not discussed here.)

6.2	What needs to be Tested, Measured and Evaluated?

This section discusses the potential problems VoIP call elements can introduce, usually when trying to perform under heavy traffic demands; connection failure, latency (delay), jitter (variable delays) and dropped packets:

- Connection Failure: The endpoint applications and devices discussed above need to be able to place and receive calls, so this capability needs to be verified.  A gateway needs to be able to receive and send circuit-switched traffic on one side and packet-switched traffic on the other and this basic functionality needs to be verified as well.

- Latency: Voice signals need to be processed for transport over a packet-switched network.  The necessary compression and packetising (and the reverse of these processes) is done either by the intelligent endpoints or a gateway.  Execution of these functions requires a small amount of time, which can vary depending on the architecture of the device (DSPs, compression algorithms) and the amount of traffic to be processed.  This processing time introduces delay, which is called latency.  The human ear, being a subjective evaluator, can tolerate some latency, usually up to around 250 ms, before perceiving a drop in the quality of a connection.  So knowing how much latency an endpoint or gateway introduces, especially when traffic load is high, is important to test in order to ensure the 250 ms threshold is not exceeded.  As it happens, the major portions of the delays are introduced after the packets leave the endpoint or gateway.  Depending on how busy each successive router in the network is, it can introduce another few milliseconds or more into the cumulative latency.  Outside of a carefully managed Intranet, there is no control over the number of router-to-router legs (or hops), a packet takes.  Therefore, monitoring the total end-to-end latency that packets are experiencing is necessary in maintaining a good quality VoIP transmission.

- Jitter: Not only is it impossible to predict or control (using current networks) how many hops packets from a VoIP call will traverse, packets from the same call can be assigned different routes, with varying numbers of hops and different traffic volumes along the way.  Because of this, packets from the same conversation can experience different amounts of delay on their way to their destination.  These variable delays produce a condition called jitter, where packets arrive at their destination at different intervals.  Most gateways have buffers to collect packets and return acceptable continuity to the data, and these must be tuned so that the process itself does not create excessive delay.  So another area of testing would involve monitoring jitter to make sure it is being dealt with effectively.

- Dropped Packets: When a router becomes overloaded with traffic, it may intentionally drop packets to relieve the congestion.  With traditional data traffic, for which these networks are optimised, there are error-checking methods built into the protocols to address these situations and maintain data integrity.  These methods require some overhead not conducive to real-time traffic, and were not implemented for voice transport.  Again, a certain number of missing packets (generally between 1% - 3%, depending on the data represented) can be tolerated by the human ear.  Beyond this, the call quality can degrade to unacceptable levels, so it is important to monitor and test for dropped packets.

6.3	How to Test, Measure and Evaluate

We have identified several conditions, which, if they occur, can negatively impact a user’s perception of the quality of the VoIP transmission; connection failure, latency, jitters and dropped packets.  The failure of a call to connect is an obvious and easily measured call control problem, but the effect the other conditions have on voice quality is more difficult to quantify; how humans perceive an audio signal is very subjective.  Because of this, it is important to closely simulate “real world” conditions so that testing is done on what humans are actually hearing [14].

How then do you create a test environment in which an effective evaluation of the performance of a VoIP device can be done? As stated previously, heavy load conditions are most likely to be the cause of performance degradation.  Unless the testing being done is on a system already in service in the real world, that load must be simulated.  A typical VoIP test system appears to the System Under Test (SUT) as users by generating the same type of traffic that actual users would.  In order to do this, the VoIP test system should have telephony interfaces capable of sending both analog signals (the type of traffic typically offered by home telephones) and digital signals (T1, E1 and ISDN are typically presented by calls from switches).  The amount of load needed to exercise an SUT would determine how many of these interfaces are necessary.

A test scenario would be incomplete without creating end-to-end connectivity in approximation to real world circumstances, which would involve either the same or another test system receiving the traffic and recording information on the sound quality and timely receipt of the calls.  With the physical resources in place to create the traffic load, there must be an easy way for the tester to control it.  VoIP tests should include the following:

- Connection Testing: Calls are generated, the connections between the originating and receiving ends are verified, and the calls are torn down.  Tones may optionally be sent to make sure data can be passed.

- DTMF Testing: Because users may be using VoIP services to access systems that require DTMF inputs (e.g., IVRs), and because DTMF tones are handled differently than speech when processed for IP transport, there should be a test dedicated specifically to checking their integrity.  Several DTMF tones should be played sequentially into the SUT using many telephony interface channels at once and their integrity verified at the receiving end.

- Telephony Load Testing: Varying levels of telephony traffic load should be generated, with the amount of load being “dialable”, perhaps in increasing increments or randomly, on many different channels.  Audio quality should be continuously checked to determine if there is a load level where it begins to decline.  But what is the most meaningful method of assessing audio quality? As previously stated, the true final arbiter of what is acceptable is the human judge.  Recognising the necessity of incorporating the human factor but realising the impracticality of always having a group available to evaluate transmissions, the ITU-T has developed methodology to automate the process.  This methodology is presented in two documents, the P.800 and P.861Recommendations, and is the most well established and fully realised currently existing in this area.

- Mean Opinion Scores (MOS): ITU-T Recommendation P.800 [17] describes the steps necessary to arrive at Mean Opinion Scores (MOS), which represent the benchmark assessment by a group of human judges of the sound quality of speech clips recorded and listened to under specifically controlled parameters.  The clips are sentences chosen not for their meaning, which is somewhat nonsensical, but the range of sounds they encompass.  The sentences are spoken by male and female adult voices, and children’s voices, so that wide assortments of human sounds are represented.  The clips are then played to the judges in a specially designed listening room, where noise and other environmental factors are controlled.  From their ratings on the sound quality of the speech clips, MOS scores of 1 - 5 are derived (with 5 being highest quality).

- Perceptual Speech Quality Measurement (PSQM): ITU-T Recommendation P.861 [18] introduces algorithms that automate the evaluation of sound transmission quality using repeatable, objective calculations that incorporate the necessary subjectivity of the human factor.  This method of analysis is called Perceptual Speech Quality Measurement (PSQM), in which the physical signals that constitute the original source speech clips and encoded speech (speech that has passed through a vocoder) are mapped onto psychophysical speech representations, in other words, how speech is perceived by the human brain.  Taken into account in these representations are weighted factors, to allow for the subjectivity of human perception.  An example of this would be background noise in a transmission (hiss, static) seeming worse during a silent pause than while someone is speaking.  Once the mapping of signals is complete, a “cognitive subtraction” is performed and a quantitative measurement of the results is produced.  It is then possible to algorithmically correlate these results to benchmark MOS scores.  These methods have been incorporated into the Hammer VoIP Test System, and any test system should incorporate an industry-defined and accepted procedure for measuring speech quality that includes recognition of the human factor, in a quantifiable and repeatable way.

Having discussed how to stress a VoIP system with telephony traffic, what is this type of traffics’ relationship to the conditions described previously?  A heavy traffic load is the primary contributor to system and network delay, jitter and dropped packets.  Therefore, an effective test system should be measuring these manifestations of performance degradation while the VoIP system is dealing with telephony load.  In order to detect these conditions, the test system must be able to “sniff” packets on the network with audio content, and understand routing and control information embedded in the packet.  For instance, out of place or missing sequence numbers would indicate a level of jitter or that packets were missing.  By time stamping call events as they synchronised with the clock upon receipt, end-to-end latency could be measured.  As these indications worsened, the effect on voice quality would increase.  It would make sense to incorporate some thresholds-based analysis into the capability of the test system.  In addition, at any time a tester should be able to actually listen to an audio transmission, to gain an understanding of what a ‘real-world’ user would be hearing under a variety of conditions.  What if a test system could actually create latency, jitter, packet loss and packet corruption, as well as create IP traffic load on top of the telephony load? Now the VoIP tester would be able to simulate a larger array of the potential real world circumstances, making the testing environment truly comprehensive.  A company that is serious about providing test systems to the VoIP industry should have these capabilities in their plans.

6.4	Test Methodology

A test methodology is defined in [14] for testing speech quality in TIPHON compliant networks and terminals.  Figure 6.1 shows the methodology used for simulation.

Figure 6.1: Simulation Methodology for Testing TIPHON Speech Quality

A set of speech signals, designed according to ITU-T Recommendation P.800, was used as input to the simulation path.  The simulation path includes the terminal side (electrical part) and the network itself.  The influence of the terminal side was focussed to the speech conversion and IP packet size issue.  The influence of the network side was simulated by different packet loss rates.  After the simulation the speech samples were recorded and stored in a database.  The subjective assessment was carried out according to the ITU-T Recommendation P.800 method.

6.5	Test Cases

The test cases consisted of a group of single codec scenarios (references), phone-to-phone scenarios in fixed network environments and a group of tandeming conditions.  The tandeming conditions were based on real scenarios where a mobile customer is connected to an ordinary telephone via IP.  For these cases, the GSM Full-Rate Codec (GSM-FR) and the GSM Enhanced Full-Rate Codec (GSM-EFR) were used.  In such scenarios, mainly the influence of the IP network was taken into account.  Only one condition was chosen to simulate a voice transmission from a mobile phone to an ordinary telephone via an impaired radio channel and via an IP network with packet loss.

| Codec | Packet Loss | Speech Frame Size | No. of Frames / Packet | Substitution |
| --- | --- | --- | --- | --- |
| Single Codec |  |  |  |  |
| G.711 | 5%, 10%, 15%, 20% | 0,125 ms | 80, 320, 480, 800 | Silence |
| G.729B | 5% | 10 ms | 1, 4, 6, 10 | G.729 internal |
| G.723 (5.3) | 0%, 5%, 10%, 15%, 20% | 30 ms | 1, 2, 3 | G.723 internal |
| G.728 | 0%, 5%, 10%, 15%, 20% | 0,625 ms | 16, 64, 96, 160 | Proprietary |

Table 6.1: G.711+ Codec + G.711

(Phone-to-Phone Scenario in Fixed Network Environments)

Figure 6.2: Processing Scenario

| Tandem with GSM-FR | Packet Loss | Speech Frame Size | No. of Frames / Packet | Substitution |
| --- | --- | --- | --- | --- |
| GSM-FR + G.723 (5.3) | 0%, 5%, 10%, 15% | G.723: 30 ms | 2 | G.723 internal |
| GSM-FR + G.729B | 0%, 5%, 10%, 15% | G.729: 10 ms | 6 | G.729 internal |
| GSM-EFR + G.723(5.3) | 0%, 5%, 10% | G.723: 30 ms | 2 | G.723-internal |
| GSM-EFR + G.729B | 0%, 5%, 10% | G.729: 10 ms | 6 | G.729-internal |
| GSM-EFR EP2 + G.723(5.3) | 0%, 5%, 10% | G.723: 30 ms | 2 | G.723-internal |

Table 6.2: Tandem Configuration with GSMxFR

(Phone-to-Phone Scenario including Mobile Networks)

Figure 6.3: Simulated Tandem Processing Scenario

6.6	What is Pathchar?

‘Pathchar’ [15] is a special tool from Van Jacobson, (IETF), developed in 1997.  It helps to find path characteristics and it is similar to ‘traceroute’ but designed to isolate congestion and loss problems rather than routing strangeness.  It determines the bandwidth, propagation delay, queue and loss rate for each hop on an Internet path and the optimal TCP window size to match the performance of a path (especially important for inter-continental links).  It uses the same mechanism as ‘traceroute’, (i.e., modulates the TTL (Time To Live) and looks for ICMP (Internet Control Message Protocol) “time exceeded” messages) and works wherever ‘traceroute’ works.  This tool is often used to determine bottlenecks.

6.6.1	Understanding Pathchar

Pathchar sends packets with different packet sizes from hop to hop and measures the response times.  By default, Pathchar is:

“doing 32 probes at each of 45 sizes (64 to 1500 by 32 )”

which means that for all intermediate hops of the route, each hop sends 32 packets of 64 bytes to the next hop, then again 32 packets of 64 + 32 = 96 bytes to this next hop, etc., up to 32 packets of 1500 bytes.

Thus it measures the bandwidth of each hop-to-hop connection and sends the bandwidth and some other information back to the source host.  For calculating the bandwidth, the program computes the gradient (response times/packet sizes) and smoothes these gradients with complex statistical mathematics.  Problems are sometimes non-linearities of the gradients, then the output is marked as “?? Mbps”.

6.6.2	Bandwidth Tests with Pathchar

This section contains some bandwidth tests to demonstrate the effect of packet loss on the public Internet.  The TIPHON-Net [11] contains several VoIP Gateways and can be used to demonstrate VoIP over the public Internet (see Figure 6.4).  Tests with a tool called Pathchar show packet loss and latency between several TIPHON VoIP gateways.

Figure 6.4: TIPHON-Net

6.6.3	Pathchar Execution Output Descriptions

Input: pathchar <host>

Per-Link Output (especially from hop 3 to 4)

The lines starting with ‘|’ are the characteristics of the link between the hops listed above and below.  The first number is the ‘link bandwidth’, the 2nd is the ‘one-way propagation time’, (i.e., for a zero length packet) and the number in brackets is the ‘Round Trip Time (RTT)’ you would get if there were no queues and you sent a path Maximum Transmission Unit (MTU) sized packet from the source to this hop, (i.e., the unloaded data RTT).  If there is a queue larger than 1 packet, its ‘size’ in both time and bytes is printed.  If the drop rate is more than 1%, it is printed.  ‘hinge’ is ratio of the Inter-Quartile Distance (IQQ) to the median (this ratio is the robust statistics equivalent of the standard deviation, is the equivalent of the standard error of the mean).  For normal ‘statistical’ fluctuations, it should be around 1, and Pathchar prints it if it’s larger.  In this case, it implies that at least 25% of the probes saw a queue larger than 16ms; statistically it’s essentially the same as having 1/4 of the data more than 13 standard deviations from the mean (unusual, perhaps some bimodal delay process due to router idiosyncrasy).

Summary output:

If at a given node, Pathchar prints something like

9:   6  1208  28     102       ->206.34.78.27 (54358)

This is a multipath output.  (Some packets try different hops.)  Pathchar prints that hop 9 tries 54358 packets to 206.34.78.27 in this example.

6.6.4	Terminal Output of Pathchar from France to Austria

The following is a terminal output of Pathchar from the ETSI VoIP Gateway in Sophia Antipolis, France to the Telekom Austria VoIP Gateway in Vienna, Austria. Pathchar always begins sending of 32 probes with different packet sizes from one hop to the next hop.  The results show a minimum of 10 % packet loss from each hop to the next hop on the public Internet.

pathchar to 195.3.82.115 (195.3.82.115)

doing 32 probes at each of 45 sizes (64 to 1500 by 32)

0 localhost

|   4.3 Mb/s,   340 us (3.49 ms),       9% dropped

1 195.101.208.62 (195.101.208.62)

|    504 Kb/s,   4.04 ms (35.4 ms),  10% dropped

2 marseille1.ESPLANADE3000.NET (195.115.3.129)

|    956 Kb/s,   7.80 ms (63.5 ms),  12% dropped

3 nnc2.ESPLANADE3000.NET (195.115.0.133)

-> 195.115.0.40 (1829)

|    31 Mb/s,   71 us (64.1 ms),        25% dropped

4?nnc1.ESPLANADE3000.NET (195.115.0.10)

|    31 Mb/s,   49.6 ms (164 ms),    10% dropped

5 cegeglobe.ESPLANADE3000.NET (195.115.0.86)

-> 204.59.18.193 (6)

|    57 Mb/s,   499 us (165 ms),  11% dropped

6?Teleglobe.NET (207.45.201.129)

-> 207.45.223.41 (16)

|    93 Mb/s,   6.89 ms (179 ms),       12% dropped

7?gin-mtt-bb2.Teleglobe.NET (207.45.223.61)

-> 207.45.206.162 (9)

|    68 Mb/s,   7.51 ms (194 ms),      12% dropped

8?gin-ppt-bb2.Teleglobe.NET (207.45.223.41)

|    34 Mb/s,   43.7 ms (282 ms),  +q 1.79 ms (7.51 KB),  6% dropped

9 PTAustia-gw.Teleglobe.NET (207.45.206.162)

10  *

-> 195.3.82.115 (19)

|   1.5 Mb/s,   1.25 ms (292 ms),  14% dropped

11?172.16.94.110 (172.16.94.110)

-> 62.156.131.109 (1)

|    16 Mb/s,   -484 us (292 ms),  +q 15.7 ms (31.2 KB) *3,  19% dropped

12?gw1.imm.AON.at (195.3.82.115)

12 hops, rtt 242 ms (292 ms), bottleneck  504 Kb/s, pipe 18392 bytes

6.6.5	Terminal Output of Pathchar from France to Germany

The following is a terminal output of Pathchar from the ETSI VoIP Gateway in Sophia Antipolis, France to the Deutsche Telekom VoIP Gateway in Berlin, Germany.  Pathchar always begins sending of 32 probes with different packet sizes from one hop to the next hop.  The results show a minimum of 10 % packet loss from each hop to the next hop on the public Internet.

pathchar to 141.39.12.150 (141.39.12.150)

doing 32 probes at each of 45 sizes (64 to 1500 by 32)

0 localhost

|   4.3 Mb/s,   353 us (3.51 ms),  12% dropped

1 195.101.208.62 (195.101.208.62)

|    201 Kb/s,   5.44 ms (74.1 ms),  12% dropped

2 212.234.242.29 (212.234.242.29)

-> 194.250.4.50 (54)

|   9.6 Mb/s,   -14 us (75.4 ms),  +q 2.05 ms (2.46 KB) *2,  11% dropped

3?Marseille.rain.fr (194.250.4.49)

-> 194.250.181.205 (792)

|   1.9 Mb/s,   8.36 ms (98.5 ms),  18% dropped

4?DAR2-s-4-1-3.rain.fr (194.250.2.5)

|    30 Mb/s,   174 us (99.2 ms),  +q 4.60 ms (17.4 KB) *2,  11% dropped

5 rbs2.rain.fr (194.51.0.152)

|    11 Mb/s,   344 us (101 ms),  +q 28.3 ms (39.5 KB) *2,  13% dropped

6 gip-raspail-1-fastethernet0-0.gip.NET (204.59.18.193)

|   4.6 Mb/s,   14.0 ms (132 ms),  +q 111 ms (63.9 KB),  15% dropped

7 paris5.att-unisource.NET (195.206.66.221)

-> 195.206.66.221 (29)

|   5.9 Mb/s,   -3504 us (127 ms),  +q 30.9 ms (22.9 KB) *2,  11% dropped

8?195.206.64.57 (195.206.64.57)

|   8.5 Mb/s,   -847 us (126 ms),  +q 12.4 ms (13.2 KB) *2,  5% dropped

9 amsterdam5.att-unisource.NET (195.206.65.89)

-> 195.206.65.89 (2)

-> 195.206.64.22 (207)

|    19 Mb/s,   5.70 ms (138 ms),  +q 56.4 ms (136 KB),  12% dropped

10?frankfurt5.att-unisource.NET (195.206.64.18)

|   202 Mb/s,   -169 us (138 ms),  +q 9.60 ms (242 KB) *5,  12% dropped

11 422.ATM11-0-0.CR2.NYC2.ALTER.NET (137.39.31.101)

-> 195.206.64.42 (5)

|    17 Mb/s,   -13 us (139 ms),  +q 4.80 ms (9.92 KB) *2,  12% dropped

12?DE.ten-34.NET (195.206.64.62)

-> 195.206.64.62 (2)

|   ?? b/s,   1.82 ms (141 ms),  11% dropped

13?ir-frankfurt1.win-ip.dfn.de (193.203.227.18)

-> 193.203.227.18 (3)

|    28 Mb/s,   -302 us (141 ms),  +q 5.54 ms (19.2 KB) *2,  12% dropped

14?ZR-Frankfurt1.WiN-IP.DFN.DE (188.1.144.98)

-> 188.1.144.98 (1)

|   ?? b/s,   4.51 ms (150 ms),  15% dropped

15?ZR-Berlin1.win-ip.dfn.de (188.1.144.105)

-> 188.1.144.105 (2)

-> 188.1.1.106 (210)

-> 141.39.12.150 (19)

|   ?? b/s,   79.0 ms (305 ms),  37% dropped

16?FU-Berlin1.win-ip.dfn.de (188.1.162.10)

-> 141.39.12.150 (17)

-> 188.1.162.10 (7)

-> 194.250.4.50 (3)

|   5.4 Mb/s,   9.80 ms (326 ms),  +q 85.7 ms (58.0 KB),  40% dropped

17?KR-DeTeBerkom.win-ip.dfn.de (188.1.1.106)

-> 188.1.1.106 (8)

|   3.2 Mb/s,   5.65 ms (342 ms),  +q 105 ms (41.3 KB),  37% dropped

18?orion.berkom.de (141.39.12.150)

18 hops, rtt 261 ms (342 ms), bottleneck  201 Kb/s, pipe 8695 bytes

References

| [1] | Steven Blake, David Black, Mark Carlson, Elwyn Davies, Zeng Wang, Walter Weiss, “An Architecture for Differentiated Services”, draft-ietf-diffserv-arch-02.txt |
| --- | --- |
| [2] | John Wroclawski; “Differential Service for the Internet”, http://diffserv.lcs.mit.edu |
| [3] | K. Nichols, S. Blake, F. Baker, D. Black; “Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers”, <draft-ietf-diffserv-header-02.txt>, August 1998 |
| [4] | ATM Forum Technical Committee, Baseline Text for MPOA, af-95-0824r9, September 1996 |
| [5] | RFC2105, “Cisco Systems’ Tag Switching Architecture Overview”, Y. Rekhter, B. Davie, D. Katz, E. Rosen, G. Swallow, February 1997 |
| [6] | RFC2205, “Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification”, R. Braden (ed.), L. Zhang, S. Berson, S. Herzog, S. Jamin, September 1997 |
| [7] | RFC791, “Internet Protocol – DARPA Internet Program Protocol Specification”, Jon Postel (ed.), September 1981 |
| [8] | R. Yavatkar, D. Hoffman, Y. Bernet, F. Baker, M. Speer, “SBM (Subnet Bandwidth Manager): A Protocol for RSVP-based Admission Control over IEEE 802-style networks”, Internet-Draft, November 1998 |
| [9] | White Paper: Introduction to QoS Policies www.qosforum.com |
| [10] | www.iphighway.com |
| [11] | www.etsi.org/tiphon |
| [12] | ITU-T Recommendation H.323, (Feb. 98), Packet-based multimedia communications systems |
| [13] | [RFC 1889], “RTP: A Transport Protocol for Real-Time Applications”, H. Shulzrinne, S. Casner, R. Frederick, V. Jacobson, January 1996 |
| [14] | TIPHON WG5:DTR/TIPHON 05001 V3 |
| [15] | Pathchar ftp://ftp.ee.lbl.gov/pathchar |
| [16] | www.packeteer.com |
| [17] | ITU-T Recommendation P.800, (Aug. 96), Methods for subjective determination of transmission quality |
| [18] | ITU-T Recommendation P.861, (Feb. 98), Objective quality measurement of telephone-band (300-3400 Hz) speech codecs |