|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.872 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Draft 1.2 (03/00) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Optical transport networks

Architecture of optical transport networks

This Draft was produced during the SG13 meeting in 
Kyoto (28 February – 10 March 2000)

ITU-T  Recommendation  G.872

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.872 (Draft 1.2 03/00) architecture of optical transport networks

Summary This Recommendation describes the functional architecture of optical transport networks using the modelling methodology described in Recommendation G.805. The optical transport network functionality is described from a network level viewpoint, taking into account an optical network layered structure, client characteristic information, client/server layer associations, networking topology, and layer network functionality providing optical signal transmission, multiplexing, routing, supervision, performance assessment, and network survivability.

Source ITU-T Recommendation G.872 was prepared by ITU-T Study Group 13 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of February 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope	4

2	References	4

3	Terms and definitions	5

4	Abbreviations	6

5	Transport functional architecture of optical networks	7

5.1	General principles	7

5.2	Optical transport network layered structure	8

5.3	Optical channel layer network	9

5.3.1	Optical channel trail termination	10

5.3.2	OCh transport entities	12

5.4	Optical multiplex section layer network	12

5.4.1	Optical multiplex section trail termination	13

5.4.2	OMS transport entities	13

5.5	Optical transmission section layer network	13

5.5.1	Optical transmission section trail termination	14

5.5.2	OTS transport entities	15

5.6	Client/server associations	15

5.6.1	OCh/client adaptation	15

5.6.2	OMS/OCh adaptation	16

5.6.3	OTS/OMS adaptation	16

5.7	Optical network topology	16

5.7.1	Unidirectional and bidirectional connections and trails	16

5.7.2	Point-to-multipoint connections and trails	17

6	Subdividing of the Optical Transport Network	17

6.1	Subdividing into domains	17

6.2	Subdividing of domains	21

7	Optical network management	21

7.1	Generic requirements	21

7.1.1	Generic fault, configuration and performance management	21

7.1.2	Generic management communications	22

7.1.3	Generic client/server interaction management	22

7.2	Optical layer network management requirements	22

7.2.1	Connection supervision	24

7.2.2	Signal quality supervision	26

7.2.3	Adaptation management	26

7.2.4	Protection control	26

7.2.5	Management communications	26

7.2.6	Other communication needs	27

7.3	Connection supervision techniques 	27

7.3.1	Inherent monitoring	27

7.3.2	Non-intrusive monitoring	27

7.3.3	Intrusive monitoring	28

7.3.4	Sublayer monitoring	28

7.4 	Connection supervision applications	29

7.4.1	Monitoring of unused connections	29

7.4.2	Connection monitoring	29

8	Optical network survivability techniques	31

8.1	Protection	31

8.1.1	Trail protection	31

8.1.2	Subnetwork connection protection	32

8.1.3	1+1 OCh SNCP	34

8.1.4	OCh shared protection ring	34

8.1.5	OMS shared protection ring	34

8.2	Network restoration	35

9	Interconnection and interworking between different administrative domains	35

Annex A	Impairment mitigation and regeneration	36

Appendix I		Examples of Optical Network functionality	38

I.1		Wavelength conversion	41

I.2	Cross-connect	41

I.3	Regeneration	41

Recommendation G.872

ARCHITECTURE OF OPTICAL TRANSPORT NETWORKS

(Geneva, 1999)

# 1	Scope

This Recommendation is restricted to the functional description of optical transport networks that support digital signals. The support of analogue or mixed digital/analogue signals is outside of the current scope.

It is recognized that the design of optical networks is subject to limitations imposed by the accumulation of degradations introduced by the number of network elements and their network topology. However, many of these degradations and the magnitude of their effects are associated with particular technological implementations of the architecture described in this Recommendation and are therefore subject to change as technology progresses. As such the description of these effects is outside the scope of this Recommendation.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation G.652 (1997), Characteristics of a single-mode optical fibre cable.

–	ITU-T Recommendation G.653 (1997), Characteristics of a dispersion-shifted single-mode optical fibre cable.

–	ITU-T Recommendation G.655 (1996), Characterization of a non-zero dispersion shifted single-mode optical fibre cable.

–	ITU-T Recommendation G.681 (1996), Functional characteristics of interoffice and long-haul line systems using optical amplifiers, including optical multiplexing.

–	ITU-T Recommendation G.707 (1996), Network node interface for the synchronous digital hierarchy (SDH).

–	ITU-T Recommendation G.803 (1997), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

–	ITU-T Recommendation G.805 (1995), Generic functional architecture of transport networks.

–	ITU-T Recommendation G.957 (1995), Optical interfaces for equipments and systems relating to the synchronous digital hierarchy.

–	ITU-T Recommendation I.326 (1995), Functional architecture of transport networks based on ATM.

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	adaptation management: The set of processes for managing client layer network adaptation to/from the server layer network.

3.2	connection supervision: The set of processes for monitoring the integrity of a connection which is part of a trail. This set consists of the processes associated with connectivity and continuity supervision.

3.3	connectivity supervision: The set of processes for monitoring the integrity of the routing of the connection between source and sink trail terminations.

3.4	continuity supervision: The set of processes for monitoring the integrity of the continuity of a trail.

3.5	Inter-Domain Interface (IrDI): A physical interface that represents the boundary between two administrative domains.

3.6	Intra-Domain Interface (IaDI): A physical interface within an administrative domain.

3.7	maintenance indication: The set of processes for indicating defects in a connection which is part of a trail in downstream and upstream directions.

3.8	management communications: The set of processes providing communications for management purposes.

3.9	OTN compliant interface: An interface for the optical transport network based on the architecture defined in this Recommendation (G.872).

3.10	OTN non-compliant interface: An interface that does not comply to the interface recommendations that will be defined for the optical transport network based on the architecture defined in this Recommendation (G.872).

3.11	overhead information: Six types of overhead information are defined:

1)	Trail Termination Overhead Information is the information generated by the trail termination source and extracted by the trail termination sink to monitor the trail. This overhead information is specific to a layer network and is independent of any client/server relationship between network layers.

2)	Client-Specific Overhead Information is associated with a particular client/server relationship and is therefore processed by a particular adaptation function.

3)	Auxiliary Channel Overhead Information is information that may be transferred by an optical network layer but which does not by necessity have to be associated with a particular connection. An example of such an auxiliary channel is a data communications channel for the purposes of transferring management data between management entities.

NOTE – These management entities are not trail termination and adaptation functions.

4)	Reserved Overhead Information for national use.

5)	Unassigned Overhead Information. This overhead may be of types 1, 2, 3 or 4 as defined above.

6)	Network Operator-Specific Overhead Information that may be used by an operator to support its unique optical networking needs and/or for service differentiation. The information content is not standardized.

3.12	optical transport network: A transport network bounded by optical channel access points.

3.13	Optical Supervisory Channel (OSC): The optical supervisory channel is an optical carrier that transfers overhead information between optical transmission section transport entities. The optical supervisory channel supports more than one type of overhead information and some of this overhead information may be used by one or more transport network layers.

3.14	protection control: The information and set of processes for providing control of protection switching for a trail or subnetwork connection.

3.15	signal quality supervision: The set of processes for monitoring the performance of a connection that is supporting a trail.

3.16	subnetwork connection supervision: The set of processes providing connectivity supervision and/or continuity supervision and/or signal quality supervision for a subnetwork connection that is supporting a trail.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AP	Access point (see Recommendation G.805)

APS	Automatic protection switching

ATM	Asynchronous transfer mode (see Recommendation I.326)

BDI	Backward defect indication

CP	Connection point (see Recommendation G.805)

FDI	Forward defect indication

IaDI	Intra-Domain Interface

IrDI	Inter-Domain Interface

LOC	Loss of continuity

MPCP	Multipoint connection point

NE	Network element

NRZ	Non return to zero

OCh	Optical channel

OCh/Client_A	Optical channel/client adaptation

OCh_LC	Optical channel link connection

OCh_NC	Optical channel network connection

OCh_SN	Optical channel subnetwork

OCh_SNC	Optical channel subnetwork connection

OCh_TT	Optical channel trail termination

OMS	Optical multiplex section

OMSn	Optical multiplex section of order n

OMS/OCh_A	Optical multiplex section/Optical channel adaptation

OMS_LC	Optical multiplex section link connection

OMS_NC	Optical multiplex section network connection

OMS_TT	Optical multiplex section trail termination

OSC	Optical supervisory channel

OTM	Optical transport module

OTMn	Optical transport module of order n

OTN	Optical transport network

OTS	Optical transmission section

OTSn	Optical transmission section of order n

OTS/OMS_A	Optical transmission section/Optical multiplex section adaptation

OTS_LC	Optical transmission section link connection

OTS_NC	Optical transmission section network connection

OTS_SN	Optical transmission section subnetwork

OTS_SNC	Optical transmission section subnetwork connection

OTS_TT	Optical transmission section trail termination

OTU	Optical transport unit

OTUGn	Optical transport unit group of order n

PDH	Plesiochronous digital hierarchy

PTI	Payload type identifier

RS	Regenerator section (see Recommendation G.803)

SDH	Synchronous digital hierarchy (see Recommendation G.707)

SNC	Subnetwork connection (see Recommendation G.805)

SNC/I	Subnetwork connection protection with inherent monitoring

SNC/N 	Subnetwork connection protection with non-intrusive monitoring

STM-N	Synchronous transport module level N (see Recommendation G.707)

TCP	Termination connection point (see Recommendation G.805)

TDM	Time division multiplexing

WDM	Wavelength division multiplexing

# 5	Transport functional architecture of optical networks

## 5.1	General principles

Optical networks are comprised of functionality providing transport, multiplexing, routing, supervision and survivability of client signals that are processed predominantly in the photonic domain. This functionality for optical networks is described from a network level viewpoint using the generic principles defined in Recommendation G.805. The specific aspects concerning the optical transport network layered structure, characteristic information, client/server layer associations, network topology, and layer network functionality are provided in this Recommendation. This

Recommendation uses the terminology, functional architecture and diagrammatic conventions defined in Recommendation G.805.

In accordance with Recommendation G.805, the optical transport network is decomposed into independent transport layer networks where each layer network can be separately partitioned in a way which reflects the internal structure of that layer network.

In the following functional description, optical signals are characterized by wavelength (or central frequency) and may be processed per wavelength or as a wavelength division multiplexed group of wavelengths. The functional description of other optical multiplexing techniques (e.g. optical code division multiplexing) in optical networks is for future study.

## 5.2	Optical transport network layered structure

The optical transport network layered structure is comprised of the optical channel, optical multiplex section and optical transmission section layer networks, as illustrated in Figure 1. Motivation for this three-layer structure is as follows:

Optical channel layer network: This layer network provides end-to-end networking of optical channels for transparently conveying client information of varying format (e.g. SDH STM-N, PDH 565 Mbit/s, cell based ATM, etc.). The description of supported client layer networks is outside the scope of this Recommendation. To provide end-to-end networking, the following capabilities are included in the layer network:

–	optical channel connection rearrangement for flexible network routing;

–	optical channel overhead processes for ensuring integrity of the optical channel adapted information;

–	optical channel supervisory functions for enabling network level operations and management functions, such as connection provisioning, quality of service parameter exchange and network survivability.

Optical multiplex section layer network: This layer network provides functionality for networking of a multi-wavelength optical signal. Note that a "multi-wavelength" signal includes the case of just one optical channel. The capabilities of this layer network include:

–	optical multiplex section overhead processes for ensuring integrity of the multi-wavelength optical multiplex section adapted information;

–	optical multiplex section supervisory functions for enabling section level operations and management functions, such as multiplex section survivability.

These networking capabilities performed for multi-wavelength optical signals provide support for operation and management of optical networks.

Optical transmission section layer network: This layer network provides functionality for transmission of optical signals on optical media of various types (e.g. G.652, G.653 and G.655 fibre).

The capabilities of this layer network include:

–	optical transmission section overhead processing for ensuring integrity of the optical transmission section adapted information;

–	optical transmission section supervisory functions for enabling section level operations and management functions, such as transmission section survivability.

Physical media layer network: The physical media layer network for an optical network is a defined optical fibre type. This physical media layer network is the server of the optical transmission section. The detailed description of this layer is outside the scope of this Recommendation.

The detailed functional description of the optical layer networks is given in the following subclause.

Figure 1/G.872 – Client server associations in an optical transport network

## 5.3	Optical channel layer network

The optical channel layer network provides for the transport of digital client signals through an optical channel trail between access points. The characteristic information of an optical channel layer network is composed of two separate and distinct logical signals:

–	a data stream that constitutes the adapted information of a client layer network;

–	a data stream that constitutes the optical channel trail termination overhead.

This characteristic information is an Optical Transport Unit (OTU). The optical channel layer network contains the following transport functions and transport entities (see Figure 2):

–	Optical channel trail;

–	Optical channel trail termination source (OCh_TT_Source);

–	Optical channel trail termination sink (OCh_TT _Sink);

–	Optical channel network connection (OCh_NC);

–	Optical channel link connection (OCh_LC);

–	Optical channel subnetwork (OCh_SN);

–	Optical channel subnetwork connection (OCh_SNC).

Figure 2/G.872 – OCh layer network example

### 5.3.1	Optical channel trail termination

The following generic processes may be assigned to the optical channel trail termination:

–	validation of connectivity integrity;

–	assessment of transmission quality;

–	transmission defect detection and indication.

The requirement for these processes is outlined in detail in 6.2.

There are three types of optical channel trail termination:

–	Optical channel bidirectional trail termination: consists of a pair of co-located optical channel trail termination source and sink functions.

–	Optical channel trail termination source: accepts adapted information from a client layer network at its input, inserts the optical channel trail termination overhead as a separate and distinct logical data stream and presents the characteristic information of the optical channel layer network at its output.

–	Optical channel trail termination sink: accepts the characteristic information of the optical channel layer network at its input, extracts the separate and distinct logical data stream containing the optical channel trail termination overhead and presents the adapted information at its output.

The optical channel is subdivided into sublayers.

The description used in this section is based on work done for ATM (in I.732) and models sublayers as optional functionality that may or not be present at any particular location. Of course the “balance” of the model is preserved in that functions may only be present or absent in pairs of Source and Sinks (though these pairs need not be located in the same equipment).

A possible set of OCh atomic functions, their ordering and applications is depicted in Figure 2a.

- "OChP" refers to the end to end OCh-Path,

- "OChT" refers to the OCh-Tandem Connection sublayer used for administrative and protected domain monitoring,

- "OChF" refers to the FEC sublayer, while

- “OChR” refers to the OCh Regenerator section sublayer used to monitor the links between 3R regeneration points.

The non intrusive monitors are represented by "OChPm" and "OChTm".

Editors Note: Q19/13 has currently no agreement on the overall model for the OCh layer network and the OCh characteristic information. §5.3 has to be updated accordingly when agreement on these issues has reached. Contributions are requested

Figure 2a/G.872:	OCh atomic functions and associated applications

Notes to the figure:

- The sublayer functions are depicted with a connection across the function, indicating optionality. A network element does not need to support all depicted functions, though certain combinations of function are not possible.

- OCh FEC functions and OCh Tandem Connection (i.e. domain monitoring) functions may be present at multiple locations in the logical flow of processing. These functions may be either “in line” in which case they can be active or bypassed, or they can be connected via the connection function only when required. The former case is typically chosen when all interfaces are required to support the functionality. The latter case is useful when a restricted number of signals (out of all interface units) need to have the function.

- The management requirements for each of the sublayers are detailed in Table 1/G.872.

### 5.3.2	OCh transport entities

Network connections, link connections and trails are as described in Recommendation G.805.

The OCh subnetwork, OCh_SN, provides flexibility within the optical channel layer. Characteristic information is routed between input (termination) connection points [(T)CPs] and output (T)CPs. The connection function may be used by the network operator to provide routing, grooming, protection and restoration.

## 5.4	Optical multiplex section layer network

The optical multiplex section layer network provides the transport of optical channels through an optical multiplex section trail between access points. The characteristic information of an optical multiplex section layer network is composed of two separate and distinct logical signals:

–	a data stream that constitutes the adapted information of the optical channel layer. The data stream contains a set of n optical channels which taken as a set have a defined aggregate optical bandwidth;

–	a data stream that constitutes the optical multiplex section trail termination overhead.

Each channel has a defined carrier wavelength (frequency) and optical bandwidth (the supported optical channel bandwidth plus source stability). Individual optical channels within an optical multiplex may be either in-service or out-of-service. Out-of-service channels are either lit or unlit.

The characteristic information of the optical multiplex section is an Optical Transport Unit Group of order n (OTUGn).

The optical multiplex section layer network contains the following transport functions and transport entities (see Figure 3):

–	OMS trail;

–	OMS trail termination source (OMS_TT_Source);

–	OMS trail termination sink (OMS_TT_Sink);

–	OMS network connection (OMS_NC);

–	OMS link connection (OMS_LC).

Figure 3/G.872 – OMS layer network example

### 5.4.1	Optical multiplex section trail termination

The following generic termination processes may be assigned to the optical multiplex section trail termination:

–	assessment of transmission quality;

–	transmission defect detection and indication.

The requirement for these processes is outlined in detail in 7.2.

There are three types of optical multiplex section trail termination:

–	OMS bidirectional trail termination: consists of a pair of co-located optical multiplex section termination source and sink functions.

–	Optical multiplex section trail termination source: accepts adapted information from the optical channel layer network at its input, inserts the OMS trail termination overhead and presents the characteristic information of the OMS layer network at its output.

–	Optical multiplex section trail termination sink: accepts the characteristic information of the OMS layer network at its input, extracts the OMS overhead and presents the adapted information at its output.

### 5.4.2	OMS transport entities

Network connections, link connections and trails are as described in Recommendation G.805. There is no OMS subnetwork defined, as there is no flexibility in this layer network.

## 5.5	Optical transmission section layer network

The optical transmission section layer network provides for the transport of an optical multiplex section through an optical transmission section trail between access points. An optical transmission section of order n supports a single instance of an optical multiplex section of the same order. There is a one-to-one mapping between the two layers. The OTS defines a physical interface, with optical parameters such as frequency, power level and signal-to-noise ratio. The characteristic information of the OTS is composed of two separate and distinct logical signals:

–	the adapted information of the OMS layer;

–	the OTS trail termination-specific management/maintenance overhead.

Physically it consists of the following.

–	an optical multiplex of order n;

–	an optical supervisory channel.

This characteristic information is an Optical Transport Module of order n (OTMn).

NOTE – In the case of an OTS-1 in a system without back-to-back OTS terminations, or in the case of an OTS-1 used as an OTN_IrDI (see clause 8), alternatives to an OSC for carrying overhead information are for further study.

The OTS layer network contains the following transport functions and transport entities (see Figure 4):

–	OTS trail

–	OTS trail termination source (OTS_TT_Source)

–	OTS trail termination sink (OTS_TT_Sink)

–	OTS network connection (OTS_NC)

–	OTS link connection (OTS_LC)

–	OTS subnetwork (OTS_SN)

–	OTS subnetwork connection (OTS_SNC).

NOTE – OTS_SN and OTS_SNC exist only in the case of OTS 1+1 NC protection.

Figure 4/G.872 – OTS layer network example

### 5.5.1	Optical transmission section trail termination

The following generic processes may be assigned to the optical transmission trail termination:

–	validation of connectivity;

–	assessment of transmission quality;

–	transmission defect detection and indication.

The means of providing these processes is described in 7.2.

There are three types of optical transmission section trail termination:

–	OTS bidirectional trail termination: consists of a pair of co-located optical transmission section trail termination source and sink functions.

–	OTS trail termination source: accepts adapted information from a client layer network at its input, adds the OTS trail termination overhead and generates the optical supervisory channel, and adds the optical supervisory channel to the main signal. The trail termination function conditions the information for transmission over the physical medium and ensures that the optical signal meets the physical interface requirements. The output of the OTS trail termination source is the characteristic information of the optical transmission section layer network. This characteristic information is referred to as an optical transport module (OTM).

–	OTS trail termination sink: accepts the characteristic information of the transmission section layer network at its input, reconditions the information to compensate for signal degradation resulting from transmission over the physical medium, extracts the optical supervisory channel from the main optical signal, processes the OTS trail termination overhead contained within the optical supervisory channel and presents the adapted information at its output.

### 5.5.2	OTS transport entities

Network connections, link connections and trails are as described in Recommendation G.805.

The OTS subnetwork, OTS_SN, provides NC protection, within the optical transmission section layer. Characteristic information is routed between input (termination) connection points [(T)CPs] and output (T)CPs.

## 5.6	Client/server associations

A principal feature of optical transport networks is the possibility of supporting a wide variety of client layer networks. Examples of these client layer networks include an SDH STM-N, and a contiguous ATM cell stream. Restrictions or rules that limit the capability of an optical channel to transfer a particular client layer network are for further study.

The structure of the optical layer networks and the adaptation functions are shown in Figure 1. For the purposes of description of the optical transport network, the interlayer adaptation is named using the server/client relationship.

### 5.6.1	OCh/client adaptation

The OCh/Client adaptation (OCh/Client_A) is considered to consist of two types of processes: client-specific processes and server-specific processes. The description of the client-specific processes is outside the scope of this Recommendation.

The bidirectional OCh/Client adaptation (OCh/Client_A) function is performed by a co-located pair of source and sink OCh/Client adaptation functions.

The OCh/Client adaptation source (OCh/Client_A_So) performs the following processes between its input and its output:

–	all the processing required to generate a continuous data stream that can be modulated onto an optical frequency carrier. The processes required are dependent upon the particular client/server relationship and may be null. For a digital client the adaptation may include processing such as scrambling and channel coding (e.g. NRZ). For a digital mapping the adapted information is a continuous data stream of defined bit rate and coding scheme;

–	generation and termination of management/maintenance signals as described in 6.2.

The OCh/Client adaptation sink (OCh/Client_A_Sk) performs the following processes between its input and its output:

–	recovery of the client signal from the continuous data stream. The processes are dependent upon the particular client/server relationship and can be null. For a digital client the adaptation may include processes such as timing recovery, decoding and descrambling;

–	generation and termination of management/maintenance signals as described in 6.2.

### 5.6.2	OMS/OCh adaptation

The bidirectional OMS/OCh adaptation (OMS/OCh_A) function is performed by a co-located pair of source and sink OMS/OCh adaptation functions.

The OMS/OCh adaptation source (OMS/OCh_A_So) performs the following processes between its input and its output:

–	modulation of an optical carrier by the optical transport unit signal by means of a defined modulation scheme;

–	wavelength (or frequency) and power allocation to the optical carrier;

–	optical channel multiplexing to form an optical multiplex;

–	generation and termination of management/maintenance signals as described in 6.2.

NOTE – The adaptation function is considered as having two data streams associated with it, one regarding the main optical payload and a second associated with that part of the overhead that is not processed by the OMS_TT. This is also true for the sink adaptation function.

The OMS/OCh adaptation sink (OMS/OCh_A_Sk) performs the following processes between its input and its output:

–	optical channel demultiplexing according to carrier wavelength (or frequency);

–	termination of the optical carrier and recovery of the optical transport unit;

–	generation and termination of management/maintenance signals as described in 6.2.

### 5.6.3	OTS/OMS adaptation

The bidirectional OTS/OMS adaptation (OTS/OMS_A) function is performed by a co-located pair of source and sink OTS/OMS adaptation functions.

The OTS/OMS adaptation source (OTS/OMS_A_So) performs the following process between its input and its output:

–	generation and termination of management/maintenance signals as described in 6.2.

NOTE – The adaptation function is considered as having two data streams associated with it, one regarding the main optical payload and a second associated with that part of the supervisory channel that is not processed by the OTS_TT. This is also the case for the sink adaptation function.

The OTS/OMS adaptation sink (OTS/OMS_A_Sk) performs the following process between its input and its output:

–	generation and termination of management/maintenance signals as described in 7.2.

## 5.7	Optical network topology

Optical network layers can support unidirectional and bidirectional point-to-point connections, and unidirectional point-to-multipoint connections.

### 5.7.1	Unidirectional and bidirectional connections and trails

A bidirectional connection in a server layer network may support either bidirectional or unidirectional client layer network connections, but a unidirectional server layer network may only support unidirectional clients.

A bidirectional optical transmission section layer network connection may be supported by one optical fibre for both directions (single fibre working), or each direction of the connection may be supported by different fibres.

Operation, administration and maintenance and overhead transfer in single fibre working is currently not considered in this Recommendation and is for further study.

### 5.7.2	Point-to-multipoint connections and trails

A unidirectional point-to-multipoint connection broadcasts the traffic from the source to a number of sinks. This is illustrated in Figure 5 where a point-to-multipoint connection is provided in the optical channel layer by means of a multipoint connection point (MPCP). The MPCP is a reference point that binds a port to a set of connections. It represents the root of a multipoint connection. The broadcast function provided by the MPCP binding is limited to the subnetwork in which it exists. It may form part of a multicast (selective broadcast) function within a larger (containing) subnetwork. The multipoint connection is restricted to a unidirectional broadcast multipoint connection in optical transport networks. This type of connection can be applied in the optical channel layer network.

Figure 5/G.872 – Point-to-multipoint optical channel connection

# 6	Subdividing of the Optical Transport Network

## 6.1	Subdividing into domains

Figure 6 illustrates how the OTN could be subdivided. Three administrative domains are shown. Administrative domain A is interconnected to administrative domain B via an OTN compliant IrDI, whilst B and C are interconnected via an OTN compliant IrDI.

Administrative domain A contains an optical channel subnetwork which contains four other optical channel subnetworks OCh-SNa to OCH-SNd. OCh-SNa to OCh-SNd may each be a matrix within individual network elements (and  OCH-SNe is therefore an optical channel ring) or they may represent subnetworks within the optical channel layer network that can be further decomposed to reveal subnetworks within itself.

Optical channel subnetworks a to d are connected together by optical channel links. This is further illustrated in Figure 7 which shows part of Figure 6 in greater detail. Each of the optical channel links is supported by links in the OMS layer which is supported by OTS links. Associated which each OTS link is an IaDI.

Within such environment it is necessary to consider where 3R regeneration can be utilised. Two cases are assumed:

- retiming and digital pulse regeneration associated with an OMS/OCh adaptation function or

- retiming and digital pulse regeneration in the OCh/client adaptation function.

Case a is within the OTN whilst case b is outside as it is on the client side of an optical channel access point.

Adaptation functions that employ a) are located at the end of optical channel link connections or network connections. They are not however associated with OCh subnetwork connections. The need to use a) within an OTN depends on network topology and performance requirements. In general, the following rules apply:

- The OMS/OCh adaptation functions that delimit and OCh link connection need not contain 3R regeneration processes. An example is an optical line system.

- Where 3R regeneration is used it can be applied to one adaptation function at one end of the OCh link or in both adaptation functions at each end of a link.

- Where optical channels are used for interworking between administrative domains, 3R regeneration shall be used in the OMS/OCh adaptation function at the end of the link.

- Individual optical channels that follow the some route through th network need not to be regenerated at the same point. For example, whilst a high capacity channel may be regenerated at the end of each link a lower rate channel may be regenerated less often. As such 3R regeneration within an OMS/OCh adaptation function is activated on a per channel basis.

Figure 6/G.872 - Example of subdivision of a transport network 
in terms of domains and subnetworks

Figure 7/G.872 - Examples of 3R

Figure 8 illustrates an example of a network element containing 3 OTN compliant interfaces and one OTN non-compliant interface. The Optical channel access point represents the logical boundary of the OTN. Non-OTN functions are outside the OTN as shown.

Figure 8/G.872 - OTN and non-OTN layer networks inside a Network Element

## 6.2	Subdividing of domains

Larger administrative or management domains may be subdivided into smaller domains where 3R regeneration shall be used in the OMS/OCh adaptation at both sides or either side of the IaDI. The domain should be dimensioned such, that full routing flexibility exists within a domain without the need of 3R regeneration. 3R is fixed assigned to the IaDI forming the boundaries of such domains..

# 7	Optical network management

This clause describes network management for the optical transport network. In particular, it describes the generic requirements for fault, performance and configuration management. The management processes required in each of the layer networks are outlined in 7.2 and summarized in Table 1. This clause also describes techniques for connection supervision.

## 7.1	Generic requirements

### 7.1.1	Generic fault, configuration and performance management

The optical transport network shall provide support for fault, configuration and performance management end-to-end and also within and between administrative boundaries.

It shall provide a means of detection and notification in the event of a misconnection.

The optical transport network shall provide facilities to:

–	ensure interconnection of transport network entities that have compatible adapted or characteristic information;

–	detect faults, isolate faults and initiate recovery actions where applicable. The optical transport network shall provide facilities for single-ended maintenance.

In the event of a signal within the server layer being interrupted, upstream and downstream network entities in the server layer shall be notified.

The optical transport network shall be able to detect performance degradations to avoid failures and verify quality of service.

### 7.1.2	Generic management communications

The optical transport network shall support communications between:

–	personnel at remote sites;

–	OSs and remote NEs;

–	craft terminals and local or remote NEs.

These forms of communication may also be supported externally to the optical transport network.

### 7.1.3	Generic client/server interaction management

The optical transport network shall detect and indicate when a signal is not present at a client layer, within the OTN, also in the case where the server layer is operating normally.

In order to avoid unnecessary, inefficient or conflicting survivability actions, escalation strategies (e.g. introduction of hold-off times and alarm suppression methods) are required:

–	within a layer;

–	between the server and client layer.

## 7.2	Optical layer network management requirements

Editors Note: Q19/13 has currently no agreement on the overall model for the OCh layer network and the OCh characteristic information. §7.2 has to be updated accordingly when agreement on these issues has reached. Contributions are requested

Requirements for management capabilities with respect to the Optical Channel, Optical Multiplex Section and Optical Transmission Section layer networks are identified in this subclause. A summary of the optical layer network management requirements is given in Table 1 and discussed in detail below.

| Table 1/G.872 – Optical transport network – Network level management requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Management capability | Process |  |  | Func. |  | Layer network |  |  |  |  |  |  |  |  |  |  |  |  | Comments |  |  |
|  |  |  |  |  |  |  | OCh b) |  |  |  |  |  |  |  | OMS |  | OTS |  |  |  |  |
|  |  |  |  |  |  |  | P |  | T |  | F |  | R |  |  |  |  |  |  |  |  |
| Continuity supervision |  | • Loss of continuity detection |  |  | TT |  |  | r |  |  |  |  |  |  |  | r |  | r |  |  | See 7.2.1 |
| Connectivity supervision |  | • Trail trace identification |  |  | TT |  |  | r a) |  |  |  |  |  |  |  | nr |  | r |  |  | See 7.2.1 |
| Maintenance information |  | • Forward defect indication • Backward defect indication • Backward quality indication |  |  | TT TT TT |  |  | r r ffs |  |  |  |  |  |  |  | r r ffs |  | r r ffs |  |  | See 7.2.1 |
| Signal quality supervision |  | • Performance monitoring (parameters are for further study) |  |  | TT |  |  | r |  |  |  |  |  |  |  | ffs |  | r |  |  | See 7.2.2 |
| Adaptation management |  | • Payload type indication |  |  | A |  |  | r |  |  |  |  |  |  |  | ffs |  | na |  |  | See 7.2.3 |
| Protection control |  | • Automatic protection switching protocol |  |  | A/T |  |  | ffs |  |  |  |  |  |  |  | ffs |  | nr |  |  | See 7.2.4 |
| Subnetwork/tandem/unused connection supervision |  | • Connection supervision |  |  | A/T |  |  | ffs |  |  |  |  |  |  |  | ffs |  | ffs |  |  | See 7.3 |
| Management communications |  | • Message-based channel • Auxiliary channel |  |  | A A |  |  | nr nr |  |  |  |  |  |  |  | ffs nr |  | r o |  |  | See 7.2.5 |
| Other communication needs |  | • Operator-specific • National use |  |  | A A |  |  | nr nr |  |  |  |  |  |  |  | nr nr |  | r ffs |  |  | See 7.2.6 |
| r = required |  |  | TT = Trail termination function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nr = not required |  |  | A = Adaptation function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| o = optional |  |  | A/T = Process may be assigned to one or both functions; allocation is ffs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ffs = for further study |  |  | a) = The only exception to this requirement is described in 7.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| na = not applicable |  |  | b) = P = end-to-end Path; F = FEC sublayer; T = TCM sublayer R = 3R regenerator section sublayer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Editors Note: The need to define an OCh sublayer for 3R regeneration was questioned, since such a sublayer would only change the signal quality rather than doing information processing. Contributions are requested. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.1	Connection supervision

It is a management requirement to provide supervision of the integrity of network connections that are supporting the trails in any layer network. A link connection supported by a server layer network is supervised by means of continuity supervision. The subnetwork connections that result from the flexible association of connection points across the subnetwork are supervised by means of connectivity supervision. For the particular case that there is no possibility to rearrange network connections between a group of OCh source and a group of OCh sink trail terminations connectivity supervision is not required.

Continuity supervision

Continuity supervision refers to the set of processes for monitoring the integrity of the continuity of a trail.

The following process is identified for continuity supervision:

•	Detection of Loss of Continuity (LOC).

In general, the failure of a link connection in a server layer will be indicated to a client layer through some form of server signal fail indication. The OTS layer, the lowest layer of the OTN, is a special case since its network connections are supported directly by the optical physical media layer. Since the latter does not contain active components, the OTS trail termination sink will not receive server fail indications – as is the case for trail terminations in higher layers – and has to detect failures in the optical physical media layer by itself.

Optical network failures include fibre disruptions and equipment failures. Equipment failures as such will be detected and reported by equipment monitoring capabilities.

The fibre disruption case is the most important failure scenario to consider from a network level view. Following a fibre disruption, loss of the aggregate signal may be observed at the first downstream OTS trail termination sink. The aggregate signal consists of the multiplexed wavelengths carrying the optical channels and the wavelength carrying the optical supervisory channel. Loss of the aggregate signal therefore results in loss of continuity of the multiplexed wavelengths and loss of continuity of the optical supervisory channel. Subsequently, the detection of the loss of the aggregate signal will be indicated towards the client layer. Note that loss of continuity of the optical supervisory channel by itself shall not initiate consequent actions on the client signal. In general, the same philosophy should be adopted in any layer network where payload and overhead have independent failure mechanisms.

At the OTS layer an optical component failure may lead to the loss of optical channels but may not lead to the loss of the optical supervisory channel. This will generate a server signal fail indication to the OMS layer and a forward defection indication within the OTS layer, the same consequent actions as in the fibre disruption case.

A server signal fail detected by the OMS trail termination sink will lead in turn to a server signal fail towards the OCh layer. In the OMS adaptation source the server signal fail will lead to a forward defect indication of the affected optical channels. It is conceivable that the OMS trail termination sink will detect a loss of continuity of the OMS trail without a loss of continuity being detected in the OTS trail. Consequent actions are the same as for the server signal fail case.

A server signal fail detected by the OCh trail termination sink will lead in turn to a server signal fail towards the client layer. The processing in the OCh adaptation source of the server signal fail is client specific. It is conceivable that the OCh trail termination sink will detect a loss of continuity of the OCh trail without a loss of continuity being detected in the OTS or OMS trail. Consequent actions are the same as for the server signal fail case.

Note that failure conditions within the OTN and/or unused (unlit) optical channel layer connections can result in missing optical payload for downstream server layer trails (e.g. the fibre disruption at the input of an optical amplifier results in missing channels at the output of the optical line amplifier). This shall not result in loss of continuity for that trail (e.g. loss of channels at following OTS trail terminations in the example above). Appropriate maintenance signalling shall be used to prevent this.

Connectivity supervision

Connectivity supervision refers to the set of processes for monitoring the integrity of the routing of the connection between source and sink trail terminations.

Connectivity supervision is necessary to confirm proper routing of a connection between trail termination source and sink during the connection set-up process. Furthermore, connectivity supervision is needed to ensure that connectivity is maintained while the connection is active.

The following process is identified for connectivity supervision:

•	Trail Trace Identification

Trail Trace Identification is necessary to ensure that the signal received by a trail termination sink originates from the intended trail termination source. The following requirements are identified:

–	Trail Trace Identification is necessary at the OTS layer to ensure proper cable connection.

–	Trail Trace Identification is not needed at the OMS layer because there is a one-to-one relationship between the OTS and OMS layers, i.e. connectivity at the OMS layer is fixed; therefore, the OMS connection is already covered by the OTS trail trace identification. Flexible connectivity at the OMS layer is not envisaged.

–	Trail Trace Identification at the OCh layer is only needed where there is a possibility of channel rearrangement between OCh source/sink trail terminations.

Detection of connectivity defects will lead to the same consequent actions as described above for the detection of loss of continuity for the characteristic information.

Maintenance indication

Maintenance indication refers to the set of processes for indicating defects in a connection, which is part of a trail. The defect indications are given in downstream and upstream directions of a bidirectional trail.

Two maintenance indication processes are identified:

•	Forward Defect Indication (FDI).

•	Backward Defect Indication (BDI).

These processes enable defect localization and single-ended maintenance.

FDI is used to indicate downstream that a defect condition has been detected upstream. This allows the suppression of superfluous failure reports due to the defect.

BDI signals the state of the trail at the trail termination sink back to the remote trail termination sink. This assists in the maintenance of Inter-Domain Interfaces (see clause 8). In addition, BDI supports the real-time requirements of bidirectional performance monitoring.

In general, FDI and BDI are associated with the activation of server signal fail. Detailed requirements for individual layers are for further study.

FDI and BDI are applicable at the OCh, OMS and OTS layers.

NOTE – FDI and BDI terminology is used instead of the traditional AIS and RDI terminology in order not to prejudge the fault maintenance indications and functionality required by the OTN.

### 7.2.2	Signal quality supervision

Signal quality supervision refers to the set of processes for monitoring the performance of a connection, which is supporting a trail.

Signal quality supervision is necessary for determining the performance of connections. Generic processes include parameter measurement, collection, filtering and processing. In terms of network level management, signal quality supervision is needed to manage channels and multiplexed channels. Thus, performance parameter monitoring at the OCh and OTS layers is required. Identification of specific parameters required to be monitored for determining the quality of OCh and OTS connections is for further study.

The requirement for backward quality indication is for further study.

The requirement to monitor parameters in OMS layer is for further study.

### 7.2.3	Adaptation management

Adaptation management refers to the set of processes for managing client layer network adaptation to/from the server layer network.

The following process is identified for adaptation management in the OTN:

•	Payload Type Identifier (PTI)

This process is necessary to ensure the client layer is assigned at connection set-up to the appropriate source and sink OCh/Client adaptations. A payload type identifier mismatch detected at source or sink adaptations would indicate an incorrectly provisioned or altered client-OCh server layer adaptation.

Application of the PTI process at the OMS layer is for further study.

The PTI process is not applicable at the OTS layer. A client of the OTN is transparent at this layer.

The OCh/Client adaptation may contain client-specific supervision processes. Definition of these processes is outside the scope of this Recommendation.

### 7.2.4	Protection control

Protection control refers to the information and set of processes for providing control of protection switching for a trail or subnetwork connection. Protection switching is controlled on the basis of local criteria generated by the trail or subnetwork connection supervision and by the TMN/OS. Additionally, control from the remote network element using an automatic protection switching protocol (APS) is possible depending on the protection switching architecture.

Subclause 7.1 describes only protection architectures that are controlled by local information of the NE. An automatic protection switching (APS) protocol is not required for these protection architectures. The need for an APS protocol supporting additional protection architectures is for further study.

### 7.2.5	Management communications

Two types of management communication processes are identified:

•	message-based channel for supporting a data communications network;

•	auxiliary channel for supporting personnel voice and voiceband data communications.

A message-based channel is required at the OTS layer to support OTN-related data communications for, for example, management of line optical amplifiers. The need for an additional channel at the OMS layer to support, for example, OMS layer management information exchange, is for further study. A message-based channel at the OCh layer is not required.

The need for an auxiliary channel at the OTS layer supporting, for example, an engineering orderwire, is optional. Additional auxiliary channels at the OMS and OCh layers are not envisaged.

### 7.2.6	Other communication needs

Capacity needs to be made available for other communication channels, e.g. for operator-specific overhead, and/or national use.

## 7.3	Connection supervision techniques

Connection supervision is the process of monitoring the integrity of a given connection in the optical transmission section, optical multiplex section or optical channel layer networks. The integrity may be verified by means of detecting and reporting connectivity and transmission performance defects for a given connection. Recommendation G.805 defines four types of monitoring techniques for connections.

The connection supervision process can be applied to network connections and connection segments, where the latter is defined as an arbitrary series of subnetwork connections and link connections.

### 7.3.1	Inherent monitoring

Connections may be indirectly monitored by using the inherently available data from the server layers and computing the approximate state of the client connection from the available data.

Optical channel layer connections may be indirectly monitored by using the inherently available data from the optical multiplex section and computing the approximate state of the optical channel connection from the available data.

Optical multiplex section layer connections may be indirectly monitored by using the inherently available data from the optical transmission section and computing the approximate state of the optical multiplex section connection from the available data.

Inherent monitoring is not applicable in the optical transmission section as the server layer is the physical media and provides no data.

### 7.3.2	Non-intrusive monitoring

The connection is directly monitored by use of listen only (non-intrusive) monitoring of the original data and overhead. The approximate state of the connection can be determined by the information provided at each of the monitoring points.

Non-intrusive monitoring of the characteristic information transported by a connection is an application that can be used to provide fault localization. If a trail termination sink function detects a disturbance, it may not be immediately obvious where this disturbance first originated. The trail termination sink function therefore indicates that there is a disturbance of a certain kind but not where it is. In order to locate such a disturbance, the trail is viewed as a series of link connections. At the end of every link connection, a non-intrusive monitoring termination sink function (TTm) may be used to monitor the characteristic information at that point. The TTm does not provide any adapted information at its output. An example of the application of non-intrusive monitoring is illustrated in Figure 9. Traversing from the trail termination sink function and going towards the trail termination source, the fault is located between those two termination sink functions of which the upstream function reports disturbance free performance while the other reports the disturbance condition.

Connections may be directly monitored by means of the relevant overhead information in the optical multiplex section and optical channel layers and then computing the approximate state of the connection from the difference between the monitored states at each end of the connection. Non-intrusive monitoring is not required in the OTS, unless OTS-level network connection is employed in systems without line amplifiers.

Figure 9/G.872 – Example of subnetwork connection supervision 
using non-intrusive monitoring

### 7.3.3	Intrusive monitoring

A connection is directly monitored by breaking the original trail and introducing a test trail that extends over the connection for the duration of the test. This allows all parameters to be monitored directly; however, the user trail is not complete and this technique is therefore restricted to the beginning of trail set-up, or intermittent testing.

Intrusive monitoring may be used for testing fibre continuity and for fault localization.

### 7.3.4	Sublayer monitoring

Some portion of the original trail's overhead capacity is over-written such that the part of the connection that is of interest can be directly monitored by a trail created in a sublayer. With this technique all parameters can be tested directly. This scheme can provide for nested sublayer trail monitored connections.

Optical channel layer (network, subbnetwork, tandemed link and link) connections may be directly monitored by means of insertion of connection monitoring overhead at the ingress of the connection and extraction and processing of this overhead at the egress of the connection.

Sublayer monitoring is not available for optical multiplex sections and optical transmission sections.

## 7.4 	Connection supervision applications

### 7.4.1	Monitoring of unused connections

Monitoring the integrity of unused connections in optical transport network layers is for further study.

### 7.4.2	Connection monitoring

The intended role of optical channel connection monitoring is to represent that portion of an optical channel connection that .  requires independent monitoring from other parts of the optical channel connection.

Optical channel connection monitoring can be applied at:

- the network connection, establishing the layer network's trail

- any subnetwork connection, establishing a serving operator administrative domain tandem connection

- any tandem link connection or link connection, establishing a service requesting administrative domain tandem connection or a protected domain tandem connection

- any link connection, for fault and performance degradation detection for network maintenance purposes.

Optical channel connection monitoring can be established for a number of nested connections, up to the maximum level defined by Recommendation G.709. The number of connection monitoring levels that can be used by each operator/user involved in an optical channel connection must be mutally agreed between these operators and users.

An example (with five levels of nested connection monitoring) is presented in Figure 10 An optical channel signal generated in user network 1 is transported via the networks of 3 network operators to user network 2. Both user networks contain multiple OTN network elements.

The three network operators offer an optical channel leased circuit to the user network. The user network is capable to monitor the quality of service of the optical channel at an end to end basis and between the edges of the two user networks. The network operators monitor the quality of service of the jointly offered optical channel leased circuit, of the optical channel connection in each network operator domain and of a working and protection connection in a protected domain within the network of network operator C.

Monitoring the interconnection between any two adjacent networks (USR1-NO A, NO A-NO B, NO B-NO C, NO C-USR2) can be performed by means of the activation of additional connection monitors at each end of the interconnection (not illustrated). This will prevent white spots (unmonitored portion) to be present in the connection.

Figure 10 – Example of OCh connection monitoring

Connection monitoring provides the following functions (between the ends of the connection):

- connection near end fault management and performance monitoring (error performance and failure/alarm conditions);

-	connection server signal fail verification (i.e. FDI/AIS);

-	connection continuity verification (i.e. loss of continuity);

-	connection connectivity verification (i.e. trace);

-	connection error verification (i.e. error detection code);

- connection far end fault management and performance monitoring (error performance and failure/alarm conditions);

-	connection backward defect indication

-	connection backward error indication

- connection monitoring independent of the content/status of the incoming signal itself;

- addition or tearing down of a connection monitoring level without interruption of the traffic, and (as an objective) without introducing of monitoring errors at the other connection monitoring levels.

- non intrusive monitoring of any connection monitoring level at any at any intermediate point within the specific connection for:

-	monitoring the total connection from this point based on the backward indications (note: this is possible only in case of bidirectional connections)

-	monitoring of the status of the connection at this point.

Optical channel connection monitoring can be applied to unidirectional and bidirectional connections. For the case of unidirectional connections, far end fault management and performance monitoring is not supported and the addition or tearing down of a connection monitor level may introduce a traffic interruption.

# 8	Optical network survivability techniques

This clause describes the architectural features of network strategies that may be applied to enhance the survivability of optical transport networks from network link and node impairments. The survivability techniques considered for optical transport networks encompass both protection and network restoration capabilities.

The following network objectives for the selection of self-healing architectures are seen as the predominant ones:

- Heal quickly (on the order of SDH rings).

- Coexist in harmony with possible client layer schemes (e.g. SDH rings).  An example would be the ability to enable/disable the OTN protection scheme on a per Optical Channel basis.

- Heal single points of failure.

- Minimize the rerouting distance (to avoid physical layer impairments on the signals).

- Gracefully accommodate multiple failures.

- Avoid hits on traffic unaffected by the failure.

- Minimize the amount of protection bandwidth required.

- Minimize the amount of signalling complexity required.

- Support prior path verification.

- Take into account OTN ring interworking.

## 8.1	Protection

A protection application makes use of pre-assigned capacity between nodes. The simplest architecture has 1 working and 1 protection capacity (11); the most complex architecture has n working and m protection capacities (m:n).

Unidirectional protection is defined as a protection switching method which switches only the affected traffic direction in the event of a unidirectional failure. Bidirectional protection switches both directions of traffic in the event of a unidirectional failure.

Two types of protection architecture are considered: trail protection and subnetwork connection protection.

### 8.1.1	Trail protection

Trail protection is a dedicated end-to-end protection mechanism that can be used on any physical structure (i.e. meshed, ring or mixed). It can be applied in both the OCh and OMS layers. Trail protection is not recommended for use in the OTS layer. A working trail is replaced by a protection trail if the working trail fails or if the performance falls below the required level. Trail protection can operate in a unidirectional or bidirectional manner.

Trail protection may also be 11, where the dedicated protection trail is only used for protection purposes, or 1:1 where extra traffic may be supported.

The following type of trail protection may be used in optical transport layers:

11 unidirectional trail protection

In this architecture, a permanent bridge is utilized at the transmit end. At the receive end of the trail, a protection switch is effected by selecting one of the signals based on purely local information. This architecture can be applied in either the OMS or OCh layers. This architecture is illustrated in Figure 11. It may be used without an automatic protection switching protocol.

All other types of trail protection including OMS and OCh trail protection in other configurations is for further study.

### 8.1.2	Subnetwork connection protection

Subnetwork connection protection is a dedicated protection mechanism that may be used on any physical structure (i.e. meshed, ring, or mixed). It may be used to protect part or all of a network connection. Subnetwork connection protection using inherent monitoring (SNC/I) protects against failures in the server layer. The switching process and defect detection process are performed by two adjacent layers, with the server layer providing the defect detection process and the client layer receives Server Signal Fail (SSF) generated by the server layer. Subnetwork connection protection using non-intrusive monitoring (SNC/N) uses client layer information to protect against failures in the server layer and failures and degradations in the client layer.

The following SNC protection architectures have been identified for optical networks:

11 unidirectional SNC/N

In this architecture, a permanent bridge is utilized at the transmit end. At the receive end, a protection switch is effected by selecting one of the signals based on purely local information. This architecture can be applied in the OCh and OTS network layers. In the latter case, its application is restricted to network connection protection rather than subnetwork connection protection and is therefore suitable for short-haul optical line systems without in-line amplifiers. This architecture is illustrated in Figure 12. It may be used without an automatic protection switching protocol.

1+1 unidirectional SNC/I

This architecture can be applied in the optical channel layer network.

Other architectures are for further study.

Figure 11/G.872 – 11 unidirectional trail protection

Figure 12/G.872 – Network connection protection with non-intrusive monitoring 
in the optical transmission section layer network

### 8.1.3	1+1 OCh SNCP

In an OCh SNCP, each OCh is duplicated at the transmit end: one acts as a working OCh, the other acts as a protection OCh. Under failure conditions, the corresponding protection OCh is selected rather than the disrupted working OCh at the receive end.

### 8.1.4	OCh shared protection ring

This architecture virtually provides each OCh with a pre-assigned 1:1 protection route and capacity. The protection OCh itself does not carry a copy of the working OCh under non-failure conditions; therefore, the capacity is not occupied and can be used for the extra traffic. This protection capacity can be shared by other protection OChs on a link-by-link basis. To restore a network failure, the affected working OChs are switched to counter-directional routes on an end-to-end basis with pre-assigned wavelengths. Necessary APS protocols will be given in other recommendation.

### 8.1.5	OMS shared protection ring

OMS shared protection rings are characterized by dividing the total payload per OMS equally into working and protection capacity. For example, for a two fiber OTM-N ring, there are N/2 OTUs available for working and N/2 OTUs available for protection, while in a four fiber OTM-N ring, there are N OTUs available for working and N OTUs available for protection. The ring protection capacity can be accessed by any OMS of a multinode ring under a section or node failure condition. Thus, the protection capacity may allow an OMS shared protection ring to carry extra traffic with a lower priority. This extra traffic is not itself protected. Necessary APS protocols will be given in other recommendations.

## 8.2	Network restoration

Optical network restoration techniques are based on optical channel cross-connection. In general, the algorithms used for restoration involve rerouting. To restore an impaired connection, alternative facilities may be chosen among the available capacity of the optical layer network.

Optical transport network restoration techniques require further study.

# 9	Interconnection and interworking between different administrative domains

As optical networking technology is evolving, so will the methods by which interconnection and interworking between different administrative domains takes place. In this context we refer to interconnection to describe a physical interface between two administrative domains. Interworking refers to the agreed networking level between domains and is described in terms of the characteristic information that is transferred transparently across domains. The following scenarios are foreseen:

a)	Initially as WDM point-to-point line systems and more complex optical network elements are introduced, they will be operated as OTN islands contained within administrative domains. Interconnection with existing transport networks (e.g. PDH and SDH networks) may take place at one of the physical interfaces which have been standardized for these networks. Such interconnection generally involves modifying the physical characteristics of the signal which is passed over an inter-domain interface, such as a G.957 optical signal for SDH-based transport networks, so that the adapted information of the signal is OTN compliant. This method of interconnection is illustrated in Figure 13 with a non-OTN Inter-Domain Interface (non-OTN_IrDI) between administrative domains A and B. Domain B contains an OTN while domain A may or may not. Also shown in Figure 13is an OTN Intra-Domain Interface (OTN_IaDI). For Intra-Domain applications, no need is currently foreseen for the standardization of fully transverse compatible interfaces.

Interworking takes place at some agreed upon client layer and its supervision is based upon client-specific maintenance signals.

b)	As a second step, as capacity for interconnect increases, OTN compliant systems may be applied to interconnect administrative domains. This is depicted in Figure 14. The interconnection point is referred to as an OTN Inter-Domain Interface (OTN_IrDI). This interface may be single channel or multichannel.

For this first application, standardization of a transverse compatible short distance (e.g. 40 km) interface has first priority. Second priority should be given to transverse compatible interfaces that span longer distances.

Obviously such OTN_IrDIs may also be applied to Intra-Domain applications (OTN_IaDI).

For this step, limited optical channel overhead may be applied. Interworking will continue to take place at some agreed upon client layer as in case a).

c)	Finally, when the standards for the overhead are in place and implemented, it will become possible to provide continuity for the OCh at the interconnection point between different administrative domains as shown in Figure 15. The OTN_IrDI is intended to be used for this purpose. This use of an OTN compliant interface is driven by the need to provide OCh continuity. Therefore, the IrDI may be a single-channel or a multichannel interface.

Figure 13/G.872 – Scenario 1: Interconnection of different administrative domains
via a non-OTN interface

Figure 14/G.872 – Scenario 2: Interconnection of different administrative domains
via an optical island with an OTN inter-domain interface

Figure 15/G.872 – Scenario 3: Interconnection of OTN subnetworks 
in different administrative domains via an OTN inter-domain 
interface supporting OCh interworking

Annex A

Impairment mitigation and regeneration

The transmission of information over an optical network is hindered by the accumulation of impairments that need to be mitigated against to maintain signal quality. It is recognized from a modelling viewpoint that these compensations need to be described in terms of processes. In particular the description of processes involved in so-called 1R, 2R and 3R regeneration are of interest. A transport function must be described in terms of the processes associated with the relevant adaptation and termination functions in each layer and a simple statement of 1R, 2R or 3R regeneration is insufficient. However, because 1R, 2R and 3R regeneration are commonly used terms, the following classification is provided as an aid to understanding them.

These forms of regeneration are composed of a combination of the following processes:

a)	Equal amplification of all frequencies within the amplification bandwidth. There is no restriction upon client layers.

b)	Amplification with different gain for frequencies within the amplification bandwidth. This could be applied to both single-channel and multichannel systems.

c)	Dispersion compensation (phase distortion). This analogue process can be applied in either single-channel or multichannel systems.

d)	Noise suppression.

e)	Digital reshaping (Schmitt Trigger function) with no clock recovery. This is applicable to individual channels and can be used for different bit rates but is not transparent to line coding.

f)	Complete regeneration of the pulse shape including clock recovery and retiming within required jitter limits.

As can be seen in Figure A.1, 1R regeneration is described as any combination of processes a) to c). 2R regeneration is considered to be 1R regeneration together with processes d) and e), whilst 3R regeneration is considered to be 2R regeneration together with process f).

An informal description of 1R regeneration is that 1R regeneration is based on analogue techniques; 2R involves digital processing of the signal levels while 3R regeneration also involves digital processing of the signal timing information.

Figure A.1/G.872 – Regeneration classification

Appendix I

Examples of Optical Network functionality

This appendix describes examples of functional groupings that may be applied to the optical network.

## I.1	Wavelength conversion

Figure I.1 shows the functional model for single channel wavelength conversion. The OTS and OMS trails are terminated and the wavelength conversion is performed by the OMS/OCh adaptation function. At the OCh layer the wavelength is undefined. The OMS/OCh_A source assigns a specific wavelength to the optical channel.

## I.2	Cross-connect

Figure I.2 shows the functional model of a cross-connect and two optical amplifiers, one single channel and one multichannel. The OCh layer signals can be cross-connected between OTN interfaces or to appropriate client layer interfaces. The cross-connect may also include wavelength/frequency conversion.

## I.3	Regeneration

The processes involved in 1R, 2R and 3R regeneration as detailed in Annex A and their assignment to the appropriate optical transport network function are for further study.

Figure I.1/G.872 – Example of optical wavelength conversion

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |