Contribution Number: 	OIF00.011

Working Group:	Architecture

TITLE: 		Multi-protocol Lambda Switching: Issues in Combining MPLS

Traffic Engineering Control With Optical Crossconnects

<draft-basak-mpls-oxc-issues-00.txt>

SOURCE:		Debashis Basak

Marconi

Daniel O. Awduche

UUNet (MCI WorldCom)

John Drake

Marconi

Yakov Rekhter

Cisco

DATE: 	January 31, 2000

ABSTRACT:	This document describes domain specific enhancements needed to adapt

MPLS traffic engineering control plane constructs to control optical

crossconnects in emerging automatically switched optical transport

networks. These enhancements are within the framework of

Multiprotocol LAMBDA switching proposed in [1]. Specifically, this

memo investigates the behavior of the MPLS based control plane

technique for OXCs, and identifies enhancements required to both OXCs

and to existing MPLS control constructs (routing and signaling

protocols) to support the application to OXCs.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

Connectivity information at the network side for Figure 2.

Basak                      Expires July, 2000                   [Page 1]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

1. Introduction

Recently, a new paradigm for the design of control planes for OXCs

intended for data-centric automatically switched optical transport

networks was proposed in [1]. This new paradigm is termed

Multiprotocol Lambda Switching and exploits recent advances in MPLS

traffic engineering control plane technology to foster the expedited

development and deployment of a new class of versatile OXCs that

specifically address the optical transport needs of the Internet.

This memo describes a number of domain specific enhancements required

to map the MPLS control plane constructs to OXCs. The memo describes

enhancements to OXCs and WDM devices to the support MPLS based

control plane approach. It also discusses extensions to the existing

MPLS control plane constructs to better accommodate the needs of the

optical transport network elements.

2. Definitions

The MPLS control plane has been used (adapted) earlier in conjunction

with data planes with specific limitations e.g. ATM-LSRs, FR-LSRs.

The adaptation of MPLS control plane to OXCs, resulting in OXC-LSRs,

needs to consider specific limitations of the OXC data plane, as

identified in [1]. This section presents some definitions that take

into account such peculiarities of the OXC data plane.

A termination-capable (TC) interface on a label switching router

(LSR) is one which is capable of terminating an LSP and

demultiplexing data carried by the LSP to make further

routing/switching decisions. A point-to-point interface terminating

on a router that implements MPLS is an example of a TC interface.

A termination-incapable (TI) interface is one which is incapable of

terminating LSPs and demultiplexing data carried by the LSP to make

further routing/switching decisions. Thus, an LSP incoming on a TI

interface of a network device cannot be carrying data destined to

different egress points of the device. A fiber connected to an OXC is

an example of a TI interface.

For a given unidirectional link the interfaces associated with the

link's endpoints may be of different types with respect to their

ability to terminate an LSP. For example, for a link from a (frame-

based) LSR (e.g., router) to an OXC, the interface with the OXC is TI

while the one with the LSR is TC. The property of TI or TC is

associated with the head of a link, as well, because in a given MPLS

Basak                      Expires July, 2000                   [Page 2]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

box it may be possible for an LSP to come in through one interface,

be switched across the box, and then terminated at the far end (at

the head of a link).

If all the interfaces of an LSR are TI interfaces, we'll refer to

such an LSR as a TI-LSR. An OXC-LSR today is an example of a TI-LSR

(an ATM-LSR is another example of a TI-LSR).

If an LSR has at least one TC interface, we'll refer to such an LSR

as TC-LSR. A router that implements MPLS is an example of a TC-LSR.

A single box may have TC and TI interfaces.  For example, one could

imagine an optical box that on some interfaces forwards data based on

the wavelength/optical trail the data was received and on other

interfaces based on the label information carried by packets. One can

also envision a hybrid physical interface in which data on certain

wavelengths/optical trails is forwarded based on the

wavelength/optical trail the data was received and on other

wavelengths/optical trails based on the label information carried by

packets. Such physical interfaces may be considered as two separate

logical interfaces, one TC and the other TI.

A TI-LSR domain is a set of TI-LSRs which are mutually interconnected

by TI interfaces. An OTN made of a mesh of today's OXC-LSRs is an

example of a TI-LSR domain.

The Edge set of an TI-LSR domain is the set of TC-LSRs which are

interconnected to members of the domain by links with a TC interface

on a TC-LSR and a TI interface on a TI-LSR. A TC-LSR which is a

member of an Edge set of a TI-LSR domain is called an Edge LSR.

Examples of Edge LSRs are access devices to an OTN.  Figure 1 below

depicts an example network with a single TI-LSR domain consisting of

OXCs (O1 through O8) surrounded by an Edge set of TC-LSRs consisting

of access routers (M0 through M4).

By definition LSPs cannot start/terminate on the LSRs within a TI-LSR

domain. LSPs can start/terminate at TC-LSR belonging to the Edge set

or beyond.

Basak                      Expires July, 2000                   [Page 3]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

_________________

M0---/--O2-----O9      \

/   /       \       \

M1--|--O1 --O3--O4---O6-|---M3

\   \       /       /

M2--\--O5 -- O7 --O8--/---M4

\_______________/

TI-LSR domain

Mk: A TC-LSR (access routers), Ox: A TI-LSR (OXC)

Figure 1: A sample network with a TI-LSR domain

surrounded by an Edge set of TC-LSRs.

3. Required Enhancements to OXCs and WDM devices to support MPLS

The following contains the list of required enhancements to OXCs and

other WDM devices to support MPL(ambda)S:

- there should be a way to exchange control information

between OXCs, either using the same links (fiber) that

is used to carry data, or via a separate network.

- an OXC must be able to provide the MPLS control plane with the

information about the state of individual fibers attached

to that OXC, as well as with the state of individual ligthpaths

within each such fiber.

- an edge LSR may not have build-in WDM capabilities, but rather

may use external WDM device that acts as SONET-to-WDM

multiplexor/demultiplexor. That still should allow that LSR

to exchange control information with the OXCs.

4. Required Enhancements to the current MPLS control plane.

The section describes enhancements to the MPLS TE control component

(ISIS/OSPF and RSVP) in order to support MPL(ambda)S. The required

enhancements are divided into two sets (A) and (B).

A) The first set concerns enhancements to adapt to the

peculiarities of OXCs:

Basak                      Expires July, 2000                   [Page 4]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

- the RSVP Label Object, in combination with already available

IGP and RSVP information, should be able to provide sufficient

information to program cross-connect matrix on OXCs.

- when a pair of OXCs are connected by multiple links (fibers),

an IGP needs to carry information about physical diversity

of the fiber.

- since bandwidth allocation on an OXC is discrete, an IGP should

be able to carry the information about bandwidth granularity of

a particular link. This information should allow support for

multiple granularities within a single link, as well as for

different granularities per priority.

Additional requirements imposed by OXCs that can't perform

wavelength translation are outside the scope of the discussion

in this document.

B) As indicated in [1], essential to the use of MPLS with OXCs

is the ability to aggregate LSPs by using the notion of

"nested" LSP.

MPLS allows several ways to implement nested LSPs. One way to

accomplish this is to have a single MPLS control plane, but

allow this control plane to treat some of the LSPs created and

maintained by the control plane as links for the purpose of

establishing new LSPs (by the same control plane).  That is the

MPLS control plane could use an LSP as a link to form another

LSP, and that other LSP, in turn, could be used as a link to

form some other LSP, etc...

Another way to accomplish this is to have multiple instances of

the MPLS control plane, and allow LSPs created by one instance

of the control plane to be used as links by some other instance

of the control plane.

Regardless of whether one uses a single or multiple instances of

the MPLS control plane, the LSPs that are used as links could be

established by either (a) means outside of the MPLS control

plane (e.g., by configuration), or (b) by means of the MPLS

control plane itself.

The following contains the list of required enhancements to the

MPLS TE control component (ISIS/OSPF and RSVP) in order to

support the ability to aggregate LSPs in MPL(ambda)S:

Basak                      Expires July, 2000                   [Page 5]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

- an IGP should carry information about whether a particular

link is termination capable (TC) or not (TI). OSPF should be

able to take this information into account when computing

paths for optical trails.

- the LSP setup procedures should include support for an LSR at

the edge of TI-LSR domain to aggregate multiple LSPs coming

from outside of the TI-LSR domain into an LSP that is

an optical trail.

- an LSR should be able to advertise into an IGP a link that

is formed out of an LSP originated by the LSR, and SPF/OSPF

procedures should be able to use such a link for path

computation.

- one instance of MPLS should be able to present LSPs

created/maintained by that instance as links to another

instance of MPLS. The two instances may reside either

on the same, or on different devices.

Note that the ability to aggregate LSPs by nesting may be useful

outside of the OXC environment as well. The enhancements specified

above in support of aggregate LSPs have to implemented in such a

way as to allow non-OXC environments.

The specifications of the enhancements to the MPLS TE control

component in support of MPL(ambda)S will be covered in a

supplimentary document to follow.

5. Security Considerations

Security considerations are for future study.

6. References

[1] D. Awduche, Y. Rekhter, J. Drake, R. Coltun, "Multi-Protocol

Lambda Switching: Combining MPLS Traffic Engineering Control With

Optical Crossconnects", Internet Draft, Work in Progress.

[2] D. Awduche, J. Malcolm, J. Agogbua, M. O'Dell, and J. McManus,

"Requirements for Traffic Engineering Over MPLS," RFC-2702, September

1999.

Basak                      Expires July, 2000                   [Page 6]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

[3] D. Awduche, "MPLS and Traffic Engineering in IP Networks,"

IEEE Communications Magazine, December 1999.

[4] D. Awduche, L. Berger, D. Gan, T. Li, G. Swallow, and V.

Srinivasan, "Extensions to RSVP for LSP Tunnels," Internet Draft,

Work in Progress, 1999

[5] B. Jamoussi et al, "Constraint-Based LSP Setup using LDP,"

Internet Draft, Work in Progress, 1999

[6] R. Callon, P. Doolan, N. Feldman, A. Fredette, G. Swallow, A.

Viswanathan, "A Framework for Multiprotocol Label Switching,"

Internet Draft, Work in Progress, 1999

[7] E. Rosen, A. Viswanathan, R. Callon, "Multiprotocol Label

Switching Architecture," Internet Draft, Work in Progress, 1999

[8] T. Li, G. Swallow, D. Awduche, "IGP Requirements for Traffic

Engineering with MPLS," Internet Draft, Work in Progress, 1999

[9] H. Smith and T. Li, "IS-IS extensions for Traffic Engineering,"

Internet Draft, Work in Progress, 1999

[10] D. Katz, D. Yeung, "Traffic Engineering Extensions to OSPF,"

Internet Draft, Work in Progress, 1999

7. Author's Addresses

Debashis Basak

Marconi

1000 FORE Drive

Warrendale, PA  15086-7502

Phone:  724-742-7026

Email:  dbasak@fore.com

Daniel O. Awduche

UUNET (MCI WorldCom)

Loudoun County Parkway

Ashburn VA

Phone: 703-886-5277

Email: awduche@uu.net

John Drake

Marconi

1000 FORE Drive

Warrendale, PA  15086-7502

Basak                      Expires July, 2000                   [Page 7]

Internet Draft     draft-basak-mpls-oxc-issues-00.txt           Jan 2000

Phone:  724-742-6798

Email:  drake@fore.com

Yakov Rekhter

Cisco Systems

170 W. Tasman Dr.

San Jose, CA 95134

Email: yakov@cisco.com

Basak                      Expires July, 2000                   [Page 8]