Contribution Number: 	OIF00.014

Working Group:	Architecture

TITLE: 		Multi-protocol Lambda Switching: Combining MPLS

Traffic Engineering Control With Optical Crossconnects

<draft-awduche-mpls-te-optocal-01.txt>

SOURCE:

Daniel O. Awduche

UUNet (MCI WorldCom)

Yakov Rekhter

Cisco

John Drake

Marconi

Rob Coltun

Redback

DATE: 	January 22, 2000

ABSTRACT:	This document describes an approach to the design of control planes for optical crossconnects (OXCs), which leverages existing control plane techniques developed for MPLS Traffic Engineering. The proposed approach combines recent advances in MPLS traffic engineering control plane constructs with OXC technology to: (1) provide a framework for real-time provisioning of optical channels in automatically switched optical networks, (2) foster the expedited development and deployment of a new class of versatile OXCs, and (3) allow the use of uniform semantics for network management and operations control in hybrid networks consisting of OXCs and label switching routers (LSRs). The proposed approach is particularly advantageous for OXCs intended for data-centric optical internetworking systems. In such environments, it will help to simplify network administration. This approach also paves the way for the eventual incorporation of DWDM multiplexing capabilities in IP routers.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 2000 Optical Internetworking Forum

Connectivity information at the network side for Figure 2.

Internet Engineering Task Force

INTERNET-DRAFT

Daniel O. Awduche

Expiration Date: May 2000                    	 UUNET (MCI Worldcom)

Yakov Rekhter

Cisco Systems

John Drake

Fore Systems (Marconi)

Rob Coltun

Siara Systems

November 1999

Multi-Protocol Lambda Switching:

Combining MPLS Traffic Engineering Control With Optical Crossconnects

draft-awduche-mpls-te-optical-01.txt

Status of this Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026 except that the right to

produce derivative works is not granted.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF), its areas, and its working groups.  Note that

other groups may also distribute working documents as Internet-

Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet- Drafts as reference

material or to cite them other than as "work in progress."

To view the list Internet-Draft Shadow Directories, see

http://www.ietf.org/shadow.html.

Awduche/Rekhter et al                                           [Page 1]

draft-awduche-mpls-te-optical-01.txt          May 2000

Abstract

This document describes an approach to the design of control planes

for optical crossconnects (OXCs), which leverages existing control

plane techniques developed for MPLS Traffic Engineering.  The

proposed approach combines recent advances in MPLS traffic

engineering control plane constructs with OXC technology to:  (1)

provide a framework for real-time provisioning of optical channels in

automatically switched optical networks, (2) foster the expedited

development and deployment of a new class of versatile OXCs, and (3)

allow the use of uniform semantics for network management and

operations control in hybrid networks consisting of OXCs and label

switching routers (LSRs).  The proposed approach is particularly

advantageous for OXCs intended for data-centric optical

internetworking systems. In such environments, it will help to

simplify network administration.  This approach also paves the way

for the eventual incorporation of DWDM multiplexing capabilities in

IP routers.

1. Introduction

This document describes an approach to the design of control planes

for optical crossconnects (OXCs), which is based on the Multiprotocol

Label Switching (MPLS) traffic engineering control plane model. In

this approach, the main idea it to leverage recent advances in

control plane technology developed for MPLS traffic engineering (see

[1,2,3,4,8,9,10]). This approach is driven by pragmatic

considerations, as it exploits an existing technology base to foster

rapid development and deployment of a new class of  versatile OXCs that

address the optical transport needs of the rapidly evolving Internet.

This approach will assist in optical channel layer bandwidth

management, dynamic provisioning of optical channels, and network

survivability through enhanced protection and restoration

capabilities in the optical domain.

As used in this document, an OXC is a path switching element in an

optical transport network that establishes routed paths for optical

channels by locally connecting an optical channel from an input port

(fiber) to an output port (fiber) on the switch element. Additional

characteristics of OXCs, as used in this document, are discussed in

Section 4.1.

The proposed OXC control plane uses the IGP extensions for MPLS

traffic engineering (with additional enhancements) to distribute

relevant optical transport network state information, including

topology state information. This state information is subsequently

Awduche/Rekhter et al                                           [Page 2]

draft-awduche-mpls-te-optical-01.txt          May 2000

used by a constraint-based routing system to compute paths for

point-to-point optical channels through the optical transport

network.  The proposed OXC control plane also uses an MPLS signaling

protocol (see [3,4]) to instantiate point-to-point optical channels

between access points in the optical transport network.

This document does not specify the details of the extensions and

domain specific adaptations required to map the MPLS traffic

engineering control plane model onto the optical domain. These

aspects will be covered in a number of supplementary documents that

will follow. However, in Section 7 of this memo, we provide a high

level overview of the architectural issues involved in making such

adaptations.

2. Advantages

The advantages of the proposed approach are numerous.

- It offers a framework for optical bandwidth management

and the real-time provisioning of optical channels in

automatically switched optical networks.

- It exploits recent advances in MPLS control plane technology

and also leverages accumulated operational experience with IP

distributed routing control.

- It obviates the need to reinvent a new class of control

protocols for optical transport networks and allows reuse

of software artifacts originally developed for the MPLS

traffic engineering application. Consequently, it fosters

the rapid development and deployment of a new class of

versatile OXCs.

- It facilitates the introduction of control coordination concepts

between data network elements and optical network elements.

- It simplifies network administration in facilities based service

provider networks by providing uniform semantics for network

management and control in both the data and optical domains.

- It paves the way for the eventual introduction of DWDM

multiplexing capabilities on IP routers

- Lastly, it establishes a preliminary framework for the notion

of an optical Internet.

Awduche/Rekhter et al                                           [Page 3]

draft-awduche-mpls-te-optical-01.txt          May 2000

3. Background

The growth, performance, and survivability requirements of the

Internet (which is also becoming very mission critical) are mandating

IP-centric networks to be cost effective, survivable, scalable, and

to provide control capabilities that facilitate network performance

optimization. Some of these requirements are being addressed by the

Multiprotocol Label Switching (MPLS) traffic engineering capabilities

under development by the IETF [1,2,3,4].

The underlying optical transport network also needs to be versatile,

reconfigurable, cost effective, and to support a variety of

protection and restoration capabilities due to the rapidly changing

requirements of the Internet.

A result of these trends, therefore, is the evolution of optical

transport networks from simple linear and ring topologies to mesh

topologies. By a mesh (not necessarily fully meshed) topology, we

mean a connected (not necessarily fully connected) network of

arbitrary topology in which the node degree is typically more than

two. In mesh optical networks, optical crossconnects engender

versatility and reconfigurability by performing switching and

rearrangement functions.

Under-scoring the importance of versatile networking capabilities in

the optical domain, a number of standardization organizations and

interoperability forums have initiated work items to study the

requirements and architectures for reconfigurable optical networks.

Refer, for example, to ITU-T recommendation G.872 [5].  This document

defines a functional architecture for an optical transport network

(OTN) that supports the transport of digital client signals.  ITU-T

G.872 speaks of an OTN as "a transport network bounded by optical

channel access points"[5].  The ITU-T G.872 OTN architecture is based

on a layered structure, which includes:

(a) an optical channel (OCh) layer network,

(b) an optical multiplex section (OMS) layer network, and

(c) an optical transmission section (OTS) layer network.

The optical channel layer is the most relevant to the discussions in

this document. The optical channel layer network supports end-to-end

networking of optical channel trails between access points. The OCh

layer network provides the following functions: routing, monitoring,

grooming, and protection and restoration of optical channels.  In

this situation, programmable Optical crossconnects, with

rearrangeable switch fabrics and relatively smart control planes,

will be critical to the realization of the OCh layer functions,

especially in mesh optical networks. In the data network domain,

Awduche/Rekhter et al                                           [Page 4]

draft-awduche-mpls-te-optical-01.txt          May 2000

routing, monitoring, and survivability are crucial functions

performed by the MPLS traffic engineering control plane (see

[1,2,3,4,8,9,10]).

Note: Although we mention the ITU-T recommendation G.872, the OXC

control plane design approach described here is not restricted to

G.872 compliant networks. Instead, it can be applied to any mesh

optical network that uses programmable and reconfigurable OXCs.

Other standards organizations and interoperability forums that are

actively pursuing projects related to dynamically reconfigurable

optical networks include the ANSI T1X1.5 committee, the Optical

Internetworking Forum (OIF), and now by virtue of this memo the IETF.

Recent contributions to ANSI T1X1.5 emphasize the need for automation

of the OCh layer network by using appropriate signaling protocols to

establish optical channels in real time (see [14] and [15]).

The Optical Internetworking Forum (http://www.oiforum.com), an

international organization engaged in the development and promotion

of interoperability agreements for optical internetworking systems,

is also evaluating architectural options related to reconfigurable

optical internetworks. At a technical meeting held in California on

October 19, 1999, the Architecture Working Group of the OIF adopted a

motion to define requirements for signaling protocols that allow

rapid provisioning and efficient restoration in optical

internetworking systems.

In all these cases, the successful realization of the vision of

versatile optical networking depends very much on the synthesis of

appropriate control plane technologies with programmable and

reconfigurable OXCs.

4. OXCs, LSRs, Optical Trails, and Explicit LSPs

Consider a hybrid, IP-centric optical internetworking environment

consisting of both label switching routers (LSRs) and OXCs, where the

OXCs are programmable and support wavelength conversion/translation.

At a level of abstraction, an LSR and an OXC exhibit a number of

isomorphic relations. It is important to enumerate these relations

because they help to expose the reusable software artifacts from the

MPLS traffic engineering control plane model. Architecturally, both

LSRs and OXCs emphasize problem decomposition by decoupling the

control plane from the data plane.

The data plane of an LSR uses the label swapping paradigm to transfer

Awduche/Rekhter et al                                           [Page 5]

draft-awduche-mpls-te-optical-01.txt          May 2000

a labeled packet from an input port to an output port (see e.g.,

[6,7]). The data plane of an OXC uses a switching matrix to connect

an optical channel trail from an input port to an output port.

An LSR performs label switching by first establishing a relation

between an <input port, input label> tuple and an <output port,

output label> tuple. Likewise, an OXC provisions optical channel

trails by first establishing a relation between an <input port, input

optical channel> tuple and an <output port, output optical channel>

tuple. These relations are determined by the control plane of the

respective network elements, and are locally instantiated on the

device through a switch controller. In the LSR, the next hop label

forwarding entry (NHLFE) maintains the input-output relations. In the

OXC, the switch controller reconfigures the internal interconnection

fabric to establish the relations.  These relations cannot be altered

by the payload of the data plane.

The functions of the control plane (for both LSRs and OXCs) include

resource discovery, distributed routing control, and connection

management.  In particular, the control plane of the LSR is used to

discover, distribute, and maintain relevant state information

associated with the MPLS network, and to instantiate and maintain

label switched paths (LSPs) under various MPLS traffic engineering

rules and policies.  An LSP is the path through one or more LSRs

followed by a specific forwarding equivalence class (FEC) (see [7]).

An explicit LSP is one whose route is defined at its origination

node.

The control plane of the OXC, on the other hand, is used to discover,

distribute, and maintain relevant state information associated with

the OTN, and to establish and maintain optical channel trails under

various optical internetworking traffic engineering rules and

policies. An optical channel trail provides a unidirectional point-

to-point optical connection between two access points.  An optical

channel trail may consist of just one wavelength or a concatenation

of multiple wavelengths. If an optical trail consists of just one

wavelength, then it is said to satisfy the "wavelength continuity

property." At each intermediate OXC along the route of an optical

channel trail, the trail is routed from an input port to an output

port.

A distinction between the current generation of OXCs and LSRs is that

the former do not perform packet level processing in the data plane,

while the later are datagram devices which may perform certain packet

level operations in the data plane.  The really significant

conceptual difference, however, is that with LSRs the forwarding

information is carried explicitly as part of the labels appended to

data packets, while with OXCs the switching information is implied

Awduche/Rekhter et al                                           [Page 6]

draft-awduche-mpls-te-optical-01.txt          May 2000

from the wavelength or optical channel.

4.1 Review of Relevant OXC Characteristics

The following section contains a brief overview of relevant OXC

characteristics, focusing on the switching functions and underlying

technologies.

The switching function of an OXC may be electrical or optical. If the

switching fabric is purely electrical, then the crossconnect is

referred to as a digital crossconnect (DXC), or a broadband digital

cross-connect (BBDXC) -- if the capacity and port density are

sufficiently high.  Optical-Electronic-Optical (OEO) conversion is

required in BBDXCs.  A BBDXC may or may not have WDM multiplexing

capabilities. If a BBDXC has WDM multiplexing capabilities, then it

may be connected directly to other compatible WDM devices through

optical fiber links that carry multiple wavelengths per fiber. If a

BBDXC does not have WDM multiplexing capabilities, then it may be

connected to an external DWDM multiplexer through a set of discrete

fibers, where each fiber carries only one wavelength. A BBDXC may

also perform regeneration, reshaping, and re-timing functions.

If the switching fabric of an OXC is completely photonic, then we

refer to the cross-connect as a pure OXC. If the granularity of

channel switching is the wavelength, then the OXC is called a

wavelength routing switch (WRS), or simply a wavelength router. The

problem of establishing optical channel trails using WRS is called

the "Routing and Wavelength Assignment problem" (RWA) [13].

An OXC may also be equipped with both electrical and optical

switching capabilities. In this situation, some channels may be

switched in the electrical domain and others in the optical domain.

Other terms commonly used within the context of optical transport

network switching elements include: wavelength selective

crossconnects (WSXC) and wavelength interchanging crossconnects

(WIXC).

In this document, we use the generic term OXC to refer to all

categories of programmable and reconfigurable crossconnects for

optical transport networks, irrespective of the technologies that

underlie them.

The OXC control plane design approach described in this document is

independent of the underlying OXC switch technologies. It is also

independent of specific OXC implementation details. Local adaptation

mechanisms can be used to tailor the control plane onto various OXC

Awduche/Rekhter et al                                           [Page 7]

draft-awduche-mpls-te-optical-01.txt          May 2000

implementations with different hardware capabilities. As an example,

a local adaption function can map a channel/port input/output

relation into specialized low level instructions to actuate a

rearrangement of the crossconnect switch fabric such that the

required input/output relation is realized.

A number of forthcoming supplementary documents will describe in some

detail the extensions needed to adapt the control plane approach

described in this memo to various OXC technologies and optical

transport network contexts.

4.2 Explicit LSPs and Optical Channel Trails

At a conceptual level, explicit LSPs and optical channel trails

exhibit certain commonalities. Essentially, they are both

fundamentally unidirectional, point-to-point virtual path connection

abstractions.  An explicit LSP provides a parameterized packet

forwarding path (traffic-trunk) between an ingress LSR and an egress

LSR. Correspondingly, an optical channel trail provides a (possibly

parameterized) optical channel between two endpoints for the

transport of client digital signals.

The payload carried by both LSPs and optical trails are transparent

to intermediate nodes along their respective paths. Both LSPs and

optical trails can be parameterized to stipulate their performance,

behavioral, and survivability requirements from the network. A set of

LSPs induce a virtual graph on a data network topology, while a set

of optical trails induce a virtual graph on the topology of a fiber

plant.

A constraint-based routing scheme can be used to select appropriate

paths for both LSPs and optical trails. Generally such paths may

satisfy some demands and policy requirements subject to some

constraints imposed by the operational environment.

There are also commonalities in the allocation of labels to LSPs and

in the allocation of wavelengths to optical trails. Two different

LSPs that traverse through a given LSR port or interface cannot be

allocated the same label.  The exception is for LSP aggregation using

label merge or label stacking.  Similarly, two different optical

trails that traverse through a given OXC port cannot be allocated the

same wavelength. It is significant to note, however, that an analog

to label stacking does not exist in the optical domain at this time.

Awduche/Rekhter et al                                           [Page 8]

draft-awduche-mpls-te-optical-01.txt          May 2000

5. Generic Requirements for the OXC Control Plane

The following section contains the requirements for the OXC control

plane, with emphasis on the routing components of these requirements.

There are three key aspects to these requirements:

(1) the capability to establish optical channel trails

expeditiously, (in seconds or even milliseconds rather

than days or months).

(2) the capability to support traffic engineering functions,

(see note below)

(3) the capability to support various protection and restoration

schemes.

Note: the introduction of DWDM and automatically switched optical

networks is unlikely to eliminate the need for traffic engineering.

Instead, it will simply mandate OXCs to also support some traffic

engineering capabilities.

Historically, the "control plane" of optical transport networks has

been implemented via network management.  This approach has the

following detrimental effects:

(1) It leads to relatively slow convergence following failure

events (typical restoration times are measured in minutes,

or even days and weeks especially in systems that require

explicit manual intervention). The only way to expedite

service recovery in such environments is to pre-provision

dedicated protection channels.

(2) It complicates the task of interworking equipment from

different manufacturers, especially at the management level

(generally, a custom "umbrella NMS or OSS" is required to

integrate otherwise incompatible Element Management Systems

from different vendors)

(3) It precludes the use of distributed dynamic routing control

in such environments.

(4) It complicates the task of inter-network provisioning (due

to the lack of EDI between operator NMSs).

Thus, another important motivation for the approach described in this

document is to improve the responsiveness of the optical transport

network, and to increase the level of interoperability within and

between service provider networks.

Awduche/Rekhter et al                                           [Page 9]

draft-awduche-mpls-te-optical-01.txt          May 2000

6. MPLS Traffic Engineering as a Generic Control Plane for OXCs

The requirements for the OXC control plane described in the previous

section have already been addressed by the MPLS Traffic Engineering

control plane, under development by the IETF (see e.g.,

[1,2,3,4,8,9,10]).

6.1 Overview of the MPLS Traffic Engineering Control Plane

Let us now discuss the components of the MPLS traffic engineering

control plane model. The MPLS traffic engineering control plane is a

synthesis of new concepts in IP traffic engineering (enabled by MPLS)

and the conventional IP network layer control plane. The high level

requirements for traffic engineering over MPLS were articulated in

RFC-2702 [1]. It is the combination of the notions defined in RFC-

2702 (including relevant extensions) with the conventional IP control

plane constructs that effectively establishes a framework for the

MPLS traffic engineering control plane model [1] (see also [2]).

The components of the MPLS traffic engineering control plane model

include the following modules:

- Resource discovery

- State information dissemination, which is used to distribute

relevant information concerning the state of the  network,

including topology and resource availability information.

In the MPLS context, this is accomplished by extending

conventional IP link state interior gateway protocols to carry

additional information in their link state advertisements

(see [9,10]).

- Path selection, which is used to select an appropriate route

through the MPLS network for explicit routing. It is implemented

by introducing the concept of constraint-based routing which is

used to compute paths that satisfy certain specifications subject

to certain constraints, including constraints imposed by the

operational environment (see [1]).

- Path management, which includes label distribution, path

placement, path maintenance, and path revocation. These are

used to establish, maintain, and tear down LSPs in the MPLS

context. The label distribution, path placement, and path

revocation functions are implemented through a signaling

protocol, such as the RSVP extensions [3] or through CR-LDP [4].

Awduche/Rekhter et al                                          [Page 10]

draft-awduche-mpls-te-optical-01.txt          May 2000

These components of the MPLS traffic engineering control plane are

separable, and independent of each other. This is a very attractive

feature because it allows an MPLS control plane to be implemented

using a composition or synthesis of best of breed modules.

In RFC-2702 [1], several new MPLS control plane capabilities were

proposed that allow various traffic engineering policies to be

actualized in MPLS networks.  Many of these capabilities are also

relevant and applicable to automatically switched optical transport

networks with reconfigurable OXCs.

We summarize some of these capabilities below, focusing on the set of

attributes that can be associated with traffic-trunks. A traffic-

trunk is an aggregation of traffic belonging to the same class which

are forwarded through a common path. In general, the traffic-trunk

concept is a technology independent abstraction. In [1], it was used

within the context of MPLS and allowed certain attributes of the

traffic transported through LSPs to be parameterized. The traffic-

trunk concept can also be extended, in an obvious manner, to the

optical transport network.

As stipulated in RFC-2702 [1], the attributes that can can be

associated with traffic-trunks include: (1) traffic parameters which

indicate the bandwidth requirements of the traffic-trunk, (2)

adaptivity attributes which specify the sensitivity of the traffic-

trunk to changes in the state of the network and in particular

indicates whether the traffic-trunk can be re-routed when "better"

paths become available, (3) priority attributes which impose a

partial order on the set of traffic-trunks and allow path selection

and path placement operations to be prioritized, (4) preemption

attributes which indicate whether a traffic-trunk can preempt an

existing traffic-trunk from its path, (5) resilience attributes which

stipulate the survivability requirements of the traffic-trunk and in

particular the response of the system to faults that impact the path

of the traffic-trunk, and (6) resource class affinity attributes

which further restrict route selection to specific subsets of

resources and in particular allow generalized inclusion and exclusion

policies to be implemented. Other policy attributes and options are

also defined by Awduche et al in RFC-2702 [1] for traffic-trunks,

including policing attributes [1] (policing is irrelevant in the OXC

context). Concepts of subscription (booking) factors are also

supported to either bound the utilization of network resources

through under-subscription or to exploit statistical multiplexing

gain through over-subscription (this aspect is also not very relevant

in the OXC context).

It should be clear that a subset of these capabilities can be mapped

onto an optical transport network by substituting the term "traffic-

Awduche/Rekhter et al                                          [Page 11]

draft-awduche-mpls-te-optical-01.txt          May 2000

trunk" with the term "optical channel trail."

The MPLS control plane also supports the notion of abstract nodes.

An abstract node is essentially a set of nodes (e.g., a subnet, an

autonomous system, etc) whose internal topology is opaque to the

origination node of an explicit LSP. So, in the most general manner,

the route of an explicit LSP (or traffic-trunk) can be specified as a

sequence of single hops and/or as a sequence of abstract nodes.

The MPLS control plane is very general and is also oblivious of the

specifics of the data plane technology. In this regard, the MPLS

control plane can be used in conjunction with a data plane that (a)

does not necessarily process IP packet headers and (b) does not know

about IP packet boundaries. For an existence proof, note that the

MPLS control plane has been implemented on IP-LSRs, ATM-LSRs, and

Frame Relay-LSRs.

The MPLS control plane may also be implemented on OXCs as discussed

in this document.

6.2 Synthesizing the MPLS Traffic Engineering Control Plane with OXCs

Given that that both OXCs and LSRs require control planes, one option

would be to have two separate and independent control planes - one

for OXCs, and another for LSRs. To understand the drawbacks of this

approach, especially in IP-centric optical internetworking systems,

one need to look no further than the experience with IP over ATM,

where IP has its own control plane (BGP, IS-IS, OSPF), and ATM its

own control plane (PNNI) [12]. For some of the drawbacks see [1,2].

Given that the control planes for both OXCs and LSRs have relatively

similar requirements, an alternative approach is to develop a uniform

control plane that can be used for both LSRs and OXCs. Such a uniform

control plane will eliminate the administrative complexity of

managing hybrid optical internetworking systems with separate,

dissimilar control and operational semantics. Specializations may be

introduced in the control plane, as necessary, to account for

inherent peculiarities of the underlying technologies and networking

contexts.

All of the above observations suggest, therefore, that the MPLS

Traffic Engineering control plane (with some minor extensions) would

be very suitable as the control plane for OXCs. An OXC that uses the

MPLS traffic engineering control plane would effectively become an IP

addressable device. The establishment of optical channel trails, OTN

traffic engineering functions, and protection and restoration

capabilities would be provided by the MPLS Traffic Engineering

Awduche/Rekhter et al                                          [Page 12]

draft-awduche-mpls-te-optical-01.txt          May 2000

control plane.

An out-of-band IP communications system can be used to carry and

distribute control traffic between the control planes of OXCs,

perhaps through dedicated supervisory channels (using e.g., dedicated

wavelengths or channels, or an independent out-of-band IP network).

In this environment, SNMP, or some other network management

technology, could be used for element management. From the

perspective of control semantics, an OXC with an MPLS Traffic

Engineering control plane would resemble a Label Switching Router.

If the OXC is a wavelength routing switch, then the physical fiber

between a pair of OXCs would represent a single link in the OTN

network topology.  Individual wavelengths or channels would be

analogous to labels. IS-IS or OSPF, with extensions for traffic

engineering ([9] or [10]) would be used to distribute information

about the optical transport network topology and information about

available bandwidth and available channels per fiber, as well as

other OTN network topology state information. This information will

then be used to compute explicit routes for optical channel trails.

An MPLS signaling protocol, such as RSVP extensions (see [RSVP]),

will be used to instantiate the optical channel trails. Using the

RSVP extensions, for example, the wavelength information or optical

channel information (as the case may be) will be carried in the LABEL

object, which will be used to control and reconfigure the OXCs.

The use of a single control plane for both LSRs and OXCs introduces a

number of interesting (and potentially advantageous) possibilities.

A single control plane (MPLS Traffic Engineering) would be able to

span both routers and OXCs. In such an environment a Label Switching

Path could traverse an intermix of routers and OXCs, or could span

just routers, or just OXCs.  This offers the potential for real

bandwidth-on-demand networking, in which an IP router may dynamically

request bandwidth services from the optical transport network.

To bootstrap the system, OXCs must be able to exchange control

information.  One way to support this is to pre-configure a dedicated

control wavelength between each pair of adjacent OXCs, or between an

OXC and a router, and to use this wavelength as a supervisory channel

for exchange of control traffic. Another possibility, which has

already been mentioned, is to construct a dedicated out of band IP

network for the distribution of control traffic.

Even though an OXC equipped with an MPLS traffic engineering control

plane would (from a control perspective) resemble a Label Switching

Router, there are some important distinctions and limitations.  One

distinction concerns the fact that there are no analogs of label

merging in the optical domain. This implies that an OXC cannot merge

Awduche/Rekhter et al                                          [Page 13]

draft-awduche-mpls-te-optical-01.txt          May 2000

several wavelengths into one wavelength.  Another distinction is that

an OXC cannot perform the equivalent of label push and pop operations

in the optical domain. This is because the analog of a label in the

OXC is a wavelength or an optical channel, and the concept of pushing

and popping wavelengths is infeasible with contemporary commercial

optical technologies.

In the proposed control plane approach, an OXC will maintain a WFIB

(Wavelength Forwarding Information Base) per interface (or per

fiber). This is because lambdas and/or channels (labels) are specific

to a particular interface (fiber), and the same lambda and/or channel

(label) could be used concurrently on multiple interfaces (fibers).

The MPLS traffic engineering control plane is already being

implemented on data plane technologies that exhibit some of the

aforementioned distinctions. For example, an ATM-LSR supports only a

subset of the MPLS functionality.  In particular, most ATM-LSRs are

incapable of merging Label Switching Paths, and may not be able to

perform label push/pop operations as well. Also, similar to the

approach proposed here for OXCs, ATM-LSRs have per interface LFIB

(Label Forwarding Information Base).

Yet another important distinction concerns the granularity of

resource allocation. An MPLS Label Switching Router which operates in

the electrical domain can potentially support an arbitrary number of

LSPs with arbitrary bandwidth reservation granularities (bounded by

the maximum reservable bandwidth per interface and the amount of

required control overhead). In sharp contrast, an OXC can only

support a relatively small number of optical channel trails (this may

change as the technology evolves), each of which will have coarse

discrete bandwidth granularities (e.g.,OC-12, OC-48, and OC-192). A

special degenerate case occurs when the control plane is used to

establish optical channel trails which all have a fixed bandwidth

(e.g., OC-48).

If the bandwidth associated with an LSP is small relative to the

capacity of an optical channel trail, then very inefficient

utilization of network resources could result if only one LSP is

mapped onto a given optical channel trail. To improve utilization of

resources, therefore, it is necessary to be able to map several low

bandwidth LSPs onto a relatively high capacity optical channel trail.

For this purpose, a generalized notion of "nested LSPs" may be used.

Note that since an OXC cannot perform label push/pop operations, the

start/end of a nested LSP has to be on a router (as nesting requires

label push/pop). Also note that in this nesting situation, it is the

wavelength of the "container" optical channel trail itself that

effectively constitutes the outermost label.

Awduche/Rekhter et al                                          [Page 14]

draft-awduche-mpls-te-optical-01.txt          May 2000

The transparency and multiprotocol properties of the MPLS Control

Plane approach would allow an OXC to route optical channel trails

carrying various types of digital payloads (including IP, ATM, Sonet,

etc) in a coherent and uniform way.

7. Control Adaptation

This section provides a high level overview of the architectural

considerations involved in tailoring the MPLS traffic engineering

control plane model to the optical domain. More detailed discussions

of these issues will be provided in a number of supplementary

documents to follow.

In adapting the MPLS traffic engineering control plane model to OXCs,

a number of critical issues needs to be considered. One critical

issue concerns the development of OTN specific domain models which

abstracts and captures relevant characteristics of the OTN. The

domain models help to delineate the design space for the control

plane problem in OXCs, and to construct domain specific software

reference architectures.

A domain model includes functional and structural aspects. For the

purpose of the present discussions, however, we have grouped the

considerations pertaining to OTN domain models into two broad

categories: (1) a horizontal dimension and (2) a vertical dimension.

The horizontal dimension pertains to the specific networking

requirements of the OTN environment. It indicates the enhancements

needed to the MPLS TE control plane model to address the peculiar OTN

networking requirements.  The vertical dimension pertains to specific

localized hardware and software characteristics of the OXCs, which

helps to determine the device specific adaptations and mechanisms

needed to port the MPLS TE control plane model software artifacts

onto an OXC.

Horizontal dimension considerations include the following aspects:

- What type of OTN state information needs to be discovered and

disseminated to support path selection for optical channel

trails?  Such state information may include domain specific

characteristics of the OTN (encoded as metrics), such as

attenuation, dispersion (chromatic, PMD), etc. This aspect will

determine the type of additional extensions that are required

for IGP link state advertisements to distribute such information.

- What infrastructure will be used to propagate the control

information?

Awduche/Rekhter et al                                          [Page 15]

draft-awduche-mpls-te-optical-01.txt          May 2000

- How are constrained paths computed for optical channel trails

which fulfill a set of performance and policy requirements

subject to a set of system constraints?

- What are the domain specific requirements for setting up optical

channel trails and what are the enhancements needed to existing

MPLS signaling protocols to address these requirements?

Vertical dimension considerations include the aspects required to

practically port MPLS control plane software onto an OXC.  In terms

of vertical dimensions, a candidate system architecture for an OXC

equipped with an MPLS control plane model is shown in Figure 1 below.

Figure 1: Candidate OXC Systems Architecture

Awduche/Rekhter et al                                          [Page 16]

draft-awduche-mpls-te-optical-01.txt          May 2000

8. Architectural Considerations for Deployment in Operational Networks

This section provides a high level overview of the architectural

considerations for deployment of the proposed control plane in

operational networks consisting of LSRs and OXCs. These architectural

issues have implications on the degree of control isolation and

control cohesion between LSRs and OXCs.

Essentially, there are two basic architectural options for deployment

of the proposed control plane in an operational context consisting of

LSRs and OXCs.

- One option is to use different instances of the control plane

in the OTN (OXC) and IP (LSR) domains. In this situation, each

instance of the control plane will operate independent of the

other. Interworking (including control coordination) between the

two domains can be established through static configuration or

through some other procedures that are outside the scope of

this document. This partitioned deployment option allows maximal

control isolation between the OTN and IP domains. This scheme is

conceptually similar to the model in use today, whereby the OTN

simply provides point-to-point channels between IP network

elements with very minimal control interaction between the two

domains.

- Another option is to use a single instance of the control

plane that subsumes and spans LSRs and OXCs.

To improve scalability the control plane may use routing hierarchy

(e.g., routing areas). Hierarchy may be applied in either situation.

Furthermore, in the option with multiple control plane instances,

hierarchy could be enabled for each control plane instance

independent of the others.

In the deployment option with a single instance of the control plane,

each routing area may maintain a link state database that contains:

(1) physical LSPs (fiber links), (2) optical LSPs (optical channel

trails), and (3) logical LSPs (conventional label switched paths). As

a general rule, all these path-oriented connection entities could

simply be considered as LSPs with different characteristics. The

origination LSR (the head-end) of each LSP entity may locally decide

whether to advertise the LSP (with appropriate attributes), so that

other LSRs could use it as a link for subsequent path computations.

There are significant tradeoffs to the above deployment options,

including aspects related to fault isolation. There are also some

details that have been left out of these discussions. These aspects

Awduche/Rekhter et al                                          [Page 17]

draft-awduche-mpls-te-optical-01.txt          May 2000

will be covered in supplementary documents to follow.

One of the advantages of the control plane design approach described

in this memo is that it potentially allows network administrators the

option to make these deployment architectural decisions based on

their specific objectives and service models.

9. Summary

This document outlined how the MPLS Traffic Engineering control plane

could be used as the control plane for optical crossconnects.  Such a

control plane would be used to distribute optical transport network

topology state information and to setup optical channel trails. Such

a control plane would support various traffic engineering functions

in the optical domain, and enable a variety of protection and

restoration capabilities.  Furthermore, such a uniform control plane

technology would expedite the development and deployment of a new

class of versatile data-centric OXCs.  Additionally, the proposed

control plane approach would simplify integration of OXCs and label

switching routers. Finally, the proposed control plane approach would

provide coherent semantics for network management and operations

control in hybrid optical internetworking systems consisting of LSRs

and OXCs.

References:

[1] D. Awduche, J. Malcolm, J. Agogbua, M. O'Dell, and J.

McManus,"Requirements for Traffic Engineering Over MPLS," RFC-

2702,September 1999.

[2] D. Awduche, "MPLS and Traffic Engineering in IP Networks," To

Appear, IEEE Communications Magazine, December, 1999.

[3] D. Awduche, L. Berger, D. Gan, T. Li, G. Swallow, and V.

Srinivasan, "Extensions to RSVP for LSP Tunnels," Internet Draft,Work

in Progress, 1999

[4] B. Jamoussi et al, "Constraint-Based LSP Setup using

LDP,"Internet Draft, Work in Progress, 1999

[5] ITU-T G.872, "Architecture for Optical Transport Networks," 1999.

[6] R. Callon, P. Doolan, N. Feldman, A. Fredette, G. Swallow,A.

Awduche/Rekhter et al                                          [Page 18]

draft-awduche-mpls-te-optical-01.txt          May 2000

Viswanathan, "A Framework for Multiprotocol Label Switching,"Internet

Draft, Work in Progress, 1999

[7] E. Rosen, A. Viswanathan, R. Callon, "Multiprotocol Label

Switching Architecture," Internet Draft, Work in Progress, 1999

[8] T. Li, G. Swallow, D. Awduche, "IGP Requirements for Traffic

Engineering with MPLS," Internet Draft, Work in Progress, 1999

[9] H. Smit and T. Li, "IS-IS extensions for Traffic

Engineering,"Internet Draft, Work in Progress, 1999

[10] D. Katz, D. Yeung, "Traffic Engineering Extensions to

OSPF,"Internet Draft, Work in Progress, 1999

[11] Y. Rekhter and T. Li, "A Border Gateway Protocol 4 (BGP-

4),"RFC-1171, March 1995

[12] ATM Forum, "Private Network-Network Interface Specification:

Version 1.0," March 1996

[13] B. Mukherjee, "Optical Communications Networks," McGraw-Hill,

1997.[14]

[14] G. Newsome and P. Bonenfant, "The Automatic Switched Optical

Network," Contribution to T1 STANDARDS PROJECT - T1X1.5,  1999.

[15] P. Bonenfant and and X. Liu, "A Proposal for Automatically

Switched Optical Channel Networks (ASON)," Contribution to T1

STANDARDS PROJECT - T1X1.5, 1999.

[16] "T. Worster, "General Switch Management Protocol," Internet

Draft, Work in Progress, 1999.

[17] E. Crawley, R. Nair, B. Rajagopalan, and H. Sandick, "A

Framework for QoS-Based Routing in the Internet," RFC-2386, August,

1998.

Security Considerations

It is imperative to guarantee the integrity and confidentiality of

control information used by the proposed OXC control plane. This can

be accomplished by using existing security mechanisms for the various

components of the MPLS traffic engineering control plane model.

Awduche/Rekhter et al                                          [Page 19]

draft-awduche-mpls-te-optical-01.txt          May 2000

Author's Addresses

Daniel O. Awduche

UUNET (MCI Worldcom)

22001 Loudoun County Parkway

Ashburn, VA 20147

Phone: 703-886-5277

Email: awduche@uu.net

Yakov Rehkter

Cisco Systems

170 W. Tasman Dr.

San Jose, CA 95134

Email:  yakov@cisco.com

John Drake

Fore Systems (Marconi)

1000 FORE Drive

Warrendale, PA  15086-7502

Phone:  724-742-6798

EMail:  drake@fore.com

Rob Coltun

Siara Systems

300 Ferguson Drive

Mountain View, CA 94043

Phone: (650) 390-9030

Email: rcoltun@siara.com

Awduche/Rekhter et al                                          [Page 20]