Contribution Number: 	OIF99.160

Working Group:	Architecture

TITLE:  UNI requirements for topology discovery.

SOURCE:	Greg Bernstein & Theodore Tedijanto

Ciena Corporation, Core Switching Division

10201 Bubb Road

Cupertino, CA 95014

(408) 865-6213

Fax: (408) 865-6292

Email: greg@ciena.com

DATE: December 3, 1999

ABSTRACT:

This contribution addresses the area of topology discovery at the UNI. It suggests a minimalistic set of parameters and conditions needed for topology discovery and a method for their conveyance compatible with the options described in OIF99.136.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

Introduction

Topology discovery at the User Network Interface (UNI) eases the provisioning of protection groups in rings and linear automatic protection switching (APS) and is essential for accurate and rapid provisioning of circuits.  Topology discovery answers the questions (a) who is in the network, (b) how are they connected (details of which port is connected to which port), and (c) with what are they connected (i.e., the type of interface, e.g., OC-48 at a specific ).  Resource utilization, e.g., number of active paths in an OC-48 or available s in a DWDM span, is an important secondary function that maps resource attributes to the node and link information obtained via topology discovery.

Topology discovery has been traditionally performed by network management applications or routing protocols.  Interoperability is one problem with network management for topology discovery. The other is that across the UNI the equipment may lie in two different network management domains, i.e., no access for network management may be given across the UNI. Routing protocols have done a good job of network discovery but do not extend to the UNI, i.e., they would want to add drop side links since this does not add useful information and increases the size of topology databases and update traffic bandwidth.

Typically presence of an end system across a UNI is the most that communication protocol have provided (10BaseT, ISDN).  ATM introduced a true UNI discovery protocol within its Integrated Local Management Interface (ILMI). However, ATM’s ILMI was as much needed for dealing with a host of ATM parameter constraints that each side must honor such as number of VPI bits, number of VCI bits, max VCs, etc., as it was needed for topology discovery. This resulted in a rather large information base (information potentially exchanged between the two sides), a management protocol (SNMP) for exchanging this information, a data communication protocol (AAL-5) for transporting management protocol, and finally a reserved (virtual) channel for the transporting this information. Needless to say this probably isn’t the approach to be taken if we want the rapid adoption of a UNI topology discovery mechanism.

Minimalistic Topology Discovery for the UNI

In figure 1 we show the relationships between an end system, its corresponding network equipment, and the network.

Figure 1 Network relationships

For topology discovery we can break up the discussion into the type of information to be transferred and then how it may be conveyed. The following information is either crucial or very helpful in the topology discovery process:

- Node Identifier – This is a number that uniquely identifies a node in a network.  Later we may include a node name but we consider a node identifier more fundamental. To later support rapid provisioning this will be an address of some form, such as the NSAP format used in ATM [3].  In one possible addressing scenario the network equipment contributes to the “network portion” of the address so this identifier or address only needs to be unique amongst those end systems connected the same network equipment.  Typically within a NNI discovery protocol end system addresses attached to a network node are summarized and then advertised throughout the routing domain.

- Port Identifier – This number (or name but we recommend a number) uniquely identifies the transmit port on either side of the UNI. This information is very helpful in the configuration of linear APS ring groups or rings.  This information is also important in conventional and rapid provisioning.

- IP Address – Although not as important as the preceding information. An IP address is very useful in the case where the both sides of the UNI fall under the same management domain.  In addition, in the case of “transparent option” from reference [1] this address and the previous information can be used to establish an out of band signaling channel for use in end system based rapid provisioning.

- Managed Element Name – A text string identifying the node. Used for network management purposes. This information, like the IP address, would be optional.

Methods for Conveying Topology Information

In contribution OIF99.136 [1] three options for optical interworking interfaces were described. Each of these options gives successively more choices and bandwidth in both directions across the UNI.

Option 1 SDH data element interfaces, SDH protocol transparency in optical transport network

In this option the network side of the UNI can only passively monitor the SDH/SONET section level overhead. It cannot insert information into any of the overhead bytes, e.g, J0 or B1.  This leads to only a one way in-band transfer of topology information from the user side to the network side. Figure 2 shows how this transfer works and how the network side will have complete topology information.

Figure 2. Topology information flow in the passive monitoring case.

In the example of figure 2 topology information (Node ID, Port #) will be conveyed in-band (e.g. J0 byte) on each of the links comprising the UNI. Since option 1 only permits passive monitoring the information flows from the user side to the network side. After the information transfer the network side of the UNI will have the following connectivity (topology) information:

Table 1. Connectivity information at the network side for Figure 2.

| Node ID (User) | Port ID (User) | Node ID (Network) | Port ID (Network) |
| --- | --- | --- | --- |
| 1701 | 1 | 2112 | 3 |
| 1701 | 3 | 2112 | 7 |
| 1701 | 4 | 2112 | 1 |
| 1701 | 12 | 2112 | 2 |

Note that wavelengths and transmission rates would be explicitly known at the network side for each of these links.

A prime candidate for conveying this information is the Section Trace byte J0. Existing requirements documents such as GR-253 [2] indicate that “detailed criteria concerning the use of these bytes… is for further study”. Reference [2] indicates that support for J0 access on drop side interfaces is not required but that access to J0 bytes for line side is optional. This is in contrast to the STS path trace byte J1, which in reference [2] is used to convey a 64 byte string or in ITU-T SDH standards a 16 byte identifier using E.164 format.

For the reverse direction, i.e., from network to user, the only option is to set up an out of band communications channel.  If the user sides IP address is included with the topology information then the network side can initiate a procedure to setup an out-of-band communications channel.  Note that for accurate and up to date topology information basic connectivity information must be conveyed in-band at the port level in at least one  direction.

The disadvantage of the section trace is that it may be over written by any regenerator that happens to be between the two equipment. It would be preferrable to use line level overhead for this function. Currently the only general purpose communications mechanism at the line layer is the line data communications channel (DCC). Using the line DCC would place quite a processing burden on the network side of the optical interworking interface (HDLC and higher layer processing). Hence this approach is less desirable. In the future, option 3 of [1], we may want to investigate the use of the line overhead Growth bytes Z1 for use as a “line trace”.

Option 2 SDH data element interfaces, SDH termination in optical transport network

In the case of section terminating equipment on the network side we can also consider using section layer DCC to convey topology information.  However for compatibility with option 1 we will most likely want to utilize the same mechanism but provide for two-way information flow. Note that the network to user direction for topology information flow is not as important to overall network topology discovery, but it can allow the user to place provisioning requests and trouble reports in terms of “network centric” parameters.

Conclusion

Support for the in-band conveyance of two parameters, node ID and port ID, in the user to network direction across the UNI allows for fairly complete automated network topology discovery. An additional optional third parameter, the equipment’s IP address, would be helpful for both network management and the future possibility of an out-of-band communications channel for reverse topology information flow, rapid provisioning signaling and possibly other uses. The use of the section trace byte (J0) using a format similar to that used for the STS path trace byte (J1) was also discussed.

Motions

- Auto-discovery (topology discovery) should be a recommended UNI function under all three of the interface options of reference [1].

- The recommended approach to interoperable topology discovery at the UNI consists of the in-band conveyance of the two parameters node ID and port ID in the user-to-network direction.

- The in-band conveyance of the two previous topology parameters also allows for the conveyance of an optional third parameter, the node’s IP address.

- For the “transparent option” of reference [1], the section trace byte, J0, be used to convey the topology information in a format similar to that of the STS path trace with details for further study.

References

[1] Joseph Berthold, “Optical Internetworking interfaces based on SDH/SONET”, OIF99.136.

[2] Synchronous Optical Network (SONET) Transport Systems: Common Generic Criteria, GR-253-CORE, Issue 2, December 1995.

[3] Raif O. Onvural, Rao Cherukuri, Signaling in ATM Networks, Artech House, Inc., 1997.