Contribution Number: 	OIF00.013

Working Group:	Architecture

TITLE:  UNI requirements and mechanisms.

SOURCE:	Greg Bernstein

Ciena Corporation, Core Switching Division

10201 Bubb Road

Cupertino, CA 95014

Email: greg@ciena.com

Larry McAdams

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134

Email:lmcadams@cisco.com

DATE: January 31, 2000

ABSTRACT:

This contribution addresses optical internetworking interface requirements and mechanisms. These requirements include the need for end system identification/registration (discovery), parameters to be signaled for rapid provisioning of optical and/or SONET/SDH layer channels, and enhanced circuit routing via additional optical layer topology information. The mechanisms include methods for end system discovery, signaling channel establishment, and signaling conveyance.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

# Introduction

This contribution addresses the requirements for optical internetworking between end system (routers, switches and muxes) and an optical network and discusses possible signaling mechanisms for their realization. OIF00.007 summarizes previous approved motions and proposed requirements for the interface.  This contribution seeks to add another level of detail to these requirements and suggest some possible implementation choices.  In addition this contribution looks at the ways in which various interface options can be combined and utilized.

The requirements for an optical internetworking interface are driven by demands for: (a) rapid provisioning, (b) easier network inventory and management, and (c) enhanced reliability and reliability options. These general requirements have led us to the following areas of interface functionality: end system identification/registration, connection control signaling, signaling channel establishment, connection parameter information, and exchange of topology information, in particular from the optical layer. This contribution reviews these areas of interface functionality and possible mechanisms for their implementation.

# End System Discovery

## Introduction

Understanding who is connected at the other side of a link is a precursor to allowing the optical network to interact with end systems.  This simply means that both devices recognize each other and maintain information concerning the state of links between the two devices.

## Parameters for End System Discovery

In figure 1 we show the relationships between an end system, its corresponding network equipment, and the network.

Figure 1 Network relationships

For end system discovery we can break up the discussion into the type of information to be transferred and then how it may be conveyed. The following information is either crucial or very helpful in the end system discovery process:

- Node Identifier/IP Address – This is a number that uniquely identifies a node in a network.  Later we may include a node name but we consider a node identifier more fundamental. To support rapid provisioning this will be an address of some form, such as the IP address. Typically within a NNI discovery protocol end system addresses attached to a network node are summarized and then advertised throughout the routing domain.

- Port Identifier – This number (or name but we recommend a number) uniquely identifies the transmit port on either side of the UNI. This port identifier could also be an IP address.

- Managed Element Name – A text string identifying the node. Used for network management purposes. This information would be optional.

## Methods for Conveying Discovery Information

In contribution OIF00.007 three options for the optical interworking UNI are described. We include them here to clearly show how the methods we propose relate to these options. Each of these options gives successively more choices and bandwidth in both directions across the UNI.

Option 1 Physical Layer Regenerator Circuit (PLR-C)

In this option the network side of the UNI can only passively monitor the SDH/SONET section level overhead. It cannot insert information into any of the overhead bytes, e.g, J0 or B1.  This leads to only a one way in-band transfer of topology information from the user side to the network side. Figure 2 shows how this transfer works and how the network side will have complete topology information. OIF99.160 gives more in depth discussion of this case.

Figure 2. End system information flow in the passive monitoring case.

For the reverse direction, i.e., from network to user, the only option is to set up an out of band communications channel.  If the user sides IP address is included with the topology information then the network side can initiate a procedure to setup an out-of-band communications channel.  Note that for accurate and up to date topology information basic connectivity information must be conveyed in-band at the port level in at least one  direction.

Option 2 STE Circuit (STE-C)

In the case of section terminating equipment on the network side we can also consider using section layer DCC to convey topology information.  However for compatibility with option 1 we will most likely want to utilize the same mechanism but provide for two-way information flow. Note that the network to user direction for end system information flow is not as important to overall network topology discovery, but it can allow the user to place provisioning requests and trouble reports in terms of “network centric” parameters.

Option 3 LTE Circuit (LTE-C)

In the case of multiplex-section (line) terminating equipment on the network side we can also consider using the line layer DCC to convey topology information.  This has the advantage of not being "swallowed" by section level regenerators.

# Signaling Channel Establishment

The signaling channel is used to convey control information, e.g., setup and tear down requests between the end system and the network.  Note that the signaling channel can only operate on end systems that have either been discovered or configured, i.e., those that are either automatically or manually configured as to their connectivity to the network. The signaling channel may be via a separate mechanism between the end system and the network equipment. This channel may be either in-band or out-of-band.

To run through the options for signaling channel establishment we can look at whether the signaling channels are manually or automatically configured and whether they run "in-band" or "out-of-band".

An additional complication arises when it is desired that a single signaling channel carry signaling traffic for multiple interconnected ports. Figure 3 illustrates the case where the network and the end system are connected via four links. Only one of these links is actually carrying signaling traffic, but this signaling traffic is applicable to all interfaces between the two systems. See options 1 and 2 below.

Figure 3. Multiple links between end system and network only one carrying signaling.

- Manually configured in-band signaling: In this case for each SONET/SDH line (or optical channel) whether signaling information is conveyed across the in-band channel is manually configured.  If this in-band channel were to convey signaling information for other SONET/SDH lines (or optical channels) the port to port connectivity information would need to be manually configured.

- Automatically configured in-band signaling: This case is similar to the above in the single line signaling mode, i.e., one in-band signaling channel per line, except that it starts, by default, with the in-band signaling channel enabled.  In the multiple line per one in-band signaling mode, i.e., multiple lines share one in-band signaling channel this mode would automatically configure this in-band signaling channel, i.e., would select which lines in-band communication channel out of the multiple would be used, ensure that port to port correspondences where known, etc…

- Manually configured out-of-band signaling: In this case an out-of-band signaling channel is manually configured.  For example via a management system a signaling association between the user and network sides of the UNI is established along with sufficient topology information, i.e., which ports on the end system are connected to which ports on the network, to allow meaningful signaling interactions to take place. One approach would be to run a signaling protocol on TCP over an out of band IP network connected to both sides of the UNI. Advantages: Doesn't require any in band communications and may be only option available.  Disadvantages: Requires out of band IP network; Mixes two potentially separate IP domains; manual configuration of port connectivity information can be error prone.

- Automatically configured out-of-band signaling: In this case information gained via end system discovery  protocol  as discussed in the previous section is used to "bootstrap" an out-of-band signaling channel.  This information at the network side of the UNI can allow the network side to initiate the creation of a signaling channel (perhaps via exchanging info on previously agreed upon UDP or TCP port).

The above options can be combined with the UNI services defined in OIF00.007 in the following manner:

- Physical Layer Regenerator Circuit (PLR-C)

Since only passive monitoring of SONET/SDH overhead is available, signaling must be out-of-band since there isn't a two-way in-band channel available. As discussed in the previous section the section trace or other overhead can be monitored by the Network side of the UNI to learn about the User side port number and IP address and use that to automatically configure an out of band signaling channel.

- STE Circuit (STE-C)

At this point the section DCC may be available for use for both end system discovery and in-band signaling.  Multiple lines between the end system and the network could share such an in-band channel.  The mechanisms defined for the PLR-C case still apply in this case.

- LTE Circuit (LTE-C)

At this point the line DCC may be available for use for both end system discovery and in-band signaling.  Multiple lines between the end system and the network could share such an in-band channel. The mechanisms applicable to the STE-C and PLR-C case also apply in this case.

# Signaling Parameters and Operations

The fundamental actions or operations to be conveyed over the signaling channel are: (a) channel or light path creation, (b) channel or light path tear down, and (c) channel or light path query, i.e., obtain current status information about the channel or light path.  To allow these operations to take place various required and optional parameters need to be conveyed in the signaling list.  The following lists some of these parameters their meaning and use.

## Endpoint Identification

End point identification is essential in identifying where the channel or light path is going! The following parameters are defined for this purpose. Note that this differs from end system discovery, i.e., end system discovery is concerned with finding out who is connected to the network while end point identification is how one specifies the endpoints of a connection.

IP address –The IP address will be used to identify the end system and/or possibly a port on the end system. This may be all the end point information required depending on the element addressing strategy.

Port index – If individual ports are not given IP addresses on an end system then a port index is required to identify the particular desired port on an end system.

Sub-Port index – When the network or end system allows multiplexing or switching at a finer granularity below the port level then the sub-port index is used to refer to specific channels below the port level. In future, more highly integrated systems there may be a need for additional levels of sub-port indexes.

## Light Path Attributes

Below is a list of the possible attributes a light path, or channel, through the optical network may exhibit.  It is envisioned that these attributes would be negotiated over the signaling channel between the end system (router, switch, mux) and the OXC to which they are connected.  In some cases not all attributes may be available, depending on the services offered and the equipment available.

Encoding – This attribute describes the line protocol carried on the light path.  Examples of such protocols includes SONET/SDH and Ethernet (GE and 10GE).  This is a required attribute.

Capacity – This attribute defines the bandwidth associated with the light path.  For example, a light path used to transport SONET encoded signals may be configured to have OC-N capacity, or STS-M capacity.  This is a required attribute.

Pre-emptability – This attribute determines a light path’s priority during a restoration event.  It is related to, but independent from the method of protection used.  It may be as simple as deciding whether a light path can carried on the optical network’s “protection” bandwidth.  Alternatively in an optical network with 100% “working” bandwidth, it can be a multi-level attribute to define which light paths can pre-empt others.  The intent in either case is to provide a hierarchy for determining the order light paths get restored when bandwidth is limited.  A simple form of this attribute is required, while more elaborate forms are optional and may require further study.

Protection  - This attribute determines the type of protection the optical network provides for the light path.  Several options exist for this attribute.  The simplest is a binary decision; is the light path protected or not.  However, some users may wish to go further and define how long the restoration will take (e.g. 50 msec, 250 msec, 2 seconds, etc.).  In its finest granularity, the protection attribute would allow user to define the type of protection desired.  For example, shared versus dedicated protection, or even the specific preferred implementation such as linear (1+1, 1:1, 1:N), ring (UPSR, BLSR), or mesh (gold, silver, bronze).  As with the preemption attribute, a simple form of the preemption attribute is required, while more elaborate forms are optional and may require further study.

Delay – This attribute specifies the temporal characteristics of the light path.  There are a number of ways to characterize the temporal behavior.  A simple metric is the total round trip delay; however, it may be desirable to create a more detailed description of the temporal behavior.  For example, in some cases, the differential delay between the incoming and outgoing directions of the light path may be important.  Finally, the change in delay caused by a protection event may also be important.  In other words, it may be desirable to not only establish the temporal behavior of the “working” light path, but also the temporal behavior of the light path after it has been restored.  This attribute is optional and may require further study to define the more advance delay metrics.

Light path Bundling - This attribute allows multiple physical light paths to be treated as one logical light path.  The attribute can be used in several ways.  One purpose is to allow end systems (router, switch, mux) to communicate over the optical network without regard to the underlying physical limitations (e.g. capacity) of the optical circuits supported by the particular optical network.  Another purpose is to ensure that a set of light paths is treated uniformly during a protection event.  This attribute is optional now, but may become required if the demands for bandwidth from end systems outstrip the capacity of the circuits in the optical network.  Further study may be needed to fully define this attribute.

Diversity – This attribute provides a means for ensuring a level of diversity between two or more light paths.  The diversity may take a number of forms.  The light paths may be diverse in the sense that they are carried in separate fibers, separate cables, separate conduits, or even entirely separate routes through the optical network.  This attribute is optional and the specific options supports may require further study.

Availability – This attribute allows a user to request a light path with a known availability, or uptime.  Such capability may be desirable in defining specific service offerings.  Typical metric for this attribute would include, for example, 99.999% availability.  This attribute is optional.

# Topology Exchange

In addition to the light path attributes and actions described above, it may also be desirable to allow end systems to obtain more detailed information about the state and topology of the optical network over the signaling channel.  The exchange of this information can provide several benefits.

- Tighter integration between optical network elements and end systems.

- Ability of end systems to better optimize light path selection.

- Opportunity for a common, open control plane for both end systems and optical networking elements.

As a foundation for the migration from an overlay model towards a peer model, we propose two additional actions.

Query Topology and State of Light paths in Use – This action is used by an end system to obtain topology and state information for all the active light paths in the optical network.  It is envisioned that this information will be conveyed using a standard data structure.  The details of this exchange are the subject of further study, but some form of this action is required to implement a peer model.

Query Topology of Unused Resources/Fibers – This action is used by an end system to obtain topology information about the unused resources within the optical network, such as available fibers, lambdas, and circuits. It is envisioned that this information will be conveyed using a standard data structure.  The details of this exchange are the subject of further study, but some form of this action is required to implement a peer model.

Connectivity information at the network side for Figure 2.

# Project Definition

The intent of this proposal is to stimulate the formation of, and contribute technical material to a new project to define the UNI between end systems (routers, switches, muxes) and network equipment comprising the optical network.  Towards this end we offer the following project definition.

Project Name: Requirements and Signaling Mechanisms for the UNI

Set of Working Groups Involved: Architecture and (proposed) Signaling groups

Problem Statement: Signaling across a UNI is needed to allow end systems to inter-operate intelligently with an optical network

Scope:  This project will include the requirements and signaling mechanisms needed to define the UNI and other related interfaces between the end systems and the optical network.

Expected Output: Technical documents defining the requirements and signaling mechanisms for implementing the UNI and the stimulation of follow-on work .

- Technical document to define one or more mechanisms for end system identification.  At least one of these mechanisms will provide for automated in-band configurations over SONET links.

- Technical document to define the requirements for light path establishment, including the desired light path attributes.

- Technical document on the potential benefits of supporting topology exchange between end systems and the optical network, resulting in a recommendation to expand the UNI to support this capability, or not.

Proposed Schedule: Draft document by late summer, OIF approved document by end of year.

# Motion

Based on the ideas and technical information contained within this document, we recommend the technical committee approve the above project.

References

[1] Joseph Berthold, “Optical Internetworking interfaces based on SDH/SONET”, OIF99.136.

[2] Synchronous Optical Network (SONET) Transport Systems: Common Generic Criteria, GR-253-CORE, Issue 2, December 1995.

[PPP] The Point-to-Point Protocol (PPP), Network Working Group, W. Simpson, Request for Comments: 1548, December 1993.

[IPCP] The PPP Internet Protocol Control Protocol (IPCP), Network Working Group, G. McGregor, Request for Comments: 1332, May 1992.

[OIF99.160] Greg Bernstein, Theodore Tedijanto, "UNI requirements for topology discovery", December 3rd, 1999 (interim meeting).