Contribution Number:

Working Group: Signaling, Architecture

TITLE:  Some Routing Constraints

SOURCE:  Monica Lazer And John Strand

AT&T

Email: mlazer@att.com, jls@research.att.com

DATE: May 26, 2000

ABSTRACT: This contribution discusses three topics likely to effect routing in the optical network: (i) Wavelength routed networks, (ii) Diversity, and (iii) Restoration.

________________________________________________________________________________

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

Some Routing Constraints

Monica Lazer* and John Strand+

*AT&T, 900 Route 202/206N , Bedminster, NJ 07921, mlazer@att.com

+AT&T Labs – Research, 100 Schulz Drive, Red Bank, NJ 07701, jls@research.att.com

# Introduction

It is important from a services perspective that Optical Layer constraint based routing algorithms are indeed applying proper constraints and are also operating on an appropriate model of a real optical network. This contribution discusses three important topics which are likely to effect the Optical Layer’s constrained routing architecture. The topics covered are: (i) Wavelength-Routed Networks, (ii) Diversity, and (iii) Restoration.  Determining how best to deal with requirements arising from these three areas should throw light on another important topic: The proper relationships between the new control plane, the network operator’s Management Plane (e.g., OS’s), and Element Management Systems.

Although strawman requirements are proposed for each topic covered, we realize that there are other points of view which need to be integrated before stability is reached.  To this end, procedural motions are proposed in a final section that hopefully will help the OIF provide much needed direction in these areas.

# Wavelength Routed Networks

A wavelength routed network routes and switches optical signals (wavelengths) based on their wavelengths. (See [1], Chap. 8 for an introductory discussion.)  These networks can be classified according to the degrees of transparency and wavelength conversion that they provide.  At one extreme are “Opaque Networks”, which optically isolate each network element. (See [2].)  This is achieved using electronic signal regeneration - either using electronic switching fabrics or transponders.  The introduction of electronic signal regeneration provides wavelength conversion, allowing any input wavelength to be converted to any output wavelength.  Electronic switching also allows TDM, so that a single wavelength may carry multiple "channels", each swtiched independently. This sort of architecture, which imposes no optically driven routing constraints, is implicitly or explicitly assumed in most optical layer control plane discussions.  This is appropriate since this architecture is what is currently being deployed.  However transparent all-optical (sub)networks may be implemented without wavelength conversion.  Also, the lack of signal regeneration will result in signal quality being degraded over long distances, and passing through optical cross-connects.  The limitations introduced by all-optical networks and due to the lack of wavelength conversion need to be considered.

## All-Optical Subnetworks Without Wavelength Converters ("Domains of Transparency")

Technical limitations (see [2])  limit the scalability of all-optical networks and point toward national-scale networks being composed of a number of such subnetworks, optically isolated from each other. At present all-optical wavelength conversion is not practical, so in this section “subnetwork” will refer to an all-optical subnetwork without wavelength converters, which are also frequently called "Domains of Transparency".

From a routing perspective there are two critical aspects of such a subnetwork:

- Adaptation: These are the functions done at the edges of the subnet which transform the incoming optical channel into the physical wavelength to be transported through the subnet.

- Connectivity: This defines which pairs of edge Adaptation functions can be interconnected through the subnet.

To explore these considerations more specifically, we consider two simple examples, which are modeled on announced products. First, an optically isolated optical transport system with an Optical Add/Drop Multiplexer (OADM)

Figure 2-1 OADM Functional Architecture

In this figure D and E are DWDM’s and X and Y are OADM’s. The boxes labeled “A” are adaptation functions. They map one or more input optical channels (the ’s, here assumed to be standard short reach (SR) signals) into a long reach (LR) wavelength or wavelength group which will pass transparently to a distant adaptation function.   Adaptation functionality which affects routing includes:

- Multiplexing: Either electrical or optical TDM may be used to combine the input channels into a single wavelength.  This is done to increase effective capacity:  A typical DWDM might be able to handle 100 2.5 Gb/sec signals (250 Gb/sec total) or 50 10 Gb/sec total); combining the 2.5 Gb/sec signals together thus effectively doubles capacity. After multiplexing the combined signal must be routed as a group to the distant adaptation function.

- Channel Grouping: In this technique, groups of k (e.g., 4) wavelengths are tightly spaced, with wider spacings between groups. Wavelength groups are then managed as a group within the system and must be added/dropped as a group.

- Laser Tunability: The lasers producing the LR wavelengths (the i in the figure) may have a fixed frequency, may be tunable over a limited range, or be tunable over the entire range of wavelengths supported by the DWDM.  Tunability speeds may also vary.

Connectivity between adaptation functions may also be limited:

- As pointed out above, TDM and/or channel grouping by the adaptation function forces groups of input channels to be delivered together to the same distant adaptation function.

- Only adaptation functions whose lasers/receivers are tuned to compatible frequencies can be connected.

- The switching capability of the OADM’s may also be constrained.  For example:

- There may be some wavelengths that can not be dropped at all.

- There may be a fixed relationship between the frequency dropped and the physical port on the OADM to which it is dropped.

- OADM physical design is likely to put an upper bound on the number of wavelengths dropped at any single OADM.

Thus, for a fixed configuration of the OADM’s and the adaptation functions connectivity between the DWDM’s and OADM’s will be fixed: Each input port will essentially be hard-wired to some specific distant port.  However this connectivity can be changed by changing the configurations of the OADM’s and adaptation functions. For example, a different channel group might be dropped or a tunable laser be retuned. In each case the port-to-port connectivity is changed.

This capability can be expected to be under software control. Today the control would rest in the vendor-supplied Element Management system, which in turn would be controlled by the operator’s OS’s.  However in principle it could be under real-time control of the routing process. The constraints on reconfiguration are likely to be quite complex, dependent on the vendor design,  and also on exactly what line cards, etc. have been deployed.

Systems with OADM's such as those described above should be supported by the proposed control plane.  Requirements are:

Proposed Requirements:

- Routing constrained by TDM and channel grouping shall be supported.

- Routing constrained by system-specific constraints on adaptation connectivity shall be supported.

- Management plane/EMS reconfiguration of system connectivity between adaptation functions shall be supported. Convergence timeafter a such a reconfiguration shall not exceed TBD.

Control plane reconfiguration control would also be desirable but raises a number of issues:

- The constraints defining possible connectivity are vendor-specific and in rapid flux. They also can depend on details of the specific line cards installed, e.g. the capabilities of the lasers and filters. Thus the state information that would need to be disseminated is likely to be voluminous, possibly vendor specific, and likely to be hard to pin down.

- Line systems will frequently be bought from a vendor other than those being used for OLXC’s.

However it is very desirable to solve these issues, possibly by treating the total configuration as a separate domain or OSPF area as discussed in ([Ref]).

The second simple example is based on the architectures of announced Optical Transport Systems with an integrated optical cross-connect.  The architecture is illustrated in the following Figure:

(a) System View			      (b) Optical Cross-Connect						Figure 2-2:  OTS Functional Architecture

The system is optically isolated with transponders placed at the boundary. Within, it is optical without wavelength conversion.  The OXC’s may be envisioned as a stack of N (the number of wavelengths) small transparent cross-connects each operating at a single frequency.  From a routing perspective, this system can be thought of as N separate networks, one for each wavelength, with no internal connectivity between them. A “tree” architecture is shown, but more complex meshes might be constructed.

The distinguishing characteristics of such a “domain of transparency” relevant to routing are:

- Routing in such a network is constrained by the accumulation of impairments such as noise, polarization mode dispersion and chromatic dispersion. The constraints vary based on the bit-rate of the wavelength and the characteristics of the physical fiber being used.  Their impact is also affected by vendor design choices such as wavelength spacing, power levels, modulation methods, and Forward Error Correction methods used ([3]). Both the physical impairments and the specific vendor design choices need to be considered in routing decisions and the state information describing both impairments and design choices needs to be available to the routing algorithm.

- The lack of wavelength conversion changes the nature of the routing problem because a common wavelength must be available on all the links used in the DOT. Surveys of the algorithmic alternatives  may be found in ([1] or [5]). It is generally accepted that wavelength conversion does not provide significant performance improvements in rings ([5]).  However, recent simulations of routing algorithms in a large national-scale optical mesh without wavelength conversion ([4]) indicate that use of a shortest path first (SPF) algorithm followed by a wavelength selection procedure along the selected path leads to badly non-optimal network utilization. The quantitative results obtained are summarized in the following figure:

Figure 2-3: Effect Of Number Of Paths On Utilization

The vertical axis is a measure of the additional bandwidth needed when compared  with a network with full wavelength conversion; thus a value of 2.0 indicates that twice as much bandwidth is needed. As can be seen, much better results were obtained when a large number (over 5) distinct paths were selected and then a simultaneous choice  is made of one of these paths and the specific wavelength to use along the selected path.

This suggests that significantly different routing procedures will be needed for sub-networks without wavelength conversion. The algorithms tested assume that detailed information about the state of every wavelength on every link in the routing domain is available. Chaudhuri et al ([6]) point out that propagating this information to every node in the domain would not scale.  Instead they propose an alternative method which probes along a chosen path to determine which wavelengths are available. This suggests that additional enhancements are required, perhaps involving probing along multiple candidate paths.

Proposed Requirements for routing in a subnetwork without wavelength conversion

- The routing algorithms used shall generate routes which meet transmission constraints and are sensitive to vendor design choices.

- The routing algorithms used shall make effective use of the available bandwidth.

- The routing algorithms and associated propagation of link state information need to scale.

## Subnetworks With Limited Wavelength Conversion

A major attraction of all optical networks without wavelength conversion is the avoidance of wavelength conversion costs, which are very substantial with today's technology. However, as noted in the previous section the lack of wavelength conversion is likely to significantly impact network utilization and efficiency in large mesh networks unless  significant routing changes are made.

It is premature to speculate on the routing impacts of the introduction of limited wavelength conversion until specific technological proposals are on the table; however it is probably safe to say that at a minimum routing procedures that treat wavelength conversion as an expensive, constrained, and topologically sparse capability are likely to be needed.

# Diversity

A number of topics related to diversity are discussed in Section 3.1.  Requirements suggested by these discussions are summarized in Section 3.2.

## Facets Of Diversity

Diversity Is A Network, Not Link Property

Diversity is often equated to routing two lightpaths between a single pair of points so that no single route failure will disrupt them both. This simple formulation is inadequate for many networks in several respects.

First, a sophisticated network manager will want to derive diversity needs from his/her end customers’ availability requirements. These often lead to more complex diversity requirements than simply providing diversity between two lightpaths.  For example, a common requirement is that no single failure should isolate a node or nodes. Consider the following examples:

Figure 3-1: Diversity Examples

In the 4 node/5 link example on the left the dashed lines indicate which links need to be diverse. The matrix underneath gives the same information in a different form; a “1” in position (i,j) means that circuits i and j must be diverse.

Figure 3-1(b) shows a more complex network. In this case the X-Y and U-V links need to be diverse, in addition to whatever constraints are needed within the subnetworks.

Note that there is no transitivity – for example, in Fig. 3-1(b) A-C must be diverse from C-D and C-D from B-D but A-C need not be diverse from B-D.

These examples are representative of what many sophisticated data networks require today.  The popular AT&T AccunetTM  Enhanced Diversity Routing Option (EDRO) is an example of a service providing this level of diversity.

It is frequently important to achieve the desired diversity with as few constraints as possible.  This is due to the generally sparse nature of the graph describing fiber route connectivity.  In an intercity network in the U.S., most nodes will have nodal degree 2 (i.e., there will be just two fiber routes from the node). In Fig. 3-1(a), if node A has degree 2 it would not be possible to have its 3 circuits mutually diverse.

Diversity constraint matrices  such as the one shown in Figure 3-1(a) express diversity constraints as binary relationships: 2 circuits should not fail simultaneously.  This does not always capture user requirements adequately: For example, in Fig. 3-1(b) if there were N circuits connecting the two subnets the requirement might be that at least K<N survive any single failure.  If K=N-1 the diversity matrix formulation works, but if  1 < K < N-1 this formulation is overly constraining.

Algorithms For Diverse Routing

Problems of the sort illustrated in Fig. 3-1 are NP-hard ([7]) when the diversity requirements are "hidden constraints" such as Shared Risk Link Groups (see next section). The heuristics that the authors are aware of require the generation of a very large number of shortest paths under different constraints to have reasonable confidence that a "good enough" solution has been found.  This would appear to make them unsuited for running in a real-time environment.

Diversity Is In The Eye Of The Beholder

Two circuits that might be considered diverse for one application might not be considered diverse for in another situation. Diversity is usually thought of as a reaction to interoffice route failures.  High reliability applications may require other types of failures to be taken into account. Some examples:

- Office Outages: Although less frequent than route failures fires, power outages, and floods do occur.  Many network managers require that diverse routes have no (intermediate) nodes in common.  In other cases an intermediate node might be acceptable as long as there is power diversity within the office.

- Shared Rings: Many applications are willing to allow "diverse" circuits to share a SONET ring-protected link; presumably they would allow the same for optical layer rings.

- Natural Disasters: Earthquakes and floods can cause failures over an extended area.  Defense Department circuits might need to be routed with nuclear damage radii taken into account.

Conversely, some networks may be willing to take somewhat larger risks.  Taking route failures as an example: Such a network might be willing to consider two fiber cables in heavy duty concrete conduit as having a low enough chance of simultaneous failure to be considered “diverse”.  They might also be willing to view two fiber cables buried on opposite sides of a railroad track as being diverse because there is minimal danger of a single backhoe disrupting them both even though a bad train wreck might jeopardize them both.

These considerations strongly suggest that the routing algorithm should be sensitive to the types of threat considered unacceptable by the requester.

Inter-Office Configurations

We will use the following terms: A fiber cable is a uniform group of fibers contained in a sheath.  Such a cable will be placed in a sequence of conduits – buried honeycomb structures through which fiber cables may be pulled – or  buried in a right of way (ROW).  A ROW is land in which the network operator has the right to install his conduit or fiber cable.  It is worth noting that for economic reasons, ROW’s are frequently obtained from railroads, pipeline companies, or thruways.  It is frequently the case that several carriers may lease ROW from the same source; this makes it common to have a number of carriers’ fiber cables in close proximity to each other. Similarly, in a metropolitan network, several carriers might be leasing duct space in the same RBOC conduit.

In a typical intercity facility network there might be on the order of 102 offices that are candidates for  OXC’s. To represent the inter-office fiber network accurately a network with an order of magnitude more nodes is required.  In addition to Optical Amplifier (OA) sites, these additional nodes include:

- Places where fiber cables enter/leave a conduit or right of way.

- Locations where fiber cables cross.

- Locations where fiber splices are used to interchange fibers between fiber cables.

An example of the first might be:

Figure 3-2:  Fiber Cable vs ROW Topologies

Here the A-B fiber cable would be physically routed A-X-Y-B and the C-D cable correspondingly.   This topology might arise because of some physical bottleneck: X-Y might be the Lincoln Tunnel, for example, or the Bay Bridge.

Fiber route crossing (the second case) is really a special case of this, where X and Y coincide.  In this case the crossing point may not even be a manhole; the fiber routes might just be buried at different depths.

Fiber splicing (the third case) often occurs when a major fiber route passes near to a small office. To avoid the expense and additional transmission loss only a small number of fibers are spliced out of the major route into a smaller route going to the small office.  This might well occur in a manhole or hut.  An example is shown in Fig. 3-2(a), where A-X-B is the major route,  X the manhole, and C the smaller office.  The actual fiber topology would then look like Fig. 3-2(b), where there would typically be many more A-B fibers than A-C or C-B fibers, and where A-C and C-B might have different numbers of fibers. (One of the latter might even be missing.)

Figure 3-3.  Fiber Cable vs Fiber Topologies

The imminent deployment of ultra-long (>1000 km) Optical Transport Systems introduces a further complexity: Two OTS's could interact a number of times.  To make up a hypothetical example: A New York - Atlanta OTS and a Philadelphia - Orlando OTS might ride on the same right-of way for x miles in Maryland and then again for y miles in Georgia. They might also cross at Raleigh or some other intermediate node without sharing right of way.

Diverse Routings Often Can’t Be Done A Circuit At A Time

Even when routing two circuits between the same two nodes it frequently happens in practice that if the circuits are routed separately diverse paths cannot be found. This is illustrated in the following example:

Figure 3-4.  Diverse Routing Example

Suppose 2 diverse circuits are required from A to C. The shortest single route would be

A-X-Y-C, but if this route is chosen the second circuit cannot be routed diversely.

This is even more true in a subnetwork situation such as was illustrated in Fig. 4.

## Proposed Requirements

- Diversity compromises between two links being used for routing should be defined in terms of Shared Risk Link Groups (SRLG - see [6] ), a group of links which share some resource, such as a specific sequence of conduits or a specific office;

- A SRLG is a relationship between the links. It should be characterized by two parameters:

- Type of Compromise: Examples would be shared fiber cable, shared conduit, shared ROW, shared optical ring, shared office - no power sharing, etc.)

- Extent of Compromise:  For compromised outside plant, this would be the length of the sharing.

- Two links could be related by multiple SRLG's.

- When doing diversity routing, the relevant SRLG's would then be identified based on the types of compromises not desired by the requester.

- The control plane routing algorithms should be able to route a single demand diversely from N previously routed demands, where diversity would be defined to mean that no more than K demands (previously routed plus the new demand) should fail in the event of a single covered failure.

- In cases where a totally conforming routing cannot be found because of topological limitations or lack of capacity, the routing could then minimize a diversity violation where each SRLG would be weighted by some function f(type of compromise, extent of compromise).

These requirements do NOT cover the more complex types of diversity discussed in Sec. 3.1.  More ambitious algorithms are desirable but for the reasons discussed (complexity, computational requirements) it does not seem prudent to require the Control Plane under definition to deal with them at this time.

One final note – there will be a LOT of shared risk link groups. In the current AT&T core network there are about 8,000 spans, which roughly correspond to SRLG’s. The number of metro spans is several times this, and is growing rapidly. A single transcontinental link could be in several hundred SRLG’s. To accurately capture risks in an area such as downtown Manhattan might require a number of SRLG’s per block because of the number of cables entering/leaving buildings along the block.

# Restoration

Many announced Optical Layer Cross-Connect products come bundled with restoration capabilities.   These capabilities are proprietary and vary considerably between vendors. Approaches inspired by SONET line-switched and path-switched rings, PNNI, and various IP schemes are being launched as are a number of mesh restoration schemes. Deployment is imminent.

The routing impacts of all this depend on the specific type of restoration in question.  For example, if routing over analogs to SONET line-switched rings is specified then every effort should be made to route over ring-protected capacity.  For some other restoration schemes, such as those analogous to SONET path-switched rings, it may be necessary to find a pair of diverse routes. In the case of mesh restoration things are more complex.

- There is no general mapping to a desirable set of links, as in the line-switched ring case,

- Whether a route meets the Service Level Agreement (SLA) will depend on the global amount and distribution of spare capacity usable for restoration, the restoration demands on this capacity from other circuits, and the specific restoration algorithm implemented by the vendor.

- The determination of the amount and distribution of restoration capacity needed is likely to be a global problem requiring computationally intense and restoration method specific algorithms.

- The applicable SLA may allow the network time to reconfigure its restoration.

Proposed Requirements:

- Routing should be based on the restoration level specified by the client.

- The Control Plane architecture should allow vendor-proprietary restoration enhancements to be effectively utilized.

- Determination of whether the circuit provided meets the client SLA needs should be made.

# Motions

Motion 1:

The OIF Signaling WG should view the establishment of requirements on the constraints to be applied by constraint-based routing, and also the specification of application critical aspects of the network model to which the constraints will be applied,  as “in-scope”.

Motion 2:

The Carrier Services Framework and Requirements group should be asked to recommend to the Signaling WG the appropriate requirements and models necessary accomplish the goal of Motion 1.

Motion 3:

After approval by the Signaling WG and the OIF these requirements and models should be formally submitted to the IETF and the ITU.

References

- Ramaswami, R. and Sivarajan, K. N., Optical Networks: A Practical Perspective, Morgan Kaufmann Publishers, 1998.

- R. Tkach, E. Goldstein, J. Nagel and J. Strand, “Fundamental Limits of Optical Transparency”, Optical Fiber Communication Conf., Feb. 1998, pp. 161-162.

- Michael Eiselt, Lightwave Networks Research Dept., private communication, 4/7/2000.

- Strand, J., Doverspike, R., and Li, G., “Importance Of Wavelength Conversion In An Optical Network”, to appear.

- Yates, J. M.,  Rumsewicz, M. P. and Lacey, J. P.  R., “Wavelength Converters in Dynamically-Reconfigurable WDM Networks”, IEEE Communications Surveys, 2Q1999 (online at www.comsoc.org/pubs/surveys/2q99issue/yates.html).

- Chaudhuri, S., Hjalmtysson, G., and Yates, J., “Control of Lightpaths in an Optical Network, Optical Interworking Forum OIF2000.004.0; similar document available as Internet Draft draft-chaudhuri-ip-olxc-control-00.txt.

- Garey, M. R> and Johnson, D. S., Computers and Intractability: A Guide to the Theory of NP-Completeness, W. H. Freeman and Co., 1979.