# OIF Contribution Cover Sheet

Contribution Number:   OIF00.015.0

Working Group:	Architecture

TITLE:	ODSI Technical Overview

SOURCE:	John Moy and Rick Barry

Sycamore Networks

10 Elizabeth Drive

Chelmsford, MA 01824

rick@sycamorenet.com

john.moy@sycamorenet.com

DATE:		January 23, 2000

ABSTRACT:  We propose an architecture for UNI signaling based on MPLS.  This technical overview was presented at the January 2000 Optical Domain Service Interconnect (ODSI) meeting.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the right to at any time add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

Optical Domain Service Interconnect (ODSI)

Technical Overview

Foreword

The goal of ODSI is to allow higher-layer service devices to dynamically

request bandwidth from the optical network.	 The optical network

consists of optical transmission and switching equipment that can

provide point-to-point connections to attached internetworking

devices. These connections can be of various shapes and sizes, such

as SONET-encoded leased line service, gigabit ethernets, even

dedicated optical wavelengths where the encoding need not be known

to the optical network itself.  Typical interconnecting devices

requesting these bandwidth services from the optical network would

include IP routers, SONET add/drop muxes, and ATM switches.

Bandwidth received from the optical network will then be used as

point-to-point connections between routers, or trunks between

add/drop muxes and ATM switches.

Current provisioning of bandwidth within the optical core is

basically static; a slow and painstaking operation that takes much

configuration and often requires redesign of optical network

internals. However, two simultaneous developments in networking

technology make the dynamic request of bandwidth from the optical

core both feasible and desirable. First, a new generation of optical

switches is enabling dynamic point-and-click bandwidth provisioning

by network operators. Second, traffic engineering [Ref10] and

constraint-based routing enhancements to IP routers [Ref11, Ref12]

and/or ATM switches are allowing these devices to dynamically

determine when and where they need additional bandwidth. Allowing

these devices to make requests to the optical core directly reduces

the workload on the network operator.

This document provides a brief technical overview of the ideas behind ODSI,

but does not provide detailed protocol specifications, which will be forthcoming

in future collaborative document(s).

ODSI Technical Overview

Table of Contents

1	     Introduction ........................................... 3

2	     Service definition ..................................... 4

2.1	     Bandwidth characteristics ........................ 4

2.2	     Service definition ............................... 4

2.3	     Actions .......................................... 5

3	     Protocols .............................................. 5

3.1	     Service discovery ................................ 6

3.2	     Address registration ............................. 6

3.3	     Signaling ........................................ 6

4	     Example usage .......................................... 7

5	     Traffic engineering example ............................ 9

6	     Accounting/security .................................... 10

7	     Q&A .................................................... 10

References ............................................. 12

ODSI Technical Overview

1.  Introduction

The Optical Domain Service Interconnect (ODSI) defines an interface

to the optical network whereby electronic devices, such as routers,

SONET add/drop muxes, can dynamically request bandwidth from

the optical network. This allows leased lines to be created (and

destroyed) dialed up in an on-demand fashion.

This is not a new concept. For example, creation of bandwidth in an

on-demand fashion is currently possible through ISDN and ATM Switch

Virtual Circuits (SVCs), to name a couple of existing technology.

Now, as mentioned in the foreword, advances in technology allow the

extension of bandwidth-on-demand concepts to the higher bandwidth

optical core.

Since bandwidth-on-demand schemes have been around for years, there

are a number of protocols which can be used to signal bandwidth

requests.  These protocols must be modified/enhanced to support

characteristics unique to the optical domain. Since TCP/IP has

become the universal control protocol for data communications, we

choose to extend IP addressing and protocols to signal bandwidth

requests to the optical core, as discussed in Sections 2 and 3.

This document is organized into the following Sections:

o	Section 2, "Service specification", provides a description of

what can be requested from the optical core: how to identify the

endpoints to be connected, and the characteristics of the

bandwidth channel that will connect them.

o	Section 3, "Protocols", describes the enhancements to standard

IP protocols that are used to make bandwidth-on-demand requests

of the optical core.

o	Section 4, "Example usage", gives a simple example of how the

protocols are used to bring up dynamic leased line service

between routers.

o	Section 5, "Traffic engineering example", gives a more detailed

example.  In particular, this example shows how traffic

engineering and constraint based routing within the electrical

devices attached to the optical domain can be used to detect

when and where to request additional bandwidth.

o	Section 6, "Accounting/security", describes protocol options

that enable bandwidth requests to be verified and logged,

possibly for charging purposes.

o	Section 7, "Q&A", poses some questions and then answers them,

hopefully further illuminating the ODSI proposal.

2.  Service Definition

The service provided to the electrical devices attaching to the

optical network is the dynamic creation of point-to-point bandwidth

channels between electrical devices. There are three components to

the service definition: 1) How do you specify the endpoints of the

bandwidth channel? 2) What are the characteristics of the bandwidth

and 3) What actions can you request on the bandwidth channel. These

components are expanded upon in the following subsections.

2.1.  Endpoint identification

Each endpoint of the requested bandwidth is specified as a

combination of one or more of the following parameters.

o   IP address. An IP address is associated with an electrical

device, and can be attached to one, some, or all of that

device's physical port attachments to the optical network.

o   Port index. Identifies a particular physical interface

connecting into the Optical core. In IP MIBs, this is

commonly an IfIndex.

o   Channel. When establishing sub-rate bandwidth connections on

multiplexed interfaces, this identifies which sub-rate

channel should be connected.  For example, on a SONET

interface this would be a timeslot.

The only required parameters when requesting bandwidth are the

IP addresses of the two endpoints. When port/channel are not

explictly specified, any port associated with the IP address and

port/channel matching the requested bandwidth characteristics

will be connected.

2.2.  Bandwidth characteristics

The characteristics of the dynnamic bandwidth channel connecting

two electric devices can be specified by the following

parameters:

o   Encoding. Example encodings include SONET (path, section,

line), ethernet, and unspecified (raw wavelength).

o   Size. Examples include OC-x, 1 or 10 megabit (ethernet).

Specified in bits/second.

o   Priority. Whether the circuit can be preempted by other,

higher priority circuits (or vice-versa).

o   Protection. Whether the bandwidth should be protected

against failures, and if so, the speed at which the

protection will restore bandwidth service after a failure.

Encoding and bandwidth size may be omitted when implied by

endpoints.

2.3.  Actions

There are three actions that you can request regarding

allocation of dynamic bandwidth. All actions are performed by a

signaling protocol, as outlined in Section 3.3.

o   Create (dial). Create a point-to-point bandwidth channel

between two electronic devices connected to the optical

network. The two endpoints of the channel and the

characteristics of the bandwidth must be specified, as

described in Sections 2.1 and 2.2.

o   Destroy (hang-up). Destroy an existing bandwidth channel.

Either one of the endpoints can be specified.

o   Query (caller ID). Query the status of an existing bandwidth

channel. Either one of the endpoints is specified, and the

status (established or not) and other endpoint of the

channel is returned.

3.  Protocols

The protocols involved in ODSI perform three functions. First, they

allow an electrical device to detect that one of its ports is

connected to an optical switch that is capable of dynamic bandwidth

allocation; we call this "service discovery". Second, they allow the

electronic device to give and address to the port, so that the port

can be used as an endpoint in a dynamic bandwidth channel creation

request; we call this function "address registration". Lastly there

is a signaling function, which the electronic device uses to

dynamically connect its ports to other ports connecting to the

optical network.

Of these functions, the only required function is signaling. The

other two protocols are optional, and can be replaced through manual

configuration in the electronic devices and/or optical switches.

As mentioned earlier, all protocols are simple extensions to

existing IP protocols. Service discovery and address registration

are accomplished via a PPP session running in an in-band channel

associated with the port.  This in-band channel should however be

separate from the user data that will be carried by the trunk, and

should be available even if user traffic is present. One example is

an in-band channel built out of SONET overhead bytes from an OC-192

port. ODSI signaling is an extension of either RSVP or LDP, and can

be sent either over the in-band control channels or through out-of-

band paths.

The three protocol functions are described in more detail in the

following subsections.  IP some optional, and can be replaced by

configuration. best if in-band connection is always available, even

when user traffic on link.

3.1.  Service discovery

When a physical port of an electrical device connects to an

optical network, the device wishes to know whether it can use

ODSI to dynamically connect the port, through the optical

network, to another port on a remote device. This discovery is

done on an in-band channel of the physical port - for example,

using SONET overhead bytes on a SONET interface. The PPP

protocol [Ref1] is then run over the in-band channel; the

termination points for the PPP session being the electronic

device and the Optical switch to which it directly connects.

A new PPP control protocol will then be defined, to negotiate

the use of ODSI on the link. This control protocol will be

similar in scope and spirit to CHAP [Ref14]. In addition, this

new protocol will allow the electronic device and the optical

switch to exchange port numbers, which are used to identify the

endpoints of dynamic connections. The electronic device will

provide a digital signature, so that its identity can be

verified and used for accounting purposes.

If an in-band channel is not available, the above information

must be configured in the electrical devices and attached

optical switches.

The in-band channel should be available even when the port has

been connected, and is transporting user traffic. For example,

the port may have been connected via ODSI, or by other network

management means, and then the electrical device restarted. The

electrical device would still like to know that ODSI is

available on the port, in case it is desirable to reconnect that

port to a different destination at some later time.

3.2.  Address registration

This protocol function again uses PPP running over a port's in-

band control channel. The PPP IP Control Protocol [Ref2] is used

unaltered to exchange IP addresses of the electrical device port

and the optical switch.	 The IP address of the electrical device

is part of the endpoint identification in dynamic bandwidth

connection requests (see Section 2.1), and is used as the IP

source address in signaling requests. The IP address of the

Optical switch is destination address used for signaling

requests.

If the electronic device does not have an IP address, one will

be assigned to it per standard IPCP [Ref2].

3.3.  Signaling

The signaling protocol is used to request connection of two

ports (or even subchannels of ports), disconnect a port, or

query the connection status of a port connecting to the optical

network. The protocol is a straightforward extension to either

of the signaling protocols used by MPLS: RSVP [Ref3, Ref4,

Ref6] or LDP [Ref7, Ref8].  New data elements are defined for

these protocols to define the endpoints to be connected (Section

2.1), and the bandwidth characteristics of the connection

(Section 2.2).

Signaling is intrinsically third-party; neither of the

connection endpoints need belong to the requesting electronic

device. When the optical network receives the connection

request, it first forwards the request to a management station

which validates the request. The network determines whether

a path exists for the connection that meets the requested

bandwidth constraints. If so, it then informs the two endpoints

of the pending connection, and upon their acceptance sets up the

bandwidth and informs the requestor of the result.

Signaling requests are IP messages. They can be forwarded on

any IP data path between the requesting electronic device and

one of the optical switches. This could be through an in-band

control channel. Or it could be through connections that were

reserved for control traffic - for example, a 10-base T ethernet

connecting the electronic device and the optical switch.

The identification of the endpoints may be only partially

specified. In this case, the optical network will choose

appropriate endpoints, and this choice will be conveyed to the

endpoint devices before the bandwidth is fully established.

There are a number of differences between ODSI signaling and

that performed in MPLS by RSVP and LDP. However, the

modifications to these protocols are no more significant than,

for example, those already performed on RSVP to support traffic

engineering [Ref4, Ref6].

o   ODSI signaling is intrinsically third-party.

o   ODSI signaling messages need not follow the same path as

the bandwidth connection that will be established.

o   In ODSI there are no labels to set up. The resulting

bandwidth connection will look like a leased line (or full

duplex gigabit ethernet, etc.) to the electrical devices,

and will be space-division switched, as opposed to packet-

switched, by the optical switches.

An ODSI connection query mechanism could be a simple extension

of RSVP Diagnostic messages [Ref5]. Validation and admission of

bandwidth requests can be performed through COPS [Ref15].

4.  Example usage

This section presents a simple example of ODSI functionality.  Three

IP routers, A, B, and C are attached to an optical network through

three Optical switches SW1, SW2 and SW3, via OC-48 SONET links, as

pictured in Figure 1. The goal of ODSI is to allow Router A to

dynamically choose whether it is connected to Router B or C (or

neither). When connected, the connection will be essentially a

leased line between the routers.

Assume that Router A can support an on-band control channel on its

OC-48, but that B and C cannot. Router A can dynamically discover

that ODSI is available on its OC-48 link into the optical network,

can tell SW1 its IP address, and can learn SW1's address, which will

be the destination of any signaling requests. In contrast, these

items must be configured in Routers B and C and their attached

Optical switches SW2 and SW3. In addition, in order to participate

in signaling, an out-of-band control connection must exist between

these latter two routers and their switches, pictured in the diagram

as 10baseT ethernet segments.

If Router A now wants a leased line to B, it sends an ODSI

signaling request to SW1 over the in-band channel, requesting a

SONET OC-48 connection to Router B's IP address. The optical network

then verifies that such a connection can be established to B,

confirms with B that this is OK (over the 10baseT), sets up the

connection and returns a success indication to A.

+********+

*	    *

*	 +---+	OC-48  +--+

*	 |SW2|---------|RB|

*		 +---+------+  +--+

*			*   |	|

+--+	   +---+     Optical	 * +-----------+

|RA|-----|SW1|     Network	  *   10baseT

+--+OC-48+---+		  *   10baseT

*			 * +-----------+

*			*   |	|

*	 +----------+  +--+

*	 |SW3|---------|RC|

*	 +---+	OC-48  +--+

*	    *

+*******+

Figure 1: Simple ODSI example

Two questions come up in this example. How does Router A know Router

B's IP address, and how does it know that it wants to connect to

Router B. Router A may have some a priori knowledge (such as a human

configuring the information), but as we shall see in the next

section, a bootstrapping scheme using MPLS traffic engineering may

also supply the needed information.

5.  Traffic engineering example

Figure 2 shows a sample optical network of three switches,

interconnected by WDM links and providing leased line services for

four IP routers.

Assume that OC-48s leased lines have been provisioned between the

routers, using traditional means, producing a linear topology

amongst the routers as shown in Figure 3.  This allows IP

connectivity to be established between the routers, enabling them to

learn each others' IP addresses.

On top of this, a combination of MPLS and constraint-based routing

allows you to overlay a full mesh of Label Switched Paths (LSPs)

between routers for traffic engineering purposes, as shown in Figure

4. Assume at first that it is estimated that the trffic between each

pair of routers is an OC-12 (i.e., each LSP is sized at OC-12). Then

the router topology in Figure 3 suffices. However, if the estimate

of the IP traffic between Routers A and D increases to 2 OC-12s,

constraint-based routing will tell the routers that the LSPs no longer fit.

Dialing up a new OC-48 link through ODSI between A and D (or B and C) fixes

The problem.

+--+--------+---+	      +---+--------+--+

|RA|--------|SW1|-----------|SW2|--------|RD|

+--+ OC-48s +---+    WDM    +---+ OC-48s +--+

\	       /

\	      /

WDM \	     /WDM

\  +---+  /

+-|SW3|-+

/+---+\

OC-48s/ /	  \ \OC-48s

/ /	   \ \

+--+	   +--+

|RB|	   |RC|

+--+	   +--+

Figure 2: Simple Traffic Engineering example

+--+	     +--+	+--+	   +--+

|RA|-------|RB|-------|RC|-------|RD|

+--+ OC-48 +--+ OC-48 +--+ OC-48 +--+

Figure 3: Bootstrap router topology

6.  Accounting/security

Service discovery and bandwidth requests carry digital signatures,

allowing identities of bandwidth endpoints and requestors to be

verified.

+--+	     +--+

|RA|-------|RB|

+--+	     +--+

|  \ ----/  |

|   /---\   |

+--+/	    \+--+

|RC|-------|RD|

+--+	     +--+

Figure 4: LSP overlay (OC-12 LSP size)

Admission control and accounting of bandwidth requests is

accomplished through the COPS protocol [Ref15].

7.  Q&A

Q: Why is the requested bandwidth channel bi-directional (transmit

and receive size the same)?

A: Because the control protocols used by the electrical devices

attaching to the optical network, and most of the applications

running over them, assume symmetric bandwidth. For example, standard

IP routing protocols do not work over asymmetric links.

Q: Why specify the use of both RSVP and LDP for signaling bandwidth

requests?

A: Current market reality. Until the market decides on a single IP

signaling protocol, most vendors will be forced to implement both

RSVP and LDP.

Q: How does ODSI relate to "Multi-Protocol Lambda Switching:

Combining MPLS Traffic Engineering Control With Optical

Crossconnects" [Ref9]?

A: They are Complementary. [Ref9] provides an outline of how to

implement the internals of an Optical network. ODSI provides an

interface whereby you can dynamically request services from the

optical network.

Q: Why not open up the topology and protocols used by the switches

and transmission equipment in the optical core?

A: Two reasons: Time to market and infrastructure robustness.

Q: How does ODSI compare to other bandwidth-on-demand schemes?

A: It is very similar to ATM SVCs, ISDN, and even the PSTN.	 The

main difference is that ODSI is dealing with much larger bandwidth

channels, is data-centric, and is going to be used by electronic

devices that are already aggregating a lot of traffic.

Q: What do you lose by not opening up the core topology and

protocols?

A: Possibly wasting signaling requests that the electronic devices

would otherwise know to be unsatisfiable.

Q: Is ODSI adding another layer to MPLS traffic engineering?

A: That other layer already exists. ODSI is allowing the optical

layer to be integrated into the larger MPLS traffic engineering

scheme.

Q: What if I don't want to assign IP addresses to my electrical

device?

A: You wouldn't have to. During address registration, your optical

switch would dynamically assign you an IP address. You would use this

address as the source address in your signaling packets, but you

wouldn't want people to use it as the endpoint of a bandwidth

connection request since it's been allocated from a temporary pool.

So you would instead use you switch's IP address and port number as

the endpoint of connection requests.

Q: Could ODSI be extended to provide a VPN service?

A: Yes. A VPN ID [Ref13] could be added to the endpoint identifier.

The VPN ID, together with an IP address in the VPN, could then be

advertised by the electronic device in the service discovery

protocol.  Public IP addresses are still discovered in the address

discovery phase, and are used as source and destination addresses

for the signaling protocols.

References

[Ref1]  Simpson, W., "The Point-to-Point Protocol (PPP)", RFC 1661,

July 1994.

[Ref2]  McGregor, G., "The PPP Internet Protocol Control Protocol

(IPCP)", RFC 1332, May 1992.

[Ref3]  Braden R., L. Zhang, S. Berson, S. Herzog and S. Jamin,

"Resource ReSerVation Protocol (RSVP) -- Version 1

Functional Specification", RFC 2205, September 1997.

[Ref4]  Berger, L., D. Gan, G. Swallow, P. Pan, "RSVP Refresh

Reduction Extensions", work in progress, October 1999.

[Ref5]  Terzis, A., B. Braden, S. Vincent and L. Zhang, "RSVP

Diagnostic Messages", work in progress, June 1999.

[Ref6]  Awduche, D., L. Berger, D. Gan, T. Li, G. Swallow, V.

Srinivasan, "Extensions to RSVP for LSP Tunnels", work in

progress, September 1999.

[Ref7]  Andersson, L., P. Doolan, N. Feldman, A. Fredette, B.

Thomas, "LDP Specification", work in progress, October 1999.

[Ref8]  Jamoussi, B., "Constraint-Based LSP Setup using LDP", work

in progress, September 1999.

[Ref9]  Awduche, D., Y. Rekhter, J. Drake and R. Coltun, "Multi-

Protocol Lambda Switching: Combining MPLS Traffic

Engineering Control With Optical Crossconnects", work in

progress, November 1999.

[Ref10] Awduche, D., J. Malcolm, J. Agogbua, M. O'Dell and J.

McManus, "Requirements for Traffic Engineering Over MPLS",

RFC 2702, September 1999.

[Ref11] Smit, H., T. Li, "IS-IS extensions for Traffic Engineering",

work in progress, May 1999.

[Ref12] Katz, D. and D. Yeung, "Traffic Engineering Extensions to

OSPF", work in progress.

[Ref13] Fox, B. and B. Gleeson, "Virtual Private Networks

Identifier", RFC 2685, September 1999.

[Ref14] Simpson, W., "PPP Challenge Handshake Authentication

Protocol (CHAP)", RFC 1994, August 1996.

[Ref15] Boyle, J., R. Cohen, D. Durham, S. Herzog, R. Rajan and A.

Sastry, "The COPS (Common Open Policy Service) Protocol",

work in progress, November 6, 1999.