Contribution Number:

Working Group: Architecture

TITLE:  IP Centric Control and Signaling for Optical Lightpaths

SOURCE:  K. Bala, S. Giebl, R. Ramamurthy, W. Russ and B. Tang

Tellium, Inc.

www.tellium.com

DATE: Jan 24 2000

ABSTRACT:

The objective of this document is to propose a control architecture for provisioning, maintaining, and restoring switched lightpaths] in an optical network. We consider a network architecture where routers and other network elements, such as SONET ADMs, are interconnected via Optical Switches or Optical Crossconnects (OXCs). The optical mesh provides connectivity between IP routers, ATM switches and SONET ADMs over switched optical paths. The OIF and other standards forums are currently in the process of defining and standardizing a control architecture for creating and managing switched lightpaths. A number of contributions submitted to OIF, IETF, and other standards bodies have already addressed some of the important aspects of such an architecture. In this document we have attempted to take the next step forward and have proposed an architecture that captures some of the intricacies of an optical network.

# IP Centric Control and Signaling for Optical Lightpaths 
K. Bala, S. Giebl, R. Ramamurthy, W. Russ and B. Tang

Tellium

www.tellium.com

# Introduction

The objective of this document is to propose a control architecture for provisioning, maintaining, and restoring switched lightpaths [1] in an optical network. We consider a network architecture where routers and other network elements, such as SONET ADMs, are interconnected via Optical Switches or Optical Crossconnects (OXCs) as shown in Figure 1. The optical mesh provides connectivity between IP routers, ATM switches and SONET ADMs over switched optical paths. The OIF and other standards forums are currently in the process of defining and standardizing a control architecture for creating and managing switched lightpaths. A number of contributions [2,3] submitted to OIF, IETF, and other standards bodies have already addressed some of the important aspects of such an architecture. In this document we have attempted to take the next step forward and have proposed an architecture that captures some of the intricacies of an optical network. The proposed architecture is based on the following fundamental tenets:

- Reuse the MPLS framework and protocols from Layer 3 as much as possible. This not only obviates the need to reinvent new protocols, but also provides a platform for coordinating services in data as well as optical domain.

- Enhance MPLS based traffic engineering framework for provisioning, managing, and restoring switched lightpaths taking into account specific characteristics of optical-networking elements and special requirements of switched lightpaths.

- Enhance signaling protocols such as LDP/CR-LDP to dynamically create, maintain, and terminate lightpaths meeting specific service provider requirements for such services.

- Use link-state routing protocols such as OPSF and IS-IS for dynamic topology discovery and link state advertisements.

# Network Architecture

Figure 1 illustrates the architecture of an optical network where multiple OXCs are interconnected via WDM links. The IP routers and other network elements are clients of this optical core and are connected to their peers over switched optical paths spanning potentially multiple OXCs.  We assume that the OXCs are within one administrative domain and run inter-domain routing protocols, such as OSPF[11] and IS-IS[12], for the purpose of dynamic topology discovery. For optimal use of network resources the routing protocols have to be enhanced to disseminate additional link-state information than that currently done by standard OSPF and IS-IS protocols. Fortunately, these protocols are extensible and efforts are already underway in the IETF to extend them for traffic engineering purposes. We can leverage this work to add enhancements necessary for discovering optical network topology. For example, we can make use of Opaque LSAs in OSPF to disseminate information about availability of physically separate lightpaths between two OXCs. In Section 4 we discuss several practical issues related to dynamic topology discovery using link-state routing protocols.

The request to setup a path can either come from a router or an ADM connected to the OXCs or a management system, such as an EMS. The request must identify the ingress and the egress OXCs between whom the lightpath has to be setup. Additionally, it may also include traffic related parameters such as bandwidth  (OC-48/OC-192), reliability parameters and restoration options, setup and holding priorities for the path etc. On receipt of the request, the ingress OXC computes a suitable route for the requested lightpath following applicable policies and constraints. Once the route has been computed, the ingress invokes a signaling protocol to setup the lightpath. We propose the use of LDP and CR-LDP as signaling protocols. It may be possible to use other signaling protocols, such as RSVP. The signaling protocol may also be used to maintain and monitor the lightpath and restore it in the event of a failure. In the following sections we discuss potential adaptations and enhancements to the signaling architecture and protocols that may be required to setup lightpaths in optical networks using LDP/CR-LDP as an example.

Figure 1. Example of an Optical Mesh Network Architecture

# Naming and Addressing

Figure 2 shows the physical plant hierarchy between two neighboring OXCs. A signaling protocol designed to setup and manage switched lightpaths needs to take into account this physical hierarchy and make use of it whenever necessary. For example, primary and backup paths between ingress OXC and an egress OXC in an optical should follow routes that are physically separate. Control architecture and signaling protocols used in MPLS, such as LDP and RSVP, are not cognizant of the details of the physical plant connecting the network elements (LSRs). In fact no standard terminology to describe the physical plant hierarchy exists. In this section we define the naming and addressing convention for different elements of the physical plant hierarchy. We assume that all connections are unidirectional point-to-point unless otherwise specified.

.

- Sub-channels.  This is the smallest granularity lightpath between two OXCs. Multiple TDM channels are multiplexed in the time domain and are carried over a single optical channel. For example, an OC-192 signal may carry four OC-48 TDM channels.

- Optical Channel or s.  An Optical Channel can generally be associated with a specific wavelength in the WDM system. However, the add and drop ports might all be of the same wavelength (e.g. 1.3 m short reach interface).

- Fibers: Several optical channels might be multiplexed onto the same fiber using Wavelength Division Multiplexing.

- Cable: Several Optical Fibers may be grouped into Cables.

- Sub-Segment or Conduit.  A Sub-Segment consists of a collection of Cables that are located in the same conduit or right of way. If there is a cut in the Sub-Segment, failures would potentially be experienced on all Optical Fibers within all Cables of this Sub-Segment. All Cables in a Sub-Segment have the same route. Several Sub-Segments can exists between two OXCs.

- Segment. All Sub-Segments connecting a pair of OXCs, not necessarily routed along the same physical route constitute a Segment. If route diversity is implemented between two OXCs, then multiple Sub-Segments may exist between the same OXCs.

Figure 2. Physical plant hierarchy between neighboring OXCs

As shown in Figure 2, two OXCs may be connected at multiple ports/interfaces via multiple fibers, cables, and sub-segments. In order to take full advantage of the physical topology, the physical plant hierarchy has to be encoded and advertised. It is important to standardize the naming and addressing of different elements that constitute the physical segment. It may also be necessary to expose other relevant physical characteristics of a segment and its constituting elements. For example, the information that two sub-segments are physically separate and are routed diversely can be used to provision backup paths that are routed over different sub-segments.

In traditional IP networks each physical port/interface on a router is typically identified by a unique IP address. Given the potentially large number of physical ports on an OXC it is impractical to assign a unique IP addresses to every port of the OXC. Instead, each OXC can be assigned a unique address and each end of a segment can be assigned unique addresses. If the routing protocol supports unnumbered links (e.g. OSPF), it may not even be necessary to assign IP addresses to the segments. In many ways a segment connecting two OXCs is analogous to a point-to-point link connecting two routers. In either of these two cases, relying on OSPF alone is insufficient to discover network topology to the level of OXC port connectivity.

# Topology and Link/Port State Discovery

For the optimal use of network resources, it is critical to have network states being dynamically updated and available whenever needed. The network states may include the network physical connectivity and topology, ports/wavelengths already used at an OXC, and ports/wavelengths still available at an OXC, or any other information deemed necessary for efficient provisioning, protection, and restoration of an optical network.  We can use link-state routing protocols such as OSPF and IS-IS for this purpose.  For the sake of the convenience, we focus our discussion only on OSPF throughout the rest of this article. Arguments made for OSPF generally apply to IS-IS as well. OSPF already provides network topology information through its LSA flooding mechanism. OSPF extensions for traffic engineering using opaque LSA can be used with necessary modifications and/or extensions for OXC nodes to disseminate node resource utilization information.

| Local port-ID | Peer port-ID | SCID | OCID | FID | CID | SSID | SID |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 1 | 1 | 1 | 2 | 7 | 1 |
| 2 | 10 | 3 | 2 | 2 | 9 | 5 | 3 |
| 3 | 101 | 4 | 1 | 1 | 2 | 7 | 1 |

We also propose to use a link layer Auto-Discovery Protocol (ADP) to further facilitate automatic topology discovery and link/segment-state update. ADP runs between two OXCs that peer at the link layer and uses unused SONET overhead bytes to exchange port and channel-state information. Peer port mapping information is incorporated into Port State Database. The database contains the state information for each OXC port such as its remote peer OXC port, its segment-ID (SID), its sub-segment-ID (SSID), its cable-ID (CID), its fiber-ID (FID), its optical-channel-ID (OCID) and its sub-channel-ID (SCID) as shown in the table above. The Port-state database from the ADP supplements the Link-state database from the OSPF to provide sufficient information needed for the network topology discovery and switched optical path management. The following is a list issue that needs to be debated and explored further.

- Specific link/segment-state information that needs to be advertised and what encoding to use for that. It is always possible to keep this opaque. However, having a consensus and/or standardization will improve interoperability among  OXCs from different vendors.

- Specifics of link-layer ADP: Specifically, what information needs to be exchanged. We do not propose any specific unused overhead bytes in the SONET frame. We suggest that equipment vendors allow this to be arbitrarily programmable by the network operators.

- Enhancements to routing protocols to carry additional link state information and their impact on the operation and performance of the protocols.

# Signaling  Architecture and Protocols

MPLS has been designed to use a number of signaling protocols including LDP and RSVP. Recently, CR-LDP has been proposed as an enhancement to LDP to facilitate traffic engineering and provisioning. Although both LDP and RSVP (with extension) can be used as the signaling protocol to setup and manage lightpaths, we believe that LDP, especially with extensions proposed in CR-LDP, is more suitable for this purpose.  However, we need to adapt and enhance CR-LDP to support all desirable traffic engineering features of switched lightpaths. The following is an incomplete list of items that requires further exploration.

- Mapping of physical plant hierarchy into a label structure so that OXCs form different vendors can inter-operate.

- Defining TLVs for traffic engineering parameters that applies to lightpaths in arbitrary connected optical networks.

- Possible extensions to LDP/CR-LDP to facilitate dynamic restoration and partial restoration.

- Explore the details of interaction between IP routers and OXCs, specifically how a lightpath can be setup as a part of setting up and end-to-end IP trunk between two routers.

# Conclusion

We propose the following:

- The formation of a study group to explore an MPLS-based IP-centric control architecture for provisioning and managing switched lightpaths in optical networks.

- “Standardize” the terminology used for various physical components of an optical network. Create a framework for naming and addressing of the physical plant hierarchy to facilitate traffic engineering.

- We propose the use of protocols, such as OSPF and IS-IS, for dynamic topology and link/segment state discovery. We propose enhancements such as use of Opaque LSAs in OSPF to distribute the link/port state information.

- We propose to use a link layer auto-discovery protocol for exchanging port interconnection information between OXCs directly connected to each other.

- We propose the use LDP/CR-LDP for signaling switched optical paths.

# References

- T. E. Stern and K. Bala, Multiwavelength Optical Networks: A Layered Approach, Addison Wesley Longman, May 1999

- D. Awaduche, Y. Rekhter, J. Drake, and R. Coltun, Multi-Protocol Lambda Switching: Combining MPLS Traffic Engineering Control with Optical Crossconnects, Internet Draft, Work in Progress, 1999.

- S. Chaudhuri, G. Hjalmtysson, and J. Yates, Control of Lightpaths in an Optical Network, OIF Contribution, Work in Progress, January 2000.

- D. Awduche, J. Malcolm, J. Agogbua, M. O'Dell, and J. McManus, Requirements for Traffic Engineering Over MPLS, RFC-2702, September 1999.

- B. Jamoussi et al, Constraint-Based LSP Setup using  LDP, Internet Draft, Work in Progress, 1999

- R. Callon, P. Doolan, N. Feldman, A. Fredette, G. Swallow, A. Viswanathan, A Framework for Multiprotocol Label Switching, Internet Draft, Work in Progress, 1999

- E. Rosen, A. Viswanathan, R. Callon, Multi-protocol Label Switching Architecture, Internet Draft, Work in Progress, 1999

- H. Smit and T. Li, IS-IS extensions for Traffic Engineering, Internet Draft, Work in Progress, 1999

- D. Katz, D. Yeung,  Traffic Engineering Extensions to OSPF, Internet Draft, Work in Progress, 1999

- E. Crawley, R. Nair, B. Rajagopalan, and H. Sandick, A Framework for QoS-Based Routing in the Internet,  RFC-2386, August, 1998.

- J. Moy, OSPF version 2, RFC 2328, April 1998

- D. Oran, IS-IS, RFC 1142, February 1990