Contribution Number:  OIF00.007.3

Working Group:	Architecture Working Group

TITLE:     Proposed Architecture Working Group Requirements Document (Project 1)

SOURCE:	Andy Wright			Rick Barry

Technical Editor			Working Group Chair

Williams Communications		Sycamore Networks

One Williams Center, RC3-J		10 Elizabeth Drive

Tulsa, Oklahoma 74172		Chelmsford, MA 01824

Phone: (918) 573-5714		Phone: (978) 367-2004

Email: awright@labrats.com		Email: rick@sycamorenet.com

DATE:	January 31, 2000

Document Status:  DRAFT version 3.2, approved by architecture working group.

Project Name: Architecture Requirements Project

Project Number: AWG Project #1

Notice: This Technical Document has been created by the Optical Internetworking Forum (OIF).  This document is offered to the OIF Membership solely as a basis for agreement and is not a binding proposal on the companies listed as resources above. The OIF reserves the rights to at any time to add, amend, or withdraw statements contained herein. Nothing in this document is in any way binding on the OIF or any of its members.

The user's attention is called to the possibility that implementation of the OIF implementation agreement contained herein may require the use of inventions covered by the patent rights held by third parties. By publication of this OIF implementation agreement, the OIF makes no representation or warranty whatsoever, whether expressed or implied, that implementation of the specification will not infringe any third party rights, nor does the OIF make any representation or warranty whatsoever, whether expressed or implied, with respect to any claim that has been or may be asserted by any third party, the validity of any patent rights related to any such claim, or the extent to which a license to use any such rights may or may not be available or the terms hereof.

For additional information contact:
The Optical Internetworking Forum, 39355 California Street,
Suite 307, Fremont, CA 94538
510-608-5990 phone  info@oiforum.com

© 2000 Optical Internetworking Forum

## Table of Contents

0	Cover Sheet	1

1	Table of Contents	2

2	List of Figures	4

3	List of Tables	5

4	Document Revision History	6

5	Project Summary	7

5.1	Working Group project(s)	7

5.2	Working Group(s)	7

5.3	Date Approved	7

5.4	Original Document	7

5.5	Problem Statement	7

5.6	Scope	7

5.7	Expected Outcome	7

5.8	Schedule	7

5.9	Merits to OIF	8

5.10	Merits to working group	8

5.11	Relationship to other Working Groups	8

5.12	Relationship to other Standards Bodies	8

6	Introduction	11

7	Reference Diagrams and Terminology	12

7.1	Reference Diagrams	12

7.2	Terminology	13

8	Requirements	15

8.1	General Network Requirements	15

8.2	Network Services	15

8.3	User Network Interface Requirements	16

8.4	Network to Network Interface Requirements	17

9	Summary	18

10	References	19

10.1	Normative references	19

10.2	Informative references	19

11	Appendix A: Glossary	20

12	Appendix B: Open Issues / current work items	21

13	Appendix C: List of companies belonging to OIF when document is approved	22

## List of Figures

Figure 1 Optical Network Reference Diagram	12

Figure 2 Optical Internetwork Reference Diagram showing two management domains.	13

## List of Tables

## Document Revision History

| Version | Date | Author | Editor | Notes | Contribution |
| --- | --- | --- | --- | --- | --- |
| 1.0 | July 20, 1999 | Andy Wright | Andy Wright | Accepted as a draft of the AWG Project 1 document | OIF99-105.1 |
| 2.0 | July 27, 1999 | Andy Wright | Andy Wright | Updated to reflect changes voted on in July OIF meeting | OIF99-105.2 |
| 3.0 | January 16, 2000 | Rick Barry | Andy Wright | Adapted text to new standard format for technical documents. Included requirements agreed to at Oct. 99 meeting. Included requirements generally agreed to at Dec. 99 interim meeting. Text is referenced to indicate if it is old text [99.105.2], agreed to content [Oct. 99], proposed content from interim meeting [Dec. 99], or simply newly proposed text [Proposed]. The latter two are also displayed in Red. References are provided at the end of a paragraph or an entire requirements. | OIF00-007.0 |
| 3.1 | January 31, 2000 | Rick Barry | Andy Wright | Document approved in technical details by AWG. Minor editorial changes required before presenting to the TC. | OIF00-007.1 |
| 3.2 | February 1, 2000 | Rick Barry | Andy Wright | Document sent to technical committee. Approved to begin straw man process. | OIF00-007.3 |

## Project Summary

### Working Group project(s)

Architecture Requirements Project (AWG Project #1).

### Working Group(s)

Architecture Working Group

### Date Approved

### Original Document

OIF.99.105.0

### Problem Statement

To define the internetworking requirements associated with designing and operating an optical internetwork.

### Scope

The scope of this project is to clearly define the internetworking requirements associated with designing an optical network. These requirements have been divided into two categories: User-Network interfaces and Network-Network Interfaces. UNIs are defined as interfaces between the “optical network cloud” and clients to the optical network (such as IP routers). NNIs are defined as interfaces within the “optical network cloud”, specifically between optical networks that are a member of different management domains.

### Expected Outcome

Project 1 will result in a document that defines the internetworking requirements of the optical network.

### Schedule

| Outcome | Target Date | Actual Date | Notes |
| --- | --- | --- | --- |
| Draft of UNI Requirements for IP Clients approved by AWG and presented to TC | Q1 ‘00 |  |  |

### Merits to OIF

Defining the requirements is the first logical step in designing the internetworking

standards. The requirements set forth by this project will help to guide the work of Project 2 in the Architecture working group, which is the development of an optical networking reference model. Once the requirements and the reference model are defined, the OAM&P working group and the Physical Link Layer working group will have a clear objective.

### Merits to working group

Defining the requirements is the first logical step in designing the internetworking

standards. The requirements set forth by this project will help to guide the work of Project 2 in the Architecture working group, which is the development of an optical networking reference model. Once the requirements and the reference model are defined, the OAM&P working group and the Physical Link Layer working group will have a clear objective.

### Relationship to other Working Groups

It is expected that this document provide input to the PLL, OAM&P, and Signaling working groups.

#### Overlaps

There are no identified overlaps with PLL, Signaling, or OAM&P.

#### Unique viewpoints

This project defines at a high level the requirements of the optical internetwork.

### Relationship to other Standards Bodies

There has been a great deal of discussion about how this project relates to the work of the ITU-T.

#### Overlaps

It has been generally agreed that there is potentially some overlap between this project and the work that has been done in ITU-T G.873, but there has not been any consensus on exactly how the two projects overlap. Since ANSI T1X1 is working closely with the ITU on the subject of Optical Networking, there have not been any concerns that this project conflicts with T1X1 independently of conflicting with the ITU. In other words, if it is decided that this project does not overlap with the ITU then it should follow that this project does not conflict with T1X1.

#### Unique viewpoints

This project is unique in many aspects with respect to the work being done at other standards bodies.  In particular, it addresses the optical internetworking requirements for

- IP clients

- rapid provisioning of SONET/SDH Framed and Rate circuits through an electro-optic optical internetwork.

- Client to optical network signaling

- Optical network to optical network signaling

## Introduction

With the widespread deployment of DWDM, a new layer has begun to evolve in fiber optics based telecommunications networks: the optical networking layer. In its initial form, the optical network uses SONET and SDH as the framing and transport protocol, but the optical network is not limited to these protocols in the future. By definition, the Optical Internetworking Forum has a charter of developing interoperability agreements between the emerging optical layer of the network and other layers already defined in the Open Systems Interconnection (OSI) model. The OIF has also been tasked with developing interoperability agreements between different vendors within the optical network layer.

To effectively serve the telecommunications community, the OIF implementation agreements must reflect the needs of service providers who are building networks.  Based on the charter of the Architecture working group of the OIF, the first project, The Architecture Requirements Project, a.ka. Project #1, is to define the architectural requirements of the optical internetwork.  This document defines those requirements. The OIF recommendations will inevitably flow out of the architectural requirements laid out in this document.

The initial focus of Project #1 is to define the requirements of UNI and the network services offered across the UNI, particularly with respect to the requirements of internetworking IP with the optical network layer thru SONET framed and rate circuits.  Although the group needs to create an open architecture that can internetwork with a variety of clients (e.g. IP, SONET, Gigabit Ethernet, Frame Relay,...), the working group must also prioritize which clients will be defined first. It has been decided that the initial focus should be on IP as a client to the optical network.

The rest of this document is organized as follows.  Section 7 contains reference diagrams and terminology for reference.  Section 8 contains the optical internetworking requirements.  Section 8 is divided into four subsections.  Section 8.1 contains high level general network requirements.  Section 8.2 contains descriptions of the network services.  Section 8.3 contains requirements specific to the UNI.  Section 8.4 contains requirements specific to the NNI. Section 9 is a summary.  Section 10 contains references.  Appendix A is the glossary.  Appendix B is the open issues / current work items list.  Appendix C is the list of companies belonging to the OIF when this document is approved.

## Reference Diagrams and Terminology

### Reference Diagrams

Figure 1 Optical Network Reference Diagram

Description of Figure 1: Figure 1 shows an optical network as a collection of connected optical subnetworks.  The UNI and NNIs may be electro-optical or all-optical.

Figure 2 Optical Internetwork Reference Diagram showing two management domains.

Description of Figure 2: Figure 2 shows an optical internetwork crossing multiple management domains (squares).  A carrier may or may not have multiple management domains.  Also shown for completeness is an IP to IP direct connection.  Such a connection is not within the scope of this document.   It is recognized that Figure  2 does not represent all possible scenarios.  For example, a router/IP entity in one administrative domain may request that a router/IP entity in another administrative domain perform some signaling event on behalf of the first administrative domain.  Further, this request might occur over a non-optical medium such as Ethernet.  However, Figure 2 will serve as a reference diagram for many scenarios of interest to this working group.

### Terminology

Client: A piece of networking equipment connected to an optical network at a UNI.  Examples of clients include IP routers, ATM switches, Ethernet Switches, SONET ADMs, SONET Cross-connects, SONET Terminal Multiplexers.  The term client neither implies an overlay nor a peer architecture for the optical internetwork.  The term edge device is used interchangeably with client.

Optical Network:  For the purposes of this document, the optical network is a cloud managed as one system and defined by the functions and services it provides across and between the UNIs and NNIs.  No other definition is required for this document.  However, for reference and better understanding, a reference diagram showing details “inside the cloud” is shown in Fig. 1.  As shown, the optical network is a collection, possibly only one, of connected optical subnetworks.  The optical subnetworks may be connected all-optically or electro-optically.  The connections between subnetworks may be direct or may be thru an optical or electro-optical cross-connects.  In the latter case, the cross-connects may form an overlay optical network built on top of more basic physical layer optical networks, or there may be a peer relationship.  For simplicity, an optical network is assumed to be composed of network elements from a single vendor.

Optical subnetworks: See Fig. 1.  An optical subnetwork is a collection of optical networking elements providing a set of unspecified network services.  Examples of optical networking elements include, but are not limited to optical or electro-optic cross-connects, WDM terminals, WDM add/drop nodes, and their associated transponder line cards.  Examples of an optical subnetwork include a WDM point-to-point link with interlink add/drop nodes, WDM ring networks, WDM mesh networks, and their interconnection.  An optical network may contain an optical transport network (OTN) as defined by G.872, or may contain some other implementation of an all-optical network.

Optical internetwork: Two or more connected optical networks, or clients connected to  one or more optical networks.

UNI: The user-network interface is the interface between a client and an optical network.

NNI: The network-network interface is the interface between two optical networks.

## Requirements

### General Network Requirements

This section identifies the generic networking requirements of the optical internetwork.

The optical internetwork will:

(R1) Support at least the following types of network elements (clients of the optical network a.k.a. clients): IP Routers, SONET/SDH ADMs, SONET/SDH Cross-connects, and SONET/SDH Terminal Multiplexers, Ethernet Switches,  Ethernet Multiplexers, ATM Switches.  Other clients are possible.

(R2) Support rapid provisioning of circuits between clients.

(R3) Support various levels of circuit protection and restoration.

(R4) Support a mechanism for real time signaling between the clients and the optical network to support automatic topology discovery, rapid provisioning and configuration of circuits, and fault detection, localization, and notification.

### Network Services

This section identifies end-to-end services provided by the optical internetwork between two or more UNIs.  The initial focus is on SONET rate and framed signals in point-to-point connections (two UNIs); however, this does not exclude other options from future consideration.  Possible future extensions include multi-casting, non-SONET framed or rate signals (e.g. Gigabit Ethernet), or all-optical interfaces (WDM or single wavelength).

Not all UNIs need support all defined services.

The following 3 services have been defined:

- Physical Layer Regenerator Circuit (PLR-Circuit)

- STE Circuit (STE-Circuit)

- LTE Circuit (LTE-Circuit)

A PLR-Circuit is a SONET/SDH rate and framed point-to-point circuit between two UNIs.  The circuit preserves all SONET overhead bytes between clients.  The SONET/SDH signal may be concatenated or channelized but cannot be SONET/SDH TDM demultiplexed or multiplexed within the optical internetwork. [OIF-99-161, Option 1].

An STE-Circuit is a SONET/SDH rate and framed point-to-point circuit between two UNIs.  The circuit preserves all SONET/SDH line overhead bytes between clients but is not required to preserve the section overhead bytes. The SONET/SDH signal may be concatenated or channelized but cannot be SONET/SDH TDM demultiplexed or multiplexed within the optical internetwork. [OIF-99-161, Option 2].

An LTE-Circuit is a SONET/SDH rate and framed point-to-point circuit between two UNIs.  The circuit preserves the SONET/SDH payload, but is not required to preserve the section or line overhead bytes. The SONET/SDH signal may be concatenated or channelized and may be SONET/SDH TDM demultiplexed or multiplexed within the optical internetwork to allow the subrate circuits to be individually routed, or to allow multiple LTE-Circuits to be multiplexed within the network to better utilize a network link.  Thus, an LTE-Circuit implies timing and synchronization requirements not required in PLR-Circuits or STE-Circuits. [OIF-99-161, Option 3].

### User Network Interface Requirements

This section identifies the requirements of the User to Network Interface.  The following are the requirements.

(R5) The UNI shall be provisionable to support different types of services, protection levels, etc.

(R6a) The optical internetwork shall offer at the UNI a signaling mechanism to enable

- The automatic discovery of client devices to the optical internetwork

- The rapid provisioning of circuits based on a request from the client devices.

- The configurations of parameters of circuits including at least 1) type of circuit (e.g. PLR-Circuit, STE-Circuit, LTE-Circuit, ...), and 2) protection levels of the circuits.

There may be a variety of signaling mechanisms, in particular, signaling may be in-band or out-of-band, may or may not use SONET overhead bytes, etc.  The forms of this mechanism are to be determined.

It is recognized that not all circuit types will support duplex in-band signaling; it is therefore further recognized that it may be that not all circuit types utilize the full functionality of the optical internetwork.

(R6b) The optical internetwork will implement management mechanisms other than signaling to implement the desired optical internetwork functionality.

(R7) The UNI shall be specified in such a way that it supports  detection of failures, localization of the failure (in or out of cloud), and notification of the failure to the appropriate network management entity.

(R8) The UNI shall be specified in such a way that it supports the continual verification of the integrity of a circuit.

### Network to Network Interface Requirements

This section identifies the requirements of the Network to Network Interface.

The requirements are for further study.

## Summary

## References

### Normative references

### Informative references

OIF-99-122 Mark Jones.  Takes a contrary view to many of the requirements in this document.

OIF-99-161 Joe Berthold.  Presents basis and motivation for the defined services.

## Appendix A: Glossary

## Appendix B: Open Issues / current work items

## Appendix C: List of companies belonging to OIF when document is approved

This section will be filled out if/when TC approves document.