Contribution Number: 	OIF99.161

Working Group:	Architecture

TITLE:	Optical internetworking requirements for SDH framed UNIs

SOURCE:	Joe Berthold

CIENA Corporation

920 Elkridge Landing Road

Linthicum, MD 21090

Phone: 410 865 8611

Fax: 410 865 8905

Email: berthold@ciena.com

DATE:	November 29, 1999

ABSTRACT:

This document provides input to the OIF Architecture Working Group Project 1, Optical Internetworking Requirements. It builds on work introduced in contribution OIF99.136, and on motions passed in the October 1999 Architecture working group meeting relating to SDH/SONET framed UNIs. It proposes a number of general requirements for optical internetworking UNIs, and then discusses three UNIs based on SDH/SONET framing. These three UNIs were contained in Options 1 and 2 of contribution OIF99.136.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

# Optical Internetworking Architecture Overview

The Optical Internetworking Architecture provides carriers with a means to rapidly scale network capacity to meet increasing traffic demands, while at the same time reducing both capital and operations costs. The elements of an optical internetwork are data networking elements interconnected across an optical transport network. In general the optical transport network will consist of links employing DWDM transmission systems and switches.

The optical internetworking architecture enables service providers to grow network capacity in the most cost-effective way through the direct connection of line-rate data networking element interfaces with DWDM transport systems. This data element to optical element interface is called a user network interface, or UNI. The UNI specification must be chosen to allow interoperability to be achieved as quickly as possible due to the rapid investments carriers are making in their network infrastructure. The UNI will address physical layer interfaces and network control and management. The UNI physical layer interfaces should be simple to implement, flexible and based on components that are widely available. The UNI will be based on single channel SDH/SONET framed signals for several reasons that will be discussed in more detail later. The major reason for a SDH/SONET based interface is that both data equipment suppliers and optical transport equipment suppliers are using this interface today, and it provides a set of management features that are supported by service provider’s operations systems.

The major reason for a single channel interface for the UNI rather than a multichannel DWDM interface is flexibility and simplicity of achieving interoperability. Data networking elements interface rates and total capacity are growing rapidly. DWDM transmission systems capacity is growing rapidly, both due to increases in optical channel rate and in the number of channels carried on a DWDM transmission system. A new generation of transport switches is being introduced that will allow rapid reconfiguration of the links in an optical internetwork. Single channel interfaces rather than multichannel interfaces allow both backward and forward compatibility between the rapidly changing worlds of data elements and transport systems.

The optical transport network is a circuit switched network that provides reconfigurable, high capacity, very low latency connections. The determinism of circuit switching allows the realization of very large scale, non-blocking switches with simple control methods and predictable behavior.  Scalability and simplicity are important characteristics of transport switches, where total network capacity is expected to be extremely large and where ultra-high availability is required. Switching is provided to rapidly establish and rearrange connections amongst data networking elements, and to provision private line services for a service provider’s customers. Fast switching allows carriers to rapidly provision high capacity links, and to rapidly restore services in the event of network faults.

Traditional transport networks do not support fast switching. The control of switches in transport networks has been traditionally done through network management systems (NMS) and element management systems, (EMS). These largely proprietary systems have not been designed for rapid provisioning and network reconfiguration, nor have they been used to automate the management of connections spanning multiple administrative domains.

Optical internetworks achieve rapid reconfigurability by applying technologies proven in data networks, and adapting them to transport network requirements. These technologies include means to automatically discover network topology, network resources and their utilization, and rapidly disseminate changes in the state of the network. They also include automatic means to request network services and establish connections. Basing the control of optical internetworks on automatic mechanisms proven over the course of many years in the field will help service providers scale network capacity while decreasing operations costs.

# End-user service perspective

The end-user of an optical internetwork desires circuit connections between data networking elements at different locations, potentially separated by a large distance. They want the service activated at a time that meets their needs, and they want to make changes to their connectivity as their traffic demands change. The end-user needs to have performance guarantees, and needs a means to verify that these guarantees are met. When there are faults in the network the end-user needs to be able to determine whether the faults are in the service provider’s domain or in their own domain. The end-user may require high availability connection services for some of their traffic. When the service provider experiences faults in their network and is unable to provide service, the end-user expects rapid notification so it can manage the service outage by other means.

# General Requirements for Optical Internetworks

This section contains general requirements for optical internetworks and a description of several representative examples of optical internetwork applications.

## Multiple protocol support

End-user customers and service providers will deploy networks that support a variety of data protocols. IP may be carried on optical internetworks directly through packet-over-SONET mapping (POS) or over ATM. ATM may be used to carry voice, video and other real-time (non-IP) services. The IEEE 802.3 High Speed Study Group (HSSG) is working on specifications for 10 G Ethernet. For wide area networking applications the HSSG is considering SDH framing as the basis for a physical layer specification. Additionally, carriers will continue to sell SDH/SONET private lines that support customers’ private data networks. In summary optical internetworks need to have the flexibility to support layer 1, 2 and 3 client equipment over SDH framed interfaces.

## Multiple administrative domains

The optical internetworking architecture supports interoperability across multiple administrative domains. Many of the aspects of internetworking among networks of different service providers are covered by NNI specifications.

## Circuit model

Optical transport networks provide circuits between UNIs. Circuits are chosen for transport because of their simplicity, low latency and guaranteed bandwidth. One type of circuit is an optical channel, where the data from a UNI is carried on a DWDM channel across the network without further time division multiplexing. Optical channels may have the data rates of the SDH, starting as low as STM-1. The optical internetworking UNI may support a channelized interface, wherein the data element constructs an SDH multiplexed circuit. In this case circuits have a higher degree of granularity than an optical channel. The constituent elements of the multiplexed circuit may be routed to different destination UNIs. Also, the service provider may choose to use time division multiplexing to increase the capacity of their backbone network, so the timing and synchronization of optical channels needs to be adequate to support SDH time division multiplexing.

## Network dimensions

Optical internetworks must span global distances. Signals must be carried from origin to destination with regeneration at intermediate points as necessary to remove impairments. Where optical channels are carried in a digitally transparent mode, regeneration must be accomplished making use of line-derived reference clock signals. Jitter specifications must be adequate so that SDH multiplex section termination and external timing is not required, although it may be used. An example reference connection would include an access network, a regional transport network, a wide area transport network possibly including submarine links, and then regional and access networks at the destination.

## Connection identification

At the UNI there shall be a means to identify the data element and port of the connection.

## Rapid configurability

The networking elements of optical internetworks shall have mechanisms to support rapid configurability. Rapid configurability will allow service providers to provision services, both to meet customers needs and increase the level of automation of service provider operations. Rapid configurability will allow service providers to support shorter-lived connections and provide a mechanism for traffic engineering. Lastly, rapid configurability will provide a variety of mechanisms for network survivability, including protection and generalized restoration.

One mechanism needed to support rapid configurability is a means to automatically determine network topology, network resources, and the state of operation of network resources. Changes of network state information must be communicated rapidly. A second mechanism to support rapid configurability is a signaling protocol. A signaling protocol should support both provisioning and survivability applications.

# SDH framed UNIs

SDH framing is used at optical internetworking interfaces because of its ubiquity, the availability of a rich set of functionality in ICs, and because service providers’ operations staff and operations systems currently support the SDH standards. Basing initial interface specifications on SDH will speed the availability of interoperable optical internetworks, and preserve the investments service providers have made in support for a SDH infrastructure.

The SDH framed UNI has three modes of operation; an SDH transparent UNI, an SDH regenerator section termination UNI, and an SDH multiplex section termination UNI. The SDH transparent UNI will provide management functionality through non-intrusive monitoring. A device that includes multiplex section termination functionality also includes regenerator section termination functionality. A multiplex section termination UNI may be software provisioned to operate as a regenerator section termination UNI only, or a transparent UNI only. Likewise a regenerator section termination UNI may be software provisioned to operate as a transparent UNI only.

The areas of application of these three UNI types are discussed in the sections below.

## SDH transparent UNI

The SDH transparent UNI is provided to avoid interoperability problems with SDH client signals that may use proprietary features or management techniques. One example is an optical internetwork providing a transmission segment of a SDH multiplex section shared protection ring. The transparent UNI may use non-intrusive monitoring of the SDH signal, but may not alter any of the information payload or SDH overhead.

## SDH regenerator section termination UNI

The SDH regenerator section termination UNI applies in cases where an optical channel is the unit of information to be managed across the UNI and where there is no need for protection signaling across the UNI. Regenerator section termination provides additional management capabilities over the transparent UNI in that B1 can be written by the optical network element on one side of the UNI, and AIS can be generated in the event of loss of signal from the network.

## SDH multiplex section termination UNI

The SDH multiplex section termination UNI applies in several instances. If the UNI supports protection mechanisms requiring a protection signaling protocol, the K1/K2 bytes can be used. A UNI carrying a time division multiplex of channels that are to be routed separately in the network may use multiplex section termination at the UNI. B2 can be written and AIS can be generated at loss of signal. Also, remote defect indicators and remote failure indicators can be forwarded using currently SDH specifications.