Contribution Number: 	OIF2000.082

Working Group:	Signaling

TITLE:  MPLS and SONET

SOURCE:	Greg Bernstein

Ciena Corporation, Core Switching Division

10201 Bubb Road 10480 ***after May 1st: 10480 Ridgeview Court Cupertino

Cupertino, CA 95014

(408) 865-6213 ***after May 1st: (408) 366-4713

Fax: (408) 865-6292 *** after May 1st: (408) 366-4867

Email: greg@ciena.com

DATE: April 21, 2000

ABSTRACT:

This contribution discusses the application and extension of MPLS protocols for use in the control of SONET based optical networks. This work is somewhat tutorial in that it describes some of the decisions needed in choosing how to extend the MPLS protocols to circuit switching in general and then gives examples specialized to the SONET domain. This work was also presented at the 47th IETF meeting in Adelaide Australia.

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the rights to at any time to add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

Connectivity information at the network side for Figure 2.

Note that wavelengths and transmission rates would be explicitly known at the network side for each of these links.

A prime candidate for conveying this information is the

Network Working Group                               				Greg Bernstein

Internet Draft                                      				Ciena Networks

Expiration Date:  							August 2000

Some Comments on the Use of MPLS Traffic Engineering for SONET/SDH Path Establishment

draft-bernstein-mpls-sonet-00.txt

1. Status of this Memo

This document is an Internet-Draft and is in full conformance with all provisions of Section 10 of RFC2026.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as ``work in progress.''

The list of current Internet-Drafts can be accessed at    http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at    http://www.ietf.org/shadow.html.

2. Abstract

In [Awduche] the MPLS Traffic Engineering control plane was applied    to the creation of light paths (optical circuits). Due to the general  hierarchical capabilities of MPLS, and the flexibility of the label     switching paradigm the same techniques used to apply the MPLS control     plane to the optical layer can be used to apply it to the SONET/SDH  path layer and in fact any form of circuit switching. This note     discusses advantages of such an approach and some of the issues     involved in its application.

3. Introduction

In [Awduche] the MPLS Traffic Engineering control plane was applied

to the creation of light paths (optical circuits). This initial work

along with the formation of a new optical signaling working group at

the Optical Internetworking Forum (OIF), and a new industry

coalition, the Optical Domain Service Interconnect(ODSI)has very much

underscored the desire to automate the control of optical networks.

Due to the hierarchical capabilities of MPLS and the flexibility of

the MPLS paradigm, MPLS techniques can be applied to the general

problem of control of hierarchical circuit switched networks. Given

the number of recent internet drafts concerning the optical domain

[Kompella, Wang, Fan, Krishna]  the comments here will be

concentrated on time division multiplexed hierarchies. Before

focusing on some SONET specific issues (note that [Mannie] gives a

good overview of the SDH specific issues) we review some of the

problems to be solved.

3.1 Transport Network Issues

3.1.1 Multi Vendor Topology/Resource Discovery

Although modern transport networks based on SONET/SDH excel at

interoperability in the performance monitoring (PM) and fault

management (FM) areas, they do not inter-operate in the areas of

topology discovery or resource status.  Although link state route

protocols, such as IS-IS and OSPF, have been used for some time in

the IP world to compute destination based next hops for routes

(without routing loops). Their value in providing timely topology and

network status information in a distributed manner, i.e., at any

network node, is immense. If resource utilization information is

disseminated along with the link status (as was done in ATM’s PNNI

routing protocol) then a very complete picture of network status is

available to a network operator for use in planning, provisioning and

operations. Note that in the circuit switch domain bandwidth

utilization and connection admission control is much simplified over

similar concepts in the packet switching world .

Other items of interest for circuit based network control include

switching capabilities of the nodes (granularity, signal types,

etc.), protection properties of the links (linear 1+1, linear 1:N,

ring), failure risk of the links (which links have a tendency to fail

at the same time - ).  At this point the main difference in this

application of link state routing versus that for IP is that no

routes have actually be calculated.

3.1.2 Multi Vendor Connection Control

Traditionally end-to-end circuit connections have been set up via an

equipment vendor’s element management system (EMS). Only limited

interoperability has been achieved via management systems. Hence,

end-to-end circuits in a multi-vendor environment typically require

the use of multiple management systems and the infamous configuration

via “yellow sticky notes”. A common signaling protocol such as RSVP

with TE extensions or CR-LDP appropriately extended for circuit

switching applications can solve this interoperability problem.

3.1.3 Customer/Edge Connection Control

This is the case where the edge device, by desire, does not fully

participate in the routing protocol, i.e., does not receive or share

topology information with the rest of the network. Such a device

would typically be discovered/register through a separate protocol

from the routing protocol.  The edge device would then typically use

a signaling protocol similar to that used in the network to request

services associated with circuits (setup, clear, query). Note that

the multiplex hierarchy used in TDM networks generally alleviates the

scaling issues that could otherwise be troublesome, i.e., a core

SONET switch with OC-192 trunks will not be dealing with signals of

DS0 or DS1 granularity. Defining some these protocols is the type of

work of initial interest at both ODSI and the OIF’s signaling working

group.

3.1.4 Path Computation

Although a link state route protocol can be used to obtain network

topology and resource information, this does not imply the use of an

“open shortest path first” route. The path must be open in the sense

that the bandwidth must be available, however the switches along the

path must also be capable, in some way, of transporting the desired

signal type, i.e., we’ve got an additional constraint.  Other

constraints may include hop count, total delay (mostly propagation),

and hop count. In addition, in addition it may be desirable to route

traffic in order to optimize overall network capacity, reliability or

some combination of the two. Dikstra’s algorithm computes the

shortest path with respect to link weights for a single connection at

a time. This can be much different than the paths that would be

selected in response to a request to set up a batch of connections

between a set of endpoints in order to optimize network link

utilization. One can think along the line of global or local

optimization of the network. Due to the complexity of some of the

route algorithms (high dimensionality non-linear integer programming

problems) and various criteria by which one may optimize their

network it may not be possible or desirable to run these algorithms

on network nodes. However, it may still be desirable to have some

basic path computation ability running on the network nodes,

particularly in restoration situations. Such an approach is in line

with the use of MPLS for traffic engineering but is much different

than typical OSPF or IS-IS usage where all nodes must run the same

route algorithm.

3.2 Decomposition of the MPLS/Circuit Switching Problem Space

Although those familiar with MPLS may be familiar with its

application in a variety of application areas, e.g., ATM, Frame

Relay, etc.  We quickly review its decomposition when applied to the

circuit switching problem space.

(i)	Information needed to compute paths must be made globally

available throughout the network.  Since this is done via the

link state route protocol any information of this nature must

either be in the existing link state advertisements (LSAs) or

the LSAs must be supplemented to convey this information.  For

example, if its desirable to offer different levels of service

in a network based on whether a circuit is routed over SONET

lines are Ring protected vs. not being protected

(differentiation based on reliability). Then the type of

protection on a SONET line would be an important topological

parameter that should be distributed via the link state route

protocol. Other important parameters are described in

[Kompella].

(ii) Information that is only needed between two “adjacent” switches

for the purposes of connection establishment is appropriate for

distribution via one of the label distribution protocols. In

fact this information may form the “virtual” label. For example

in SONET if we are distributing information to switches

concerning an end-to-end STS-1 path traversing a network. It is

critical that adjacent switches agree on the “time slot” used

by this STS-1 (but this information is only of local

significance between the two switches). Hence the time slot

number in this case can be used as a virtual label. Note that

it is virtual in that it is not appended to the payload in

anyway, but it is still a label in the sense that it uniquely

identifies the signal local to the link between the two

switches.

(iii) Information that all switches in the path will need to know

about a connection will also be distributed via the label

distribution protocol. Example of such information can include

bandwidth, priority, and preemption information.

(iv) Information intended only for end systems of the connection.

Some of the payload type information in [Mannie] may fall into

this category.

4. SONET Considerations

SONET/SDH is a TDM based multiplexing technology that has a number r

of standard options that a network user may choose to use. In n

addition a number of extensions have been proposed [Jones] or are e

beneficial for network operations, e.g., eliminate the need for link k

grooming.  This draft reviews the SONET multiplex structure, options s

and possible extensions with respect to the information that is s

required to be shared between MPLS LSRs working at the SONET layer r

for the establishment of SONET layer LSPs.

4.1 SONET Structure and Extensions

The fundamental signal in SONET is the STS-1 (about 51 Mbps). This signal consists of a transport overhead and a Synchronous Payload Envelope (SPE). The SPE floats within its allotted space within the

SONET frame structure with the pointer bytes (H1, H2 and H3) in the Line Overhead of the SONET transport overhead pointing to the beginning of the SPE.  An STS-N signal is formed from a SONET STS-(N- 1) signal and an STS-1 signal via byte interleaving.  The transport

overhead structures are frame aligned prior to interleaving but this

is not required of the SPEs, i.e., there is no special relationship

between the payload envelopes.  To transport signals in excess of

about 50Mbps the SPEs can be concatenated, i.e., glued together. In

this case their relationship with respect to each other is fixed in

time and hence this relieves, when possible, and end system of any

inverse multiplexing bonding processes.

Due to the previously describe structure the end points of SONET

connections can be identified by the “time slots” (position) that

they occupy within the interleaved frame structure. In the standard

SONET case we are concerned with which of the M STS-1 paths within an

STS-N signal will be used to transport our data (M <= N, and N = 3,

12, 48, 192,...). The SPEs of these M STS-1s can be concatenated to

form an STS-Mc. The STS-Mc notation is really a short hand way of

describing an STS-M signal whose SPEs have been concatenated.

In BellCore GR-253 [GR-253] section 6.1.2 (requirement R6-3) two

conventions are given for identifying an STS-1 within an STS-N:

- A two-level "STS-3 #, STS-1 #"

- A single-level "1 to N in order of appearance at the input to

the byte-interleaver"

For example STS-1 number 23 within an OC-48 can also be represented d

by the tuple (8, 2).  We will be using the single-level numbering g

scheme in our discussion, but this is not imply an encoding format.

A second complication is in dealing with concatenated signals. In n

Bellcore GR-253 section 5.1 the multiplexing procedures for SONET are e

given. Constraints are imposed on the size of STS-Mc signals, i.e. ,

they must be a multiple of 3, and on their starting location and d

interleaving.  This has the following advantages: (a) restriction to o

multiples of 3 helps with SDH compatibility (there is no STS-1 1

equivalent signal in SDH,  an STM-1 is equivalent (essentially) to an

STS-3c); (b) the restriction to multiples of 3 reduces the number of f

connection types; (c) the restriction on the placement and

interleaving could allow more compact representation of the “label”;

The major disadvantage of these restrictions is: (a) Limited d

flexibility in bandwidth assignment (somewhat inhibits finer grained d

traffic engineering). (b) The lack of flexibility in starting time e

slots for STS-Mc signals and in their interleaving (where the rest of f

the signal gets put in terms of STS-1 slot numbers) leads to the e

requirement for re-grooming (due to bandwidth fragmentation).

4.2 SONET Concatenation Extensions

Due to these disadvantages some SONET framer manufacturers now

support “arbitrary” concatenation, i.e., no restrictions on the size

of an STS-Mc (as long as M <= N) and no constraints on the STS-1

time slots used to convey it. It is recommended that arbitrary

concatenation be supported in the format for SONET labels. It is also

recommended that the use of arbitrary concatenation or “standard”

concatenation be negotiated as part of the label assignment process

between two SONET LSRs.

Note that arbitrary concatenation as used here is a network service

that is similar in nature to the SONET end system service of higher

order virtual concatenation [Jones], [Mannie]. In one example of

virtual concatenation two end systems supporting this feature could

essentially “inverse multiplex” two STS-1s into a virtual STS-2c for

the efficient transport of 100Mbps Ethernet traffic.  The burden in

virtual concatenation is on the end systems while in arbitrary

concatenation it is on the network.  In addition arbitrary

concatenation includes arbitrary interleaving which avoids the need

for link regrooming between any pair of nodes supporting this

feature.

4.3 SONET Connection Bundling

Connection Bundling is the process of routing a set of non--

concatenated STS-1s together as a group, i.e., they are all contained

within the same SONET line (or WDM signal) and receive essentially

the same delay and propagation. This simplifies connection

establishment (especially for batches of DS-3s that are being

wholesaled) and speeds re-routes. Such bundling may be important when

establishing STS-1s that will be used between end systems

implementing virtual concatenation. It is recommended that the labels

chosen for SONET paths can incorporate the concept of STS-1 bundling.

Whether it is desirable to bundle larger signals, i.e., groups of

STS-Mc, is for further study.

4.4 SONET Transparency

One last transport technique that bears mentioning since it can be

viewed as a service is that of transparent multiplexing or switching.

The SONET over head is broken into three layers: Section, Line and

Path. All these layers are concerned with fault and performance

monitoring. Section overhead is primarily concerned with framing and

Line overhead is primarily concerned with multiplexing and

protection. To perform multiplexing a SONET network element should be

line terminating. However not all SONET multiplexers/switches perform

SONET pointer adjustments on all the STS-1s contained within them or

if they perform the pointer adjustments they do not terminate the

line overhead. For example a multiplexer may take four SONET STS-48

signals and multiplex them onto an STS-192 without performing

standard line pointer adjustments on the individual STS-1s.  This can

be looked at as a service since it may be desirable to pass SONET

signals, like an STS-12 or STS-48, with some level of transparency

through a network and still take advantage of TDM.  Transparent

multiplexing and switching can also be viewed as a constraint since

some multiplexers and switches may not switch at as fine a

granularity as others.  The levels of transparency and their

representation is for further study.

4.5 SONET Protection

SONET and SDH networks offer a variety of protection options at both

the SONET line and SONET path level.  Standardized SONET line level

protection techniques include Linear 1+1 and Linear 1:N automatic

protection switching (APS)[GR-253] and both two fiber and four fiber

bi-directional line switched rings (BLSRs) [GR-1230]. At the path

layer SONET offers unidirectional path switched ring protection.

Both ring and 1:N line protection also allow for “extra traffic” to

be carried over the protection line when it is not being used, i.e.,

not carrying traffic for a failed working line. It may be desirable

to route some connections over lines with protection of a given type,

unprotected lines, or primarily over protection lines as “extra

data”. In order to do this the method of protecting a line (if any)

or whether a line is a protection line is useful topology information

that can be disseminated via the link state route protocol. In

addition, a signaling protocol should allow the optional

specification of link protection types as one of the connection

attributes.

5. Summary

This note has discussed some of the advantages of a circuit switching

control plane based on MPLS in terms of the current interoperability

problems that MPLS can help solve. An overview of the application of

MPLS to circuit switching and its decomposition into routing and

label distribution components was presented. And, finally a few of

the subtleties particular to SONET that need to be taken into account

when an MPLS control plane is applied to this application were

detailed.

6. Acknowledgements

The author would like to thank Yakov Rekhter and Jeff Weiss for a

number of enlightening and stimulating discussions that prompted

these notes.

7. References

[Awduche] Awduche, D., Rekhter, Y., Drake, J., Coltun, R., “Multi-Protocol Lambda Switching: Combining MPLS Traffic Engineering Control With Optical Crossconnects”, draft-awduche-mpls-te-optical-01.txt

[Jones] Jones, N., Murton, C., “Extending PPP over SONET/SDH, with virtual concatenation, high order and low order payloads”, draft-ietf-pppext-posvcholo-01.txt.

[GR-253] Bellcore Generic Requirements, GR-253-CORE, Synchronous s

Optical Network (SONET) Transport Systems: Common Generic Criteria ,

Issue 2, December 1995.

[Mannie] Mannie, E., “MPLS for SDH control”, draft-mannie-mpls-sdh-

control-00.txt.

[Kompella] Kireeti Kompella, et. al., “Extensions to IS-IS/OSPF and

RSVP in support of MPL(ambda)S”, draft-kompella-mpls-optical-00.txt.

[Wang] Gouqiand Wang, et. al., “Extensions to OSPF/IS-IS for Optical

Routing”, March 2000, Internet Draft, draft-wang-ospf-isis-lambda-te-

routing-00.txt.

[Fan] Yanhe Fan, et. al., “Extensions to CR-LDP and RSVP-TE for

Optical Path Set-up”, March 2000, draft-fan-mpls-lambda-signaling-

00.txt.

[Krishna] Murali Krishnaswamy, et. al., “MPLS control plane for

Switched Optical Networks”, 2/25/00,draft-krishnaswamy-mpls-son-

00.txt.

8. Author Information

Greg Bernstein

Ciena Core Switching Division

10201 Bubb Road, Cupertino, CA 95014

***after May 1st: 10480 Ridgeview Court Cupertino, CA 95014

e-mail: Greg@ciena.com

Phone: (408) 865-6213 ***after May 1st: (408) 366-4713