Contribution Number:

Working Group: Signaling Working Group

TITLE:     User to Network Interface (UNI) service definition and lightpath attributes

SOURCE:	Larry McAdams	Jennifer Yates	Working Group Chair

Technical Editor	Technical Editor	Working Group Chair

Cisco Systems	AT&T Labs	Company

170 West Tasman Drive	180 Park Avenue	Address 1

Address 2

San Jose, CA, 95134	Florham Park, NJ, 07932	City, State, Zip

Phone: (408) 525 4628	Phone: (973) 360 7036	Phone:

Email: lmcadams@cisco.com	Email: jyates@research.att.com	Email:

DATE: 2/15/00	Date of this Draft/ revision

Document Status:  DRAFT   revision number or APPROVED

Project Name: Signaling Requirements and Mechanisms for the UNI

Project Number:

Notice: This Technical Document has been created by the Optical Internetworking Forum (OIF).  This document is offered to the OIF Membership solely as a basis for agreement and is not a binding proposal on the companies listed as resources above. The OIF reserves the rights to at any time to add, amend, or withdraw statements contained herein. Nothing in this document is in any way binding on the OIF or any of its members.

The user's attention is called to the possibility that implementation of the OIF implementation agreement contained herein may require the use of inventions covered by the patent rights held by third parties. By publication of this OIF implementation agreement, the OIF makes no representation or warranty whatsoever, whether expressed or implied, that implementation of the specification will not infringe any third party rights, nor does the OIF make any representation or warranty whatsoever, whether expressed or implied, with respect to any claim that has been or may be asserted by any third party, the validity of any patent rights related to any such claim, or the extent to which a license to use any such rights may or may not be available or the terms hereof.

For additional information contact:
The Optical Internetworking Forum, 39355 California Street,
Suite 307, Fremont, CA 94538
510-608-5990 phone  info@oiforum.com

© 2000 Optical Internetworking Forum

## Table of Contents

## Abstract

This document specifies the attributes associated with a lightpath and their use in service definition.

## List of Figures

## List of Tables

## Document Revision History

3/3/00 Created by Jennifer Yates and Larry McAdams.

## Project Summary

Note: This section is removed when the document is forwarded to the TC for vote.  It is used to help maintain the progress of the document.  It is expected that appropriate key content form this section is contained in the Introduction.

### Working Group project(s)

Signaling Requirements and Mechanisms for the UNI

### Working Group(s)

Signaling

### Date Approved	2/1/00

### Original Document

### Problem Statement

[From project description] Signaling is needed to allow end systems to inter-operate intelligently with and within an optical network  ******

### Scope

Lightpaths are defined to be fixed bandwidth connections established between two network elements via optical layer crossconnects.  These lightpaths may have a number of different attributes, which define the associated service. The lightpath attributes are defined by the client when the lightpath is requested.

### Expected Outcome

This document on lightpath attributes is intended to have the following content:

- The definition of lightpath attributes and their use for service definition

### Schedule

[From Project Description] Draft documents by late summer, OIF approved document by end of year.

### Merits to OIF

[From Project Description] Fosters interoperability between edge systems and optical networks. Supports the signaling portion of the UNI 1.0 goal of the OIF for year 2000.

### Merits to working group

The definition of lightpath attributes and their use for service definition is crucial in establishing a signaling relationship across the UNI.

### Relationship to other Working Groups

#### Overlaps

Currently none.

#### Unique viewpoints

### Relationship to other Standards Bodies

[From Project Description] Closely related to work at IETF, ITU-T and T1. Technical documents will be exchanged as appropriate.

## Introduction

The goal of this document is to specify lightpath attributes and their use in service definition.  However, the full scope of this document needs to evolve as other related documents are created.

The generic requirements for the Optical Internetwork [OIF00.07.0] specify that a number of different types of clients must be supported; that rapid provisioning should be supported; that various levels of circuit protection be provided; that some form of mesh restoration be supported; and that a high level of manageability and provisioning flexibility be provided for SDH/SONET framed circuits.

OIF00.07.0 defines the UNI as the interface between the optical network and a “client” to the optical network. It is required that the UNI be provisionable to support different types of services, protection levels, etc.; that a signaling method be provided between the client and the UNI that might be implemented in a number of ways (in-band, out-of-band, may or may not use SONET overhead bytes); that not all circuit types will support duplex in-band signaling; and that the UNI shall be specified so as to allow failure detection and localization (in or out of the Optical Network) and notification of the failure to the appropriate network management entities, and also continual verification of the integrity of the circuit.

At the first meeting of the Signaling WG during the January 2000 OIF meeting there was general consensus based on a proposal from AT&T (OIF 00.004.0) to adopt IP mechanisms and algorithms for the control of the optical layer.  Adopting a unified protocol across both the optical network and its clients fundamentally enhances the flexibility of the control plane.  Now, depending on the nature of the network and the service offered, both a client-server or overlay model of the network is possible (i.e. UNI) and a peer-to-peer model is possible (i.e. NNI).  In some cases, the NNI may be simply viewed as an extension of the UNI.

The scope of this document will need to evolve as other related documents are extended.  As such, we have included sections here that may be moved to other documents at a later stage. In this document, red text is used to highlight sections that the editors are particularly uncertain about their relevance to this document.

## Lightpath services

The optical network is primarily offering high bandwidth connectivity in the form of lightpaths, where a lightpath is defined to be a fixed bandwidth connection between two network elements, such as IP routers or ATM switches, established via the optical network.  The behaviors of the lightpaths are defined by their lightpath attributes.

The definition of an atomic lightpath request needs to be discussed.  In particular, whether the atomic lightpath request is for a single connection, or could be for multiple lightpaths, needs consideration.  The simplest approach is to define the atomic lightpath request to be for a single lightpath.  However, more flexibility might be achieved using a more complicated definition, in which multiple lightpaths may be simultaneously requested.

## Lightpath operations

The fundamental actions or operations related to a lightpath are:

- request for a new lightpath

- disconnection or teardown of an established lightpath

- lightpath query (i.e. obtain current status information re: lightpath)

- lightpath attribute change (i.e. change a parameter regarding an established lightpath – e.g. restoration requirements)

These operations are to be conveyed across the UNI, along with other information (such as topology information), which should be defined within other OIF Signaling Group documents.  These operations are currently defined here, but could be moved to other documents at a later stage.

## Lightpath attributes

A lightpath request, query, disconnect or attribute change has associated attributes.  The following is a list of the attributes associated with a lightpath.

It is envisioned that these attributes will be negotiated over the signaling channel between the client and the network (i.e. over the UNI or in some cases with the network’s management plane).  Many of the attributes are denoted as being required attributes, meaning that their definition is essential for the control plane operation.  However, other attributes are optional.  In some cases, not all of the attributes will be available, depending on the services offered and the equipment available. However, the UNI specification should not preclude any attribute.

Connection-related attributes

The first group of attributes are important in the connection establishment process and in ongoing capacity management and maintenance activities.  This material might overlap with other Signaling Working Group documents; until these documents are further fleshed out it is difficult to tell.

- globally unique end-to-end lightpath identifier: an identifier for the lightpath. This identifier may be assigned by either the client, or by the network.  This is a required attribute in the sense that it must be known or derivable by the network.

- client identifier: an identifier by which we can refer to the business entity which “owns” the lightpath. It could also identify a specific “virtual optical network” (analogous to IP VPN’s) if the need arose.  This is important for connection acceptance, billing, etc.  It appears to differ from the information obtainable by end system discovery, which is focused on topological information. (See [OIF 2000.052.0].) In some cases it may be derivable from port information. This is a required attribute in the sense that it must be known or derivable by the network.

- security object: required for authentication, but could be closely related to the client identifier.  The necessity and details of this needs further study.

- desired availability period: the date/time when the lightpath is desired to be in service, and the earliest and latest date/time when the lightpath will be disconnected.  The disconnect information is needed for capacity management; it might also be a factor in determining charges. It could be set to infinity.  The inclusion of this attribute within the optical network as opposed to a higher layer management system should be further investigated.

- destination address: the address of the destination to which the lightpath is to be established. It is proposed that this address be an IP address. However, it is unclear whether additional addressing schemes need to be understood by the optical network to handle non-IP aware equipment, such as ATM switches.  Should the optical network understand multiple higher layer addressing schemes, or should the non-IP aware boxes (e.g. ATM switches) be assigned IP addresses by which they are referred to in the optical network? These IP addresses could be associated with the ports to which the clients are attached to the optical network. This is a required attribute.

- destination port index: if individual ports are not given unique addresses, then a port index is required to identify them.  The destination port index used by a lightpath may be assigned by either the client, or by the network.  This is a required attribute.

- destination sub-port index: when the network or end system allows multiplexing or switching at a finer granularity below the port level then the sub-port index is used to refer to specific channels below the port level.  In future more highly integrated systems there may be a need for additional levels of sub-port indexes.  The destination sub-port index may be assigned by either the client or the network.  This is an optional attribute.

- source address: the address from which the lightpath is to be established. This is a required attribute in the sense that it must be known or derivable by the network.

- source port index: similar to destination port index.  The source port index may be assigned by either the client or the network.  This is a required attribute in the sense that it must be known or derivable by the network.

- source sub-port index: similar to destination sub-port index.  The source sub-port index may be assigned by either the client or the network.  This is an optional attribute.

Physical attributes

The next group of attributes relate to the physical and transmission characteristics of a lightpath.

- framing type and use: the line protocol carried on the lightpath.  Examples of such line protocols include SONET/SDH and Ethernet (GbE and 10GbE).  This parameter may not be required in an optically transparent network.  The proposed digital wrapper may allow some simplification of this attribute to the extent that it is used to encapsulate line protocols, but it cannot be assumed to be ubiquitous.  This is a required attribute.

Other attributes need to be associated with specific line protocols.  For example, transparency and drop side protection attributes need to be associated with lightpaths established using SONET/SDH framing.

transparency: Because of its ubiquity, SONET/SDH framing is particularly important.  Three modes of operation that could be supported at the UNI are identified in OIF 99.161 (Joe Berthold):

- Transparent: All SONET/SDH overheads would be transported unchanged. This would avoid interoperability problems with client signals using proprietary features or management techniques.

- Regenerator Section (SONET Section) Termination: AIS could be inserted, and a segment of a SDH MS Shared Protection Ring (SONET BLSR) could be transported.  Protection would be limited to 1+1.

- Multiplex Section (SONET Line) and Regenerator Section Termination: The tributaries within a channelized facility could be routed separately, and the K1/K2 bytes would be available for use within the Optical Layer.  M:N Linear Automatic Protection Switching could also be supported.

Performance monitoring and fault handling for each of these options needs further study. This is a required attribute.

drop side protection: This refers to protection between the client network element and the optical network.  In the case of co-location, the choices would seem to be 0:1 (unprotected), 1:N, M:N and 1+1.  The non-colocated case needs further study.  This is a required attribute.

Similar attributes will need to be investigated / defined for other framing types.

- capacity: bandwidth associated with the lightpath.  For example, a lightpath used to transport SONET encoded signals may be configured to have OC-N capacity, or STS-M capacity.  STS-1 granularity is suggested for SONET encoded signals, although this does not imply that a vendor has to implement such a granularity. This parameter may not be required in an optically transparent network.  Asymmetric capacity might be provided by combining bi-directional and uni-directional capacity assignments; however this requires further study.  This is a required attribute.

- directionality: lightpaths may be either uni-directional or bi-directional. This is a required attribute.

- priority and pre-emptability: This multi-level attribute determines a lightpath’s treatment during a restoration and provisioning events.  This attribute would specify whether or not the resources used by a lightpath are reusable by other lightpaths in the event that there is insufficient capacity to handle a failure or the provisioning of a new lightpath with a higher priority.  A simple, single level form of this attribute is required, while more elaborate multi-level forms are optional and may require further study.

- protection and restoration: numerous possible definitions of protection level exist. The simplest is to have a binary decision as to whether the lightpath is restored or not.  At a finer granularity, different types of restoration can be specified.  These need to be defined in terms of sub-attributes, such as:

- restoration time,

- what restoration capacity would be used (dedicated or shared, for example),

- what restoration method should be used (mesh or BLSR, for example),

- what failures are targeted (single link, single node, two links, ...), and

- reversion strategy –  does the lightpath get restored to its original route? An “optimal” available route?  When?

It is preferable that the definition of this attribute be functional (e.g., 100 mSec restoration time) rather than solution-specific (e.g., SONET BLSR).  However, the above attribute provides the ability for solution-specific restoration requirements.  Maximum flexibility should be left for vendor and network operator innovation without requiring a standards change. A simple form of this attribute is required, while more elaborate forms will require further study.

Routing attributes

- relationships between lightpaths: Various relationships may be defined between lightpaths.  For example, a client may request that multiple lightpaths be diversely routed, that multiple lightpaths be routed along the same physical route, that multiple lightpaths be bundled together and treated as a single entity with a common set of attributes, or that a given lightpath be routed on the same path as an existing lightpath.

Two lightpaths are diverse if they have no Shared Risk Link Groups in common. (See OIF 2000.04.0 for more on Shared Risk Link Groups.) Diverse routing is frequently a requirement of sophisticated enterprise networks whose availability objectives may require that no single failure isolate a node or disconnect the network, for example.  The diversity requirement for such a network is best expressed by a matrix: If there are N lightpaths involved, then ajk specifies whether lightpaths j and k must be diverse.  This attribute is for further study.

This attribute requires further study.  In particular, the definition of how multiple lightpaths may be bundled together requires further work.

- specified routing: client specification for specific physical routing of a lightpath.  This routing may be specified in terms of specific links or nodes which must/must not be traversed.  Constraints such as the maximum delay or number of hops should also be supported.

The delay is of particular importance.  There are a number of ways to characterize delay.  A simple metric is the total round trip delay; however it may be desirable to create a more detailed description of the temporal behavior.  For example, in some cases, the differential delay between the incoming and the outgoing directions of the lightpath may be important. Finally, the change in delay caused by a protection event may also be important. In other words, it may be desirable to not only establish the temporal behavior of the primary lightpath, but also the temporal behavior of the lightpath after it has been restored, so that the delay is bounded on both the primary and restoration routes.  Delay along a lightpath will be directly proportional to the route mileage of the lightpath, so it may be appropriate to formulate this constraint in mileage terms.

This attribute requires further study.

- client constraints: client constraints may have to be propagated across the UNI - particularly in all-optical networks without wavelength conversion available between the client and the network.  In particular, information may have to be conveyed regarding the set of wavelengths accessible from the client and whether the client can re-tune its wavelengths in the face of a failure within the network.  This parameter requires further investigation.

## Summary

This document specifies lightpath attributes and their use in service definition.  Note that the scope of this document needs to evolve as other documents form.

## References

### Normative references

### Informative references

## Appendix A: Glossary

## Appendix B: Open Issues / current work items

## Appendix C: List of companies belonging to OIF when document is approved

Figure 1 This is the figure 1 caption.
Note:  Figure and figure title are in text box

Note:  Figure and figure title are in text box

| Parameter 1 | Value 1 | Value 2 | Value 3 |
| --- | --- | --- | --- |
| Row 1 | a | b | c |
| Row 2 | d | e | f |
| Row 3 | g | h | i |

Table 1 Title of table
Note:  Table and table title are in text box