# OIF Contribution Cover Sheet

Contribution Number:   OIF2000.085

Working Group:	Signaling

TITLE:	ODSI Functional Specification, v1.3

SOURCE:	Rick Barry

Sycamore Networks

10 Elizabeth Drive

Chelmsford, MA 01824

rick@sycamorenet.com

DATE:		April 22, 2000

ABSTRACT:  This document contains the ODSI Functional Specification agreed to at the April 6th 2000 ODSI meeting in Chicago, IL. (version 1.3)

The editors are G. Bernstein (Ciena),  J. Weiss (Ciena), R. Coltun (Siara Systems), J. Moy (Sycamore Networks), A. Sodder (Tenor Networks), K. Arvind (Tenor Networks).

Notice: This contribution has been created to assist the Optical Internetworking Forum (OIF).  This document is offered to the OIF solely as a basis for discussion and is not a binding proposal on the companies listed as resources above. Each company in the source list, and the OIF, reserves the right to at any time add, amend, or withdraw statements contained herein.

This Working Text represents work in progress by the OIF, and must not be construed as an official OIF Technical Report.  Nothing in this document is in any way binding on the OIF or any of its members.  The document is offered as a basis for discussion and communication, both within and without the OIF.

For additional information contact:

The Optical Internetworking Forum, 39355 California Street,

Suite 307, Fremont, CA 94538

510-608-5990 phone  info@oiforum.com

© 1998 Optical Internetworking Forum

ODSI Coalition                                             		     G. Bernstein

April 2000                                                     		            J. Weiss

Version 1.3                                                        	   	  Ciena

R. Coltun

Siara Systems

J. Moy

Sycamore Networks

A. Sodder

K. Arvind

Tenor Networks

Editors

Optical Domain Service Interconnect (ODSI)

Functional Specification

Foreword

The goal of ODSI is to allow user devices to dynamically request

bandwidth from the Optical network.  The Optical network consists of

transmission and switching equipment that can provide point-to-point

connections to attached user devices. These connections can be of

various shapes and sizes, such as SONET-encoded leased line service,

gigabit ethernet, even dedicated optical wavelengths where the

encoding need not be known to the optical network itself.  Typical

user devices requesting these bandwidth services from the Optical

network would include IP routers, SONET add/drop multiplexers, and

ATM switches.  Bandwidth received from the Optical network will then

be used as point-to-point connections between routers, or trunks

between add/drop muxes and ATM switches.

Current provisioning of bandwidth within the Optical network is

basically static; a slow and painstaking operation that takes much

configuration and often requires redesign of optical network

internals. However, two simultaneous developments in networking

technology make the dynamic request of bandwidth from the optical

network both feasible and desirable. First, a new generation of

optical switches is enabling dynamic point-and-click bandwidth

provisioning by network operators. Second, traffic engineering

[Ref5] and constraint-based routing enhancements to IP routers

[Ref6, Ref7] and/or ATM switches are allowing these devices to

dynamically determine when and where they need additional bandwidth.

Allowing these devices to make requests to the Optical network

directly reduces the workload on the network operator.

[Page 1]

ODSI Functional Specification                                 April 2000

This document provides a technical overview of ODSI.  Detailed

protocol specifications can be found in a separate series of

documents ([Ref11], [Ref12] and [Ref13]).

C Copyright ODSI Coalition.

THIS DOCUMENT WAS CREATED AFTER EXTENSIVE COLLABORATION AMONG THE

AUTHORS AND CONTAINS COPYRIGHT MATERIAL.  DISTRIBUTION IS LIMITED TO

ODSI MEMBERS.  REPRODUCTION OF THIS DOCUMENT, IN WHOLE OR IN PART,

FOR USE OTHER THAN IN CONNECTION WITH ODSI IS STRICTLY PROHIBITED

WITHOUT PERMISSION OF ODSI.

[Page 2]

ODSI Functional Specification                                 April 2000

Table of Contents

1        Introduction ........................................... 4

1.1      Terminology ............................................ 4

1.2      Document Organization .................................. 5

2        Service Definition ..................................... 6

2.1      Endpoint identification ................................ 7

2.2      Bandwidth characteristics .............................. 8

2.3      Actions ................................................ 9

2.4      Unsupported services .................................. 10

3        Protocols ............................................. 11

3.1      Service discovery ..................................... 12

3.2      Address registration .................................. 14

3.3      Signaling ............................................. 14

4        Example usage ......................................... 16

5        Traffic engineering example ........................... 17

6        Accounting/security ................................... 18

7        Network management .................................... 19

A        SONET-specific ODSI attributes ........................ 20

A.1      Channel numbering ..................................... 20

A.2      In-band control channels .............................. 21

A.3      Detection of established connections .................. 24

A.4      Interaction with SONET protection schemes ............. 24

B        Digital wrappers ...................................... 25

B.1      In-band control channels .............................. 25

C        User devices without IP addresses ..................... 26

D        SONET ADMs ............................................ 27

References ............................................ 28

Editors' Addresses .................................... 30

[Page 3]

ODSI Functional Specification                                 April 2000

1.  Introduction

The Optical Domain Service Interconnect (ODSI) defines a interface

to the Optical network whereby user devices, such as routers, SONET

add/drop multiplexers, and ATM switches can dynamically request

bandwidth from the optical network. This allows leased lines to be

created (dialed) and destroyed in an on-demand fashion.

This is not a new concept. For example, creation of bandwidth in an

on-demand fashion is currently possible through ISDN, ATM and Frame

Relay Switch Virtual Circuits (SVCs), as well as the PSTN. The main

difference between ODSI and these technologies is that ODSI is

dealing with much larger bandwidth channels and is going to be used

by user devices that are already aggregating a lot of traffic.  As

mentioned in the foreword, advances in technology are enabling the

extension of bandwidth-on-demand concepts to the higher bandwidth

optical network.  You can think of ODSI as an Optical UNI.

Since bandwidth-on-demand schemes have been around for years, there

are a number of protocols which can be used to signal bandwidth

requests.  These protocols must be modified/enhanced to support

characteristics unique to the Optical network. Since TCP/IP has

become the universal control protocol for data communications, we

choose to extend IP addressing and protocols to signal bandwidth

requests to the optical network, as discussed in Sections 2 and 3.

Complete specification of these protocol extensions will be the

subject of a separate document.

ODSI is complementary to the proposal "Multi-Protocol Lambda

Switching: Combining MPLS Traffic Engineering Control With Optical

Crossconnects" [Ref4], which provides an outline of how to implement

the internals of an Optical network. ODSI integrates the two levels

of traffic engineering, that provided by the IP routers [Ref5] and

the traffic engineering implemented in the optical network (one

possible framework for which is presented in [Ref4]), while

maintaining a separation that allows the two levels to evolve

separately and protects one level from the other's failures.

1.1.  Terminology

Throughout this document we use the following terminology.  Note

that some terms, such as "admission control" and "third-party",

are used somewhat differently than their traditional sense.

o   The endpoints at the end of the dynamic bandwidth connection

are called "user devices". These can be IP routers, ATM and

Frame Relay switches, SONET ADMs, etc.

[Page 4]

ODSI Functional Specification                                 April 2000

o   The collection of network devices creating the bandwidth

connection are termed the "optical network". Individual

components of the optical network are called "network

devices".  In particular, a user device connects to the

optical network through a network device.  Note that the

optical network can contain non-optical components, such as

traditional SONET cross-connects and ADMs. Likewise, some of

the user devices may employ optical technology, such as IP

routers with optical interfaces.

o   The dynamically created bandwidth connecting two user

devices is called a "bandwidth channel" or "circuit".

o   "Third-party" signaling means simply that a user device

(such as a network management station or provisioning

server) can request that bandwidth be created between two

user endpoints, neither of which is the requester itself.

This does not imply that the endpoints themselves are

incapable of signaling. In fact, in ODSI third-party

signaling the two endpoints participate in signaling, they

just don't initiate the request.

o   By "admission control" control is meant the authorization

function that the network performs to ensure that the

requester is allowed to setup up a connection between two

endpoints. This authorization is based on the user group

that the requester and endpoints belong to.  "Admission

control" is not used to denote the function whereby the

optical network determines that there are sufficient

resources for the connection.

o   A control channel available for ODSI messaging that is

directly related to the user device's port into the optical

network is called an "in-band control channel". These

channels also go by the name facility-associated signaling

in the literature; one example is the SONET Line Data

Communication Channel (DCC) contained within an OC-48 port's

SONET Line overhead bytes.  Channels unrelated to any port

into the optical network, such as a 10-base-T hub connecting

the user and network device, are called "out-of-band control

channels" (or non-facility associated signaling).

1.2.  Document Organization

This document is organized into the following Sections:

o   Section 2, "Service specification", provides a description

of what can be requested from the optical network: how to

[Page 5]

ODSI Functional Specification                                 April 2000

identify the endpoints to be connected, and the

characteristics of the bandwidth channel that will connect

them.

o   Section 3, "Protocols", gives a brief overview of the

protocols that are used to make bandwidth-on-demand requests

of the optical network.

o   Section 4, "Example usage", gives a simple example of how

the protocols are used to bring up dynamic leased line

service between routers.

o   Section 5, "Traffic engineering example", gives a more

detailed example.  In particular, this example shows how

traffic engineering and constraint-based routing within the

user devices attached to the optical network can be used to

detect when and where to request additional bandwidth.

o   Section 6, "Accounting/security", describes protocol options

that enable bandwidth requests to be verified and logged,

possibly for accounting or billing purposes.

o   Section 7, "Management", describes the Management

Information Base (MIB) that can be used to configure ODSI

services and monitor the state of ODSI-related functions.

Various appendices provide ODSI information that is specific to

various physical layer interfaces (e.g., SONET) over which ODSI

functionality will be supported.

2.  Service Definition

The service provided to the user devices attaching to the optical

network is the dynamic creation of point-to-point bandwidth channels

between user devices. There are three components to the service

definition: 1) how do you specify the endpoints of the bandwidth

channel 2) what are the characteristics of the bandwidth and 3) what

actions can you request on the bandwidth channel. These components

are expanded upon in the following subsections.

ODSI assumes the following model. A user device is connected to the

optical network through one or more physical interfaces, such as

OC-48 links. The user device wishes to dynamically connect these

interfaces to other user devices attached to the optical network.

When such a connection is established, it looks like a leased line

connecting the two user devices. The optical network is providing a

bandwidth service, but one that is not packet-based. The optical

network has no concept of queueing or other packet-based quality of

[Page 6]

ODSI Functional Specification                                 April 2000

service features; these functions are left to the user devices. If

the user device's interface supports channelization, parts of the

interface may be connected to separate destinations; for example,

each of say 4 STS-12cs contained in an OC-48 interface may be

connected by ODSI to separate remote routers. While the network

devices do not examine the data packets transmitted over the user

device's interface to the optical network, the physical layer of the

interface may support a separate in-band control channel that can be

used by the packet-based ODSI protocols described in Section 3. One

example of an in-band control channel is the SONET Line Data

Communication Channel (DCC) contained within the SONET Line overhead

bytes.

ODSI is based on IP protocols and addressing, assuming that the

control network used by the Optical network, and extended to user

devices through the use of ODSI, is IP-based.

The IP addresses assigned to the user devices serve multiple

purposes, as endpoint IDs of circuits, and as the source and

destination of ODSI protocol packets. The latter function implies

that the IP addresses are routable, at least in the context of the

Optical network's IP control network.

2.1.  Endpoint identification

Each endpoint of the requested bandwidth is specified as a

combination of one or more of the following parameters.

o   IP address. An IP address is associated with a user device,

and can be associated with one, some, or all of that

device's physical port attachments to the optical network.

o   Port index. Identifies a particular physical interface

connecting into the Optical network. In IP MIBs, this is

commonly an IfIndex.

o   Channel. When establishing sub-rate bandwidth connections on

multiplexed interfaces, this identifies which sub-rate

channel should be connected.  For example, on a SONET

interface this would be a timeslot, as specified in Section

6.1.2 of [Ref10].

o   User group. A further qualification on the IP address; a

port is only allowed to attach to IP addresses associated

with the same user group, similar to the closed user group

concept in X.25. For example, if there were 10 ISPs whose

routers were all attached to the same optical network, each

ISP would have a separate user group identifier. See Section

[Page 7]

ODSI Functional Specification                                 April 2000

6 for further details.

The only required parameters when requesting bandwidth are the

IP addresses and user group of the two endpoints. When

port/channel are not explicitly specified, any port associated

with the IP address and port/channel matching the requested

bandwidth characteristics will be connected.

2.2.  Bandwidth characteristics

The characteristics of the dynamic bandwidth channel connecting

two user devices can be specified by the following parameters:

o   Physical layer. The encoding of data in the channel

connecting the user device to the Optical network. Example

physical layers include SONET (path, section, line), SDH,

full-duplex ethernet, and unspecified (raw wavelength). The

"network services" Physical Layer Regenerator Circuit (PLR-

Circuit), STE Circuit (STE-Circuit) and LTE Circuit (LTE-

Circuit) from [Ref14] are included as physical layer

specifications.

o   Size. Examples include STS-1, OC-48, and 1 or 10 megabit

(ethernet). Specified in bits/second. Any size bandwidth may

be requested, although it is not required that all sizes be

supported by the optical network. When a size is requested

that the optical network does not support, it may return a

larger bandwidth connection, but never a smaller. However,

if the amount of bandwidth requested is not available, the

optical network will indicate the maximum amount of

bandwidth that can be obtained to the specified endpoint.

o   Priority. Whether the circuit can be preempted by other,

higher priority circuits (or vice-versa).

o   Protection. Whether the bandwidth should be protected

against failures, and if so, the speed at which the

protection will restore bandwidth service after a failure.

Restoration speed is specified in milliseconds.

o   Propagation delay. The user device can specify the maximum

amount of propagation delay to be tolerated on the circuit.

Specified in milliseconds.

o   Jitter. The user device can specify the maximum amount of

jitter to be tolerated on the circuit. Specified in

microseconds.

[Page 8]

ODSI Functional Specification                                 April 2000

o   Error rate. The user device can specify the maximum error

rate to be tolerated, expressed as an exponent of a bit

error rate.  For example, 10**-9 would be specified as 9.

o   Availability. The user device may ask for particular

availability guarantees, in terms of seconds down per year.

o   Diversity. The user device may request that the new circuit

be diverse (that is, share no common facilities) from an

existing set of ODSI-created circuits.

o   Vendor extensions. Vendors may create their own proprietary

bandwidth descriptions. One example might be more specific

protection modes, such as ring or linear SONET APS.

Mechanisms will be provided so that vendor extensions can be

expressed during service discovery (Section 3.1) and by ODSI

signaling (Section 3.3).

Some network operators may support a smaller set of options for

ODSI bandwidth requests. For example, a network operator may

decide to offer only OC-48 service, or limit availability to two

choices: 4 hours per year or 2 days per year.

Details concerning particular physical layers, for example how

ODSI encodes channel identifiers, or how ODSI protection

mechanisms interact with protection schemes already defined for

the physical layer, are defined in Appendices.

Encoding and bandwidth size may be left unspecified when implied

by endpoints of the bandwidth channel.

2.3.  Actions

There are three actions that you can request regarding

allocation of dynamic bandwidth. All actions are performed by a

signaling protocol, as outlined in Section 3.3.

o   Create (dial). Create a point-to-point bandwidth channel

between two user devices connected to the optical network.

The two endpoints of the channel and the characteristics of

the bandwidth must be specified, as described in Sections

2.1 and 2.2.

o   Destroy (hang-up). Destroy an existing bandwidth channel.

Either one of the endpoints can be specified.

o   Query (caller ID). Query the status of an existing bandwidth

channel. Either one of the endpoints is specified, and the

[Page 9]

ODSI Functional Specification                                 April 2000

status (established or not) and other endpoint of the

channel is returned.

o   Modify. Increase or decrease the amount of bandwidth

associated with actively connected circuit. The user device

may also request to modify other bandwidth parameters.

o   Non-destructive modify. Identical to the modify request,

except for the following provision. For the network to

successfully execute a non-destructive modify request, the

requested changes must be performed in a hitless fashion:

the existing bandwidth connection must continue to be

presented to the user endpoints without errors.

o   Directory lookup. Obtain a list of endpoints to which the

user device can establish circuits. Each endpoint is

identified by an IP address.  Only IP addresses registered

with the same user group as the device making the directory

lookup will be returned. In addition, the device can

restrict the lookup to only those endpoints supporting a

specified physical layer interface to the optical network.

Note the difference between a modify request and a destroy

followed by a create with new parameters. If the former fails,

the user device still has a connection. If the latter fails, the

user device has no connection.

2.4.  Unsupported services

The goal of ODSI is to provide an interface to the optical

network that can be quickly implemented and deployed. We have

chosen to limit the feature set accordingly, eliminating

features whose utility is not yet obvious, or which can be

derived from other features that are included by ODSI. In

particular, the following features are not supported.

o   Asymmetric bandwidth requests. The bandwidth requested of

the optical network is assumed to be the same in the

transmit and receive directions.  This is because the

control protocols used by the user devices attaching to the

optical network, and most of the applications running over

them, assume symmetric bandwidth. For example, standard IP

routing protocols do not work over asymmetric links.

o   Point-to-multipoint bandwidth channels.  Routers and ATM

switches will provide the multicast service on top of the

point-to-point bandwidth provided by the Optical network. We

are not going to try to specify the equivalent of a LAN

[Page 10]

ODSI Functional Specification                                 April 2000

emulation service at the Optical layer.

o   VPN services.  These too will be provided by the routers and

ATM switches connected to the Optical network. The IP

addresses which specify the endpoints of the dynamically

created bandwidth channels must be routable within the

optical networks control network. (Note: ODSI user groups

provide some VPN functions by restricting which endpoints

can be connected. However, unlike VPNs all ODSI user devices

connected to a single optical network must use a single

routable IP address space).

o   Connections based on time of day. These services can be

built upon the basic ODSI services outlined in Section 2, as

vendor value-added features.

o   NNI connections. These come in various degrees. ODSI does

not support the connection between two elements internal to

the optical network; instead, technology as outlined in

[Ref4] would be more appropriate.  However, ODSI could be

used at the connection between two Optical networks. ODSI

signaling would transit the boundary between two optical

networks unaltered. You would also need to exchange endpoint

identification. Since we are identifying endpoints by IP

addresses, this could be accomplished in a limited scope by

existing ODSI registration mechanisms, although a future

extension to an IP routing protocol (such as BGP) would be

better in the long term, as it would enable arbitrary inter-

connection topologies.

o   Diagnostic and test access. Examples include the ability to

trace the path of an ODSI-supplied circuit, and test access

to a circuit through a bridging capability. While not part

of the ODSI specification, this support may be supplied

through vendor extensions.

o   More specific protection modes. One example is the ability

to specify ring or linear APS protection for the created

bandwidth.  Note however that these modes could be supported

by an optical network through vendor extensions.

3.  Protocols

The protocols involved in ODSI perform three functions. First, they

allow a user device to detect that one of its ports is connected to

an optical network that is capable of dynamic bandwidth allocation;

we call this "service discovery". Second, they allow the user device

to give an address to the port, so that the port can be used as an

[Page 11]

ODSI Functional Specification                                 April 2000

endpoint in a dynamic bandwidth channel creation request; we call

this function "address registration". Lastly, there is a "signaling"

function, which the user device uses to dynamically connect its

ports to other ports connecting to the optical network.

ODSI is implemented using IP protocols, extending PPP and MPLS

signaling. ODSI protocol packets are transmitted over the IP control

network composed of the optical network's network devices, the user

devices and selected links interconnecting the two. These links are

classified as either in-band, which use part of the same physical

port that has been registered with ODSI for dynamic bandwidth

creation, or out-of-band, which use entirely separate physical

paths. SONET DCC overhead bytes were the in-band example mentioned

previously; an example of an out-of-band control connection would be

a 10-base-T ethernet hub connecting a user device's management

ethernet to a network device's management ethernet interface.

Service discovery and address registration are accomplished via a

PPP session running in an in-band channel associated with the port.

This in-band channel should be available even if user traffic is

present.  For example, the port may have been initially connected

via ODSI, or by other network management means, and then the user

device restarted. The user device would still like to know that ODSI

is available on the port, in case it is desirable to reconnect that

port to a different destination at some later time.  If an in-band

channel is not available, information that would have been derived

from service discovery and address registration must be configured

in the user devices and their directly attached network devices.

Signaling messages are sent over the IP control network, possibly

traveling multiple IP hops over both in-band and out-of-band control

links.

The three protocol functions are described in more detail in the

following subsections.

3.1.  Service discovery

When a physical port of a user device connects to an Optical

network, the device wishes to know whether it can use ODSI to

dynamically connect the port, through the optical network, to

another port on a remote device. This discovery is done using

the PPP protocol [Ref1] over the in-band channel; the

termination points for the PPP session being the user device and

the network device to which it directly connects.

The user and network devices should periodically test the PPP

connection for liveness, using PPP's Link Quality Monitoring

[Page 12]

ODSI Functional Specification                                 April 2000

Protocol [Ref14].  If the PPP connection fails, any address

registrations (Section 3.2) for the associated port are removed,

but any existing bandwidth connections to the port are

maintained.

The following pieces of information are exchanged over the

port's in-band control channel during service discovery.

o   The user device indicates the port's number, which is used

to specify that this port be used for dynamically created

bandwidth.

o   The user device indicates the port's user group, used to

restrict incoming connections to the port and to identify

the user device for accounting purposes. The user device may

need to provide a digital signature with the user group, so

that its membership can be verified (see Section 6).

o   The network device indicates that the port is available for

dynamic bandwidth creation.

o   The network device indicates its corresponding port number,

which can be alternatively specified during bandwidth

requests (see Appendix C).

o   The network device provides its IP address, which can be

used in combination with the network device's

port number as an alternative endpoint identification.

o   The network device provides the address of an "Optical

networking controller", to which signaling requests can be

addressed.

o   Any physical characteristics of the port which cannot be

dynamically determined are exchanged. For example, the user

device can indicate whether it is capable of splitting the

port into separate channels, and the network device can

indicate whether the optical network is capable of routing

these channels separately.

o   The network device tells the user device whether address

registrations and signaling requests must be authenticated

(see Section 6).

o   The network device indicates to the user device the typical

bandwidth connection setup time, in milliseconds. The user

device should treat this as advice as to whether it should

set up a connection, and when it should give up on a pending

[Page 13]

ODSI Functional Specification                                 April 2000

connection.

3.2.  Address registration

Address registration is a port-specific process, whereby the

user device associates one or more IP addresses to the port;

these IP addresses can then be used as endpoint identification

in dynamic bandwidth connection requests (see Section 2.1).  A

user group is associated with each registered address. Address

registration also uses PPP running over the port's in-band

control channel.

A user device can advertise the same IP address for multiple

ports. In this case, if a specific port is not specified in

signaling requests for the advertised address, the optical

network is allowed to pick any one of the associated ports.

The user device can also advertise multiple addresses for a

single port. This capability is useful when implementing a

simple NNI function (see Section 2.4). In this case, some (or

all) of the addresses are being advertised by proxy for other

(remote) user devices.

3.3.  Signaling

The signaling protocol is used to request connection of two

ports (or even subchannels of ports), disconnect a port, query

the connection status of a port, modify the bandwidth of any

existing connection or obtain a list of valid connection

endpoints.  Data elements carried by the signaling include

identification of the endpoints to be connected (Section 2.1),

and the bandwidth characteristics of the connection (Section

2.2).

Signaling is intrinsically third-party: neither of the

connection endpoints need belong to the requesting user device.

When the optical network receives the connection request, it

performs the following functions:

o   The optical network validates the connection request.

o   The network determines whether a path exists for the

connection that meets the requested bandwidth constraints.

o   The network informs the two endpoints of the pending

connection, and upon their acceptance sets up the bandwidth

and informs the requester of the result.

[Page 14]

ODSI Functional Specification                                 April 2000

Signaling requests are IP messages. They can be forwarded on any

IP data path between the requesting user device and one of the

network devices. This could be through an in-band control

channel. Or it could be through connections that were reserved

for control traffic - for example, a 10-base T ethernet

connecting the user device and the network device.

The identification of the endpoints may be only partially

specified. In this case, the optical network will choose

appropriate endpoints, and this choice will be conveyed to the

endpoint devices before the bandwidth is fully established.

Once a bandwidth connection has been established, it can be

identified in signaling messages by either a) a complete

specification of either endpoint, down to the channel number or

b) a combination of one endpoint's IP address and a connection

ID assigned by the signaling protocol.

The following points should be kept in mind concerning ODSI

signaling:

o   ODSI signaling is intrinsically third-party. Bandwidth can

be requested by a device other than the endpoints.

o   ODSI signaling messages need not follow the same path as the

bandwidth connection that will be established.

o   The endpoints should be able to influence the connection,

including refusing the connection, or specifying particular

ports.

o   The implementation of the optical network is unspecified; it

can be either distributed or centralized circuit placement

algorithms.

o   User and network devices are part of a single IP network for

control purposes.

o   Devices are not required to support the MIB. In particular,

SNMP sets cannot be used as a replacement for signaling.

o   Unlike MPLS, in ODSI there are no labels to set up. The

resulting bandwidth connection will look like a leased line

(or full duplex gigabit ethernet, etc.) to the user devices,

and will be space-division switched, as opposed to packet-

switched, by the optical network. (The endpoint or channel

number could be considered the equivalent of an MPLS label,

but does not appear anywhere in the data stream).

[Page 15]

ODSI Functional Specification                                 April 2000

4.  Example usage

This section presents a simple example of ODSI functionality.  Three

IP routers, A, B, and C are attached to an optical network through

three Optical switches SW1, SW2 and SW3, via OC-48 SONET links, as

pictured in Figure 1. The goal of ODSI is to allow Router A to

dynamically choose whether it is connected to Router B or C (or

neither). When connected, the connection will be essentially a

leased line between the routers.

Assume that Router A can support an in-band control channel on its

OC-48, but that B and C cannot. Router A can dynamically discover

that ODSI is available on its OC-48 link into the optical network,

can tell SW1 its IP address, and can learn SW1's address, which will

be the destination of any signaling requests. In contrast, these

items must be configured in Routers B and C and their attached

network devices SW2 and SW3. In addition, in order to participate in

signaling, an out-of-band control connection must exist between the

latter two routers and their network devices, pictured in the

diagram as 10baseT ethernet segments.

If Router A now wants a leased line to B, it sends an ODSI signaling

request to SW1 over the in-band channel, requesting a SONET OC-48

connection to Router B's IP address. The Optical network then

verifies that such a connection can be established to B, confirms

+********+

*          *

*        +---+  OC-48  +--+

*         |SW2|---------|RB|

*          +---+------+  +--+

*                  *   |   |

+--+     +---+     Optical     * +-----------+

|RA|-----|SW1|     Network      *   10baseT

+--+OC-48+---+                  *   10baseT

*                   * +-----------+

*                 *   |         |

*         +----------+  +--+

*        |SW3|---------|RC|

*       +---+  OC-48  +--+

*         *

+*******+

Figure 1: Simple ODSI example

[Page 16]

ODSI Functional Specification                                 April 2000

with B that this is OK (over the 10baseT), sets up the connection

and returns a success indication to A.

Two questions come up in this example. How does Router A know Router

B's IP address, and how does it know that it wants to connect to

Router B. Router A may have some a priori knowledge (such as an

operator configuring the information), but as we shall see in the

next section, a bootstrapping scheme using MPLS traffic engineering

may also supply the needed information.

5.  Traffic engineering example

Figure 2 shows a sample optical network of three optical switches,

interconnected by WDM links and providing leased line services for

four IP routers.

Assume that OC-48 leased lines have been provisioned between the

routers, using traditional means, producing a linear topology

amongst the routers as shown in Figure 3.  This allows IP

connectivity to be established between the routers, enabling them to

learn each others' IP addresses.

+--+--------+---+           +---+--------+--+

|RA|--------|SW1|-----------|SW2|--------|RD|

+--+ OC-48s +---+    WDM    +---+ OC-48s +--+

\             /

\           /

WDM \         /WDM

\  +---+  /

+-|SW3|-+

/+---+\

OC-48s/ /   \ \OC-48s

/ /     \ \

+--+     +--+

|RB|     |RC|

+--+     +--+

Figure 2: Simple Traffic Engineering example

+--+       +--+       +--+       +--+

|RA|-------|RB|-------|RC|-------|RD|

+--+ OC-48 +--+ OC-48 +--+ OC-48 +--+

Figure 3: Bootstrap router topology

[Page 17]

ODSI Functional Specification                                 April 2000

+--+       +--+

|RA|-------|RB|

+--+       +--+

|  \ ----/  |

|   /---\   |

+--+/     \+--+

|RC|-------|RD|

+--+       +--+

Figure 4: LSP overlay (STS-12c LSP size)

On top of this, a combination of MPLS and constraint-based routing

allows you to overlay a full mesh of Label Switched Paths (LSPs)

between routers for traffic engineering purposes, as shown in Figure

4. Assume at first that it is estimated that the traffic between

each pair of routers is an STS-12c (i.e., each LSP is sized at

STS-12c). Then the router topology in Figure 3 suffices. However, if

the estimate of the IP traffic between Routers A and D increases to

2 OC-12s, constraint-based routing will tell the routers that the

LSPs no longer fit.  Dialing up a new OC-48 link through ODSI

between A and D (or B and C) fixes the problem.

[Editors' note: The routers in this example could also bootstrap

themselves by using the ODSI "Directory lookup" to find the other

routers connecting to the Optical network.]

6.  Accounting/security

All ODSI protocol exchanges contain user group identifiers that can

used for both security and accounting. Bandwidth will only be

established between user devices that can be verified to belong to a

common user group. The requester of the bandwidth must also belong

to the same user group. Note however that a single device may belong

to multiple user groups, as may be desirable for a provisioning

server used by multiple user communities.

User group identifiers are administered by the operator of the

optical network. Verification of user group membership is optional,

at the discretion of the network operator. When user group

membership is to be authenticated, address registrations and

signaling requests carry digital signatures in order to prove

membership.

Admission control and accounting of bandwidth requests is

accomplished through extensions to the COPS protocol [Ref9], and is

again both optional and based on user group.

[Page 18]

ODSI Functional Specification                                 April 2000

7.  Network management

A Management Information Base will be created for ODSI, consisting

of the following tables. These tables are present in both the user

and network devices.

o   A port table, containing the user group identifier, port ID,

network device port ID, network device IP address, and a pointer

to a table for physical layer parameters (which may include

information concerning channelization, in-band control, and so

on).

o   Port address table. The list of addresses that have been

registered for a port, and their user groups.

o   Connection table. Information about current connections. Indexed

by the connection identifiers in Section 2.2.

o   Available endpoint table. The list of endpoints that are

available in a given user group.

[Page 19]

ODSI Functional Specification                                 April 2000

Appendix A. SONET-specific ODSI attributes

This Appendix provides ODSI information specific to the SONET

physical layer.

A.1 Channel numbering

The fundamental signal in SONET is the STS-1 (about 51 Mbps).

This signal consists of a transport overhead and a Synchronous

Payload Envelope (SPE). The SPE floats within its allotted space

within the SONET frame structure with the pointer bytes (H1, H2

and H3) in the Line Overhead of the SONET transport overhead

pointing to the beginning of the SPE.  An STS-N signal is formed

from a SONET STS-(N- 1) signal and an STS-1 signal via byte

interleaving.  The transport overhead structures are frame

aligned prior to interleaving but this is not required of the

SPEs, i.e., there is no special relationship between the payload

envelopes.  To transport signals in excess of about 50Mbps the

SPEs can be concatenated, i.e., glued together. In this case

their relationship with respect to each other is fixed in time

and hence this relieves, when possible, and end system of any

inverse multiplexing bonding processes.

The end points of SONET connections can be identified by the

"time slots" (position) that they occupy within the interleaved

frame structure. In the standard SONET case you must specify

which of the M STS-1 paths within an STS-N signal will be used

to transport the data (M <= N, and N = 3, 12, 48, 192,...). The

SPEs of these M STS-1s can be concatenated to form an STS-Mc.

The STS-Mc notation is really a short hand way of describing an

STS-M signal whose SPEs have been concatenated.

In BellCore GR-253 [Ref10] section 6.1.2 (requirement R6-3) two

conventions are given for identifying an STS-1 within an STS-N:

-   two-level "STS-3 #, STS-1 #"

-   A single-level "1 to N in order of appearance at the input

to the byte-interleaver"

For example STS-1 number 23 within an OC-48 can also be

represented by the tuple (8, 2). ODSI uses the single-level

numbering scheme to identify endpoints.

A second complication is in dealing with concatenated signals.

In Bellcore GR-253 section 5.1 the multiplexing procedures for

SONET are given. Constraints are imposed on the size of STS-Mc

signals, i.e., they must be a multiple of 3, and on their

[Page 20]

ODSI Functional Specification                                 April 2000

starting location and interleaving.  This has the following

advantages: (a) restriction to multiples of 3 helps with SDH

compatibility (there is no STS-1 equivalent signal in SDH, an

STM-1 is equivalent (essentially) to an STS-3c); (b) the

restriction to multiples of 3 reduces the number of connection

types; (c) the restriction on the placement and interleaving

could allow more compact representation of the "label"; The

major disadvantage of these restrictions are: (a) Limited

flexibility in bandwidth assignment (somewhat inhibits finer

grained traffic engineering). (b) The lack of flexibility in

starting time slots for STS-Mc signals and in their interleaving

(where the rest of the signal gets put in terms of STS-1 slot

numbers) leads to the requirement for re-grooming (due to

bandwidth fragmentation).

Due to these disadvantages ODSI will support "arbitrary"

concatenation, i.e., no restrictions on the size of an STS-Mc

(as long as M <= N) and no constraints on the STS-1 timeslots

used to convey it. However, the use of arbitrary concatenation

or "standard" concatenation will be negotiated as part of ODSI

service discovery process.

A.2 In-band control channels

SONET currently references three types of spans, Section, Line

and Path.  A section covers the span between two intelligent

repeaters (such as OEO regenerators) or between terminating

equipment and an intelligent repeater, or between terminating

equipment for which there are no intelligent repeaters.  A

"Line" covers the span between two pieces of terminating

equipment such as SONET Add Drop Multiplexers (ADMs) or Optical

Cross Connects (OXCs) supporting Optical to Electrical to

Optical Conversion (OEO).  A "Path" covers the end to end span

through a SONET network and terminates on both end points, for

example IP routers. This concept is illustrated in Figure 5.

Telcordia and ANSI have defined specific overhead bytes which

are to be terminated and processed at network elements

corresponding to these designations.

The frame structure of SONET allocates approximately 3% of the

framing format for maintenance purposes.  In particular, in each

810 byte STS-1 frame, 27 bytes are reserved for section and line

maintenance.  An additional 9 bytes are reserved for path

maintenance on a per concatenated payload basis.  See ITU-T

G.709 or Telcordia GR-253 for additional information on framing

formats.

[Page 21]

ODSI Functional Specification                                 April 2000

Router       OXC          Regen      Regen         OXC         Router

______      ______                                 ______      ______

|      |    |      |                               |      |    |      |

|      |    |      |        |\         |\          |      |    |      |

|      |    |      |        | \        | \         |      |    |      |

|      |____|      |________|  \_______|  \________|      |____|      |

|      |    |      |        |  /       |  /        |      |    |      |

|      |    |      |        | /        | /         |      |    |      |

|      |    |      |        |/         |/          |      |    |      |

|      |    |      |                               |      |    |      |

|      |    |      |                               |      |    |      |

------      ------                                 ------      ------

|<--------->|<---------->|<--------->|<------------>|<---------->|

|  Section  |   Section      Section     Section    |   Section  |

|           |                                       |            |

|           |<------------------------------------->|            |

|                             Line                               |

|                                                                |

|<-------------------------------------------------------------->|

Path

Figure 5: SONET span types

The Section, Line and Path Overhead bytes are labeled as

follows:

Name          Description

------------------------------------------------------------------

A1,A2         Section Framing bytes

B1            Section BIP-8 error monitoring for previous payload

B2            Line BIP-8 for each STS-1 individually (BIP is a

parity computation)

B3            Path BIP-8 over payload

C1            STS-1 Identifier (fixed value)

C2            Signal Label, indicates construction of STS Nc ATM

D1,D2,D3      Section network management channel 192 Kbps

(Section DCC)

D4->D12       Line network management channel, 576 Kbps

(Line DCC)

E1            Section orderwire, used for mux to mux handsets

E2            Line orderwire

F1            Section User defined

F2            User Channel

[Page 22]

ODSI Functional Specification                                 April 2000

Name          Description

------------------------------------------------------------------

G1            Path FEBE, RDI, Loss of Cell delineation for ATM

H1,H2,H3      Line structure pointer and pointer action

H4            Indicator for various SONET boundaries

J0            Section Trace-J0

J1            Path Trace, provides connectivity verification

K1,K2         Line automatic protection switching. K2 also

carries line AIS, line RDI, RDI removal

M0 or M1/Z2    Line Far end block error count (FEBE).

Reflects received errors back to source.

S1            Line Sync Status Indicates quality of clock source

Z0            Section Growth

Z1,Z2,Z3,Z4   Future growth

Z5            Tandem Connection Maintenance

Each overhead byte position utilizes 64 kbps bandwidth

SONET byte interleaves multiple STS-1s to achieve higher data

rates.  For SONET data streams greater than a single STS-1, the

Section, and Line overhead for the first STS-1 is processed, and

the overhead of subsequent STS-1s is predominantly unused except

for framing and parity..

Most commercial hardware framers support termination of Section

(D1,D2 and D3 - 192 Kbps) and Line (D4->D12 576 Kbps) HDLC

communications channels.  This implies that most existing router

and ADM interfaces are already able to communicate on these

channels.  The GR253 specified control channel between adjacent

SONET Network Elements is the section DCC.  To minimize

interference with existing SONET NE control, this specification

recommends the use of Line DCC as the default for the Optical

UNI control channel, unless administratively configured to use

alternative overhead bytes.

SONET overhead bytes can be used to provide an in-band control

channel for ODSI service discovery, address registration and

signaling.  The default in-band control channel for an ODSI

interface will occupy the SONET Line Overhead bytes labeled D4,

D5 through D12 of the first STS-1 of the SONET structure.  (576

Kbps).  The configured overhead bytes are then concatenated to

form a synchronous, bit-stuffed HDLC-framed stream, over which

the PPP protocol is then run.

It is recommended that both user devices and network devices

allow the configuration of an in-band control channel to

[Page 23]

ODSI Functional Specification                                 April 2000

comprise one or more alternative overhead bytes: D1, D2, D3, E1,

E2, F1, F2, Z1, Z2, Z3, Z4.

A.3 Detection of established connections

When the SONET port has not yet been connected via ODSI to a

remote endpoint, a SONET AIS pattern will be received on the

port. Circuit failures can also be detected via standard SONET

mechanisms.

A.4 Interaction with SONET protection schemes

Standard SONET protection schemes can be used between the user

devices and their attached network devices, providing local

protection of a port and its ODSI connection. For example, ports

A and B of a user device can be connected to a network device,

and standard 1:1 protection can be configured with port A

working and port B protect. Separate ODSI connections can then

be established on ports A and B. However, if A fails its ODSI

connection will be rerouted to B, replacing B's previous

connection. (This naturally gives port B's traffic lower

priority, which is in accordance with the "extra data" traffic

on rings and linear APS per Bellcore, ANSI and ITU

specifications.)

[Page 24]

ODSI Functional Specification                                 April 2000

Appendix B. Digital wrappers

This Appendix provides ODSI information specific to physical links

employing Digital wrappers. If the wrapped information is SONET,

most of the information in Appendix A applies. However, Digital

wrappers employ a different in-band control channel.

B.1 In-band control channels

The optical channel overhead (sometimes referred to as the user

data channel) can be used to provide an in-band control channel

for ODSI service discovery, address registration and signaling.

There are 40Mb/sec of overhead bytes available for this purpose,

which can be viewed as four 10Mb/sec channels.  Both user

devices and network devices should allow the configuration of

one of these channels as an in-band control channel, which

should be encoded using 10Mb full-duplex ethernet framing, over

which the PPP protocol is then run.

[Page 25]

ODSI Functional Specification                                 April 2000

Appendix C. User devices without IP addresses

All user devices which implement ODSI need IP addresses, so that

they can exchange ODSI protocol packets, which are encapsulated

within the IP protocol.  However, the network devices can

dynamically assign IP addresses to a user device, using the standard

IP Control Protocol [Ref2] of PPP, running over a port's in-band

control channel.

The user device would use this address as the source address in its

signaling packets, but it wouldn't want to use the address as the

endpoint of a bandwidth connection request since it's been

dynamically allocated from a temporary pool. Instead, the network

device's IP address and port number can be used as the endpoint of

connection requests.

[Page 26]

ODSI Functional Specification                                 April 2000

Appendix D. SONET ADMs

SONET Add/Drop multiplexers can be either user or network devices.

When they are user devices, they cannot request specific SONET

protection modes such as ring or Linear APS for their dynamic

bandwidth connections.  Instead, they are forced to build their own

protection using ODSI primitives as building blocks: allocating sets

of diverse connections to build their own ring and linear protection

schemes.

[Page 27]

ODSI Functional Specification                                 April 2000

References

[Ref1]  Simpson, W., "The Point-to-Point Protocol (PPP)", RFC 1661,

July 1994.

[Ref2]  McGregor, G., "The PPP Internet Protocol Control Protocol

(IPCP)", RFC 1332, May 1992.

[Ref3]  Rosen, E., A. Viswanathan and R. Callon, "Multiprotocol

Label Switching Architecture", work in progress.

[Ref4]  Awduche, D., Y. Rekhter, J. Drake and R. Coltun, "Multi-

Protocol Lambda Switching: Combining MPLS Traffic

Engineering Control With Optical Crossconnects", work in

progress, November 1999.

[Ref5]  Awduche, D., J. Malcolm, J. Agogbua, M. O'Dell and J.

McManus, "Requirements for Traffic Engineering Over MPLS",

RFC 2702, September 1999.

[Ref6]  Smit, H., T. Li, "IS-IS extensions for Traffic Engineering",

work in progress, May 1999.

[Ref7]  Katz, D. and D. Yeung, "Traffic Engineering Extensions to

OSPF", work in progress.

[Ref8]  Simpson, W., "PPP Challenge Handshake Authentication

Protocol (CHAP)", RFC 1994, August 1996.

[Ref9]  Boyle, J., R. Cohen, D. Durham, S. Herzog, R. Rajan and A.

Sastry, "The COPS (Common Open Policy Service) Protocol",

work in progress, November 6, 1999.

[Ref10] Bellcore Generic Requirements, GR-253-CORE, Synchronous

Optical Network (SONET) Transport Systems: Common Generic

Criteria, Issue 2, December 1995.

[Ref11] ODSI Coalition, "ODSI Service Discovery and Address

Registration", work in progress.

[Ref12] ODSI Coalition, "Definition of Managed Objects for ODSI

Management", work in progress.

[Ref13] ODSI Coalition, "Optical Domain Service Interconnect (ODSI)

Signaling Specification", work in progress.

[Ref14] Simpson, W., "PPP Link Quality Monitoring", RFC 1989, August

1996.

[Page 28]

ODSI Functional Specification                                 April 2000

[Ref15] Wright A. and R. Barry, "Proposed Architecture Working Group

Requirements Document (Project 1)", OIF00.007.3, January 31,

2000.

[Ref16] Bernstein, G. and T. Tedijanto, "UNI requirements for

topology discovery", OIF99.160, December 3, 1999.

[Page 29]

ODSI Functional Specification                                 April 2000

Editors' Addresses

Greg Bernstein

Ciena Corporation, Core Switching Division

10201 Bubb Road

Cupertino, CA 95014

Email: greg@ciena.com

Jeffrey Weiss

Ciena Access Systems Division

400 Nickerson Road

Marlborough Mass 01752

Phone: 508 486-3502

Email: jweiss@ciena.com

Rob Coltun

Siara Systems

300 Ferguson Drive

Mountain View, CA 94043

Phone: (650) 390-9030

EMail: rcoltun@siara.com

John T. Moy

Sycamore Networks

10 Elizabeth Drive

Chelmsford, MA 01824

Phone: (978) 367-2161

Email: jmoy@sycamorenet.com

Arnold N. Sodder

Tenor Networks, Inc.

100, Nagog Park, Acton, MA 01720

Phone: 978-264-4900 x110

Fax: 978-264-0671

Email: asodder@tenornetworks.com

K. Arvind

Tenor Networks, Inc.

100, Nagog Park, Acton, MA 01720

Phone: 978-264-4900 x124

Fax: 978-264-0671

Email: arvind@tenornetworks.com

[Page 30]