UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

| Commission d'études  Study Group  13 Comisión de Estudio  | Contribution tardive  Delayed Contribution  D. Contribución tardía  |
| --- | --- |

Q.19/13 Rapporteur’s Meeting, September 2000, Turin

| Questions: 19/13 | Texte disponible seulement en  Text available only in  E Texto disponible solamente en  |
| --- | --- |

| SOURCE*: | Lucent Technologies |
| --- | --- |
| Title: | OTN Overhead - Architectural principles |

______________

ABSTRACT

G.872 (February 1999) describes the Architecture of the OTN, and provides requirements for overhead to be carried by the OTN. A particular distinction is made between associated and non-associated overhead This contribution notes that  certain types of overhead are not actually associated with any particular OTN layer at all, and proposes that such overhead be grouped into requirements for a management overlay network and should be specified in a different recommendation. A candidate recommendation could be G.dcc, a new recommendation dealing with the general requirements for a technology independent management overlay network.

# Introduction

G.872 (February 1999) describes the Architecture of the OTN, and provides requirements for overhead to be carried by the OTN. A particular distinction is made between associated and non-associated overhead This contribution notes that  certain types of overhead are not actually associated with any particular OTN layer at all, and proposes that such overhead be grouped into requirements for a management overlay network and should be specified in a different recommendation. A candidate recommendation could be G.dcc, a new recommendation dealing with the general requirements for a technology independent management overlay network.

# Discussion

## Overhead types

G.872 describes overhead as being assigned to one of the OTN layers, and makes the distinction between associated overhead (which must be carried in the Optical Channel frame or in fibre), and non associated overhead, which may be carried by any means as long as communication takes place between the correct computational entities. Earlier contributions discussed the need for digital means for achieving the association required to meet the requirements for Performance Monitoring and Trace accuracy, and made the point that the OTN will evolve by reducing the need for regeneration as Optical Interface reach extends.  A contribution "Maintenance of the OTN, a reminder", (Lucent Technologies) to this meeting discusses Maintenance Philosophy for the OTN, in particular it recommends separating the operations having to do with fault location from those having to do with protection or restoration activities, so that actions can be taken in appropriate time scales.

In addition to the overhead for performance measurements and for connection identification (Trace), G.872 defines several overheads for general purpose communications and G.709 provides allocations for that overhead in the OCh frame. It must be noted that this latter overhead has nothing to do with the operation of the OTN layers as the communications being carried are actually associated with equipment, or even equipment locations.  Therefore it seems incorrect to assign these general overhead functions to any of the OTN layers. These functions should rather be part of a management overlay network.

## Management Overlay Network

So where should such functions be assigned given that the general overhead functions associated with the management overlay network need to be available at equipment or equipment sites? The only "layer" which is certain to interconnect all equipment is the fibre layer, so such overhead can only be reasonably assigned to the fibre layer. G.872 discusses the Optical Supervisory Channel (OSC) which was envisaged as a bearer sharing the same fibre as the OTS and OMS and carrying the overhead associated with those layers. The OSC can also be used as the bearer carrying the general overhead functions of the management overlay network.

Note that COM13-R68, the Report of the Kyoto meeting, contains a liaison from SG13 to SG15 in Annex 19. This liaison summarises the discussion of D.999 and D.1001 and discusses the transport of management information through the domain of an intervening operator. This contribution is not suggesting that this liaison is incorrect as this contribution is discussing the interconnection of domains, not the provision of connectivity through an intervening domain.

Considering this overlay network in more detail, it seems very likely that not all fibres need to carry the management overlay network. There is a strong similarity between the overlay network and the signalling network required for ASON, and G.ason discusses the need to be able to dimension that network so that availability and performance can be dimensioned independently of the transport network. The strong similarity, including the possibility that ASON signalling might perhaps be just one application of the management overlay network, leads to the same conclusion - the management network topology, performance and availability should be independent of the transport network.

There are also several valid diametrically opposed views of overlay capacity being discussed in Q11/15, which also implies the need to independently dimension the management overlay network and the overlay capacity provided by the transport network.

In order to independently dimension the overlay network, we must consider what is the shared resource, and we must conclude that overlay transport capacity must be provided between equipment of a single vendor, and that means must be provided to reach all the equipment at a particular site from one or more overlay link connections terminating at that site. Such means would most likely be a packet switch (router). In most cases, this router would provide the connectivity between equipment collocated at that site, and in this case there is no need for the overlay link connections to cross the IrDI (see Figure 1).

What must be noted is that such an overlay network is not one of the OTN layers. In fact such a network would have nothing specific to do with the OTN, and so it seems inappropriate to define overhead for this purpose in G.872. What must also be noted is that the connections between equipment at a particular site never carry the overlay network link connection, which is always terminated at a switch?

Figure 1: Overlay DCN Link Connection Needs For IaDI and IrDI

## A Standard Solution?

The problem we are faced with is to provide a solution that allows for standard interfaces to overlay network capacity, while allowing performance and availability to be operator policy decisions. We cannot standardise a physical signal (rate, format etc) on the fibre carrying the IaDI because 1) the IaDI is not subject to standardisation and 2) such a rate and format would not allow operator policy to determine the capacity of the link connection. (Note that the emphasis here is on physical standards; it is quite likely that logical information flows will be standardised.)

What is needed is a means to decouple the standard interface rate and format from the capacity provided. The only technology that seems to offer this possibility is to define a packet interface to the overlay network link connections, as the bit rate and the packet rate are now independent decisions (up to the limits provided by the bit rate).

As this overlay network is independent of the OTN layers, there seems to be no good reason to burden G.872 with describing its functions which removes overlay network concerns from G.709.

# Conclusion

This contribution demonstrates that general overhead functions not associated with the operation of the OTN layers cannot be usefully assigned to the OCh layer, as those functions cannot be reliably accessed at the necessary points in the OTN. It proposes that such functions be provided by a general overlay network which has nothing to do with OTN layering, and can be better specified in a different recommendation. Finally the discussion suggests that the only technology for this overlay network that allows for the capacity of link connections to be a matter of operator policy and allows for those connections to be provided over interfaces that are not subject to standardisation is a packet technology.

# Proposal

G.872 should remove all layer requirements for overhead that is not intrinsic to the operation of that layer. Those requirements can be used to start a new recommendation for a more general approach to providing management overlay capacity. A suitable new recommendation could be G.dcc.

# References

- G.872 Architecture of the OTN, ITU-T, February 1999

- COM13-R68, Report of the Kyoto Meeting, February 2000

- Maintenance of the OTN, a reminder, WD ... this meeting

- First Draft of Rec G.ason, “Architecture for the Automatic Switched Optical Network”  SG13, Kyoto, 2000