UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

13	WD Siemens-02

Turin, 13 – 15 September  2000

Question(s):	Q.19

SOURCE*:	Siemens AG, Germany

TITLE:	OIF Contribution

___________________

# INTRODUCTION

At the last OIF meeting it was agreed to align the OIT UNI terminology with ITU terminology. Attached is the document I did sent out on the OIF mailing lists on this issue. It represents our view of the UNI functional architecture concerning the connected signal. Note that it is not limited to "optical signals".

Contribution Number: oif2000.xxx.0 draft

Working Group:	Architecture, OAM&P, Signaling

TITLE:     UIT - Secteur de la normalisation des télécommunications

SOURCE:

| Juergen Heiles Siemens AG Hofmannstr. 51 81379 Munich, Germany Phone: +49 89 722 48664 e-mail: juergen.heiles@icn.siemens.de |  |  |
| --- | --- | --- |

DATE:	September 1st,  2000

Document Status:  Draft, for discussion

Project Name: UNI 1.0

Project Number:

Notice: This Technical Document has been created by the Optical Internetworking Forum (OIF).  This document is offered to the OIF Membership solely as a basis for agreement and is not a binding proposal on the companies listed as resources above. The OIF reserves the rights to at any time to add, amend, or withdraw statements contained herein. Nothing in this document is in any way binding on the OIF or any of its members.

The user's attention is called to the possibility that implementation of the OIF implementation agreement contained herein may require the use of inventions covered by the patent rights held by third parties. By publication of this OIF implementation agreement, the OIF makes no representation or warranty whatsoever, whether expressed or implied, that implementation of the specification will not infringe any third party rights, nor does the OIF make any representation or warranty whatsoever, whether expressed or implied, with respect to any claim that has been or may be asserted by any third party, the validity of any patent rights related to any such claim, or the extent to which a license to use any such rights may or may not be available or the terms hereof.

For additional information contact:
The Optical Internetworking Forum, 39355 California Street,
Suite 307, Fremont, CA 94538
510-608-5990 phone  info@oiforum.com

© 2000 Optical Internetworking Forum

# Introduction

- At the last OIF meeting it was decided to align the UNI1.0 terminology with the ITU functional architecture.

- This document provides a short overview on the functional architecture/modeling, applies the functional architecture to the OIF UNI and proposes specific terms and terminology for the UNI based on the functional architecture.

# Introduction to functional architecture/modeling

- As transport networks can be large complex networks with various components an appropriate network model with well defined functional entities is essential for its design and management. In order to simplify the description ITU-T has developed a transport network model, based on the concepts of layering and partitioning that allows a high degree of recursiveness. This model is applied to the various transport network technologies and is implementation independent. ITU-T G.805 " Generic Functional Architecture of Transport Networks" defines the generic model approach. The functional architectures for specific transport network technologies are defined in G.803 for SDH, G.872 for the OTN and I.326 for ATM. This functional architecture has been proven very useful over the years for the description and modeling of networks.

- Functional modeling decomposes the network both vertically and horizontally.

## Layer Networks

- Vertically the network is decomposed into layer networks (see Figure 1). A layer network represents the complete set of identical access points (AP) that may be associated for the purpose of transferring information The information transfer is specific for its transport capability (e.g. bandwidth) and end-to-end supervision. The end-to-end supervision (e.g. connectivity, continuity, signal quality) is performed by the trail termination functions, which are associated with the APs. A trail termination source inserts for example specific supervision overhead like trace and parity and the trail termination sink evaluates this overhead. A separate, logically distinct layer network exists for each trail termination type. The combination of the transport capability offered at the AP and the end-to-end supervision of the trail termination is specific for a layer network and constitutes its characteristic information CI. The CI is the information that passes the termination connection points (TCP) and Connection Points (CP) of the layer network.

- Examples for layer networks are SDH VC-4s, SDH Multiplex Sections and ATM VPs. The SDH VC-4 layer network for example offers a transport capacity of 149,76 kbit/s (= 260 columns by 9 rows) with the end-to-end supervision capabilities provided by the VC-4 POH (e.g. J1 trace for connectivity supervision, B3 BIP-8 for signal quality supervision). At the VC-4 AP the client information adapted to the specific needs for the transport via the VC-4 layer network (e.g. 140 Mbit/s PDH signal mapped into the VC-4 payload area using bit stuffing, lower order VC-11/12/3 mapped into the VC-4 payload area using point processing) plus payload specific overhead (e.g. signal label C2, multi-frame counter H4) is available. At the VC-4 TCP the whole VC-4 including the supervision overhead (J1, B3, ...) is available.

Figure 1 - Layer Networks

## Client/server relationship

- Between layer networks a client/server relationship exists (see Figure 1). A server layer network provides transparent transport for a client layer network. Transparent transport in this case means that the client CI is not changed between input and output other than that resulting from degradation in the transfer process. Adaptation functions, located between layer networks adapt the client layer network characteristic information into a form suitable for transport over the server layer network.

- The client/server relationship can be one-to-one, many-to-one and one-to many as shown in Figure 2.

- In a one-to-one relationship a server layer trail supports one client layer link connection. An example is the adaptation between SDH RS and MS layer.

- The many-to-one relationship represents multiplexing as a server layer trail supports several client layer link connections. The link connections need not to be of the same layer network. An example is the adaptation between SDH higher order and lower order VCs.

- The one-to-many relationship represents inverse-multiplexing/virtual concatenation as a client layer link connection is supported by several server layer trails in parallel. The trails need not to be of the same layer network.

Figure 2 – Client/server relationship

## Topological components

In addition to the layer network several other topological components are defined (see Figure 3).

### Subnetwork

- A subnetwork exists within a single layer network. It is defined by a set of CPs/TCPs that might be connected for the transfer of CI between them. Subnetworks might be defined for administrative or management reasons.

- Subnetworks may be partitioned into smaller subnetworks interconnected by links.

### Link

- A link consists of a subset of the ports at the edge of one subnetwork or access group which are associated with a corresponding subset of the ports at the edge of another subnetwork or access group for the purpose of transferring characteristic information. The link represents the topological relationship and available transport capacity between a pair of subnetworks, or a subnetwork and an access group or a pair of access groups. Multiple links may exist between any given subnetwork and access group or pair of subnetworks or access groups. While links are established and maintained at the time scale of the server layer network, they are not limited to being provided by a server trail and can be also provided by client layer network connections.

### Access group

- An access group is a group of co-located trail termination functions that are connected to the same subnetwork or link.

Figure 3 – Topological components

## Transport Entities

The transport entities (see Figure 4) provide transparent information transfer between layer network reference points (e.g. AP, TCP, and CP). There is no information change between input and output other than that resulting from degradation in the transfer process.

The trail represents the end-to-end transport through a layer network, including integrity supervision (e.g. a VC-4 connection including the end-to-end supervision for connectivity, signal quality, ...). The trail is defined between connected APs of a layer network.

The Network Connection NC represents the end-to-end connection between trail termination functions (e.g. a VC-4 connection without the end-to-end supervision). The NC is defined between connected TCPs of a layer network.

The SubNetwork Connection SNC represents the connection between ports of a subnetwork (CPs or TCPs). The smallest, indivisible SNC is the matrix connection (cross-connect). In general subnetwork connections are constructed from a concatenation of subnetwork connections and link connections (i.e. recursive).

A link connection LC is capable of transferring information transparently across a link. It represents the fixed relation between the ends of the link A link connection represents a pair of adaptation functions and a trail in the server layer network. It is delimited by CPs or TCPs.

Figure 4 - Transport Entities

## (Tandem) Connection Monitoring

While the trail termination monitors the network connection to establish the trail, it is sometimes required to perform supervision/monitoring of subnetwork connections, link connections or an arbitrary series of contiguous “link connections” and/or “subnetwork connections” (= tandem connection). Examples are:

- monitoring of a carrier subnetworks;

- monitoring of working and protection subnetwork connections for protection switching criterias.

Several tandem connection monitoring techniques are available.

Figure 5 – Tandem Connection Monitoring

Inherent Monitoring (Figure 5 a) makes use of the supervision information inherent available form the server layer trails. If a server layer fails (detected for example in the server layer trail termination), the information is made available for the link connection. This information is forwarded over all following link connections. The information has to be monitored at the ingress (A) and egress (B) in order to evaluate the state of the tandem connection. As a disadvantage monitoring is limited to the supervision capabilities of the server layer network. Misconnections on the layer itself for example are not detected.

Non-intrusive Monitoring (Figure 5 b) monitors the CI of the connection directly, using the same criterias (overhead) as the trail termination. A non-intrusive monitor provides information about the whole connection from the trail termination to the monitor point. In order to get information on the specific tandem connection, the information of non-intrusive monitors at the ingress (A) and egress (B) of the tandem connection has to be combined. As a disadvantage information form the trail terminations (e.g. trace) is required at the non-intrusive monitors.

Intrusive Monitoring (Figure 5 c) can only be used if the tandem connection is not in service, as it open the connection and inserts a test signal.

Sublayer Monitoring (Figure 5 b) is the most flexible technique if supported by the layer network. Some part of the original trails capacity (overhead) is used to establish a sublayer for supervision of the tandem connection. At the ingress (A) this information is inserted at the sublayer trail termination source. At the egress (B) this information is supervised at the sublayer trail termination sink. Sublayer monitoring can be nested if enough free capacity is available in the original trail.

SDH provides one level of sublayer monitoring (called tandem connection monitoring TCM) for all VC layers.

The Optical Channel Data Unit ODU of the OTN supports up to 6 nested levels of TCM.

## Example

Figure 6 shows an example for an SDH network connecting two IP routers. Router A has a STM-16 interface with a single contiguous concatenated VC-4-16c. Router B has a STM-64 interface with 4 concatenated VC-4-16c. Only the SDH layers of the routers are shown as they are of interest for the transport. The VC4-16c Layer Network is relevant for the interconnection between the two routers. It is the server layer network for the IP signals, that are mapped into the VC-4-16c using HDLC/PPP (POS) as intermediate layers. A VC-4-16c trail provides the transparent transport for the HDLC payload. According to the SDH structure the VC-4-16c is transported via Multiplex Section (MS16), Regenerator Section (RS16) and Optical Section (OS16). The OS16 is the bottom layer of the STM-16 interface and represents the optical signal. The SDH higher order VC cross-connect terminates all these server layers in order to perform cross-connection of the VC-4-16c (VC-4-16c CPs). The VC-4-16c is transported together with 3 other VC-4-16c via the MS64, RS64 and OS64 layer network of a STM-64 signal.  The SDH STM-64 regenerator terminates only the OS64 and RS64 layer and pass-through the MS64 layer transparently. Router B terminates all SDH trails including the VC-4-16c for access to the HDLC /PP mapped IP payload.

Figure 6 – Example SDH network connecting two IP Routers

# Functional architecture of the UNI

- The UNI is the interface between the networks of users and carriers, between the user edge device and the network edge device shown in Figure 7. Note the term user edge device is used to identify the network element at the user side of the UNI and the term network edge device is used to identify the network element at the carrier side of the UNI.

Figure 7 – UNI

- The user and carrier networks can be decomposed into layer networks as shown in Figure 8. The specific layer networks depend on the transport technology used in the user and carrier network.

- At the UNI we have to distinguish between the signaling and the transport signal itself that is connected. Both are part of the UNI. As G.805 defines transport networks and not the signaling, this document concentrates on the transport signal part of the UNI. Note that different physical connections can be used between the user and network edge device for the transport signal and signaling.

- In the layer model of G.805 the transport signal represents a specific characteristic information CI. At the UNI the user requests to connect this characteristic information CI of a certain layer network transparently through the carrier network to another user edge device. In the example of Figure 8 it is the CI of layer network C. This layer network C spans therefore from the user network via the UNI over the carrier network into the other user network. The carrier network and both UNIs have to support transparent transport for the requested CI. On the UNI request a subnetwork connection for the specific layer network is established in the carrier network.

- The following behavior applies to the CI at UNIs, client and carrier networks:

- The requested CI can be of any layer network passing through the UNI. It needs not to be the bottom layer network. However the carrier network and both UNIs have to support transparent transport for the requested CI.
(e.g. at a STM-16 UNI a the connection request could be for the whole STM-16 signal including all SOH (i.e. RS16 layer network CI), for a STM-16 including the MSOH (i.e. MS16 layer network CI) or for individual VCs within the STM-16 signal (i.e. VC-4/3/2/12/11 layer network CI))

- Both UNI interfaces can have different signal structures as long as the requested CI is part of it.
(e.g. a VC-4-16c  can be requested at a STM-16 interface for one UNI and a STM-64 interface for the other UNI)

- The carrier can terminate all layer networks of the UNI that are below the requested CI.
(e.g. if a VC-4 transport is requested, the OS, RS and MS layer can be terminated)

- The carrier is free to choose appropriate server layer networks for the CI as long as they provide transparent transport for the CI. The server layer networks used in the carrier network are independent of the server layer networks used at the UNI and in the user network. The carriers choice depends on existing infrastructure, economical and technical reasons and may change over time. The only requirement is to support transparent transport of the requested CI. It is therefore also 
(e.g. a request for a RS16 connection can be supported in the carrier network with a single STM-16 signal on a fiber, with WDM systems with STM-16 or G.709 ODU1 (digital wrapper) channels or with TDM systems that combine several STM-16 transparently (proprietary SDH or ODU TDM)).

- The trail termination for the requested CI has not to be located in the user edge device. It can be located anywhere in the user network or even in a different network. The UNI sets up a connection; it doesn't care about the trail.
(e.g. if a VC-4-16c connection is requested and the user edge device is a router, the trail termination is located in the router. If the user edge device is a SDH cross-connect the trail termination might be located at a downstream router)

- Note, that although it is not required to have access to the specific layer network (via CPs) in the user edge device and the network edge device, it will normally be the case as the carrier wants to monitor the service offered to the user and the user wants to monitor the service provided by the carrier using a tandem connection monitoring technique.

- As the UNI itself is a point-to-point connection between the user and network edge device the physical interfaces (bottom layer network) on both sides need to be compatible. However the processing of all other layers (on top of the physical layer) could be different. A user edge device that terminates the OS16, RS16, MS16 and VC-4-16c layer networks (e.g. IP router) can be connected to a network edge device that terminates the OS16 layer only (e.g. transponder).

- Figure 9 shows the layer network view for the example in Figure 8. The layer network C, for which the UNI connection was requested spans over the carrier network and the user networks on both sides of the UNIs. The carrier network represents a subnetwork. The user networks will also have subnetworks if the layer network is not terminated in the user edge device.

Figure 8 – UNI functional model; example

Figure 9 – UNI layer network view; example

# Terminology for the UNI (Motions)

## Client vs. User

The term user instead of client shall be used in the UNI definition as the term user is part of the name UNI "user network interface" itself. In addition it avoids confusion with the ITU terminology client. The user network (could be a single network element) makes use of the services offered via the UNI.

## Ligthpath vs. Connection/Link

- As discussed above the carrier is free to choose the server layers for a specific connection request. These must not be necessarily pure optical. Depending on the requested CI the carrier has the choice between direct fiber connections and/or multiplexing using WDM or TDM techniques. The term lightpath seems therefor not appropriate. Furthermore it is not true that the Optical Network always starts in the network edge device. In the future with non-SONET/SDH UNIs the Optical Network can already start in the user network. Relating the UNI to a specific transport technology seems therefore too restrictive. According to ITU terminology a connection setup is requested via the UNI, not more and not less. In the carrier network a UNI-UNI subnetwork connection is established on this request. The user edge device sees a UNI-UNI link to the user edge device on the other side.

## Termination points vs. Connection point

- According to the ITU terminology a connection is located between connection points CP and between connection points CP and termination connection points TCP. These terms shall be used instead of termination point.

- Note that we have to distinguish here between the transport signal that is connected via the UNI and the UNI signaling. The above applies to the transport signal and not to the signaling. UNI signaling starts always at the user edge device in case of the overlay model, however the signal that is connected can come from somewhere else.

Consider a SDH cross-connect or ADM as user edge device. If a VC-4 connection is requested via the UNI, the VC-4 itself can start somewhere else in the user network, for example at a voice switch. On the other hand if the open model is used for signaling, a VC-4 still starts at the user edge device if it is an IP router, while the connection request may come from an IP router somewhere else.

## Transparency

ITU defines transparent transport as,

- the information (e.g. CI) is not changed between input and output of a transport entity (e.g. connection, trail) other than that resulting from degradation in the transfer process (e.g. bit errors, attenuation).

A connection is therefore always transparent for its specific CI. The CI defines/restricts the signal format that is transported:

- A connection for a VC-4-16c CI supports transport of the VC-4-16c frame. The payload of the VC-4-16c (e.g. ATM, Ethernet, PPP/HDLC) doesn't matter.

- A connection for a MS16 CI supports transport of the STM-16 frame, except for the RSOH. The payload of the STM-16 (e.g. 16xVC-4, 4xVC-4-4c, VC-4-16c or any mixture) doesn't matter

- A connection for a RS16 CI supports transport of the full STM-16 frame. The payload of the STM-16 (e.g. 16xVC-4, 4xVC-4-4c, VC-4-16c or any mixture) doesn't matter

- A connection for a CBR2.48832G (constant bit rate 2.48832 Gbit/s) supports transport of a STM-16 or any other digital signal with this line bit rate.

- A connection for CBR1-2.5G supports transport of any digital signal within the line bit rate range of 1 to 2.5 Gbit/s (e.g. STM-16, 1GbE).

The term transparent shall be used in the ITU terminology.

## Connection attributes

### Connection type

- To identify the type of connection the CI has to be specified. The CI name clearly identifies the signal format and. It is not necessary to subdivide the type into framing, bit rate and transparency.

- Furthermore ITU and T1X1 have tried over the last years to align SDH and SONET such that interworking between both is possible. Note that SDH also defines a multiplex hierarchy identical to the SONET multiplex hierarchy (e.g. higher order VC-3 and STS-1-SPE).

- For SDH/SONET signals the following CI names shall be used:

| - SDH name | - SONET name | - transported signal |
| --- | --- | --- |
| - RS64 | - STS-192 Section | - STM-64 (STS-192) signal without termination of any OH. |
| - RS16 | - STS-48 Section | - STM-16 (STS-48) signal without termination of any OH. |
| - MS64 | - STS-192 Line | - STM-64 (STS-192); termination of RSOH (section OH) possible. |
| - MS16 | - STS-48 Line | - STM-16 (STS-48); termination of RSOH (section OH) possible. |
| - VC-4-64c | - STS-192c-SPE | - VC-4-64c (STS-192c-SPE); termination of RSOH (section OH), MSOH (line OH) and VC-4-64c TCM OH possible. |
| - VC-4-16c | - STS-48c-SPE | - VC-4-16c (STS-48c-SPE); termination of RSOH (section OH), MSOH (line OH) and VC-4-16c TCM OH possible. |
| - VC-4-4c | - STS-12c-SPE | - VC-4-4c (STS-12c-SPE); termination of RSOH (section OH), MSOH (line OH) and VC-4-4c TCM OH possible. |
| - VC-4 | - STS-3c-SPE | - VC-4 (STS-3c-SPE); termination of RSOH (section OH), MSOH (line OH) and VC-4 TCM OH possible. |
| - VC-3 | - STS-1-SPE | - VC-3 (STS-1-SPE); termination of RSOH (section OH), MSOH (line OH) and VC-3 TCM OH possible. - Note: In SDH it could be a higher order or lower order VC-3, this is identified by the sub-addressing scheme. In case of a lower order VC-3 the higher order VC-4 OH can be terminated. |
| - VC-2 | - VT6-SPE | - VC-2 (VT6-SPE); termination of RSOH (section OH), MSOH (line OH), higher order VC-3/4 (STS-1-SPE) OH and VC-2 TCM OH possible. |
| - - | - VT3-SPE | - VT3-SPE; termination of section OH, line OH, higher order STS-1-SPE OH and VC3-SPE TCM OH possible. |
| - VC-12 | - VT2-SPE | - VC-12 (VT2-SPE); termination of RSOH (section OH), MSOH (line OH), higher order VC-3/4 (STS-1-SPE) OH and VC-12 TCM OH possible. |
| - VC-11 | - VT1.5-SPE | - VC-11 (VT1.5-SPE); termination of RSOH (section OH), MSOH (line OH), higher order VC-3/4 (STS-1-SPE) OH and VC-11 TCM OH possible. |

- Note that the same code shall be used for SDH and SONET.

### Addressing

- Addressing of network elements and ports is independent of the signal structure, but the addressing of a specific CI within the UNI signal depends on the UNI interface signal format and multiplex structure. If a one-to-one relationship exists between the CI and the bottom/physical layer network of the UNI no further sub-addressing is required. In case of a many-to-one relationship (multiplexing) sub-addressing is required. Note that several levels of multiplexing could be supported at the UNI. The UNI addressing scheme should therefore be flexible in order to support various signal types and multiplex hierarchies in the future.

- For SDH and SONET a common sub-addressing format shall be used based on the ITU-T G.707 addressing scheme.

#### Addressing of higher order VC-4/VC-3 (STS-3c/1-SPE) in a STM-N (STS-3*N) signal

The addressing shall use a 3-figure address scheme (K,M,L) based on the TU/TUG structure.

Figure 10 – Sub-addressing of VC-4/3 in a STM-N signal

A:	identifies the VC-3 within the AUG-1/STM-1 (STS3)
number range: 1 to 3, 0 for VC-4 (STS3c-SPE)

B:	identifies the AUG-1 within the AUG-4/STM-4 (STS-12)
number range: 1 to 4, 0 in case of VC-4-4c (STS-12c-SPE)

C:	identifies the AUG-4 within the AUG-16/STM-16 (STS-48)
number range: 1 to 4, 0 in case of VC-4-16c (STS-48c-SPE)

D:	identifies the AUG-16 within the AUG-64/STM-64 (STS-192)
number range: 1 to 4, 0 in case of VC-4-64c (STS-192c-SPE)

E:	identifies the AUG-64 within the STM-256 (STS-768)
number range: 1 to 4, 0 in case of VC-4-256c (STS-768c-SPE)

#### Addressing of lower order VC-3/2/12/11 in a VC-4

The addressing shall use a 3-figure address scheme (K,M,L) based on the TU/TUG structure.

Figure 11 – Sub-addressing of VC-3/2/12 in a higher order VC-4

K:	identifies the TUG-3 within the VC-4
number range: 1 to 3

L:	identifies the TUG-2 within the TUG-3
number range: 1 to 7, 0 in case of TU/VC-3

M:	identifies the TU-12/11 within the TUG-2
number range: 1 to 4 for TU/VC-11, 1 to 3 for TU/VC-12, 0 in case of TU/VC-2 and TU/VC-3

#### Addressing of lower order VC-2/12/11 (VT6/3/2/1.5-SPE) in a higher order VC-3 (STS-1-SPE)

The addressing shall use a 2-figure address scheme (M,L) based on the TU/TUG structure.

Figure 12 – Sub-addressing of VC-2/12/11 & VT3-SPE in a higher order VC-3

L:	identifies the TUG-2 (VT group) within the VC-3 (STS-1-SPE)
number range: 1 to 7

M:	identifies the TU-12/11 (VT3/2/1.5) within the TUG-2 (VT group)
number range: 1 to 4 for TU/VC-11 (VT1.5), 1 to 3 for TU/VC-12 (VT2), 1 to 2 for VT3, 0 in case of TU/VC-2 (VT6)

# References

ITU-T G.707 "Network node interface for the Synchronous Digital Hierarchy (SDH)"

ITU-T G.805  "Generic functional architecture of transport networks"

ITU-T G.803 "Architecture of transport networks based on the Synchronous Digital Hierarchy (SDH)"

ITU-T G.872 "Architecture of the Optical Transport Network (OTN)"

ITU-T I.326 "Functional architecture of transport networks based on ATM"