UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

13	  D.xxx

Turin, 13-15th September 2000

E

Question(s):	19

SOURCE*:	BT

TITLE:	The Optical Transport Network Architecture

(The circle that cannot be squared)

___________________

Introduction

This contribution discusses the architecture of the optical transport network and highlights its current limitations. This is in part due to very different and competing requirements from different operating agencies coupled with rapid advances in technology and an ever changing landscape with regard to the point at which one technology becomes more cost effective than another.

The contribution details a number of issues that need to be resolved within the architecture.

Unacknowledged Assumptions

In terms of the network architecture there are very few assumptions that are explicitly written down. It appears that there are three underlying and unacknowledged assumptions that are shaping the development of the OTN

- Efficient use of optical bandwidth

- Networking

- Support for multiple bit rate tributaries

The original design of G.872 was predicated on a) and b) with a simple flat multiplexing hierarchy. There is no limitation on the mix of bit rates carried on individual wavelengths. This was the design based on the view of the technology three years ago. The architecture did not however take into account c).

A number of carriers, particularly long haul carriers in the US have recently laid more emphasis on a) and c). In doing so b) becomes almost impossible. This is particularly the case with PDH type multiplexing schemes – frames within frames.

Consider the case of a number of WDM line systems connected to an all-optical switch based for example on MEMs technology. In this architecture the line systems are terminated with transponders. However in order to network between any input and any output with fast provisioning all transponders need to have the same bit rate and protocol. Hence part of the drive toward optical channel multiplexing (there are other factors as well such as the number of ports on optical cross-connects versus the number of channels that can be supported on line systems. One solution is to up the bit rate and reduce the number of channels). However, this just moves the problem. Consider wanting to access a 2.5G frame inside a 40G frame and every transponder on the line system being at the higher rate. The only solution is to drop the higher rate channel and then demultiplex everything down to 2.5G – especially as there is no byte visibility. This is a return to PDH and is not networking. In the absence of electronics a different scenario can be envisaged and reflects what is likely to be more common in metropolitan networks.

Finally for some of us b) and c) are the most important.

In conclusion the existing optical transport architecture allows us to have any two but not all three. But we require a solution that meets all three needs.

The Fourth Horseman

There is a further requirement that needs to be factored in, namely the ability to support inverse multiplexing. Whilst router manufacturers are beginning to examine the possibility of 40G concatenated interfaces there is, nevertheless, the problem that there is a considerable amount of fibre infrastructure already installed that cannot support such an interface directly. There are two principle solutions

- To provide inverse multiplexing within the router whilst maintaining 40G ports at the IP level

- To allow for inverse multiplexing within the transport infrastructure

- The architecture for the optical transport network must build in support for the latter.

- What is meant by optical networking?

- Bearing in mind the rapid changes in technology and their impact on how we design and manage the network it is worth asking if there is only one implementation of G.872 or whether G.872 provides a high level architectural framework for different implementations. A set of simple examples include the introduction of the all-optical regenerator or the all-optical wavelength converter with in built regeneration, alternative multiplexing schemes etc.

- A List of Issues Regarding the Optical Transport Network Architecture

- This section highlights a number of significant issues that need to be addressed within the architecture and its subsequent implementation. None of the issues has a solution within the current network architecture.

- Ownership of the bits and bytes within the overhead. Currently there are eight levels of connection associated with an optical channel and within the overhead only one automatic protection switching channel and three general communications channels. This leads to the problem of ownership. Do the bits belong to the operator at the top of the stack or can they be used somewhere else? Once assigned at one level these bytes cannot be used at lower levels of connection.  It also leads to the problem of what is the service that is being sold –and that will depend upon who owns the bits, resulting in a difficulty in defining the service being offered. This will lead to complete confusion and is unacceptable. There is nothing to stop different bytes being owned at different levels. In short the issue can be summarised as a recursive problem coupled with a non-recursive solution.

- Limited recursion in infinite stacks. The situation of one party requiring bandwidth from another is a recursive problem yet the stack has limited depth. At the same time the intent is to use this to trigger protection.

- The bottom of the network. It is always tempting to add overhead to overhead – to create a new frame for carrying the old frame. This creates its own problems. In the case of selling the SDH frame without a frame to carry it the network operator removes the ability to manage the network except by snooping and creates a service that cannot be carried by the existing SDH network. The optical channel frame can be used to solve this problem. However, what is there to stop the optical channel frame (rather than its payload) being sold as a service? Indeed this is not that far fetched, existing non-standardised metropolitan networks can carry different protocols and bit rates directly on wavelengths. This may in certain circumstances be more cost effective than putting everything into an optical channel frame. In such circumstances the optical channel frame may be seen as just another protocol and bit rate to transfer. Optical networking was supposed to be about carrying the frame and minimum overhead to manage it – not solve the separate problem of selling TDM services – optics doesn’t care.

- Growing optical islands. Consider what happens as “all-optical” optical networks grow in size and electronics moves toward the edge and away from being in the middle. For the purposes of illustration imagine an optical network with 10 optical nodes in series, with the middle eight providing transparent optical connections (with no electronic conversion) and the end two providing electronic processing of the frame. At the intermediate points there is no access to the associated overhead – so the general communications channels can only be accessed at the end points. Therefore such a facility is potentially not going to be available at all nodes – especially if we have all optical transit nodes for all wavelengths (not unimaginable). This becomes worse as the island gets bigger. The problem with this approach is that different operational processes are required depending on whether electronics is present or not. There is no clear requirement defined for the general communications channels in such a network and what is so special about the two end nodes. Exactly the same problem occurs with performance monitoring information and its transfer. It is preferable to have a common process. If the requirement is to provide any node to any node connectivity for certain services it can be achieved by other means.

- A networking problem best avoided A solution based upon multiplexing frames within frames recreates a problem that continues to vex network engineers, namely that multiple protection schemes that have the same reaction time to faults and act autonomously create an escalation problem. When coupled with multiple levels of connection within each layer this becomes increasingly difficult operationally. The case where the higher level paths all either all protected or unprotected is the exception rather than the rule.

- Introduction of new technologies. The ability to support new technologies that allow an increase in bit rate and/or increase optical technology need to be reflected in the architecture and the impact on the operational processes understood.

- Support for new clients. G.872 can be interpreted in two ways – one being that a new technology has to be found that meets the requirements of the optical channel. This is the approach we have adopted. A second alternative is that any digital signal that meets the requirements of G.872 can be used as the format for the optical channel. Therefore there may be more than one type of implementation of the “optical channel” that can be envisaged. In other words can the optical multiplex section have a more than one type of client.

- Ensuring ASON requirements are met. Essentially the model for the control plane should work regardless of the degree to which the optical network is optically transparent. In a digital network topology discovery can be carried out using in-band or in frame techniques. Topology cannot be discovered in an optical network if the information is carried in frame and the frames are not digitally terminated at each node.

- Fibre type and distance. The solution a carrier chooses will heavily depend on distance and fibre type. It is unlikely that there is a one glove fits all solution.

Conclusion

This contribution has described a number of competing factors that highlight some of the limitations of the current architecture. Each of these has to be understood so that a common framework can be developed which meets the requirements of different operating agencies and different types of network. As currently defined the architecture is incapable of supporting the different requirements – a circle that cannot be squared.

In companion contributions to this meeting BT propose a number of extensions to the architecture that deal with each of the points raised above.

___________________