UIT - Secteur de la normalisation des télécommunications	WD

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Turin, 12 – 15 September 2000

Experts Meeting Q.19/13

SOURCE:	Deutsche Telekom

TITLE:	Connectivity of OMS

# Introduction

It is a question whether the architecture G.872 is future proved or it needs same extension in order to permit groups of OCh and cross connection of such groups.

# Discussion

Transparent optical cross connects (e.g. based on MEMS technology) are going to enter ther market. Such devices could cross connect the whole signal (single or multi channel) on a fibre. Due to the fact, that these devices have a fixed price per cross connection, it would be an advantage to maximise the total bandwidth for the cross connection. G.709 and G.959.1 define a 16-channel interface, which channels could be cross connected as a group. However, Recommendation G.872, subclause 6.2.1 states that there is a fixed relation between OTS and OMS and, therefore, connectivity supervision is not required for the OMS layer network.

# Proposal

G.872 should be modified in order to allow cross connections of multi-channel signals.

Connectivity supervision should be required also for the OMS.