UIT - Secteur de la normalisation des télécommunications	WD

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Turin, 12 – 15 September 2000

Experts Meeting Q.19/13

SOURCE:	Deutsche Telekom

TITLE:	Need for higher and lower order ODU services

# Introduction

During the last SG13 meeting the issue of ODU multiplexing was discussed, but no agreement was reached. Is there a market need for TDM in the OTN that cannot be provided by SDH?

# Discussion

There is a growing market for transport services for ISPs, Application Service Provider (ASPs), carrier etc. The bandwidth request moves to higher bit rates ( VC-4). Even when the number of narrow band leased lines ( VC-12) dominates the capacity is dominated by broad band leased lines, i.e. higher order VCs.

A carrier could use a VC-4 leased line for SDH services (lower order VCs) or non-SDH services (ATM, IP, PDH, etc.). On the other hand, SDH cannot provide a leased line connection that could arbitrary structured for various higher order VCs. Today only a dark fibre or WDM could provide such transport service. The OTN will provide it in future, too.

It is expected that the OTN will be deployed rapidly. Subsequently ODU leased lines will be demanded. With the currently agreed OTN multiplexing structure an ODU leased line connection can only be used for non-OTN services/clients such as ATM, Ethernet IP and SDH, i.e. an OTN transport service cannot be used for ODUs of different bit rates.

The carriers carrier business bases on two levels of networking:

- higher order VC and lower order VC,

- lambda (ODU) and higher order VC or

- higher order ODU and lower order ODU.

Among other aspects like supervision capabilities, optimisation of nodes and spans of a network there is a market need for TDM in the OTN.

# Proposals

- G.872 should define multiplexing of ODUs.

- Two levels, lower and higher order ODUs should be defined.

- The multiplexing structure should be as illustrated in Figure 1.

Figure 1: Proposed TDM structure for the OTN