UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

13	WD Nortel-01

Turin, 13 – 15 September 2000

Question(s):	Q.19/13

SOURCE*:	Nortel Networks

TITLE:	Considerations for Multiplexing in the OTN

___________________

# INTRODUCTION

In recent meetings of both SG 13 and SG 15 there has been considerable discussion regarding multiplexing of OTN client signals once they have been wrapped into an ODU.  Multiplexing issues were first brought up at the Kyoto meeting of Q.19/13 [1] resulting in the creation of Study Point 8 of the Living List for G.872. This has lead Q.11/15 to develop bit rate requirements amenable to multiplexing.  However, the discussion within SG 13 on network architectural considerations for the OTN has only begun recently in earnest.   This contribution raises additional considerations that require thorough vetting before multiplexing can be incorporated in the architectural requirements, Recommendation G.872, for the OTN.

# Background

With the ever increasing deployment of dense wavelength division multiplexed networks and the requirement to transport new, non-traditional services, a means of efficiently transporting these services in a well managed way was required. This spawned the creation of a suite of OTN standards.  Although a number of recommendations apply, Recommendations G.872, G.873 and G.709 are of a particular importance due to their maturity.

Q.11/15 has determined G.709 and is progressing it to its final approval.  At the Q.9&11/15 most recent meeting in Ottawa, progress was made in establishing the bit rates and mappings for the ODU1, ODU2 and ODU3 corresponding to approximately 2.5 Gb/s, 10 Gb/s and 40 Gb/s respectively.

Although support of TDM multiplexing was discussed in Q.11/15 at the Geneva meeting in March 2000, the agreement reached was that the current version of G.709 would not incorporate the capability. Given this agreement and the fact that no agreement has been reached in Q.19/13 on this issue, a decision was made in Ottawa with regards to bit rates and client mappings without the benefit of examination of the requirement for multiplexing from Q.19/13.  The majority of participants favoured development of bit rates for the OTN that would support possible TDM multiplexing in future versions of G.709. The bit rates incorporated now support synchronous and asynchronous mapping without precluding future support for PDH multiplexing. The bit rates now support multiplexing of 4xODU1 into ODU2 and 4xODU2 into ODU3 including TDM multiplexing 16xODU1 directly into ODU3 [1].

# Discussion

There are two aspects overall that need to be examined.  First there is adaptation of the client. Adaptation can be either synchronous or asynchronous.   The current version of G.709 supports both methods with the identical bit rate.

Second there is the multiplexing of adapted clients. Once again the multiplexing can be achieved by either synchronous or asynchronous methods.  The current bit rates in G.709 are such that they also support synchronous and asynchronous multiplexing of adapted clients with the most stringent requirements placed on the bit rates by the asynchronous multiplexing case.

The decision to incorporate TDM multiplexing within the OTN Recommendations requires the selection of a multiplexing method (i.e. synchronous or asynchronous).  However, the multiplexing method will have a significant impact on the overall OTN architecture. In this regard, following are a number of key issues that require discussion to progress the OTN work within ITU-T.

1. What are the drivers for TDM multiplexing in the OTN?

2. If TDM multiplexing is required, should it be based on synchronous or asynchronous

multiplexing methods.

3. Is there a requirement for true ADD/DROP capability in a TDM OTN?

Bit rates in G.709 do not preclude future versions of G.709 from supporting PDH multiplexing.

4. Will the use of back to back PDH terminals be sufficient to support add and drop?

6. What are the implications of PDH multiplexing to ASON?

The choice of synchronous or asynchronous multiplexing will have considerable impact on the

ability to dynamically configure connections through a network.

Multiplexing of clients can be achieved in a variety of ways.

5. Is there a need to support a mixture of ODU1 and ODU2 payloads within a single ODU3?

The current version of G.709 supports multiplexing of 4xODU1 into an ODU2 and 4xODU2 into an       ODU3 or 16xODU1 into an ODU3.

7. Are there requirements for inverse multiplexing?

(e.g. How are 40G service rates handled over 10G lines? With additional reference to SP 10

in the Living List of G.872.)

Such a network will need to be managed.

8. Are there explicit supervisory channel requirements to support TDM multiplexing?

9. How are they handled across domains?

Future considerations for the OTN.

10. Will there be any need to support rates higher than 40G?

# Proposal

As we have seen, for the benefit of expediently progressing G.709, some decisions regarding the functionality of the OTN were made without thorough consideration to the architectural requirements.

The current discussions related to TDM multiplexing have significant architectural impact to the OTN.  Hence it is proposed that the issues discussed in the previous section be thoroughly studied to ensure the OTN architecture provides for a network that can be deployed to meet the growing needs of carriers and the uncertainties their customers face in their respective requirements.  To this end, the aforementioned 10 points be added as sub-points to SP8 in the Living List of G.872.

# References

[1] ITU-T SG13 COM13-R 74-E "Study Group 13 – Report R74", April 2000

[2] ITU-T SG15, Q.9&11 Experts Meeting, WD 12,  “Proposal for OTN Rates and SDH Client Adaptation Mechanism”, Tyco Submarine Systems Ltd., July 2000

____________________