- UIT - Secteur de la normalisation des télécommunications

- ITU - Telecommunication Standardization Sector

- UIT - Sector de Normalización de las Telecomunicaciones

- Study Period 1997-2000

Commission d'étudesStudy Group  13	Working Document WDxx

- Turin, 13 - 15 September 2000

- Question(s):	19

SOURCE:	Ciena Corporation

TITLE:	Use of the Optical Auxiliary Channel (OAC) to Carry G.ASON Messages

___________________

ABSTRACT:

This contribution compares various possible methods for transporting the messages needed for the OTN control plane (G.ASON).  A recommendation is made to use the Optical Auxiliary Channel (OAC) for this purpose.

I.  Introduction

Q11 of SG15 has just started discussions on the requirements for the Optical Auxiliary Channel (OAC), known earlier as the Optical Supervisory Channel (OSC).  Till now, the OAC has been envisioned to provide OMS and OTS overheads (e.g. global AIS) as well as carry some non time-critical OCh overheads. In addition, the OAC is also expected to carry non-standardised, carrier-specific, as well as national/administrative traffic.  A list of requirements for the OAC has been provided by WorldCom [1].  Corning has suggested using the OAC to transport Automatic Protection Switching (APS) messages at the IrDI [2]. One of the OAC requirements that WorldCom requested was for carrying ASON traffic. Our contribution concurs and provides added rationale for this position.

II.  Discussion

Q19 of SG13 has recently started work on defining the automatically switched optical network (ASON). However, there has been very little discussion to determine the mechanism for transport of the Signaling, Routing and Address/ Service Discovery messages within the ASON optical control plane. The objective of this contribution is to stimulate discussion of this topic.

There are, essentially, two possible methods that can be used to carry the ASON messages:

- in-band: here the signaling/ routing messages are carried over a communication channel embedded in the optical link between the optical network elements in the OTN. This could, for example, be the overhead bytes in a data channel or a dedicated in-band wavelength on an optical data link.

- out-of-band: here the signaling/ routing messages are carried via a separate communications infrastructure from the information bearing channels of the OTN. Examples of this include the use of an Ethernet or TCP/IP network or the use of a dedicated out-of-band wavelength.

A.  In-band solution:

The in-band solution requires the ASON control and routing messages to be carried by a communication channel embedded within the information bearing channels of the OTN.  Two possible in-band solutions are possible: (a) one using the overhead bytes of the revenue bearing channel, or (b) using a dedicated in-band wavelength.  A number of problems exist with the first solution  (using the overhead bytes of the revenue bearing channel). These include:

- The possibility of creating a large number of  ASON signals from the embedded channels, and the consequent need for co-ordinating all these signals. For example, a 100 X 100 optical cross-connect switch employing an OEO switching fabric could require co-ordination of  up to 10,000 signals at the cross-connect.

- In the case of all-optical networks, this method would require tapping off , demultiplexing and detecting the optical signals to get the ASON signals. These add-drop taps could severely  restrict the optical power budget for the system.

- Defining new overhead bytes in the optical channel for ASON purposes may not always be possible and could lead to interoperability problems. For example, in the case of SONET/SDH, defining a new set of overheads for ASON may create interoperability problems with legacy SONET/SDH systems.

B.   Out-of-band solution:

A number of out-of-band solutions to carry ASON signals are possible.

- One possible method is to create a completely separate control transport network, for example using a TCP-IP network or an Ethernet network, different from the one used to transport revenue-bearing channels. This solution has often been used in the past, for example in ISDN networks or ATM networks. A key drawback to this solution is that this requires building and maintaining not only the OTN but also a parallel separate control network.

- Another possibility is to carry the ASON traffic on the same optical network used for carrying information traffic, but to designate a specific wavelength, different from the one used for the OSC or OAC, to carry this traffic. This method, aside from requiring the standardization of a new wavelength specifically for ASON traffic, also would result in increased add/drop losses, which would have impact on the optical power budget.

- The solution we favour, and the one that avoids almost all of the problems of the other solutions, is to piggyback the ASON messages onto the OAC. The bandwidth requirements to carry ASON messages are likely to be small, as they would only be sent during (re)-provisioning of the optical channels.

An in-band solution is also possible.  In this case, a wavelength that would normally be used for carrying revenue-bearing traffic is dedicated to carrying the ASON messages. However, this would utilise what otherwise would have been a revenue-bearing channel - a major drawback.  On the other hand, if an in-band OAC is already being used for other traffic (e.g. OMS or OTS related indicators or protection switch messages), the ASON traffic could be piggy backed on to it.

Thus, we believe the ASON messages should be carried by the OAC, irrespective of whether the OAC is in-band or out-of-band.  Worldcom [1] also makes a similar recommendation. They show ASON related messages as a part of the carrier specific bandwidth.  However, the carrier specific bandwidth is not subject to standardization, whereas, a standard ASON message format is necessary to control an OTN which may involve multiple carriers.

III.  Recommendation and Conclusion

This contribution suggests that the OAC, in addition to carrying various non-channel associated overheads, carrier specific OMS/OTS overheads and national/administrative overheads, should also be used for transporting the ASON messages. We suggest that a separate portion of the OAC bandwidth be assigned specifically for transporting standardised ASON message for Signaling, Routing, Address/ Service Discovery, and (possibly) Restoration.  A GFP-like Data Link layer could be used to identify the Type of message carried on the OAC.  Similar to the SAPI field in Q.921 LAPD, the Type field code point would identify the OAC client protocol/ overhead, e.g. for ASON: Signaling, Routing, Address/Service Discovery; OMS overhead, OTS overhead, national/ administrative overheads, APS messages, etc

IV.   References

[1] T1X1.5/2000-138, “Optical Networking- OSC Requirements”, July 10, 2000, Lazlo Szerenyi, Worldcom

[2] T1X1.5/2000-171, “Domain Interfaces, Protection Switching, and the Optical Supervisory Channel, Mark Soulliere, Corning