- UIT - Secteur de la normalisation des télécommunications

- ITU - Telecommunication Standardization Sector

- UIT - Sector de Normalización de las Telecomunicaciones

- Study Period 1997-2000

Commission d'étudesStudy Group  13	Working Document WDxx

- Turin, 13 - 15 September 2000

- Question(s):	19

SOURCE*:	Ciena Corporation

TITLE:	 Expanding the scope of G.ASON  to include SONET/SDH networks

ABSTRACT:

This contribution suggests the need for optical control plane mechanisms (i.e. G.ASON) in SONET/SDH networks.  It reflects a unanimous vote of the Optical Internetworking Forum (OIF) at their August 17th TC Plenary meeting.  A G.ASON- like control plane will be especially important for new SONET/SDH Network Elements, such as intelligent cross connects, service access multiplexers, and multi-service provisioning platforms, which support a combination of TDM, virtual concatenation,*  and Layer 2/layer 3 functionality (e.g. distributed L2/L3 switching, MAC frame multiplexing, and L2 add-drop capabilities) over SONET/SDH channels.

*  Virtual concatenation standardization – both low order and high order- has already been completed (T1.105, T1.105.02 G.707, G.783).

The Data over SONET (DoS) specifications being developed by T1X1.5 include:  L2 Add/Drop (via L2 ADMs) over SONET paths, L2 Protection (by L2 ADMs) of DoS channels in a ring topology, and a Generic Framing Procedure (GFP) to map variable length frames to SONET/SDH,.  These capabilities permit multiple L2 address pairs (per SONET/SDH path) to co-exist on a GFP ring, in either  point to multi-point or multi-point to multi-point transmit-to-receive mode.

We specifically suggest the use of G.ASON like principles to automatically provision: SONET paths, the L2 Source-Destination address pairs between L2 ADMs on a GFP Ring, the endpoints of a Virtual Concatenated channel, to provide the time slot maps (frame and sequence numbers) to the Source PTE supporting Virtual Concatenated channels.

We also suggest G.ASON functions for:  L2 protection on a GFP Ring, and to convey bandwidth changes (i.e. Variable Bandwidth Allocation (VBA)) to PTEs on the virtual concatenated channels that have been previously provisioned.  Note that VBA includes: time of day re-provisioning, bandwidth on demand, and a graceful degradation mode of protection/ restoration.  Please see references [1, 2, 3, 10].

I.   Introduction

SONET/SDH has often been criticised, for its rigid fixed time slot bandwidth hierarchy and for its slow, manual provisioning process for setting up links and circuits. The fixed channel sizes are cited as “stranding bandwidth” for frame/ packet based data clients and for inhibiting efficient transport of higher speed clients (e.g. Gig Ethernet, ESCON, FICON, Fiber Channel based Storage Area Networks).  The manual, data base/ records provisioning process can often take days or weeks to install a long distance OC48 link with its constituent circuits.

These perceived problems with SONET/SDH resulted in many long haul carriers and equipment vendors to use L2/L3 switches to first aggregate traffic from multiple sources into one SONET/SDH contiguous concatenated payload (e.g. sts-12c or 48c or 192c) using Packet over SONET (POS).  The L2/L3 POS equipment was then interfaced directly to a DWDM Network Element, without any  intervening SONET/SDH ADM/RTs.  Doing so required use of a DWDM vendor specific (proprietary) Element Management System (EMS) for provisioning wavelengths.  With Optical Cross Connects (OXCs) being deployed as hubs within the network core, there becomes yet another proprietary EMS to contend with.  This results in severe interoperability problems when provisioning an end-to- end light-path.  Specification of the optical control plane will solve this provisioning interoperability problem.  We believe that this is the prime objective of the G.ASON functional requirements specification for the OTN (see Figure 5.).  A parallel effort is underway for fast provisioning of SONET framed optical channels in the Signaling WG of the Optical Internetworking Forum’s (OIF), which is being driven by a set of Carrier Requirements [6].

Despite premature obituaries, SONET/SDH has proved to be an extremely cost effective and resilient transport technology for the access and metropolitan areas (inter-office and inter-POP).  Some of the earlier criticisms, such as the rigid bandwidth structure are being addressed in the T1X1.5 DoS work:  the introduction of the Variable Bandwidth Allocation (VBA) mechanism [1,2], the use of (the now approved) high-order and low-order virtual concatenation [3,4] for “right sizing” the SONET channel to the data client’s requirement, and the SONET MAC for point to point, linear chain, and ring topologies.  The approval of the Generic Frame Procedure (GFP), facilitates mapping of Ethernet as well as other variable frame data clients to SONET.  The DoS work also permits statistical multiplexing of multiple data sources and sinks with L2 Add-Drop capability over a SONET/SDH ring [5].

Upon recognition of the T1X1.5 DoS work and consideration of the need for SONET access networks to support an optical control plane, the following motion was unanimously passed (55/0/8) at the OIF TC Plenary meeting on August 17th:

A letter to be sent to T1X1 and ITU-T recommending the scope of G.ASON be extended to include transport other than the OTN, in particular SONET/SDH.

It was suggested that this “letter” could be a company contribution to the Sept 13-15 ITU-T q19/SG13 meeting on G.ASON and OTN Architecture. That is the primary purpose of this contribution, which also provides motivation and examples of  new SONET/SDH functions that could make use of a G.ASON control plane.

II.   Discussion and Recommendations

- Control plane needed for provisioning SONET/SDH  paths (PTE-to-PTE channels):

With the introduction in T1X1.5 of concepts such as VBA, virtual concatenation and GFP, the concerns regarding the rigidity of SDH hierarchy have been allayed and these concepts allow for more scalability in SONET/SDH networks.  However, the problems of SONET/SDH’s slow provisioning and circuit set-up times remain.  Provisioning of SONET/SDH typically requires network engineers to first review asset inventory and channel assignments to locate available capacity. Then network operators must manually configure cross-connect switches and add/drop multiplexers (ADMs) between the end points. This can take days if not weeks to accomplish. Clearly, there is need for a mechanism that automates this procedure and make setting-up, tearing-down and provisioning circuits much more simplified and quick.

Almost all carriers and service providers alike have determined that there is an urgent need for fast provisioning of circuits, as was enumerated in the Carrier’s Requirements document presented at the  OIF’s most recent meeting (Aug16-17, Barcelona) [6].  The relevant OIF Carrier requirements:

- Promote a standardized optical network control plane, with its associated interfaces and protocols.

Ensure that intelligent optical networking (ON) products from different vendors or employing different technologies can interwork at the control plane level.
This is not meant to preclude vendor specific extensions so long as the extensions do not degrade total network performance. However it is unacceptable to expect carriers to write software to conceal OL vendor or technology incompatibilities.

- Provide rapid automatic end-to-end provisioning including routing by the control plane.
“End-to-end” is an important part of this objective; the value of rapid provisioning in the core network is greatly reduced if traditional provisioning intervals are encountered in metro and access networks.  Hence inter-domain interworking is viewed as being key to realizing many of the hoped-for benefits. It is recognized that rapid inter-domain provisioning must be built upon rapid automatic provisioning within a single domain. In this document “provisioning” refers to network provisioning only and does not necessarily include other customer-facing aspects of provisioning like the establishment of  a new account.

- Provide restoration, diverse routing, and other Quality of Service features within the control plane on a per-service-path basis.
Per-circuit control of these capabilities is important because of the anticipated diversity of needs of the Optical Layer clients.

 We suggest that q19/13 seriously consider defining the control plane mechanisms in G.ASON for establishing SONET/SDH circuits, as well as for OTN provisioning.  This would potentially solve the interoperability problem between intelligent optical cross connects, POS routers, and other intelligent network equipment with SONET/SDH interfaces (e.g. ATM switches, 10 Gig Ethernet switches, Multi-service provisioning platforms -with multiple switch fabrics. etc).

- Use of G.ASON for Provisioning L2 Source-Destination Address pairs on a GFP ring

The need for fast and scalable provisioning becomes even more evident when one considers the use of a SONET MAC (i.e. GFP) for both the point-to-point as well as ring topologies. Nortel [7,8], points out that large statistical gains and improved bandwidth efficiency can be achieved through the use of multiple link approaches – either point to multi-point or multi-point to multi-point with L2 add/drop capability (please see Figures 1-3).   However, these gains and bandwidth utilization efficiency can only take place if the channels (SONET/SDH paths) that are being added or dropped at the various nodes (L2 ADMs) are set-up, torn-down and re-configured rapidly through some type of control-plane mechanism.  We suggest G.ASON for provisioning the permitted GFP Source-Destination Addresses on a GFP ring.  Please see Figures 1-4 for L2 multiplexed connections over SONET paths, between L2 ADMs on a DoS ring.

- Use of G.ASON for L2 Protection Functions

T1X1.5/2000-163 raised a number of open issues with respect to Layer 2 protection for a Data over SONET ring [9].  Most of those issues, such as the location of L2 ADMs on the ring (for proper fault isolation), provisioning of protection and reserved bandwidth, as well as congestion management mechanisms, can best be addressed by the use of G.ASON, i.e. address/ topology discovery, signaling and routing procedures to setup protection paths (see our companion contribution on Proposed Additional Functionality for G.ASON).

Note:  GFP control messages have been proposed by Nortel for APS signaling.  However, those messages might be considered as part of the G.ASON control plane.

D.   Use of G.ASON for provisioning of Virtual Concatenated Channels and for managing Bandwidth Change (AKA Variable Bandwidth Allocation or VBA)

SONET/SDH Sending and Receiving PTE supporting Virtual Concatenated Channels must be informed of the channel size (e.g. value of n in n x vt1.5 or value of N in sts-Nc).  The sending PTE must be informed of the time slot map (frame and sequence numbers for each of the constituent low order or high order signals making up the virtual concatenated channel).   With VBA, the virtual concatenated channel size and time slot map will change, and this must be rapidly conveyed to sending and receiving PTE so that they may “hitlessly” synchronize to the new rate [1,2].   EMS or PTE requests for more (or less) bandwidth –either via preset “time of day re-provisioning” or by  real time requests from the PTE- must also be made, through some yet to be defined mechanism. We suggest use of G.ASON principles for these VBA capabilities.

When a SONET/SDH  PTE supporting virtual concatenation (n x vt1/5 or sts-NC or y x sts-3c) detects a failure on a constituent vt/sts, control signaling is used to remove the constituent signal to provide a “graceful degradation” mode of operation for the virtual concatenated channel [10].  When the failure is restored (via a new protection/ restoration scheme), control signaling is once again used to convey the new constituent signals to the PTE.  The sending and receiving PTE co-ordinate the rate changover on a superframe boundary, so that they may once again send/receive at the rate given by the original virtual concatenated channel size.

III.  Conclusion:  Single Control Plane for Data over SONET/SDH and the OTN

We believe that the definition of a new Control Plane is required only for the new Data over SONET/SDH systems, especially the multi-service access boxes, cross-connects, and L2 ADMs that plan to use GFP, VBA and/or virtual concatenation mechanisms.  These systems are currently being designed with vendor specific EMS’s for provisioning/ re-configuration.  In all likelihood, these new SONET/SDH systems will eventually become subsets of the Optical Transport Network (OTN).  It does not make sense to have two different Control Planes, one for SONET/SDH and one for the OTN (G.ASON).  It seems the G.ASON model (Figure 5.) is appropriate to both networks.

Consequently, we strongly urge the q19/13 and other standards bodies to expand the scope of G.ASON to include SONET/SDH control plane mechanisms.

Note: Figures 1-4 are from Nortel contributions presented at the March 2000 T1X1.5 meeting [8]

Figure 1: Multiple L2 Connections Sharing SONET Paths: GFP Source-Destination Addresses used for each L2 connection

Figure 2: A Hypothetical Example of a SONET Ring with 5 nodes, and each node connected to 3 service nodes (e.g. L2 switches) plus TDM equipment

Figure 3.  Meshed Traffic Patterns via L2 connections over SONET paths

Case 1: Full Mesh traffic demand, and the mapped traffic demand on each SONET link

Case 2: Full homing traffic demand & the mapped traffic demand on each SONET link

Figure 4.  Pt-Multi-Pt L2 connection (over multiple ring L1 networks)

ADM

ADM

ADM

ADM

ADM

ADM

ADM

L2X

L2Y

L2X

L2Y

tribs

Figure 5.  Logical view of Automatic Switched Optical Network (ASON) architecture

References

[1] T1X1.5/2000-156, “A Proposed Variable Bandwidth Allocation (VBA) Methodology for SONET Virtually Concatenated SPEs)”, July 10-14 2000, Lucent/Nortel, Nevin Jones, Huub Van Helvoort and Trevor Wilson.

[2] T1X1.5/2000-157R1, “A Justification for a Variable Bandwidth Allocation methodology for SONET Virtually Concatenated SPEs)”, July 10-14 2000, Lucent/Nortel, Nevin Jones and Trevor Wilson.

[3] T1X1.5/2000-126R2, “SONET-Payload Mappings (inclusion of Virtual Concatenation))”, July 10-14 2000, ANSI T1.105.02 Update, T1.105.02a.

[4] T1X1.5/2000-146, “Transparent Mappings into SONET Paths for Block-Coded Data Formats)”, July 10-14 2000, Lucent/Nortel, Tim Armstrong, Chris Murton, Enrique Hernandez-Valencia and Nevin Jones.

[5] T1X1.5/2000-147, “Generic Framing Procedure (GFP) for Data over SONET/SDH (DoS)”, July 10-14 2000, Lucent/Nortel/Data Communications Technology (DCT) and PMC-Sierra, Enrique Hernandez-Valencia, David Kirk, Alan J Weissberger & Gordon Cheng.

[6] OIF2000.155, “Carrier Optical Services Framework and Associated Requirements for UNI”, Aug. 16-17, Barcelona, OIF Meeting, OIF Carrier Group (AT&T, Cable & Wireless, Deutsche Telekom, Enron, GTS, KDD, Korea Telecom, NTT, Palm, Sita, Sprint, TDK, Telecom Italia (CSELT), Tycom, UUNET, Worldcom).

[7] T1X1.5/98-086, “Point-to-Point vs Multiple-Link Approach for a New Link Layer Protocol”, July 1998, Nortel, Xinyi Liu

[8] T1X1.5/2000-104, “Realizing the Benefits of Shared SONET Paths”, March 21-24, 2000, Nortel/PMC-Sierra, Tim Armstrong and Gordon Cheng.

[9] T1X1.5/2000-163, “Open Issues on L2 Protection for Data over Sonet (DoS)”, July 11-14, 2000, Data Communications Technology, Alan J Weissberger.

[10] T1M1.3/99-141, “Protection and Restoration Requirements for Virtual Concatenated SONET Channels (lower order and higher order)”, August 4, 1999,  Data Communication Technology, Alan J Weissberger.