UIT - Secteur de la normalisation des télécommunications                                                     ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

- Study Period 1997-2000

Commission d'étudesStudy Group  13	Working Document WDxx

- Turin, 13 - 15 September 2000

- Question(s):	19

SOURCE:	Ciena Corporation

TITLE:	Proposed Additional Functionality for G.ASON

ABSTRACT:

This contribution reflects the Source Company’s view on additional functionality for G.ASON.  It is based on analysis of work that took place at the ITU-T q11/15 July 17-20 meeting, the IETF July 31- Aug 4 meeting, and the OIF Aug 16-17 meeting.

We first provide our understanding of optical control plane functions and an overview of OIF work in this area (Carrier Group Requirements document [1]).  Next, we propose several new or different functions that we believe should be included in the G.ASON functional requirements.  We conclude by suggesting that the G.ASON model should be consistent with the OIF control plane model, even though different framing schemes are used for the optical channels (G.709 digital wrapper for the OTN vs SONET framed signals in the OIF UNI 1.0).

- Introduction:

We believe that the optical control plane may be decomposed into the following processes [2]:

-Selection of an Address plan to identify the Endpoints of an optical channel (known in the                    OIF as a “lightpath”)

-Address Registration of all OTN Endpoints

-Address (neighbor) discovery between Endpoint and Optical edge NE

-Service discovery between Endpoint and Optical edge NE

-Routing protocol with topology and link state advertisements (e.g. OSPF with optical network extensions)

-UNI Signaling to request setup, modify, release, query, and scheduling of optical channels

-Inter-node Signaling (e.g. RSVP or CR-LDP) for optical channel connection control and attribute management

–Route/ path computation algorithm (not subject to standardization) for optical channel establishment or re-arrangement

-Restoration using routing protocol and inter-node signaling protocol noted above

We believe that all of the above procedures should be considered in the G.ASON work, as the OIF and IETF are currently evaluating them as part of their respective optical control plane specification work.

# II.   Discussion:

# The OIF Model for Optical Control plane is specified in the Carrier Group requirements document [1] and will be reflected in OIF UNI 1.0, scheduled for completion by year end 2,000.  The relevant sections of [1] are reproduced here, for information only.

# Definitions:

# -Signaling is the collection of processes using messages to achieve communication between entities connected through dedicated communication channels using well-defined protocols.

# -Routing is the collection of processes for dynamic dissemination and propagation of the network information among within the network based on well-defined protocols.

# -Resource and Service discovery is the automatic process between the connected network devices using protocols to determine the equipment, software, networking and other available features to support a customer offering.

B.    OIF Carrier Group Objectives:

(1)  Promote a standardized optical network control plane, with its associated interfaces and protocols.                                                                                                           Intelligent optical networking (ON) products from different vendors or employing different technologies can inter-work at the control plane level.
This is not meant to preclude vendor specific extensions so long as the extensions do not degrade total network performance. However it is unacceptable to expect carriers to write software to conceal Optical Layer vendor or technology incompatibilities.

(2)	Provide rapid automatic end-to-end provisioning including routing by the control plane.
“End-to-end” is an important part of this objective; the value of rapid provisioning in the core network is greatly reduced if traditional provisioning intervals are encountered in metro and access networks.  Hence inter-domain inter-working is viewed as being key to realizing many of the hoped-for benefits. It is recognized that rapid inter-domain provisioning must be built upon rapid automatic provisioning within a single domain. In this document “provisioning” refers to network provisioning only and does not necessarily include other customer-facing aspects of provisioning like the establishment of a new account.

(3)	Provide restoration, diverse routing, and other Quality of Service features within the control plane on a per-service-path basis.
Per-circuit control of these capabilities is important because of the anticipated diversity of needs of the Optical Layer clients.

(4)	Provide policy-based call acceptance and peering policies and support usage-based accounting.

(5)	Offer carrier-specific “branded” services.
A fundamental carrier business need is the ability to put together packages of features, quality of service, support, and pricing which they can then market.

(6)	Rapid deployment of new technologies and capabilities with no network service disruption.
In particular deployment of a new vendor X capability should not depend on vendor Y’s willingness to upgrade their control plane software. It is recognized that the part of the network served by vendor Y equipment may not get the benefits of the new capability in this case.

(7)	Protect the security and reliability of the optical layer, and particularly the control plane.

(8)	Preserve carrier’s control over its network resources.

(9)	Reduce the need for carrier written OS software through heavy use of open protocols and vendor and third-party software, particularly for network provisioning and restoration.

(10)	Ensure the scalability of the Optical Network.

Several aspects of scalability are highly important in a carrier’s network:

- Node scalability - the size of a single node is expected to be sized anywhere between several hundred and several thousands optical terminations.

- Network scalability - the size of an inter-connected network is expected to be anywhere from several to hundred or thousands of nodes.

C.       Optical Control Plane Requirements

- R1 The control plane must have the capability to establish, tear-down and maintain the end-to-end light-path in real-time.

R2 The control plane must have the capability to establish, tear-down and maintain the hop-by-hop light-path segments between two end-points.

R3	The control plane must have the capability to support traffic-engineering requirements including resource discovery and dissemination, constraint-based routing and path computation.

R4	The control plane must have the capability to support various protection and restoration schemes for the optical channel establishment.

- Additional capabilities recommended for G.ASON:

In light of the above Carrier Group Requirements and considering the work done at recent meetings (ITU-T q11/15 July 17-20, the IETF July 31- Aug 4, and the OIF Aug 16-17 meeting), we propose the following additional functionality for G.ASON:

- UNI Signaling Messages and their Attributes:

In addition to connection Setup, Release, and Modify, we recommend two other UNI  Signaling  messages:

-Query: The UNI shall support the client edge device having an UNI signaling channel to request for connection query (R6 of [1]).  The parameters or attributes to be queried are for further study.

-Schedule an optical channel for subsequent setup, including a reversion mechanism (e.g. time of day auto setup and release).  We think this should be a separate message and not an attribute of the Setup message, as it may also involve a connection release component in the event or reversion.

-------------------------------------------------------------------------------------------------------------------------

We also recommend the following Signaling message attributes/ parameters be considered:

-Protection/ Restoration in the Setup and Modify messages.  For protection: 1:1, 1:n, OCh shared protection ring, other protection mode.  For restoration: high priority, low priority, none, pre-emptable, etc

-Ability to negotiate parameters in the Setup, Modify, or Schedule messages.  Note that the Modification of certain parameters, such as optical channel bandwidth, also require the destination device to accept the request before the network will perform the changes.

- 3rd Party Signaling

The difference between third party call control and UNI Signaling is that a third party (i.e., neither source nor destination endpoint of the network) initiates the connection. Consider a carrier’s carrier network or a public optical network used to interconnect one or more private networks composed of IP routers or ATM switches.  In this case, the Signaling messages will usually not originate at the public UNI, but at a private network interface behind it.        For example, suppose that a set of IP routers are interconnected with SONET STS-12c paths..  Furthermore, assume that a separate traffic engineering application in each router monitored the traffic between them.  Such an application could use a third party signaling interface to add or delete STS-12c connections between the routers to optimize the IP traffic flow, doing load balancing where appropriate.  Note that this application has knowledge of the IP layer topology and traffic patterns, but not of the SONET layer topology.

From [1]:  “Third party signaling is a special signaling scenario and is required to support clients unable to signal to the optical network via a direct communication channel. In this situation a third party systems containing will initiate and process the information exchange on behalf of the client device and the network device. The UNI-C and UNI-N entities in this case reside outside of the client and network devices in separate signaling systems.

- End System and Service Discovery - from [1]:

Service discovery is the process of information exchange between directly connected end-system and network edge ONE equipment. The objective is for network to obtain essential information about the end-system and thereby provide the available services to the end-system.

End-System discovery is the process of information exchange between directly end-system and network edge ONE equipment. The objective is for each side to obtain essential information about the network element at the other side of the optical link and understand the link between them.

The information exchanged in the end-system discovery process will enable both network elements to recognize the existence and identity of each other, obtain topological and functional information  from each other, and understand how they are connected and what the link states are.

The list of essential information obtained in the discovery process should be accessible from both network management interface and management plane interface of the network element for troubleshooting purpose.

Service and End-system discovery processes are defined by the type of information to be exchanged, the procedure that governs the exchange process and the communication mechanism to be used to realize the information exchange.

Service and End-system discovery are key to facilitating interoperability, configuration automation and identifying signaling entities and their relationships for automatic service provisioning.

The communication mechanism for the service and end-system discovery should be selectable through configuration or through automatic negotiation.

The communication mechanism for service and end-system discovery is usually independent of the communication mechanism for control signaling channel, but that does not preclude the possibility for them to share the same channel End-system-discovery may be done via in-band communication or the information may be provisioned through out-of-band communication channel

Note:  At the August OIF Signaling WG meeting, there were two competing proposals for end system/neighbor discovery. The editors of the proposals merged them, thereby establishing the technical direction.  The merged proposal will be posted on the OIF Contribution web site.

- Co-ordination of Optical Channel Setup with G.709 Connection Monitoring Endpoint (CME) Activation

At the July Q11/15 meeting Lucent proposed text for activation of CME activation along with ACT messages structure and ACT field coding  [3].  A companion contribution [3] suggests adding an additional G.709 appendix for scenarios concerning the activation of a CME.  However, neither contribution identified how the CME Activation process was initiated, what effect this might have on a connection setup request if one or more CME Activation’s failed, or how CME Activation would be coordinated with the G.ASON control plane.  In particular, we raise the following open issues:

-How should the network request CME Activation to be initiated after an end-to-end light path has been calculated for the requested optical channel?

-Should the SETUP Acknowledge message be delayed until all CME endpoints have completed the Activation process?

-What if one or more CME sessions fail to activate?  Should a new path be calculated and new CME sessions initiated?  Or should the existing path be maintained with fewer CME’s operational?

-How to report and resolve incorrect IDs of CMEs which are detected during CME activation?

We propose that these matters be considered as part of the G.ASON work.

- Restoration:

OIF Carrier Group Objective 3 (above) should be considered as a G.ASON control plane function: Provide restoration, diverse routing, and other Quality of Service features within the control plane on a per-service-path basis.

Restoration is a reasonable objective of G.ASON, assuming the use of a topology and link state routing protocol (e.g. OSPF with optical network extensions defined by IETF) along with inter-node signaling (the same type used for optical channel provisioning).  The goal is  to rapidly restore service after a detection of a hard failure.  This work will involve the following:

-specification of restoration attributes (see a. above)

-ensurance of diverse routed alternate paths

-use of shared risk link groups (SRLGs) which have the same vulnerability

-failure reporting status and periodicity of link state routing protocol (e.g. OSPF with IETF extensions?)

-definition of fault indicators that trigger restoration

-selection of pre-calculated or a dynamically re-calcualated path

In particular, links that have differing SRLGs need to be grouped separately to allow diverse routing to take place, i.e., an SRLG would be a property of the bundle that gets advertised since individual links within a bundle do not get advertised by the routing protocol [2].

- Sub-rate multiplexing:

The optional capability to multiplex multiple client signals (either the same or different types) into an optical channel, in order to make more cost effective use of network resources (wavelengths, ports, 3Rs, etc).  This capability would be initially requested at optical channel Setup time, but could be Scheduled or use the Modify message to add or delete a sub-rate channel.

Sub-rate multiplexing provides much more efficient use of OCh transport, especially over a long distance link between a pair of optical NE’s.  It is not at all economical to carry a single client signal of <2.4Gb/s on a wavelength that is capable of transporting 10G b/s).  Sub-rate multiplexing also uses fewer optical channels and optical NE (edge node, OXC, and intermediate node)  ports than would otherwise be necessary

While different client signals could be multiplexed over an optical channel, they would likely all share common quality of service parameters (e.g. same protection/ restoration, delay variation, etc.  An example of sub-rate clients sharing the same optical channel could be IP-MPLS, IP-PPP, Gig Ethernet, 10Gig Ethernet.  Another example might be ATM CBR traffic and SDH TDM traffic.

Note:  We do not agree that SDH/SONET multiplexing will in all cases be used to multiplex  subrate clients into an optical channel.  If that was the case, then the OTN would be a single client - SDH/SONET- network.  Consider the case of multiplexing GigE or 10GigE with the LAN PHY.  Neither of these employs SONET/SDH framing.  Further, despite a recent Nortel proposal in T1X1.5, there is no standard mapping of SANs/FC, ESCON, or FICON (all of these are 8B10B PHYs) to SONET/SDH.

To facilitate this sub-rate multiplexing capability, we suggest the following:
- UNI Signaling messages for Och Set-up, Release, Modify, and Schedule need to have unique sub-rate attributes specified (e.g. channel size, client type, etc)

-Network’s support of sub-rate multiplexing could be determined by a Query message

-Query messages would need to be able to determine how many sub-rate channels could be multiplexed on a given point to point OCh, and how many were currently active.

-Set-up, schedule or modify  to add/delete a sub-rate channel could be negotiable, e.g. a smaller channel size than requested could be offered by the network to the user (endpoint of the OTN).

- Co-ordination and co-operation between G.ASON Optical Control plane and OTN Management plane

We assume that static provisioning will continue to use the Management plane (EMS-NE or EMS-NMS-EMS-NE) while dynamic provisioning or “optical channels on demand” will use the G.ASON Optical Control plane.  How will these planes be coordinated such that the Management plane is made aware of optical channel and sub-rate connections set-up via the G.ASON control plane?

A few questions we have are the following:

-What impact does this have on an EMS, NMS, and Gateway NE in the Management Plane?

-When one plane sets up an OCh (light path connection) does it inform the other plane?  How?  By  using primitives and parameters?

-If the EMS is responsible for static provisioning, is it possible for the OTN client to subsequently Modify an attribute or Query the network (via the Control plane)?                                                              -If protection (e.g 1:1, 1:n, Shared protection Och ring, etc) is done in the Management plane, but Restoration (for mesh, linear, ring, mixed topologies) in the Control plane, can both be available in the same network?  If so, how to decide which one to use for resiliency when a detected failure causes a service outage?  Which one should be tried first?  What are the hold -off timer values?

## Summary and Conclusions:

## We suggest that the G.ASON model be consistent with the OIF control plane model, even though different framing schemes are used for the optical channels (G.709 digital wrapper for the OTN vs SONET framed signals on the lightpaths in OIF UNI 1.0).  In addition, we note that in G.709, CME Activation should be coordinated with G.ASON provisioning process and that sub-rate (multiplexed Optical Channel) connections, if available, should be handled by the G.ASON control plane.  Finally, we suggest coordination and cooperation between Control and Management planes.

## References:

## [1] OIF-00.155, Carrier Optical Services Framework and Associated Requirements for UNI,  OIF Carrier Group

## [2]  Paper to be published in IEEE Communications:  IP Centric Control and Management of Optical Transport Networks, Greg Bernnstein, Jennifer Yates, Deb Saha

## [3]  Q11/15 July 2000  WD-04: Hitless Activation of Connection Monitoring Endpoints, Lucent

## [4] Q11/15 July 2000  WD-16:  Text Proposal for subclause 15.6.2.4/ G.709, Lucent-Nederlands