UIT - Secteur de la normalisation des télécommunications	WD

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Turin, 12 – 15 September 2000

Experts Meeting Q.19/13

SOURCE:	Deutsche Telekom

TITLE:	ASON interface structure

# Introduction

A companion contribution (WD ……) showed that different type of ASON equipment and further OTN equipment will coexist in future networks. It was also shown that a global switched optical network would base on switched ODUs. Up to now it was not discussed whether a switched ODU service needs some specific design of an ONNI.

# Discussion

Draft G.709 specifies an interface where ODU and OTU are frame synchronous. This concept leads to the known problem that a phase and frequency jump occurs in the event replacement of AIS by a normal signal. Such a phase and frequency jumps cause short-term interruption of CM7 and running TCM function. A workaround may be a appropriate solution for a transport network, because activation and deactivation of AIS should occur seldom.

ASON switches provide connections for different ODU sources which have there own timing. Any establishing (maybe also releases) of a connection causes a phase and frequency jump and a hit. This is not acceptable since switched connections may be established and released frequently.

Also for ASON it would be better to have lower and higher order ODUs. The higher order ODU would provide a permanent connection between adjacent ASON switches and between ASON switches and ASON terminals. The timing of the higher order ODU is independent on the timing of the switched lower order ODUs due to asynchronous mapping.

A signalling channel needs a fixed connection be adjacent ASON switches and between ASON switches and ASON terminals. It cannot be assigned to the OTU since there may be a chain of OTUs. An allocation to the overhead of the ODU would imply that at least one user channel needs to be established between adjacent ASON switches. With a lower and higher order ODU structure it would be possible to assign the signalling channel to the overhead of the higher order ODU.

# Proposal

G.872 should define higher and lower order ODUs.

G.ason should specify switched lower order ODUs over higher order ODUs.