UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

13	WD Siemens-01

Turin, 13 – 15 September  2000

Question(s):	Q.19

SOURCE*:	Deutsche Telekom
Siemens AG

TITLE:	TDM of ODUs

___________________

# INTRODUCTION

Several contributions to the last SG13 and SG15 plenary meetings (COM13-D.947 [1], COM13-D.998 [2], COM15-D.808 [3]) have indicated a need for TDM of Optical Channels/ODUs. This contribution tries to justify the proposal in addition to the arguments already mentioned in COM13-D.947 [1], e.g. generation of OTN islands, limitations of SDH multiplexing and transparent transport of SDH signals

# Discussion

The essential business for a carrier is to over transport services to its customers. The bandwidth for leased line services has increased over the years from 64 kbit/s over 2 Mbit/s to VC-4 and beyond today. Carriers carrier over already transparent transport of full SDH signals. With the decision of Q.19/13 to map IP, ATM and Ethernet Signals directly into the Optical Channel/ODU, the ODU will become the next level of leased line service as soon as OTN interface will become available at IP routers, ATM and Ethernet switches. Carriers carrier will even see requests for transparent ODU transport as soon as G.872 based OTN networks will be installed by carriers. With the introduction of automatic switched transport networks the demand for such high bandwidth transport services will even increase.

The service bandwidth is normally de-coupled from the line bit rates used within the carrier network. Multiplexing techniques are used to aggregate several services on a line signal. This allows the operator to select the economical best solution for its transport network and to de-couple its transport network evolution from the services offered.

Possible multiplexing techniques are for example TDM and WDM. From a technical point of view TDM is limited today to 40 Gbit/s while WDM can aggregate much higher capacities. On the other hand TDM has cost advantages in case of lower line bit rates (e.g. 2.5 Gbit/s and below). As we all know technical limits and costs change over time. In addition the costs are different from carrier to carrier depending on the network size, required transport capacity, existing infrastructure and offered services. The choice for one or the other technique will therefore be different from operator to operator and may change over time.

The OTN as defined in G.872 is currently a WDM based hierarchy. However both WDM and TDM will be used in the future in order to cope with the increasing bandwidth demands. Today's multi-operator/private network environment with connections crossing several, even nested operator domains stimulates the need for multiple levels of multiplexing in order to allow every operator its own network optimisation and to offer transparent transport services. A carrier aggregates the signals from its customer to a fat pipe. A carriers carrier wants to aggregate several of these already fat pipes coming from its customer (the carrier) to even fatter pipes as shown in Figure 2. The aggregation should be service transparent. Limiting multiplexing only to WDM restricts the carrier in its choice. With WDM the ODU services are limited to the channel bandwidth used in the transport system. An upgrade to transport systems with higher channel bandwidth would require an upgrade of the ODU services also if the available bandwidth shall not be wasted. Service and transport are no longer independent.

Figure 1 – Multi-operator transport

It is often suggested to use client TDM instead, e.g. SDH multiplexing. However this is not possible for an ODU service. An ODU service doesn't care about the client and cannot use TDM at the client layer. Furthermore client multiplexing would require support of all possible types of clients, not to count the security problems of multiplexing for example at the IP level. On the other hand direct mapping of IP, ATM, SDH and Ethernet into the ODU doesn't make much sense without ODU transport services. Note that only a ODU transport service and TDM of ODUs provides transparent transport for whole SHD signal without the need to terminate RSOH, MSOH and timing if multiplexing to higher bit rates will be used.

Although WDM sits normally on top of TDM (below from a layer network view), both are interchangeable. A 2.5 Gbit/s ODU-1 service that is transported via a 80 Gbit/s WDM system (32x2.5 Gbit/s) in one part of a network can be transported over a 40 Gbit/s TDM system in another part of the network or in a combination of both (e.g. 80x10 Gbit/s) as shown in Figure 2. The management and supervision of this service shall not be influenced or interrupted when changing between the different transport systems. Only the choice between WDM and TDM for ODU services allows this. Without TDM of ODUs either I am restricted to capacity increase using WDM only or I cannot offer ODU services. Only a combination of both WDM and TDM allows to build future prove networks and to leave the choice to the carrier for its best solution.

Figure 2 – Mixed WDM and TDM transport

With the 3 OCh bit rates of ~2.5, ~10 and ~40 Gbit/s the issue of moving up to the next bit rate has to be solved as otherwise no interworking between OTN networks with different OCh bit rates is possible. The interworking should be transparent for and independent of the OTN clients. This is only possible with TDM of ODUs and not with client TDM.

Often the "multiplex mountain" is mentioned as argument against multiple levels of TDM. Looking at today's environment with private networks, carriers and carriers carrier every network in this hierarchy has different demands for the multiplexing level. It depends on the operators choices for offered services and line bit rates which levels of multiplexing are required. Limiting the choices by removing some multiplex options is not a good approach. Furthermore with today's technology is it not a problem to integrate several TDM levels on a single card.

A flat multiplex hierarchy would look similarly to SDH, just with higher bit rates and the same problems. With SDH it is not possible to offer a client independent transparent 2.5 Gbit/s service over a STM-64 system in a standard way. The 2.5 Gbit/s could be either a single VC-4-16c or a mixture of VC-4 and VC-4-4cs (or even VC-3 in the SONET world). Multiplexing to STM-64 in a standard way requires processing of the individual VCs. The VC structure has to be known and supervision is only possible per VC but not for the 2.5 Gbit/s service as a whole.

With a flat ODU multiplex structure as shown in Figure 3 a) an ODU2 service transported over a 40G ODU3 system has the same problems. Depending on the ODU2 client the ODU2 can be directly multiplexed (non-ODU client) or has to be terminated (ODU1 clients) and the ODU1 has to be multiplexed. The ODU2 service is no longer client independent. Only the multi level multiplex hierarchy (Figure 3 b) allows to offer transparent ODU2 services over ODU3 systems and as such de-couples service and channel/line rates.

Figure 3 – ODU multiplex structures

TDM of ODUs will also not limit the application of ODU/OCh protection schemes as mentioned in COM13-D.992 [4]. OCh shared protection rings are possible with TDM and WDM as the protection switching criterias and protocol are solely based on the information and overhead of the protected OCh itself using protection sub-layers. It doesn't care about the server layer.

# Proposal

It has been shown that a new future prove transport hierarchy like the OTN should allow both WDM and TDM in order to allow the optimum solutions for services and line bit rates today and in the future. Allowing only WDM in the OTN restricts the availability of ODU transport services, which contradicts Q.19/13 decision from the last meeting to have direct mapping of IP, ATM and Ethernet into the ODU.

It is therefore proposed to introduce TDM of OCh/ODU into G.872.

Note that Q.11/15 has already considered TDM of ODUs in the definition of the ODU bit rates. With the currently defined 3 ODU bit rates the following multiplex schemes are supported:

4xODU-1 -> ODU-2

4xODU-2 -> ODU-3

16xODU-1 -> ODU-3

Both the flat and multi-level multiplex hierarchy are supported. The carrier can chose its best solution.

# References

[1] ITU-T SG13 Delayed Contribution COM13-D.947 "Time division multiplexing in the OTN", Siemens AG, March 2000

[2] ITU-T SG13 Delayed Contribution COM13-D.998 "Multiplexing of Clients into an OCh versus TDM in the OTN", Deutsche Telekom, March 2000

[3] ITU-T SG15 Delayed Contribution COM15-D.808 "Multiplexing of Client Signals into one high bit rate OCh and TDM approach in the core OTN", Alcatel, April 2000

[4] ITU-T SG13 Delayed Contribution COM13-D.992 "Optical channel multiplexing", BT, March 2000

____________________