UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

13	WD Siemens-02

Turin, 13 – 15 September  2000

Question(s):	Q.19

SOURCE*:	Siemens AG, Germany

TITLE:	OCh functional architecture

___________________

# INTRODUCTION

Q.11/15 is defining the Optical Channel Data Unit ODU and Optical Channel Transport Unit OTU as the signal format for the Optical Channel. In addition an OCh that supports also other digital data formats (e.g. SDH signals) is discussed. This contribution discusses the functional architecture for the OCh.

# Discussion

## Characteristic information and connections

Contributions and discussions at the last meeting have shown that we have to differentiate between the characteristic information CI of a layer network and the transport capabilities of a connection.

The CI of a layer network is defined by the trail termination. It defines the specific signal structure needed for the transport capacity offered at the access point and the trail supervision terminated by the trail termination. For the SDH VC-4 path layer it is the VC-4 frame with the POH and container C-4 included

A connection however needs not to be specific on the signal structure. Some examples:

- An optical fiber connection is not specific for a certain optical signal like an STM-16 (OS16). It can support a wide range of optical signals. While the STM-16 optical section layer OS16 has a certain bit rate, coding and signal format as CI, the optical fiber connection requires only a subset; e.g. the signal has to be optical.

- A coax or 2 wire connection is not specific for a certain electrical signal like an STM-1 (ES1). ). It can support a wide range of electrical signals

- A connection consisting of SDH MS trails, MS/S4 adaptations and S4 connections supports a digital frame of 18792 bits repeated every 125 s (VC-4 frame), but is not specific on the content of the frame. It doesn't care about the POH and container C-4.

- A link connection consisting of a SDH VC-3 trail and S4/P31x adaptations supports a digital signal with 34 Mbit/s, but doesn't care about the specific frame structure while a S3/P31e or S3/P31s adaptation requires the specific PDH or SDH frame (see Figure 1).

A connection is in most cases not specific for the whole CI of a layer network, but only for a subset. It can therefore support all layer networks that have this subset as part of their CI. E.g. a P31x connection is specific for the bit rate of 34 Mbit/s, but it doesn't care about the frame structure. It therefore supports the P31e (34 Mbit/s PDH frame) and P31s (34 Mbit/s SDH frame) layer network CI as shown in Figure 1. The adaptation function between the server layer and the P31x connection point performs bit rate adaptation, but no framing. An additional adaptation process with a framing circuit is required in order to get the CI of the specific layer networks P31s or P31e. The P31x connection is a transport entity that falls between layer networks.

Figure 1 – P31x connection

## OCh connection

For the OCh we have a similar case as discussed above. An OCh connection can support various signal formats and bit rates, depending on the processing along the connection. A pure optical connection is transparent for a wide range of signals modulated onto an optical carrier. Even analog signals are possible. Wavelength conversion with o-e-o processing terminates the optical carrier and is often limited to digital signals of a certain bit rate range. 3-R regeneration might be limited to a single bit rate or a small bit rate range. A framing process in the adaptation function requires a specific frame format. Figure 2 shows the different formats of the OCh information at the different processing levels. Note that the information itself is the same for the various formats. Modulation of a digital signal on an optical carrier for example adds no additional information. It is therefore not needed to define a layer network for each of the different formats.

Figure 2 – OCh signal formats

## OTU/ODU

The OTU/ODU frame format is defined by Q.11/15 as a specific implementation for the OCh layer network defined in G.872. It provides the necessary overhead and processing in order to support the OCh transport, management, monitor and survivability requirements defined in G.872. An OTU or ODU termination requires the specific frames shown in Figure 3.

Figure 3 – OTU/ODU frame

## OCh functional model

The ODU is the OCh trail according to G.872. It supports the transport of various client signals like SDH, IP, ATM, and Ethernet and provides the trail supervision (e.g. connectivity, signal quality, and continuity). If allows 6 levels sub-layer monitoring (tandem connection monitoring) for domain monitoring and protection switching. In addition the operators had identified in Q.11/15 a need for supervision of 3-R spans. This is achieved by the OTU, which provides also FEC. While the TCM sub-layers are optional, the OTU is required. As its frame format is different from the ODU it can be defined as a layer network of its own.

Note that the location of OTU terminations is defined by the physical topology of the network (e.g. 3-R regeneration is required after a certain transmission distance) and not by a transport service. The ODU provides the transport service.

Figure 4 shows the proposed functional model. The ODU layer network includes the ODU trail termination and sub-layer functions. The OTU layer network includes the OTU trail termination.

Between the OMS and OTU layer different adaptation functions exists. The OMS/OTU adaptation performs FEC, framing, clock recovery, e/o, o/e-conversion and wavelength assignment. The OMS/OCh includes wavelength assignment, optional with o/e/o conversion. An OCh/OTU adaptation performs the additional functions like clock recovery and framing if the OTU is terminated in the same network element. The OCh represents a connection that is not limited to the OTU frame format.

Flexible connectivity is available for OCh and ODU connections. For the OTU it makes not much sense as 3-R regeneration which is part of the server/OTU adaptation requires termination of the OTU according to the current agreements.

In draft G.709 the OTU is further decomposed into an OTU and OTUS layer network as shown in Figure 5. As the OTU and OTUS trail terminations are always co-located and always required we don't see a need for a sub-layer. Furthermore e/o conversion, clock recovery and LOS supervision are not related to the OTU trail. The wavelength assignment, which includes the generation of the optical carrier, is part of the adaptation to the OMS layer. As the LOS detection normally detects a loss of the optical carrier and not a loss of the information itself it should be related to the termination of the carrier. Loss of an individual channel can occur for example due to wavelength (laser) drift, filter drift or attenuation of the optical carrier below the receiver (o/e conversion) limits. LOS supervision should therefore be located at the o/e conversion, which is part of the adaptation and can be located at any point along an OCh connection. Note that LOS is here more like a LOP in SDH as it looks on the carrier signal necessary for the transport via the server (OMS) layer and not on the OTU information itself.

Figure 4 – OCh functional model

Figure 5 - Processes performed by OTUk functions

# Conclusion

The functional model as discussed above shall be used for the OCh. Instead of increasing the number of management entities by defining layers and sub-layers for different representations of the same information, a more flexible concept for connections (e.g. connections that support several layer networks) and adaptations (splitting of adaptation) shall be used.

____________________