UIT - Secteur de la normalisation des télécommunications	WD

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Turin, 12 – 15 September 2000

Experts Meeting Q.19/13

SOURCE:	Deutsche Telekom

TITLE:	Multiplexing and invers multiplexing of ODUs

# Introduction

The current draft of G.709 defines the bit rates of ODU1, ODU2 and ODU3. Each of them is mapped directly into the associate OTUk (see Figure 8-1/G.709). Once an ODUk is assembled it determines the line rate and cross connect rate of the transport network.

Transport services need common bit rates and formats. On the other hand, spans and nodes of transport networks have to be optimised in terms of number and bit rate of channels. SDH and PDH network meet both requirements, since they have TDM capabilities. Such capabilities are currently not specified for the OTN. However, there is a need for mechanism for changing bit rates in the OTN. Multiplexing or inverse multiplexing could do this. What do we need, multiplexing, inverse multiplexing or both?

Figure 8-1/G.709 - OTM0 IrDI structure

# Discussion

The transport service across the Atlantic were restricted to 64 kbit/s or E1/T1 rates before the first SDH undersea cable was in service (just a couple of years ago). However, the line rates across the Atlantic and at both sides were much higher. Today the service (source) rates grow faster than the line rates. It may happen that 40 Gbit/s signals have to be transported before 40 Gbit/s networks are available everywhere.

TDM of ODUs would provides common bit rates and formats in the network independent on the line rates. Inverse multiplexing would also offer common service rates and formats that are independent on the line rates. However, there are important differences. The bit rate of an ODU2 is greater than 4 times ODU1 and an ODU3 is greater than 4 times ODU2. Therefore, the transport of high a bit rate ODU over lower bit rate ODUs is ineffective. Examples: ODU2 over 5 ODUs, ODU3 over 17 ODU1s or over 5 ODU2s. In addition, monitoring of an ODUg when transported over n ODUj (g>j) cannot be monitored at intermediate points.

Inverse multiplexing for data clients such as ATM or GFP is a different subject that does concern the OTN architecture (the OTN starts at the access point).

# Proposal

G.872 should specify TDM of ODUs. Lower and higher order ODUs should be defined.

Inverse multiplexing of ODUg over n ODUj (g>j) should not be specified.