UIT - Secteur de la normalisation des télécommunications	WD

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Turin, 12 – 15 September 2000

Experts Meeting Q.19/13

SOURCE:	Deutsche Telekom

TITLE:	Management of OTN and WDM

# Introduction

Transparent WDM rings are deployed in Metropolitan Areas (see example in Figure 1). They provide connectivity, transparent and/or client specific interfaces for various clients and, in the normal case, they do not provide 3R regeneration. Furthermore, such rings do not have OTN functions like OTUs and ODUs. In future, they may be able to generate generic AIS at client interfaces, may use propriety kinds of the Digital Wrapper (e.g. synchronous with the client bit rate) or my use the standardized Optical Overhead Signal (OOS).

Figure 1: Example of a Metropolitan Area Network

Due to the transparent characteristic, transparent WDM rings could be interconnected with OTN equipment. An OTUk presented at OTM0.k interfaces could be transported over such rings like other signals (STM-N, Gigabit Ethernet, ESCON, etc). Also existing WDM point-to-point systems may be capable to transport OTUs when the are present at OTM0 interfaces.

Transparent WDM rings and WDM point-to-point systems are not part of the OTN, but part of a multi-technology transport network. As such, the management of the WDM rings should fit seamlessly the management of the transport network.

# Discussion

From a management point of view WDM provides a subset of the OTN functions. The OTS (with or without OOS) and OTS (with or without OOS) should be managed identical in WDM and OTN. Recommendation G.872 describes the OCh as the layer network that provides connectivity and adaptation to various clients. This is also valid for WDM rings. Otherwise, the management of WDM rings has to handle cross connections at client layer such as STM-N (RS), OTM0 (OTU), Gigabit Ethernet and ESCON. Subsequently the transport of any new client would require an additional cross connect capability.

# Proposal

OTS, OMS and OCh should be specified as common layer network in OTN and WDM.

Annex: Illustration of some client connections over WDM and OTN as shown in Figure 1

Figure 2: Examples for the transport of SDH-RS over OTN and WDM

Figure 3: Examples for the interconnection between OTN and WDM

Figure 4: Examples for the transport of Ethernet over OTN and WDM