UIT - Secteur de la normalisation des télécommunications	WD

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Turin, 12 – 15 September 2000

Experts Meeting Q.19/13

SOURCE:	Deutsche Telekom

TITLE:	ASON architecture and interfaces

# Introduction

The relations between OTN and ASON and between different ASON functions need to be defined.

# Discussion

It is expected that OTN devices will be deployed in the network before ASON devices are available. Therefore, ASON has to coexist with an OTN.

There could be switching devices with routing capabilities and devices without routing capabilities, i.e. devices that are just able to request an outgoing connection and/or accept an incoming connection. The later ones may be computer and storages, which are data source and sink but no transfer devices such as switches. Therefore, three different types of functions should be considered for ASON (Figure 1).

OTN-NE: 	A device that provides only permanent connection and/or client adaptation functions. ONNIs that are defined in ITU Recommendations G.709 and G.959.1 interconnect these devices.

TE:	A switching device that is capable to request outgoing connections OR accept incoming connection. ASON-UNIs interconnect TEs with switches. The signalling at the UNI does not include routing functions.

Switch:	A device that provides switched connections. ASON-NNI interconnect switches. The signalling at the NNI includes routing functions.

Figure 1: Conceptual illustration of a switched network

It should be noted that any of these functions and also client functions could be integrated within a single device. For instance, an IP router that is capable to request outgoing connections, accept incoming connections and switch connection for other devices.

Both ASON-UNI and ASON-NNI should base on the ONNI, i.e. ONNI plus a signalling channel of defined protocols.

G.ason should define a switched ODU network, since a switched ODUs are not restricted in distance.

# Proposal

G.ason should define a switched ODU network that coexist with the OTN

G.ason should define TE and UNI function without routing capabilities and switch and NNI functions with routing capabilities.